CHKCHG Copyright (C) 1993 Dimitri Vulis DLV@DM.COM

When OS/2 reads data (such as the root directory or the file allocation table)

from a diskette, and needs to refer to it again soon (e.g., to open another

file), it may not have to re-read it if the same diskette is still in the drive

and the data is still in the memory buffer. To determine whether the diskette

has been changed, OS/2 uses the hardware feature called "diskette change line".

Under normal circumstances, if the disk drive has been opened and closed and

therefore the diskette may have been changes, the diskette drive tells the OS

on a subsequent read that the diskette may have been changed, and the OS will

know the data in memory buffers cannot be reused and must be re-read.

Unfortunately, this mechanism does not work correctly on some clones. Sometimes

the drive fails to report that the diskette has been changed. With two drives,

sometimes diskette change is reported for the wrong drive. Sometimes no media

change is reported when it fact there's no media in the drive and the drive is

not ready. This is also a malfunction, but it seems not to cause problems for

OS/2. OS/2 is also more sensitive than DOS to the drive incorrectly identified

in CMOS configuration (e.g., 1.2MB instead of 1.44MB).

If OS/2 assumes that a diskette has not been changed, when it fact it has, data

may be lost. For example, the root directory or the file allocation table of

the previously inserted diskette may be written into the new diskette.

The program CHKCHG will report the diskette change status returned to the

operating system by the diskette drive. You can open and close your drives and

see whether the program's output corresponds to what is happening in reality.