Nifty James' Famous (Enhanced) Expanded Memory RAM Disk Drive

Version 1.20  of  16 Oct 1987

(C)  Copyright 1987 By Mike Blaszczak.  All Rights

Reserved.  This is a Shareware Program. A $15 donation

is requested.

THIS PROGRAM MAY NOT BE RESOLD WITHOUT THE PRIOR WRITTEN

CONSENT OF THE AUTHOR.  "REPUBLISHING" SHAREWARE PROGRAMS

WITHOUT THE AUTHOR'S KNOWLEDGE IS A CRIME.

Introduction

------------

NJRAMD is a device driver that allows you to partition a

part of your EMS or EEMS memory as an extremely

efficient, extremely fast "RAM disk" drive.  Having a

part of your memory configured as a RAM disk will allow

you to store temporary files, intermediate files, and

other information quickly.

NJRAMD is compatible with the Lotus/Intel/Microsoft EMS

standard, as well as the AST/Quadram/Ashton-Tate EEMS

standard.  NJRAMD will function equally well with both

systems.

This program will also run with any version of DOS,

staring with version 2.00 or later.  I wrote it under

PC-DOS 3.1, and tested it under PC-DOS 2.00 and PC-DOS

2.10.  The program should work with MS-DOS, but it

hasn't been "officially" tested.

NJRAMD takes up very little DOS space -- the program,

depending on the configuration that you use, takes

around 720 bytes of DOS memory, regardless of how large

the disk space is made.  The smallest configuration of

NJRAMD takes under 700 bytes of DOS memory.  Thus, you

can partition a very large RAM disk, and only take up a

very small portion of precious DOS memory.

Use and Installation

--------------------

NJRAMD is installed as a DOS device driver when DOS is

first loaded.  Thus, you must specify the device driver

in the CONFIG.SYS file of your boot disk or diskette.

The entry in the CONFIG.SYS file must be anywhere

*after* the specification for the EMS device driver.

For example, the EMS driver that came with my Taiwanese

EMS clone is called INITIAL.SYS.  I also use ANSI.SYS,

which I specify in the CONFIG.SYS file, also.  Thus, my

CONFIG.SYS file looks like this:

device=\sys\initial.sys

device=\sys\ansi.sys

device=\sys\njramd.sys

|  Starting with version 1.20, this version, NJRAMD also

|  comes with a file specially prepared for computers with

|  extended instruction sets.  This includes processors

|  like the 80186, 80286, and 80386.  You can also make

|  use of this version if you have a NEC V-20 or V-30

|  installed and running in your system.

|  The version is called NJRAMDXP.SYS.  "XP" stands for

|  "Enhanced Processor".  This version of NJRAMD is coded

|  to take advantage of the special, extra instructions that

|  some CPU's have.  It is slightly smaller, and

|  considerably faster than the standard version.  If this

|  version doesn't work on your system, use the regular

|  version.

|  Even if you only have a stock PC with a NEC V-20, you

|  can expect the newver version of NJRAMD to be up to 5%

|  faster in benchmark tests.  On a real 80286 system, the

|  program will run up to 9% faster.

|  See the paragraph just before "Technical Information" if

|  you have a PC's Limited computer.

You might also have settings for the BUFFERS, FILES,

STACKSPACE, and BREAK settings in your CONFIG.SYS file.

These settings can remain unchanged, as they are

independent of NJRAMD.

You can find more information on the uses of the

CONFIG.SYS file in the DOS manual.  The section is

called "Configuring your system".

If you install NJRAMD as shown above, NJRAMD will

default to a disk of 32 EMS Pages, which is 512

kilobytes.  (One EMS Page is equal to 16 kilobytes of

storage space.  16 kilobytes stores 16384 bytes, or

characters, of information.)  If you would like to have

a larger or smaller RAM disk, you must specify the /P

option on the same line as NJRAMD in your CONFIG.SYS

file.  NJRAMD allocates space in one-page increments.

Since each EMS page is 16k bytes long, you can adjust

the size of NJRAMD in 16k increments.

For example, to specify a RAM disk that is 100 pages

long, use this line in your CONFIG.SYS file:

device=njramd.sys /P100

NJRAMD will try to take 100 pages of EMS memory -- this

will give you 1600k of RAM disk space.

Of course, you must have enough free EMS memory at the

time that NJRAMD is loaded to accomodate the requested

size.  If there isn't enough memory, NJRAMD will

terminate with an error, and not install itself at all.

If you have selected a size that is too big to fit, you

will see the message

Device not installed.

Requested size too big to fit.

Remember that if you don't tell NJRAMD otherwise, it

will try to create a 512k RAM Disk.

|  In version 1.20 of NJRAMD, you can use the /A switch to

|  use *all* of the available EMS memory, automatically,

|  instead of specifying a /P option.  Note, though, that

|  the /A option will not leave any memory for other EMS

|  programs, such as disk cache programs or for 1-2-3.

If there is no EMS memory available, at all, when NJRAMD

tries to load, you will see this message:

Device not installed.

No free EMM Memory.

If you see this message:

Device not installed.

The EMM is not installed.

the EMM for your system was not present when NJRAMD

loaded.  NJRAMD depends on the EMM for access to the EMS

memory.  The specification for your EMM driver *must*

appear before the specification for NJRAMD in your

CONFIG.SYS file.

Once you have entered the proper information in your

CONFIG.SYS file, you must reboot your computer so that

the information in the file will take effect.  When

NJRAMD installs, it will print a copyright message, and

some status information reguarding the memory that it

has taken, and the amount of memory that is available as

a RAM disk.

If you have any problems installing NJRAMD, please feel

free to write to me.  I'd be happy  to help!

Clicking

--------

I programmed NJRAMD to click the system's speaker every

time the disk was accessed.  Since a RAM disk is silent,

it's often hard to tell when it is working.  To

compensate for the fact that there is no "whirring" or

"chirping" going on while the RAM disk is active, the

speaker clicks while the program accesses the memory.

If you find this annoying, you can turn it off by

specifying the "/S" option in the CONFIG.SYS entry.  For

example,

device=njramd.sys /P16 /S

would create a 16-page (256k byte) RAM disk that does

not click when it is accessed.  If you specify the /S

option, NJRAMD will print the message

Clicking Suppressed.

when it is installed.

|  If you have an PC's Limited computer, with a Smart-Vu

|  display, try using the program NJRAMDPL.SYS.  This

|  version is the same as the other versions, but it

|  writes to the Smart-Vu display.  When NJRAMD is writing,

|  the display will show "NJ-W", and when it is reading, it

|  will show "NJ-R".  Other than the statements that take

|  care of the display, this version of the program is

|  identical to NJRAMDXP.SYS.

|  If you have a PC's Limited and do not want NJRAMD to

|  access the Smart-Vu display, use the NJRAMDXP.SYS

|  program.

Technical Information

--------- -----------

I have included the assembly language source code in

this .ARC file (or on your white distribution disk) for

your reference.  You do not need it to run the program.

If you are a programmer, or simply have an interest in

"what makes it go", you can take a look at the

NJRAMD.ASM file.  I've heavily commented it, and

therefore will not describe it in greater detail here.

Shareware

---------

This program is a work of Shareware.  If you use the

program, I expect you to end a donation.  I'm asking $15

for the program, but, if you would like to send $5 or

$25, that's fine, too.

Registering your copy of NJRAMD will automatically

entitle you to free updates.  There currently is one

change planned for later release, and that is making the

drive survive "warm boots".  That is, the next version

of NJRAMD will not be erased if you use

<CTRL>+<ALT>+<DEL> to reset your system.

This planned change is unfeasible, however.  Since the

EMS manager overwrites the information in the expanded

memory when it is installed, the RAM Drive's contents

will always be erased after a warm boot.

If you would like to write to me, my address is

Mike Blaszczak

112 Verlinden Drive

Monroeville, PA   15146-2041

I'm also available on Participate On The SOURCE as NIFTY

JAMES.  My SOURCE ID is BCB922, and you're welcome to

send me SMAIL.  I'm also available on the NWI

Conferencing System, as NIFTY JAMES or MIKE B.

Please feel free to send your comments, questions,

suggestions, donations, marriage proposals, and

get-rich-quick schemes.

I'm really interested in hearing any ideas that you

have about NJRAMD, as well as any questions you have

about it.  If you run into any problems, please feel

free to call or write.  (I don't give my phone number,

but, if you're serious enough, call directory

assistance.)

A Special Note

--------------

This program would not have been possible without the

outstanding generosity and good will of Doctor File

Finder and Bob Brody.

Hats off to a couple of wonderful people!

Doctor File Finder managed to find a bug in the first

version that sometimes caused the program to trash the

files on the disk -- the GetSector procedure wasn't

working properly.  The problem has been corrected in

this version, and the program's optimization in the

GetSector proceudre is now fully functional.

Again, the Good Doctor helped me track down the bug that

sometimes caused this device driver to ignore the /Page

option.  Thanks, Doc!  The fix was made, and this is

now Version 1.15.

|  Version 1.20 just let me flex my 80286 knowledge by

|  adding the additional instructions.  The program also is

|  the first version to be developed under Microsoft Macro

|  Assembler Version 5.00.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 800+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.