TIMEPARK

TIMED HARD DISK PARKER

by

ALPHA COMPUTER SERVICE

( Sanford J. Zelkovitz )

714-894-6808

VERSION 1.1   03/19/86

One of the major concerns about using a hard disk is its continu-

ous rotation during its usage.  I, like many other SYSOPs, have

hard disks spinning 24 hours a day, 7 days a week.  Even though

there are many periods in the day that the BBS is being used on a

continuous basis, there are also periods where it isn't used for

some time.  The heads are positioned on the recording surface and

may touch for one reason or another.  Therefore, this program is

one answer to the problem.  What this will do is park the heads of

the hard disk after so much time of non-use.  The heads will be

re-positioned after the next disk access is requested.  The event

timer is automatically reset after each and every hard disk access

( INT 13H ). Therefore, it is quit possible that you may never see

it park during normal operation.  The program will park drive C

and, if present, drive D.

To use the program, simply type or put in your batch file:

TIMEPARK N

Where N is the number of minutes that you will allow before the

disk parking takes place.

Please note that the range of N is 1-9; therefore you can select

one minute to nine minutes before the heads park.  I could have

made the range longer; however, I felt that 9 minutes without

usage was MORE than enough.

An example of a five minute period is:

TIMEPARK 5

Please note that this software was first placed on my BBS on the

13th of March, 1986.

Pro-Tech BBS    Fido 103/506

714-898-8634  300/1200/2400

7 days a week,  24 hours a day

Enjoy,

Sandy

ERRATA:  Version 1.1

The software NOW takes over part of the BIOS 1C interrupt!  Also,

it now performs a seek instead of a verify.  The parking software

is also removed from the interrupt service routine and is a STAND

ALONE memory resident program which is executed prior to the

return to the main program.

In the original version, it was possible under certain applica-

tions to loose time will the disks are parking, in this version,

NO TIME SHOULD BE LOST NOR SHOULD YOU NOTICE A KEYBOARD LOCKUP

WHILE THE DISKS ARE PARKING!