HERCULES VIDEO CARD OWNERS

(AND CGA USERS)

I. HERCULES BACKGROUND

The Hercules Graphics Adapter has proven to be a very

popular alternative to the IBM Monochrome Display Adapter. It

is relatively inexpensive and, in addition to providing the

crisp character display of the MDA, it also allows quite high

resolution graphical displays.

The original Hercules card has been cloned by every

garage in Taiwan, and essentially every non-IBM computer with a

monochrome display has one installed.

Normally, the Herc card may be set up in two modes: the

first mode uses the upper 32K for text/graphics display and

allows a Color Graphics Adapter (CGA) to run in the same system.

The second mode uses the full 64K to provide two pages of

graphics and cannot be used in this mode with a CGA card.  Most

users don't use two monitors, anyhow.

The selection of mode may be made by hardware jumpers, or

it may be controllable with software.

There have been a number of small utilities developed to

enable Herc card owners to utilize the card's graphics capability

to emulate a CGA, thus providing the best of both worlds.  The

two I am most familiar with are SIMCGA and HGCIBM.  Both of these

programs are widely available from shareware houses and BBS's.

II. HERCULES MEMORY USAGE

When a Herc card is used in monochrome mode, it emulates

the IBM MDA.  That is, the only memory available for video use is

the first 4K, from B000 to B0FF - the remainder of the memory,

from B100 to BFFF, or 60K, is available for other uses.

When the Herc card is used with one of the CGA

simulators, it uses the bottom 32K, from B800 to BFFF.  Depending

on the particular program in use, all of this memory may be used

or just the first 4K.  In either case, the upper 32K is still

available, from B000 to B7FF.

When using a Herc card for Hercules graphics, most

programs will try to take advantage of the entire 64K, so no

'extra' memory will be available for other uses.

With the above, admittedly brief, explanation in mind,

you may determine for yourself whether it would be feasible to

use the Herc card's 'free' memory to HILOAD your TSRs up there.

#

By the way, some TSRs let you 'uninstall' them.  It would

seem that you could load some program, say SideKick, up into the

video memory when working on a text application, and then

uninstall it when you want to do graphics work.  My advice is,

don't try it! Uninstalling TSRs is tricky at best, and when

they're loaded outside their 'normal' locations, this process

will almost always fail.

III. CGA USERS

Finally, CGA users may have noted that, when working in

a text mode, thay also use only the first 4K of their memory,

from B800 to B8FF, leaving 28K free for experimentation.  The

same cautions apply here as to the Hercules card when going into

graphics mode.

III. TRY IT OUT

To try out the HILOAD software, you could use the Herc

or CGA 'extra' memory to see if the stuff really works, and then

decide if it would be worth it to get the upper memory in real

hardware.

To see if the memory is addressable, run the utility

MEMPEEK (you'll recognize it by Bernie Tern's logo).  When the

display screen comes up, press "S" which will prompt you for a

segment input.  Type B000 and press ENTER.  The display should

switch to segment B000, and what is displayed will be what is on

the screen at that point.  Then press "S" again, and enter a

segment value of B100.  If the display repeats, then the card

is remapping the memory into the upper 4K, providing what I call

an overly slavish emulation of the IBM MDA.  Exit from the

program (ESC) and either run the Hercules utility you got with

the card, or dig out the documentation to set the jumper

switches.

If all is well, running MEMPEEK at B100 and beyond should

display random characters, indicating that memory is there and

may be addressed but is currently not filled with anything.

By the same technique, examine memory at segment B800 and

beyond. A Hercules card in FULL mode will show real memory at

these locations, while a CGA card should show real memory from

B800 to BFFF.

As a final check, run the program ZERORAM with a block

input of B (i.e, type in ZERORAM B).  This will fill segment

B000 with 64K of zero's.  Running MEMPEEK again should now show

all zeros except in the upper 4K where the text is.

When you are sure that real memory is available and will

not conflict with the video display, you can try loading TSR's in

this newfound space.

#

The utility VIDEOTSR looks at the interrupt vector table

and notices if any of them point to video memory. See the .DOC

file for details.  I occasionally use the video memory to load a

TSR, and my graphics programs are executed from batch files.  This

utility is in all of them to remind me if I have a potential

disaster on my hands.

IV. SET UP THE ENVIRONMENT VARIABLE

As noted in the HILOAD documentation, the program uses a

DOS environment variable to determine where to start loading the

TSRs.  The SET command is used to establish values for the

environment.

A. Using a Hercules card for text mode -

SET HILOAD=B100                 ;starting point

HILOAD                          ;reset program

HILOAD \TSR1                    ;load 1st TSR

HILOAD \DIR1\TSR2               ;load 2nd TSR

etc.

Up to 60K of TSRs can be installed

B. Using a Herc card for CGA simulation -

SET HILOAD=B000                 ;starting point

HILOAD                          ;reset program

HILOAD \TSR1                    ;load 1st TSR

HILOAD \DIR1\TSR2               ;load 2nd TSR

etc.

Up to 32K of TSRs can be loaded

C. Using a CGA card (in text mode) -

SET HILOAD=B900                 ;starting point

HILOAD                          ;reset program

HILOAD \TSR1                    ;load 1st TSR

HILOAD \DIR1\TSR2               ;load 2nd TSR

etc.

Up to 28K of TSRs can be loaded

#

V. FINAL NOTES ON TSR SIZES

Examining the size of the candidate TSR file using DIR

or some such does not give a reliable indicator of the amount

of memory a TSR will consume when actually loaded. The best way

is to load the TSR normally, and then use the MAPMEM utility

provided to see how much memory is actually taken up when in

use. Remember, some TSR's create buffers and such which will not

show up in the size of the program file.

Also, HILOAD provides each TSR with a Program Segment

Prefix (PSP) just like DOS, so even a 10 byte TSR will take up

at least 266 bytes (256 bytes for the PSP + 10 bytes for the

program).  However, HILOAD does not copy the environment to

upper memory, so a small saving is effected here.

Finally, HILOAD aligns programs on paragraph boundaries

(16 byte increments), so as many as 15 bytes per TSR may be

wasted.