Reason Why I Wrote This

Well, I wrote "TEST.WK1" to find out how fast different computers and

different hard disk controller cards would handle file save and retrieves.

I have found that different tests currently available do not give me a true

benchmark.

For example, I was looking at a Paterson Labs controller card that

supports a one to one interweave. According to coretest, the data transfer

rate was well over 400 kb per second vs. the Western Digital at 240 kb per

second with a 2 to 1 interweave. (Approx. 160 some odd with 3 to 1)

The Paterson Labs controller is a XT type, installed in a Wintech AT 12 MHZ

clone with 1.2 meg floppy controller card separate. With a data transfer rate

like that I was going to switch till I tried running this test on my Wintech

AT 12 MHZ computer. The results were quite surprising. The Western Digital was

over 40% faster than the Paterson Labs even though CoreTest Disagreed.

Each AT was equipped with MiniScribe 40 meg. 28 msec. drives.

How Test Works

The file Test.Wk1 is only 9 K, Once you start the test, it automatically

expands to just over 500 K. Once expanded, the timer kicks on and program does

two file saves and one file retrieve. This will give you an idea how long the

system you're testing can save and retrieve 1.5 megs of data. As soon as the

last file retrieve is done, the timer stops and then calculates the results.

This is done this way to minimize the speed of your machine to actual data

transfers and not extra processing time.

Please note that the speed of your computer WILL affect how fast lotus can

accept the data transfer and handles I/O routines. This may be handy to show

your boss when your system tests at 400 % slower !

Bench Mark Times

I ran this test over 100 times and used the results that came up most often as

the Benchmark to calculate any other results. A test with 1.0 performance

index would be identical to this system, and one with a 1.5 performance index

would be 100% faster in doing the routine.

It would vary by one second from time to time due to @second rounding the

results of @now. See Example of test screen below:

========================================================================

Bench Mark on WinTech AT 12 MHZ Clone With MiniScribe 6053 28msec.

Hard Disk & Western Digital Controller Card with 2 to 1 Interweave.

[ File Save & Retrieve Times Omit As Much Macro Time As Possible ]

========================================================================

One Moment Please!

========================================================================

Bench Mark Times

System as Listed Above, 1st File Save Took        : 19 Seconds

System as Listed Above, Both File Saves & File Retrieve Took: 57 Seconds

========================================================================

Your System Times

1st File Save Started :11:17:46 PM |1st File Save Was      : 18 Seconds

2nd File Save Started :11:18:04 PM |Both File Saves and File

File Retrieve Ended   :11:18:42 PM |Retrieve From Start Was: 56 Seconds

========================================================================

Your System Results

Your System Tested at       1.79% Faster !!! Performance Index 1.018

Your System Took          1 Seconds Shorter !!

========================================================================

General Notes:

1. The file Test.wk1 must be in the default worksheet directory.

2. The program saves the 500K file as testbak.wk1 (BE SURE AND NOT

HAVE FILE ALREADY NAMED THAT AS THIS PROGRAM WILL OVERWRITE IT!)

3. The program will automatically delete the file testbak.wk1 when

done with test.

4. You can print out your results to Printer or File by selecting any

of those options in the menu.

5. The program watches for errors such as printer not installed, disk

drive full, memory full, etc. and will automatically abort the

test.

6. This test REQUIRES about 512 k of free ram to work ! (do chkdsk at

DOS prompt is not sure).

7. Requires Lotus 123 rel 2.X

8. Can be used to test floppy drive system if using 1.2 meg floppies

720 K 3-1/2". (The results of that test will make you wonder how

you ever put up with floppies!) Also requires that your Worksheet

Default Directory be A: or B:

I think you will be surprised as to the difference in times reported on

different systems configured with different equipment.

Al Thomas

Compuserve ID # 75746,33