M Y S P E E C H

User-supported software by

Norman  Newman

Kibbutz Tsor'a

M.P.  Shimshon

ISRAEL   99705

Fax:  972-2-908555

Legal note

----------

This program is copyright 1992 by Norman Newman, Kibbutz Tsor'a, M.P.

Shimshon, ISRAEL 99705; all rights reserved, all wrongs deserved.  The program

may be freely distributed and copied, provided that no changes are made in

either MYSPEECH.COM or MYSPEECH.DOC (this  file).  The author denies all

responsibility for any damage caused by the use  or misuse of this program.

This software is provided as shareware. This means that you have the

opportunity to try MYSPEECH before having to register.  Should you decide to

keep and use this program, you are requested  to  register your copy with the

author by sending  a  cheque  for  $20.  This  will entitle you to receive the

source code (written in assembly language) and free support.

This program is "user-driven" and suggestions for additional features are

welcome. This documentation describes version 1.3, which is the current

released version (1/92).

What is MYSPEECH?

-----------------

MYSPEECH is a program designed to help vision-impaired computer users. It is a

Terminate and Stay Resident program (occupying just under 2K memory) which

reads keystrokes and speaks them via the Speech Thing (manufactured by Covox

Inc, 675 Conger Street, Eugene, Oregon USA 97402). This enables the user to

hear exactly which keys s/he has pressed.

Prior to invoking MYSPEECH, you must load into memory the SPEECHV2 driver

program which came with the Speech Thing. MYSPEECH will detect whether the

driver is absent, but is not (presently) capable of loading it into memory. It

is preferable to load both SPEECHV2 and MYSPEECH into memory from the

AUTOEXEC.BAT file in order to save problems.

Program invocation

------------------

MYSPEECH is executed in the following manner:

MySpeech [/?] [/u] [/a] [/r] [/p:n] [/s:n] [/v:n]

The command line switches are not compulsory, and their meanings are explained

below:

/?        Produces a help message

/u        Unactivate MySpeech whilst leaving it in memory

/a        Activate MySpeech after a previous unactivation

/r        Remove MySpeech from memory

/p:n      Set pitch, where n = value, 0 = low, 9 = high

/s:n      Set speed, where n = value, 0 = slow, 9 = fast

/v:n      Set volume, where n = value, 0 = quiet, 9 = loud

MYSPEECH may be invoked several times, although it will be loaded into memory

only once. The subsequent invocations communicate with the resident copy,

changing either the voice parameters, or the function of MYSPEECH (ie

unactivating or activating it).

Cautions

--------

MYSPEECH captures the int 16h keyboard interrupt; should you load into memory

another program which captures this interrupt, you will not be able to free the

memory occupied by MYSPEECH until this latter program is removed. An

unsuccessful attempt to remove MYSPEECH will cause it to be unactivated,

however.

Although this program has been checked on several computers using several BIOS

chips and with all of Covox's products (Speech Thing, Sound Master and Voice

Master), it may be that this program will not work on a specific computer.

Unless you are willing to send me the money to buy an exact copy of your

computer, I am afraid that you will not be able to use this program. The

development of MYSPEECH was stymied for many weeks because it would work on my

development system, but not on my blind friend's computer; this caused several

major design changes until I found one which worked on both, and so it may be

that there are other hardware combinations on which this program will not work.