***** CPLOT.COM

There was a bug in the port initialization. If you typed <ENTER> to accept

the default port (instead of typing <1>), the port would not always be set

correctly. This has been rectified in the included file.

***** PENPLOT.COM

This program is essentially the same as CPLOT.COM except that it will

pause and request pen changes at the appropriate sections of your drawing. This

program should not be used with multi-pen plotters as it will intercept the pen

changes that the plotter would normally receive. It was designed to work with

Houston Instruments DMP-52 plotters and (unlike CPLOT) has not been tested with

others.

The trick is to get pen change requests into your plot file. This is

accomplished by configuring AutoCAD to think you have a DMP-52MP before you

create the plot file. When you enter the pen number assignments, be sure to

make a note of which pen number corresponds to which pen you intend to use.

Only pen numbers 1 - 8 should be used. When the pen change is encountered in

the file, you will be asked to change to the respective pen number, after the

pen stops.

***** BOTH PROGRAMS

The status of bit 4 (CLEAR TO SEND) in the selected port is continuously

checked to determine when to hold up the data transmission. If the program goes

directly into the "WAITING" mode, your port has most likely not been

initialized properly and you will need to break out with a <CONTROL-C> and try

again. Files are plotted in the foreground.

If you are not familiar with plot files, consider the following:

In addition to your computer and disk drives, the location and type of items

being plotted affect the speed of the plot. Arcs, circles, and text are

actually composed of numerous vectors, whereas straight lines are single

vectors and are rapidly transmitted to the plotter. Things that are plotted

near the plotter's origin go much faster than if they were near the opposite

corner.