CLASER

Version 2.03

GRAHAM SYSTEMS

Hewlett Packard LaserJet

RESIDENT CONTROL PROGRAM

Table of Contents

CHAPTER                           PAGE

CLASER                         i

1.   Installing Cartridge Control   1

2.   Starting CLASER                4

3.   CLASER Main Menu               7

4.   CLASER Help                    9

5.   Primary and Alternate Fonts   10

6.   Selecting Cartridge Fonts     12

7.   Selecting Font Attributes     14

8.   Resetting the LaserJet        15

9.   Graphic Print Screens         16

10.  Translation Functions and

Epson FX Emulation            18

11.  Switching Fonts In Text       23

12.  Port Redirection              25

13.  Miscellaneous Functions       27

14.  Exiting CLASER                29

15.  Things That Can Go Wrong      30

CLASER

Hewlett Packard LaserJet

Resident Control Program

CLASER is a control program written for

the IBM PC and PC compatible machines to

control the Hewlett Packard LaserJet printer.

It overcomes some of the problems of using the

LaserJet and allows users to get their money's

worth out of a fine machine. CLASER was written

in an attempt to make use of some of the

features of the LaserJet and to make selection

of these features convenient. It also was

designed to make existing software written to

support the Epson FX printer series compatible

with the LaserJet without modifying software or

patching complicated printer tables.

CLASER is a resident program, which

means that it attaches itself to DOS and in

effect becomes an extension of DOS. Since it is

resident, the program only has to be executed

once at the beginning of a session and it will

continue to be available as long as the PC is

left on and the the PC is not re-booted. CLASER

is accessible by pressing the Alt and the left

shift key and holding them down and tapping the

L key. (L for LaserJet).

<i>

CLASER is a pop-up screen program which

allows you to stop an ongoing program and call

up the CLASER menus. The menus disappear when

you are finished with them and the program you

were in restarts as though it never heard of

CLASER. The CLASER pop-up menus are always

accessible to you, even when in a LOTUS 1-2-3

spreadsheet. You can control you LaserJet and

thereby maximize your investment.

CLASER offers several different

features. It offers font selection supporting

15 font cartridges through the use of the

CLFONT program. CLASER gives control over your

printer ports allowing you to direct printer

output from LPT1: and LPT2: to any other port

or the serial ports. This feature allows you to

shift easily from any printer attached to your

PC to the LaserJet and back again. CLASER

allows control over such features as lines per

inch and margins. It allows the LaserJet to

print in 66 lines per page mode. CLASER allows

you to access the Raster Graphics built into

you LaserJet by giving you the ability to

perform graphics screen dumps much faster than

the DOS GRAPHICS.COM program could dump to an

Epson printer.

One of the most important features of

CLASER is its ability to emulate an Epson

printer. The LaserJet is a good printer, but

most software was written first for the Epson

printers. Getting software to work with any

other printer very often involves making

complicated patches to the printer tables or

buying expensive printer driver upgrades. If

<ii>

you own a LaserJet you need CLASER even without

the Epson emulation, but the Epson Emulation

feature can save many times over the price of

CLASER in software upgrades. CLASER will allow

your Epson based software to work immediately

and without fuss with the LaserJet.

An important feature of CLASER is the

abaility to include a font change flag in any

text file using ordinary editors or word

processor systems. This is done through the

use of the tilde character (~) in combination

with a font number.

The manual which follows goes into

detail on how to use CLASER. I would suggest

you read chapters 1 and 2 and save the other

chapters for later. Try out CLASER and

experiment with the menus. I think that you

will find it easy enough to use that you will

not need 30+ pages of documentation to make it

work. If you are unclear as to what a certain

function does, then you can read the

appropriate chapter in the manual. There is a

built in Help function in CLASER that should

give a good quick answer to most questions. The

manual is pretty much the same as the help

screens except that the manual goes into quite

a bit more detail.

<iii>

CLASER NOW AND THEN

CLASER was written when I received an early

version of the first LaserJet. It is written

for this version and not any LaserJet models

which followed. There are still alot of older

LaserJets out there so CLASER is still useful.

CLASER was sold with moderate success from

mail order ads for about a year and then sat on

the shelf for about a year. I recently dusted

it off and fixed a few things that I didn't like

and I am putting it out as a user supported

product, hoping to extend its life. It represents

about 3 man-months of work so I hope that it

will be appreciated. If I receive any support

I will go back and enhance it. I would like to

add soft font support and EGA support. These

are very easy to add, but I will need some

indication that it is worth the effort. If you

use CLASER, please send in the registration fee.

If you would like to see CLASER improved, drop

into PC-Rockland with your modem at (914) 353-2176

and leave a message for Keith Graham. Shareware

authors have support from fewer than 1% of their

users. Leave me a message to let me know you are

out there even if you can't pay for registration.

Your ideas and comments are as valuable as the

registration fee (although I'd like both).

Thanks,

Keith P. Graham

<iv>

CHAPTER 1

Installing Cartridge Fonts

CLASER.COM comes with Cartridge 92286B

installed, but most users will require another

cartridge option than the B cartridge. There is

a program included on the CLASER disk which

will allow the installation of any one of

cartridges A-H, J, L-N, P, Q, and T. These

cartridges are the only cartridges supported as

of the publication of this manual. Any other

cartridges can be supported by updates to the

system or by selecting individual font

characteristics.

* note: CLFONT.COM is available to registered

users of CLASER for $10.00.

Before installing any cartridge data a

backup copy of the CLASER disk should be made

following the instructions for using DISKCOPY

in the IBM PC-Dos manual. You should put the

original of CLASER away in a safe place.

Installing the font cartridge data is the

process of creating a new version of CLASER.COM

with a different name. The new version will

have the proper commands and the screen menu

for the chosen cartridge. Although you can name

a program anything you want it is convenient to

name the newly created program CLASERn.COM

where n is the cartridge letter of the

cartridge installed. For instance a CLASER

program with cartridge T installed could be

called CLASERT.COM. Instead of using the CLASER

command to start up LaserJet control you would

use CLASERT.

<1>

To install a cartridge you would boot

the PC from IBM PC-Dos and put the CLASER disk

in drive A:. From the A> prompt you would enter

the command:

CLFONT

followed by a carriage return.

When the CLFONT program is loaded in it

asks three questions. It wants to know first

the name of the program to use as a model for

making the new program. Normally this would be

the CLASER.COM program. (If you hit enter it

will take CLASER.COM automatically, saving you

some typing.) Next the CLFONT program needs to

know what cartridge you will be installing. You

would type a T if you were installing the T

cartridge. Then you press enter. Last of all,

CLFONT needs to know the name of the new

program it is creating. If you like the

convention of adding the extra letter to the

end of CLASER then you would just hit enter.

CLFONT will then work a few seconds installing

the menu into the program and making a copy of

the program. When the DOS prompt returns CLFONT

is finished. If you use the DIR command on the

a disk you will see the program CLFONT has just

created.

CLASER is not copy protected in any way,

so you can copy the programs from the CLASER

disk anywhere you like and use them. Don't pass

copies out to your friends. If someone can

afford to pay for a LaserJet printer they can

spring for the cost of their own copy of

CLASER.

<2>

Here is a sample CLFONT session for the

installation of the J cartridge.

A>CLFONT

CLFONT.COM Laserjet cartridge installation

Version 1.1 November 1, 1987

Copyright 1987 by

Keith P. Graham

ENTER PROGRAM TO COPY [CLASER.COM]:

ENTER FONT LETTER [A-T]:J

ENTER PROGRAM TO BE CREATED [CLASERJ.COM]:

CLFONT COMPLETE, PROGRAM SUCCESSFULLY CREATED

A>

<3>

CHAPTER 2

Starting CLASER

CLASER is installed by typing the CLASER

command followed by certain parameters. CLASER

checks to see if it has been previously loaded

into memory (you can't load CLASER twice). If

CLASER is not already resident it then checks

the command line parameters. CLASER expects

certain things to follow the CLASER command. If

CLASER accepts the command line it will load

itself into memory and quietly wait for any

data destined for the printer and ship it off

to the LaserJet.

It is best to start CLASER when the PC is

booted up. This is done by putting the CLASER

command in the AUTOEXEC.BAT file. CLASER.COM is

about 30,000 bytes and should fit on a IBM

PC-Dos backup disk for booting purpose for

floppy based systems. If you have a hard disk

the CLASER program should be either on the

root, or preferably in a Dos or utility

subdirectory which is on the path. CLASER does

not have to be present on the current

subdirectory in order to install itself in

memory, just on the path. If Sidekick is to be

used it must be installed after CLASER.

NOTE: CLASER requires IBM PC-Dos 2.0 or better

to run.

<4>

The command line containing the CLASER

command must have either a 1, 2, or an N

corresponding to the serial port where the

LaserJet is installed. The command line CLASER

1 would indicate that CLASER will redirect

LPT1: to COM1:. CLASER 2 indicates that CLASER

will redirect LPT1: to COM2:. CLASER N

indicates that the CLASER program will do no

redirection of serial ports. you must have

either a 1 2 or n or CLASER will give you an

error and not install itself in memory.

If the CLASER sound effects become

annoying they can be turned off by including

the character q on the command line after the

port number.

Here are examples of two AUTOEXEC.BAT's

which install CLASER (among other things).

For a floppy based system with 2 disk

drives:

ECHO OFF

CLS

REM 2 disk drive autoexec.bat

PATH A:\;B:\

DATE

TIME

REM install CLASER with no redirection

CLASER n

<5>

For a Hard disk based system with

subdirectories for DOS, Batch files, Utilities

and Sidekick.

ECHO OFF

CLS

REM Hard disk based autoexec.bat

PATH C:\DOS;C:\UTIL;C:\BATCH

REM install all memory resident utilities

REM before Sidekick

REM resident program for clock card

SETCLOCK

REM make the print spooler resident

REM CR is a file created with EDLIN which

REM has nothing but one blank line.

PRINT <CR

REM install CLASER for COM1: quiet mode

CLASER 1 Q

REM initialize COM2: for another serial printer

MODE COM2:1200,E,7,1

REM install Sidekick last

CD \SK

SK

CD \

CLS

ECHO AUTOEXEC.BAT COMPLETE

VER

<6>

CHAPTER 3

CLASER and the Main Menu

After CLASER has been made memory resident

it is possible to access the functions of

CLASER by pressing the key combination of

Alt-Left Shift and the letter L. Press the Alt

key and the left shift key (the left shift key

is on the left and for the purposes of CLASER

is different from the right shift). Hold both

the keys down and tap the L key. The CLASER

menu should appear with a cheery bloop sound.

Sometimes CLASER will not appear and the reason

is, either it isn't installed, or the program

that you are running is using the Alt-L

sequence for its own purposes. Some programs

such as MultiLink do not recognize the

combination of Alt and Shift and will not allow

CLASER to start. If you exit the program you

can use CLASER. In most word processors which

use Alt-L you can usually exit to one of the

menus and use CLASER there. CLASER's main menu

will normally appear whenever the Alt-Left

Shift-L sequence is pressed no matter what

program is running at the time.

Almost all CLASER functions are controlled

by using the function keys. The function keys

are listed down the left of the menu and the

function they control are on the right. Many

function keys will give a sub-menu from which

to select a function by use of the function

keys. The function keys always apply to the

most recent screen, even though new menus

<7>

appear, the most recent one is the one that

applies.

If you select an option, an asterisk (*)

appears next to the function. In many cases the

asterisk will go away if you leave CLASER or

select another option. For instance, that

cheerful bloop is annoying to many people (I

can't imagine why) and can be toggle off. If

you press F6 from the CLASER main menu, the

sound will no longer be heard, and an asterisk

will appear next to the F6 indicating that the

sound is toggled off. If you hit F6 again, the

sound will come back and the asterisk will

disappear.

Hitting the Esc key or F10 will exit you

back to your program.

<8>

CHAPTER 4

CLASER HELP

Pressing the F1 key from the CLASER

main menu gives a series of help screens. The

help screens cover in a very brief way some of

the information contained in this manual and

the Hewlett-Packard LaserJet manual. The help

is not designed to be a replacement for the

hard copy documentation, but it acts instead as

a quick reference and a tutorial on certain

topics. In addition to general help, the help

screens have a listing of the LaserJet error

codes that you may encounter. It does not

include the error codes that are written on the

front of the LaserJet itself and it does not go

into detail as to possible causes or fixes for

error codes.

Help screens are too big to fit on 40

column displays and will not appear on them. If

you are in 40 column mode and wish to have help

you must return to an 80 column mode before

help will work.

<9>

CHAPTER 5

Primary and Alternate Fonts

On main menu options F2 and F3 a window

will come up and ask your preference as to

which font the selections will apply to,

primary or alternate. This question applies to

the LaserJet's ability to use 2 fonts a once

and shift back and forth between them without

resetting any of the other characteristics of

the printer. The main font is called the

primary font and the font that you can switch

to is called the alternate font. You usually

will want to make changes apply to the primary

font.

You can switch between primary and

alternate fonts (after they have been set) by

using characters shift-in and shift-out (SI and

SO). These characters are usually imbedded in

the text to be printed by the word processor.

On the Epson SI will start condensed print

mode. (in Lotus 1-2-3 it is the setup string

015). On the Epson SO will start enlarged print

mode.

On the LaserJet the SI and SO characters

are used to flip back and forth between

selected fonts. SO begins the alternate font

and SI returns to the primary font.

<10>

There is one warning concerning the

selection of fonts. If you select 2 fonts which

are incompatible with one another (eg. primary

font landscape and alternate font portrait)

then one font will default out. The LaserJet

tries to make the best fit of the attributes

you want to the attributes it can give you, and

some of the attributes must be the same for

both fonts (such as orientation). If the

LaserJet cannot give the combination of

characteristics which you requested, it will

cause the incompatible characteristics to

default to the last valid selection.

The following are the various ways of

encoding SI and SO.

CHARACTER       HEXADECIMAL   DECIMAL

SI              0F          015

SO              0E          014

SO is can usually be entered as a Ctrl-O. SI

can usually be entered as Ctrl-N. Your word

processor, however, may have decided to use

these control codes for other purposes. You

must consult you word processor documentation

to determine the best way to enter SI and SO.

(Note: the LaserJet Manual will refer to the

alternate font as the secondary font.)

<11>

CHAPTER 6

Selecting Cartridge Fonts

Choices F2 and F3 from the main menu allow

you to choose primary or secondary font

attributes, the difference is that the F2

screen can be customized using the CLFONT

program specifically for the cartridge that you

plan on using. F2 allows the selection of any

of the fonts on the cartridge you selected when

you installed CLASER. If you see the Cartridge

B menu when you press F2 and you have any other

cartridge then you must read chapter 1:

Installing CLASER.

Only one version of CLASER can be loaded

into memory at one time, this is to prevent

memory from being loaded up with LaserJet

control programs to the exclusion of any other

program. This means that you can only specify

one cartridge using the F2 function. If you

have more than one LaserJet cartridge you can

only control one of them using the F2 function.

You must re-boot and load another version of

CLASER to get convenient control over another

cartridge. Re-booting is not practical for most

users. My suggestion is to install the

cartridge most often used as the CLASER

cartridge. You can then use F3 (see chapter 7)

to select the attributes of other cartridges

when you find it necessary to switch

cartridges. It will not be necessary to select

all of the attributes of the required font, and

most selections will take no more than four

<12>

extra keystrokes. F3 may be slightly less

convenient yet the cost in terms of memory

usage in adding extra screens makes up for the

inconvenience.

If you have no cartridges there is no

reason to install a cartridge screen. The B

cartridge screen (which is the default) has the

options for selecting between the two LaserJet

internal fonts. Just ignore all options except

the landscape and portrait courier fonts.

Use the Escape key (Esc) to exit from the

font selection screen.

<13>

CHAPTER 7

Selecting Font Attributes

Pressing function key F3 from the CLASER

main menu allows the selection of fonts by

attribute. This function is useful for

selecting very quickly a certain attribute

without selecting a whole font. For example, if

you want to use the font which is currently

active, but you would like to switch to a

landscape orientation then you could use this

menu rather than the font selection menu.

Another reason for using F3 would be to select

fonts which are in a different cartridge than

the cartridge supported by F2.

Font characteristics should be set in

the order in which they appear in the menu. The

LaserJet tries to give a font which is the best

fit to the characteristics currently selected.

The LaserJet's best fit is based on priorities.

The F3 menu is listed according to the

LaserJet's priorities with the most important

items first. Always select the font attributes

in the order shown working from F1 down.

Another reason for using the F3 function

would be in the cases where a font cartridge

overrides the default internal courier font. In

this case it is easy to return to the internal

font just by specifying the USASCII character

set.

<14>

CHAPTER 8

Resetting the LaserJet

Pressing F4 from the main menu results

in sending the Esc @ string to the printer. In

the Epson, this results in a reset of all of

the selected options. If CLASER is emulating an

Epson, Esc @ will be translated to the LaserJet

Esc E reset command. The two forms of reset do

different things to the Epson and to the

LaserJet.

When you reset an Epson it reverts to

its power on characteristics such as normal

font and 6 line per inch. When the LaserJet is

reset, it also reverts to its power on state.

The problem is that a LaserJet will also print

the current page if there is one and then issue

a page eject. The Epson does not eject the

page. Remember emulation does not mean that all

things will be the same.

<15>

CHAPTER 9

Selecting Graphic Print Screen Modes

CLASER allows you to do graphic screen

dumps to the LaserJet. This feature can be

disabled, or redirected by using choice F7 from

the main menu. Normally pressing the PrtSc key

and the shift key will result in the text

characters on the screen being dumped to the

printer on LPT1:. In addition to this built in

feature of the PC, CLASER will allow you to

dump graphic output to either LPT1: or LPT2:.

When graphic screen dumps are enabled,

CLASER will sense the screen mode of the PC's

color graphics adaptor. If the PC is in a valid

graphics mode, the screen will be dumped, using

LaserJet raster graphics to the printer port

specified in the F7 menu. If the PC is not in a

valid graphics mode or the graphics print

screen function is disabled then the normal

print screen function takes over. (This is the

function which comes with your PC that only

goes to LPT1:.)

There are times when programs either

disable the PrtSc key or they use a

nonstandard way of developing graphics. If a

program has turned off the print screen

function and Shift PrtSc no longer works it is

still possible to dump screens, graphic or

text, through the us of Alt-Left Shift-P. This

does the same thing as pressing Shift PrtSc

with the exception that programs can't turn it

off as easily as they can turn off the Print

Screen function.

<16>

Lotus 1-2-3 graphics screens are

developed in a nonstandard way and the print

screen function is disabled. Pressing the

Alt-Left Shift-P key combination results in a

text screen dump which is a because CLASER can

not sense that the screen is in a graphics

mode. In the Lotus 1-2-3 graphics screens (and

only Lotus 1-2-3 graphics screens!) you may

press the Alt-Left Shift-G combination to force

a screen dump. Forcing a graphics screen dump

allows you to dump graphic screens which were

not previously accessible to you. Alt-Left

Shift-G is not discriminating! It will force a

graphics dump of non-graphics screens which

will result in a garbled mess, both on the

screen and on the paper. Do not use Alt-Left

Shift-G unless you are certain that the screen

you are looking at is a color graphics screen.

If you use the Alt-Left Shift-G

combination on a text screen, be patient.

CLASER will return the screen to its original

condition when it is finished. If you just wait

and watch. It will be done in a few minutes.

Save the LaserJet output as a reminder of what

happens when you don't read the instructions.

<17>

CHAPTER 10

EPSON FX Emulation

Option F5 on the main menu allows you to

toggle Epson FX emulation on and off. CLASER

emulates many of the functions of an EPSON FX

(see limits below), but assumes that only one

LaserJet is attached to the PC at one time. For

this reason, emulation takes place on LPT1:.

You must specify to your software that the

LaserJet is on LPT1:. If you have need to hook

up some other printer to the port normally

assigned to LPT1: you may use function F8 to

swap ports (see chapter 11).

The LaserJet and the Epson Fx series of

printers are very different pieces of hardware

and deep down are not compatible in many of

their features. CLASER tries to iron out some

of the very obvious differences between the two

types of printers, but it cannot change the

hardware. Certain things that the Epson

printers do a LaserJet cannot be forced to do.

Some of the functions of an Epson are

impractical to try and duplicate on the

LaserJet.

CLASER will try its best to duplicate the

functions of an Epson Fx printer, including

graphics with the following exceptions: CLASER

does not attempt to emulate the Epson's ability

to redefine characters. CLASER does not attempt

to emulate nine pin graphics. It was felt that

the LaserJet has much better character sets

than the matrix graphics characters of the

Epson and to have the LaserJet emulate these

<18>

functions would be to a step backwards. CLASER

would have to be nearly 3 times the size that

it currently is, just to have room to remember

the combinations of characters that it could be

sent. Nine pin graphics is rarely implemented

because it is awkward to manipulate the 9 bits

of data. Most users will never miss this

function.

On the following pages is a list of all

of the Epson Functions. An X next to the

function means that the function is emulated,

an I means that the function is captured and

not acted upon. A P means that CLASER passes

the function to the LaserJet unaltered.

<19>

CLASER        EPSON FUNCTION

===========================================

X          Carriage return

P          Line feed

P          Form feed

X          Tentative n/216 line spacing

X          Reverse feed

I          Set Elite character set

I          Set Pica character set

P          Shift out

I          Cancel enlarged mode

I          Set enlarged mode

P          Shift in

I          Condensed mode cancel

X          Escape shift out

X          Escape shift in

X          Underline mode

I          Emphasized mode

I          Cancel emphasized

I          Double strike

I          Cancel double strike

I          Superscript/subscript

I          Superscript/subscript cancel

I          Mode selection

P          Backspace

X          Alternate character selection

X          Alternate character cancel

I          Internat. character Selection

X          1/8 line spacing

X          7/72 line spacing

X          1/6 line spacing

X          n/216 line spacing

X          n/72 line spacing

X          Horizontal tab

X          Horizontal tab set

<20>

CLASER        EPSON FUNCTION

===========================================

I          Vertical tab

I          VFU channel selection

I          Set vertical tab

I          VFU position setting

X          Form length setting

I          Skip over perforation

I          Skip over perforation cancel

X          Right Margin

X          Left Margin

I          Select printer

I          Deselect printer

I          Delete

I          Cancel

I          MSB  0

I          MSB  1

I          MSB control sequence

I          Download character definition

I          CG select

I          ROM cg copy

I          Expand printable code

I          Cancel expand printable code

I          Control code select

P          Bell

I          Paper end detector disable

I          Unidirectional printing

I          Half speed printing

I          Incremental printing

X          Initialize

X          Normal density graphics

X          Dual density graphics

X          Quad density graphics

X          Bit image selection

I          Bit image mode assignment

I          9 pin graphics

<21>

CHAPTER 11

Port Redirection

Choice F8 from the CLASER main menu gives

you control over printer ports LPT1: and LPT2:.

Ports LPTn are normally assigned to the

parallel ports. These are the ports on the IBM

monochrome printer adapter and the IBM printer

adapter. With CLASER you can capture

information before it gets to the printer port

and send it out one of the serial ports. Serial

ports are usually used for communications, but

they can be used for printers with serial

interfaces. The LaserJet has a serial interface

and requires that the serial port communicate

at a speed of 9600 baud. When you start up

CLASER using option 1 or 2 the appropriate

serial port is initialized to the correct

speed.

F1 through F4 control printer port

reassignment to serial ports. You may reassign

the normal printer assignments of LPT1: and

LPT2: and have them go to either the COM1 or

COM2 serial ports. You can have both LPT1: and

LPT2: going to the same serial port if you

like.

F7 and F8 allow you to initialize the

serial ports. If you assign a printer to a com

port it is likely that the com port is not

correctly initialized to 9600 baud so that the

LaserJet can recognize the data being sent to

it. If you are using CLASER and try to print

<22>

something and the LaserJet gives an error code

40 it means that the serial port was not

correctly initialized and the LaserJet can't

read the information being sent to it. You must

always initialize the serial port that the

LaserJet is attached to when you select that

the printer will be redirected to the serial

port.

In order to test the printer you may use

option F9 from the port control screen to print

a test line out LPT1:.

Option F9 and all CLASER control commands

expect to talk to the LaserJet using LPT1:.

CLASER always assumes that it is talking to

LPT1:. Sometimes it is necessary to have

another printer on LPT1: and use the LaserJet

on LPT2:. In these case you can use F6 to swap

port assignments. All that this does is

whenever CLASER sees data going to LPT1: it

makes the data go to LPT2: and vice versa.

(This can be used to have the PC's normal

screen dumps go to printer on LPT2:).

It should be noted that CLASER will not

recognize any control function, including Epson

emulation, on LPT2:. Use F6 to swap printers if

the LaserJet is to be attached as LPT2: before

issuing any control commands.

<23>

CHAPTER 12

Miscellaneous Functions

Option F9 from the main menu allows the

control of several diverse but handy items.

Normally you would use the setup abilities of

you word processor to control margins and lines

per inch using the Epson emulation features of

CLASER, but sometimes it is necessary to issue

a command to the CLASER directly to override

the word processing options or to do things

such as print ascii text files. F9 allows for a

a few commonly needed functions.

F1 sets 66 lines per page. This is useful

because an Epson is normally set to 66 lines

per page. This function is not automatically

set when Epson emulation begins and must be

specified from this screen. Since a LaserJet

cannot print 66 lines per page at 6 lines per

inch and normal margins, the lines per inch is

changed and the top and bottom margins are

reduced. This should be rememberd if any

subsequent printing is to be done where it is

expected that the printer be set at 6 lines per

inch.

F2 sets 6 lines per inch.

F3 sets 8 lines per inch.

F4 sets 12 lines per inch.

<24>

Font selection (F2 from the main menu)

does not change the lines per inch, and some of

the smaller fonts will look much better at 8

lines per inch. 12 lines per inch is for when

you use very small fonts, but can be handy if a

double spaced document is to printed as single

spaced without altering the document.

F5, F6 and F7 give you three common

options for setting the left margin. The

LaserJet normally prints a document too far to

the left to allow room for punching holes for

putting documents in binders. F6 or F7 should

leave room for punching holes in pages without

ruining the document.

F8 gives a line feed.

F9 gives ejects the page. F9 is handy to

eject the last page of documents that get left

in the printer without getting up and pressing

the buttons on the LaserJet. I find I use this

function very often so I made it easy to get

to. Press Alt-Left Shift L, press F9 twice and

F10 twice and you are back to where you started

and the page is ejected. Use F9 when printing

Lotus 1-2-3 spreadsheets before using the 1-2-3

align option on the 1-2-3 print menu.

<25>

CHAPTER 13

Exiting CLASER

Once you get into CLASER, as much fun as it is,

you may want to escape from the windows. Escape

will always get you out of any situation. Hit

escape a few times and you will exit CLASER.

You must back out the way you got in. CLASER is

3 screens deep at the deepest so you would have

to hit escape for each menu that you passed

through to get where you are. F10 will

sometimes work to exit you from the current

menu, but some menus require that F10 be used

for control functions and in these cases you

must use the escape key to exit CLASER.

REMEMBER: Don't Panic - Escape

It is also possible, beginning with version 2.03

to uninstall CLASER. This is done by pressing

the key combination of CTRL and K when at any of

the CLASER menus (not help screens). CLASER will

restore DOS to the condition it was in when CLASER

was started. This will kill any programs running

as well as any memory resident programs which were

installed after CLASER. You may want to be careful

about uninstalling CLASER while in another program.

Some files may not be closed correctly if you do

this. It is best to exit a program and uninstall

CLASER from the DOS prompt.

<26>

CHAPTER 14

Things That can Go Wrong

From time to time there will be problems

as a result of using CLASER. Most problems come

from compatibility problems of CLASER with

other software or misunderstandings as to how

CLASER works. Here is a list of some of the

common problems.

PROBLEM: The computer stops or "hangs up"

when a package is loaded in. The package never

hung the system before and still works fine

when CLASER is not loaded.

SOLUTION: Some packages are not very careful

about how they use the various methods to

capture characters from the keyboard. The main

place where this can be a problem is when

CLASER is run alongside of Borland's Sidekick.

CLASER should be loaded in before other

keyboard type resident programs. Sidekick

should be loaded in last. Check the memory

requirements of your program. CLASER takes up

about 30k of memory. Your program may be

bumping its head against the top of memory. If

you have 256K of memory or less I would suggest

that you buy a memory expansion board for you

PC.

PROBLEM: The Alt-Left Shift-L sequence does

nothing, yet I know CLASER is there.

SOLUTION: Some programs grab the keyboard

for their own uses, but they give it back when

they are done. Multi-Mate and some other word

processors use Alt-L in their program.

<27>

CLASER is always benign and will allow all

other programs to have first crack at a key

that is typed. These programs get the Alt-L

first and don't pass it on to CLASER. The

CLASER menus will be accessible from the menus

in these programs and should always be

available from IBM PC-Dos.

Another possible problem is that CLASER may

be busy printing. CLASER will not interrupt

itself in the middle of Epson emulation. If you

interrupted a program that was printing by

using the Ctrl-Break key you may have confused

CLASER and CLASER will be waiting for some more

data. The Solution is to try a PrtSc an see if

the printer works. If it does not, try again.

You should not interrupt CLASER and a print or

graphics program while they are working

together.

CLASER will not display its main menu on a

graphics screen. If you are in graphics mode,

CLASER will know it and ignore the Alt-Left

Shift L sequence. Return to Text mode and

CLASER will work.

PROBLEM: When I press Alt-Left Shift-L I

get a clock like tick.

SOLUTION: The click is Sidekick getting

upset about CLASER. Make sure that CLASER is

always loaded in before Sidekick.

PROBLEM: I get the CLASER Main menu but F1

Help does not work.

SOLUTION: Help screens contain too much

information to fit on a 40 column screen.

CLASER will show all the other menus, but Help

will not work in 40 column mode. Change to an

<28>

80 Column mode and F1 will give you the help

you need.

PROBLEM: I get graphics but they are

distorted. Circles look like eggs.

SOLUTION: The HP LaserJet is not an Epson

and was not designed to be Epson compatible. By

trying to make it Epson compatible certain

compromises had to be made. The differences in

the density of the Epson and LaserJet graphics

results in differences in aspect ratios. Most

good graphics packages allow you to adjust the

aspect ratio. To get good graphics out of your

LaserJet you will have to determine the density

of the graphics, the page size and the aspect

ratio. The vertical density of the dots on the

LaserJet will be the same as the Epson, that is

48 dots per inch. The horizontal density will

be different from the Epson. The LaserJet will

give either 50, 75, 150 or 300 dots per inch.

The corresponding Epson densities are 60, 80,

120 and 240. The Epson normal density yields

graphics on the LaserJet that are 20% wider.

CRT graphics are 6% fatter. Double density

graphics and high resolution graphics are 20%

thinner than Epson graphics. You must adjust

your page size, density and aspect ration until

the graphics looks good. This may require

experimentation.

PROBLEM: The LaserJet waits for graphics

but after a few minutes flashes a 20 or 21.

SOLUTION: The LaserJet has limited ability

to do graphics, especially in high density

modes. You have exceeded the amount of graphics

that the LaserJet can handle at once. You must

<29>

use a lower density graphics or clean up the

graphics page. CLASER intercepts white space

and does not send it out to the LaserJet. You

will find that graphics with more white space

look better and print faster. One thing that

has cleared up a lot of the error 20 problems

is not using page borders in you graphics. The

page borders are useless and I find that it

limits the impact of graphics if you put them

in a box.

New LaserJets with more memory and memory

upgrades for the older LaserJets are becoming

available, and you may consider getting the

upgrade if you do much graphics.

For the time being you can press the

continue button on the LaserJet and continue

with the graphics on the next page.

PROBLEM: Alt-Left Shift-G gives very

strange results. The screen goes all screwy and

the LaserJet prints strange bars and lines.

SOLUTION: Alt-Left Shift-G is only used for

graphics screens such as the Lotus 1-2-3

graphics where Shift-PrtSc does not work. It is

an override type of function and will attempt a

graphics dump of any screen. But Alt-Left

Shift-G has no conscience and does not care if

you really do have a text screen, it treats it

like a graphics screen. If you wait it will

finish all by itself in about a minute. Please

don't press the Alt-Left Shift-G again unless

you are sure that you have a graphics screen.

PROBLEM: I have a text with imbedded

LaserJet Escape codes. When I print it I see

strange combinations of characters where the

<30>

codes were placed, but the LaserJet does not

change fonts as I have asked it.

SOLUTION: When CLASER is emulating an Epson

it does not recognize LaserJet escape sequences

(just as your Epson would not recognize them).

You must use menu option 5 to turn off Epson

emulation.

PROBLEM: I am using a word processor that

works very well with my LaserJet without

CLASER, but prints garbled data at the top of

each page when I am using CLASER.

SOLUTION: The word processor is using

LaserJet escape sequences to control the page

and print options of the LaserJet, but CLASER

is trying to convert these commands and getting

confused. Either turn off Epson emulation using

CLASER menu option 5 or tell your word

processing package that you are attached to an

Epson.

PROBLEM: I have a graphics program which

works with the Epson which sends garbage to the

LaserJet or does not work at all.

SOLUTION: CLASER does not emulate 9 pin

graphics (which is rarely used) or downloadable

characters (which is used in packages such as

Lettrix, Sideways and a few other packages).

The Epson downloadable fonts feature was not

supported for three reasons. First, they do not

look good on the LaserJet and I can't imagine

anyone preferring them to the LaserJet

cartridge fonts. Second, Hewlett-Packard is

supporting its own downloadable fonts on the

new model LaserJets which are not compatible

with Epson type fonts. Third, supporting

<31>

downloadable Epson fonts would have added more

than 60K to CLASER's size making it too large

for a memory resident program. I would suggest

that you rely on the LaserJet font cartridges

for extra character sets.

PROBLEM: Graphics come out with ragged

edges and broken lines.

SOLUTION: CLASER is trying its best to

emulate Epson Quad density graphics and is

failing. The LaserJet cannot position a dot to

the precision that is necessary to perfectly

emulate Epson Quad density graphics. The ragged

edges are accumulated rounding errors from from

your graphics package sending out multi-strike

lines to the LaserJet. The solution is to

always use low or medium resolution graphics

when given a choice. Packages such as

Select-A-Font can produced fuzzy looking

results in high resolution modes.

PROBLEM: IBM DisplayWrite 3 will not work with

CLASER. It gives a message telling me that it

doesn't have enough free memory.

SOLUTION: IBM DisplayWrite 3 needs all of 256K

to run. If you have DisplayWrite, I would

suggest that you buy a memory expansion board

for your PC.

PROBLEM: The CLASER menu is not available from

within IBM DisplayWrite word processing

packages.

SOLUTION: It's not DisplayWrite's fault, but

the KQE.COM keyboard enhancer. Try running

DisplayWrite without the KQE command (look at

the DW3.BAT file and make the KQE commands into

remarks).

<32>

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 1,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 721-6104 or (713) 721-5205