FONTVIEW Documentation

(Version 3.0)

June 19, 1988

FONTVIEW is

Copyright (c) 1988

by

S.H. Moody & Associates, Inc.

1810 Fair Oaks Ave.

South Pasadena, CA 91030

All Rights Reserved.

FONTVIEW reads HP-specification LaserJet downloadable PORTRAIT or

LANDSCAPE orientation font files and displays the magnified characters

in the file on your EGA screen in increasing ASCII order.  You must

have an EGA display card installed in your computer for FONTVIEW to

function.

FONTVIEW automatically magnifies the font character images as

much as possible while retaining the font's original horizontal and

vertical proportions on the screen.  User options include manually

setting X and Y magnifications, setting the display time of each

character before proceeding to the next character, and overriding the

default grid-like display to instead display characters as solid

figures.

FONTVIEW and the accompanying FNTCLEAN are not free, but

Shareware.  If you find FONTVIEW or FNTCLEAN useful, you are requested

to forward $17.00 to the above address to register your copy and

receive two useful additional font programs (see REGISTRATION, below).

USING FONTVIEW

The syntax for running FONTVIEW is:

FONTVIEW  fontfile.ext [/S] [/Mx,y]

Where the parameters in brackets are optional and may be omitted

from the command line.

"fontfile.ext" is the complete name of the font file you

wish to view.  DOS paths are supported if you are using DOS

2.0 or higher.

/S  tells FONTVIEW to display solid characters on the

screen.  The default for the program ( no "/S") is to

display characters using a dot plot shape which includes a

background-colored line on the top and right sides to allow

you to clearly distinguish the individual dots forming the

character.

/Mx,y  allows you to manually set the screen magnification

in the X and Y directions instead of having FONTVIEW

determine these values.  The values to substitute for "x"

and "y" specify the amount of magnification to use in the

direction indicated.  For example,  /M9,6  tells FONTVIEW

to display 9 dots horizontally on the screen for each

horizontal dot in the font file, and 6 dots vertically for

each vertical dot in the font file.

After FONTVIEW displays a character, pressing any key causes the

program to proceed to the next character.  All characters are

displayed in increasing ASCII order, regardless of their order in the

font file.  If you want to view all the characters in a file without

manual intervention, you can hit any of the numbers between 0 and 9

and FONTVIEW will display a character, pause for the number of seconds

you have input, and proceed to the next character.  To shut this

"auto-display" feature off once it has begun, merely hit any non-

numerical character and FONTVIEW will again await a keystroke before

proceeding to the next character.  To change the amount of delay

between the display of characters, just press a different numerical

value and the program will use the new value after the next character

display.

To view a particular character in a font file, press "E" or "e"

(for ENTER Character) and at the lower left corner of the screen the

program will prompt you to enter the character from the keyboard.  To

view characters not on the keyboard, press and hold down the ALT key

while entering the character's ASCII number from the NUMERIC KEYPAD,

then release the ALT key.  The screen will display the appropriate

character.  Following the display of the character, FONTVIEW will

resume its display sequence order beginning with the character with

the next higher ASCII number.  If you change your mind after pressing

"E", merely hit RETURN without entering a number and FONTVIEW will

resume its display sequence from where it left off.  If you enter a

character or ASCII number of a character which is not in the font

file, FONTVIEW will beep, display "NOT THERE.  TRY AGAIN" for two

seconds then prompt you to "Enter Character" again, all in the lower

left hand corner of the screen.

To exit FONTVIEW, hit the Escape key and you will exit after the

next character has been displayed.  Upon exiting, FONTVIEW will

display the LaserJet escape code sequence to use to call the font into

use after it has been downloaded.

To check that FONTVIEW is compatible with your hardware, I

suggest you first try FONTVIEW on a file which is known to be

specified correctly.  The file DEMO.SF2 (included in this archive) is

a twelve character (6 portrait, 6 landscape) font file which will

establish that your hardware is suitable.  Type FONTVIEW DEMO.SF2 to

display the 6 characters ("A", "B", "W", "p", "x" and "y"), first in

landscape, then in portrait orientation.  DEMO.SF2 is not a valid

LaserJet font file since it contains a mixture of portrait and

landscape characters.  The landscape characters are stored in the file

as ASCII characters 1-6 and so appear first in the display sequence.

THE FONTVIEW DISPLAY SCREEN

The display window is divided into two elements: a text portion

along the left side, and a graphics window occupying the rest of the

screen.

THE GRAPHICS WINDOW

Inside the graphics window you'll see a magenta rectangle within

which the magnified character is displayed.  This rectangle, or cell,

displays the boundaries within which all characters in the font

normally fit.  Also shown within this font cell are two horizontal

lines.  The lower, solid line is the baseline which is the bottom row

of dots for most characters.  For example, the bottom of the "A", "B",

and "x" of DEMO.SF2 coincide with the baseline.  Descenders fall below

this baseline.  In many fonts (particularly the larger fonts) the

bottom of the "C", "G", "O" and others extend up to a few dots below

the baseline to improve their attractiveness.  The second, dotted line

shown in the cell is the "x-height" line.  The top of the lower case

"x" should coincide with the x-height line while the bottom of the

letter coincides with the baseline.  The x-height is generally the top

of lower-case characters except for those with ascenders ("b", "d",

etc.), however, again in many typefaces the top of the "c" and "o" and

the top of the loops in "b", "d", "p" and others often extend 1-3 dots

above the x-height line to improve their appearance.

Finally, and most important, the magnified character is dis-

played.  Characters are placed within the character cell using the

"left offset" value stored in the character header unless the left

offset value is 0 or negative.

THE TEXT WINDOW

The left, text portion of the screen presents data found in the

font and character headers.  HP-specification laserjet soft fonts are

comprised of three types of data:  first is the FONT header, which

contains information for the font as a whole; second are CHARACTER

headers (one for each character) which contain width, height, and

spacing information for each character; and finally, the bit image

data for the character.

FONT DATA

The top portion of the text portion of the screen displays a

portion of the font header information.  This information is displayed

once for each font, and thus will not change as you view each

character.

The first line shows the DOS filename of the font which you

entered on the command line.

The second line shows the name of the font if it is included in

the header.  Often, particularly with shareware fonts, this

information is not included so nothing will appear after "NAME:" on

this line.

The third line shows the typeface (Times Roman, Helvetica,

Optima, Garamond, etc.).  This information is decoded from the font

header.  HP has assigned codes for over 20 typefaces, which I have

incorporated into FONTVIEW.  Nonetheless, many soft fonts put a code

value in the header which FONTVIEW does not recognize. in this case,

"UNKNOWN" will appear on this line.

The fourth line Shows the orientation of the font, either

"Portrait" or "Landscape".

The fifth line shows the spacing of the font and should be either

"Fixed", or "Proportional".

The sixth line shows the symbol set of the font, which identifies

which character set the font is using (USASCII, PC-8, HP Roman

Extensions, etc.).  Again, HP has assigned codes for over 30 such

sets, which are included in FONTVIEW, but if the font maker uses a

code which is not recognized, the word "UNKNOWN" will appear on this

line.

The seventh line, "Points", shows the height of the font, in HP

points.  HP fonts are defined based on 72 points per inch, as opposed

to the standard printer's point which is 1/12 of a Pica or 72.27 per

inch.  The body size of a font, in points, is the vertical distance

measured from the bottom of a descender, such as the stem of a "p" to

the top of an ascender, such as the stem of a "b".  To convert points

to 300 dots/inch dots, multiply by 4.167.

The eighth line, "Default Pitch" shows the horizontal spacing of

characters in a font in characters per inch.  A 12 pitch font prints

12 characters per inch.  Actually the default pitch figure defines

character spacing only for fixed spaced fonts.  For proportionally

spaced fonts the figure displayed is the reciprocal of the width of

space character.  For example a default pitch of 27.27, as in the

DEMO.SF2 file, means that 27.27 spaces will fit in one horizontal

inch.  The width of the individual characters of a proportional font

is shown on the lower part of the screen under CHARACTER DATA.

The ninth and tenth lines show the dimensions of the font's cell,

in HP dots.  Since HP uses 300 dots per inch in their font specifica-

tions, dividing the displayed cell dimensions by 300 will provide you

the physical dimensions of the cell.  For example, if the cell width

is 41 dots, it is 41/300 = 0.137 inches wide.  These dimensions are

used to draw the cell on the graphics portion of the screen.

The eleventh line, "BaseLine dst" shows the distance from the top

of the cell to the baseline, in dots.  This value is used to position

the baseline in the FONTVIEW graphics window.

The twelfth line, "x" height displays the distance, in dots, from

the baseline to the x-height line, as described earlier.  Many

shareware fonts leave this field blank; and, if this has been done, no

x-height line will appear in the FONTVIEW graphics window.

The final, thirteenth line of the font data area shows the dot

magnification used by FONTVIEW to display the font characters.  As

discussed earlier, an X-magnification of 9 means that 9 dots are

displayed horizontally, for each single horizontal dot printed by

your laserjet printer.  The true physical magnification, naturally

depends on the size of your screen.  For my NEC Multisynch monitor,

with a horizontal image of about 10 inches and vertical image of

about 8 inches, the overall image magnification is about 4.7 times

the X-magnification shown.  Thus, for a horizontal dot magnification

of 9, the actual physical magnification is 4.7 * 9 or about 42:1.

Similarly, the vertical magnification is about 6.9 times the displayed

amount, or 41:1 for a Y-magnification of 6.  If the ratio of X-

magnification to Y-magnification is maintained at 3:2, the screen

image aspect ratio (vertical to horizontal proportions) matches the

printer aspect ratio within about 3%.  FONTVIEW automatically

maintains this 3:2 ratio unless the font is so large (more than 30

points) that the image cannot fit on the screen.  If this occurs, the

program will shift to a one dot:one dot magnification which will allow

fonts of over 60 points to be displayed, although fonts will appear

tall and skinny on the screen due to the distortion in aspect ratio.

Also, at 1:1 magnification, all characters are displayed as solids

since each "display dot" cannot be further reduced in size to produce

the grid-like dot display appearance.

CHARACTER DATA

Below the font data area of the text window is the Character

data.  This data changes for each character displayed and is updated

immediately before each character is displayed in the graphics window.

The first line shows the ASCII number of the character.  To the

right of this, in parenthesis, is the EGA text-mode representation of

the character.  Above ASCII 126 this may not match the actual

character shown in the graphics area, because EGA text mode displays

the equivalent of HP's PC-8 symbol set.  If your font contains one of

the more than 30 other symbol sets, the characters will not be the

same, particularly above ASCII 127.  Nonetheless, the more commonly-

used characters below ASCII 128 should match.  If your font contains

characters below ASCII 32, the EGA text mode character may not be

shown in parenthesis since many of these characters are control

characters which would scroll or otherwise distort the screen if

displayed.

The second and third lines show the dot width and height of the

character.  The HP LaserJet II technical reference manual indicates

that a character height or width larger than the corresponding

dimension of the cell specified in the font descriptor will invalidate

the character download, although my experience suggests the rule is

somewhat more complex than it appears:  for fixed pitch fonts, if the

left offset (see below) plus the character width does not exceed the

cell width, the character is OK; for proportional fonts, if the "delta

X" (discussed below) does not exceed the cell width, it's OK.  It is

possible, therefore, for a valid LaserJet character to extend beyond

the right boundary of the graphics cell, although in practice this is

rare.  FONTVIEW will beep when it reads a character whose height or

width violates the LaserJet cell rules, but will attempt to display

the character.  If FONTVIEW terminates abruptly, immediately following

a beep, it is likely that the character was larger than the cell and

would not fit on the screen at the dot magnification being used.

The fourth line displays the left offset, in dots.  This is the

number of dots that the LaserJet printer moves to the right before

printing the character displayed on the screen, and thus is how

character spacing is performed in the printer.  Note that this value

is negative for characters which are moved to the left before

printing.  FONTVIEW uses the left offset value to position characters

within the graphics screen cell, except that negative values are

ignored for display purposes.

The fifth line shows the top offset, in dots.  This is the number

of dots above the baseline at which the top of a character begins.

For example the portrait-orientation "A" in DEMO.SF2 has a character

height of 29 dots and a top offset of 28 dots.  This means that the

top of the character begins 28 dots above the baseline.  Since the

character is 29 dots high, this results in the bottom of the character

coinciding with the baseline.

The sixth and last line shows the Delta X value in dots.  This

is the number of dots the printer moves from where it left off at the

end of the previous character to where it moves after printing the

character being displayed.  This Delta X defines the width of a

character on the printer; this is the value which many word

processors include in Width Tables so that they can properly space

text which will use a downloaded font.

TROUBLESHOOTING

If FONTVIEW abruptly terminates before displaying any characters,

it may be that extraneous bytes are contained in the file.  Run

FNTCLEAN to clean up the file and then run FONTVIEW on the new file

created.

If FONTVIEW terminates before displaying any characters, and you

are attempting to view a file already processed by FNTCLEAN, recheck

the path and filename you typed in on the command line.  Particularly

check that the path exists since I have bombed FONTVIEW many times by

mistyping the path name.

If FONTVIEW terminates in the middle of a file, it may be that

the manual magnification is set too high, or because some character

dot data overflows the "cell" boundary defined in the font header and

cannot fit on the screen.  Try running FONTVIEW at reduced

magnification.  For example, if the original magnification was 9 by 6

(x-mag, y-mag), try using /M6,4 on the command line.  This is the next

lower magnification which preserves the 3:2 x- to y- magnification

ratio which results in correct vertical and horizontal proportions on

the screen.

REGISTRATION

You will soon notice that many shareware and even commercial

fonts don't sit on the baseline.  For some unknown reason, many fonts

sit one dot above the baseline, although I've never observed this in a

HP-designed font. This offset does not impair the font's performance

in your Laserjet printer since if all characters are situated one dot

(1/300 inch) too high, it is not noticeable, however it is annoying

when you view the font using FONTVIEW.  Also, if the character sits

one dot too high, it is likely the x-height line will appear one dot

too high as well.  Furthermore, many font makers do not bother to set

the x-height since it does not affect the font's performance in a

printer.  Again, though, it is annoying since the x-height line is

helpful when viewing any lower-case letter in FONTVIEW.  Finally,

sometimes a font will contain characters which violate the LaserJet

cell boundary rules or contain dots so far outside the cell that

FONTVIEW bombs.

If you register FONTVIEW and FNTCLEAN, you will receive the

latest versions of the two programs on a diskette, PLUS, you will

receive a copy of the programs FONTFIX and LJROTATE.

FONTFIX, which works on portrait soft fonts, is a very hard-

working program.  It has all of the features of FNTCLEAN, plus:

1.   Displays the hexadecimal values and interpretation of all 64

bytes of the LaserJet II font header.

2.   Displays the HP LaserJet escape sequence which completely

identifies the font and can be used by your word processor

or spreadsheet to designate the font for use, once it has

been downloaded.

3.   Sorts its output file by ascending ASCII numbers.  Amazing-

ly, I have viewed many font files which do not have their

character data sorted in ASCII order.

4.   Has a Command line option to move the Baseline for all

characters Up or Down by a user-selected number of dots.

5.   Has a Command line option to set the font's "x-height"

either manually (in dots), or have the program set the font

x-height using data from the lower-case "x" character (ASCII

120) if it is included in the font file.

6.   Goes through each character in a font on a bit (pixel)

level, along the top, bottom, left and right edges,

eliminating superfluous rows of dots which are all "off";

then adjusts left offsets, top offsets, character widths and

heights appropriately, and reconstitutes the character.  It

then returns to the FONT header, and sets the cell width,

cell height, and baseline distance such that these values

are correct.  This results in a font which uses the minimum

number of bytes to fully describe each character, thus

minimizing download time, LaserJet memory requirements, and

disk storage requirements.  Several shareware fonts do not

make effective use of top offsets or left offsets.  If you

display a font which uses the same height and width for each

character, it is likely that the size of the file can be

reduced.  For example, the shareware font LARGEHLW.SFP was

34,773 bytes long originally, but only 16,992 bytes long

after processing by FONTFIX.  Naturally, the modified font

performed identically on my LaserJet printer.

LJROTATE creates a LANDSCAPE font from your PORTRAIT font input

or a PORTRAIT font from your LANDSCAPE font.  It processes characters

in a manner similar to FONTFIX to set correct cell dimensions, sort

output characters into ascending ASCII order, and minimize the number

of bytes to define each character, then rotates the character.  Since

many fonts are available in only one orientation (portrait or

landscape), LJROTATE can be very useful.  I now don't bother to

download both orientations of the same font.  Instead, I download the

portrait font, set the baseline and x-height with FONTFIX, and then

run LJROTATE to create a landscape font with the correct baseline

distance and x-height.

You may register both FONTVIEW and FNTCLEAN, and receive FONTFIX

and LJROTATE by forwarding $17.00 (California residents add $1.11

sales tax) to:

S.H. Moody & Associates, Inc.

1810 Fair Oaks Ave.

South Pasadena, CA 91030

Please note that you are registering for FONTVIEW so I can

distinguish your order from orders for other products.

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

The Public (Software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  The P(s)L cannot de-

bug programs over the telephone.

Disks in the P(s)L are updated monthly, so if you did not get

this disk  directly from the P(s)L,  you should be aware that

the files in this set may no  longer be the current versions.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

The Public (Software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

(713) 665-7017