Title: More Than 15 Files Open at Once in DOS Versions 3.30 and Later

Document Number: Q31509

Publ Date:  5-MAY-1989

Product Name: Microsoft QuickBASIC Compiler

Product Version:  4.00 4.00b 4.50

Operating System: MS-DOS

Summary:

To open more than 15 files at once in a program, you must do the following:

1. Run under DOS Version 3.30 or later. (You can double check the DOS

version number with the VER command at the DOS prompt.)

2. Add the statement FILES=n in the DOS CONFIG.SYS file, where "n" is

an odd number ranging from 21 to 255. (Even numbers may make the

interrupt fail under DOS Version 3.30. This problem was corrected

in MS-DOS Version 3.30a, where you can use even or odd numbers for

"n".)

3. Call interrupt 21 Hex with function 67 Hex from the BASIC program,

as shown in the example further below.

4. If you are using the SHARE.EXE utility, you must also invoke

SHARE/F:nnnnn to increase the area for file-sharing information

above the default of 2048 bytes. Please refer to your MS-DOS manual

for more information about the SHARE utility.

The following is an example of using SHARE/F:

SHARE/F:16384

This information applies to QuickBASIC Versions 4.00, 4.00b, and 4.50 for MS-DOS

and to the Microsoft BASIC Compiler Versions 6.00 and 6.00b for MS-DOS and OS/2.

More Information:

Please note that even if you follow the above steps and specify FILES=255 in the

DOS CONFIG.SYS file, you may not be able to access that many files at once in

your program because there may not be enough memory available inside the DGROUP

data segment to allocate file buffers. Also note that five file handles are

taken up by the DOS standard input/output devices.

We recommend that you use the BASIC SETMEM function to reduce the size of memory

available to BASIC as it loads. This method ensures the provision of more memory

available to the operating system; therefore, more files can be opened at the

same time.

The following is a BASIC code example of the DOS interrupt necessary to access

more than 20 DOS file handles:

' $INCLUDE: 'qb.bi'

DIM InRegs AS RegType, OutRegs AS RegType

InRegs.ax = &H6700         'SetFileHandles function

InRegs.bx = x

'x is the number of files to open

CALL INTERRUPT(&H21, InRegs, OutRegs)

FOR I% = 1 TO x - 5   '5 file handles are reserved for DOS standard I/O.

File$ = "Junk" + STR$(I%)

OPEN File$ FOR OUTPUT AS I%

PRINT I%

NEXT

END

COPYRIGHT Microsoft Corporation, 1989.