////////////////////////////////////////////

// This is the documentation for the      //

// FASTDRAW v1.0 library for C++.         //

////////////////////////////////////////////

Developed by: Greg Dolley

Date of this documentation: 10-6-1994

Tested under Turbo C++ v3.0 for DOS.

Included in this archive:

1.) FASTDRAW.H ................................... Function prototypes

2.) FASTDRAW.LIB ................................. Library of functions

3.) RANDLINE.EXE ................................. Demo

4.) PALETTE.EXE .................................. Demo

5.) FASTDRAW.DOC ................................. Documentation

(what you're reading)

Some legal crap:

These files are distributed AS IS!!! There is no warranty expressed or

implied. I take no responsibility for any damages that may be caused by these

files. But as far as I know, they won't cause any damage.

This archive is distributed as freeware. You may distribute this archive,

as long as the files are intact, not corrupted and no charge is asked for

(except for disk costs). If you receive an archive that does not contain one

or more of the files listed above, do not distribute it any further.

Desription of the functions:

The two functions you receive with this archive are:

1.) void linedraw(int x1, int y1, int x2, int y2, int color,

char far *screen)

2.) void setgmode(int mode)            // The mode is in hex.

///////////////////////////////////////////////////////////////////////

// The LINEDRAW function:                                            //

//                                                                   //

//    The LINEDRAW function will draw a line from one coordinate to  //

// another VERY QUICKLY. Note: Draws lines in mode 13h only.         //

//                                                                   //

// Arguments:                                                        //

//                                                                   //

//    x1, y1 ................................. Starting coordinates. //

//    x2, y2 ................................. Ending coordinates.   //

//    color .................................. Color of the line.    //

//    screen ................................. Place in RAM where    //

//                                             you want your line to //

//                                             be drawn (usually the //

//                                             screen - VIDEO RAM).  //

//                                                                   //

//    If you aren't familiar with making a pointer point to video    //

// ram, the screen, here's how you would implement it:               //

//                                                                   //

//    char far *screen = (char far *)MK_FP(0xa000, 0);               //

//                                                                   //

// This tells C++ to make a character far pointer, called screen,    //

// point to the address in parentheses of the MK_FP macro.           //

///////////////////////////////////////////////////////////////////////

///////////////////////////////////////////////////////////////////////

// The SETGMODE function:                                            //

//                                                                   //

//    The SETGMODE function sets the graphics to the hexadecimal     //

// number that you specify in parentheses.                           //

//                                                                   //

// Arguments:                                                        //

//                                                                   //

//    mode ................................... Number of the         //

//                                             graphics mode.        //

//                                                                   //

// Graphics modes:                                                   //

//                                                                   //

//    HEX             TYPE       RESOLUTION      COLORS              //

//    -------------------------------------------------              //

//    00H, 01H      | TEXT     | 40 x 25       | 16                  //

//    02H, 03H      | TEXT     | 80 x 25       | 16                  //

//    04H, 05H      | GRAPHICS | 320 x 200     | 4                   //

//    06H           | GRAPHICS | 640 x 200     | 2                   //

//    07H           | TEXT     | 80 x 25       | MONO                //

//    08H, 09H, 0AH | GRAPHICS |        (PCjr only)                  //

//    0BH, 0CH      | (Used internally by EGA BIOS)                  //

//    0DH           | GRAPHICS | 320 x 200     | 16                  //

//    0EH           | GRAPHICS | 640 x 200     | 16                  //

//    0FH           | GRAPHICS | 640 x 350     | MONO                //

//    10H           | GRAPHICS | 640 x 350     | 16                  //

//    11H           | GRAPHICS | 640 x 480     | 2                   //

//    12H           | GRAPHICS | 640 x 480     | 16                  //

//    13H           | GRAPHICS | 320 x 200     | 256 (DOOM GRAPHICS!)//

//    6AH           | GRAPHICS | 800 x 600     | 16                  //

//    100H          | GRAPHICS | 640 x 400     | 256                 //

//    101H          | GRAPHICS | 640 x 480     | 256                 //

//    102H          | GRAPHICS | 800 x 600     | 16                  //

//    103H          | GRAPHICS | 800 x 600     | 256                 //

//    104H          | GRAPHICS | 1024 x 768    | 16                  //

//    105H          | GRAPHICS | 1024 x 768    | 256                 //

//    106H          | GRAPHICS | 1280 x 1024   | 16                  //

//    107H          | GRAPHICS | 1280 x 1024   | 256                 //

///////////////////////////////////////////////////////////////////////

Later versions:

I'm currently working on enhancing these functions and adding many more.

The line drawing function that I'm working on now will draw lines in any

graphics mode. If you would like the latest version of this library, and

source code for the demos, please send $5.00 to: