HIGH PRECISION TIMER

DOCUMENTATION

Version 5.5

January 14, 1990

Public Domain

by Eagle Performance Software

TIMERH High Precision Timer                      Documentation, Version 5.5

T A B L E   O F   C O N T E N T S

Features .  . . . . . . . . . . . . . . . . . . . . . 3

Distribution Files  . . . . . . . . . . . . . . . . . 3

Demonstration . . . . . . . . . . . . . . . . . . . . 3

Simple Programming  . . . . . . . . . . . . . . . . . 3

How It Works  . . . . . . . . . . . . . . . . . . . . 4

Support . . . . . . . . . . . . . . . . . . . . . . . 5

Reference . . . . . . . . . . . . . . . . . . . . . . 5

Revisions . . . . . . . . . . . . . . . . . . . . . . 6

2

TIMERH High Precision Timer                      Documentation, Version 5.5

FEATURES

This high precision timer written in Pascal allows you to time events or

programs with 1 micro-second resolution up to 24 hours.  The program

simply uses the extended resolution of the Intel 8253/8254 timer chip

which is the same chip that generates an interrupt for the system timer

(every 1/18.2... seconds).

Compatibility - TIMERH works on all IBM PC, XT, AT, PS/2, and 3270 PC

compatible computers.  The source code can be compiled with either Turbo

Pascal 5.0, 5.5, or QuickPascal.

DISTRIBUTION FILES

In this version, TIMERH55.ARC contains:

TimerH  .pas:  Timer unit source code.

TimerH  .doc:  This file.

TimeDemo.arc:  Demonstration file to show elapsed time.

DEMONSTRATION

To get a feeling of the timer, let's run the demonstration program that

came with the utility.  Do the following steps:

1. Make and run TIMEDEMO.PAS.

2. Press RETURN for elapsed time since the beginning of the

program.

3. Press ESC to quit the program.

SIMPLE PROGRAMMING

Basically, to time a given event in your program, the source code should

look like the following:

...

begin

...

Timer (Sync);   { Optional one time use }

Timer (Start);

...

{ Code to be tested goes here. }

...

Timer (Stop);

...

end.

Start - Timer(Start) resets the initial starting value of T1 so that all

subsequent splits will be referenced to this point in time.  You can use

this more that once.

Stop - Timer(Stop) takes a split in reference to the last Timer(Start)

TIMERH High Precision Timer                                         Page 3

TIMERH High Precision Timer                      Documentation, Version 5.5

which changes the value of T2.  Subsequent uses of Timer(Stop) are

permitted, but there is a little overhead for its execution that is not

accounted.

Sync - Timer(Sync), placed just before the first start, resets the timer at

a zero count.  It is recommended that you use the default Sync parameter

for consistent results.  The Sync parameter starts the chip timer zero.

This puts the next interrupt (which uses about 15-200 microseconds to

update the system timer) at 1/18.2 seconds away.  This makes quick events

(<1/18.2 seconds) uninterrupted.  Since the system timer is independent of

the real time clock, it doesn't affect the time of day on your computer.

Overhead - The overhead for the elapsed time between the start and stop is

kept in the value T0.  The global value ElapsedTime excludes the overhead

after Timer(Stop).

Jitter - Most events will be within plus or minus 3 microseconds depending

on your CPU.

HOW IT WORKS

To work both on IBM PC which use the 8253 chip and on the IBM AT using the

8254 chip, it is necessary to use  the common characteristics of both

chips:

1. The output port to the timer for counter 0 which is a utility counter

is $43 and the input port is $40.  The latch code is $00 to read a

value from the chip.

2. The system timer is a four-byte value and updated in memory at

0040h:006Ch, but only the first three bytes (21 bits) are used because

it only counts up to 1573040 (1800AFh) ticks per day.  At that

resolution, it only amounts to:

1573040/(60*60*24) = 18.206481481 ticks/sec

3. The chip counts down from $0000 negatively until it wraps to zero

again which then sends the usual interrupt every 1/18.2... seconds to

update the system timer in the BIOS data area.

4. Using the chip ports, we can access and lock on to the count down

timer and get a value.  Since it has one full word in the count, the

total extended resolution of 16 bits is:

1573040 * 2^16 = 103090749440 ticks/day

= 1193179.9704 ticks/sec

5. This total resolution requires a five-byte value to hold the 37 bit

number.  If two days are needed in the calculation which is required

when passing midnight, a 38 bit value will be needed, but notice that

it never reaches the full 40 bits of five bytes.  This means that

6 byte reals are fully capable of handling the value with no loss in

resolution.

TIMERH High Precision Timer                                         Page 4

TIMERH High Precision Timer                      Documentation, Version 5.5

6. Interrupt control while taking splits was a problem.  Since the chip

and CPU are independent, it is possible that the chip may try to send

an interrupt on the bus just as the OUT code is being executed.  The

chip will read 0-2 microseconds, but the DOS timer will be one tick

short.  An easy solution is to take two readings of the DOS timer low

byte and see if they differ - one before the chip is latched and

another after.  If they differ, an extra tick is needed for the system

timer part of the split only the timer value is small enough (<10000

microseconds) to confirm that the interrupt did occur.

7. It is also important that no conditional statements are used in the

code so that the overhead is constant.

8. The wake up mode of all timer chips is mode 3, but there is no way to

use mode 3 on the 8253 with software.  The 8254 chip made mode 3

superior with access to its status register, but software accessing

this register is incompatible with the IBM PC.  So, TIMERH initializes

with the common mode 2.  When the program is terminated, it is

restored back to mode 3 in case anything else may expect the default

mode.

SUPPORT

If you have questions, comments, or suggestions, the Eagle can be contacted

by four means - (1) CompuServe, (2) telephone, (3) The Eagle BBS, or

(4) mail.

CompuServe - The most dependable way to contact the Eagle is through

CompuServe.  James (Jim) H. LeMay has written the Pascal version of TIMERH,

but the person to contact is Jordan Gallagher who can be contacted on the

Borland Forum by typing GO BPROGA from the CompuServe main menu.  You will

enter the Forum for Turbo Pascal.  You can contact Jordan with his PPN

number of 73557,2342.  Messages can also be left through EasyPlex.

Telephone - Jordan can also be reached by phone at (214) 539-7855 on

weekdays and Saturday from 9:00 a.m. to 8:00 p.m CST.

The Eagle BBS - You can also contact us on our 24-hour BBS at (214) 539-

9878, 1200/2400 N81.

Mail - For problems, please write:

Eagle Performance Software

P.O. Box 292786

Lewisville, TX  75029-2786

REFERENCE

For more information on the Intel 8253/8254 timer chips, refer to the Intel

Microsystem Component Handbook, vol 2.  You can also call Intel literature

sales at 1-800-548-4725.

TIMERH High Precision Timer                                         Page 5

TIMERH High Precision Timer                      Documentation, Version 5.5

REVISIONS

Version 5.5 (01-15-90):

Original release.

TIMERH High Precision Timer                                         Page 6

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 2,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

1-800-2424-PSL

MC/Visa/AmEx

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.