MIDI POPup v1.0                    9/25/87

by Jack A. Orman

ARMADA SOFTWARE

Box 858

Southaven, MS  38671

As  a  progammer  working with MIDI, I often have to refer to the MIDI

Spec document to find out what bytes are used for the many commands so

I can put them in my programs. The commands are in binary format and a

calculator  is  needed  to convert them to decimal. Therefore, I wrote

this small pop-up utility that will terminate and  stay  resident  and

produce a list of MIDI commands at the touch of a key.

Not  every command is included in the pop-up, a few had to be left off

due to space limitations. Also in the language of MIDI, Channel  1  is

specified  by  a data byte of 0000 , Channel 2 by 0001 .... Channel 16

by 1111 (15 decimal or F hexadecimal). Therefore, for any  Channel  N,

the  proper  data  to use to designate that channel is equal to N - 1.

For example, a Note Off command is 128(decimal) + the channel number.

For Channel  1, note off =  128 + 0

Channel  2, note off =  128 + 1

Channel  3, note off =  128 + 2

...

Channel 16, note off =  128 + 15

The note and velocity must also be sent after the note off command, of

course.  The  same  numbering system applies to the channels when used

with all of the other commands also.

The active sensing command is optional but once it is sent, the  synth

will expect it to continue to be sent at least every 300 ms. It's best

to  stay away from this since it is just more data to clog up the MIDI

channel.

The Tune Request command is for analog synths that will need  to  tune

their oscillators.

Hitting  the Alt-Left Shift key calls the MIDI POPup to the screen and

escape will remove it. This program should have  no  problem  with any

IBM compatibles but there are NO guarantees.

MIDIhex.com is identical to MIDIpop.com except the numerical values

are all in hexadecimal. Both programs were created with TSRmaker.

The MIDI POPup is a copyrighted work and is for private non-commercial

use  only!  Site  licenses  and tech support are available for a small

fee.

**** REGISTRATION ****

The registration fee for MIDIpop.com is $20.00 which entitles the user

to a disk containing the complete collection of our utilities. Some of

the  programs  on the registration disk have never  been  released  as

shareware.

Send your check or money order for $20.00 to:

Jack A. Orman

Armada Software

Box 858

Southaven, MS  38671

Comments and suggestions are welcome...

On GEnie send e-mail to J.Orman

On Delphi send to username JAO.

On Compuserve to ID# 72261,677

----------------end-of-author's-documentation---------------

Software Library Information:

This disk copy provided as a service of

Public (software) Library

We are not the authors of this program, nor are we associated

with the author in any way other than as a distributor of the

program in accordance with the author's terms of distribution.

Please direct shareware payments and specific questions about

this program to the author of the program, whose name appears

elsewhere in  this documentation. If you have trouble getting

in touch with the author,  we will do whatever we can to help

you with your questions. All programs have been tested and do

run.  To report problems,  please use the form that is in the

file PROBLEM.DOC on many of our disks or in other written for-

mat with screen printouts, if possible.  PsL cannot debug pro-

programs over the telephone, though we can answer questions.

Disks in the PsL are updated  monthly,  so if you did not get

this disk directly from the PsL, you should be aware that the

files in this set may no longer be the current versions. Also,

if you got this disk from another vendor and are having prob-

lems,  be aware that  some files may have become corrupted or

lost by that vendor. Get a current, working disk from PsL.

For a copy of the latest monthly software library newsletter

and a list of the 3,000+ disks in the library, call or write

Public (software) Library

P.O.Box 35705 - F

Houston, TX 77235-5705

Orders only:

1-800-2424-PSL

MC/Visa/AmEx/Discover

Outside of U.S. or in Texas

or for general information,

Call 1-713-524-6394

PsL also has an outstanding

catalog for the Macintosh.