SYSTEMATIC NAVIGATION

EXCEL APPLICATION FOR NAVIGATION PLANNING

| (1) INTRODUCTION | Page 2 |
| --- | --- |
| (2) INSTALLATION | 3 |
| (3) ASTRO NAVIGATION | 4 |
| (4) WORKED EXAMPLES | 11 |
| (5) BIBLIOGRAPHY | 18 |
| (6) LICENCE AGREEMENT | 18 |
| (7) REGISTRATION DOCUMENT | 19 |

Systematic Navigation

Winkworth House

11 High Street

Bramley

Surrey GU5 0HF

Telephone : 0483 894890

Telefax : 0483 894078

14 February, 1994

'SHAREWARE' VERSION V1.0 (1994) : 14 Feb 94 : NavigShr.doc

(1)  INTRODUCTION

Systematic Navigation is a complete set of applications written in Microsoft Excel with full macro and graphics support for astro and terrestrial navigation.  This 'Shareware' version contains one of the twelve options - Astro Navigation.  You may make as many copies of this disk as you wish and pass it and the documentation on to others, provided that you make no charge for the software.  We would encourage you to register in order to receive the latest edition of Astro Navigation.  This includes all the other astro and terrestrial navigation options with a full manual.  These comprise other methods of Astro Navigation, Sun and Moon Rise and Set, Estimated Position, Course to Steer, Tidal Heights, Passage Planning and Satellite Positioning for Astra and the other TV Satellites.

Registration costs only £30.00 including all VAT, postage and packing for the full version or £15.00 for Astro Navigation only; (See form at the end of this manual, together with Read.Me and Register.Txt documents on the disk).  Please write to Alastair Day at the address below to register your copy, or if you have any problems with any of the programmes, or need any further information.

Astro Navigation provides:

(1) Position fixes from two or three sextant sights without an Almanac

(2) Automatic calculation of hour angles, declination and sight reduction

(3) Star Charts for all 59 stars giving SHA, elevation and bearing

(4) Star Identification from angle and bearing

(5) Chart plots and full results print-outs of all angles and position lines

This guide does not try to teach astro navigation and a knowledge of the basic theories is assumed.  A bibliography is given at the end of the guide, which contains explanations of the formulae and algorithms used by the system.

We strongly recommend that you review each of the examples in turn with this manual and make sure that you understand the results produced.

Note: All rights reserved.  All intellectual contents and any derivatives or improvements on the Systematic Navigation disk are the property of Systematic Navigation.  The programmes have been extensively tested, however, no liability whatsoever can be accepted regarding the use or accuracy of the programmes and associated data.  We will make every effort to correct such software errors that are found and reported to us at the address below.

| This disk is provided for: | 'Shareware' Disk with limited evaluation licence only |
| --- | --- |
| Date / Reference: | V1.0 (1994) 14 Feb 1994 - 20 Feb 1994 SHAREWARE |
| Expiry Date: | 31 December 1994 |

| Systematic Navigation | German Office: |
| --- | --- |
| Winkworth House | Jesuitengasse 7 |
| 11 High Street | D-86152 Augsburg |
| Bramley | Germany |
| Surrey GU5 0HF |  |

| Telephone: | 0483 894890 | (49) 0821 / 15 20 38 |
| --- | --- | --- |
| Telefax: | 0483 894078 | (49) 0821 / 15 20 38 |

Systematic Navigation is a division of Systematic Finance Limited of the same address.

| Registered Number: | 2443802 |
| --- | --- |
| VAT No: | GB 572 5542 32 |
| Registered Office: | Langton Priory, Portsmouth Road, Guildford, Surrey GU2 5EH |

(2)  INSTALLATION

2.1  Description

The system requires an IBM compatible computer with minimum 3.0 mb spare hard disk capacity and 4 mb RAM.  The software required is DOS 3.1 (or later), Windows 3.0 (or 3.1) and Microsoft Excel for Windows Version 4.0 (or later).  Files can be supplied in another Excel format on request.

The 3½" (90mm) 1.44 mb disk contains all the files for this application.

2.2  Installation

(1)	Using File Manager, make a directory under call it 'Navigtn' - 'C:\Navigtn'.

(2)	Copy the files to this sub-directory.  Use the command line: 'A: Copy_*.*_C:\Navigtn'.

(3)	Keep the disk as supplied as your back-up security copy.

(4)	Set up a separate icon on your Applications Menu to run Systematic Navigation directly from your Main Windows Applications Group.

(a)	Click 'File New'.

(b)	Select 'Programme Item' and press 'OK'.

(c)	At the next box, the description should be entered as Systematic Navigation.

(d)	Command line: 'C:\[Excel Directory]\Excel.exe_C:\Navigtn\Openup.xls'.

Note: the space between the two parts of the command to open Excel and run Openup.  The working directory is 'C:\Navigtn'.

(5)	The icon file to be loaded is 'NavApps.ico' and should now be in the path 'C:\Navigtn'.  It will appear as a logo to distinguish it as a separate application to the standard Excel icon.

(6)	When exiting Windows remember to 'Save Changes' to set up this application permanently.

(7)	Click the icon twice quickly to activate Systematic Navigation.  A copyright box appears.  Click 'OK' and the system displays a description of the information needed.  Full details on Astro Navigation start on the next page.

(3)  ASTRO NAVIGATION

3.1  Introduction / Background

Astro Navigation fixes positions with up to three sextant sights of the sun, 59 navigational stars, the moon or the planets of Venus, Mars, Jupiter and Saturn.  This dispenses with manual calculation of corrected altitudes, hour angles and declination from the Almanac and the plotting of position lines on a chart.  You can view or print results and you should use the following sub-sections to check the procedure for each object selected.  This is not intended as a manual of astro navigation and you should refer to simplified explanations such as Basic Astro Navigation by Conrad Dixon (see Bibliography).

The model requires this information to determine a fix from individual sextant sights:

(a)	Dead Reckoning (DR) Position and Time

(b)	Name of each body observed

(c)	Observed Angle and any adjustments required e.g. height, temperature to derive the corrections

(d)	Ephemeris i.e. the Greenwich Hour Angle (GHA) and Declination (DEC) - the model calculates these variables in almost all cases

(i)	Declination (DEC) is equivalent to the latitude of the body observed, and measured in Degrees North or South.  For example: the sun moves from the furthest position in the South on 21st December to the 21st June in the North.

(ii)	Hour Angles are the equivalent of longitude except that longitude is measured up to 180 degrees whereas hour angles have values up to 360 degrees.  Three kinds of hour angle are used to calculate positions:

(a)	Sidereal Hour Angle Hour angles (SDA) are based on a meridian line (c.f. Greenwich for longitude) and in the heavens this is the first point of Aries.  The SDA is the degree's difference to Aries (the hour equivalent of the Greenwich meridian).

(b)	Greenwich Hour Angle (GHA) is the addition of the Aries position and the Sidereal Hour Angle.

(c)	Local Hour Angle (LHA) completes the calculation as the addition of the GHA and Longitude (West is subtracted).

Therefore LHA = GHA(Aries) + SHA +/- observer's longitude

The system displays position lines and a fix for two sights or more and you can select printing, viewing, increasing/reducing the number of sights or closing files.

Note: All distances in this manual are given in (sea/nautical) miles.  One nautical mile is:

| Metres | 1,852.00 metres | (1.85 kilometres) |
| --- | --- | --- |
| Yards | 2,026.08 yards | (1 metre is 1.094 yards) |
| English Miles | 1.15 miles | (1760 yards to the mile) |

A nautical mile corresponds to one minute of latitude and is made up of ten cables; thus sixty miles is equal to one degree of latitude.  Units of longitude vary with latitude and cannot be calculated in this manner.

3.2  Example Astro Navigation

This set of three sights illustrates the outline method from the previous page and uses data from first example in Section 4.  The model uses three sextant sights from assumed positions on a straight course and reduces the sights based on the calculated position of the body observed.  A statistical fix position is shown with its relative distance and bearing from the DR position:

3.3  Method Described in Detail

(A) Sight Reduction

The model uses the classic Marcq Saint Hilaire method to reduce the sights as the mathematical link between the observer and the celestial body.  If you know your latitude and longitude, you can predict the true bearing and the height of the object above the horizon.  This angle can then be compared to your corrected sextant angle to produce a position line and a measure of distance along this line.  With several sights, the model plots a fix through the statistical intersection of these position lines.

The following sight reduction formulae are used:

(1) Computed Altitude

Asin [ (Sin Latitude * Sin Declination) + (Cos Latitude * Cos Declination * Cos Local Hour Angle) ]

(2) Azimuth or True Bearing:

Acos [ (Sin Declination-(Sin Computed Altitude * Sin Latitude) ) / (Cos Computed Altitude*Cos Latitude ) ]

If the Local Hour Angle is less than 180' then the Azimuth is 360' less the product of the above expression.

(B) Observed Angle

All sextant angles need to be corrected for index error and altitude to produce the Apparent Altitude.  The model computes these adjustments automatically and calculates the Corrected Altitude by adding and subtracting the corrections below.  For the Sun, Moon, Venus and Mars only a correction for parallax is applied and for the Sun and Moon a further adjustment for semi-diameter is required.

This example assumes a height of 5.4 metres, temperature of -3.0' C, pressure of 982 mb.  Sun HP is assumed to be 0.0024' and the HP for the Moon 54.4'' i.e. 0.9067'.  Note that the corrections for temperature and pressure are small and can be ignored.  The model is set for a temperature of 9.8' C and pressure of 1010 mb which produces a factor of 1.0 under the formula below.

| Procedure | Sun | Moon |
| --- | --- | --- |
| Observed Angle / Initial Sextant Altitude (Hs) | 21.5000' | 33.0000' |
| (1) Height (DIP) = 0.0293*SQRT(Hs) | 0.0681' | 0.0681' |
| (2) Index Error (I) | 0.0000' | 0.0000' |
| Apparent Altitude (H) =Hs+I-DIP | 21.4319' | 32.9319' |
| (3) Refraction(a) (Ro) = 0.0167/ Tan (H+7.31/(H+4.4)) | 0.0348' | 0.0256' |
| Adjustment for Pressure & Temp (f) = 0.28P/(T+273) | 1.0184' | 1.0184' |
| R = f*Ro | 0.0354' | 0.0261' |
| (4) Parallax (HP) | 0.0024' | 1.0055' |
| Parallax (PA) = HP *Cos H | 0.0022' | 0.8439' |
| (5) Semi-diameter (S) |  |  |
| Sun (S) = 15.76"/60 : Moon (S) = 0.2724 * HP | 0.2627' | 0.2740' |
| Subtract for Upper Limb / Add for Lower Limb |  |  |
| (6) Oblateness - Moon only (OB) = -0.0017 * Cos H |  | 0.0014' |
| (7) Corrected Altitude Ho = H-R+PA-OB+/-S | 21.6614' | 34.0251' |
| (8) Variance: Corrected Altitude - Observed Angle | 0.1614' | 1.0251' |

(a) Bennett's Formula.  The model is preset with a temperature of 9.8' C and 1010.0 mb air pressure.

(C) Epherimedes / Celestial Positions

Astro Navigation incorporates two different methods of computing the Local Hour Angle and Declination of the observed body. The two methods are denoted as 'A' or 'B' throughout the model.

(A)	Almanac / Calculated.  The model will compute the position of the sun and moon automatically and will prompt for Almanac data on  the GHA and DEC for stars and planets.  The user can retrieve this information  from the page in the Almanac containing the relevant day.

(B)	Polynomial Coefficients.  These are extracted from Compact Data for Navigation and Astronomy for the Years 1991-1995 by Yallop and Hohenkerk published by Cambridge University Press (ISBN 0 521 38731 0), (Compact Data).  The model derives a series of coefficients to calculate the GHA, Declination and if applicable the horizontal parallax and semi diameter of the body observed.  The model computes the DEC and GHA and adds the longitude to convert to LHA (Local Hour Angle).

The most convenient method for calculating GHA and Declination for each body is shown below.  The model will prompt you for the method to be used on all sights.  Similar to Methods 'A' and 'B', the celestial bodies are referred to throughout the model by their numbers below:

(1)	Sun: Under 'A' GHA and DEC are calculated, and under Method 'B' the coefficients for 1992-1995 are contained in the model.  If a sight on another year under Method 'B' is chosen then the model will prompt you for the coefficients applicable to the month and year.

(2)	Stars:  The model requires input SHA and DEC together with hourly corrections under Method 'A' however the data under Method 'B' for the years 1991-1995 is contained in the model.  The apparent position of the stars does not vary month to month and therefore the model can calculate the coefficients for all 59 navigational stars.

(3)	Moon: Method 'A' is calculated, however the coefficients for the month and year under Method 'B' must be retrieved from Compact Data together with the horizontal parallax.

(4)	Planets:  The planets have complicated movements and therefore all data must be input.  For Method 'A' this means the Sidereal Hour Angle and the Declination and under Method 'B' the relevant coefficients.

The above provides some flexibility in the model and the results will be very similar.  To reduce the complexity of the above, you can decide to compute the GHA and DEC automatically and in this case the routine will choose either 'A' - Calculated / Almanac or 'B' - Compact Data as follows:

(1)	Sun: Method 'B' for 1992-1995 otherwise Method 'A'.  The model derives the coefficients for 1992-1995, and will also calculate the semi-diameter accurately as opposed to using the single figure of 16" used in Method 'A'.  This will account for any resulting small differences between the two methods.

(2)	Stars: Method 'B' (Compact Data calculates the coefficients for the years 1991-1995).

(3)	Moon: Method 'A' with no inputs needed.

(4)	Planets: Method 'A' and prompts the user to enter data.

3.4  Procedure for Each Type of Sight

(1)	Select 'Astro Navigation' on the Main Navigation Menu.

(2)	The system displays a box requesting the user to select general options for the sights.  These are set to true and they must be clicked to de-select them.  The options are :

(a)	Keep DR (Dead Reckoning) Position and Time

(b)	Calculate GHA and DEC automatically (this is explained above)

(c)	Keep existing sight results.

Clicking (a) or (c) will cancel the sights held in the system.  If the selection is left untouched, the system will load the currently saved entries into the dialog boxes for cancellation, editing and amendment.  Sight Data can be changed and this option does not prevent future editing.  The effect of clicking (b) is to give the user a choice on the method of computation of GHA and DEC used.

Select 'OK' to continue or 'Re-Do' to return to the Introductory Menu.

(3)	A box requesting the DR position and zone time together with any course and speed between the DR position and the sights (if applicable).  The zone time is calculated with reference to longitude and no account is taken of any local time differences.  You are also prompted for the number of sights to a maximum of three.  However if one or two are entered and you choose the retention of current data, any sight numbers above the number of sights chosen will be erased when calculating fixes.  Press 'OK' to continue or 'Re-Do' to return to the main Astro Navigation Menu.

(4)	The model prompts you for the body observed and the local sight time.  GMT is calculated by reference to longitude e.g 100' W = 100' W / (360' per day / 24 hours ) = 6.67 hours rounded to 7.0 hours.  If the calculation of GHA and DEC is set to automatic, the explanation is dimmed and the relevant method is shown.  'A' for Calculated/Almanac and 'B' for Compact Data.  Press 'OK' to continue or 'Re-Do' to return to the Main Astro Navigation Menu.

(5)	The system now calculates GHA and DEC or prompts you for Almanac data for the day of the sight.  The sections below deal in detail with each body and method in turn showing worked examples.

(6)	Input the Observed Angle.  Input the angle in degrees and minutes together with the height of the user's eye  i.e. the height above sea level.  There are also prompts for the Index error and whether the upper of lower limb was observed.  The semi-diameter is only required for the planets.  Press 'OK' to continue of 'Re-Do' to re-enter this information.

(7)	The information to calculate a position line is now complete and system prompts for:

(a)	View result for this sight before continuing with other sights

(b)	Increase the number of sights to a maximum of three

(c)	Enter no more sights and view final results (and fix if two or more sights entered)

(d)	Continue with more sights

The default setting continues with more sights if the sight number is less than the total entered on commencement.  If (a) is selected then the worksheet is calculated, and the object bearing and azimuth is displayed together with the calculated latitude and longitude position of the intercept.  The model allows you to re-enter the sight details or press 'OK' to accept this result and continue with the next sight.  When all sights have been entered, the option is greyed and the model defaults to calculating a fix.

(8)	The screen shows a 'best-fit' calculated fix with the latitude and longitude and the distance and true bearing from the DR position.  The model uses the least squares method from Compact Data et al. to produce a most probable position within a confidence eclipse of 95%.  To produce an accurate fix, three sights are normally required with some angle between them to provide a 'good cut'.  A summary of the position lines for other sights is included.  If any sights are inaccurate such that the intercept is greater than 500 miles, then no fix can be calculated and an error message is displayed.  Similarly no fix can be computed with only one sight.  Press 'OK' to continue to print, view and 'Re-Do' options.

(9)	An options box is displayed repeating the fix position and requesting the next action.  These are:

(a)	View Worksheet (for thirty seconds)

(b)	View Summary and Chart combined.  Data from the main worksheet is copied to a summary together with a chart plot for future reference

(c)	View Chart of one, two or three position lines

(d)	Print Worksheet Results (four pages containing the sight answers, intermediate results and Star positions)

(e)	Print Preview Star Chart (see below for explanation of stars)

(f)	Print Preview Chart Plot

(g)	Print Preview Summary Sheet of results and chart plot

(h)	Re-Run Astro Navigation

(i)	Quit Astro Navigation and return to Windows (and close files normally i.e. prompted to save any data changes)

Under (h) you will be prompted either to re-run the whole routine or an individual sight number if you wish to check or amend any of the data.

3.5  Explanation of GHA and Declination

The calculation of Epherimedes can seem complicated. The following section provides an explanation of each body by method with the inputs required and the options available at each stage.  If 'Automatic Calculation' on the first options box (2 above) is left selected, the model will compute Epherimedes without recourse to you for any data inputs. The following section assumes that you have chosen to select the manual method of derivation of GHA and DEC.

In each case, you can select the Method 'A' or 'B' in the 'Sight and Method' dialog box when inputting the body observed and the actual sight time.  With the calculation of GHA and DEC set to your choice, the text remains highlighted and you select the method to be used. 'A' signifies Almanac or Calculated as applicable and 'B' the use of Compact Data coefficients.

(1) Sun

Method 'A':	Select 'A' and the routine will move to the Observed Angle box and thereafter a box will be displayed informing you that the GHA and Declination are automatically calculated.  Press 'OK' to continue and the option box for the end of the sight will be displayed.

Method 'B':	As 'A' above.  Note that the information box on the calculation is not displayed if the user has selected the calculation to automatic ('B').  For years other than 1992-1995, the procedure is the same as for Planets under Method 'B' - see below for a description of the coefficients box since these figures need to be input from the relevant month in Compact Data.

(2) Stars

Method 'A':	 The system displays a box requesting the Sidereal Hour Angle and Declination for the day of the sight.  Daily/hourly corrections are dimmed since these are only needed for the planets.  The user therefore need only input two numbers for this method which can be used to check any sight using the Compact Data where the result appears to be inaccurate.

Method 'B':	This option is very flexible and allows you three sub-options for checking observed star names and their numbers.

(a)	Star Identification: input the altitude and bearing to identify the star number and name.

(b)	Star Finder: input the star number to calculate its true bearing and altitude at sight time.

(c)	Omit calculation and input a star number:  number and names are shown in the example on the following pages.  This option is the fastest since no calculations are performed at this stage.

(a)	Star Identification: A further box requests the altitude and bearing of the star.  Press 'Re-Do' to return to the Options Menu or 'OK' to continue.  The system calculates the star number and name and this is displayed.  Enter the star number and 'OK' to continue or 'Re-Do' to re-enter the altitude and bearing.  The model then moves to request the Observed Angle.

(b)	Star Finder: You are prompted to enter the star number on the Star Identification box.  The identification is dimmed.  Press 'OK' to continue or 'Re-Do' to return to the Options box.  The altitude and bearing of the star is calculated.  The options are then  'OK' to  continue or 'Re-Do' to re-enter the Star Options.  An alternative is to click the Stars Chart which will print preview the current SHA and Declination for all stars and then a plot of bearings and altitudes at the DR Time.  This is an overlay graph with the bearing on the left-hand Y scale and the altitude on the right-hand scale.  This can be reviewed in more detail using the magnification within Print Preview or printed for future reference.  This option is repeated in the End Options if missed at this stage.  You have a further option to sort the stars by bearing or altitude e.g. to check which stars should be visible.  Select the option or continue to omit and continue to input the Observed Angle.  This sort option displays a further plot for printing or reviewing on the screen with the bearing, altitude and sidereal hour angle.  The routine then moves to request the Observed Angle.

(c)	Input Star Number: The star number is input and no calculations are performed.  The routine moves directly to the Observed Angle.

(3) Moon

Method 'A':	Select Method 'A' and no inputs are required.  After the Observed Angle, a box reminds you that no inputs are required.

Method 'B':	The routine moves to the Observed Angle and displays a box requesting input of the coefficients.  The moon requires A0 to A2, for the GHA and DEC and the two coefficients for horizontal parallax.  Enter 'OK' to continue or 'Re-Do'.  The sum of the coefficients is shown as a check.  Enter 'Re-Do' if there is an error or 'OK' to continue to the Options box at the end of the sight.  To Re-enter the sight, select 'View this Sight' and then 'Re-Do' in this box.

(4) Planets

Method 'A':	After Observed Altitude the GHA and DEC box requests the Greenwich Hour Angle and Declination for 00:00 GMT on the day of the sight.  Planets also need the correction per hour and this is also found on the daily page in the Almanac.  Press 'Re-Do' to re-enter information or 'OK' to continue to the Options Box at the end of the Sight.

Method 'B':	All coefficients A0 to A4 for GHA and DEC are needed together with A0 and A1 for the semi-diameter.  These should be input and the addition checked on the next box.  Press 'Re-Do' to re-enter information or 'OK' to continue to the Options Box.

4.0  Worked Examples

4.1  Example of a Fix with the Sun, Moon and a Star

The following pages give a worked example and the results are printed on the next pages.  This is the example on an earlier page

A ship at 22:0:0 on 21 June 1994 calculated its DR Position as 32' 0'' N 15' 0'' W and has followed a course of 325.00' T at a speed of 8 knots per hour for several hours.  Three sights have been taken and a fix is now sought to check the transferred positions.

The individual sights are :

(1) Sun at 18:00:00 at an altitude of 11' 57.0''

(2) Moon at 18:23:30 at an altitude of 11' 34.0''

(3) Star thought to be Kochab (40) at 21:23:20 at an altitude of 46' 31.0'' and a bearing of 356' 0.0".

At 15' W the model calculates the local time as:

GMT + 15' W / (360 / 24) = GMT - 1 hour (the usual convention for West and South is negative).

The star finder and identification routines are used to ensure the accurate selection of the star.  A following page shows the location of all Navigational Stars for the DR time and position in this example.  Summary of results:

|  | Sun | Moon | Star (Kochab 40) |
| --- | --- | --- | --- |
| Time of Sight (GMT) | 19:00:00 | 19:23:00 | 22:23:20 |
| Difference to DR Time | (4 h 00 m) | (3 h 36 m) | (0 h 36 m) |
| Observed Angle | 11' 57.0'' | 11' 34.0'' | 47' 31.0'' |
| Adjusted Altitude | 12' 8.35'' | 12' 44.94'' | 47' 30.09'' |
| Declination | 23' 26.4'' N | 20' 17.4'' S | 74' 10.8'' N |
| GHA | 104' 33.7'' | 314' 48.56'' | 23' 1.60'' |
| LHA | 89' 55.34'' | 300' 8.13'' | 8' 4.91'' |
| Azimuth /Bearing | 290' 14.29'' | 123' 45.21'' | 356' 44.69'' (a) |
| Intercept / Distance | 3.60 Miles | 4.70 Miles | 3.26 Miles |
| Direction | Towards | Towards | Away |

The Model uses the 'Method of Least Squares' to compute a fix at the DR Time of 31' 55.65'' N 15' 1.94'' W.  This position is on a bearing of 203.34' T 4.65 miles from the DR Position.  The following page shows the worksheet print-out with the Moon and the Star calculated by each available method and the amount of user input required.

The results also show the confidence eclipse using standard deviation at 95% with its angle bearing 42.46' T.  The length of the eclipse is 8.56 miles and its width 13.33 miles.  This example has been chosen to illustrate the method and the size of the eclipse is less on the later examples.

(a) The angle is reversed since the plot is 'Away'.  Thus the corrected position line angle is 176' 44.69"

Worksheet Results

Method 'A':  Automatic Calculation of GHA and Declination: Sun / Star = Polynomial Data, Moon = Calculated.

Method 'B':  Manual Method using input data for the moon (Polynomial) and the star (Almanac)

The Moon requires the Polynomial Coefficients from Page 42 of Compact Data for 21/6/94 and the Star Sight the Sidereal Hour Angle only for Kochab (40).  The sun is calculated using Method 'A' (no inputs needed).

Chart Plot and Star Positions

A calculated fix is gained from the three position lines based on the least squares method.  The bodies observed are the Sun, the Moon and the Star Kochab (40).

Apparent Positions of the Stars for the position are given below for the DR local time.  This data is used to determine the star number from the altitude and bearing or alternatively to supply the expected altitude and bearing for a particular star.  Star Charts are shown on the next page.

The data is derived from a table of Polynomial Data which lists the coefficients necessary for calculating the apparent positions of the stars for the years 1991-1995 (see bibliography). Note: the Sidereal Hour Angles show the usual order of the 57 navigation stars here in reducing order.

Star Charts for Sight Planning

Star Positions for 32' 0"'N 15' 0"W on 21/6/94 at 22:0:0 Local (Zone) Time.  Stars are listed in number order as per the previous sheet.

In Star Planning it is useful to order stars by altitude (for visibility) and also by bearing.  Astro Navigation contains a sub-routine allowing you to review the stars in either order.  The example below is ordered by bearing and a further plot of the sidereal hour angle (SDA) is included.

4.2  Example from The Nautical Almanac

This example is taken from pages 282 and 283 of The Nautical Almanac 1994 (HMSO).

Required: calculate the position of a ship on 4 July 1994 at 21:0:0 UT which has been travelling on a constant course of 325' T for several hours at a speed of 20 knots.

Three observations have been taken:

| Sight No | Number/Star | Universal Time | Adjusted Altitude |
| --- | --- | --- | --- |
| Sight (1) | (26) Regulus | 20:39:23 UT | 27.0109' |
| Sight (2) | (42) Antares | 20:45:47 UT | 26.0764' |
| Sight (3) | (40) Kochab | 21:10:34 UT | 47.4449' |

The time is 21:0:0 UT and the estimated position (DR) is 32' N 15' W.  The Almanac calculates a final estimated position of 31.6193' N 15.0204' W (31' N 37.158" 15' W 1.224").  The plot here is 31' 37.14'' N 15' 1.06'' W.

No Hour Angle, Declination and corrections to the Observed Angle are needed with Systematic Navigation and the intermediate results using the same format as The Nautical Almanac 1994 (HMSO) are:

| Star: | Regulus | Antares | Kochab |
| --- | --- | --- | --- |
| Star Number: | 26 | 42 | 40 |
| Time of Observation | 20:39:23 UT | 20:45:47 UT | 21:10:34 UT |
| Adjusted Altitude (Ho) | 27.0109' | 26.0764' | 47.4449' |
| Greenwich Hour Angle | 80.4516' | 346.7984' | 17.6023' |
| Local Hour Angle | 65.5291' | 331.8518' | 2.5626' |
| Time Difference to DR | -0.3436 hours | -0.2369 hours | +0.1761 hours |
| Revised Latitude | 31.9062' N | 31.9353' N | 32.0481' N |
| Revised Longitude | 14.9225' W | 14.9466' W | 15.0397' W |
| Azimuth (Z) | 267.5722' | 152.0216' | 358.9596' |
| Calculated Altitude (Hc) | 26.9878' | 25.7700' | 47.8456' |
| Distance (p) | +0.0231' | +0.3064' | -0.4007' |
| Intercept (I) | 1.39 miles | 18.38 miles | 24.04 miles |
| Direction | Towards | Towards | Away |

The fix obtained with the model varies with the Almanac by 0.0004' S and 0.0028' E at 31.689' N 15.0176' W (31' N 37.1368" 15' W 1.0569").  This is due to iteration and slight differences on rounding.  The system print-out is as follows:

4.3  Automatic Position Fix with the Sun, Moon and Vega (49)

This example appears in Compact Data.  On 19/6/91 at 21:00 GMT the DR position of a ship is 32' 45'' N 15' 30'' W and it has followed a course of 325' T at a constant speed of 12 knots for several hours.  Three sights have been taken:

| Sight | Time | Altitude | Height |
| --- | --- | --- | --- |
| (1) Sun | 18:15:24 GMT | 21' 25.398'' | 6.0 metres |
| (2) Moon | 20:30:45 GMT | 46' 32.700'' | 6.0 metres |
| (3) Vega (49) | 20:42:23 GMT | 29' 27.132'' | 6.0 metres |

Using a further sight of Dubhe (27) the fix gained by Compact Data is 32' 39.1'' N 15' 34.2'' W and this compares well with the answer below, given that the Sun and Moon are calculated using another method ('A').

(5)  BIBLIOGRAPHY

Our thanks are given to the authors of the following publications which proved to be invaluable reference guides in the preparation of Systematic Navigation.

(1)	The Nautical Almanac 1994 (HM Nautical Almanac Office ISBN 0-11-772737-7)

(2)	The Macmillan and Silk Cut Nautical Almanac 1994 (Macmillan ISBN 0-333-58729-4)

(3) 	Astro Navigation by Pocket Computer by Mike Harris (Adlard Coles)

(4)	Compact Data for Navigation and Astronomy for the years 1991-1995 by B D Yallop and  C Y Hohenkerk (Cambridge University Press ISBN 0-521-38731-0)

(5)	Basic Astro Navigation by Conrad Dixon (Adlard Coles)

(6)	Astronavigische Navigation by Walter Stein and Werner Kumm (Delius Klasing Verlag / Klasing & Co., GmbH ISBN 3-87412-095-3)

(7)	Handbuch der Navigation by Winfried Böhm (BusseSeewald ISBN 3-512-00845-3)

(6)  LICENCE TERMS AND CONDITIONS

This notice is intended to be a 'no nonsense' agreement between you ('the licencee') and Systematic Navigation ('Systematic').  The software and associated documentation ('software') are subject to copyright law.  They are protected by the laws of England.  If you use this software, you are deemed to have accepted the terms and conditions under which this software was supplied.

You have a limited licence to use the software for the period of time stated on the software copyright notice and to make as many copies of the software as you wish.  This is a 'Shareware' copy software licence granted by Systematic for evaluation purposes only.

The phrase 'Shareware' is used to give the licencee maximum flexibility in the use of the licence.  This means, for example, that this software can be used by any number of people, or freely moved between computers.  You may however not make any changes or modifications to the licenced software.  You may not decompile, disassemble, or reverse-engineer the licenced software or sell the software to others.  You may freely copy the software and pass it on to others provided that you do not pass it off as your property or charge any fee.  You must include all the files and documentation on the disk without any omissions.

A full Systematic Navigation licence contains twelve Astro and terrestrial navigation options together with a full 50 page manual at a cost of £30.00 including VAT.  A full licence, for Astro Navigation only, costs £15.00.  Registration entitles you to receive all updates and additions.  Use the form on Page 19 to register your copy.

Systematic warrants that the software will perform substantially in accordance with the documentation supplied with it.  Systematic does not warrant that the functions contained in the software will meet your requirements or that the operation of the software will be uninterrupted or error free.  This warranty does not extend to changes made to the software by third parties, nor does it extend to liability for data loss, damages or lost profits due to the use of the software.

| Systematic Navigation |  | German Office: |  |  |
| --- | --- | --- | --- | --- |
| Winkworth House |  | Jesuitengasse 7 |  |  |
| 11 High Street |  | D-86152 Augsburg |  |  |
| Bramley, Surrey GU5 0HF |  | Germany |  |  |
| Telephone: | 0483 894890 |  | (49) 0821 / 15 20 38 |  |
| Telefax: | 0483 894078 |  | (49) 0821 / 15 20 38 |  |

REGISTRATION DOCUMENT

To:

Systematic Navigation

Winkworth House

11 High Street

Bramley

Surrey GU5 0HF

Attention of Mr Alastair Day

From:

Please register my 'Shareware' copy and supply a registered disk and manual:

copy/ies* of 'Systematic Navigation' at £30.00 incl. VAT each.

copy/ies* of 'Astro Navigation' at £15.00 incl. VAT each.

Our remittance in the sum of £		is enclosed

Computer Make and Type:

Please send a VAT invoice to (if different to above):

Delivery Address (if different to above):

Signed:

Authorised Signatory

Date:

Order Number:

* Insert and delete as appropriate.

Systematic Navigation: Tel 0483 894890 : Fax 0483 894078