VERSAMAP (version 1.50)

1 November 1995

Copyright 1991-1995 by Charles H. Culberson

8 Ritter Lane

Newark, Delaware  19711

U.S.A.

Phone: 302-731-0694

Internet: 70511.3134@compuserve.com

CompuServe: 70511,3134

CONTENTS                                           PAGE

1.      Introduction                                         2

1.1.    Computer Requirements for Running VERSAMAP           3

1.2.    Disclaimer                                           3

1.3.    Registration                                         3

1.4.    Plotting the Default World Map                       4

1.5.    Finding your Way with Latitude and Longitude         5

1.6.    Keys with Special Functions in VERSAMAP              6

1.6.3.  Exit the program                                     6

2.      Running VERSAMAP                                     9

3.      Projection menu                                     10

4.      Map Design menu                                     16

5.      Input/Output menu                                   32

6.      Run menu                                            41

7.      Sources of Digital Map Data for use with VERSAMAP   42

7.1.    World Data Bank II                                  42

7.2.    Micro World Data Bank II                            46

7.3.    VERSAMAP Micro World Database                       47

7.4.    USGS Digital Line Graph Data                        47

7.7.    Importing Your Own Data into VERSAMAP               52

8.      Use of VERSAMAP with Graphing Programs              55

9.      Program Compatibility with Software and Hardware    56

9.1.    Running VERSAMAP under Windows                      56

10.      Installing VERSAMAP                                 57

11.      VERSAMAP Order Form                                 58

12.      Acknowledgements                                    61

13.      Glossary                                            62

14.      References                                          65

15.      Index                                               66

VMAP.DOC version 1.50                                    Page  1

1.        Introduction

Welcome to VERSAMAP, a versatile mapping program for drawing outline

maps on 14 different map projections.  Maps can be printed on dot

matrix or laser printers; and exported as PCX, CGM, DXF, PIC, or

ASCII files, for use with paint programs, word processors, and

presentation or technical graphics programs.  VERSAMAP is supplied

with Courier and Helvetica fonts which allow text to be added to

maps displayed on the screen.

The shareware version of VERSAMAP contains the following files:

VMAPREAD.ME  --  how to get started

VMAP.EXE  --  the mapping program

VMAP4.MAP  --  digital map of the world (16,000 points)

VMAP.DOC  --  instruction manual

REGISTER.DOC  --  order form

VMTAIWAN.PKV  --  sample map of Taiwan from World Data Bank II

VMOAHU.GRV  --  sample map of Oahu from USGS DLG database

VMAPUSER.PRN  --  ASCII file containing latitude, longitude data,

study this file to learn how to plot your own

data with VERSAMAP

VMAPUSER.UTM  --  ASCII file containing coordinates in meters,

study this file to learn how to plot your own

UTM data with VERSAMAP

VM_FIX.MPS  --  utility file used to concatenate World

Digitized binary map files

VM_FIX.MP1  --  utility file used to concatenate World

Digitized ASCII map files.

VMVENDOR.DOC  --  information for shareware vendors

VENDINFO.DIZ  --  information for shareware vendors

FILE_ID.DIZ  --  information for BBS operators

COUR_EGA.FON  --  fixed-space Courier font for EGA

COUR_VGA.FON  --  fixed-space Courier font for VGA

The following additional files are supplied with the registered

version of VERSAMAP:

HELV_EGA.FON  --  proportional Helvetica font for EGA

HELV_VGA.FON  --  proportional Helvetica font for VGA

VMAP3.MAP  --  digital map of the world (31,000 points)

VMAP5.MAP  --  digital map of the world  (6,000 points)

VMCITY.PRN  --  ASCII file that can be plotted from VERSAMAP

containing positions and names of 606 world

cities

VMAP.DOC version 1.50                                    Page  2

Additional, more detailed maps are also available; see Section 7 for

details.  Please make backup copies of the VERSAMAP distribution

disks before running the program.

1.1.      Computer Requirements for Running VERSAMAP

MS-DOS 3.0 or higher;

EGA or VGA;

640K memory;

hard disk.

A math coprocessor is not required to run VERSAMAP but the program

will run 3-4 times faster if one is installed.

VERSAMAP will not change your AUTOEXEC.BAT or CONFIG.SYS files, it

will not alter your keyboard settings, and it will not write any

files to disk without your permission.

Instructions for installing VERSAMAP are given in the file

VMAPREAD.ME on the VERSAMAP distribution disk.

1.2.      Disclaimer

Users of VERSAMAP must accept this disclaimer of warranty:

"VERSAMAP is supplied as is.  The author disclaims all warranties,

expressed or implied, including, without limitation, the warranties

of merchantability and of fitness for any purpose.  The author

assumes no liability for damages, direct or consequential, which may

result from the use of VERSAMAP."

Do not use this program for navigation!  Use the proper aeronautical

or nautical charts for navigation.

1.3.      Registration

Shareware distribution gives users a chance to try software before

buying it.  VERSAMAP is a "shareware program" and is provided at no

charge to the user for evaluation.  If you find this program useful

and find that you continue to use VERSAMAP after a reasonable trial

period, you must register the program.  The registration fee will

license one copy for use on any one computer at any one time.  You

must treat this software just like a book.  An example is that this

software may be used by any number of people and may be freely moved

from one computer location to another, so long as there is no

possibility of it being used at one location while it's being used

at another.  Just as a book cannot be read by two different persons

at the same time.

VMAP.DOC version 1.50                                    Page  3

Registration of the program is $15 for which you will receive a copy

of the most recent version of the program; Helvetica fonts for EGA

and VGA; the additional digital map files VMAP3.MAP (31,000 points)

and VMAP5.MAP (6,000 points); and a gazetteer, VMCITY.PRN,

containing the names and positions of 606 major world cities.

Registration entitles you to support by mail, telephone, and

electronic mail, concerning problems you encounter with VERSAMAP.

An order form is included in the file REGISTER.DOC, and in Section

11 of this manual.

You are encouraged to share a copy of VERSAMAP with your friends for

evaluation.  Please encourage them to register their copy if they

find it useful.

This program is produced by a member of the Association of Shareware

Professionals (ASP).  ASP wants to make sure that the shareware

principle works for you.  If you are unable to resolve a

shareware-related problem with an ASP member by contacting the

member directly, ASP may be able to help.  The ASP Ombudsman can

help you resolve a dispute or problem with an ASP member, but does

not provide technical support for members' products.  Please write

to the ASP Ombudsman at 545 Grover Road, Muskegon, MI 49442-9427,

USA; FAX 616-788-2765; or send a CompuServe message via CompuServe

Mail to ASP Ombudsman 70007,3536.

1.4.      Plotting the Default World Map

Before reading the rest of the manual, let's plot a map of the

world.  This will familiarize you with the operation of VERSAMAP.

The rest of the manual should make more sense once you have actually

used VERSAMAP.

VERSAMAP has built in defaults which plot a world map, from 90 N to

90 S latitude, and from 180 W to 180 E longitude, on an

Equirectangular projection.  The default settings will not print any

files to disk.

To run VERSAMAP with the default settings:

1.  change to the drive and directory containing VERSAMAP;

2.  type VMAP;

3.  press the Enter key.

A screen with a menu bar across the top will appear once the program

has loaded.  The pull-down menus are selected by pressing the Alt

key, followed by the highlighted letter of the desired menu; or by

pointing and clicking the left mouse button.

To plot the world map:

1.  press the Alt key, to access the menus

2.  press R, to select the Run menu

3.  press Enter, to select the Size 100% option and begin the plot.

VMAP.DOC version 1.50                                    Page  4

When the map begins plotting, the top line of the screen will

display (1) a reminder that pressing the Esc key will terminate

plotting and return you to the menu bar; (2) the name of the map

input file being read, VMAP4.MAP in this case; and (3) the number of

points which have been read from the input file, VMAP4.MAP.

The menu bar will reappear when the map is finished plotting.  The

default map takes 11 seconds to plot on a 33 MHz 486 computer.

To exit the program and return to DOS:

1.  press the Esc key, to halt plotting;

2.  press the Alt key, to access the menus;

3.  press P, to select the Projection menu;

4.  press X from the Projection menu to exit the program.

Pressing the Esc key halts program execution and returns you to the

menu bar.  Pressing Esc will not exit the program.

Please read the rest of the instruction manual before attempting to

plot maps which do not use the default parameters.

1.5.      Finding your Way with Latitude and Longitude

To use this program you should be familiar with the concepts of

latitude and longitude.

Latitude is the angular distance (in degrees) of a point on the

earth's surface north or south of the equator.  The maximum values

of latitude are 90 N (north pole) and 90 S (south pole).  The

equator is 0 N/S latitude.

Longitude is the angular distance (in degrees) of a point on the

earth's surface east or west of the meridian that passes through

Greenwich, England.  The maximum values of longitude are 180 W and

180 E (the international date line).  The meridian passing through

Greenwich is 0 E/W longitude.

Latitude and longitude values in VERSAMAP are entered as degrees,

minutes, seconds.  There are 60 seconds in 1 minute, and 60 minutes

in 1 degree.  One degree of latitude is equal to 60 nautical miles

(1 nautical mile = 6080 feet = 1.8532 kilometers = 1.1515 statute

miles).  One minute of latitude is equal to 1 nautical mile.  One

second of latitude is equal to 101 feet (31 meters).

For example, the position for Rio de Janeiro, Brazil, written as

degrees, minutes, and seconds (22 degrees, 53 minutes, 43 seconds

south latitude; 43 degrees, 13 minutes, 22 seconds west longitude)

is,

VMAP.DOC version 1.50                                    Page  5

Deg Min Sec

latitude   =   22  53  43  S

longitude  =   43  13  22  W

The position for Paris, France (48 degrees, 50 minutes, 14 seconds

north latitude; 2 degrees, 20 minutes, 14 seconds east longitude)

is,

Deg Min Sec

latitude   =   48  50  14  N

longitude  =   02  20  14  E

If you are unfamiliar with the use of latitude and longitude to

locate points on the surface of the earth, please consult an

encyclopedia or an atlas.

1.6.      Keys with Special Functions in VERSAMAP

1.6.1.    Esc

Pressing the Esc key halts program execution and returns you to the

menu bar.  Pressing Esc will not exit the program.

If you are in the process of plotting or printing a map, the program

checks the Esc key each time the record number or point, shown in

the upper right corner of the screen, is updated.  Consequently, the

program may take several seconds to respond after Esc is pressed.

1.6.2.    Alt

Pull-down menus are selected by pressing the Alt key, followed by

the highlighted letter of the desired menu.  Individual menu items

are selected by pressing the highlighted letter of each item; by

moving through the menu with the up or down cursor keys, and then

pressing Enter to select the item; or by moving the mouse cursor

over the desired item and pressing the left mouse button.

1.6.3.    Exit the program

To exit the program and return to DOS:

1.  press the Esc key, several times if necessary, to halt plotting

or erase the current window(s);

2.  press the Alt key, to access the menus;

3.  press P, to select the Projection menu;

4.  press X from the Projection menu to exit the program.

VMAP.DOC version 1.50                                    Page  6

If a window is on the screen or a map is in the process of being

drawn, you must press Esc before you can access the menus and exit

the program.

1.6.4.    Letter B

Pressing the letter B erases the menu bar and changes the background

color of the menu bar to the background color of the map.  This

feature is useful if you use a screen-capture program to save maps

displayed on the screen.  You must press B a second time, or press

Esc, to redisplay the menu bar and reactivate the pull-down menus.

1.6.5.    Letter T

Pressing the letter T toggles the menu bar on and off; and replaces

it with the name of the current projection, and the time taken to

draw the current map.  You must press T a second time, or press Esc,

to redisplay the menu bar and reactivate the pull-down menus.

1.6.6.    Using a mouse

Most VERSAMAP features can be accessed by pointing and clicking the

left mouse button.  Windows and pull-down menus can be closed by

clicking the left mouse button when the mouse cursor is outside the

window or menu box.

1.6.7.    Moving around windows

The windows which pop up when menu selections are made contain

several types of items: (1) input fields; (2) vertical list boxes;

and (3) pushbuttons.

Input fields allow numeric and alphabetic input into the program.

An example of a window with numeric input fields is the LATITUDE AND

LONGITUDE LIMITS window displayed after selecting the Map Boundaries

option in the Map Design menu.  You can move within an input field

with the right, left, Home, and End cursor keys.  To enter data into

an input field, type the correct characters from the keyboard, and

press Enter.  Use the Tab key to move between lines in this window.

Vertical list boxes allow one or more items in a list to be chosen.

An example of a window with a vertical list box is the SELECT MICRO

WORLD DATA BANK II FEATURES window displayed after selecting the

Geographic Features option in the Map Design menu.  Move through the

list with the up and down cursor keys, and select the correct

item(s) by pressing the Space Bar or Enter.  The item(s) selected

will be highlighted or indicated with a checkmark.

VMAP.DOC version 1.50                                    Page  7

If the number of items in the list is too large to fit inside the

box, a scroll bar will appear along the right side of the box,

allowing all items to be viewed.

Pushbuttons, indicated by text inside brackets, such as < ok >,

allow the user to control program execution.  An example of a window

with a pushbutton is the LATITUDE AND LONGITUDE LIMITS window

displayed after selecting the Map Boundaries option in the Map

Design menu.  The text inside the brackets indicates the action to

be taken when the pushbutton is selected.  The brackets of the

currently active pushbutton are highlighted, and its text is shown

in all capital letters.  To select a pushbutton, tab to that button

and press Enter.

Use the Tab key to move between items when more than one list box or

pushbutton occur in a window.

VMAP.DOC version 1.50                                    Page  8

2.        Running VERSAMAP

Each map projection in VERSAMAP has built in latitude/longitude

defaults which are active the first time you access the projection

during each session with VERSAMAP.  These defaults allow you to plot

maps using each map projection without getting error messages.  The

defaults are only active the first time you access a particular map

projection.  For instance, if you access the Mercator projection,

then the Equirectangular projection, and then return to the Mercator

projection, the latitude limits of the grid will be those of the

previous Equirectangular projection, 90 N/S, and you will get an

error message if you attempt to plot a Mercator projection without

changing the grid.

Sections 3 through 6 provide a detailed discussion of each VERSAMAP

pull-down menu.  For your first experience with VERSAMAP you should

run the program with its default settings (see Section 1.4).

When VERSAMAP is started, the following menu bar appears at the top

of the screen.

-----------------------------------------------------------------

Projection      Map Design      Input/Output      Run

-----------------------------------------------------------------

When you begin plotting a map, by selecting an option in the Run

menu, the menu bar is replaced by the following line.  This line is

not a menu bar, it is for your information only.

-----------------------------------------------------------------

Esc: goto menu         Input file:             Reading record:

-----------------------------------------------------------------

While a map is plotting, the top line of the screen displays (1) a

reminder that pressing the Esc key will terminate plotting and

return you to the menu bar; (2) the name of the current map input

file;  and (3) the number of points which have been read from the

input file.

Once the map is completed, the menu bar reappears on the top line.

VMAP.DOC version 1.50                                    Page  9

3.        Projection menu

-----------------------------------------------------------------

Projection      Map Design      Input/Output      Run

+----------------------------------------------------------------

| Mercator                  |

| Equirectangular           |

| Hammer equal-area         |

| Robinson                  |

|---------------------------|

| Equidistant conic         |

| Conformal conic           |

| Albers equal-area         |

|---------------------------|

| Gnomonic                  |

| Stereographic             |

| Orthographic              |

| Vertical Perspective      |

| Azimuthal equidistant     |

| Azimuthal equal-area      |

|---------------------------|

| UTM                       |

|---------------------------|

| Exit                      |

+---------------------------+

This menu lists the map projections on which you can plot your map.

VERSAMAP assumes that the earth is a sphere of radius 6,371 km for

all projections except the Universal Transverse Mercator (UTM).  The

UTM projection assumes that the earth is an ellipsoid.

Maps can be classified by the type of projection (azimuthal, conic,

cylindrical) used in their construction; or by their common

properties (azimuthal, conformal, equal-area, and equidistant).

On azimuthal projections, the direction from the central point of

the map to any other point is correct, and the scale of the map is

constant along any circle centered on the map center.

On conformal projections, the map scale is the same in all

directions at a given point.

On equal-area projections, the relative areas of all regions on the

map are correct.  However, shapes of particular areas may be greatly

distorted.

On equidistant projections, the scale is constant along all great

circles from one or two points.

VMAP.DOC version 1.50                                    Page  10

3.1.      Mercator

A conformal projection on which meridians and parallels are parallel

lines perpendicular to each other.  Meridians are equally spaced,

parallels are unequally spaced.  The north and south poles cannot be

plotted because they project to infinity.

The scale is true along the equator, or along two parallels

equidistant from the equator.  Since the scale increases rapidly

away from the equator, the areas of polar regions are greatly

exaggerated compared to those of equatorial regions.

The Mercator projection is commonly used for maps of the world, and

is designed so that straight lines on the map are rhumb lines (lines

of constant compass bearing).  This is a useful property in

navigation and many nautical charts use the Mercator projection.

3.2.      Equirectangular

Meridians and parallels are equally spaced parallel lines

perpendicular to each other.  The scale is true along the two

standard parallels and along all meridians.  The scale along

parallels is too small between the equator and the standard

parallels, and too large between the standard parallels and the

poles.

The map can be visualized as projected on a cylinder which

intersects the globe at two standard parallels equidistant from the

equator.  VERSAMAP allows the user to select the latitude of the

standard parallels.

The fact that meridians and parallels are equally spaced makes

interpolation easy.  The projection is useful for maps of small

regions, since distortion is not large if one of the standard

parallels is chosen to bisect the region under consideration.

3.3.      Hammer Equal-Area

An equal-area projection useful for maps of the entire world.

The central meridian is a straight line.  Other meridians are curves

concave to the central meridian and intersecting at the poles.  The

equator is a straight line; other parallels are arcs concave toward

the nearest pole.

The scale of the projection is only correct along the equator and

the central meridian.

VMAP.DOC version 1.50                                    Page  11

3.4.      Robinson

This projection is useful for general purpose world maps with a

pleasing visual appearance.  The National Geographic Society uses

the Robinson projection for world maps.

The central meridian and all parallels are straight lines.  Other

meridians are curves concave to the central meridian.  The map scale

is true along 38 N and 38 S latitudes.

3.5.      Equidistant Conic

An equidistant conic projection on which meridians are equally

spaced lines converging at a point beyond one of the poles.

Parallels are equally spaced circular arcs centered on the point

where the meridians converge.  The poles are circular arcs of

infinite scale.  Distortion on the projection is intermediate

between that of equal-area and conformal conic projections.

The scale is true along meridians and along two standard parallels.

VERSAMAP allows the user to select the latitudes of the standard

parallels.

3.6.      Conformal Conic

A conformal conic projection on which meridians are equally spaced

lines converging at a point, which is one of the poles.  Parallels

are unequally spaced circular arcs centered on the point where the

meridians converge.  The pole opposite the point of convergence

cannot be shown.

The scale is true along two standard parallels, which VERSAMAP

allows the user to select.

3.7.      Albers Equal-Area Conic

An equal-area conic projection on which meridians are equally spaced

lines converging at a point beyond one of the poles.  Parallels are

unequally spaced circular arcs centered on the point where the

meridians converge.  The poles are shown as circular arcs of

infinite scale.  This projection is commonly used to maps of the 48

conterminous United States.

The scale is true along two standard parallels, which VERSAMAP

allows the user to select.  Since the projection is equal area, the

scale at any point on a meridian is the reciprocal of the scale

along a parallel at that point.

VMAP.DOC version 1.50                                    Page  12

3.8.      Gnomonic

An azimuthal projection with the characteristic that great circles

plot as straight lines.  This projection is commonly used for long

distance navigation, since the shortest distance between two points

on the projection is the straight line connecting them.

The range of the Gnomonic projection must be less than 90 degrees,

relative to the center of the map.  The scale of the projection is

only correct at the map center, and distortion increases rapidly

away from the center.

3.9.      Stereographic

A conformal azimuthal projection.  The polar aspect of this

projection is commonly used for maps of polar regions.

The range of the projection cannot be more than 90 degrees, relative

to the center of the map.

3.10.     Orthographic

An azimuthal projection which has the appearance of a globe, and

which resembles the earth as seen from a great distance in space.

The range of the projection cannot be more than 90 degrees, relative

to the center of the map.  Map scale is only true at the center of

the map.

3.11.     Vertical Perspective

An azimuthal projection which shows the earth as it appears from any

point above the earth's surface.  The projection provides views of

the earth resembling those seen from space.

The area of the earth's surface that can be displayed by this

projection is less than 1 hemisphere.  VERSAMAP allows the user to

select the distance above the surface from which the earth is

viewed.  The default distance is 35,840 km (the distance of

geosynchronous communication and weather satellites above the

equator).

3.12.     Azimuthal Equidistant

An azimuthal projection on which the distance between the map center

and any other point on the map is true.  By centering the map at the

location of interest, you can determine the distance from that point

to any other place on the earth.

VMAP.DOC version 1.50                                    Page  13

This projection can show the entire globe.  The scale of the map is

true along any straight line radiating from the center of the

projection.

3.13.     Azimuthal Equal-Area

An equal-area azimuthal projection which is recommended for maps of

nearly circular regions of the globe.

The projection can show the entire globe.  The scale of the

projection is only correct at the center of the map.

3.14.     UTM - Universal Transverse Mercator

The Transverse Mercator projection is a normal Mercator projection

which has been rotated 90 degrees so that the projection is tangent

to the earth along a central meridian, rather than along the

equator.  If the scale along the central meridian is made slightly

less than 1, the projection becomes tangent to the earth along 2

small circles equidistant from the central meridian.  The scale

factor of the central meridian is often set equal to 0.9996. In this

case, lines of true scale (scale 1.0000) occur 180 km (97 nautical

miles) on either side of the central meridian.  Because scale errors

on the Transverse Mercator are small near the central meridian, this

projection is commonly used to map areas near the central meridian.

The projection is not used to map large regions.

When applied to an ellipsoidal earth, the Transverse Mercator is

also called the Gauss-Kruger projection.  The UTM is a special case

of the Gauss-Kruger projection.

The UTM projection differs from the other projections in VERSAMAP in

several respects.

1.  It assumes that the earth is an ellipsoid, not a sphere.

Although this assumption complicates the mathematics, it is

necessary for accurate mapping of small areas.  All other

projections used by VERSAMAP assume that the earth is a sphere

of radius 6,371 km.

2.  Positions are expressed in meters, not as degrees of latitude

or longitude.

3.  Positions can be located more precisely on the UTM than on the

other projections used by VERSAMAP.  Positions on the UTM can

be located to fractions of a meter; whereas positions on the

other projections, which are rounded to the nearest second of

latitude and longitude, are precise to about 30 meters.

VMAP.DOC version 1.50                                    Page  14

The UTM was adopted by the U.S. Army in 1947 for large scale maps of

the world.  For use with the UTM, the earth is divided into 60

longitudinal zones, each 6  wide, running east from 180 W.  The

first zone is centered on 177 W longitude; the 30th zone on 3 W; the

31st zone on 3 E; and the 60th zone is centered on 177 E.

Latitudinal bounds for the UTM extend from 84 N to 80 S.  The

Universal Polar Stereographic (UPS) projection is used for maps of

the polar regions extending north of 84 N or south of 80 S.

The UTM projection in VERSAMAP can also be used to plot maps on the

Gauss-Kruger projections used for various national grids.  The user

may select the ellipsoid; the scale factor of the central meridian;

the longitude of the central meridian; the latitude of the map

origin; the false easting; and the false northing.

VMAP.DOC version 1.50                                    Page  15

4.        Map Design menu

-----------------------------------------------------------------

Projection      Map Design      Input/Output      Run

-----------------------------------------------------------------

| Map Boundaries        |

| Standard Parallels    |

| Great Circle          |

| Grid Type             |

| Geographic Features   |

|-----------------------|

| Add Text to Map       |

| Select Font           |

| Display Previous Text |

|-----------------------|

| Labels                |

|-----------------------|

| Background Color      |

| Grid Color            |

|-----------------------|

| Save Map Design       |

| Recall Map Design     |

+-----------------------+

This menu allows you to modify the design of your map.  You may

change the boundaries of the map (ZOOM in or out); select the

standard parallels, if any; define a Great Circle to be plotted;

define the type of grid superimposed upon the map; select the

geographic features that you wish to plot; add text to the map;

display labels associated with map points; change the background and

grid colors of the map; and save your selected map parameters for

recall at a later time.

4.1.      Map Boundaries

This option allows you to increase or decrease (ZOOM in or out) the

area covered by a map.  After selecting this item, one or more

windows appear in which the boundaries of the map projection are

entered.  Data entered define the area to be plotted, and the

spacing of grid marks on the finished map.

4.1.1.    Non-azimuthal projections

The following items are displayed in the LATITUDE AND LONGITUDE

LIMITS window for non-azimuthal projections (Mercator,

Equirectangular, Hammer equal-area, Robinson, Equidistant conic,

Conformal conic, Albers equal-area).

VMAP.DOC version 1.50                                    Page  16

Northernmost latitude of the map?

Southernmost latitude of the map?

Degrees of latitude between grid marks?

Westernmost longitude of the map?

Easternmost longitude of the map?

Degrees of longitude between grid marks?

For the non-azimuthal projections, you define the map boundaries by

specifying the northern and southern latitudes, and western and

eastern longitudes of the map.

VERSAMAP requires that the grid spacing you select for non-azimuthal

projections yield equally spaced divisions between the northern- and

southernmost latitudes, and between the western- and easternmost

longitudes.  You will get an error message, requiring you to edit

the data, if the degrees between grid marks that you have chosen do

not yield equally spaced divisions along the axes.  For example, the

northernmost latitude of the default Equirectangular projection is

90 N and the southernmost latitude is 90 S.  The difference between

the northernmost and southernmost latitudes is 180 degrees of

latitude.  The Degrees of latitude between grid marks for the

default Equirectangular projection is 30 degrees, which divides

evenly into the distance between the northernmost and southernmost

latitudes (180/30 = 6.00).  However, you would get an error message

if you tried to set the Degrees of latitude between grid marks to 19

degrees, since 19 does not divide evenly into 180 (180/19 = 9.47)!

Once you have entered the items in the LATITUDE AND LONGITUDE LIMITS

window and pressed the < ok > button, the program checks for errors

in the latitude/longitude limits of your map projection, and for

errors in grid spacing.  If the program detects an error, it prints

an error message on the screen telling you to correct the mistake.

For instance, the program will not allow a Mercator projection to be

plotted if the Northernmost latitude is 90 N (north pole) or if the

Southernmost latitude is 90 S (south pole).  Try entering 90 N as

the Northernmost latitude limit for a Mercator projection.  In this

case, an error message will appear, forcing you to edit the data.

Pressing the < cancel > button allows you to exit the window and

ignore any changes made to the latitude and longitude limits of the

map.  Pressing < cancel > restores the latitude and longitude limits

in use when the window was opened.

4.1.2.    Azimuthal projections

For azimuthal projections (Gnomonic, Stereographic, Orthographic,

Vertical Perspective, Azimuthal equidistant, Azimuthal equal-area)

the following items are displayed in the LATITUDE AND LONGITUDE

LIMITS window.

VMAP.DOC version 1.50                                    Page  17

Central latitude of the map?

Degrees of latitude between grid marks?

Central longitude of the map?

Degrees of longitude between grid marks?

Range of map in degrees?

The map boundaries of azimuthal projections are defined by

specifying the position at the center of the map (central latitude,

central longitude), and the range of the map.  The range of the map

is the arc distance, in degrees, from the center of the map to the

edge of the map.  Note, that in VERSAMAP, all azimuthal map

projections plot with a circular shape.  Since the central meridian

of the azimuthal projections is a straight vertical line, the Range

of map in degrees is best thought of as the distance in latitude,

along the central meridian, from the center of the map to the top

edge of the map.

For azimuthal projections, VERSAMAP requires that the Degrees of

latitude between grid marks that you select, yield equally spaced

divisions when divided into 180 degrees; and that the Degrees of

longitude between grid marks that you select, yield equally spaced

divisions when divided into 360 degrees.  You will get an error

message, requiring you to edit the data, if the degrees between grid

marks that you have chosen do not yield equally spaced divisions

when divided into 180 or 360 degrees.  For example, the Degrees of

latitude between grid marks for the default Stereographic projection

is 30 degrees, which divides evenly into 180 degrees (180/30 =

6.00). However, you would get an error message if you tried to set

the Degrees of latitude between grid marks to 25 degrees, since 25

does not divide evenly into 180 (180/25 = 7.20)!

For the Vertical Perspective projection (satellite view), the item

Range of map in degrees is replaced by Distance above surface in

kilometers.  As the distance from the earth's surface increases, the

area (range) of the Vertical Perspective projection increases.  The

default distance, 35840 km, is the distance of geosynchronous

communications and weather satellites above the equator.

4.1.3.    UTM projection

Three windows are used to define the parameters needed to plot a map

on the UTM projection.

VMAP.DOC version 1.50                                    Page  18

4.1.3.1.  UTM spheroid

The first window, SELECT UTM SPHEROID, allows the user to select 1

of 9 ellipsoids that define the earth's shape.  The following table

lists spheroids used for particular regions of the earth.

Spheroids used to map different regions of the earth in the UTM

and the Gauss-Kruger projections.

Spheroid               Scale       Region

Factor

Spheroids for the UTM Projection

Clarke (1866)          0.9996      North America, Philippines

Clarke (1880)                      France, most of Africa

Everest (1830)                     India, Burma, Pakistan,

Afghanistan, Thailand

Bessel (1841)                      Most of Central Europe,

Chile, China, Indonesia

International (1924)               Remainder of world

Spheroids for Gauss-Kruger Projections

Airy (1830)            0.9996      Britain, Ireland

Krasovsky (1940)       1.0000      Soviet Union, Warsaw Pact

Once a spheroid has been selected, the parameters defining its shape

(equatorial radius and flattening) are listed in the Map Parameter

Report option of the Input/Output menu.

4.1.3.2.  UTM parameters

The second window, SELECTION OF UTM PARAMETERS, allows the user to

select the following map parameters.  The default values shown below

plot a map of Ireland.

Central longitude of UTM zone?  009 00 00 W

Latitude of map origin?   00 00 00 N

False  easting?   0500000

False northing?  00000000

Scale factor of central longitude?  0.9996

For the UTM projection, the only parameters which vary are the

central longitude of the UTM zone and the false northing.  All other

parameters are constant.

VMAP.DOC version 1.50                                    Page  19

The central longitude of a given UTM zone is calculated as follows,

Central longitude of UTM zone = -183 + 6 * (zone number)

where negative values represent west longitude.  Ireland is in zone

29, which runs from 12 W to 6 W.

The false easting and false northing are added to the calculated x,y

coordinates so that positions on the map are always positive.  On

the UTM projection, the false northing is 0 for maps in the northern

hemisphere, and 10,000,000 for maps south of the equator; the false

easting is always 500,000.

For the Gauss-Kruger projection, the above parameters may vary

depending on the national grid chosen.  The following table gives

values for Britain, Ireland, and countries of the former Warsaw

Pact.

Gauss-Kruger Projection Parameters for National Grids

Britain     Ireland     Warsaw Pact

Central longitude:   02 00'W      08 00'W      UTM zone

Latitude of map origin:   49 00'N      53 30'N      00 00'N

False  easting:   0400000      0200000      0500000

False northing:  00100000     00250000     00000000

Scale factor:   0.9996       0.9996       1.0000

Spheroid:     Airy        Airy       Krasovsky

The central longitude for the Warsaw Pact countries is the same as

the equivalent UTM longitude.

4.1.3.3.  UTM boundaries

The third window, BOUNDARIES OF UTM PROJECTION, allows the user to

define the map boundaries and grid spacing.  The default values

shown below plot a map of Ireland.

Northern boundary of map in meters?  06200000

Southern boundary of map in meters?  05600000

Meters between north/south grid marks?  00100000

Western boundary of map in meters?   0300000

Eastern boundary of map in meters?   0800000

Meters between east/west grid marks?   0100000

VMAP.DOC version 1.50                                    Page  20

The boundaries of the UTM projection are defined by specifying their

coordinates in meters.  The x-axis runs from left to right; the

y-axis from bottom to top.

VERSAMAP requires that the grid spacing you select for the UTM

projection yield equally spaced divisions between the northern and

southern boundaries, and between the western and eastern boundaries.

You will get an error message, requiring you to edit the data, if

the meters between grid marks that you have chosen do not yield

equally spaced divisions along the axes.  In the above example, the

northern boundary of the default UTM projection is 6,200,000 meters

and the southern boundary is 5,600,000 meters.  The difference

between the northern and southern boundaries is 600,000 meters.  The

Meters between north/south grid marks for the default UTM projection

is 100,000 meters, which divides evenly into the distance between

the northern and southern boundaries (600,000/100,000 = 6.00).

However, you would get an error message if you tried to set the

Meters between north/south grid marks to 125,000 meters, since

125,000 does not divide evenly into 600,000 (600,000/125,000 = 4.8)!

4.1.3.4.  Estimation of UTM coordinates

The conversion between geographic coordinates (latitude and

longitude in degrees) and UTM coordinates (meters) is not obvious.

The following paragraphs show how to estimate the x,y coordinates of

a UTM projection given the latitude and longitude limits of the map.

Since 90 degrees of latitude is nearly equal to 10,000,000 meters,

the y coordinate corresponding to a given latitude is approximately,

y coordinate = false northing + (10,000,000/90)*latitude

= false northing + 111111*latitude

For example, the y coordinate for a point at 50 N latitude is

approximately 5,555,550 meters; for a point at 50 S the y coordinate

is about 4,444,450 meters (10,000,000 - 111111*(-50)).

To determine the x-axis limits of a UTM projection, first determine

the central longitude of the UTM zone in which the map occurs,

Central longitude of UTM zone = -183 + 6 * (zone number)

Use the following criteria to check your calculated value for the

central longitude: central longitude of UTM zone plus 3 must be

evenly divisible by 6.  In equation form,

(Central longitude of UTM zone + 3)/6 = integer

VMAP.DOC version 1.50                                    Page  21

Once you have determined the central longitude, set the Western

boundary of map in meters to 0 meters, and the Eastern boundary of

map in meters to 1,000,000 meters.  This will draw a map 1000 km

wide, centered on the central longitude.  You can then gradually

increase the western boundary and decrease the eastern boundary

until the map covers the area of interest.

4.2.      Standard Parallels

After selecting this item, a window appears in which the latitudes

of the standard parallels are entered.

4.2.1.    Conic Map Projections

The conic projections in this program (equidistant conic, conformal

conic, Albers equal-area) require that you select 2 standard

parallels of latitude.  The scale of the map is true along the

standard parallels; the spacing of the parallels determines the

angle between meridians on the map.

There are 2 constraints on selection of the standard parallels:

1.  they must lie between the northern- and southernmost latitudes

of your grid; and

2.  they cannot be symmetrical about the equator.  Standard

parallels at 20 N and 20 S are not allowed, because this would

turn the cone into a cylinder.

The program places default values for the standard parallels at 1/6

and 5/6 of the distance between the northern- and southernmost

latitudes of the grid.

When this screen appears, the cursor will be in the first position

of the degrees column of the northernmost standard parallel.  If the

default values are acceptable, tab to the < ok > button and press

Enter to continue.

To change the values, tab to the line to be edited, enter the new

value in the field you wish to edit, and press Enter.  The cursor

will skip to the next field.  Enter a new value if desired and press

Enter.  Continue until you have edited the last field.  When done,

tab to the < ok > button and press Enter.

If you attempt to enter standard parallels which are equally spaced

about the equator, the program will print an error message and force

you to change the latitude of one or both parallels.

Pressing the < cancel > button allows you to exit the window and

ignore any changes made to the standard parallels.  Pressing

< cancel > restores the standard parallels in use when the window

was opened.

VMAP.DOC version 1.50                                    Page  22

4.2.2.    Equirectangular Map Projection

This projection requires 2 standard parallels at equal distances

north and south of the equator.  You may only select the northern

standard parallel.  The other parallel is automatically placed in

the southern hemisphere at an equal distance from the equator.

The program places default values for the standard parallels at 1/2

the distance between the northern- and southernmost latitudes of the

grid if the northern- and southernmost latitudes lie in the same

hemisphere.  If the northern- and southernmost latitudes of the grid

lie in opposite hemispheres, then the standard parallels are placed

at 1/2 the distance between the equator and whichever of the

northern- or southernmost latitudes of the grid lies closer to a

pole.

When this screen appears, the cursor will be in the first position

of the degrees column of the northernmost standard parallel.  If the

default values are acceptable, tab to the < ok > button and press

Enter to continue.

To change the values, enter a new value in the field being edited

and press Enter.  The cursor will skip to the next field.  Enter a

new value if desired and press Enter.  Continue until you have

edited the last field.  When done, tab to the < ok > button and

press Enter.

Pressing the < cancel > button allows you to exit the window and

ignore any changes made to the standard parallels.  Pressing

< cancel > restores the standard parallels in use when the window

was opened.

4.3.      Great Circle

This option allows you to define the endpoints of a Great Circle,

the shortest distance between 2 points on the earth's surface.  The

default endpoints will plot the Great Circle between San Francisco

and Kuwait.  The course and distance at selected waypoints along the

Great Circle can be viewed by selecting the Great Circle Report

option from the Input/Output menu.

To plot a Great Circle, enter the starting and ending latitudes and

longitudes, tab to the < ok > button, and press Enter.  Once you

press < ok >, a second window titled SELECT GREAT CIRCLE COLOR will

appear, allowing you to plot the Great Circle in any 1 of 16 colors.

The Great Circle will be displayed on the next map to be plotted,

and will be shown on all successive maps.  To display a Great Circle

you have defined, select one of the Size options from the Run menu.

To stop plotting a Great Circle that you have previously defined,

tab to the < cancel > button, and press Enter.

VMAP.DOC version 1.50                                    Page  23

4.4.      Grid Type

This window allows you to select the type of grid that you wish

superimposed on your map.  You have several choices,

1.  the outer boundary of the grid shown as a solid line and the

interior grid points plotted as individual points (the

default);

2.  the outer boundary of the grid shown as a solid line and no

interior grid points plotted;

3.  all grid points connected by lines;

4.  all grid points plotted as individual points with no lines

connecting them; and

5.  no grid (the map will be plotted without any latitude or

longitude markings).

To get a quick view of the appearance of the different grid choices,

select a grid type, then enter a null string (i.e. no file name) for

the map input filename, and finally plot the map.  In this case, the

program will plot the grid, but no map, on the screen.

In VERSAMAP, the azimuthal map projections (Gnomonic, Stereographic,

Orthographic, Vertical Perspective, Azimuthal equidistant, Azimuthal

equal-area) are truncated to fit a circular shape, and points that

lie outside the bounding circle are not plotted.  Due to the complex

shape of the grid for these projections, the outside boundary of the

map may not be well marked when grid type 4 (all grid points plotted

as individual points) is selected.  To indicate the edge of the map

for azimuthal projections with grid type 4, VERSAMAP plots a

bounding circle, composed of discrete points spaced 3 degrees apart.

4.5.      Geographic Features

After selecting this item, a window appears which contains a list of

geographic features (coastlines, islands, lakes, rivers, countries,

U.S. States, Canadian Provinces, Australian States) in Micro World

Data Bank II (MWDB-II) files which can be plotted by VERSAMAP.  Move

through the vertical list with the up and down cursor keys, and

toggle the features on or off with the space bar.  Checkmarked

features will be plotted.  The defaults plot all features.  Move to

Proceed and press Enter to exit the window.

You may select a different color for each geographic feature.  The

color in which a geographic feature will be plotted is displayed to

the left of the feature name.  As you move through the geographic

feature list box, a second list box appears, allowing you to select

the color of each geographic feature.

VMAP.DOC version 1.50                                    Page  24

The map files supplied with VERSAMAP (file extension .MAP) are an

enhanced version of MWDB-II and can be used to plot the different

features.  Please note that the original MWDB-II files (file

extensions .ALL, .CCI, .PNT) do not contain data for Canadian

Provinces or Australian States.

VERSAMAP has the ability to plot as many as 9 map files, one after

the other, on a map.  These 9 map files represent different layers

or features on your map.  Features selected by accessing the

Geographic Features option in the Map Design menu only affect

features plotted for the first map file (which will be the default

map input file, VMAP4.MAP, until you enter a new filename in the Map

Input Filenames option of the Input/Output menu).  When you enter a

MWDB-II filename in the Map Input Filenames option, VERSAMAP

automatically prompts you for the geographic features that you wish

to plot for that particular file.

In addition to MWDB-II, VERSAMAP allows geographic features to be

plotted from 2 other digital map databases: (1) World Data Bank II

(WDB-II), a digital map database covering the entire world, which

contains 36 different geographic categories; and (2) U.S. Geological

Survey Digital Line Graph (USGS DLG) data, a database covering the

United States, containing 72 different geographic features.  These

are the most detailed map databases that can be plotted with

VERSAMAP; they are described in Sections 7.1 and 7.4.  Geographic

features for WDB-II and USGS DLG map files are selected when you

enter their filenames in the Map Input Filenames option of the

Input/Output menu.

4.6.      Add Text to Map

You may chose the size, color, orientation, and content of text to

be entered on the map.  When you select this option, a window

appears containing 6 boxes labeled Text #, Size, Color, Angle, PIC

Font, and Text.  The use of each box is described below.

Text #:

VERSAMAP allows you to store 100 text strings (and their associated

size, color, angle, PIC font, and position) in memory to be recalled

later.  The Text # box allows you to select the number of the text

string to be entered.

Size:

This box allows you to select the point size of the text that will

be written on the map.  The fixed-space Courier font has 3 possible

sizes: 8, 10, 12 point.  The proportionally spaced Helvetica font

has 6 sizes: 8, 10, 12, 14, 18, and 24 point.  The height of the

font in inches is equal to the point size divided by 72.  A 24 point

font has letters 1/3 inch high.

VMAP.DOC version 1.50                                    Page  25

Color:

You may print text in any 1 of 16 colors.

Angle:

You may print text in 1 of 4 orientations.  Selecting 0  prints the

text horizontally from left to right; selecting 90  prints the text

vertically with the first letter at the bottom; selecting 180

prints the text horizontally, upside down, from right to left;

selecting 270  prints the text vertically with the first letter at

the top.

PIC Font:

The PIC Font number has no effect on maps which are not exported as

PIC files.

The text that you enter on the map will be incorporated in any Lotus

PIC graphics file that you export.  Text in PIC files can be printed

in 1 or 2 fonts.  These Lotus fonts are not the same as the Courier

and Helvetica fonts supplied with VERSAMAP.

The PIC Font box allows you to select the font number, 1 or 2, with

which you want the text string to be printed when you print your PIC

file.  For instance, the PrintGraph utility from Lotus 1-2-3 version

2.2 allows you to chose 2 fonts from a list of 11 possible fonts

(block1, block2, bold, forum, italic1, italic2, Lotus, Roman1,

Roman2, script1, script2).

If you plan to export maps as PIC files, I recommend that you

prepare a test map annotated with both the Courier and Helvetica

fonts in all possible point sizes.  Export this test map from

VERSAMAP as a PIC file, import the file into your graphics program,

and print the PIC file from within your graphics program.  This will

give you an idea of the appearance of each font in its various

sizes.

Text:

Type the text that you want printed on the map in the Text box, and

press Enter to store the string.  You must press Enter to save the

text string!

Text strings can have a maximum length of 56 characters.  With 12

point or larger text, you may not be able to fit 56 characters

across the map.

VMAP.DOC version 1.50                                    Page  26

The following table lists the keystrokes necessary to print a degree

sign,  , in applications running under DOS or Windows.  To print a

degree sign for all output options except Save as DXF, hold down the

Alt key and type the number 176 or 248 on the numeric keypad.

Output Option         DOS          Windows

Screen             Alt-248        Alt-248

Print Map          Alt-248        Alt-248

Save as PCX        Alt-248        Alt-248

Save as CGM        Alt-248        Alt-176

Save as PIC        Alt-176        Alt-176

Save as DXF          %%d            %%d

The keystrokes Alt-176 will be displayed as a stippled block,  , and

an underline character, _, in VERSAMAP, but will print a degree sign

when the output file is printed.

To incorporate a degree sign in a DXF file, type the 3 character

sequence, %%d.  These 3 characters will be displayed as a degree

sign when the DXF file is imported into AutoCAD.  If %%d does not

display a degree sign in your application, try Alt-176 or Alt-248.

4.6.1.    Positioning text on the map

After selecting the text parameters, tab to the < ok > button, and

press Enter.  The window will disappear, and the text string

previously entered in the Text box will be printed in the upper left

corner of the map.  The mouse or the cursor keys are used to place

the text string in the desired position on the map.  The horizontal

and vertical pixel position of the cursor is continuously updated in

the upper right corner of the screen.

The menu bar is erased when you press the < ok > button.  It is

replaced by a reminder that pressing Esc will restore the menus.  To

access the menus again you must press Esc or click the left mouse

button on the top line of the screen.  This will terminate text

addition, and reactivate the menus.

If you have a mouse, move the mouse cursor to the position at which

you want the text to appear, then press the left mouse button.  The

mouse cursor represents the lower, left corner of the text string.

The text string will appear in the new position.  Pressing the left

mouse button toggles the text string on and off.  The program will

beep if the position you have chosen is too near the edge of the map

to print the full text string.

VMAP.DOC version 1.50                                    Page  27

You can use the up, down, right, and left cursor keys to position

the text string if you do not have a mouse.  The cursor position is

represented by a plus sign, +.  The plus sign is not visible until

you press one of the cursor keys.  The distance the cursor moves is

controlled by pressing the numbers 3, 2, or 1.  Pressing the number

3, causes the cursor to move 50 pixels each time a cursor key is

pressed; pressing the number 2, causes the cursor to move 10 pixels

each time a cursor key is pressed; and pressing the number 1, causes

the cursor to move 1 pixel each time a cursor key is pressed.  Once

the cursor is in the correct position, press Enter.  The center of

the + represents the lower, left corner of the text string.

Pressing Enter, toggles the text string on and off.  The program

will beep if the position you have chosen is too near the edge of

the map to print the full text string.

Once you have placed the text in the correct location, press Esc (or

click the left mouse button on the top line of the screen) to

terminate text addition, and reactivate the menus.

4.6.2.    Adding additional text to the map

To enter a second text string on the map, press Esc to activate the

menus and select the Add Text to Map option in the Map Design menu.

Place the highlight bar over the number 2 in the Text # box, and

press Enter.  Then proceed to modify the text parameters as

necessary.  The second text string will print in the upper left

corner of the screen when the < OK > button is pressed; this string

will overprint any text already present in the upper left corner.

However, the original text will be restored when you move the second

text string to a new location on the screen.

Text that you enter on the map will be incorporated into maps that

you print or save using the Print Map or Save as PCX, CGM, DXF, or

PIC options in the Input/Output menu.  Text entered on maps is not

incorporated into maps saved with the Save as ASCII option, since

most technical and business graphing programs have the ability to

annotate imported graphs.

4.7.      Select Font

This option allows you to select the font used to print text on the

screen.  You may select a fixed-space Courier font or a

proportionally spaced Helvetica font.

The fonts used by VERSAMAP are stored in 4 files with the file

extension .FON:

COUR_EGA.FON, COUR_VGA.FON

HELV_EGA.FON, HELV_VGA.FON

VMAP.DOC version 1.50                                    Page  28

The program will select the correct font for your video adapter.

Fonts labeled _EGA are for EGA adapters, and those labeled _VGA for

VGA adapters.  After making backup copies, you may delete the

unnecessary font files to save disk space.  The Helvetica fonts are

only included in the registered version of VERSAMAP.

The font files must be in the same directory as the VERSAMAP

program, VMAP.EXE.

After selecting this option a window titled SELECT FONT appears.

The default font is fixed-space Courier.  If this is acceptable,

press Enter.  To select the Helvetica font, highlight Helvetica with

the down arrow key, and press Enter.

You may add text in both fonts to the same map.  However, using both

fonts on the same map is not recommended.  The length of a text

string depends on the font and point size; text which fits on a map

in one font and point size may not fit on the map in the second

font.  Use only one font per map if you plan to export a map as a

CGM, DXF, or PIC vector graphics file.  The vector graphics files

exported by VERSAMAP allow only 1 font to be specified, that font

will be the last font selected before exporting the file.

4.8.      Display Previous Text

This option allows you to redisplay, on the screen, all previously

defined text strings.  Once text is redisplayed it can only be

erased by redrawing the map.

4.9.      Labels

This option allows you to import text that will be displayed next to

points plotted by VERSAMAP.  The PRN and UTM ASCII file formats

supported by VERSAMAP allow you to include a text string that can be

printed next to the latitude, longitude or x,y coordinates of the

point.  The file structure of the PRN and UTM files is discussed in

Section 7.7, Importing Your Own Data into VERSAMAP.

You may select one of the following items in the Labels option,

Do Not Display Labels

Display Labels

Convert Labels to Text

Erase Labels Converted to Text

The default setting, Do Not Display Labels, ignores labels contained

in a PRN or UTM file.

VMAP.DOC version 1.50                                    Page  29

Selecting Display Labels causes VERSAMAP to display labels contained

in a PRN or UTM file when the next map is drawn.  Labels are

displayed to the lower right of their associated point.  Labels

which would extend off the right side of the screen are displayed to

the lower left of the point.  Labels displayed by selecting Display

Labels are incorporated in maps saved with the Print Map and Save as

PCX options of the Input/Output menu; they are not incorporated into

maps saved as vector graphics files with the Save as CGM, DXF, PIC,

or ASCII options.

The Convert Labels to Text item converts labels displayed on the

screen into text objects that can be edited from within VERSAMAP.

When you select this option, labels which were previously imported

into VERSAMAP using the Display Labels item are converted into text

strings that can be edited using the Add Text to Map option.  Labels

which have been converted to text are added to the end of the list

of text strings in the Add Text to Map option.  Labels converted to

text can be edited and repositioned using the Add Text to Map

option.  In addition, labels which have been converted to text are

included in maps saved as CGM, DXF, and PIC vector graphics files.

To convert labels to text, perform the following steps,

1.  Select Display Labels

2.  Replot your map to display the labels

3.  Select Convert Labels to Text

If you now access the Add Text to Map option in the Map Design menu,

and page down through the list of text strings, you will see that

the labels have been converted to text strings and occur at the end

of the list.

The last menu item, Erase Labels Converted to Text, deletes any

labels, that you have previously converted to text, from the text

strings in the Add Text to Map option.

A detailed example showing how to display labels is given in Section

7.7, Importing Your Own Data into VERSAMAP.

4.10.     Background Color, Grid Color

These options allow you to change the background or grid color of

the map, to any 1 of 16 colors.  The new color that you select will

be displayed when the next map is plotted.  If you plan to export

maps as PCX or CGM files, and to print them as black on white,

change the background color to white and the grid color to black.

VMAP.DOC version 1.50                                    Page  30

4.11.     Save Map Design

This option allows you to save the map parameters you have selected

in a configuration file for recall at a later time.  After selecting

this item, a window titled ENTER FILENAME FOR CONFIGURATION FILE

appears.  Type in the filename and press Enter to select the

filename.  Press Enter on the < ok > button to save the

configuration file to disk.  Your configuration file will be written

to disk immediately after you press the < ok > button.

4.12.     Recall Map Design

This option allows you to recall a previously defined configuration

file.  After selecting this item, a window titled RECALL MAP DESIGN

appears.  This window contains a vertical box, listing all

configuration files (file extension .CNF) in the current directory.

Move to the desired file and press Enter to load the file into

VERSAMAP.  The configuration file will be loaded immediately after

you press Enter.

VERSAMAP does not automatically display a new map when a

configuration file is loaded into the program.  To display the new

map, you must manually plot the map by selecting the checkmarked

option in the Run menu.  If the new map includes text, you must also

redisplay the text using the Display Previous Text option in the Map

Design menu, once you have redrawn the map.

VMAP.DOC version 1.50                                    Page  31

5.        Input/Output menu

-----------------------------------------------------------------

Projection      Map Design      Input/Output      Run

-----------------------------------------------------------------

| Map Input Filenames   |

| Map Parameter Report  |

| Great Circle Report   |

| View Disk Directory   |

|-----------------------|

| Print Map             |

| Select Printer        |

|-----------------------|

| Save as PCX           |

| Save as PIC           |

| Save as CGM           |

| Save as DXF           |

| Save as ASCII         |

| Save Great Circle     |

+-----------------------+

This menu allows you to enter filenames for input and output files;

to display a report describing the currently selected map; to

display a report listing course and distance at waypoints along a

Great Circle; to view the contents of any disk directory; to print

the map on dot matrix and laser printers; and to save maps as disk

files.

You may export maps produced by VERSAMAP: (1) by copying the screen

directly to your printer; (2) as bit-mapped PCX graphics files; (3)

as vector graphics CGM, DXF, or Lotus PIC files; and (4) as vector

based ASCII files.

Maps exported by options 1 and 2 are limited by the resolution of

your monitor.  Even with a VGA monitor, the screen resolution (640

pixels horizontally, 480 pixels vertically) is fairly coarse.  Maps

exported as CGM, DXF, or PIC files (option 3) are independent of

monitor type and have much greater resolution (3200 dots

horizontally, 2311 dots vertically) than PCX files.  If you have a

choice, export your maps as CGM, DXF, or PIC files.  This is

particularly true if you have an EGA monitor.  The most detailed

maps produced by VERSAMAP are those produced by saving them as ASCII

files (option 4).  The resolution of these vector based ASCII files

is limited only by the resolution of the input map boundary files.

However, the use of ASCII files produced by option 4 requires a

technical graphing program that can import large data sets.

VMAP.DOC version 1.50                                    Page  32

5.1.      Map Input Filenames

This option allows you to select the digital map file(s) that you

wish to plot, and to define the symbols and colors used to plot

them.  After selecting this item, a window appears in which the

names of the digital map files you wish to plot are entered.  Up to

9 filenames may be entered.  You can think of these 9 input files as

representing different layers or features on your map.

The default file name for the first input file is VMAP4.MAP in the

current directory.  VMAP4.MAP is the world map included on the

VERSAMAP disk.  The default file names of the remaining 8 map input

files are null strings (i.e. no file name).  The program displays

the current path, but no filename, for input files which have not

been defined.

When the window is first displayed, the cursor is located at the end

of the first input file name.  Type in the correct path name and

press Enter to accept the path name.

Symbol Type and Size:

Once you enter a filename, a second window will appear which allows

you to specify the symbol used to plot the points in that file.

The symbol type determines whether your data is plotted as lines,

points, crosses, squares, diamonds, triangles, or circles.  If you

select the symbol type Line, all points on a particular polyline

will be connected by straight lines.  This is the default symbol

type for all files.

The other symbol types (Point, Cross, Square, Diamond, Triangle,

Circle) allow you to superimpose individual latitude/longitude

points on a map created from a digital map database.

In addition, you may chose the size of the symbols used to plot your

data.  The size that you enter is the radius of the symbol in

pixels.  The maximum size is 99 pixels.  Size has no effect on the

Line and Point symbol types, these two types are always 1 pixel

wide.

NOTE:     File types (DAT, MP1, MPS, PNT, PRN, UTM) without multiple

geographic features allow you to select the color with

which to plot the file from this window.

Once you have entered the symbol type, size, and color (if any) for

the first input file, tab to the < ok > button and press Enter.

VMAP.DOC version 1.50                                    Page  33

Label Size and Color:

ASCII files with file extensions PRN or UTM may contain text labels.

For these 2 file types, a window titled SELECT LABEL SIZE & COLOR

allows you to select the point size and color of labels you wish

printed on the map.

Geographic Features:

For file types (ALL, CCI, MAP, PKV, PKD, GRV, GRF) which contain

multiple geographic features, the program will prompt you to enter

the geographic features to be plotted.

You may select a different color for each geographic feature.  The

color in which a geographic feature will be plotted is displayed to

the left of the feature name.  As you move through the geographic

feature list box, a second list box appears, allowing you to select

the color of each geographic feature.

Once you select Proceed on the Geographic Features list box, the

window disappears, and you are prompted for the name of the second

map input file (if any).  If you wish to input a second map boundary

file, type its name and press Enter.  Continue until you have

entered all of your map input files.

Once you have entered the last map input file that you wish plotted,

tab to the < ok > button in the ENTER FILENAMES FOR MAP INPUT FILES

window, and press Enter.

The program does not require any input map boundary files.  If you

do not wish to input data, erase the default filename (up to the

rightmost backslash) of the first input file; press Enter; then tab

to the < ok > button; and press Enter.  In this case, only the map

grid (if any) will be plotted on the screen.

VERSAMAP will accept map boundary input files from the following

digital map databases: World Data Bank I, World Data Bank II, Micro

World Data Bank II, The World Digitized, and U.S. Geological Survey

Digital Line Graph Data.  In addition, VERSAMAP will accept input in

an ASCII text format.  Section 7 contains a detailed discussion of

the various map databases that VERSAMAP recognizes.

VERSAMAP recognizes the various databases by their file name

extensions.  The file names that you enter on this screen must have

one of the extensions given in Section 7.  However, the names that

you enter can have different extensions.  For instance, your first

file could be from Micro World Data Bank II with the extension .ALL,

your second file could be from the U.S. Geological Survey Digital

Line Graph database with the extension .GRF, and your third file

could be from World Data Bank II, with the extension, .PKV.

VMAP.DOC version 1.50                                    Page  34

5.2.      Map Parameter Report

The Versamap parameter report lists various information about the

map: the projection type; the grid type; the latitude/longitude

limits of the map; the maximum/minimum values of the transformed x,y

map coordinates; scale and area errors at selected points on the

map; and the names of the map input files.

The X-axis Length and Y-axis Length, listed under Relative Length of

Axes, are used to scale maps exported as ASCII file(s) to technical

and business graphics programs.  The use of these two parameters is

described in Section 8.

Use the up, down, Home, and End keys to scroll through the report.

Press F1 to print the report.  Press Esc to exit the report and

return to menu bar.

5.3.      Great Circle Report

Selecting this option displays a report listing the course and

distance at selected waypoints along a Great Circle previously

defined in the Great Circle option of the Map Design menu.

The column labeled Course gives the initial course in degrees from

the current waypoint to the next waypoint.  A course of 0  or 360

represents due north; 180  represents due south.  The last 2

columns, list the cumulative distance in nautical miles (nm) or

kilometers (km) from the initial point of the Great Circle to the

given waypoint.

Use the up, down, Home, and End keys to scroll through the report.

Press F1 to print the report.  Press Esc to exit the report and

return to menu bar.

5.4.      View Disk Directory

This window allows you to view the contents of any disk and

directory.  Disk drives are indicated by the letters <drive> to the

right of the drive letter.  To select a disk drive, move the

highlight bar over the drive letter and press Enter.  Directories

are indicated by the letters <dir> to the right of the directory

name.  To select a directory, move the highlight bar over the

directory and press Enter.  The path which has been selected is

displayed immediately above the vertical list box.

The files in the directory are listed after the last directory

listing.

VMAP.DOC version 1.50                                    Page  35

5.5.      Print Map

Maps displayed on the screen may be printed on dot matrix or laser

printers.  When you select this item, a window titled MAP PLACEMENT

ON PAGE IN INCHES appears, which allows you to define the dimensions

and page placement of the printed map.

This option copies the entire screen, except the menu bar, to the

printer.  The default settings place the map in the center of the

page, and set the width (the entire screen width) at 6 inches.  The

width and height of the printed map are dependent.  Adjusting the

width automatically fixes the height.  Any text displayed on the map

will be printed.  The grid spacing, and other information about the

printed map that you need to annotate it, can be obtained from the

Map Parameter Report option in the Input/Output menu.

Use the Tab key to move between items.  To print the map, tab to the

< ok > button and press Enter.  The map will begin printing

immediately after you press the < ok > button.  The Print Map option

is slow!  On an Epson LQ-800, 24-pin, dot matrix printer, the

default map takes 1.2 minutes to print from a 33 MHz 486 computer.

Press Esc to exit a map that is currently being printed.

5.6.      Select Printer

Before you print a map, you must select the type of printer which

will print the map.  When you select this item, a window titled

SELECT PRINTER TYPE AND PORT appears.  You may select one of three

printer types: Epson 9- or 24-pin dot matrix, or Hewlett-Packard

LaserJet II.  If your printer is not compatible with one of these

printers, select None.

You may also select the printer port.  Lpt1 is the default.

5.7.      Save as PCX

Before saving the map as a bit-mapped PCX paint file, you must

specify the filename and path of the file.  After selecting this

item, a window titled ENTER .PCX FILENAME appears.  Type in the

correct filename and press Enter to select the filename, then tab to

the < ok > button and press Enter.  Since the PCX file will be

written to disk immediately after you press Enter on the < ok >

button, the map you wish to save must be displayed on the screen.

The menu bar is not saved as part of the file.

Maps saved as PCX files are exported exactly as they are displayed

on the VERSAMAP screen.  If you plan to edit your map in an another

program, and then to print it as black on white, you can use the

Background Color and Grid Color options in the Map Design menu to

plot your map as black on white before exporting it from VERSAMAP.

VMAP.DOC version 1.50                                    Page  36

Any text displayed on the screen (except the menu bar) will be

included in the PCX file.  If you do not want text to be included in

the PCX file, redraw the map.

The grid spacing, latitude/longitude limits, and other information

that you may need to annotate the map, can be obtained from the Map

Parameter Report option in the Input/Output menu.

5.8.      Save as PIC, Save as CGM, Save as DXF

Before saving a map as a CGM, DXF, or PIC vector graphics file, you

must specify the filename and path of the file.  After selecting one

of these items, a window titled ENTER FILENAME appears.  Type in the

correct filename and press Enter to select the filename, then tab to

the < ok > button and press Enter.  Once you have specified the

filename, redisplay the map using the Size 100% or Size 70% options

in the Run menu.  The file will be written to disk as the map is

drawn.  The menu bar is not saved as part of the file.

NOTE:     Since the CGM, DXF, or PIC file is written to disk as the

map is being plotted on the screen, you must enter the

filename, and then immediately redraw the map with the

Size 100% or Size 70% option.

Any text strings which have been defined with the Add Text to Map

option in the Map Design menu will be included in the CGM, DXF, or

PIC file, even if the text strings are not displayed on the screen.

If you do not want text to be included in the exported file, erase

any previously defined text strings.

To include text in a CGM, DXF, or PIC file, perform the following

steps:

1.  draw the map on the screen,

2.  annotate the map using the Add Text to Map option,

3.  select a name for the file using the Save as PIC, Save as CGM,

or Save as DXF option, and

4.  redraw the map using the same size option (Size 100% or Size

70%) that was used in step 1.

The file will be written to disk as the map is drawn on the screen

in step 4.  Text added to the map in step 2 is not redisplayed on

the screen when the map is redrawn in step 4; however, it is

included in the CGM, DXF, or PIC file.  Select the Display Previous

Text option in the Map Design menu to redisplay text previously

added to a map.

The grid spacing, latitude/longitude limits, and other information

that you may need to annotate the map, can be obtained from the Map

Parameter Report option in the Input/Output menu.

VMAP.DOC version 1.50                                    Page  37

Maps printed on black and white printers may be difficult to

interpret because of the many geographic features shown on the map.

For instance, due to the many lines on a map, it may be difficult to

distinguish roads from railroads, or political boundaries from

rivers.  This difficulty can be alleviated by using VERSAMAP to

export each geographic feature as a separate CGM, DXF, or PIC file;

and then importing the CGM, DXF, or PIC files into a graphics

program, such as CorelDRAW!, which has the ability to import vector

graphics files as layers of a drawing.  The line width and type

(solid, dashed, dotted) of each geographic feature (layer) can then

be modified to distinguished it from other features on the map.

NOTE:     Due to their ASCII format, DXF files produced by VERSAMAP

are exceptionally large!  The default Equirectangular map

occupies 797,293 bytes of disk space when exported in DXF

format.

5.9.      Save as ASCII

This option allows your map to be exported to technical or business

graphing programs for the production of publication quality maps

(see Section 8, Use of VERSAMAP with Graphing Programs).

After selecting this option, a window titled ENTER FILENAMES FOR

ASCII OUTPUT FILES appears.  The input fields in this window allow

you to name the various grid and map files to be exported.  You may

choose any path names for these files.

To enter a filename, tab to the name to be edited, type in the

filename and press Enter.  The cursor will then move to the next

filename (if there is one), or to the < ok > button.

The window contains lines for 1 or 2 grid filenames, depending on

the grid type selected in the Grid Type option of the Map Design

menu.  In most cases, the window will contain space for a single

grid file.  The only exception, the OUTSIDE BOUNDARY CONNECTED BY

LINES, INTERIOR GRID POINTS SHOWN grid type (the default), contains

space for two files: the first file contains the interior points,

the second the outside boundary.

The window contains lines for up to 9 map filenames, one for each of

the map input files selected in the Map Input Filenames option of

the Input/Output menu.  Each map output file corresponds to the

corresponding map input file.  For instance, if you have 5 map input

files, the fifth map output file will contain the transformed

latitude/longitude coordinates from the fifth map input file.

Data records in the ASCII grid and map output files have the

following format.

x coordinate | space (ASCII 32) | y coordinate |

carriage return (ASCII 13) | line feed (ASCII 10)

VMAP.DOC version 1.50                                    Page  38

Data in most digital map databases is organized as polylines; each

polyline corresponds to a series of points which can be connected by

straight lines.  In the ASCII output files, the beginning of each

new polyline is indicated by a dummy point in which the x and y

coordinates are both equal to 5.000000 (5 times the maximum x

coordinate of 1.000000).  In an ASCII output file, any point with an

x coordinate of 5.000000 is a dummy point which indicates the

beginning of a new polyline, and which will plot outside the

boundaries of the map.

To output your map as a series of ASCII files, perform the following

steps:

1.  draw the map on the screen using one of the Size options in the

Run menu;

2.  select the Save as ASCII option in the Input/Output menu;

3.  enter a name for each of the grid files listed under Grid

filename;

4.  enter a name for each of the map files listed under Map

filename;

5.  tab to the < ok > button and press Enter;

6.  redraw the map using the same size option (Size 100% or Size

70%) that was used in step 1.

The file(s) will be written to disk as the map is drawn on the

screen in step 6.  The Save as ASCII option does not export text

defined with the Add Text to Map option.  Text displayed on the

screen is ignored when the map data is written to disk.

5.9.1.    Converting exported ASCII x,y coordinates back to

latitude, longitude in degrees

Many users have asked how to convert ASCII x,y coordinates exported

from VERSAMAP back into latitude, longitude values for use in other

applications.  This can be done by using the Equirectangular map

projection to export the data.  The x-axis of the Equirectangular

projection is directly proportional to longitude; the y-axis is

directly proportional to latitude.

For an Equirectangular projection, the longitude in degrees

corresponding a given x coordinate is,

longitude =  (longitude range of map)*(x coordinate) +

(minimum longitude of map)

If the calculated longitude is greater than 180 degrees, subtract

360 degrees from the calculated value.

VMAP.DOC version 1.50                                    Page  39

The latitude in degrees corresponding a given y coordinate is,

latitude =  (latitude range of map)*(y coordinate)/(relative

length of y-axis) + (minimum latitude of map)

The relative length of the y-axis used in the above equation is

given in the Map Parameter Report option of the Input/Output menu,

under Relative Length of Axes.  The relative length of the x-axis is

always 1.000000, corresponding to the maximum possible value of the

x coordinate.  The relative length of the y-axis may be greater than

or less than unity, depending on the shape of the map.  In your

calculations, ignore any points with x > 1; these points represent

polyline breaks.

5.10.     Save Great Circle

This option allows the currently defined Great Circle to be written

to disk for later recall.  The Great Circle is stored in the ASCII

PRN file format that can be read by VERSAMAP (see Section 7.7,

Importing Your Own Data into VERSAMAP).  The purpose of saving Great

Circles to disk is to allow more than one Great Circle to be plotted

on a map.  To plot more than one Great Circle on a map, export the

Great Circles as ASCII PRN files, and then import them back into

VERSAMAP using the Map Input Filenames option of the Input/Output

menu.

After selecting this option, a window titled ENTER GREAT CIRCLE

FILENAME appears.  Type in the correct filename and press Enter to

select the filename.  The cursor will then jump to the < ok >

button.  Press Enter on the < ok > button to write the file to disk.

The ASCII PRN file contains the same latitude/longitude waypoints

listed in the Great Circle Report option of the Input/Output menu.

Please note that before saving a Great Circle to disk, you must

define its endpoints with the Great Circle option of the Map Design

menu.

VMAP.DOC version 1.50                                    Page  40

6.        Run menu

-----------------------------------------------------------------

Projection      Map Design      Input/Output      Run

-----------------------------------------------------------------

| Size 100% |

| Size  70% |

+-----------+

This menu contains 2 items.  Selecting Size 100% will plot the

largest possible map on the screen.  Selecting Size 70% will plot a

smaller map with margins around all sides.  The Size 70% option

allows you to add text in the margins of the map without writing

over the map itself.

Your map will begin plotting immediately after either item is

selected.  The program will cease plotting and return to the menu

bar if Esc is pressed while the map is being drawn.

VMAP.DOC version 1.50                                    Page  41

7. Sources of Digital Map Data for use with VERSAMAP

VERSAMAP will generate maps from the following databases, available

commercially, as shareware, or in the public domain.  In addition,

the program will accept input in a simple ASCII format.  VERSAMAP

recognizes the various databases by their file extensions.  The most

complete databases are (1) the Central Intelligence Agency's World

Data Bank II (WDB-II), a detailed world map; (2) Micro World Data

Bank II (MWDB-II), a smaller database derived from World Data Bank

II; and (3) the U.S. Geological Survey's Digital Line Graph Data

(USGS DLG), which contains detailed maps of all 50 states.

Which digital map database you use depends on the size of the area

you wish to map.  For world maps, MWDB-II is the best choice.

Although WDB-II can be used to draw world maps, it contains so many

points that it plots very slowly when used to plot such a large

area.  WDB-II is best used for maps of continental size or smaller.

The USGS DLG data is the best digital map database for plotting maps

at the state or county level in the United States.

7.1.      World Data Bank II (WDB-II)

File extensions PKV, PKD

A database, developed by the Central Intelligence Agency, covering

the entire world.  Latitude and longitude are given to the nearest

second.  The original database is divided into 13 files containing

data for Asia, Europe-Africa, North America, and South

America-Antarctica.  The complete set of 13 files contains 6

million, 20 character ASCII records, which require 120 million bytes

of disk space.  WDB-II was last updated in January 1987.

WDB-II was digitized using all available source information.  Maps

used ranged in scale from 1:750,000 to 1:4,000,000.  Nominal scale

was 1:3,000,000.  At 1:3,000,000 scale, 1 inch equals 47 statute

miles (1 cm equals 30 km).  Successive points in the  database are

never more than 127 seconds (approximately 2 miles or 3 km) of

latitude or longitude apart.

To make WDB-II easier to use on microcomputers, Micro Doc (3108

Jackson Street, Bellevue, NE  68005) has reformatted WDB-II in a

binary format (.PKD file extension).  The binary format reduces

storage requirements by a factor of 10 (to 12 million bytes), and

vastly reduces the time required to draw maps.  VERSAMAP can plot

maps using the binary form of WDB-II (.PKD file extension), and from

an enhanced version of Micro Doc's .PKD files (.PKV file extension),

developed specifically for VERSAMAP.

VERSAMAP can plot the following geographic features from World Data

Bank II in the .PKV or .PKD file formats.

VMAP.DOC version 1.50                                    Page  42

Geographic Features in World Data Bank II

Rank       Feature Name

PKV   PKD

Major Category: International Boundaries or Limits of

Sovereignty

1     1     International Boundaries

2     2     Indefinite or in dispute

3     3     Separation line, land

4     4     Separation line, sea

5     5     Separation line, islands

6     6     Continental Shelf, Persian Gulf

7     7     Demilitarized zone, Israel

8     8     No defined boundary

Major Category: Coastlines, Islands, and Lakes

9     1     Coastlines, Islands, Lakes

10     2     Islands and lakes, large

11     3     Islands and lakes, medium

12     4     Islands and lakes, small

14     6     Intermittent lakes, large

15     7     Intermittent lakes, small

16     8     Reefs

17     9     Salt pans, large

18    10     Salt pans, small

21    13     Ice shelves, large

22    14     Ice shelves, small

23    15     Glaciers

Major Category: Rivers

24     1     Rivers

25     2     Rivers, large

26     3     Rivers, medium

27     4     Rivers, small

28     5     Rivers, double lined

29     6     Intermittent rivers, large

30     7     Intermittent rivers, medium

31     8     Intermittent rivers, small

33    10     Canals

34    11     Canals, small

36    13     Canals, irrigation

Major Category: Internal Political Boundaries

37     1     U.S. States

38     1     Canadian Provinces

39     3     Boundaries in Great Lakes

40           Australian States  (not available in)

41           Scotland and Wales (     .PKD format)

VMAP.DOC version 1.50                                    Page  43

The WDB-II .PKV files, developed specifically for VERSAMAP, differ

from Micro Doc's .PKD files in 2 respects.

First, .PKV files contain political boundaries for the new countries

which have formed from the Soviet Union and from Yugoslavia, for

Slovakia and Eritrea, for Australian States, and for Scotland and

Wales.  These borders are not included in Micro Doc's .PKD files,

nor in the original ASCII version of WDB-II.

Second, inspection of the above table shows that the ranks, of

individual geographic features in Micro Doc's .PKD files, are not

unique to a single major category (International Boundaries,

Coastlines, Rivers, or Internal Boundaries).  The duplication of

ranks between major geographic categories means that .PKD files

cannot be concatenated into a single large file without losing

information about which category a particular rank corresponds to.

To alleviate this problem, ranks in the specially modified .PKV

files have been redefined so that they are unique for each

geographical feature (see above table).  This modification allows

.PKV files to be concatenated into a single large file for each

continent.

For use with VERSAMAP, the modified .PKV files were concatenated

into 5 files, each file containing all geographic features for a

particular region.  The 5 regions are (1) Africa, (2) Eurasia

(Europe and Asia), (3) Oceania (Indonesia, Australia, New Zealand,

and the Pacific Islands), (4) Latin America (including Antarctica),

and (5) North America (Canada and the United States).  The .PKV

files for individual regions or for the entire world are available

on high density, floppy disks from the author.  See Section 11, the

VERSAMAP Order Form, for details.

As an example of the detail available in WDB-II, the file,

VMTAIWAN.PKV, which contains all data from WDB-II for the island of

Taiwan, is included on the VERSAMAP disk.  To plot the map of

Taiwan,

1.  Set the map boundaries at 26 N to 21 N latitude, and 119 E to

123 E longitude using the Map Boundaries option in the Map

Design menu (use a grid spacing of 1 degree, i.e. 001 00 00).

2.  Import the file VMTAIWAN.PKV into VERSAMAP using the Map Input

Filenames option in the Input/Output menu.  Toggle all

geographic features on in the SELECT WORLD DATA BANK II

FEATURES list box.

3.  Plot the map using the Size 100% option in the Run menu.

VMAP.DOC version 1.50                                    Page  44

When VERSAMAP plots maps using Micro Doc's packed .PKD files, it

determines the major category of data being plotted from the third

letter of the filename of the input file.  In Micro Doc's file

naming scheme, this letter is unique for each of the 4 major

geographic categories.  The third letter must be B for .PKD files

containing International Boundaries or Limits of Sovereignty; it

must be C for Coastlines, Islands, and Lakes; it must be R for

Rivers; and it must be P for Internal Political Boundaries.  This

restriction on filenames is not necessary when you use .PKV files,

since geographic ranks in .PKV files are unique.

VMAP.DOC version 1.50                                    Page  45

7.2.      Micro World Data Bank II (MWDB-II)

File extensions PNT, ALL, CCI

A database, covering the entire world, containing 178,000 latitude,

longitude points.  Latitude and longitude are rounded to the nearest

minute and stored as six byte binary records.  This database was

derived from WDB-II by Micro Doc (3108 Jackson Street, Bellevue, NE

68005).  An enhanced version of MWDB-II, containing boundaries for

the new countries which have formed from the Soviet Union and from

Yugoslavia, for Slovakia and Eritrea, for Canadian Provinces, and

for Australian States is available from the author.  See Section

7.3, VERSAMAP Micro World Database, for details.

VERSAMAP can plot the following geographic features from MWDB-II:

coastlines, islands, lakes, rivers, countries, and U.S. States.

MWDB-II contains several types of digital map files.  The first type

(file extension .PNT) contains points from a single geographic

feature at 5 levels of detail.  The second file type (file extension

.ALL) contains points for all geographic features at a single level

of detail.  A third MWDB-II file, MWDB5.CCI, contains only country,

coastline, and island points at the lowest level of detail.  The

.ALL and .CCI files are made from the .PNT files using the utility

program, SELECT.EXE, supplied with MWDB-II.  SELECT.EXE allows the

user to build customized map files containing any number of

geographic features and levels of detail.  To save disk space,

MWDB1.ALL and MWDB2.ALL, the 2 largest files, are not included in

the MWDB-II package.  They are constructed by the user from the .PNT

files and SELECT.EXE upon receiving MWDB-II.

Summary of Micro World Data Bank II Files

Filename    Size(bytes)   # Points    Comment

MWDB1.ALL    1,068,408     178,068    The entire database

MWDB2.ALL      652,500     108,750

MWDB3.ALL      157,176      26,196

MWDB4.ALL       82,560      13,760

MWDB5.ALL       27,180       4,530

MWDB5.CCI       19,110       3,185

COAST.PNT      449,802      74,967    Coastlines

ISLAND.PNT      211,026      35,171    Islands

LAKE.PNT       90,708      15,118    Lakes

RIVER.PNT      169,164      28,194    Rivers

COUNTRY.PNT      134,154      22,359    Country borders

STATE.PNT       13,554       2,259    U.S. State borders

VMAP.DOC version 1.50                                    Page  46

7.3.      VERSAMAP Micro World Database

File extension MAP

A database containing 196,000 latitude, longitude points.  Latitude

and longitude are rounded to the nearest minute and stored as six

byte binary records.  This database is an enhanced version of

MWDB-II described in Section 7.2.  Both map databases have the same

file structure.  Enhancements incorporated in the VERSAMAP Micro

World Database include (1) borders for the new countries which have

formed from the Soviet Union; (2) borders for Eritrea, Slovakia, and

the Yugoslav Republics, (3) borders for Canadian Provinces; (4)

borders for Australian States; (5) more detail for U.S. State

boundaries; (6) increased detail for the borders of Israel, Oman,

the United Arab Emirates, and Yemen; and (7) removal of the borders

between East and West Germany, and between the Yemen Arab Republic

and the People's Democratic Republic of Yemen.

VMAP1.MAP through VMAP5.MAP, the digital map files supplied with

VERSAMAP Micro World Database, are enhanced versions of the

equivalent .ALL files in MWDB-II.

VERSAMAP can plot the following geographic features from VERSAMAP

Micro World Database: coastlines, islands, lakes, rivers, countries,

U.S. States, Canadian Provinces, and Australian States.

VERSAMAP Micro World Database Files

Filename      Size      Number     Avg distance between points

(bytes)    Points        nautical miles    km

VMAP1.MAP   1,174,788   195,798             4            7

VMAP2.MAP     719,670   119,946             6           11

VMAP3.MAP     187,122    31,187            19           35

VMAP4.MAP      98,490    16,415            33           61

VMAP5.MAP      34,944     5,824            72          133

The complete set of VERSAMAP Micro World Database .MAP files is

available on floppy disk from the author.  See Section 11, the

VERSAMAP Order Form, for details.

7.4.      USGS Digital Line Graph Data (USGS DLG)

File extensions GRF, GRV

A database developed by the U.S. Geological Survey containing

detailed maps of all 50 states.  The database divides the United

States into 21 sections.  The digital data for each section contains

the following major categories: (1) streams and rivers, (2)

coastlines and lakes, (3) roads, (4), railroads, (5) political

boundaries, (6) administrative boundaries, (7) cultural features,

and (8) hypsography.  Each major category contains numerous minor

categories characterized by the size of the geographic feature.  The

most recent data in the database dates from 1980.

VMAP.DOC version 1.50                                    Page  47

Data in the USGS DLG database were derived by digitizing 1:2,000,000

scale maps from the National Atlas of the United States at a

resolution of 0.001 inch.  At 1:2,000,000 scale, 1 inch equals 32

statute miles (1 cm equals 20 km).  Successive points in the

database are never more than 127 seconds (approximately 2 miles or

3 km) of latitude or longitude apart.

The database is available on CD-ROM (US GeoData - 1:2,000,000-Scale

Digital Line Graph (DLG) Data) and can be ordered from the Earth

Science Information Center, U.S. Geological Survey, 507 National

Center, Reston, VA  22092.  Phone number: 1-800-USA-MAPS.

VERSAMAP can plot maps using DLG data in the graphic file format

(.GRF file extension) directly from the above CD-ROM.  These .GRF

files consist of 20 character ASCII records, in which latitude and

longitude are given to the nearest second.  The entire set of .GRF

files, covering all 50 states, occupies 144 million bytes of space

on the CD-ROM.

To reduce storage requirements, and speed plotting, the ASCII .GRF

files have been converted to the binary, packed WDB-II format

mentioned above.  The .GRF files in packed WDB-II format have the

file extension .GRV.  The packed .GRV files occupy 8 times less disk

space (18 million bytes) and plot several times faster than the

original ASCII .GRF files.  Packed .GRV files, for the 21 sections

of the United States or for the entire country, are available on

high density, floppy disks from the author.  See Section 11, the

VERSAMAP Order Form, for details.

As an example of the detail available in the USGS DLG data, the

file, VMOAHU.GRV, which contains all data from the US GeoData CD-ROM

for the island of Oahu in the Hawaiian Islands, is included on the

VERSAMAP disk.  To plot the map of Oahu,

1.  Set the map boundaries at 22 N to 21 N latitude, and 159 W to

157 W longitude using the Map Boundaries option in the Map

Design menu (use a grid spacing of 1 degree, i.e. 001 00 00).

2.  Import the file VMOAHU.GRV into VERSAMAP using the Map Input

Filenames option in the Input/Output menu.  Toggle all

geographic features on in the SELECT USGS DIGITAL LINE GRAPH

FEATURES list box.

3.  Plot the map using the Size 100% option in the Run menu.

VMAP.DOC version 1.50                                    Page  48

VERSAMAP can plot the following geographic features using DLG data

in either .GRF or .GRV file formats.

Geographic Features Included in USGS DLG Data

#       Feature Name

Rivers and Streams

1       River, double line, shoreline

2       River, double line, centerline

3       River, perennial,        <30 km

4       River, perennial,       <100 km

5       River, perennial,       <350 km

6       River, perennial,      >=350 km

7       River, intermittent,     <30 km

8       River, intermittent,    <100 km

9       River, intermittent,    <350 km

10       River, intermittent,   >=350 km

11       River in lake, perenn.   <30 km

12       River in lake, perenn.   <80 km

13       River in lake, perenn.  >=80 km

14       River in lake, interm.   <30 km

15       River in lake, interm.  >=30 km

16       Braided stream

17       Canal, navigable,        <20 km

18       Canal, navigable,        <80 km

19       Canal, navigable,       >=80 km

20       Canal, other,            <20 km

21       Canal, other,            <80 km

22       Canal, other,           >=80 km

23       Ditch

24       Intercoastal waterway

Water Bodies

25       U.S. coastline and Great Lakes

26       Lake, island, perennial  <30 km

27       Lake, island, perennial  <80 km

28       Lake, island, perennial >=80 km

29       Lake, intermittent       <30 km

30       Lake, intermittent       <80 km

31       Lake, intermittent      >=80 km

32       Marsh, swamp,            <50 km

33       Marsh, swamp,           >=50 km

34       Dry lake, alkali flat

35       Glacier,                 <50 km

36       Glacier,                >=50 km

Roads and Trails

37       Interstate highway

38       Interstate, proposed

VMAP.DOC version 1.50                                    Page  49

39       Divided highway, limited access

40       Other hwy, limited access

41       Other hwy, non-limited access

42       Other hwy <10 km from major hwy

43       State secondary, all weather

44       Light duty, all weather

45       Unimproved, fair or dry weather

46       Highway, tunnel

47       Auto, ferry

Railroads

48       Railroad, main line

49       Railroad, branch line

50       Railroad, other

51       Railroad, tunnel

52       Railroad, ferry

Political Boundaries

53       International treaty line

54       National boundary, land

55       National boundary, water

56       State boundary, land

57       State boundary, water

58       County, large city, land

59       County, large city, water

60       City boundary,     >1.0 million

61       City boundary,     >0.5 million

62       City boundary,            small

Note that USGS DLG sections 3, 10, 11, and 15

through 21 do not contain city boundaries

(features 60, 61, 62).

Administrative Boundaries

63       National park            <20 km

64       National park           >=20 km

65       National park closure line

66       Indian reservation       <20 km

67       Indian reservation      >=20 km

68       Indian reservation closure line

69       Military reservation    <405 ha

70       Military reservation   >=405 ha

71       Mil. reservation closure line

Cultural Features

72       Alaska pipeline

VMAP.DOC version 1.50                                    Page  50

7.5.      World Data Bank I

File extension DAT

An obsolete database, covering the entire world, developed by the

Central Intelligence Agency.  World Data Bank I contains 79,000

points representing world coastlines.  Latitude and longitude are

given to the nearest second.  Data are stored as 20 character ASCII

records on 5, 5.25 inch, 360K disks, which can be ordered from the

National Technical Information Service, United States Department of

Commerce, 5285 Port Royal Road, Springfield, VA  22161.

7.6.      The World Digitized

File extensions MPS, MP1

An obsolete, shareware database containing 100,000 latitude,

longitude points rounded to thousandths of a degree.  The database

is available (in both binary and ASCII format) from shareware

vendors.

The World Digitized database is subdivided into directories, one

directory for each continent.  Each of the World Digitized

directories contains 2 or more map files.  To plot the entire World

Digitized database from VERSAMAP, you will have to concatenate the

individual .MPS (binary) or .MP1 (ASCII) files, since VERSAMAP will

not accept more than 9 input files.  The VERSAMAP disks contain 2

files, VM_FIX.MPS and VM_FIX.MP1, which must be used to concatenate

World Digitized files for use with VERSAMAP.  These 2 files,

VM_FIX.MPS and VM_FIX.MP1, force a line break at the end of each

World Digitized file when it is concatenated.

For example, to concatenate the World Digitized binary .MPS files in

the Europe subdirectory for use with VERSAMAP, use the following

form of the DOS copy command:

COPY /B E0.MPS + VM_FIX.MPS + E1.MPS + VM_FIX.MPS + E2.MPS +

VM_FIX.MPS + E3.MPS  EUROPE.MPS

where EUROPE.MPS is the concatenated file.

A similar procedure (without the binary /B switch) is used to

concatenate the World Digitized ASCII .MP1 files for use with

VERSAMAP:

COPY E0.MP1 + VM_FIX.MP1 + E1.MP1 + VM_FIX.MP1 + E2.MP1 +

VM_FIX.MP1 + E3.MP1  EUROPE.MP1

Spurious diagonal lines will be plotted if you do not insert the

VM_FIX.MPS or VM_FIX.MP1 files between the World Digitized files

when they are concatenated for use with VERSAMAP.

VMAP.DOC version 1.50                                    Page  51

7.7.      Importing Your Own Data into VERSAMAP

File extensions PRN or UTM

In addition to the above file formats, VERSAMAP will also accept

data in 2 ASCII formats.  These ASCII formats provide an easy way

for users to plot their own data with VERSAMAP.  The file extension

of these ASCII files must be PRN for coordinates in degrees, or UTM

for coordinates in meters.  The PRN and UTM file formats are

identical, except for the difference in coordinates.

7.7.1.    Record Structure for PRN Files

Coordinates are expressed as latitude and longitude in decimal

degrees; negative values represent South latitude or West longitude.

Use the following formula to convert latitude and longitudes given

as degrees, minutes, and seconds to decimal degrees:

Decimal lat/lon = degrees + minutes/60 + seconds/3600

For example, the decimal position for Rio de Janeiro, Brazil (22

degrees, 53 minutes, 43 seconds south latitude; 43 degrees, 13

minutes, 22 seconds west longitude) is:

Decimal latitude  = -(22 + 53/60 + 43/3600) = -22.8953

Decimal longitude = -(43 + 13/60 + 22/3600) = -43.2228

The decimal position for Paris, France (48 degrees, 50 minutes, 14

seconds north latitude; 2 degrees, 20 minutes, 14 seconds east

longitude) is:

Decimal latitude  = +(48 + 50/60 + 14/3600) = +48.8372

Decimal longitude = +( 2 + 20/60 + 14/3600) = + 2.3372

Decimal latitude and longitude, input into VERSAMAP, are rounded to

the nearest second.  The program cannot distinguish between

positions less than 101 feet (31 meters, 1 second of latitude)

apart.

PRN File Format

field #1 - line header:  -1 for new polyline

0 for point within polyline

field separator:   one or more spaces (ASCII 32)

field #2 - latitude:   decimal degrees, south latitude is

negative

field separator:   one or more spaces (ASCII 32)

field #3 - longitude:   decimal degrees, west longitude is

negative

end of record:   carriage return (ASCII 13),

line feed (ASCII 10)

VMAP.DOC version 1.50                                    Page  52

7.7.2.    Record Structure for UTM Files

X,Y coordinates are expressed in meters.  Coordinates may contain

decimal points, but they may not be negative.

UTM File Format

field #1 - line header:  -1 for new polyline

0 for point within polyline

field separator:   one or more spaces (ASCII 32)

field #2 - x coordinate:  meters, value must be positive

field separator:  one or more spaces (ASCII 32)

field #3 - y coordinate:  meters, value must be positive

end of record:  carriage return (ASCII 13),

line feed (ASCII 10)

Records with these formats can be generated by most word processors,

text editors, and spreadsheets.

NOTE:     Several VERSAMAP users report getting INPUT PAST END OF

FILE error messages when plotting ASCII files.  This error

message usually indicates that the ASCII file contains

additional carriage returns (blank lines) after the last

coordinate in the file.  If you receive this error

message, delete any blank lines that occur at the end of

your file.

VMAP.DOC version 1.50                                    Page  53

7.7.3.    Plotting Sample ASCII Files

The files VMAPUSER.PRN and VMAPUSER.UTM on the VERSAMAP disk are

short files using the above ASCII formats.

VMAPUSER.PRN contains the names and locations of 4 cities which plot

as points; plus 5 points in the Indian Ocean which plot as a box

when the points are connected by lines.

VMAPUSER.UTM contains the names and locations of Belfast and Dublin

which plot as points; plus 5 points in the center of Ireland which

plot as a box when the points are connected by lines.

Use these 2 files to study the appearance of the different symbols,

fonts, and colors that VERSAMAP can plot.

To plot the files,

1.  Select the default Equirectangular projection for VMAPUSER.PRN,

or the default UTM projection for VMAPUSER.UTM.

2.  Select the Labels option of the Map Design menu, and choose the

Display Labels item.

3.  Select the Map Input Filenames option in the Input/Output menu.

4.  Tab to filename #2, type VMAPUSER.PRN or VMAPUSER.UTM for the

filename, and press Enter.

5.  When the window titled ENTER SYMBOL TYPE, COLOR, & SIZE

appears, select symbol type Solid square; the color Light

Magenta; size 03 pixels; and press Enter on the < ok > button.

6.  When the window titled SELECT LABEL SIZE & COLOR appears,

select point size 10; label color Light Magenta; PIC font 1;

and press Enter on the < ok > button.

7.  When asked for the next filename, tab to the < ok > button and

press Enter to end filename input.

8.  To display the file, select the Size 100% option in the Run

menu.

The screen will show the individual cities, marked by light magenta

squares, with their names printed at the lower right corner of the

square.

The box plots as 4 individual points, because all symbol types,

except Line, plot as points.  To plot the 4 individual points as a

box, repeat the above steps, but choose symbol type Line.  In this

case, the cities plot as light magenta dots, 1 pixel in size, which

are hard to see; but the points of the box are connected by lines,

and plot as a rectangle.

To view the contents of VMAPUSER.PRN, type the following command at

the DOS prompt and press Enter (the word TYPE is part of the

command).

TYPE VMAPUSER.PRN

To view VMAPUSER.UTM, substitute VMAPUSER.UTM for VMAPUSER.PRN.

VMAP.DOC version 1.50                                    Page  54

8.        Use of VERSAMAP with Graphing Programs

VERSAMAP can be used to prepare publication quality maps in

conjunction with suitable graphing programs.  To be useful in

preparing maps, a graphing program should be able to:

1.  handle large data sets;

2.  use more than 1 data set per graph;

3.  physically separate the X and Y axes, so that they do not show

on the figure;

4.  connect individual points on the graph with lines; and

5.  exclude data which exceeds the X- and Y-axis limits.

To print a map using a graphing program, import the map grid and the

output map boundary files into the program.

If there are two map grid files, the points in the first map grid

file should be plotted as dots, and the points in the second map

grid file should be connected by lines.  If there is only one map

grid file, you can connect the grid points by lines or not,

depending on how you want the grid to appear.

The X- and Y-axes are scaled using the values of X-axis Length and

Y-axis Length listed under Relative Length of Axes on the VERSAMAP

Parameter Report.  The relative X-axis Length is defined as 1.000000

for all maps; the relative Y-axis Length may be less than or greater

than unity, depending on the projection and grid chosen.   Starting

values of the X- and Y-axes should be set at 0.  The ending values

of the X- and Y-axes will depend on your graphing program, but in

general they will be one or both of the values listed under Relative

Length of Axes on the VERSAMAP Parameter Report.

You should configure the graphing program for data clipping (i.e.

exclude points which lie outside the limits of the X- and Y-axes).

This will prevent the dummy points (X = 5.000000), which indicate

new polylines (see Section 5.9, Save as ASCII), from being plotted.

Map and grid output files produced by VERSAMAP have been

successfully imported into the technical graphing programs, Grapher

and SigmaPlot.  For use with SigmaPlot, dummy points (X,Y =

5.000000) which indicate line breaks in map output files must be

converted to dashes (X,Y = --------), indicating missing data,

before the files are imported into SigmaPlot.  Changing the dummy

points from 5.000000 to -------- can be done using the search and

replace command in a word processor.

VMAP.DOC version 1.50                                    Page  55

9.        Program Compatibility with Software and Hardware

VERSAMAP has been tested successfully on computers running under DOS

versions 4.01, 5.00, 6.00, and 6.20.  VERSAMAP should run on any

computer with DOS 3.0 or higher.

9.1.      Running VERSAMAP under Windows

To run VERSAMAP as a DOS program under Microsoft Windows:

1.  Choose the Run option from the File menu in the Program

Manager.

2.  Type C:\VERSAMAP\VMAP in the Command Line box.  Change the

drive and path name if VERSAMAP is not stored in a directory

named VERSAMAP.

3.  Choose the OK button.

Run VERSAMAP using the full screen.  Do not run VERSAMAP in a

Window, since the mouse will not open VERSAMAP menus when the

program is running in a Window.  If you must work in a Window, use

the Alt key to open the VERSAMAP menus.

9.2.      Memory Requirements

The amount of conventional memory required to run VERSAMAP is

increased when the Courier or Helvetica fonts are loaded in the

program.  With no font loaded, VERSAMAP requires at least 443,000

bytes of free conventional memory.  With the Courier VGA font

loaded, the program requires 468,000 bytes of free memory.  With the

Helvetica VGA font loaded, VERSAMAP requires 508,000 bytes of free

conventional memory.

9.3.      Program Overlays

VERSAMAP uses program overlays to minimize conventional memory

usage.  Overlays are stored in extended or expanded memory and

loaded into conventional memory as needed.  On computers without

extended or expanded memory, overlays are loaded from disk.

Due to its use of program overlays, VERSAMAP runs fastest on

computers equipped with extended or expanded memory.  If extended or

expanded memory is not available, you will notice a delay, upon

selecting menu items, as overlays are loaded into memory.  This

delay is short for hard disks, but can be long when the program is

run from a floppy disk.

VMAP.DOC version 1.50                                    Page  56

9.4. Video Compatibility

VERSAMAP has been tested successfully on systems with EGA and VGA

adapters and monitors.

Several users have reported that VERSAMAP versions 1.20 and lower

would not run on their systems because it did not recognize the type

of video adapter (VGA or EGA) in their computer.  This version of

VERSAMAP has been modified to allow users to force the program to

run in a particular video mode.  This is done by adding an

additional parameter after the word VMAP on the DOS command line

when VERSAMAP is started.  VERSAMAP recognizes the following command

line parameters:

Command Line     Result

VMAP             Run VERSAMAP, allow program to select video mode

VMAP VIDEO       Display video monitor and adapter determined by

program

VMAP HELP        Display list of valid command line parameters

VMAP EGA         Force program to run in color EGA mode

VMAP EGAMONO     Force program to run in monochrome EGA mode

VMAP VGA         Force program to run in color or monochrome VGA

mode

If VERSAMAP cannot recognize the video system in your computer, you

will receive the error message "VERSAMAP 1.50 does not support your

monitor and/or video display adapter!".  If you receive this error

message, and you know the type of video adapter and monitor in your

system, you can force VERSAMAP to run in that video mode by using

one of the above commands.

For instance, to force VERSAMAP to run in color EGA mode on a

computer known to contain an EGA video adapter and a color EGA

monitor, type the command VMAP EGA at the DOS prompt and then press

Enter.

10.       Installing VERSAMAP

Installation of VERSAMAP is described in the file VMAPREAD.ME.

Please read VMAPREAD.ME for instructions on how to install the

program on your hard drive.  Due to its size, VERSAMAP cannot be run

from a low-density 5.25", 360K floppy disk.

To run VERSAMAP, once it has been installed, change to the directory

containing the VERSAMAP files, type VMAP at the DOS prompt, and then

press the Enter key.

VMAP.DOC version 1.50                                    Page  57

11.       VERSAMAP Order Form (version 1.50)              page 1/3

Make checks payable to Charles Culberson.  Persons living outside

the United States can register VERSAMAP by obtaining a Postal Money

Order payable in U.S. dollars from their National Post Office; or by

personal check in their local currency at the current exchange rate.

Send order form and comments to:

Charles Culberson

8 Ritter Lane

Newark, DE  19711

USA

------------------ Registration, version 1.50 ------------------

Date: __________________________________________________________

Where did you obtain VERSAMAP? _________________________________

Your Name: _____________________________________________________

E-mail Address: ________________________________________________

Postal Address: ________________________________________________

________________________________________________________________

________________________________________________________________

________________________________________________________________

Please mark the floppy disk size that you would like the VERSAMAP

files supplied on.

5.25" 1.2M                    3.5" 1.4M

--------------------------- Comments ---------------------------

VMAP.DOC version 1.50                                    Page  58

11.       VERSAMAP Order Form (version 1.50)              page 2/3

Item                                                           Cost

1.  VERSAMAP, registered copy , $15                          ______

Most recent version of mapping program plus Helvetica

fonts, maps VMAP3.MAP (31,000 points) and VMAP5.MAP

(6,000 points), and gazetteer containing positions of

over 600 world cities.

2.  Printed Manual, $15                                      ______

Printed copy of VERSAMAP instruction manual with clear

plastic cover.  The text of the printed manual is

identical to the text of the manual, VMAP.DOC, included

on the VERSAMAP disk.

3.  VERSAMAP Micro World Database, $15                       ______

Complete set of 5 maps, VMAP1.MAP (196,000 points) to

VMAP5.MAP (6,000 points).  See Section 7.3 of

instruction manual for details.

4.  World Data Bank II in VERSAMAP .PKV format.  See

Section 7.1 of instruction manual for details.

Region            Comments

Africa            Suez Canal divides Africa and Eurasia

Eurasia           includes Japan and Taiwan

Latin America     Mexico to Chile, including Antarctica

North America     Canada, United States (Hawaii is in Oceania)

Oceania           Indonesia, Philippines, Australia, New

Zealand, and Pacific Islands

------- Circle regions wanted -------

Region            Number of      Cost

Disks

Africa                2           $10

Eurasia               5           $25

North America         3           $15

Latin America         2           $10

Oceania               1           $ 5

Total cost of World Data Bank II files.   ______

VMAP.DOC version 1.50                                    Page  59

11.       VERSAMAP Order Form (version 1.50)              page 3/3

Item                                                           Cost

5.  USGS Digital Line Graph data in VERSAMAP .GRV format.

See Section 7.4 of instruction manual for details.

-------------- Circle sections wanted --------------

USGS         States on disk(s)                    Number of

Sections                                                Disks

01            CT, MA, ME, NY, VT, NH, RI                 1

02            DE, MD, OH, NJ, PA, VA, WV                 1

03, 04        FL, GA, NC, SC                             1

05            AL, AR, LA, MS, TN                         2

06            IA, IL, IN, KY, MO                         2

07            MI, MN, WI                                 1

08            TX     (TX in section 09 also)             1

09            TX, OK (TX in section 08 also)             1

10            CO, KS, NE                                 1

11            MT, ND, SD, WY (MT in section 15 also)     1

12            AZ, NM                                     1

13, 14, 21    CA, HI, NV, UT                             1

15            ID, MT, OR, WA (MT in section 11 also)     1

16 thru 20    AK                                         5

Total cost of USGS Digital Line Graph files

(number of disks x $5 per disk). ______

6.  $5 charge for shipping to addresses outside the U.S.

or Canada. ______

7.  $7 charge for checks drawn on foreign banks in foreign

currency.  Except for Canadian banks, I cannot accept

checks drawn on foreign banks in U.S. currency.  There

is no charge for checks drawn on U.S. banks.             ______

Minimum order is $15.  Total cost of items 1 through 7,

or $15, whichever is greater.                            ______

VMAP.DOC version 1.50                                    Page  60

12.       Acknowledgements

VERSAMAP is written in Microsoft Visual Basic for MS-DOS.  VERSAMAP

uses the font files COURB.FON, COURE.FON, HELVB.FON, and HELVE.FON

supplied with Microsoft BASIC 7.0.  Portions(C) 1982-1989 Microsoft

Corporation.  All rights reserved.

VERSAMAP uses routines from the PCX Programmer's Toolkit, Copyright

(c) Genus Microprogramming, Inc.  1988-1990.

VERSAMAP uses the QuickWindows Advanced User Interface Library which

is Copyrighted 1990 by Software Interphase, Inc.

The boundary files, VMAP1.MAP through VMAP5.MAP, contain data from

Micro World Data Bank II, a digital map database placed in the

Public Domain by Fred Pospeschil and Antonio Riveria.

The enhanced World Data Bank II files (.PKV file extension)

distributed with VERSAMAP are derived from the packed, binary

version of World Data Bank II (.PKD file extension) developed and

placed in the Public Domain by Fred Pospeschil (Micro Doc, 3108

Jackson Street, Bellevue, NE  68005, USA).

AutoCAD is a registered trademark of Autodesk, Inc.  CorelDRAW is

copyrighted by Corel Systems Corporation.  Epson is a registered

trademark of Epson America, Inc.  Grapher is a trademark of Golden

Software, Inc.  Hewlett-Packard is a registered trademark of

Hewlett-Packard Company.  Lotus and 1-2-3 are registered trademarks

of Lotus Development Corporation.  Microsoft is a registered

trademark and Windows, Visual Basic, and MS-DOS are trademarks of

the Microsoft Corporation.  SigmaPlot is a trademark of Jandel

Corporation.

VMAP.DOC version 1.50                                    Page  61

13.       Glossary

Aspect:   Conceptual placement of a projection in relation to the

earth's axis (polar, equatorial, oblique).

Azimuthal projection:   Projection on which the azimuth or direction

from the central point to any other point is shown correctly.

The scale of an azimuthal projection is constant along the

circumference of any circle centered on the map center.

Central meridian:   Meridian passing through the center of a

projection, often a straight line about which the projection is

symmetrical.

Conformal projection:   Projection on which all angles at each point

are preserved.  On a conformal projection the scale is the same

in all directions at a given point.

Conic projection:   Projection resulting from the conceptual

projection of the earth onto a tangent or secant cone, which is

then cut lengthwise and laid flat.

Cylindrical projection:   Projection resulting from the conceptual

projection of the earth onto a tangent or secant cylinder, which

is then cut lengthwise and laid flat.

Ellipsoid:   A solid geometric figure formed by rotating an ellipse

about its shorter axis.

Equal-area projection:   Projection on which the areas of all

regions are shown in the same proportion to their true areas.

Shapes may be greatly distorted.

Equatorial aspect:   Aspect of a projection on which the center of

projection or the origin is some point along the Equator.

Equidistant projection:   Projection that maintains constant scale

along all great circles from one or two points.

Great circle:   Any circle on the surface of the earth (assumed to

be a sphere) formed by the intersection of the surface with a

plane passing through the center of the earth.  It is the

shortest distance between any two points on the circle and

therefore important for navigation.  All meridians and the

Equator are great circles.

Grid:   Network of lines representing a selection of the earth's

parallels and meridians.

Kilometer:   Unit of length in the metric system.  1 km = 1000

meters = 0.53961 nautical miles = 3280.8 feet.

VMAP.DOC version 1.50                                    Page  62

Latitude:   Angle made by a perpendicular to a given point on the

surface of the earth and the plane of the Equator, north or south

to 90 degrees.  One of the two common geographic coordinates of

a point on the earth.

Longitude:   Angle made by the plane of a meridian passing through

a given point on the earth's surface and the plane of the (prime)

meridian passing through Greenwich, England, east or west to 180

degrees.  One of the two common geographic coordinates of a point

on the earth.

Meridian:   Reference line on the earth's surface formed by the

intersection of the surface with a plane passing through both

poles and some third point on the surface.  This line is

identified by its longitude.  On the earth, this line is half a

great circle.

Meter:   Basic unit of distance in the metric system.  Originally

defined as 1/10,000,000 of the distance from the equator to the

poles.  Consequently, 90 degrees of latitude is approximately

equal 10,000,000 meters.  1 meter = 3.2808 feet.

Nautical mile:   One minute of arc along a great circle.  For a

spherical earth of radius 6371 km, 1 nautical mile = 1853.2

meters = 6080 feet = 1 minute of arc.  Consequently, 1 degree

of arc = 60 minutes of arc = 60 nautical miles.

Oblique aspect:   Aspect of a projection on which the center of the

projection or the origin is located at a point which is neither

at a pole nor along the Equator.

Parallel:  Small circle on the surface of the earth formed by the

intersection of the surface with a plane parallel to the plane of

the Equator.  This line is identified by its latitude.  The

Equator (a great circle) is also treated as a parallel.

Polar aspect:   Aspect of a projection on which the earth is viewed

from the polar axis.

Radius of the earth:   VERSAMAP assumes that the earth is a sphere

of radius 6371 km for all projections except the Universal

Transverse Mercator.

Scale:  Ratio of the distance on a map to the corresponding distance

on the earth; usually stated in the form 1:5,000,000, for

example.

Secant cone, cylinder:   A secant cone or cylinder intersects the

earth's surface along two separate lines.

Small circle:   Circle formed by the intersection of the earth's

surface and a plane that does not pass through the center of the

earth.  Parallels of latitude, other than the Equator, are small

circles.

VMAP.DOC version 1.50                                    Page  63

Spheroid:   A solid geometric figure formed by rotating an ellipse

about its shorter axis.

Standard parallel:   A parallel of latitude along which the scale of

the map is correct.  There are one or two standard parallels on

most cylindrical and conic map projections.

Statute mile:   Unit of distance on land.  1 statute mile = 5280

feet = 1.60934 km = 0.86842 nautical miles.

VMAP.DOC version 1.50                                    Page  64

14.       References

When plotting maps with VERSAMAP it is useful to have an atlas handy

to determine the latitude/longitude coordinates of the area you wish

to plot.  An easy to use, up-to-date, and reasonably priced atlas is

the:

Quick Reference World Atlas.  1995.  Rand McNally & Company,

Chicago.  58 pages.  $5.95.  ISBN 0-528-83733-8.

A easily understandable book on the theory of map projections is:

Porter W. McDonnell.  1991.  Introduction to Map Projections.

Second edition.  Landmark Enterprises, 10324 Newton Way, Rancho

Cordova, California  95670.  198 pages.  ISBN 0-8247-6830.

The map projections and equations used in this program, along with

most of the definitions in the Glossary are taken from:

John P. Snyder.  1987.  Map Projections - A Working Manual.

U.S. Geological Survey Professional Paper 1395.  U.S.

Government Printing Office.  383 pages.

John P. Snyder and Philip M. Voxland.  1989.  An Album of Map

Projections.  U.S. Geological Survey Professional Paper 1453.

U.S. Government Printing Office.  249 pages.

A detailed, technical book on map projections is:

D. H. Maling.  1992.  Coordinate Systems and Map Projections.

Second edition.  Pergamon Press, Oxford.  476 pages.

ISBN 0-08-037234-1.

VMAP.DOC version 1.50                                    Page  65

15.       Index

Acknowledgements . . . . . . . . . . . . . . . . . . . . . . 1, 61

Add Text to Map. . . . . . . . . . . . . . .16, 25, 28, 30, 37, 39

Administrative boundaries. . . . . . . . . . . . . . . . . .47, 50

Afghanistan. . . . . . . . . . . . . . . . . . . . . . . . . . .19

Africa . . . . . . . . . . . . . . . . . . . . . . .19, 42, 44, 59

Airy . . . . . . . . . . . . . . . . . . . . . . . . . . . .19, 20

Alaska pipeline. . . . . . . . . . . . . . . . . . . . . . . . .50

Albers equal-area. . . . . . . . . . . . . . . . . .10, 12, 16, 22

ALL. . . . . . . . . . . . . . . . . . . . . . . . .25, 34, 46, 47

Alt. . . . . . . . . . . . . . . . . . . . . . . . . . 4-6, 27, 56

Antarctica . . . . . . . . . . . . . . . . . . . . . . .42, 44, 59

ASCII. . . . . . . .2, 28-30, 32, 34, 35, 38-40, 42, 44, 48, 51-55

Aspect . . . . . . . . . . . . . . . . . . . . . . . . .13, 62, 63

Association of Shareware Professionals . . . . . . . . . . . . . 4

Australia. . . . . . . . . . . . . . . . . . . . . . . . . .44, 59

Australian states. . . . . . . . . . . . . .24, 25, 43, 44, 46, 47

AutoCAD. . . . . . . . . . . . . . . . . . . . . . . . . . .27, 61

Azimuthal. . . . . . . . . . . . . . . . 10, 13, 14, 16-18, 24, 62

Azimuthal equal-area . . . . . . . . . . . . . . . .10, 14, 17, 24

Azimuthal equidistant. . . . . . . . . . . . . . . .10, 13, 17, 24

Background color . . . . . . . . . . . . . . . . . . 7, 16, 30, 36

Bessel . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Boundary file. . . . . . . . . . . . . . . . . . . . . . . . . .34

Britain. . . . . . . . . . . . . . . . . . . . . . . . . . .19, 20

Burma. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Canada . . . . . . . . . . . . . . . . . . . . . . . . .44, 59, 60

Canadian provinces . . . . . . . . . . . . . . .24, 25, 43, 46, 47

CCI. . . . . . . . . . . . . . . . . . . . . . . . . . .25, 34, 46

Central latitude of the map. . . . . . . . . . . . . . . . . . .18

Central longitude of the map . . . . . . . . . . . . . . . . . .18

CGM. . . . . . . . . . . . . . . . . . . . . .2, 27-30, 32, 37, 38

Chile. . . . . . . . . . . . . . . . . . . . . . . . . . . .19, 59

China. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Clarke . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

CNF. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .31

Coastlines . . . . . . . . . . . . . . . . . . . . . 24, 43-47, 51

Color. . . . . . . . . . . . .7, 16, 23-26, 30, 33, 34, 36, 54, 57

Command line . . . . . . . . . . . . . . . . . . . . . . . .56, 57

Compatibility. . . . . . . . . . . . . . . . . . . . . . 1, 56, 57

Configuration file . . . . . . . . . . . . . . . . . . . . . . .31

Conformal. . . . . . . . . . . . . . . . . . . . 10-13, 16, 22, 62

Conformal conic. . . . . . . . . . . . . . . . . . .10, 12, 16, 22

Conic. . . . . . . . . . . . . . . . . . . .10, 12, 16, 22, 62, 64

CorelDRAW. . . . . . . . . . . . . . . . . . . . . . . . . .38, 61

Countries. . . . . . . . . . . . . . . . . . . .20, 24, 44, 46, 47

COUR_EGA.FON . . . . . . . . . . . . . . . . . . . . . . . . 2, 28

COUR_VGA.FON . . . . . . . . . . . . . . . . . . . . . . . . 2, 28

Courier. . . . . . . . . . . . . . . . . . . 2, 25, 26, 28, 29, 56

Cylindrical. . . . . . . . . . . . . . . . . . . . . . .10, 62, 64

DAT. . . . . . . . . . . . . . . . . . . . . . . . . . . . .33, 51

Data clipping. . . . . . . . . . . . . . . . . . . . . . . . . .55

VMAP.DOC version 1.50                                    Page  66

Default. . . . . . . 1, 4, 5, 9, 13, 17-25, 29, 33, 34, 36, 38, 54

Degree sign. . . . . . . . . . . . . . . . . . . . . . . . . . .27

Degrees of latitude between grid marks . . . . . . . . . . .17, 18

Degrees of longitude between grid marks. . . . . . . . . . .17, 18

Digital Line Graph . . . . . . . . . . . 1, 25, 34, 42, 47, 48, 60

Digital map. . . . . . . . 1, 2, 4, 25, 33, 34, 39, 42, 46, 47, 61

Directory. . . . . . . . . . . . . . .4, 29, 31-33, 35, 51, 56, 57

Disclaimer . . . . . . . . . . . . . . . . . . . . . . . . . .1, 3

Disk directory . . . . . . . . . . . . . . . . . . . . . . .32, 35

Display Previous Text. . . . . . . . . . . . . . . .16, 29, 31, 37

Distance above surface in kilometers . . . . . . . . . . . . . .18

DOS. . . . . . . . . . . . . . . . 3, 5, 6, 27, 51, 54, 56, 57, 61

Dot matrix . . . . . . . . . . . . . . . . . . . . . . . 2, 32, 36

DXF. . . . . . . . . . . . . . . . . . . . . .2, 27-30, 32, 37, 38

Easternmost longitude of the map . . . . . . . . . . . . . . . .17

Easting. . . . . . . . . . . . . . . . . . . . . . . . .15, 19, 20

EGA. . . . . . . . . . . . . . . . . . . . . . 2-4, 28, 29, 32, 57

Ellipsoid. . . . . . . . . . . . . . . . . . . . . .10, 14, 15, 62

Equal-area . . . . . . . . . . . . . 10-12, 14, 16, 17, 22, 24, 62

Equatorial . . . . . . . . . . . . . . . . . . . . . . .11, 19, 62

Equidistant. . . . . . . . . . . . . . . 10-14, 16, 17, 22, 24, 62

Equidistant conic. . . . . . . . . . . . . . . . . .10, 12, 16, 22

Equirectangular. . . . . . . . . . 4, 9-11, 16, 17, 23, 38, 39, 54

Eritrea. . . . . . . . . . . . . . . . . . . . . . . . .44, 46, 47

Error message. . . . . . . . . . . . . . 9, 17, 18, 21, 22, 53, 57

Esc. . . . . . . . . . . . . . . . . . .5-7, 9, 27, 28, 35, 36, 41

Eurasia. . . . . . . . . . . . . . . . . . . . . . . . . . .44, 59

Europe . . . . . . . . . . . . . . . . . . . . . . .19, 42, 44, 51

Everest. . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Exit . . . . . . . . . . . . . . . . 1, 5-7, 10, 17, 22-24, 35, 36

Export . . . . . . . . . . . . . . . . . . . 26, 29, 30, 32, 38-40

F1 . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .35

Filename . . . . . . . . . . .24, 25, 31, 33, 34, 36-40, 45-47, 54

FON. . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 28, 61

Font . . . . . . . . . . . . . . 2, 16, 25, 26, 28, 29, 54, 56, 61

Format . . . . . . . . . . . 34, 38, 40, 42, 43, 48, 51-53, 59, 60

France . . . . . . . . . . . . . . . . . . . . . . . . . 6, 19, 52

Gauss-Kruger . . . . . . . . . . . . . . . . . . . .14, 15, 19, 20

Geographic Feature . . . . . . . . . . . . . . .24, 34, 38, 46, 47

Geological Survey. . . . . . . . . . . . . .25, 34, 42, 47, 48, 65

Germany. . . . . . . . . . . . . . . . . . . . . . . . . . . . .47

Gnomonic . . . . . . . . . . . . . . . . . . . . . .10, 13, 17, 24

Grapher. . . . . . . . . . . . . . . . . . . . . . . . . . .55, 61

Graphing program . . . . . . . . . . . . . . . . . . . . . .32, 55

Great Circle . . . . . . . . . . . . . .16, 23, 32, 35, 40, 62, 63

Great Circle Report. . . . . . . . . . . . . . . . .23, 32, 35, 40

GRF. . . . . . . . . . . . . . . . . . . . . . . . . . . 34, 47-49

Grid color . . . . . . . . . . . . . . . . . . . . . . .16, 30, 36

Grid file. . . . . . . . . . . . . . . . . . . . . . . . . .38, 55

Grid type. . . . . . . . . . . . . . . . . . . . . .16, 24, 35, 38

GRV. . . . . . . . . . . . . . . . . . . . . . . .2, 34, 47-49, 60

Hammer equal-area. . . . . . . . . . . . . . . . . . . .10, 11, 16

Hawaii . . . . . . . . . . . . . . . . . . . . . . . . . . . . .59

HELV_EGA.FON . . . . . . . . . . . . . . . . . . . . . . . . 2, 28

VMAP.DOC version 1.50                                    Page  67

HELV_VGA.FON . . . . . . . . . . . . . . . . . . . . . . . . 2, 28

Helvetica. . . . . . . . . . . . . . .2, 4, 25, 26, 28, 29, 56, 59

Import . . . . . . . . . . . . . . .26, 29, 32, 38, 40, 44, 48, 55

Index. . . . . . . . . . . . . . . . . . . . . . . . . . . . 1, 66

India. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Indonesia. . . . . . . . . . . . . . . . . . . . . . . .19, 44, 59

Input field. . . . . . . . . . . . . . . . . . . . . . . . . . . 7

Input file . . . . . . . . . . . . . . . .5, 9, 25, 33, 34, 38, 45

Input past end of file . . . . . . . . . . . . . . . . . . . . .53

Input/Output menu. . .1, 19, 23, 25, 28, 30, 32, 36-40, 44, 48, 54

Installing VERSAMAP. . . . . . . . . . . . . . . . . . . .1, 3, 57

Ireland. . . . . . . . . . . . . . . . . . . . . . . . .19, 20, 54

Islands. . . . . . . . . . . . . . . . . . . . . . . 24, 43-48, 59

Israel . . . . . . . . . . . . . . . . . . . . . . . . . . .43, 47

Japan. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .59

Krasovsky. . . . . . . . . . . . . . . . . . . . . . . . . .19, 20

Labels . . . . . . . . . . . . . . . . . . . . .16, 29, 30, 34, 54

Lakes. . . . . . . . . . . . . . . . . . . . . . 24, 43, 45-47, 49

Laser. . . . . . . . . . . . . . . . . . . . . . . . . . 2, 32, 36

Latin America. . . . . . . . . . . . . . . . . . . . . . . .44, 59

Latitude . . . . . . . . . . . .1, 5, 6, 16-22, 35, 39, 40, 52, 63

Latitude and longitude limits. . . . . . . . . . .7, 8, 16, 17, 21

Layer. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .38

Letter B . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

Letter T . . . . . . . . . . . . . . . . . . . . . . . . . . . . 7

List box . . . . . . . . . . . . . . . . .7, 8, 24, 34, 35, 44, 48

Longitude. . . . . . . . . . . . . .1, 5, 6, 16-22, 35, 39, 52, 63

Lotus. . . . . . . . . . . . . . . . . . . . . . . . . .26, 32, 61

MAP. . . . . . . . . . . . . . . . 2, 4, 5, 25, 33, 34, 47, 59, 61

Map Boundaries . . . . . . . . . . . . . . 7, 8, 16-18, 20, 44, 48

Map Design menu. . . .1, 16, 25, 28, 30, 31, 35-38, 40, 44, 48, 54

Map Input Filenames. . . . . . . . . . .25, 32, 33, 38, 40, 48, 54

Map Parameter Report . . . . . . . . . . . . . . 19, 32, 35-37, 40

Math coprocessor . . . . . . . . . . . . . . . . . . . . . . . . 3

Menu . . 1, 4-10, 16, 19, 23, 25, 27, 28, 30-32, 35-41, 44, 48, 54

Menu bar . . . . . . . . . . . . . . . . . . 4-7, 9, 27, 35-37, 41

Mercator . . . . . . . . . . . . . . . . . . .9-11, 14, 16, 17, 63

Meridian . . . . . . . . . . . . . . 5, 11, 12, 14, 15, 18, 62, 63

Mexico . . . . . . . . . . . . . . . . . . . . . . . . . . . . .59

Micro Doc. . . . . . . . . . . . . . . . . . . . . . 42, 44-46, 61

Micro World Data Bank II . . . . . . . . .1, 7, 24, 34, 42, 46, 61

Microsoft Windows. . . . . . . . . . . . . . . . . . . . . . . .56

Mouse. . . . . . . . . . . . . . . . . . . . . 4, 6, 7, 27, 28, 56

MP1. . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 33, 51

MPS. . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 33, 51

MWDB-II. . . . . . . . . . . . . . . . . . . . .24, 25, 42, 46, 47

National Grid. . . . . . . . . . . . . . . . . . . . . . . . . .20

New Zealand. . . . . . . . . . . . . . . . . . . . . . . . . . .44

North America. . . . . . . . . . . . . . . . . . . .19, 42, 44, 59

Northernmost latitude of the map . . . . . . . . . . . . . . . .17

Northing . . . . . . . . . . . . . . . . . . . . . . . . 15, 19-21

Oahu . . . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 48

Oblique. . . . . . . . . . . . . . . . . . . . . . . . . . .62, 63

Oceania. . . . . . . . . . . . . . . . . . . . . . . . . . .44, 59

VMAP.DOC version 1.50                                    Page  68

Oman . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .47

Ombudsman. . . . . . . . . . . . . . . . . . . . . . . . . . . . 4

Order form . . . . . . . . . . . . . . .1, 2, 4, 44, 47, 48, 58-60

Orthographic . . . . . . . . . . . . . . . . . . . .10, 13, 17, 24

Output file. . . . . . . . . . . . . . . . . . . . . . .27, 38, 39

Pakistan . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

Parallel . . . . . . . . . . . . . . . . . .11, 12, 22, 23, 63, 64

PCX. . . . . . . . . . . . . . . . . 2, 27, 28, 30, 32, 36, 37, 61

Philippines. . . . . . . . . . . . . . . . . . . . . . . . .19, 59

PIC. . . . . . . . . . . . . . . . . . . .2, 25-30, 32, 37, 38, 54

PIC font . . . . . . . . . . . . . . . . . . . . . . . .25, 26, 54

PKD. . . . . . . . . . . . . . . . . . . . . . . . . 34, 42-45, 61

PKV. . . . . . . . . . . . . . . . . . . . . .2, 34, 42-45, 59, 61

PNT. . . . . . . . . . . . . . . . . . . . . . . . . . .25, 33, 46

Point size . . . . . . . . . . . . . . . . . . . . .25, 29, 34, 54

Polar. . . . . . . . . . . . . . . . . . . . . .11, 13, 15, 62, 63

Political boundaries . . . . . . . . . . . . . . 38, 43-45, 47, 50

Polyline . . . . . . . . . . . . . . . . . . . .33, 39, 40, 52, 53

Print Map. . . . . . . . . . . . . . . . . . . .27, 28, 30, 32, 36

Printer. . . . . . . . . . . . . . . . . . . . . . . . . . .32, 36

Printer port . . . . . . . . . . . . . . . . . . . . . . . . . .36

PrintGraph . . . . . . . . . . . . . . . . . . . . . . . . . . .26

PRN. . . . . . . . . . . . . . . .2, 4, 29, 30, 33, 34, 40, 52, 54

Projection menu. . . . . . . . . . . . . . . . . . . . 1, 5, 6, 10

Pull-down menu . . . . . . . . . . . . . . . . . . . . . . . . . 9

Pushbutton . . . . . . . . . . . . . . . . . . . . . . . . . . . 8

Radius of earth. . . . . . . . . . . . . . . . . . .10, 14, 19, 63

Railroads. . . . . . . . . . . . . . . . . . . . . . . .38, 47, 50

Range of map in degrees. . . . . . . . . . . . . . . . . . . . .18

Recall Map Design. . . . . . . . . . . . . . . . . . . . . .16, 31

REGISTER.DOC . . . . . . . . . . . . . . . . . . . . . . . . .2, 4

Registration . . . . . . . . . . . . . . . . . . . . . 1, 3, 4, 58

Rivers . . . . . . . . . . . . . . . . . . . . . 24, 38, 43-47, 49

Robinson . . . . . . . . . . . . . . . . . . . . . . . .10, 12, 16

Run menu . . . . . . . . . 1, 4, 9, 23, 31, 37, 39, 41, 44, 48, 54

Satellite. . . . . . . . . . . . . . . . . . . . . . . . . . . .18

Save as ASCII. . . . . . . . . . . . . . . . . .28, 32, 38, 39, 55

Save as CGM. . . . . . . . . . . . . . . . . . . . .27, 30, 32, 37

Save as DXF. . . . . . . . . . . . . . . . . . . . . . .27, 32, 37

Save as PCX. . . . . . . . . . . . . . . . . . .27, 28, 30, 32, 36

Save as PIC. . . . . . . . . . . . . . . . . . . . . . .27, 32, 37

Save Great Circle. . . . . . . . . . . . . . . . . . . . . .32, 40

Save Map Design. . . . . . . . . . . . . . . . . . . . . . .16, 31

Scotland . . . . . . . . . . . . . . . . . . . . . . . . . .43, 44

Select Font. . . . . . . . . . . . . . . . . . . . . . .16, 28, 29

Select Printer . . . . . . . . . . . . . . . . . . . . . . .32, 36

Shareware. . . . . . . . . . . . . . . . . . . . . . . 2-4, 42, 51

SigmaPlot. . . . . . . . . . . . . . . . . . . . . . . . . .55, 61

Size 100%. . . . . . . . . . . . . . . . 4, 37, 39, 41, 44, 48, 54

Size 70% . . . . . . . . . . . . . . . . . . . . . . . .37, 39, 41

Slovakia . . . . . . . . . . . . . . . . . . . . . . . .44, 46, 47

Sources of digital map data. . . . . . . . . . . . . . . . . 1, 42

Southernmost latitude of the map . . . . . . . . . . . . . . . .17

Soviet Union . . . . . . . . . . . . . . . . . . . .19, 44, 46, 47

VMAP.DOC version 1.50                                    Page  69

Spheroid . . . . . . . . . . . . . . . . . . . . . . . .19, 20, 64

Standard Parallel. . . . . . . . . . . . . . . . . . . .22, 23, 64

States . . . . . . . . . . . . 12, 24, 25, 42-44, 46-48, 51, 58-60

Stereographic. . . . . . . . . . . . . . . .10, 13, 15, 17, 18, 24

Streams. . . . . . . . . . . . . . . . . . . . . . . . . . .47, 49

Suez canal . . . . . . . . . . . . . . . . . . . . . . . . . . .59

Symbol . . . . . . . . . . . . . . . . . . . . . . . . . . .33, 54

Tab. . . . . . . . . . . . . . 7, 8, 22, 23, 27, 33, 34, 36-39, 54

Taiwan . . . . . . . . . . . . . . . . . . . . . . . . . 2, 44, 59

Text . . . . . . . . . 2, 8, 16, 25-31, 34, 36, 37, 39, 41, 53, 59

Text # . . . . . . . . . . . . . . . . . . . . . . . . . . .25, 28

Thailand . . . . . . . . . . . . . . . . . . . . . . . . . . . .19

U.S. Geological Survey . . . . . . . . . . .25, 34, 42, 47, 48, 65

U.S. states. . . . . . . . . . . . . . . . . . . . .24, 43, 46, 47

United Arab Emirates . . . . . . . . . . . . . . . . . . . . . .47

United States. . . . . . . . . .12, 25, 42, 44, 47, 48, 51, 58, 59

Universal Transverse Mercator. . . . . . . . . . . . . . . .10, 14

USGS . . . . . . . . . . . . . . . . . . . 1, 2, 25, 42, 47-50, 60

USGS DLG . . . . . . . . . . . . . . . . . . . . .2, 25, 42, 47-50

UTM. . . . . . . . . . 2, 10, 14, 15, 18-21, 29, 30, 33, 34, 52-54

UTM zone . . . . . . . . . . . . . . . . . . . . . . . . . . 19-21

VERSAMAP Micro World Database. . . . . . . . . . . . 1, 46, 47, 59

Vertical perspective . . . . . . . . . . . . . .10, 13, 17, 18, 24

VGA. . . . . . . . . . . . . . . . . . . . 2-4, 28, 29, 32, 56, 57

Video. . . . . . . . . . . . . . . . . . . . . . . . . . . .29, 57

View Disk Directory. . . . . . . . . . . . . . . . . . . . .32, 35

VM_FIX.MP1 . . . . . . . . . . . . . . . . . . . . . . . . . 2, 51

VM_FIX.MPS . . . . . . . . . . . . . . . . . . . . . . . . . 2, 51

VMAP.DOC . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 59

VMAP.EXE . . . . . . . . . . . . . . . . . . . . . . . . . . 2, 29

VMAP1.MAP. . . . . . . . . . . . . . . . . . . . . . . .47, 59, 61

VMAP2.MAP. . . . . . . . . . . . . . . . . . . . . . . . . . . .47

VMAP3.MAP. . . . . . . . . . . . . . . . . . . . . . .2, 4, 47, 59

VMAP4.MAP. . . . . . . . . . . . . . . . . . . . .2, 5, 25, 33, 47

VMAP5.MAP. . . . . . . . . . . . . . . . . . . . .2, 4, 47, 59, 61

VMAPREAD.ME. . . . . . . . . . . . . . . . . . . . . . . .2, 3, 57

VMAPUSER.PRN . . . . . . . . . . . . . . . . . . . . . . . . 2, 54

VMAPUSER.UTM . . . . . . . . . . . . . . . . . . . . . . . . 2, 54

VMCITY.PRN . . . . . . . . . . . . . . . . . . . . . . . . . .2, 4

VMOAHU.GRV . . . . . . . . . . . . . . . . . . . . . . . . . 2, 48

VMTAIWAN.PKV . . . . . . . . . . . . . . . . . . . . . . . . 2, 44

Wales. . . . . . . . . . . . . . . . . . . . . . . . . . . .43, 44

Warsaw Pact. . . . . . . . . . . . . . . . . . . . . . . . .19, 20

WDB-II . . . . . . . . . . . . . . . . . . . . .25, 42, 44, 46, 48

Westernmost longitude of the map . . . . . . . . . . . . . . . .17

Windows. . . . . . . . . . . . . . . . . . . . . . . 1, 27, 56, 61

World Data Bank I. . . . . . . . . . . . . . . . . . . . . .34, 51

World Data Bank II . . . . .1, 2, 7, 24, 25, 34, 42-44, 46, 59, 61

World Digitized. . . . . . . . . . . . . . . . . . . . . 2, 34, 51

X-axis Length. . . . . . . . . . . . . . . . . . . . . . . .35, 55

Y-axis Length. . . . . . . . . . . . . . . . . . . . . . . .35, 55

Yemen. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .47

Yugoslav Republics . . . . . . . . . . . . . . . . . . . . . . .47

VMAP.DOC version 1.50                                    Page  70