MODEM INTERFACING:

������������������

The MODEM.PAS library shows how to interface a modem by using the pro-

cedures offered with SERIAL PORTS MANAGER. You could insert in it some

other sequences of HAYES commands, or write a special interfacing adapted

to some other modems NOT HAYES compatibles...

MODEM.C is the translation of MODEM.PAS in Turbo-C (C).

For the assembler, it is sufficient to proceed in a similar way.

A simple demonstration of how to use each of these libraries is distri-

-buted with SERIAL PORTS MANAGER: TESTMODP.EXE (for MODEM.PAS) and

TESTMODC.EXE (for MODEM.C).

!! ATTENTION: modify the "CommPort" variable in these programs before

!! compiling them, in order to use the serial port on which your modem

!! is physically installed !

����������������������������������������������Ŀ

�Globals variables (PASCAL and C types: PAS/C) �

������������������������������������������������

ModemUsed       SHORTINT/byte      Modem type used: Hayes or Minitel ?

Permit to send the specifics commands

associated to the modem to drive !

Its initial value is 'Hayes'.

MINITEL <---> TlTex

HAYES   <---> Hayes

DTR_Cmde        BOOLEAN/byte       Using DTR as a command prefix or NOT ?

Default is TRUE. IF this variable is

set to FALSE, the following prefix

MUST be sent first before each command.

HAYESPrefix     STRING[3]/char[4]  Command prefix for the HAYES modems.

'+++' = default value.

PulseDial       BOOLEAN/byte       TRUE if you want a pulsed call, FALSE

if you need a frequency call.

Its default value is TRUE.

connected       BOOLEAN/byte       Informes on the connection status. TRUE

if the line is connected, FALSE if not.

Is up-dated by the clock interrupt

function. FALSE is its intial value.

Show_CorF       BOOLEAN/byte       Write the connection sign on the screen?

TRUE => The sign is written each second

on the screen when the clock interrupt

procedure is installed.

Default value is FALSE.

Int1CSave       POINTER/<NIL>      Stores the INT 1Ch vector initial value.

Used to activate or disactivate the clock

interrupt procedure in the PASCAL library.

���������������������������������Ŀ

�Available procedures (PAS and C):�

�����������������������������������

1�/ To send any command to the modem:

PROCEDURE CmdeToModem(TypCmde: BYTE;DataCmde: CmdeStr);

void CmdeToModem(byte TypCmde, char *DataCmde)

Possible values for 'TypCmde':

Personal command     <---> Generique

Initialization       <---> InitMod

Connection           <---> Connect

Call a given number  <---> Appel

Hand-off the line    <---> Raccroch

Minitel "Connex/Fin" <---> ConnexFin

2�/ Clock interrupt made to check the connection status:

PROCEDURE Check_HModem_State; INTERRUPT;

3�/ To have more informations, report yourself to the listing... Specially

in C, the 2 following procedures permit to install/uninstall the clock

interrupt made to check the connection status...

void install_Clock(byte com)

void un_install_Clock()