[                 D A N P L O T   A N D   D A N C A M                ]

SHAREWARE DOCUMENTATION

FOR

DANPLOT.EXE v2.6 (tm) AND DANCAM.EXE v2.6 (tm)

THE TWO COMPANION

COMPUTER AIDED MANUFACTURING PROGRAMS

FOR USE WITH DANCAD3D v2.6 (tm)

Copyright (c) 1989-1994 by Daniel H. Hudgins, All rights reserved.

Daniel H. Hudgins, 466 Diamond Street, San Francisco, CA 94114, USA

SEE ALSO THE FILE DANCAD3D.DOC,  AND DRAWING  HOOKUP1.2D,  ON  THE

DANCAD3D  SHAREWARE  DISKS.   USE  THE  PROGRAM  PRINTDOC.EXE  ON  THE

DANCAD3D SHAREWARE DISKS TO PRINT OUT THIS FILE IN ORDER TO  HAVE  THE

TEXT CENTERED PROPERLY.

This shareware documentation may not be copied or reproduced other

than  as  a disk file distributed on the DANCAM/DANPLOT v2.6 shareware

disks.

This file contains abbreviated information from  what  is  in  the

manuals that come with the full user outfit.

Please read all disclaimers and all warnings,  notices, and danger

statements before you use DANCAM or DANPLOT.  Since DANCAM and DANPLOT

control motors,  and other electro-mechanical  hardware,  the  use  of

DANCAM  or  DANPLOT can cause serious or fatal personal injury.  It is

assumed that the user of DANCAM and DANPLOT  understands  the  dangers

involved  with  working with power driven machinery,  and high voltage

circuitry,  and will take any and all safety precautions necessary  to

protect  himself  and others from injury.  Although DANCAM and DANPLOT

are being used successfully,  DANCAM  and  DANPLOT  may  not  function

properly  at  all  times and are provided AS IS for you to USE ONLY AT

YOUR OWN RISK.

[1]

[                 T A B L E   O F   C O N T E N T S                  ]

Disclaimer and Warning................................3

Abstract..............................................6

Overview of DANCAM and DANPLOT........................9

Program features.....................................19

Applications.........................................21

General information..................................27

DOS command line and batch files.....................31

Hardware requirements................................33

Hook up connections..................................37

Configure and install................................45

Drawing tool path files..............................54

Running the CAM demo files...........................62

Help with DANCAM and DANPLOT.........................63

Electronic circuits..................................68

Where to get parts...................................81

Recommended reading..................................82

[2]

[           D I S C L A I M E R   A N D   W A R N I N G              ]

QUICK DISCUSSION OF IDEA

You use DANCAD3D,  DANCAM,  DANPLOT and all  of  DANCAD3D's  parts

totally  at  your  own  risk.  The program is sold "as is" and may not

work.  Any trade marks  mentioned  in  DANCAD3D  or  DANCAD3D's  parts

belong to their owners.

STEPS TO CARRY OUT IDEA

1) Read the disclaimer before using the programs or other information.

2) Back up your harddisk, wear protective clothing.

3) Establish safety standards and methods.

4) Start using the program if you agree to the disclaimer.

DETAILED DISCUSSION OF IDEA

Two  things  to keep in mind:  "Better safe than sorry",  and "Haste

makes waste." If you work carefully and take your time to think things

through I am sure that you will find my products fascinating, fun, and

useful.  But be aware that things can go  seriously  wrong,  and  that

automated  machinery  is  even  more  dangerous than manually operated

machinery.

DISCLAIMER

Please remember that DANCAM can DANPLOT are sold AS IS, please use

them ONLY AT YOUR OWN RISK.  Daniel H. Hudgins will not be responsible

or liable for any loss,  damage,  or injury of any kind.  Please  read

the  disclaimer that is in the DANCAD3D manual since it covers DANPLOT

and DANCAM as well.  Also read the disclaimer in the file DANCAD3D.DOC

on the DANCAD3D shareware disks,  as well as this disclaimer  and  the

warning below.

This  documentation  for DANCAM and DANPLOT may contain errors and

or  omissions.  Under  no  circumstances  or  conditions  will  Daniel

Hudgins be held accountable, or liable, for any errors or omissions.

This  documentation  includes suggested connections for hooking up

devices to your computer's parallel port.  Daniel Hudgins will not and

shall not be held accountable or liable for any loss  or  damage  that

might   occur   to  your  computer  to  other  devices.   It  is  your

responsibility  to  check  and  verify  all  the  appropriateness   of

connections  to  your computer before turning on the power.  Verifying

the appropriateness  of  connections  may  require  you  to  read  and

understand more information than this DANCAM and DANPLOT documentation

contains.

Under  no  circumstances  are  DANCAM  or  DANPLOT  to  be used in

equipment that is  used  for  medical  purposes.  DANCAM  and  DANPLOT

[3]

[           D I S C L A I M E R   A N D   W A R N I N G              ]

should  not be used in equipment that could cause injury to persons or

loss of property if  the  equipment  or  software  failed  to  operate

properly.

The  use  of  DANCAM  or  DANPLOT  is disallowed if any laws might

require Daniel Hudgins to be in any way responsible or liable for  any

loss  or  injury.  DANCAM  and  DANPLOT  may  not satisfy occupational

safety laws in your state or country.

No license to third parties is expressed or implied.  No violation

of copyrights or patents held by  third  parties  is  implied  by  the

suggested  use  and connection of DANCAM and DANPLOT.  The user should

confirm that no copyrights or patents will be violated by his  use  of

DANCAM or DANPLOT.

The  use  of  DANCAM  or  DANPLOT indicates full acceptance of all

disclaimers and an understanding that DANCAM and DANPLOT are  only  to

be used at the users OWN RISK.

In  this documentation the what is said about using stepper motors

will also generally apply to servo motors and any other type of motors

that might be used.  It is a fact that any component can and should be

expected to fail, therefore your safety depends on your not relying on

any particular component to protect you from  harm.  To  improve  your

safety  from  harm  use  several stages of protection,  so that if any

single safety measure fails you have several others to back the failed

one up.  Always be more careful than you think you need to be!

WARNING

Power driven and or automated equipment can cause  serious  injury

or  even death.  You must take serious efforts to protect yourself and

anyone else near such equipment.  Do not place  your  hands  or  other

body parts near power driven equipment without first disconnecting the

power  to  the  stepper motors (or other type of motors) in such a way

that the motors and tool  cannot  harm  you  or  others.  DANPLOT  and

DANCAM  are  only  to  be  used  by people that understand the dangers

involved and have been trained in the  necessary  safety  precautions.

Be   aware   that   failure  of  electrical  connections  or  computer

hardware/software  related  faults   can   cause   motors   to   start

unexpectedly.  To  help avoid injury caused by unexpected motor starts

you should disconnect the motor power at both  the  AC  input  of  the

motor  power supply and the DC output of the motor power supply before

you expose yourself or others to  the  equipment.  Evaluation  of  the

completeness   of   any   safety   measures   is   the   users  (your)

responsibility.

DO NOT USE any TSR program that puts a clock on the  screen  while

the   DANCAM  or  DANPLOT  programs  are  in  use!   Do  not  use  any

multitasking,  or print spooling programs while DANCAM or DANPLOT  are

in  use!  Such  programs  interfere  with the computers CPU timing and

therefore make the motors not run smoothly.  Do not use the DOS screen

[4]

[           D I S C L A I M E R   A N D   W A R N I N G              ]

dump command,  e.g.  [Shift] and  [PrtSc],  while  running  DANCAM  or

DANPLOT  since the parallel port is not connected to a printer and the

motors or auxiliary relays may be accidentally turned on (the computer

may also lock up since the screen print command will  not  be  getting

the correct printer messages!)

If the computer re-boots because of: a power failure, your turning

the computer off,  your pressing [Ctrl] & [Alt] & [Del],  or any other

reason the voltage on the output pins of the parallel port may  change

and   activate   the  auxiliary  relays  and  other  equipment  in  an

inappropriate and possibly harmful manner.  You should investigate the

default state of the  voltage  on  the  parallel  port  pins  after  a

computer  reboot  to  determine if you have the high/low state for the

output optimized to minimize any hazard.

When using servo motors be aware that  electro-magnetic  noise  or

brief  power  interruptions/serges  can upset the position counters in

the servo controller and make the motors unexpectedly start at maximum

speed and torque.  Failure  of  the  components  in  the  servo  motor

amplifier  can  also  make  the  motor  start at full torque and speed

unexpectedly (in stepper drivers component failures generally make the

motor stall.) To control the damage that might be caused  by  a  servo

controller/amplifier  accident  you  should  have extra contacts (or a

second limit switch) in your limit switches that will kill  the  power

to the servo motor if the limit switch point is exceeded.

[5]

[                         A B S T R A C T                            ]

QUICK DISCUSSION OF IDEA

DANCAM  and  DANPLOT  are  the  first widely distributed shareware

personal manufacturing programs.  They let you use your computer as  a

motion  controller  without  buying  any  special  hardware interface.

Simple and  inexpensive  circuitry  can  be  used  to  interface  your

computers  parallel  port to operate almost any stepper or servo motor

driver  module  that  uses  step  and  direction  signals.   Hook   up

information  and  printed circuit foil patterns for a Bi-Polar stepper

motor translator/driver board are included in files on  the  shareware

disks.  Additional  schmatics for low cost stepper and DC servo driver

modules you can build are available from the author.

STEPS TO CARRY OUT IDEA

1) Read this documentation.

2) Buy the parts needed to automate your machine.

3) Hook up your computer, driver modules, and attach the motors.

4) Install, configure, and use the programs.

DETAILED DISCUSSION OF IDEA

DANCAM and DANPLOT are two utility programs that you  can  use  to

automatically  manufacture  parts.  DANCAM and DANPLOT run on your IBM

(tm) compatible personal computer and  use  your  personal  computer's

parallel port to send motion control signals to stepper or servo motor

translator/driver  modules.  The  parallel  port is also used to drive

all the necessary control relays and to  read  all  of  the  necessary

home, limit and pause switches.

The  macro  BIPOLAR2.MAC  on  the  DANCAM & DANPLOT v2.6 shareware

disks will make PCB foil patterns for building low cost stepper  motor

translator/driver modules.  If you do not want to build circuit boards

you  can  buy  assembled  translator/drivers  from  your  local  motor

supplier.

Servo motor control circuits that use "step and direction" signals

can be used in place of stepper motor translator/drivers  circuits  to

obtain  greater  speed and freedom from occasional lost step problems.

A set of electronic drawings for a low cost closed  loop  servo  motor

driver compatible with DANCAM v2.6 and DANPLOT v2.6 is included in the

manuals that are available from the author.

How  to  connect the stepper or servo motor driver modules to your

computer's parallel printer port is detailed in the drawing HOOKUP1.2D

on the DANCAD3D v2.6 shareware disks and elsewhere in this document.

DANCAM and DANPLOT work by reading line segment data from  drawing

files and sending signals, corresponding to the lines, to motor driver

modules  in such a way that the modules make motors rotate to move the

[6]

[                         A B S T R A C T                            ]

tool or work-piece through motions that are  like  the  lines  in  the

drawing  file.  The  signals  from  the  parallel  port are in "step &

direction" form, which means that for each axis one wire pulses on and

off to make  the  motor  move  one  step  (generally  1.8  degrees  of

rotation)  and the direction signal is set to on or off (the direction

signal pin changes just before the  step  signal  pin  pulses  if  the

direction reverses, otherwise the direction signal pin stays on or off

while  the  step  signal pin pulses) to tell the driver module to move

the motor clockwise or counterclockwise.

The drawing file (the tool path file) to be  read  by  DANCAM  and

DANPLOT  can  be  made  (drawn)  by  using DANCAD3D.COM v2.6.  DANPLOT

v2.60+ can also read some HP-GL type files made by  raster  to  vector

conversion programs, and other CAD type programs.

You  can  attach stepper or servo motors to almost any 2 or 3 axis

machine tool or plotting type device's hand wheel (crank, knob) shafts

to automate it.  Additionally DANPLOT can be  used  to  drive  4  axis

devices  where  the forth axis (the "C" axis,  tangential knife) keeps

the cutting edge of a saber  saw  or  knife  blade  pointed  into  the

direction  of the tool motion.  You can build your own special purpose

automated tools, retrofit manually operated machine tools,  or up-date

of old NC tools to operate directly from your computer.

DANCAM  and  DANPLOT  offer  very  high possitional accuracy (when

properly installed) and can be used for  applications  requiring  fine

accuracy  to points spaced large distances (the actual accuracy of the

cutting tool location depends on the mechanical quality of the  device

you are automating, e.g. flexing in the tool post and play in the ways

may  cause  the  tool  to  be  slightly  misplaced,  if  the  error is

repeatable you  can  adjust  the  tool  path  file  with  DANCAD3D  to

compensate for the mechanical errors.)

Because  DANCAM  and  DANPLOT  are  general purpose Computer Aided

Manufacturing (CAM) programs you can use them to automate most if  not

all  of  the  types of machine tools you use.  Additionally because of

the extreme low cost  and  high  repeatable  accuracy  of  DANCAM  and

DANPLOT  you  may  become  interested  in  automating  tasks  that are

currently done with manual hand labor.  Additionally  DANPLOT  can  be

used  for  partial automation in which the work-piece is automatically

positioned and the tool is raised and lowered manually (for drilling.)

DANCAM and DANPLOT can be operated automatically  from  DOS  batch

files.  Since  DANCAD3D can also be operated from a DOS batch file you

can write a DANCAD3D  macro  to  make  your  own  user  interface  for

manufacturing (e.g.  you would end the macro with the HALT command and

save the tool path with a standard file name that would always be used

by the batch file as the file to execute in DANCAM or DANPLOT.)

DANCAM and DANPLOT v2.6 have the ability to  save  a  "tool  path"

file  of  the  coordinates of end points of line segments by using the

"teach" mode and "jogging" the  tool  around  with  the  joy-stick  or

cursor keys.  This teach mode can be used to make fine measurements of

[7]

[                         A B S T R A C T                            ]

parts, or to manually trace a part to make duplicates.

[8]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

QUICK DISCUSSION OF IDEA

Two manufacturing programs are supplied,  DANCAM and DANPLOT.  The

user interface and the  way  the  programs  operate  is  similar.  The

difference  is  in  what  type of tool is to be automated.  One or the

other of these two programs (DANCAM or DANPLOT) can be used  for  most

types of tool.

STEPS TO CARRY OUT IDEA

1) Figure out whether you want to use DANCAM, DANPLOT or both.

2) Buy the parts needed to automate your machine.

3) Hook up your computer, driver modules, and attach the motors.

4) Install, configure, and use DANCAM and or DANPLOT.

DETAILED DISCUSSION OF IDEA

To  install  the  programs  you  can  copy  the *.EXE files onto a

formatted floppy disk or install (see INSTCAM.BAT) the  DANCAM/DANPLOT

floppy  disk(s)  to  your  harddisk.  Both programs have configuration

menus that let you adjust the programs to fit any size  machine.  Once

the  programs  are  configured  you  can operate the programs in three

ways: from their menus, from the DOS command line, or from a DOS batch

file.  On the DANCAM/DANPLOT disk there are several example data files

(*.ASC) you can use for testing (see below) the  programs.  The  macro

file  BIPOLAR2.MAC  can  be  run under DANCAD3D v2.6 to print out foil

patterns for making a stepper motor translator/driver printed  circuit

board.

ABOUT THE DANCAM AND DANPLOT DISK

The  DANCAM  and  DANPLOT  disk(s)  has  several  example files in

addition to the program files.  Please read the file CAMFILES.DOC  for

an updated description of the files on the disk(s).

If  you  wish  to run the programs DANCAM or DANPLOT from a floppy

disk you will need to make a copy of  the  DANCAM.EXE  or  DANPLOT.EXE

files and then copy any drawing data files (*.ASC) that you might want

to use.

EXAMPLE: C>COPY A:*.EXE B:

C>COPY A:*.ASC B:

There  needs to be some free disk space on the disk you run DANCAM

or DANPLOT on so that DANCAM and  DANPLOT  have  room  to  save  their

configuration  files  i.e.  DANCAM.CFG and DANPLOT.CFG.  When you copy

the configured versions of the programs to another  disk  remember  to

copy  the  *.CFG  file  along  with  the  *.EXE  files.  Note that the

configuration file is  not  supplied  on  the  program  disk,  but  is

[9]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

automatically  created  by  the  program  when you use the program and

configure the set up with the configuration menu in DANCAM or DANPLOT.

EXAMPLE: C>COPY A:DANCAM.EXE B:

C>COPY A:DANCAM.CFG B:

If you want to use DANCAM and DANPLOT from  a  harddisk  just  use

INSTCAM.BAT  to install the programs and files into sub-directory DC25

or DC25U26.

EXAMPLE: C>A:INSTCAM.BAT

To print out the shareware documentation  files  use  the  program

PRINTDOC.EXE  on  the  DANCAD3D shareware disks.  If your printer uses

the serial port use the DOS MODE command to configure your serial port

before trying to print the file.  PRINTDOC.EXE  will  print  the  file

centered  on  the  page (the leading spaces have been omitted from the

shareware documentation files to save disk space.)

EXAMPLE: C>C:\DC25U26\PRINTDOC A:\DCAM2OF2\CAMPLOT.DOC PRN

C>C:\DC25U26\PRINTDOC A:\DCAM2OF2\CAMPLOT.DOC LPT2

C>C:\DC25U26\PRINTDOC A:\DCAM2OF2\CAMPLOT.DOC COM1

The manuals in the user outfit have  more  information  than  this

CAMPLOT.DOC file,  but there is enough information here for you to get

the programs hooked up and operating.

FILES ON THE DISK

Here  is  a  brief  description  of  what  files   come   on   the

DANCAM/DANPLOT  disk(s)  and  how  you  would  use  them.   Additional

utilities for use by registered users are  on  the  DANCAD3D  outfit's

utility disk.

BIPOLAR2.MAC

If  you run this macro with DANCAD3D v2.6 five DANCAD3D ASCII type

drawing files will be  produced  that  you  can  print  out  by  using

DANCAD3D  to  build a Bi-Polar stepper motor translator/driver module.

Additional information is included elsewhere in this documentation.

DANPLOT.EXE

This is a 2D motion control program that lets  your  computer  act

like  the controller in a plotter.  Additionally DANPLOT can control a

fourth motor to have the computer act like the controller in  a  vinyl

sign  cutting machine.  Other types of machine can be controlled,  see

below.  To run from DOS enter DANPLOT at the prompt.

[10]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

DANCAM.EXE

This is a 3D CAM system controller program file.  When  used  with

DANCAD3D  v2.6  this program will let you make any shape possible on a

three axis tool such as a vertical milling machine.  To run  from  DOS

enter DANCAM at the prompt.

DANPLOT.PLT

This  is  a  plotter  driver for use with DANCAD3D v2.6 to make 2D

plotter files that DANPLOT can  read  for  engraving.  The  difference

between  using  DANCAD3D's  plotter driver to make a drawing file,  or

just saving an ASCII file of the drawing is that  the  plotter  driver

will  support the line attributes such as thickness,  shape and style,

while the ASCII file will only plot simple lines.  Normally you  would

not  need  the  line  attributes  (except  color which DANPLOT uses to

control the feed rate,  and the ASCII file supports) so the ASCII file

is  the  normal choice.  In the event you need to make dashed lines or

use the other attributes I have included this plotter driver  to  fill

that  need.  Select  [H]ardcopy  [P]lot [I]nstall from DANCAD3D's main

menu to change the name of the output file created by the  DANPLOT.PLT

driver.  Select  [H]ardcopy [P]lot [P]lot from DANCAD3D's main menu to

use this driver to create the output plot file.  Line  attributes  can

also  be  plotted  from HP-GL type files that have the line attributes

expressed in PU and PD commands (DANCAD3D plots HP-GL files  with  the

attributes  expressed  as multiple line segments,  and so will work as

well as files made with the DANPLOT.PLT driver type files.)  Adjusting

the  scaling  factors  can  change the stroking pitch you get in thick

lines.

DANPLOT.OUT

This is an example 2D plotter file that  was  output  to  disk  by

using  DANPLOT.PLT driver with DANCAD3D's plotter driver command.  You

can execute this tool path by selecting menu option #2  then  #2  from

DANPLOT's main menu.

30FEEDS.ASC

This  is  an ASCII tool path file to test thirty feed rates.  What

it actually is,  is a drawing with 30 lines colored 1  through  30  so

that  you  can  test your feed rate to line color configuration.  This

tool path can be executed in either DANPLOT  or  DANCAM  by  selecting

menu option #1 from their main menu.

DANPLOT.ASC

This is an ASCII tool path to test DANPLOT's configuration to make

sure you have the motors hooked up so that the lettering comes out the

[11]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

right  way around.  And also to test if you have the steps per unit of

distance set properly,  i.e.  the circle should be as wide  as  it  is

high.  You  can  load any of the ASCII data files into DANCAD3D if you

want to see what they look like.  This tool path can  be  executed  in

DANPLOT by selecting menu option #1 from its main menu.

DANCAM.ASC

This  is an ASCII tool path to test DANCAM's configuration to make

sure you have the motors hooked up so that the cuts come out the right

way around.  And also to test if  you  have  the  steps  per  unit  of

distance  set properly.  You can load any of the ASCII data files into

DANCAD3D if you want to see what they look like.  This tool  path  can

be executed in DANCAM by selecting menu option #1 from its main menu.

CAMTEST.ASC

This  is  a simple Three-Dimensional tool path to test DANCAM hook

up such that you will see all three motors turn at once  and  reverse.

This  tool  path can be executed in DANCAM by selecting menu option #1

from its main menu.

BACKLASH.ASC

This tool path helps you to measure  and  compensate  for  machine

backlash.  You  would  need  to install a fine point pen or scribe and

mark a flat surface with this design.  The spacing of the  two  traces

shows  the  amount  of  backlash in your machine.  You can measure the

backlash in the marks and enter  the  value  found  into  DANCAM's  or

DANPLOT's  configuration  menu  to  compensate for the backlash found.

After you enter the value for the backlash you should re-execute  this

tool  path  to test that the proper value for the backlash was entered

into the configuration file.  Normally this file tests  the  X  and  Y

axis  and can be used with DANCAM or DANPLOT.  If you wish to test the

X and Z axis or the Y and Z axis you can rotate  this  file  with  the

[R]otate  command  in  DANCAD3D.  This  tool  path  can be executed in

either DANPLOT or DANCAM by selecting menu option #1 from  their  main

menus.

PLOTTEST.ASC

This  ASCII tool path is a torture test for pen plotters made with

DANPLOT.  By drawing triangles from different directions any  backlash

present  in  a  plotter  you have built will become apparent as double

images.  Use this tool path file in addition to BACKLASH.ASC  to  test

any  plotter  type  machines you may build.  Adjust the values for the

backlash compensation in DANPLOT's configuration menu until the double

images are reduced to the minimum possible with your  hardware  (on  a

perfect  machine  no  double  images  would  remain,  but there are no

[12]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

perfect machines,  so there will always be some mechanical errors that

will  cause  the tool to be slightly off position.) This tool path can

be executed in DANPLOT by selecting menu option #1 from its main menu.

DEMOCAM.BAT

This DOS batch file can be used to demonstrate running DANCAM from

DOS.  This file makes DANCAM execute the tool  path  file  DANCAM.ASC.

To  run  this batch file you would need to first install and configure

DANCAM,  and also hook up  the  motors  so  you  could  see  something

happening.  To  run  this  demo,  copy  all  DANCAM.???  files to your

working disk and enter DEMOCAM at the DOS prompt.

DEMOPLOT.BAT

This DOS batch file can be used  to  demonstrate  running  DANPLOT

from  DOS.  This  file  makes  DANPLOT  execute  the  tool  path  file

DANPLOT.ASC.  To run this batch file you would need to:  first install

and  configure  DANPLOT,  and also hook up the motors so you could see

something happening.  To run this demo: copy all DANPLOT.???  files to

your working disk and enter DEMOPLOT at the DOS prompt.

MANYPLOT.BAT

This  batch file can be used to repeatedly make identical 2D parts

from a given tool path file.  To stop making parts you  will  need  to

press  the [Ctrl] & [C] keys on your keyboard when you see the message

to ready the machine for another work-piece.  To use this  batch  file

you  will need to have an ASCII type tool path file,  a plotter output

file made in DANCAD3D, or a HP-GL file.  The batch file will pause for

you to put a new work-piece into the  machine  before  making  another

part.  The  character  after  the path file's filename is optional and

lets you select the file type,  i.e.  ASCII =  1,  DANPLOT.PLT  driver

plotter output = 2, HP-GL type file = H.

EXAMPLE: C>MANYPLOT YOURPATH.ASC 1

C>MANYPLOT YOURPLOT.OUT 2

C>MANYPLOT YOURFILE.HPG H

MANYCAM.BAT

This  batch  file  can  be  used  to  make  several  3D parts of a

particular tool path file.  To stop making  parts  you  will  need  to

press  the [Ctrl] & [C] keys on your keyboard when you see the message

to ready the machine for another work-piece.  To use this  batch  file

you  will  need  to have an ASCII type tool path file.  The batch file

will pause for you to put a new work-piece  into  the  machine  before

making another part.

[13]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

EXAMPLE: C>MANYCAM YOURPATH.ASC

CAMPLOT.DOC

This  is  the  shareware documentation for DANCAM and DANPLOT that

you can share with your friends.

CAMFILES.DOC

This is the one line file description for the DANCAM/DANPLOT disk.

You might want to get a file directory of the DANCAM/DANPLOT  disk  to

see  if  I  have  added  any  new  files  since this documentation was

written.

USING DANCAD3D AS A UTILITY

You should keep in mind that DANCAD3D can be used, as a utility to

manipulate tool path files,  from  a  DOS  batch  file  if  the  macro

DANCAD3D is running ends with the macro HALT command.

EXAMPLE: ; This is a DANCAD3D macro called DOWN005.MAC

LOAD ASCII YOURNAME.ASC

# 0 OFFSET C 0 0.005 0

# 0 SAVE ASCII YOURNAME.ASC

HALT

; End DANCAD3D macro.

EXAMPLE: REM This is a DOS batch file using DOWN005.MAC

DANCAM YOURNAME.ASC

DANCAD3D DOWN005.MAC

DANCAM YOURNAME.ASC

DANCAD3D DOWN005.MAC

DANCAM YOURNAME.ASC

When  you  use  DANCAD3D  macros to manipulate tool path files you

have a wide range of rotation,  scale change,  and position adjustment

options available.

OVERVIEW OF HOW DANCAM AND DANPLOT WORK

DANPLOT  and  DANCAM  are  two  programs  that you can run on your

computer to manufacture parts.  YOU DO NOT NEED to understand or  have

any  background in CNC or "G" codes to use DANCAM and DANPLOT.  DANCAM

and DANPLOT work in a SIMPLE AND DIRECT MANNER,  they simply move  the

tool  along lines and curves THAT YOU DRAW with DANCAD3D (or lines you

have in an HP-GL type file.) All you have to do is draw on  DANCAD3D's

drawing  screen a series of lines and curves in the order you want the

tool to trace out those motions and DANCAM or DANPLOT will FOLLOW  THE

[14]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

MOTIONS YOU HAVE DRAWN OUT.

(If  you  are  interested  in  having  DANCAM  or  DANPLOT support

execution of "G" or "M&G" codes send me an example  file  on  a  1.2MB

floppy disk,  as a "G" code text file, so I can see which commands you

would need supported.  I am interested in knowing how many users  have

an interest in a utility for conversion of "G" codes to something that

can be used with DANCAD3D or DANCAM/DANPLOT.  DANCAD3D v2.6A+ can SAVE

tool  paths  in M&G code file format for use with other types of NC or

CNC machine controller,  see the [F]iles  [S]ave  [I]ndustry  sub-menu

from DANCAD3D v2.6A+'s main menu.)

DANCAD3D and DANPLOT work by reading a disk file that contains the

end points of line segments and sending signals through the computer's

parallel printer port such that motors attached to your machine rotate

in  correspondence  to  the location of the points along lines between

the end points of the lines in the file being read.  The tool  in  the

machine is positioned by coordinates in the X,  Y and Z axis,  just as

the drawing cursor in DANCAD3D's workspace can be  positioned.  Curves

of  any complexity can be followed by making the line segments shorter

than the resolution (step size) of the motors driving the machine.

To use DANCAM and DANPLOT you simply go to a  motor  supply  house

and purchase some stepper motors and driver modules (or build your own

driver  circuit  board,  see  BIPOLAR2.MAC  on  DANCAM  shareware disk

DCAM1OF2.) You will also need a power supply (some driver modules  are

powered  from  the AC line and do not require a power supply) and a 36

pin mating connector for your parallel port cable (you can connect  to

the 25 pin connector on the parallel port directly if you want to, but

I  find  using  the printer cable easier.) Connect the motors to drive

your machine's lead screws with timing belts  and  pulleys  (or  shaft

couplings.)  Follow  the  connections  shown in the drawing HOOKUP1.2D

(found  on  the  DANCAD3D  shareware  disks),  or  use  the  port  pin

information in this documentation, and connect up the motors, modules,

and  relays  to  the  mating  36  pin  connector (note that use of the

information in this manual and the HOOKUP  drawings  is  at  your  own

risk,  Daniel  Hudgins  is not responsible for any damage or loss that

might occur.) Connect the mating 36 pin connector to the  printer  end

of  your  parallel  port printer cable.  Run DANCAM and or DANPLOT and

select menu option #4.  from the DANCAM or DANPLOT main  menu.  Answer

the  questions  in the configuration menu so that the motors will move

the proper amount when the program is used.  Press  [E]scape  to  exit

the configuration section.  Try using main menu option #3. to manually

move  the motors.  Use a dial gage if you have one to confirm that the

tool is moving the proper amount (when you test the movement make  the

tool  or work-piece move 0.25 inch two or three times in order to take

up the backlash in your machine, and ignore the first reading, you can

later correct for the backlash by entering the amount of  backlash  in

the configuration menu) Quit DANCAM or DANPLOT.

Now  that  you have an automated machine tool or plotter hooked up

to your computer you can use DANCAD3D v2.6 to  make  some  ASCII  type

data  files  for  tool  paths.  Run  DANCAD3D  and  select  DRAW  from

[15]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

DANCAD3D's main menu.  Draw the tool path.  Save the tool path element

as an ASCII type file (if you save the ASCII tool  path  to  a  floppy

disk you can take the drawing from your home or office computer to the

computer  in  the  shop  and read the drawing from the shop computer's

floppy drive or copy the file to the shop computer's harddisk and read

(execute) the drawing from the  shop  computer's  harddisk  (a  single

computer  can  be  hooked  up to many machine tools to have many parts

made simultaneously, or each machine can have its own computer and the

tool path file can be run  separately  and  or  concurrently  on  each

machine  tool.))  Note  that  you can transmit the tool path "file" to

your shop computer over a Modem,  or means  of  a  cable  between  the

serial ports of the computers. Quit DANCAD3D.

DANCAM  and  DANPLOT  v2.6+  have a teach mode that can be used to

create ASCII tool path files,  see their menu  option  #6.  The  teach

mode  has a curve fit option that allows you to mark a few points on a

surface and have the tool later follow a smooth  curved  line  through

the  points.  The teach mode can be used to "trace" objects to measure

or copy as well as mark out movements  without  a  work-piece  in  the

machine.  A  full  screen large digital read out (DRO) can be selected

to make measuring easier.  Note that in DANPLOT's  teach  mode  the  z

axis  position is ignored in the file made,  so use DANCAM if you want

to save the actual z axis value.

DANPLOT v2.6+ can directly read HP-GL commands PU, PD, PA, PR, and

SP,  and so can be used with HP-GL files created by CAD programs other

than DANCAD3D.  See DANPLOT main menu option #2, sub-menu #1.

To make parts,  turn on the power supply for the automation motors

(DANCAM and DANPLOT can control relays to turn on the motor power, but

for safety reasons you should have a  manual  switch  to  control  the

master motor power supply.) Run DANCAM or DANPLOT,  select menu option

#1.,  and enter the name  of  the  tool  path,  e.g.  SOMEFILE.ASC  or

A:\SOMEFILE.ASC, to execute (execute means trace out the path drawn by

moving  the  tool  or  work-piece  relative  to each other.) DANCAM or

DANPLOT should then make your part for you.  When  the  tool  path  is

finished  DANCAM  or DANPLOT will return the tool to home position and

be ready to make another part from the same tool path or another  tool

path file.  You can file the floppy disk copy of the tool path so that

if you ever need more parts,  or replacement parts, all you have to do

is shove the floppy disk in the computer,  and  enter  the  tool  path

name,  to  make  another  identical  part.  If  you have manufacturing

stations around the world you can send tool paths on floppy disks  (or

by  Modem)  so  that  the  remote stations can make up parts on demand

locally and reduce your shipping and inventory costs.

DOS batch  files  can  be  used  to  operate  DANCAM  and  DANPLOT

automatically  to  make  several parts.  The machine operator DOES NOT

NEED TO USE ANY OF THE MENUS  in  DANCAD3D,  DANCAM,  and  DANPLOT  to

manufacture  parts.  The  machine operator does need to understand the

meaning of some of the messages that come on the screen (such  as  the

DOS  PAUSE  command  telling  him to press [Any-Key] when ready) or at

least  know  enough  to  alert  a  more  knowledgeable  operator  when

[16]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

something  goes  wrong.  The machine operator should also be told that

the power driven tool can move unexpectedly and  hurt  him,  therefore

THE  POWER  TO  THE  MOTORS COILS SHOULD ALWAYS BE SWITCHED OFF BEFORE

EXPOSING HIMSELF TO THE EQUIPMENT (do not design equipment  such  that

the motor needs to be on to keep the load stationary, e.g. do not hang

a  weight  from  a  pulley on the motor shaft!) Safety switches on the

equipment access covers to turn off the motor coil power are advisable

to help reduce injuries (to maintain possitional  accuracy  the  logic

power  to the motor driver modules should not normally be switched off

in the middle of a tool path execution,  so when hooking up DANCAM and

DANPLOT you should try to wire up two separate power switches: one for

the motor coil power,  and the other for the motor driver module logic

power.)

To summarize, using DANCAM and DANPLOT takes these steps:

PHASE ONE: HOOKING IT UP

1. Mount automation motors and switches mechanically to machine.

2. Hoop up electrical connections to modules and switches.

3. Configure DANCAM and DANPLOT to fit your mechanical system.

4. Test the installation to make sure the motors are working.

PHASE TWO: COMPUTER AIDED DRAFTING

1. Use  DANCAD3D to draw tool paths for the tool paths needed.  DANCAM

and DANPLOT v2.6 can also create ASCII  files  with  their  "teach"

mode  (option  #6  from their main menus) by using the joy-stick or

cursor keys, or entering a series of "go-to" point movements.

2. Save the tool paths to floppy disks in ASCII file type.

3. Write a  DOS  batch  file  to  control the manufacture of the parts

needed,  and save the batch  file  to  the  floppy  disk  with  the

corresponding tool path files.  Set up the AUTOEXEC.BAT on the shop

computer  so  that  the  user will be prompted to insert the floppy

disk with the tool path and batch file to  control  the  number  of

parts  to  make.  For  safety,  the  batch  file  should prompt the

operator to turn the motor power on  and  off  at  the  appropriate

points in the manufacturing cycle.

PHASE THREE: MACHINE OPERATION

1. Turn power on to the shop computer.  The AUTOEXEC.BAT file you made

will  automatically  prompt  the  operator  to insert the tool path

disk.  The AUTOEXEC.BAT file then runs the batch file on  the  tool

path  floppy  disk  (in order to have the AUTOEXEC.BAT file run the

batch file of the  floppy  disk  you  would  always  use  the  same

filename for the batch file on the floppy disk, e.g. GO.BAT.)

2. Operator inserts the work-piece and turns the motor power on.

3. Operators presses [Any-Key] to start making a part.

4. Operator is prompted to turn the motor power  off,  insert  another

[17]

[    O V E R V I E W   O F   D A N C A M   A N D   D A N P L O T     ]

work-piece, and turn the motor power back on.  (See MANYCAM.BAT and

MANYPLOT.BAT for example batch files of this type.)

5. The  manufacturing cycle continues under the direction of the batch

files on the tool path floppy disks (i.e.  GO.BAT  can  prompt  the

user to insert further floppy disks and run sub-batch files.)

Phase  three is of course not necessary if you are going to be the

operator of the machine.  In the case that you are doing all the  work

yourself  you  would  save  the  tool  path file on your harddisk from

DANCAD3D and then run DANCAM or DANPLOT,  turn on the  power  for  the

machine,  and  enter  the  filename of the tool path you saved to your

harddisk.

[18]

[                  P R O G R A M   F E A T U R E S                   ]

QUICK DISCUSSION OF IDEA

DANCAM and DANPLOT are designed to be adaptable to a wide range of

uses.   The  range  of  features,  of  the  programs,  allows  several

arrangements  of  motors  and  relays.   The  ability  to  select  and

configure the programs to work with different hook up options lets you

select the most cost effective approach.

STEPS TO CARRY OUT IDEA

1) Figure out how many axis will need to be used.

2) Select DANCAM for 3 axis 3D work, DANPLOT for other jobs.

3) Review the parallel port pin hook-up information in this document.

4) Configure, DANCAM or DANPLOT to match your hook-up choice.

DETAILED DISCUSSION OF IDEA

DANCAM and DANPLOT both have several features that make setup  and

operation simple and relatively easy to accomplish:

* All commands displayed on screen.

* Menu driven configuration is adjustable to any units.

* Works with tool paths drawn in metric, inch, or other units.

* Motor testing utilities to check maximum motor speed under load.

* Jog menu lets you position tool manually and has position read out.

* Can be paused while making parts from keyboard or switch on port.

* Tool position can be adjusted while the tool path is executing.

* Supports up to four auxiliary relays to control anything.

* Relay default state is user selectable.

* Has automatic or manual home up to home switches.

* Will return to starting point even if home switches are not used.

* Limit switches can be used to protect against out of range movement.

* Can be operated from DOS command line automatically.

* Can be automatically operated from DOS batch files.

* Uses user accessible DANCAD3D ASCII data file type.

* Overall scaling factor to adjust part size.

* Adjusts to any pitch lead screws or drive chain.

* High accuracy over large distances of tool travel.

* Works with any motor driver using step pulse and direction signals.

* Feed rates can be installed from rapid movement to very slow.

* Feed  rate  change,  auxiliary  relay  toggle,   and  pause  can  be

controlled automatically from line colors in the tool path file.

* Installable delay after relay toggle lets equipment settle down.

* Automatic backlash compensation when needed.

* DANCAM is fully three dimensional and can make any three axis shape.

* DANPLOT supports HP-GL files, and tangential knife motion.

* DANCAM and DANPLOT will run on almost any low cost PC compatible.

* Low cost simple connection to parallel port, LPT1, LPT2, or LPT3.

* Joy-stick control and large digital read out in jog and teach modes.

* Teach mode allows manual tool motion to be automatically repeated.

[19]

[                  P R O G R A M   F E A T U R E S                   ]

When run on a 10 MHz 80286 computer DANCAM v2.60 and DANPLOT v2.60

can  produce  about  3420 step pulses per second which would run servo

motors with 200 steps per revolution at a speed  of  about  1026  RPM.

Most  stepper  motors  have  a reliable top speed less than 200 RPM so

when stepper motors are used the  reliable  operation  of  the  motors

limits the top speed obtainable, that is, DANCAM or DANPLOT are faster

than needed and do not limit the maximum speed when stepper motors are

used.  The  programs feature a rate multiplier can be used to increase

the step angle for servo drives (i.e. to 3.6 degrees) to get speeds of

2000 rpm or more since servo motors are normally drive through  a  6:1

or  8:1 reduction before driving a 5 tpi lead screw,  giving about 300

RPM at the lead screw or a feed  of  60  inches  per  minute.  (Future

versions  will  hopefully  be  able  to  run somewhat faster,  but the

performance of v2.60 is quite  adequate  for  normal  uses.)  Using  a

computer with a faster clock,  i.e.  > 10 MHz, should result in higher

maximum speeds.

[20]

[                      A P P L I C A T I O N S                       ]

QUICK DISCUSSION OF IDEA

DANCAM and DANPLOT are designed to be adaptable to a wide range of

applications.  This section will give you some idea how you might  use

DANCAM or DANPLOT to do your tasks automatically.

STEPS TO CARRY OUT IDEA

1) Review the hook-up information in this documentation.

2) Think of how you currently do your work.

3) Think of how you might use DANCAM or DANPLOT to do that work.

4) Remember that any axis can also be used for rotary motion as well.

DETAILED DISCUSSION OF IDEA

DANCAM  and  DANPLOT  are similar programs,  but each one has been

optimized  for  different  applications.   DANCAM  is   designed   for

applications  that require the tool or actuator to move between points

at any location in three dimensions.  DANPLOT is optimized for working

on flat material, e.g. sheet or plate, and will automatically lift the

tool or turn off the cutting flame,  or laser beam,  before moving  to

the  next point in the tool path (drawing file.) In other words DANCAM

is used for applications like 3D milling,  and  DANPLOT  is  used  for

plotter like work such as engraving and torch cutting.

Both  DANPLOT  and DANCAM can draw circles and curves of any type.

DANCAM can manufacture any part that can be produced on a  three  axis

machine  tool.  The  production  of  circles and curves takes place in

DANCAD3D as part of the preparation of the drawing of the  tool  path.

For  complex  contoured surfaces the tool path must define points that

are more closely spaced than for the tool paths  of  parts  than  have

flat  surfaces.  The  size of DANCAD3D v2.6's workspace is much larger

than previous versions,  and so can work with very complex curves  and

contoured  surfaces  (so  long  as  you have enough free space on your

harddisk for the workspace to expand.) DANCAM  and  DANPLOT  can  read

tool path files of any length.

DANCAM  and  DANPLOT  can  also use the COM1,  or COM2 port of the

computer as the file name  of  the  file  to  execute.  The  DOS  copy

command can be used to send (transmit) the tool path from one computer

to  another.  You  can also just save the ASCII file to the "filename"

of the serial port to transmit the tool path to the machine's computer

(where the path "file" data would be read by DANCAM or DANPLOT as  the

data  comes  in,  or stored on the machine's computer's harddisk,  and

then executed from the harddisk's stored file.)

EXAMPLE: C>REM On the sending, office, computer.

C>C:\DOS\MODE COM1:9600,N,8,1,P

C>COPY MYPATH.ASC COM1 /B

[21]

[                      A P P L I C A T I O N S                       ]

EXAMPLE: A>REM On the receiving, shop, computer.

A>MODE COM1:9600,N,8,1,P

A>DANCAM COM1

In the above example I assumed that the DOS MODE command would  be

copied  onto the working DANCAM floppy disk in the A:  drive.  At 9600

BAUD acceptable speed can be achieved for milling  and  other  not  so

fast  tasks.  You  do not actually need to exit DANCAD3D to transmit a

tool path to the shop computer,  just use COM1 as the name of the file

to  save  when  you select [F]iles [S]ave [A]SCII from DANCAD3D's main

menu (provided you used the DOS MODE command to set up the serial port

before  you  entered  DANCAD3D.)  The  DANCAD3D  CAD  manual  has  the

instructions  for  making the NUL MODEM cable required for the hook up

of two computer's serial ports.

If your shop computer does not have a harddisk you can use a print

spooling program to send a tool path file one line at a  time  to  the

shop  computer  to  execute while you use the sending computer to draw

other tool paths.

If you wish to clear the bits on  your  parallel  port  when  your

computer boots you can put DANCAM or DANPLOT in your AUTOEXEC.BAT such

that the program will exit back to DOS.

EXAMPLE: DANPLOT NUL 0

DANCAM NUL 0

If  you  use a MODEM on each end of the serial port cable in place

of the NUL MODEM cable you can have  the  manufacturing  machine  tool

located at a great distance.  In fact you could send your tool path by

telephone  line  across  the  world  to control a distant tool in real

time.  For sign shops or other types of field  work  the  part  design

could  be  done  on a portable computer at the clients site.  The tool

path could be transmitted over the phone (by modem)  to  the  shop  so

that  the  part  would  be  able  to  be  picked  up  as  finished and

immediately  delivered  and   installed.   By   using   DANCAD3D   and

DANCAM/DANPLOT  no  dimensioned  drawings  on paper or skilled machine

operators are needed  between  the  creative  design  and  the  actual

manufacturing of the finished part.

An  example  batch  file  to  store  and  execute a tool path file

transmitted over the serial port would look like this:

EXAMPLE: REM Batch file on receiving computer.

C:\DOS\MODE COM1:9600,N,8,1,P

:LABEL

COPY COM1 TEMP.ASC /B

DANCAM TEMP.ASC

GOTO :LABEL

DANCAD3D has special macro commands  that  can  produce  swept  or

contoured surfaces, e.g.  EXPAND and BETWEENS.  The macro commands for

[22]

[                      A P P L I C A T I O N S                       ]

DANCAD3D are explained in detail in the DANCAD3D outfit.

APPLICATIONS DANCAM CAN BE USED FOR

DANCAM  can  be  used  to move the tool or work-piece along a path

that requires synchronized, and simultaneous, motion of feed motors on

three axis.  DANCAM is capable of making contoured surfaces  when  the

tool  path  file  has be properly prepared.  If some inverters and AND

gates are connected to the auxiliary relay outputs it is  possible  to

control  more  that three motors with DANCAM,  such that only three of

the motors are turning at once (e.g. any three of six.)

MAIN USES OF DANCAM

* Three axis fully Three-Dimensional vertical milling.

* Operation of lathes.

* Cutting foam and other materials with hot wires.

* Flame cutting where the torch height needs to be adjusted.

* Laser cutting where the laser focus needs to be adjusted.

* Drilling holes to varying depths.

* Measure 3D shapes using jog or teach modes.

ADDITIONAL POSSIBLE USES FOR DANCAM

* Spray application where the sprayer moves in three dimensions.

* Glue application where the applicator moves in three dimensions.

* Welding, spot or arc.

* Parts insertion, screw insertion, or stapling and nailing.

* Marking with an automatic ink stamp as the tool.

* Automated warehouse loading and unloading, e.g. overhead winch.

* Three axis motion control Cine "FX" photography.

* Automation of a motion picture animation stand.

* Dipping of parts for coating by use of a gripper.

* Candle making.

* Processing film, PC boards, or micro chip wafers, using a gripper.

* More than 3 axis work where only three axis turn at once.

If a processing robot is to be  constructed  the  gripper  can  be

controlled  by  one of the control relays.  You would draw the part of

the path where the gripper will hold in a selected color, and the part

of the path where the gripper is free in another selected  color.  The

timing  of the processing operation could be controlled by setting the

feed rate and distance moved to come out to the time required for  the

processing  step.  By  drawing  zigzag  lines in the tool path,  parts

being processed in solutions could  be  agitated.  Additional  control

relays  could be used to turn on or off pumps to operate drying air or

wash sprayers.  The auxiliary control relays can also  open  or  close

solenoids on fill and drain valves to change solutions.

In  addition  to  the  linear  three axis applications noted above

[23]

[                      A P P L I C A T I O N S                       ]

DANCAM can be used to have one or  more  axis  move  in  a  non-linear

manner.  For  instance  the X axis could control a rotating table,  so

when the drawing for the motion of the X axis is drawn  with  DANCAD3D

the person drawing the tool path would use units of arc (i.e. 0 to 360

degrees)  rather  than  units  of distance (e.g.  0.001 inch as +/- 10

inches.)

For applications involving beams of light the stepper  motors  can

rotate  minors to deflect and scan out the beam,  rather than use lead

screws to move the laser around at right angles to the work-piece.

For lathes the third axis in DANCAM can be  used  to  control  the

spindle  speed  by connecting a small stepper motor to the shaft of an

SCR speed control for the spindle motor  (the  stepper  motor  on  the

speed  control  knob  would  turn  less  than 270 degrees and would be

motionless much of the time to keep the spindle  speed  constant.)  In

DANCAD3D,  when drawing the tool path, you would need to incline lines

drawn on the third axis of the tool path,  such that the stepper motor

on  the  spindle  motor  speed  control  will achieve constant surface

cutting speed.  To know how much to incline the tool  path  lines  you

would  need  to work out a table of numbers that equate the degrees of

rotation on the third axis stepper motor attached to the spindle speed

control shaft to the actual  spindle  speed  under  the  load  of  the

cutting tool (you might need to draw a curved line in the tool path to

get the desired result.) The automatic adjustment of the spindle speed

allows  your  lathe's  cutting tool to maintain constant surface speed

such that as the tool reduces the radius  of  the  part,  the  spindle

speed  would  be made to go faster.  If the spindle speed control only

has two positions such as fast and slow one of  the  auxiliary  relays

could  be  used in place of the third axis motor to switch the spindle

speed.

For thread cutting on a lathe,  a large stepper or servo motor can

be geared to the spindle in place of the spindle motor (friction drive

will not work since the belt will slip.)  In  DANCAD3D's  workspace  a

helix shaped thread on the lathe would be drawn as a straight diagonal

line, with the slope of the diagonal line determining the pitch on the

thread.  So,  in  the lathe application you could have two linear axis

and one rotary axis installed on the lathe.  If you adapt  your  lathe

to  have  a  variable  speed  motor and also the pulleys for a stepper

motor on the spindle you can do both synchronized  and  unsynchronized

turning.  Having  the  spindle  unsynchronized  makes drawing the tool

path simpler, but does not allow for turning threads.  You should note

that DANCAM can mill inside or outside threads by using  helix  shaped

tool path and the appropriate shape cutter.

If  you  use  a stepper motor to rotate the spindle of a lathe for

thread cutting and have a maximum spindle RPM less than that  required

for  a stationary cutter in the tool post,  you can still cut accurate

threads by using a tool post grinder either with an  abrasive  cutter,

or with a cutter like a gear tooth milling cutter and reduced speed of

the tool post grinder motor.

[24]

[                      A P P L I C A T I O N S                       ]

When  DANCAM  is used for plotter like applications the third axis

motor can be used,  as described  above  for  the  SCR  spindle  speed

control on a lathe, to adjust a valve or other control to regulate the

flow  of  gas  to a torch,  or the flow of liquid in high pressure jet

cutting.  Other uses for the third axis  could  be  control  of  light

intensity,  ultra-sound amplitude or frequency, heater temperature, or

cutting fluid flow rate.

THREE AXIS APPLICATIONS WHERE DANPLOT CAN BE USED

* Building a large pen plotter to plot drawings full life size.

* Engraving scales and lettering.

* Cutting fabric or leather from a pattern.

* Drilling printed circuit boards.

* Plotting PWB circuit traces with resist or conductive ink.

* Drilling holes to constant depth, slow down & fast up motion option.

* Flame cutting with automatic flame control on the Z axis.

* Laser cutting automatic laser on and off control on the Z axis.

* Automated routing of wood for cabinets and stair runners.

* Stamping or punching of sheet metal or other material.

* Glue, Grease, Potting compound, or Oil application.

With DANPLOT the third (Z) axis can be used in several  ways:  you

can  control  a  stepper  motor,  you can control a relay,  or you can

control a solenoid.  The normal use of the stepper  motor  for  the  Z

axis  would be to raise and lower the tool,  such that the tool is not

in contact with the  work-piece  when  the  tool  moves  to  the  next

starting point if the line segments in the drawing file being executed

to  not  touch.  Additionally  the Z axis stepper motor can be used to

open or close a valve or rheostat.  A relay on the  Z  axis  direction

control  signal can be used to turn a laser or other electrical device

on or off.  A solenoid triggered by the Z axis direction could be used

to:  stamp or punch the sheet material, actuate a valve to control gas

or  liquid,  close the jaws of a spot welder,  or operate a gripper to

grasp objects to move.

FOUR AXIS APPLICATIONS WHERE DANPLOT CAN BE USED

* Knife cutting of signs out of sheet vinyl.

* Saw cutting of wood with a Saber, Band or Scroll saw.

* Cutting fabric or leather from a pattern.

* Broaching of key ways, groves, or gear teeth.

* Automating nibbling tools.

The forth or "C" axis (the C axis is rotation around the  Z  axis,

such  that  the cutting edge of the tool is parallel to the Z axis) in

DANPLOT will rotate the cutting edge of a knife or recriprecating  saw

so  that  the  cutting  edge  always  faces  into the direction of the

motion,  of the tool,  as the tool moves  through  the  work-piece.  A

tolerance on how large a C axis angle change, between consecutive line

segments in the tool path file,  is required before the tool will lift

[25]

[                      A P P L I C A T I O N S                       ]

is available to speed the cutting of curves and circles  (normally  in

knife  cutting no harm comes if the knife rotates in the work-piece as

circles are cut out,  but you will want (and DANPLOT will  perform)  a

tool  removal,  tool  rotation,  and  tool  insertion when large angle

changes (such as at a 90 degree corner) need to be made.  The use of a

knife as the cutting tool connected to the C axis motor  is  sometimes

referred to as plotting with a tangential knife.  You should set the C

axis  angle  tolerance  before  lift  to  about  15  degrees  for most

applications.

AUTOMATING FIXED PATTERN MACHINES

DANCAM or DANPLOT can be used to automate manufacturing operations

where the tool path is infrequently altered.  Simple  operations  such

as  winding  tape on a composite beam (or pipe),  or winding wire on a

coil form can be done with  repeatable  accuracy  when  controlled  by

DANCAM  or  DANPLOT.  In  such a wrapping task one stepper motor would

rotate the form and the other stepper  motor  would  move  the  supply

spool  guide.  The  tool  path  for such a simple task would be just a

diagonal line, with the slope of the line determining the pitch of the

wrap, and the length of the line determining the number of turns.

Other related tasks would be to build a machine that  manufactures

a  single  part  like  making  pool cues,  table legs,  baseball bats,

bobbins,  and other mass produced shapes.  Users have  also  used  the

programs  to  cut  materiel to length and de-bur castings.  DANCAM and

DANPLOT are so inexpensive to use that tasks that would have been done

other ways in the past,  now can be  done  using  computer  controlled

automation.

[26]

[                G E N E R A L   I N F O R M A T I O N               ]

QUICK DISCUSSION OF IDEA

To  use  DANCAM or DANPLOT you need to have a tool path file.  The

tool path file consists of the  coordinates  of  end  points  of  line

segments.  Curves and such shapes are produced by groups of short line

segments, the line segments can be shorter than one motor step, and so

do  not  limit the smoothness of curves.  You can use DANCAD3D to make

the tool path file.  DANCAM or DANPLOT can also be used to make a tool

path file.  Other programs that make HP-GL type files may also be used

to make the required tool path file.  DANCAM and DANPLOT read the tool

path file and drive stepper or servo motors through the parallel  port

of your computer.

STEPS TO CARRY OUT IDEA

1) Think through how the tool will need to move to make your part.

2) Draw out your tool path file with DANCAD3D's drawing editor.

3) Quit DANCAD3D.

4) Run DANCAM or DANPLOT.

5) Turn on the power to your machine

6) Enter the name of the tool path to execute.

DETAILED DISCUSSION OF IDEA

To use DANPLOT and DANCAM you will need to create a text file that

contains information on the end points of line segments, such that the

line segments define the path you want your tool (or work-piece as the

motion  between  the  tool and work-piece is relative) to move through

space.  The easiest way to create the necessary file of  line  segment

data  is  to  use  DANCAD3D to draw the line segments in the order you

want them to have and in the direction you want the motion to proceed.

The necessary line segment data file can also  be  made  with  a  text

editor or by a data file conversion program.

The format of DANCAD3D's ASCII file is formatted as ten numbers in

ASCII  floating  point  across  each  line  of  text.  The first three

numbers on a line of text are the X1,  Y1,  and Z1,  values for a line

segment.  The fourth,  fifth, and sixth numbers are the X2, Y2, and Z2

coordinates of the same line segment.  The seventh number is the  line

color.  The  remaining  numbers are line attributes not currently used

in the CAM programs and would be set to:  1 0 0.  The  first  line  of

text  in the ASCII file contains the word ENTER.  The last line of the

ASCII file contains  ten  spaced  zeros  to  flag  the  end  of  file.

Therefore  you see that the ASCII file has two lines of text more than

the number of lines segments described.  You can use the Write command

in DANCAD3D or any other text editor to look at any ASCII  data  file.

The values in the ASCII file are absolute position coordinates.

You  can write your own programs,  in any language,  to create the

ASCII files to use with DANCAM or DANPLOT.  DANCAD3D v2.6 can be  used

[27]

[                G E N E R A L   I N F O R M A T I O N               ]

to  edit  the ASCII tool path files.  DANCAD3D's macro language can be

used to create application programs to produce ASCII tool  path  files

without having to do any manual drawing.

The  overall  scaling  factor in DANCAM and DANPLOT can be used to

change the size of the part made,  e.g.  a scale of 2 makes  the  part

twice as large, a scale of 0.5 makes the part half as large.  Normally

the  "scale"  in  DANCAD3D is set to 240 when working in inch units so

drawings will print out at  the  proper  size,  but  in  DANCAD3D  the

display  scale  has  no  effect  on the values saved to an ASCII file.

ASCII files made in DANCAD3D only contain the  values  represented  at

the top of the drawing editor screen, that is, one workspace unit ends

up  as  1.0  in  the  ASCII  file.  The numbers in the ASCII file only

relate to amounts of distance by means of the numbers you  enter  into

DANCAM's  or  DANPLOT's  configuration  menus.  If  you have the motor

steps configured to 2000 steps per inch,  and the X axis value changes

one  unit then the X motor will move 2000 steps when the overall scale

in DANCAM or DANPLOT is set to 1.0.

The location of the 0 0 0 point can be  shifted  by  changing  the

location   of   the   home  point  values  in  DANCAM's  or  DANPLOT's

configuration menu.  For the backlash compensation  to  work  properly

you  need to have the home point at one "corner" of the working volume

(area.) Mirror images can be produced by  changing  the  sign  of  the

steps  per unit entered,  i.e.  -1000 in place of 1000 (the home point

value may need to be inverted as well, to keep the tool from going off

the end of the machine.)

The maximum speed the program sends pulses  to  the  motor  driver

circuits  needs  to  be  limited  to  the  speed  that  the motors can

physically keep up with,  and so the programs let you  enter  a  pulse

width   factor  (p.w.f.).   DANCAM  and  DANPLOT  have  motor  testing

utilities built in for determining the fastest speed  the  motors  can

reliably be used.

The feed rate can be automatically controlled by the line color of

line  segments in the tool path file.  The configuration menus let you

select the feed rate for different line colors.  Relays  can  also  be

set to turn on or off automatically when special line colors are read.

In  general  the  lower  you set the maximum resolution the faster you

maximum feed rate will be.

HOW TO USE DANCAM AND DANPLOT

In general the use of DANCAM and DANPLOT follows these steps:

1. Build  or buy a machine to automate.  Generally you will  want  the

lead screws that have about 5 threads per inch so that the  stepper

motors  200  or 400 steps per shaft rotation will give a resolution

of 0.001 or 0.0005 of an inch.  Chain  or  timing  cable  drive  in

place  of  a  lead  screw  is  an  excellent choice for drives that

require less resolution  and  faster  feed  rates.  Always  try  to

[28]

[                G E N E R A L   I N F O R M A T I O N               ]

minimize inertia in components that must be driven.

2. Install  stepper motors  on  your  machine.  Use  timing  belts  or

flexible  shaft  (bellows)  couplings  between the motor shafts and

lead screws to reduce noise and lost steps caused by  vibration  in

the  motor  coupling.  Stepper  motors  get  hot  (80C  to  85C) in

operation so proper mounting would allow  the  conduction  of  heat

from  the  motor  and  circulation of cooling air.  Used or surplus

stepper motors can be purchased for a fraction of what  new  motors

cost.  In general stepper motors should have a rated holding torque

that is at least FOUR TIMES THE TORQUE that will be required at the

operating  RPM (the torque of stepper motors falls off as the motor

turns faster,  so that a motor rated at 500  oz-in  holding  torque

might only have 200 oz-in at 60 RPM, and only 100 oz-in at 120 RPM,

with  the  torque  falling  completely  to  zero at 180 RPM.) Using

stepper motors that are two small, and or running the motors faster

than they can reliable go is the major  cause  of  problems  people

have  with stepper motors.  Also stepper motors need to be operated

at 4 to 8 times their rated voltage to give satisfactory results (a

constant current stepper translator module or resistors  in  series

with  the  motor  windings  are  required to prevent the motor from

overheating when operated at high voltages,  i.e.  48 to 96 volts.)

The  type  of constant current driver circuit used:  resistor,  bi-

level,  or chopper,  does not reduce the need for  the  motor  coil

power  supply  to be 4 to 8 times the motors rated voltage (holding

torque voltage),  a 5 volt motor needs at least a 35 volt supply to

operate well.

3. Buy   or  build  the motor driver modules and motor power supplies.

Stepper  motors generally operate with two coils (phases) on so the

current draw is double the rated current,  e.g.  three motors rated

at  2 amperes will require a power supply that can deliver at least

12 amperes.  The stepper translator/driver modules  you  will  need

are  available  from  your  local  motor distributor.  Most stepper

motor manufactures supply driver modules optimized  for  their  own

brand  of  motors.  Surplus or used stepper motors can be purchased

for a fraction of what new motors cost,  e.g.  500 oz-in motors can

be  found for less than $75.  If you want to use solid state relays

to control the spindle of your machine or  other  auxiliary  device

buy  solid  state  relays that are rated for input of 3 volts DC or

less since the parallel port cannot supply more than  3  volts  (if

more power is needed than the parallel port can supply a TTL buffer

chip  such  as  the  74H07 and pull-up resistors (approximately 330

ohm) can be used amplify  the  signal.)  You  can  build  your  own

stepper  driver  modules  to drive three,  five ampere motors,  for

about $100 or less depending on how fancy  you  want  to  get  (See

BIPOLAR2.MAC  for  some  PCB  foil patterns.) Ready made translator

modules cost from $50 to $600 per axis depending on the  power  and

speed required.

4. Use the port pin hook-up information to connect the  stepper  motor

translator driver modules to your computers parallel port.  If four

auxiliary  relays  are  to  be  used a 5 volt 500 milliampere power

[29]

[                G E N E R A L   I N F O R M A T I O N               ]

supply (about a $10 item) will be required for  the  pull-up  bias,

otherwise  the auxiliary C and D outputs can be used to pull-up the

home and limit switch inputs (X, Y,  and Z home switch inputs being

pulled up through 10K ohm resistors to auxiliary C output,  and the

pause/limit input being pulled up through a 5K ohm resistor to  the

auxiliary D output pin of the parallel port connector.)

5. Run   DANCAM  and  DANPLOT  and  install  the  values for speed and

resolution  that correspond to your machine.  Test the motors under

load to make sure they can keep up with  the  movements  they  will

need to make.  If the motors "lose steps" (that is slip and fail to

rotate  to  the proper position) under load use a larger Pulse With

Factor (p.w.f.) to give the motor time to complete one step  before

another  is  requested  (if a stepper motor loses steppes the rotor

will no longer have  the  proper  rotational  position  (until  the

motors  are  sent  home again which is a good reason for installing

the home switches.)) Reducing the motor current to 80% of the rated

current can sometimes reduce lost steps caused by resonance in  the

middle speed range.

6. Run DANCAD3D,   draw a tool path, and save the tool path drawing as

an ASCII data file (or a plotter driver output file if you need the

line  width feature for DANPLOT.) DANPLOT can read HP-GL files,  so

you can generate the tool path with  a  raster-to-vector  (scanner)

third party program or some other CAD program.

7. Run  DANCAM or DANPLOT,  select menu option #1,  and enter the file

name  of  the  ASCII  (*.ASC)  tool  path  file you just saved from

DANCAD3D.  If you did the installation  properly  your  first  part

should come out as designed.

[30]

[   D O S   C O M M A N D   L I N E   A N D   B A T C H   F I L E S  ]

QUICK DISCUSSION OF IDEA

DANCAM  and  DANPLOT  are designed to be able to be activated from

either the DOS command line,  from a DOS batch  file,  or  from  their

internal  menus.  This feature lets a expert set up the programs and a

less skilled operator, who will, use the programs by following prompts

echoed to the screen by a batch file the  expert  has  written.  Batch

files  also have uses in manipulating tool paths automatically so that

the machine can run several  tool  path  files  without  the  operator

having to enter commands.

STEPS TO CARRY OUT IDEA

1) Install and configure the programs.

2) Write a batch file to prompt the operator activate the programs.

3) Create tool paths files to be manufactured.

4) Run the batch file and follow the prompts.

DETAILED DISCUSSION OF IDEA

DOS  batch  files  are simply ASCII text files that have a list of

DOS commands in them such that DOS can read the batch file rather than

the keyboard to get your commands.  So anything you can enter from the

DOS command line can also be put in a DOS batch file.

EXAMPLE OF HOW TO RUN DANCAM OR DANPLOT FROM THE DOS COMMAND LINE

To operate DANCAM or DANPLOT from  the  DOS  command  line  simply

enter  DANCAM  or DANPLOT followed by a space and then the name of the

data file to be plotted and then followed by  another  space  and  the

number  1  (or  1,  2,  H  for DANPLOT.) The number following the data

filename is filetype code.  Code 1 is optional (defaults to ASCII.)

EXAMPLE: C>DANCAM A:TOOLPATH.ASC 1

C>DANPLOT A:TOOLPATH.ASC 1

C>DANPLOT A:PLOTTER.OUT 2

C>DANPLOT A:SOMEFILE.HPG H

C>DANCAM A:TOOLPATH.ASC

C>DANPLOT A:TOOLPATH.ASC

If you want to have one of the other menu options come up from the

DOS command line you can use the dummy filename NUL.

EXAMPLE: C>DANCAM NUL 3

C>DANPLOT NUL 3

C>DANCAM NUL 6

C>DANPLOT NUL 6

[31]

[   D O S   C O M M A N D   L I N E   A N D   B A T C H   F I L E S  ]

EXAMPLE OF BATCH FILES THAT AUTOMATICALLY RUN DANCAM OR DANPLOT

The following DOS batch file will make parts from data files saved

on floppy disks.  Note that all the data files should  have  the  same

name  (when  you  save  the  DOS  batch file you should always use the

filename extension *.BAT.)

EXAMPLE: :START

ECHO OFF

CLS

ECHO PUT TOOL PATH DISK IN DRIVE A:

PAUSE

DANCAM A:\TOOLPATH.ASC 1

GOTO :START

Since the floppy drives are somewhat slow you can get  faster  and

smoother  plots by coping the data file to a harddisk or RAM disk (the

delay for the floppy drive to come up to speed when data is being read

will cause the cutter to idle for a  moment  periodically,  a  problem

that  can  be  avoided  or  reduced  by  reading  the data file from a

harddisk or RAM disk.)

EXAMPLE: :START

ECHO OFF

CLS

ECHO PUT TOOL PATH DISK IN DRIVE A:

PAUSE

COPY A:\TOOLPATH.ASC C:\DC25U26\*.*

DANCAM C:\DC25U26\TOOLPATH.ASC 1

GOTO :START

You can have DANCAM automatically run after you quit  DANCAD3D  by

using  a DOS batch file.  To break a loop in a DOS batch file you need

to press [Control] & [C] (or [Ctrl] & [Break]) while the batch file is

operating (while you are between commands listed in the bath file.)

EXAMPLE: :START

CLS

DANCAD3D

DANCAM

GOTO :START

See also the descriptions of the utility programs SQUEEZE,  CHAIN,

OFFSET, and JOBDONE given in the user outfit, since these programs are

meant to be used in batch files along with DANCAM and DANPLOT.

[32]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

QUICK DISCUSSION OF IDEA

DANCAM  and  DANPLOT  will  run on almost any PC compatible with a

parallel port.  In order to automate your machine you will need to buy

or build some electronic parts and stepper (or servo) motors.

STEPS TO CARRY OUT IDEA

1) Install and configure the programs.

2) Figure out what kind of electronics you will need.

3) Purchase the motors and electronics.

4) Wire up the motor control circuits, and switches to your computer.

5) Test the motors, and adjust the configuration as needed.

DETAILED DISCUSSION OF IDEA

Although DANCAD3D requires a fast computer and  harddisk,  DANPLOT

and DANCAM can be used with a less expensive system.  One advantage of

using  a  less  expensive  computer  in  the  shop  with the automated

equipment is that if the shop computer is damaged you will not have to

spend much to repair it. 80286 mother boards are so cheep now there is

little reason to use an 8088.  An 8088  can  be  used  though  if  the

motors do not need to run faster than about 100 RPM.

DANCAM  and DANPLOT will work on almost any IBM (tm) compatible PC

that has one floppy drive and a IBM (tm) PC  standard  parallel  port.

No  special  hardware  is  required  for  the computer.  Many standard

stepper motor driver modules and solid state relays can  be  connected

directly to the parallel port.  The signals from the parallel port can

be boosted with inexpensive parts (e.g.  a 74H07 buffer chip) to drive

almost any other stepper driver  module  or  solid  state  relay  that

requires more power than the power the parallel port can supply.

Motor  driver modules requiring pulse CW and pulse CCW can be used

if some cheep TTL chips are wired up to convert the step and direction

signals into pulse CW and pulse CCW.

COMPUTER: Use a 100% IBM PC-XT or AT compatible.

PORTS:    One Parallel Port is required, i.e. LPT1, LPT2, or LPT3.

One or two Joy-Stick ports if joy-stick(s) is (are) used.

VIDEO:    Any 80 column by 25 line video text display.

MEMORY:   512KB RAM DOS system memory, 400KB or more free.

DRIVES:   1 floppy disk drive.  Reading the tool path data file from a

RAM disk can  avoid  periodic  short  interruptions  due  to

blocks of data being loaded from the data file.  If you have

640KB on the computer you are using for DANCAM or DANPLOT  a

[33]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

RAM  disk  program  to  make  a 128KB RAM disk from your DOS

system memory and have enough left over to run  DANPLOT  and

DANCAM  (you  would  copy the ASCII tool path data file into

the RAM disk,  and have DANPLOT of DANCAM read the tool path

from the RAM disk drive.) A RAM disk is created by running a

small utility program before you run DANCAM or DANPLOT.  RAM

disk utility programs are available from ShareWare dealers.

External  to  your  computer you will need to get some electronics

wired up so the programs can operate your machine.

WIRES AND CABLES

You will need a  parallel  port  cable.  If  you  have  a  printer

connected  to  your  computer  you can use that cable.  To connect the

wires to the translator modules you will  need  a  Centronics  36  pin

connector that will mate with your printer cable.  You can then solder

wires onto the Centronics connector rather than your printer cable and

that  will  allow  you  to  disconnect  the  printer  cable  from  the

Centronics connector at any time to use  the  printer  cable  on  your

printer (reboot your computer after using DANCAM or DANPLOT before you

use  your  printer.)  If you buy a printer switch box make sure all of

the needed pins are connected since some cheaper switch boxes may  not

connect all of the pins.

The  wire  going between the parallel port pins and the input pins

of the translator modules should  be  shielded.  Microphone  or  video

coaxial  cable  can  be  used with the shields of the two cables being

tied to common (one cable for the step signal,  and one cable for  the

direction signal.) Electro-magnetic noise can be a problem (making the

motors  step  or  reverse  incorrectly) because of a high current that

flows through the wires that connect to the motor power supply and  to

the motor coils themselves.  Always keep the step and direction cables

as  separated  and  as  far away from the motor coil wires as you can.

Putting a 2.2K ohm pull up resistor from the step and direction inputs

of the translator module to a positive five volt regulated supply  can

help reduce the input impedance of the translator module and therefore

the voltage of induced electro-magnetic noise.  Opto-isolators,  or LC

or RC low pass filters on the inputs of translator  modules  can  also

help reduce incorrect triggering of the translator modules since opto-

isolators  and filters reduce the input impedance.  To filter the step

and direction signals at the input of the translator module connect  a

1000  pf  capacitor  from  the  input  to  common,  connect a 2.2K ohm

resistor from the input to +5 volts,  and connect a 100  ohm  resistor

between  the input of the translator and the parallel port signal.  If

adequately shielded and filtered the parallel port to translator cable

can be as long as 50 feet.

If you use an extension cable (15 pin type) on your  joy-stick  be

sure that all conductors are used since the "throttle" control that is

used  on  some  joy-sticks  for the Z axis may not be connected on the

extension cable!

[34]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

Wire for connecting the motors should be stranded and large enough

to handle the current required, i.e. 14 to 16 gage.  The wires for the

limit and home switches can be rather thin (22 gage) since very little

current flows through the switches.

TRANSLATOR AND MOTOR DRIVER MODULES

You will need Stepper motor translator driver modules for  stepper

motors.  Hundreds  of  different driver modules are available to drive

different  types  of  stepper  motors.   You  will  want  TTL   signal

compatible  modules  that  have MOTOR STEP (pulse) and motor DIRECTION

(cw/ccw) inputs.  TTL signals logic level is low from 0 to 0.5  volts,

and high from 2.5 to 5 volts,  at about 5 milliamperes.  Since steeper

motors need to be operated at voltages above what the motors are rated

for the stepper motor translator module you purchase should offer some

constant current or current cutback feature to keep  the  motors  from

burning up when they stop turning (stepper motors get hottest when run

slowly  or are stopped.) Two types of translator module that give good

performance are Bipolar chopper and Bilevel  Bipolar.  Bipolar  drives

give  more  torque at medium speeds than Unipolar driver modules.  You

should check that  the  stepper  motors  you  are  going  to  use  are

compatible  with  the  driver type you want to use.  Regardless of the

type of module selected the top motor speed  will  be  less  than  the

motor's  best  if the motor supply voltage is less than five times the

rated motor voltage, e.g.  a motor rated at 5 volts will run well with

a  25  volt supply,  but a 25 volt motor will need a supply voltage of

125 volts to run well.  The maximum rated voltage of the motor  driver

module  of course limits the supply voltage you can use.  Modules also

have a rated range of motor coil current they  can  handle  (the  coil

current for the motors should never exceed the rated current!) See the

information  in  this document about BIPOLAR2.MAC if you want to build

your own translator driver boards.

SOLID STATE RELAYS

Solid state relays such as the HAMLIN model  7521D  or  equivalent

can  be  used  to control auxiliary devices directly from the parallel

port.  The solid state relay selected should be able to operate from a

3 volt or smaller signal.  Solenoid relays can be controlled from  the

parallel  port  if  you  use  a Darlington amplifier transistor and DC

power supply.

POWER SUPPLIES

External power supply.  You will  need  a  power  supply  powerful

enough to drive all the stepper motors you are using.  For medium duty

tasks this would be 20 to 50 amperes at 35 to 96 volts (stepper motors

are run at voltages about 4 to 8 times their rated voltage,  so if you

have motors rated at 10 volts you would want to have  at  least  a  50

[35]

[              H A R D W A R E   R E Q U I R E M E N T S             ]

volt  power  supply  (how  high  the power supply voltage can be is of

course restricted by the maximum voltage of the translator modules you

will be using,  which is why you see stepper motors rated with odd low

voltages,  like a 3.4 volt motor that will be driven by a driver rated

at 24 volts.)) To determine the total current required figure that two

coils will be drawing current in each stepper motor so if your  motors

are  rated  at 1 ampere per coil and you have 3 motors you will need 6

amperes.  To assure that the motors can get all the current they  need

while  stepping  I  recommend that you put a 20000 mf capacitor across

the power supply input of each of your motor driver modules.  A 5 volt

500 ma power supply may also be required to bias any pull-up resistors

required for the parallel port connections.

HOME, PAUSE, AND LIMIT SWITCHES

Three Normally Open (N.O.) micro switches for home  switches.  Six

Normally  Closed  (N.C.)  micro  switches  for  limit  switches.   One

Normally Closed (N.C.) toggle switch for pause  switch.  One  Normally

Open (N.O.) push button for bypass button.  Three optical switches may

also  be required for critical applications,  to be put in series with

the home micro switches (the optical switches would be triggered by  a

rotating shutter fixed to the motor shaft.)

RESISTORS

Five 2.2K ohm 0.25 watt, for pull-up of the switch inputs (pull-up

resistors connect from TTL logic IC's inputs or outputs to a plus five

volt power supply (the common from the five volt power supply would of

course  connect  to  the  logic common or ground.)) Since the switches

connect between the input pins of the parallel  port  and  common  the

pull-up  resistors are needed to insure that the input pins are always

at logic high (logic high is about 2.8 to 5.0 volts.) If  you  do  not

want  to  use  a  special  5 volt supply for the limit and home switch

inputs you can use 10K ohm 0.25 watt resistors from the input pins  to

auxiliary  outputs C and D (of course auxiliary C and D cannot then be

used for relays since they will need to be logic high all the time.)

MECHANICAL PARTS AND HARDWARE

To mount the motors you will need flexible shaft couplings  (metal

bellows  type)  or timing belts and pulleys.  It is important that the

coupling used on the stepper motor shaft will have about 0.5 degree of

give (elasticity) so that the motor can step easily and  smoothly  and

without excessive vibration.

[36]

[                 H O O K   U P   C O N E C T I O N S                ]

QUICK DISCUSSION OF IDEA

Hooking up DANCAM and DANPLOT involves making some connections  to

the connector on your computers parallel port cable,  or the connector

on the parallel port board itself.  You do not need to hook up all  of

the pins, only those that are needed for your application.

STEPS TO CARRY OUT IDEA

1) Review the drawing HOOKUP1.2D on the DANCAD3D disks.

2) Buy a connector to connect to your parallel port board or cable.

3) Attach wires to the correct pins on the connector.

4) Connect any pull-up resistors needed to a +5 volt supply.

5) Connect the signal wires to your modules, relays, and switches.

DETAILED DISCUSSION OF IDEA

The  connections  to  the parallel port of your computer can be as

simple as six wires when building a plotter, i.e.  X axis step, X axis

direction, Y axis step, Y axis direction, the Z axis direction for the

pen up and down solenoid,  and a common (the limit switch input  would

go to common if not used.) If you want to hook up home switches, limit

switches, a pause switch, relays and additional motors you can as well

just  by  adding  more  wires  on the parallel port connector.  Always

double check the accuracy of the connections before applying power  to

avoid any possible damage to your computer or other parts.

HOOK UP CONNECTIONS TO THE PARALLEL PORT

The  complete schmatic for the basic DANCAM and DANPLOT hook up is

provided in drawing HOOKUP1.2D on the DANCAD3D disks.  You  can  print

out  the  HOOKUP1.2D  drawing by using DANCAD3D.  Studying the HOOKUP1

drawing will make the information  in  this  this  section  much  more

understandable.   This  following  information  gives  all  the  basic

connections for using DANCAM and DANPLOT.

DANPLOT and DANCAM use the same basic connections to the  pins  of

the parallel port.  The basic connections to the 36 pin printer end of

the Centronics parallel port cable are:

[37]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

All  of  the  input pins must be "pulled up" to logic high through

2.2K ohm 0.25 watt resistors to a +5 volt regulated power supply  (you

should be able to find a +5 volt 500 milliampere power supply for less

than $10 (you can use three flashlight 1.5 volt dry cells in series to

get  about  4.5 volts,  but NEVER use more than 5 volts or less than 3

volts.)) ALWAYS check that the MINUS (-) end of the five  volt  supply

connects  to the COMMON point,  since reversed connections will damage

your parallel port and possibly other parts of your computer.  If  you

do  not  want to bother with a +5 volt power supply and do not need to

use the auxiliary outputs  "C"  and  "D",  you  can  usually  use  the

auxiliary  outputs "C" and "D" to pull up the switch inputs,  as shown

here:

[38]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

PULL UP FOR PINS: 10 & 32 - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - PULL UP FOR PINS: 11, 12, & 13

The value of the pull up resistor will need to be increased to 10K

ohm when auxiliary "C" and "D" are used as the pull up source to avoid

over loading the parallel port.  When the auxiliary input on pin 32 is

not used a 4.7K ohm resistor can be connected between pin 31  and  pin

10 in place of a 10K ohm resistor.

The home switches are normally open (N.O.) and connect between the

home  switch  input pins and the common (ground) point.  The six limit

switches,  and pause  switch,  are  normally  closed  (N.C.)  and  are

connected  in  series between pin 10 and the common point.  The bypass

push button for  the  limit  switches  is  normally  open  (N.O.)  and

connects  from  pin  10  to  the  common  point.  If  one of the limit

switches opens (due to out-of-range travel, the motors will stop,  and

an  error  message  will  come  up  on  the computer screen) press the

[Control] and [X] keys on the keyboard and then press the limit switch

bypass button you  wired  from  pin  10  to  the  common  point.  When

installing  the  limit  switches at the home end of the travel be sure

that the limit switches become open several motor steps AFTER the home

switches have closed (otherwise the motors will stop before  the  home

position is reached!)

The  WINDOW  command  in  DANCAD3D should be used to clip the tool

path before you save the tool path to an ASCII file to  avoid  out-of-

range  motions.  You  can write a DANCAD3D macro to clip all your tool

path files before you save them to disk.  The  values  passed  to  the

macro WINDOW command would be the maximum distance from the home point

in  DANCAM  or  DANPLOT such that no line segments would remain in the

tool path that could cause an out-of-range error.

[39]

[                 H O O K   U P   C O N E C T I O N S                ]

EXAMPLE: ; CLIPPATH.MAC, Example macro to clip tool path in DANCAD3D.

# 1 WINDOW_ELEMENT 20000 0 0 0 5 3 6

# 1 ERASE_ELEMENT

; End macro.

DANPLOT can  be  hooked  up  in  some  additional  configurations.

DANPLOT  can  drive four motors,  two for the X and Y movement,  the Z

axis motor to raise and lower the tool,  and a C axis  to  rotate  the

tool  to  point the cutting edge into the direction of movement of the

tool.  In order to use the C axis the  auxiliary  relay  "A"  and  "B"

output  pins  are used for the C axis step and direction signals,  and

the auxiliary input is used for the C axis home switch:

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

C AXIS HOME SWITCH INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - C AXIS DIRECTION OUTPUT

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - C AXIS STEP PULSE OUTPUT

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

You still have auxiliary outputs "C" and "D" available if you need

to have control relays and use a  +5  volt  supply  for  the  pull  up

resistors  on  the  switch  inputs.  So  the  C axis above hook up can

connect to four motors and two relays.

Another option when hooking up  DANPLOT  is  to  use  the  Z  axis

direction  bit  to  control  a relay or solenoid rather than a stepper

motor.  This option gives you the possibility of two motors  and  five

relays:

[40]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

AUXILIARY INPUT - 32 14

AUXILIARY RELAY "D" OUTPUT - 31 13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 30 12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 29 11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 28 10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 27 09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 26 08 - Z AXIS RELAY OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 22 04

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01 - AUXILIARY RELAY "C" OUTPUT

When using a relay on the Z axis you need to set the default state

for the Z axis direction bit and the rotation of the Z axis motion  so

that the relay is off before the tool path starts and is automatically

set off when the tool path ends, by using the configuration setup menu

option #4 from the DANPLOT and DANCAM main menus.  Normally this means

default  to  logic  low  and have the Z axis motion -10 to -200 steps.

The pulse width factor for the Z axis can be used to control the delay

after the Z direction bit changes before the X and Y motors  start  to

turn (to allow for the time the Z axis relay/device takes to respond.)

You  can  of course use the C axis motor and a Z axis relay if you

want to (three relays and three motors.) You can  omit  the  home  and

limit switches if you wish,  since both DANCAM and DANPLOT can operate

without the switches connected.  The minimum hook  up  for  DANCAM  or

DANPLOT running three motors would be:

[41]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

32 14

31 13

TIE TO COMMON - 30 12

TIE TO COMMON - 29 11

TIE TO COMMON - 28 10 - TIE TO COMMON

TIE TO COMMON - 27 09

TIE TO COMMON - 26 08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05

TIE TO COMMON - 22 04 - Z AXIS STEP PULSE OUTPUT

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01

Or  for DANPLOT the minimum hook up for two motors and an solenoid

would be:

Pin No.

36 18

35 17

34 16

TIE TO COMMON - 33 15

32 14

31 13

TIE TO COMMON - 30 12

TIE TO COMMON - 29 11

TIE TO COMMON - 28 10 - TIE TO COMMON

TIE TO COMMON - 27 09

TIE TO COMMON - 26 08 - Z AXIS RELAY OUTPUT

TIE TO COMMON - 25 07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 24 06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 23 05

TIE TO COMMON - 22 04

TIE TO COMMON - 21 03 - Y AXIS STEP PULSE OUTPUT

TIE TO COMMON - 20 02 - X AXIS STEP PULSE OUTPUT

TIE TO COMMON - 19 01

A lathe could be  operated  with  only  two  motors  connected.  A

customer has even built a machine that only uses one motor and a relay

to  cut  sheet  material  to  length from a bulk roll.  You could even

forget about the motors all  together  and  just  use  the  relays  to

control  something  by  drawing  different  colored lines of different

[42]

[                 H O O K   U P   C O N E C T I O N S                ]

lengths to control the timing  and  sequence  that  the  relays  would

switch on and off.

If  you  wish to connect directly to the pins on your parallel port

card's 25 pin connector rather than use the printer end of a  parallel

port cable, the connections are:

Pin No.

13 - Z AXIS HOME SWITCH INPUT

TIE TO COMMON - 25

12 - Y AXIS HOME SWITCH INPUT

TIE TO COMMON - 24

11 - X AXIS HOME SWITCH INPUT

TIE TO COMMON - 23

10 - LIMIT & PAUSE SWITCH INPUT

TIE TO COMMON - 22

09 - AUXILIARY RELAY "B" OUTPUT

TIE TO COMMON - 21

08 - Z AXIS DIRECTION OUTPUT

TIE TO COMMON - 20

07 - Y AXIS DIRECTION OUTPUT

TIE TO COMMON - 19

06 - X AXIS DIRECTION OUTPUT

TIE TO COMMON - 18

05 - AUXILIARY RELAY "A" OUTPUT

TIE TO COMMON - 17

04 - Z AXIS STEP PULSE OUTPUT

AUXILIARY RELAY "D" OUTPUT - 16

03 - Y AXIS STEP PULSE OUTPUT

AUXILIARY INPUT - 15

02 - X AXIS STEP PULSE OUTPUT

14

01 - AUXILIARY RELAY "C" OUTPUT

You  can  of  course adjust connections on the 25 pin connector to

correspond to any of the alternate hook-ups, e.g.  use auxiliary input

on  pin 15 for the C axis home switch,  and or use the auxiliary relay

"D" output on pin 16 to pull up pin 10.  As you have probably  noticed

pins  1 through 13 have the same connections on both the parallel port

25 pin connector and the 36 pin parallel printer cable connector.

JOY-STICK PORT

Starting with DANCAM & DANPLOT v2.60  you  can  use  the  standard

clone computer joy-stick port to control the X,  Y, and Z axis motors.

If you wish to hook-up some other joy-stick  the  connections  to  the

joy-stick port are:

[43]

[                 H O O K   U P   C O N E C T I O N S                ]

Pin No.

08

15

07 - SW TO PIN 05, MARK END

14

06 - 100K OHM POT TO PIN 01, Y AXIS

100K OHM POT TO 01, Z AXIS - 13

05 - COMMON GROUND FOR SWITCHES

12

04

11

03 - 100K OHM POT TO PIN 01, X AXIS

10

02 - SW TO PIN 05, MARK START

09

01 - +5 VOLTS TO POTS

The  joy-stick  should  be  the type that returns,  by spring,  to

center when the control is released (so the motors will  stop  turning

when  you  let go.) Some commercially available "game type" joy-sticks

have a "throttle" control pot (wired to pin 13) that will operate  the

Z  axis  motor.  If  you connect two 2D "game type" joy sticks to your

joy-stick board one will control the X and Y axis,  and the other will

control  the  Z axis.  Remember to use the configuration menus to tell

the programs what type of joy-stick connections you  have  before  you

try to use the joy-stick.  If the joy-stick gives an error, check that

any  extension  cable  actually  has  connections for all the required

pins.

[44]

[            C O N F I G U R E   A N D   I N S T A L L               ]

QUICK DISCUSSION OF IDEA

In order for DANCAM and DANPLOT to make parts that come out to the

proper size  you  need  to  properly  answer  some  questions  in  the

configuration  menus  of DANCAM and DANPLOT.  Additionally you need to

test the motors to determine the maximum reliable speed,  set  up  the

feed  rate  to  line color table,  and set up the auxiliary relays and

home switch's options.

STEPS TO CARRY OUT IDEA

1) Measure the distance moved per motor step for your machine.

2) Enter the proper values for the distance moved on the various axis.

3) Enter the possitional offset for the home position of the tool.

4) Decide if you want to use the home switches.

5) Enter the feed rate (i.e. step delay) for each line color.

6) Set up the auxiliary control relays.

7) For DANPLOT select manual or automatic motion on the Z axis.

8) For DANPLOT select the C axis if needed.

9) Test your configuration and adjust as needed.

DETAILED DISCUSSION OF IDEA

DANCAM and DANPLOT send messages to the motor  translator  modules

in  the  form  of what are called "steps." The motor shaft rotation is

divided into fractions, usually 1.8 degrees per step, or 200 steps per

360 degrees (360/200=1.8).  Sometimes motors are run "half step"  mode

where  the  step  is divided in half,  e.g.  400 per revolution of the

motor shaft.  Micro stepping  divides  the  motor  step  into  smaller

parts,  but  does not usually offer higher actual possitional accuracy

under fluctuating loads since the motor shaft of a stepper motor  will

typically  exhibit  0.9 degrees of possitional error when driven under

full load from clockwise,  and  then  counterclockwise  rotation.  For

servo  motors the optical shaft encoder defines the minimum rotational

interval or "step."

Note that the unit name inches is given in the configuration menus

in DANCAM and DANPLOT even though any units can be used so long as all

of the set up is done in the same units of measurement.  This was done

to avoid the confusion some users have about the meaning of  the  word

units.  You  will  also  have to set the overall scaling factor in the

configuration menu of DANCAM  and  DANPLOT  to  relate  to  the  units

scaling  factor used while drawing the tool path in DANCAD3D.  If this

gets confusing,  just set the global scale  in  DANCAD3D  at  240  and

configure DANCAM and DANPLOT in steps per inch,  and set the DANCAM or

DANPLOT overall scaling factor to  1  (then  one  unit  in  DANCAD3D's

drawing  editor  will  move  the  tool  one  inch (this is the default

configuration.))

So,  since the motor shaft has a finite number of  positions  that

[45]

[            C O N F I G U R E   A N D   I N S T A L L               ]

the shaft can stop at, the driven load will also have finite positions

it can stop at.  When working out the ratio of the timing belt pulleys

for  the  motor  and  lead screw you should figure that one motor step

should move the work-piece relative  to  the  tool  by  one  half  the

smallest  amount  necessary.  If you are machining to 0.001 of an inch

and have a 10 pitch lead screw then 1:1 coupling of the stepper  motor

will give you 0.0005 inches of movement per motor step, i.e.:

200 steps per revolution

--------------------------  =  2000 steps/inch  =  0.0005 inches/step

0.1 inch per revolution

The  reason you want twice the required resolution is that stepper

motors will "over-shoot" about one half step while  running,  and  can

also lag behind about half a step under load.

Successfully  installing  DANCAM  or  DANPLOT  requires having the

motors  installed  to  give  the  coarsest  resolution  that  will  be

acceptable.  Excessively  fine resolution will limit the maximum speed

the tool can move (because the maximum steps per second is  constant.)

Reducing  the  resolution will increase the maximum speed the tool can

travel.  In our example we can calculate the maximum tool  feed  speed

in inches per minute from the motor RPM, since with 1:1 drive the tool

will  move  0.1  inch per revolution,  and 120 RPM would be a reliable

maximum speed for the motor shaft.

120 revolutions per minute

--------------------------  =  12 inches per minute

10 revolutions per inch

Feed rates for other resolutions can be worked out for  the  motor

shaft speed of 120 RPM and 1:1 shaft coupling:

----------------------------------------------------------------------

|    FEED RATE WITH FULL STEP STEPPER MOTOR RUNNING 120 RPM (1:1)    |

----------------------------------------------------------------------

|   LEAD SCREW | FEED/REV | STEPS/INCH | INCH/STEP | INCHES/MINUTE   |

----------------------------------------------------------------------

|     2.5 tpi  |   0.4"   |     500    |   .002    |      48         |

|     5.0 tpi  |   0.2"   |    1000    |   .001    |      24         |

|    10.0 tpi  |   0.1"   |    2000    |   .0005   |      12         |

|    20.0 tpi  |   0.05"  |    4000    |   .00025  |       6         |

----------------------------------------------------------------------

If  the  motor  is half stepped and the motor is coupled through a

1:2 set of timing belt pulleys the feed rate can be  doubled  and  the

resolution held the same (one turn of the motor shaft for two turns of

[46]

[            C O N F I G U R E   A N D   I N S T A L L               ]

the  lead screw.) Unfortunately when motors are half stepped they have

about half of the torque as when the motors are full stepped,  because

to  half step,  the motor alternates between two coils on and one coil

on (in full step you normally have two coils on  all  the  time.)  You

also  further divide the torque in half when you "gear up" the pulleys

1:2.  So going from full step 1:1 coupling,  to half step 1:2  pulleys

divides  the  torque  by 4.  Also when running the motor at full speed

you only get about 0.25 the rated holding torque,  therefore you  will

need  a  motor  rated for (holding torque of) 8 to 16 times the torque

required to move the tool if you want to half step the motors.

----------------------------------------------------------------------

|    FEED RATE WITH HALF STEP STEPPER MOTOR RUNNING 120 RPM (1:2)    |

----------------------------------------------------------------------

|   LEAD SCREW | FEED/REV | STEPS/INCH | INCH/STEP | INCHES/MINUTE   |

----------------------------------------------------------------------

|     2.5 tpi  |   0.8"   |     500    |   .002    |      96         |

|     5.0 tpi  |   0.4"   |    1000    |   .001    |      48         |

|    10.0 tpi  |   0.2"   |    2000    |   .0005   |      24         |

|    20.0 tpi  |   0.1"   |    4000    |   .00025  |      12         |

----------------------------------------------------------------------

Since the torque of stepper motors falls off the faster the  motor

turns  you may only be able to get reliable operation at speeds slower

than 60 RPM.  In general doubling the voltage used to drive a  stepper

motor will increase the top speed by 50%,  i.e.  if a motor works well

at 60 RPM at 24 volts you  can  probably  get  it  up  to  90  RPM  by

increasing  the  voltage  to  48  volts.  The  top voltage for stepper

motors is about five to ten times  the  rated  voltage,  you  will  of

course need to limit the current to the rated current to avoid burning

out  the motor (in the simplest scheme a series power resistor is used

to limit the current flowing through the motor winding when the  motor

is not turning to the rated current, because the motor is an inductive

load  the  resistor  will  allow  the  voltage  to rise briefly at the

beginning of each motor step before the  voltage  drops  back  to  the

static level, thereby making the motor turn faster.)

When  building  a  plotter you can drive the pen holder by using a

timing belt driven directly by a pulley.  Small motors  can  generally

run at 240 RPM in half step mode to give 1600 (half) steps per second.

----------------------------------------------------------------------

| PULLEY DIAMETER | FEED/REV | STEPS/INCH | INCH/STEP | INCHES/SECOND|

----------------------------------------------------------------------

|     0.25"       | 0.785"   |   509.55   |  0.00196" |   3.14159"   |

|     0.50"       | 1.570"   |   254.65   |  0.00392" |   6.28318"   |

|     0.75"       | 2.356"   |   169.76   |  0.00589" |   9.42477"   |

|     1.00"       | 3.141"   |   127.32   |  0.00785" |  12.56636"   |

----------------------------------------------------------------------

[47]

[            C O N F I G U R E   A N D   I N S T A L L               ]

When  selecting  stepper motors to drive your equipment you should

use motors that have a rated holding torque of AT LEAST FOUR TIMES the

torque that will be required under load and at full  speed.  Also  use

the  motor  testing  utilities built into DANCAM and DANPLOT to do the

start and stop test to make sure that the spacing of the  pulses  sent

to  the  motor  translator module are far enough apart (a larger pulse

width factor will space the pulses wider apart) for the motor to  keep

up  with the step pulses (if the pulses are to fast the motor will not

come back to the point it was at at the beginning of the  motor  test,

indicating  unreliable operation.) The ramping should be set to 0 when

testing stepper motors.  The ramping feature is  mostly  intended  for

use  with  servo motors so that the step pulses speed up and slow down

gradually to allow for the time it takes the servo motor controller to

adjust the motor drive current.

I would like to warn you that stepper motor manufactures published

speed vs.  torque curves can be very optimistic, and are not generally

applicable to use with DANCAM or DANPLOT.  As a rule of thumb based on

my  experience  you can expect these values for maximum top speeds for

reliable error free operation under working load,  with the higher RPM

number being obtained by using the maximum voltage the motors can take

(when  the  voltage  is  more than six times the rated voltage stepper

motors may become unstable in the middle of the motors speed range, so

you are limited in how much the motors can be boosted):

----------------------------------------------------------------------

|       MOTOR HOLDING TORQUE     |     MAXIMUM MOTOR SHAFT SPEED     |

----------------------------------------------------------------------

|          20 -   40 oz-in       |           240 - 300 RPM           |

|          40 -  150 oz-in       |           120 - 240 RPM           |

|         150 -  400 oz-in       |            90 - 120 RPM           |

|         400 - 1500 oz-in       |            30 -  90 RPM           |

----------------------------------------------------------------------

One problem with the stepper motor manufacture's specifications is

that they normally measure the torque using a  technique  that  allows

the motor to be lousing steps,  i.e.  the rotor is slipping, while the

motor is running.  They are not lying in as much as the motor actually

produces that torque at the specified RPM,  but they fail to point out

that  this  is  useless  information if you are interested in accurate

open loop operation of the motor.  For the operation  to  be  accurate

the  rotor  must  NEVER  be more than one half of a step (or half of a

half step) from the position dictated by the  number  of  step  pulses

sent to the driver module.  Also the motor must be able to work at all

speeds  between  the  maximum  and  stopped  without  "resonating" and

lousing steps.  Manufactures frequently rate the holding torque  at  a

current  that  is the maximum the motor can take without burning up or

demagnetizing the rotor,  rather than the current that would give  the

best  and  most accurate performance over the full range of speeds.  I

am mentioning this because I do not want you to look at the  overblown

[48]

[            C O N F I G U R E   A N D   I N S T A L L               ]

specifications  some  manufactures publish and waste a lot of money on

their expensive hardware when you can get nearly the same  performance

in  the  "real world" by using much less expensive parts.  If possible

test any hardware yourself before you pay for it.

To configure DANPLOT and DANCAM to produce the  proper  number  of

steps  per  inch  of  travel simply run DANCAM or DANPLOT from DOS and

select mode #4 from the main menu that comes up.  Since stepper motors

typically can only step 300 to 1000 full steps per second (600 to 2000

half steps) the motor step delay of 0.5 ms to 3 ms is needed  to  keep

the  motors  working properly.  The minimum motor single step delay is

controlled a value called the pulse width factor (p.w.f.).  Each  axis

has  its  own  p.w.f  so  the  motors  can  run  as  fast  as possible

(allowable) when the maximum feed rate is requested.

When you install DANCAM and DANPLOT using their main  menu  option

#4,  be  sure  to  work through the sub-menu options in order #1,  #2,

(sub-menu of #2 options #1, #2, #3 for each axis) and then #3, #4, and

#5, and so on.

When you work through the menus keep in mind the number  of  steps

the motor shaft makes per revolution.  If you run a motor in half step

mode, you will need to multiply the number of motor steps by two.  For

servo  motors  the  number of steps per revolution can be derived from

the encoder counts per revolution and any pulse rate multiplier  used.

Some  servo  motor  controllers increase resolution by multiplying the

lines in the optical encoder by 4,  such that a 500 line encoder  will

really give 2000 counts (steps) per revolution.

The  rate multiplier in configuration option #1 lets you have more

than one pulse sent for each motor step  signal.  This  is  useful  to

speed  up  some kinds of micro stepper or servo motor controllers that

have excess resolution.  If you have a micro stepper  that  has  20000

steps  per  revolution  you can set the rate multiplier to 10 or 20 to

reduce the resolution to a more productive  2000  or  1000  steps  per

revolution.  When  driving  regular  stepper motors full step the rate

multiplier should always be set to 1 pulse per step.

MOTOR TESTING

In the motor test sub-menu (#2  in  the  configuration  menu)  use

option  #1  to  find  the  smallest  p.w.f.  that  makes the motor run

smoothly under load, use #2, the start/stop test,  (#2 of #2 of #4) to

check that the motor is not missing any steps (make the p.w.f.  larger

if the motor is missing steps and test again),  and use #3 (of  #2  of

#4)  to  test  the  RPM  if needed to help in figuring the feed rates.

When the p.w.f.  has been configured for each axis press [Esc]  to  go

back to the installation sub-menu and answer the other questions.  The

ramping  should be set to 0 for testing stepper motors.  Note that the

motors do not need to be running (powered) for the RPM test to work if

you are testing "open loop" motors, i.e. stepper motors.

[49]

[            C O N F I G U R E   A N D   I N S T A L L               ]

When you perform the start/stop test,  menu selection #2 of #2  of

#4,  you  need  to  mark  the motor shaft and a fixed point before you

start the test.  After the start/stop test the marks  should  line  up

again.  If  the  marks  do  not  line up you lost some steps.  Several

things can cause lost steps (P.  stands for  problem,  S.  stands  for

solution):

1. P. The p.w.f. for the axis under test is to small.

S. Increase the p.w.f. until the error is removed.

2. P. The motor is over loaded.

S. Use a larger motor or reduce the load.

3. P. Your load has to much inertia and or not enough friction.

S. Drill lightening holes, or add some friction, viscous damper.

4. P. The motor is not getting the proper voltage and current.

S. Adjust the module current setting or the series power resistor.

5. P. Electrical noise is upsetting the module logic circuits.

S. Add a low pass filter on the input of the translator module.

6. P. Some mechanical part is loose or slipping.

S. Avoid parts that can slip like cable drives, use thread lock.

The  RPM  test  in the motor configuration sub-menu will work with

stepper motors being switched off.  The only time the motor  needs  to

be on is if you have a servo motor hooked up in "closed loop" with the

error  line  feeding  back to the pause input on the parallel port.  A

note of warning about the RPM test,  the motor will run for one minute

which  may  drive your machine to the end of its travel and beyond and

possibly break something,  so switch the motor off before running this

test  if  you  are  going  to  have  an out of range problem.  Another

problem that can occur with the RPM test is that since servo motors go

so fast  you  may  overflow  the  servo  motor  controller's  position

register  making  the motor suddenly reverse direction at full torque!

So think about how far your motor will move in one minute and  see  if

you  will  have  a  problem.  If  your limit switches are wired up the

motor will stop when the limit switch opens (if a servo  motor  is  at

full  speed  it  may over shoot the limit switch a little,  so it is a

good idea to use limit switches that can take some movement beyond the

trigger point.)

The questions in option #3 in the installation sub-menu should  be

used  to  install  the  scaling factors before you enter the feed rate

controlling delays with option #4.  In version 2.60+  of  DANPLOT  and

DANCAM  the  feed  rate  is controlled by a delay between motor steps.

Since the p.w.f.  keeps each motor working  properly,  the  feed  rate

delay  can vary from 0 to 65000000 micro seconds per step depending on

how slow you want the feed.  To get different feeds,  enter the delays

you want in the feed rate table,  then make the tool path drawing with

DANCAD3D so that the line  segments  have  the  color  attribute  that

corresponds  to  the  delay  set  in  the  feed table.  The line color

[50]

[            C O N F I G U R E   A N D   I N S T A L L               ]

attribute is set while drawing in DANCAD3D v2.6 by  pressing  the  [L]

key of the keyboard or by using the mouse to click on the [L]ine style

option of the [L]ines sub-menu from the drawing editor root menu.  Any

step  delay  value  can be assigned to any line color number.  Set all

the delays to 0 if you want the tool to move as fast as  possible  for

all  motion.  Generally  the  sixteen delays installed in the menu are

enough,  however you can edit the  configuration  file  manually  with

DANCAD3D's  [W]rite  command to set a step delay value for each of the

127 line colors available (you will see a bunch of  0's  next  to  the

numbers 0 through 127,  just change the 0's, (the number on the right)

to the value for the step delay you want.)

AUXILIARY RELAYS

Another configuration menu  option  lets  you  assign  which  line

colors,  in  the  tool  path  file,  will turn on or off the auxiliary

control relays.  Line segments used to control  the  auxiliary  relays

can  have  starting and ending points at the same location (i.e.  zero

length.)

Once a relay is turned on it remains on until a  line  segment  is

encountered  that  has  the special color to turn it off.  This allows

the four relays to be individually  turned  on  an  off  at  different

times.

When  a tool path file is being executed,  before the motors start

to turn to follow the line segment,  the line color is checked against

the  installed  options to see if one of the auxiliary relays needs to

be turned on or off.  The step delay installed for the line  color  of

the  currently executed line segment will set the feed rate (speed) of

the motors.  The auxiliary relay to line color configuration will turn

on or off the relays as configured.  The auxiliary relay  options  let

you  assign  a delay after the relay turns on so that the device being

turned on is active before the motors start to turn.  Likewise when  a

line  color  signals  a relay to turn off,  a delay can be set to make

sure that the device is fully off before, the motors continue to turn.

When a line color is read that activates a relay the relay state  will

be stable until another line segment is encountered that is configured

to  affect  the  status of the relays.  For example,  if you have line

color 3 set to turn on auxiliary relay "A" and line color #8  to  turn

off  auxiliary  relay  "A"  then  when the file is executed and a line

segment with color #3 is read the relay will toggle  on  and  stay  on

while  other  color  line  segments are read so long as the other line

segments are not color #8.

Additional axis can be added to a machine by using  the  auxiliary

relay outputs to switch the step and direction signals from the X,  Y,

or Z axis to a forth,  fifth,  sixth,  or seventh motor driver module.

You  can  arrange some TTL "AND" gates to send the step signals to one

motor when a auxiliary relay is hi,  and another when the relay output

on  the  parallel  port  is low.  The additional motors can be used to

rotate a part that is being milled for cutting of gear teeth and other

[51]

[            C O N F I G U R E   A N D   I N S T A L L               ]

sorts of "indexed" manipulations where only 3 of 7  axis  need  to  be

turning  at any given moment.  The line colors would select which axis

are turning.

Another  feature  the  line  segment  colors  can  control  is  an

automatic  pause.  Putting  an  automatic pause in your tool path will

allow you to adjust the tool position against a reference point before

cutting actually begins.  During a pause you could also  make  a  tool

change, or brush the chips away to make an inspection.

HOME POSITION

Option  #5  of  the  installation  sub-menu  asks  about  the home

switches.  Both DANPLOT and DANCAM let you install  home  switches  on

your  equipment  so  that  you can have the tool automatically home up

before  each  tool  path  file  is  read.   The  ability  to  home  up

automatically  can  be  a  great  time  saver since you do not need to

manually jog the motors to their starting position.  If you  turn  the

motor  power  off  and  then  back  on  stepper motors may "jump" to a

position several steps away,  so without the home  switches  installed

you would need to manually check the home position every time you turn

the power on.

Both DANCAM and DANPLOT allow you to select the option to have the

tool  automatically homed up to the home switches at the start of each

tool path file.  If you wish to start with  the  tool  in  some  other

position  you  can  turn  off the automatic home up feature,  manually

select the home up to  switches  from  the  menu,  then  use  the  jog

position menu to move the tool to the special starting point, and then

execute the tool path.

The  position  of  the  home  point  relative  to DANCAD3D's 0 0 0

workspace center needs to be entered into the  configuration  menu  so

that  the  proper  displacement  (offset)  will  be  added to the line

segments in the tool path.  For example if you have a plotter that  is

24"  by  36" you would install the home point as 12 and 18 so that the

tool will move to the corner of the drawing,  and the  center  of  the

drawing  would  be  0 0 0.  You can install the home point as 0 0 0 as

well,  but you would have to draw in DANCAD3D with  the  elements  off

center in the workspace.

Another  reason for having the home position at the extreme margin

of the plotting  area  is  so  the  backlash  compensation  will  work

properly.  Since the backlash compensation takes up all the "slack" on

the  first  move in the tool path the tool cannot return from "behind"

the starting (home) position.  Installing the home switches at the end

of the device travel is also the simplest method.  The  home  position

numbers in the DANCAM/DANPLOT configuration menu for the most part are

there just let you shift the relation between DANCAD3D's workspace and

the range of motion of the tool in your device.

[52]

[            C O N F I G U R E   A N D   I N S T A L L               ]

THE C AXIS

DANPLOT  has some additional special configurations.  The "C" axis

lets you mount a fourth motor with its shaft parallel to and  centered

on  the  Z  axis  such  that  a  knife,  saw,  gauge,  or  broach will

automatically rotate to point the cutting  edge  into  the  motion  of

travel.  The  tolerance  on  the  C axis lets the tool stay down while

cutting out curves.  Generally the C axis tolerance before  tool  lift

should  be  set  to  between 10 and 20 degrees.  There is an option to

display the angle for the C axis tool.  Normally the display of the  C

axis angle would be turned off to get the best speed,  however you can

use the display of the angle for testing, or for manually rotating the

tool by placing a protractor  on  the  tools  axis  and  matching  the

reading of the protractor to the displayed value.

MANUAL Z AXIS UP AND DOWN

DANPLOT  also  has  a manual up/down mode.  The manual up and down

option lets you manually activate the Z axis for use in  automating  a

drill  press  with  two motors (one for X axis and one for Y axis.) In

manual up/down mode the work-piece is positioned and then the computer

is made to pause while you drill the hole, then you press a key on the

keyboard and the work-piece will move to the next position, and so on.

The words UP and DOWN appear on the computer screen while the computer

is paused to tell the operator where the tool should be.  You can also

use the Z axis direction pin on the parallel port to turn on a lamp or

buzzer to aid the operator in keeping track of what he should be doing

with the up and down motion of  the  tool.  For  drilling  the  manual

up/down  can  be  very useful in reducing drill breakage caused by the

drill clogging during automated feed.

[53]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

QUICK DISCUSSION OF IDEA

DANCAM and DANPLOT are meant to be used  with  DANCAD3D.  DANCAD3D

lets you draw line segments in a certain order, and keep that order of

line  segments in the file that is saved.  Since you know the order of

the points you want the tool to move to,  to make your  part  all  you

have to do is draw the line segments between the points,  in the order

you want,  with DANCAD3D's drawing editor.  To make  circles,  curves,

and swept surfaces you can use some of the other commands DANCAD3D has

to  offer.  If  you  wish to use some other program to make tool paths

for use with DANCAM or DANPLOT you can (or ask me to) write a  program

to  convert  the  other  program's file format into the DANCAD3D ASCII

file type.  DANPLOT v2.6+ can directly read HP-GL files  derived  from

some   third   party   CAD  programs  and  raster-to-vector  (scanner)

conversion programs.

STEPS TO CARRY OUT IDEA

1) Figure out what type and size of cutting tool  or  tools  you  will

need to use to make your part.

2) Determine the maximum size of the raw material  you  will  need  to

use,  so  that the tool will not rapid move into a high spot on the

work-piece and stall or break off.

3) Figure out the relation  of  the  home  position  of  your  machine

running  DANCAM  or  DANPLOT and DANCAD3D's workspace center.  Also

adjust if necessary the  over  all  scaling  factor  in  DANCAM  or

DANPLOT to the units you will use in DANCAD3D (the default scale of

240  in DANCAD3D and the scale of 1.0 in DANCAM/DANPLOT lets 1 unit

in the DANCAD3D drawing editor equal one inch of  movement  on  the

work-piece.)

4) Run   DANCAD3D and draw your tool path.  If you are using DANCAM to

cut the part be sure that your first and last line  segments  guide

the  tool around the projections of the work-piece in a safe manner

from and to the home position or the tool might bang into something

and ruin the part (also watch out for the vice,  clamps,  and other

fixtures!)

5) If  you need to visualize the size of the cutter in relation to the

outline of the finished part you can use the [D]rag command to draw

line segments while you drag an element of the cutter  shape  (this

is described in more detail below.)

6) Use  the  [J]oin  command in DANCAD3D's drawing editor as needed to

join  together  elements.  Use  the  [E]rase  command in DANCAD3D's

drawing editor if needed to erase any "guide" elements that are not

part of the tool path.

7) Save the tool path as an ASCII type drawing file.

8) Run DANCAM or DANPLOT and execute the ASCII tool path file.

DETAILED DISCUSSION OF IDEA

Any of DANCAD3D's drawing features can be applied to  the  drawing

[54]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

of  a tool path file for use in guiding the motion of a tool driven by

DANCAM or DANPLOT.  Some approaches to drawing tool paths might be:

1. Draw lines manually in the [D]raw lines command.

2. Draw lines while dragging a cutter outline in the [D]rag command.

3. Use the [P]olygon, and [F]it-curve commands.

4. Use  the Alias feature to create standard tool cycles  (as  drawing

symbols) that you can quickly use over and over again.

5. Write a macro to use the BETWEENS command for swept surfaces.

6. Write a macro that uses a formula to APPEND line segments.

7. Use the [W]rite command to manually enter coordinates into an ASCII

file.

8. Use the teach mode in DANCAM/DANPLOT to make an ASCII file.

You should look at the DANCAD3D's documentation for information on

the drawing commands and the macro language.

In general tool path files look a little strange at first,  but if

you  know what type of tool is being used you should be able to figure

out what the finished part will look like.

One common characteristic of tool paths for milling is  that  they

contain  a  lot  of parallel lines or concentric circles.  The spacing

between the parallel lines or concentric circles would give the  depth

of  the  cut  to  be made.  Successive planes of parallel lines can be

used to rough out deep pockets.

When you create tool cycles for pockets or counter sunk holes  you

should put the starting and ending line segments sticking up above the

surface  of  the  work-piece.  You  will not need to connect the "flag

poles" sticking up above the plane of  the  work-piece  surface  since

DANCAM  will  automatically  move in a straight line to the next point

(i.e. to the next top of a "flag pole.")

Tool cycle symbols you  create  can  be  altered  with  DANCAD3D's

[M]agnify  and  [R]otate  commands to adjust the size of the cut made.

If you magnify a cycle for a pocket in the axis of the parallel  lines

the depth of the cut (one third the diameter of the end mill) will not

change.  If you magnify across the pocket's parallel lines (increasing

the spacing) you can still use the element if a larger diameter cutter

is  used.  You  can  write a macro in DANCAD3D that will automatically

create any type of tool cycle from prompting  you  for  the  pertinent

dimensions,  e.g.  tool radius, length and width of cut, depth of cut,

rotation or tilt if any, and so on.

Holes can be drilled by having DANPLOT read  an  ASCII  tool  path

file  that just has dots (line segments with no length) where you want

the holes to be.  The depth of the holes, and drill feed rate would be

set up in DANPLOT's configuration menu.

When no line is drawn in a tool path DANCAM and  DANPLOT  normally

go to the most rapid feed rate to travel to the next point in the tool

path.  For  lines  that  are  drawn  in the tool path the line segment

[55]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

color is used to determine the feed rate based on the  values  entered

in  the  DANCAM  or  DANPLOT configuration file.  In the configuration

file you can set the range of feeds (motor RPM) over a very wide range

from the maximum rapid move,  down to very slow.  When the motor speed

ramping  is on,  the line color determines the maximum speed that will

be achieved if the line segment is longer than the  ramping  slope  up

and down (i.e. on very short lines not enough time exists to "ramp" up

to the speed requested.)

As  mentioned before the line color in the tool path also controls

the auxiliary relays and the programed pause.  If (in  the  DANCAM  or

DANPLOT configuration menu) you set up the auxiliary relay "D" to come

on  with  line  color #1 and your tool path starts with a line in line

color #1 you can use the auxiliary relay to start the spindle motor of

your tool automatically.  Likewise you can program the  spindle  motor

to  shut  off  by using another line color,  and also have a programed

pause to do a tool change or tool position adjustment.  The status  of

the relays returns to the default at the end of the tool path file and

so  would  shut off the spindle and coolant flow automatically.  Press

[L]ine style while drawing in DANCAD3D to select the  current  drawing

line  color  number.  If you have an EGA video board you can see 15 of

the line colors with  DANCAD3D's  main  menu  [P]review  command.  The

commands  in DANCAD3D's drawing editor's [L]ine edit sub-menu's [L]ine

style command let you alter the line color of any line segment in  any

element loaded into the workspace.

DRAWING TOOL PATHS WITH CUTTER TOOL COMPENSATION

The  following  are the steps required to draw a tool path element

while being able to visually see  the  shape  of  the  cutter  on  the

computer  screen.  Please  read  the  DANCAD3D  documentation  for  an

explanation of the commands indicated below.

The most obvious method of compensating for  the  tool  radius  in

operations  such as vertical milling is to simply draw the line of the

tool path off to one side of the part by the amount of the  radius  of

the  cutter.  Selecting  the  [D]elta  reading  in  DANCAD3D's drawing

editor will show you the relative distance of the drawing cursor  from

the  last  marked  point  so  you  will  not have to do subtraction or

addition in your head.

To help in visualizing the interference of  the  cutter  with  the

finished part, fixtures, and the machine itself you can use the [D]rag

command  in DANCAD3D's drawing editor drag an element with the outline

of the tool (and any tool holder if you wish) while you trace out  the

zigzag  lines  of  the  tool  path around the outlines of the finished

part, the raw stock, and the fixtures and machine parts.

The instructions below list the steps involved in using the [D]rag

command while drawing a tool path.  In  practical  drawing  you  would

probably  exit  and enter the [D]rag command several times in order to

use the [P]olygon or other commands in the  drawing  editor.  To  make

[56]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

editing  of  the tool path easier you should have DANCAD3D's automatic

output macro feature turned on so that if you make a mistake (or  want

to  make  some  changes  later)  you can edit output macro rather than

having to fiddle with the drawn lines in the tool path file  with  the

drawing editor's commands.

It  is  assumed  that you will be using DANCAD3D v2.6A or later to

draw cutter paths with compensation for the cutter shape.

1.  Run the DANCAD3D program.  Press [Return] until you reach its main

menu.

2.  Select  [D]raw  from  the  DANCAD3D main menu.  Select the [F]ront

view.  If the drawing workspace is not empty  select  [I]nitialize

to clear the workspace.

3.  Select  [D]raw from the drawing editor root menu.  Draw an outline

of the shape of the finished work-piece you wish to make.  You can

make  the inside corners sharp if the radius will be determined by

the cutter shape.  You can make the shape outline in 3D if you are

going to do 3D work.  Remember that lathes are 2 axis devices,  so

the  tool path for the lathe will be 2D (i.e.  a profile.) You can

include an outline of the raw  material  in  the  outline  of  the

finished  piece to make visualization of the amount of material to

remove easier.  The outline of the work-piece can include outlines

of holding fixtures and other machine parts so that you can  check

for  interference  with  the cutter.  When you draw the outline of

the part keep in mind the relative location  of  the  tool's  home

position  so  that the part will actually be cut as you want it to

be.

4.  The  next step is to make a new element that is an outline for the

cutting tool.  If you are going to use an end mill you can use the

[P]olygon command (try to keep the sides total to 36 or  less)  in

the  [2]D shapes drawing editor sub-menu.  For other shape cutters

press [B] to begin a new element (element 2) and draw the  outline

of the cutter.  The center of the cutter outline element should be

on  the  cutter's  axis  of  rotation  if the cutter rotates.  The

cutter outline can be 3D and include the tool holder or tool  post

so  that  you  can  visually check for interference with the work-

piece or other parts of the machine that move with the cutter.

5.  To  draw the actual tool path element go into the drawing editor's

[E]lements sub-menu and  select  [D]rag.  Select  [E]nter  element

number  and  enter  the  element  number  for  the  cutter outline

(element 2.) Select the element center  as  the  reference  point.

Press [Return] to start the drag command.  The cutter outline will

blink.  Press  [B]egin element to begin the tool path element (the

tool path will be element number 3.) Move the  cutter  outline  to

the  home  position  with  the [G]oto drawing command,  then press

[./Del] to mark the home point.  Use the  mouse  to  position  the

cutter  outline near the edge of the raw material.  Press the left

mouse button to draw the first line  segment  in  the  tool  path.

[57]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

Move  the  mouse  for the path of the first cut and press the left

mouse  button  again.  Continue  drawing  zigzag,  concentric,  or

spiral  tool  path  lines until the blinking outline of the cutter

comes in contact  with  the  outline  of  the  final  shape.  When

drawing  tool paths be sure that you only feed the amount that can

be safely taken off (so the tool will not stall  or  break!)  Also

remember that some cutters can only cut going one way,  so you may

have to retrace without feed, into the stock,  when drawing in one

direction.  Pressing  [L] while inside the [D]rag command will let

you change the drawing line color,  and  therefore  set  the  feed

rate, auxiliary relays, and programed pause.

6.  When  the  tool path is finished press [Escape] to exit the [D]rag

command.  [Q]uit  the  drawing  editor  and  select [F]iles [S]ave

[A]SCII from the main menu.  Enter  the  element  number  for  the

element  of  the  tool path (normally element 3) as the element to

save.

7.  Make a trial  part  using  DANPLOT  or  DANCAM.  If  you  made  no

mistakes in drawing the tool  path  and  your  machine  is  sturdy

enough  to  be  accurate your part should be fine,  however in the

real world you may find that  your  machine  had  some  flex  that

displaces tool under load, and so you will need to adjust the tool

path before making a run of parts.

8.  Measure the trial part.  Record any error in the final shape.

9.  Load  the  tool  path ASCII file into DANCAD3D and use the [P]ull-

Point  command  in  the  [L]ines sub-menu of the drawing editor to

alter the tool path line segments for the final cut,  in order  to

correct  for  any  errors.  Save  the  corrected tool path element

again as an ASCII file.

10. Repeat  steps  7  to  9  if  needed  until the finished part is in

tolerance.  Make the run of parts as needed.  Store the ASCII tool

path file on a floppy disk for future use.  You should  also  copy

the  DANPLOT.CFG or DANCAM.CFG file used to make the part onto the

same floppy disk with the tool path file,  so the tool  path  will

match  up  if  used  again.  You  should also file notes about the

spindle speed and cutter seating used and also the location of the

limit switches relative to the work-piece.

If you want to change the line color attribute for any of the line

segments in the tool path such as the line segment for the  final  cut

you  can  use  the  commands in DANCAD3D's drawing editor [L]ines sub-

menu.  The conversion of the line color attributes in  the  tool  path

data  file,  into  feed  rates,  is done using the values for the step

delay that you have entered into the  DANPLOT  and  DANCAM  feed  rate

table  (option #4 in the DANPLOT and DANCAM configuration installation

sub-menu.)

You can also draw the bulk of the tool path in DANCAD3D's  drawing

editor's [D]raw lines sub-menu,  and only when the lines for the final

[58]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

cuts need to be drawn go into the [D]rag  command  in  the  [E]lements

sub-menu  to align the cutter outline to the final shape outline guide

element (to make the drawing of the lines easier and quicker.) If  you

want to add line segments to the tool path element that you started in

the [D]raw lines sub-menu use the [D]rag command as described above in

step 5.  but select command [E]lastic line in place of command [B]egin

element (the [E]lastic line is normally off in the [D]rag command, but

you can turn it on so that you will be able to see the location of the

line you are about to draw.) Be sure that you remember to [E]rase  any

guide  elements in the workspace before you save the tool path element

to an ASCII file so that the guide elements do not accidentally become

part of the tool path file!

DANCAD3D v2.6A+ supports the CalComp "Drawing Board II"  digitizer

tablet,  so if you have paper drawings of a part, you can generate the

required tool path by  moving  the  drawing  board's  "cursor"  (puck)

around  on  the  drawing  (the  drawing  should be taped down to avoid

movement.) You can (approximately) figure the cutter offset by  taping

a  transparent  scaled outline of the cutter to the cross hairs on the

"cursor."

DANCAM and DANPLOT v2.6 have a "teach" mode that allows you to use

them to create a tool path file that can be "re-played" to make parts.

Just use the joy-stick, the cursor keys,  or enter the position of end

points with the [G]o-to command.  See main menu option #6 in DANCAM or

DANPLOT.  Note that only DANCAM records the actual Z axis position for

the movements marked out.  The joy-stick buttons work like [Del/.] and

[Ins/0] in DANCAD3D to mark the start and end of line segments.  Teach

also has a curve-fitting option to draw curves in 3D space.

JOINING ELEMENTS

When you need to use commands that create new elements such as the

[P]olygon  command,  or  the [F]it-curve command in DANCAD3D's drawing

editor,  you can unite the separate  elements  created  in  DANCAD3D's

drawing  workspace  into  a single ASCII tool path file by erasing any

guide elements (e.g.  the finished part outline element,  and the tool

outline  element) and then use element number 0 as the element to save

with the [S]ave [A]SCII  command  (rather  than  just  saving  element

number 3.) The [J]oin command in DANCAD3D's drawing editors [E]lements

menu  can  also  be  used  to  unite  elements into a single tool path

element.

If you need to rearrange the order of elements that will become  a

tool  path  file  you  can save all the elements in the workspace with

DANCAD3D's  main  menu  [F]iles  [S]ave   [E]lements   command.   Then

[I]nitialize  the  workspace.  Then  use  the main menu [F]iles [L]oad

[3]D-Quick command to reload the individual elements one at a time  in

the  order you want them to be.  When all the elements are loaded back

in the proper order,  use the main menu [F]iles [S]ave [A]SCII command

to  save  element  number  0 to make the tool path file.  You can also

write a DANCAD3D macro to do these operations for you automatically.

[59]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

The [S]plit command in DANCAD3D's drawing editor's  [J]ockey  sub-

menu  lets  you  break  up  a  single element in a tool path file into

smaller elements for editing.

SETTING UP THE WORK-PIECE

Although not strictly the domain of using DANCAM or DANPLOT I must

say something about setting up the work-piece.  In order to be able to

make usable parts you will need to be very careful in your  choice  of

fixtures  for  holding the work-piece.  The position of the work-piece

relative to the tool's home position must be known to small tolerances

if you are to be able to flip a part over and cut  the  back  side  to

line  up  with  the front side.  To aid in lining up the part when you

flip it over you might make a special tool path that drills holes  all

the  way  through  the part to act as reference points,  or just makes

some finished flats on sections that will be later cut off.  You could

then make a tool path file that would move a dial gage around  in  the

tool  holder  (with  the spindle off) and pause on points on the work-

piece to help with alignment.  Various kinds of edge finder  could  be

likewise used to zero in on holes and flats.  If such a set up routine

was  put  at  the  head  of  the  tool path the [A]djust tool position

command in DANCAM and DANPLOT could be used to jog the tool around  to

line  up  before  the  cutting  starts.  You can also pause DANCAM and

DANPLOT during the rough cutting and use a dial gauge  or  caliper  to

see  if  the part is coming out properly before allowing the final cut

to be made.

To minimize errors it is always best if  the  whole  part  can  be

produced  without  un-clamping the work-piece,  or at least moving the

clamps around one at a time without letting  the  work-piece  move  at

all.  If you can make the part from one set up,  all you have to worry

about is that you have the part centered well enough that you  do  not

run out of material on one side.

To  make small three dimensional parts you can construct a fixture

that lets you bolt down a  guide  plate  with  guide  pins,  and  then

submerge  the  work-piece  in  a  block  of  firm  wax (or special low

temperature melting point metal alloy  containing  Bismuth  &  Indium)

that has guide sleeve inserts cast into the block of wax such that the

block  of  wax  can  be  rotated and centered on the inserts (over the

fixture's pins) without losing the accuracy of position  of  the  part

inside  the  block  of wax.  After making all the cuts from one side a

mold would be placed over the fixture and new melted  wax  pored  over

the  part  such that the block of wax with the work-piece inside of it

and the embedded guide sleeves can be flipped over to work  the  other

side.  Liquid plastic,  or molten lead alloy,  clay,  plaster,  or tar

could be used in place  of  the  wax  so  long  as  the  encapsulating

material  can  later  be  washed  or  melted away without damaging the

valuable part (work-piece) inside.

A three jaw chuck can be  mounted  on  the  table  of  a  vertical

[60]

[          D R A W I N G   T O O L   P A T H   F I L E S             ]

milling machine to hold round parts that need to be flipped over.

[61]

[        R U N N I N G   T H E   C A M   D E M O   F I L E S         ]

QUICK DISCUSSION OF IDEA

To give you something to test the DANCAM and DANPLOT programs with

I  have  included some demo ASCII tool path files on the disk with the

programs.  After you get your motors hooked up you can run these  demo

files to see if the motors are working properly.

STEPS TO CARRY OUT IDEA

1) Install and configure DANCAM and or DANPLOT.

2) Hook up your motors and other electronics.

3) Use the demo batch files DEMOCAM.BAT and DEMOPLOT.BAT.

DETAILED DISCUSSION OF IDEA

A  demo is provided for each program.  Enter DEMOPLOT.BAT from DOS

with the prompt changed to the drive and  directory  all  the  DANPLOT

files are in.  DEMOCAM.BAT does the same for DANCAM.

EXAMPLE: C>DEMOPLOT.BAT

You  can  also  enter the filename of the demo files manually from

the command line,  or pick menu option #1 in  the  program  and  enter

DANCAM.ASC or DANPLOT.ASC to see the programs work.

EXAMPLE: C>DANPLOT DANPLOT.ASC 1

You  can interrupt (pause) the motion of the motors momentarily by

pressing the [P] key,  press [P] or [SpaceBar] to restart the  motors.

You  can  adjust the tool position while the file executes by pressing

[A].  Take all necessary precautions to protect yourself from harm  if

you are going to expose yourself to the moving parts.

You  can  load  the example *.ASC tool path files into DANCAD3D to

look at how they were drawn.

[62]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

QUICK DISCUSSION OF IDEA

You can contact the author of the DANCAM/DANPLOT programs  if  you

have  problems or want more information.  Before you write please look

over this documentation since it can answer most questions.

STEPS TO CARRY OUT IDEA

1) Read this documentation and look at the program's menus.

2) Spend a few hours trying to figure out what you did wrong.  99%  of

problems people have reported so far have  been  traced  to  faulty

connections, or other hardware problems related to bad wiring.

3) Write what you did and what happened in detail and send the  letter

along with return postage to the author for analysis.

DETAILED DISCUSSION OF IDEA

If you have any questions or comments about DANCAM or DANPLOT feel

free to write me at:

ADDRESS: DAN HUDGINS

c/o DANCAM & PLOT v2.6

466 DIAMOND STREET

SAN FRANCISCO, CA  94114

The UNITED STATES of AMERICA

When you write PLEASE enclose some stamps for the  reply  postage.

If  you  live  outside  the  U.S.  you  can  get  some  postal coupons

(vouchers) for first class international air mail  postage  from  your

post office.

In addition to the information and schematics in the user outfit I

can  help you design custom hardware for special applications.  Let me

know what you are trying to build and I will  try  to  help  you  with

ideas of how you might get your machine to work properly.

If  you need help in locating stepper motors,  drivers,  and other

mechanical and electronic components,  I have a list of suppliers that

is included in the full user outfit that you can purchase.

If you would like to discuss something on the phone I can call you

collect  if  you  send  me  a  letter telling me what time to call and

asking me to call collect.

I am  always  working  on  a  improvements  to  the  programs  and

documentation.  If  the  disks  you  have are more that a year old you

should write to me for information about the current version (tell  me

what version you now have.) If you have a machine hooked up I would be

most appreciative if you could send me a photo of what you have built,

I  always  enjoy  seeing  what users are doing with the programs,  and

[63]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

learning what improvements would make their results better.

Feel free to send me your  "wish  list"  for  DANCAD  and  DANPLOT

improvements.

QUICK ANSWERS TO COMMON PROBLEMS

Before  you write with your problem you should check that you have

not accidentally set one of  the  values  in  the  configuration  file

improperly.  If  something  does  not  seem  to be working properly go

through all the prompts in the configuration menu  and  check  exactly

what  you  have  told  the program about your machine's dimensions and

connections.  If that does not work try erasing  the  *.CFG  file  and

start the configuration over from scratch.

BACKLASH COMPENSATION

DANCAD3D's backlash compensation works by adding a fixed number of

steps  to the motor's movement when the motors direction reverses.  If

you are going to use a dial gauge to test the  movement  of  the  tool

from DANCAM's or DANPLOT's jog menu,  and the backlash compensation is

set to zero,  the first move will give a false reading since  the  nut

was  not  tight against the lead screw.  So you should always move the

tool some distance to take up the backlash  before  zeroing  the  dial

gauge and jog the tool in the same direction a few times to get a true

reading.  You  can  measure  the amount of backlash in your machine by

reversing direction and moving one step  at  a  time  until  the  tool

begins to move backwards (when the needle on the dial gauge reverses.)

When  running  the motor tests the backlash compensation should be

off so you can keep track of where the motor starts and stops.

HOME POSITION

In order for the automatic backlash compensation to  work  when  a

tool  path  file  is  executed  you  must  set the home position value

outside the working area of the tool.  The tool cannot (should not) go

"behind" the home position.  The tool should be jogged into  the  home

position  such  that the backlash will be "in front" of the tool as it

moves from home to the first point in the tool path  file  (the  first

thing  DANCAM or DANPLOT will do when executing a tool path is to take

up the backlash.) When  you  use  the  home  switches  the  tool  will

automatically  be  positioned  to  start  properly  for  the  backlash

compensation since the motors stop while moving  away  from  the  work

area.

MOTOR SPEED RAMPING

In  general  the  ramping  should  not be used with stepper motors

[64]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

except perhaps to  reduce  vibration  caused  by  rapid  starting  and

stopping.  If  you have a problem with to much inertia on your stepper

motors you should re-design your machine to use lighter materials.

PULSE RATE MULTIPLIER

The pulse rate multiplier in option #1 of the  configuration  menu

should  be set to send 1 pulse per step when normal stepper translator

modules are used.  Since micro-stepper and servo motor driver  modules

have a position counter in them they can accept several pulses at once

and  adjust  the  motor  speed to the average rate of pulses received.

The idea of the rate multiplier is to speed up servo motors that  have

excess  resolution  by  reducing the resolution to only what is needed

(each bunch of pulses turns the motor more than a single pulse would).

By using the pulse rate multiplier you can get servo motors up to 2000

RPM or faster and still have a resolution of 0.001" at the tool.

If you accidentally set the rate multiplier to some  number  other

than  1  your  normal  (200/400  step)  stepper  motors  will not turn

properly (or at all) unless you use a number that is a muliple of  the

step cycle (4 or 8) plus one, since the translator will rotate through

the  motor  phases  rapidly  before  stopping  on  one of the possible

positions.  If you need to mix  stepper  and  servo  motors  the  rate

multiplier can be set to a number like 5,  9,  or 13 (4+1, (2*4)+1, or

(3*4)+1), the stepper motor would only make 1 step per pulse bunch and

the servo motor would make 5, 9, or 13 steps.

SOME OTHER THINGS THAT CAN GO WRONG

Since the pins used for  the  auxiliary  relays  can  be  used  in

different  ways  you  should  make  sure  that  all the prompts in the

configuration menu agree as to what should happen!

If you use auxiliary relay outputs C and D to pull up  the  switch

inputs then the auxiliary relay outputs C and D will need to be set to

default high (so that they will always put out 3 volts.)

If you use auxiliary relays A and B in DANPLOT for the C axis then

the auxiliary relays A and B should be set to default low and have the

state  (hi/low)  toggle line colors set to color numbers that will not

be used.

Your parallel port may not put out enough voltage to trigger solid

state relays.  In such a case a pull up resistor of  about  2.2K  Ohms

would be needed on the auxiliary relay outputs of the parallel port to

pull the outputs up to a 5 volt supply.

THE SOURCE OF PROBLEMS

You  need  to  separate  any  problems  you  are having into three

[65]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

groups:

1. Problems that are caused by DANCAM or DANPLOT.

2. Problems that are caused by the electronics.

3. Problems that are caused by the mechanical parts.

If you are sure  that  you  have  configured  DANCAM  and  DANPLOT

properly  you  should  assume  that  the  possitional  error is due to

electronic or mechanical causes.

If you can get the motors to come back  to  their  starting  point

when  you  do  the motor test #2 (start and stop) in DANCAM or DANPLOT

then the software and the electronics are working (and  therefore  the

problem  is  mechanical.)  If  the motors fail the start and stop test

then you need  to  isolate  the  cause.  There  are  several  possible

causes:

1. You  have the p.w.f.  to small,  so the pulses are not wide enough.

Enter a larger p.w.f. and test again.

2. You  have  electrical  noise  (E.M.I,   R.F.I.)   triggering   your

translator  modules.  Try  to  isolate  the  source  of  electronic

interference,  separate the step and direction wires from the motor

coil  wires.  Add low pass filters (or opto-isolators) on the input

of your translator modules.

3. Your  translator  modules  are  defective,  or  you  have  a  loose

connection with your power or motor coil wires.  If you connect the

motor wires wrong the motor will not turn  properly  or  will  turn

reverse of the desired direction.

4. You  have too much or too little current  going through the motors.

The  motors  will  not  work  unless  you have the coil current set

properly.

5. You have  too  little  friction  on  the  motor.  The  motors  will

resonate at certain speeds if you do not have  enough  friction  in

the load, and or have too much inertia.  Try reducing the low speed

motor current if you have excess torque.

6. Your motors are overloaded.  Use larger motors.

7. You have the step  and  direction  or  common  wires   reversed  or

otherwise hooked up to the wrong places.

8. You forgot to connect the common wires to all the necessary points.

Use separate ground wires going back to a common point.

When the problem is with the motor torque (or  p.w.f),  the  motor

will  normally  jump  to  a  multiple  of 4 whole steps off the proper

position.  If electrical interference in involved then the motor  will

be off any number of steps.

The  type  of  electronics  used  with  the motors will affect the

performance of the motors.  If you can  not  get  your  (medium  size)

motors  over  100  RPM  you  have probably done something wrong or are

using badly designed equipment.  Please do not modify the  circuits  I

have presented until you try them the way I have designed them.

If  you  can  see  that  the  motor shaft comes back to the proper

[66]

[      H E L P   W I T H   D A N C A M   A N D   D A N P L O T       ]

position when the start/stop test is done then any  problem  with  the

position of the tool is caused by your mechanical parts and not by the

software  or  electronics.  The only way to reliably know if the motor

shaft is in the proper position is to mark the shaft and the  face  of

the motor and look at the marks directly.

For  the most part DANCAM and DANPLOT work properly and you should

have no problems.  If you get stuck  as  to  what  is  wrong  you  can

contact me for help.  When you need to ask for help it tells me what I

need to improve in terms of explanation in the documentation,  or what

needs improvement in the way the programs operate.

[67]

[               E L E C T R O N I C   C I R C U I T S                ]

QUICK DISCUSSION OF IDEA

Although stepper and servo motor driver modules are available from

motor distributors all over the world,  you may want to build your own

in  order  to  save  a substantial amount of money.  Also you may find

that no commercially available product meets your needs as well  as  a

circuit you can easily modify and repair yourself.

STEPS TO CARRY OUT IDEA

1) Determine the speed and power requirements of your machine.

2) Purchase new, surplus, or used motors with enough torque.

3) Pick the appropriate driver circuit for the motors you purchased.

4) Buy the necessary parts to build the circuit.

5) Assemble the circuit.

6) Check the wiring to make sure you made no mistakes.

7) Test the circuit.

8) Hook up the circuit to your parallel port.

9) Use DANCAM/DANPLOT's motor test utilities to configure your set up.

DETAILED DISCUSSION OF IDEA

The ability to build,  repair and modify a motor driver translator

module yourself,  rather than just buy a ready made module,  can  save

you  money.  Stepper  translator  module  manufactures  have  taken to

"potting" their modules in black plastic  so  that  if  anything  goes

wrong and a part burns out you are out the price of the whole unit!

If  you run the macro file BIPOLAR2.MAC (supplied on the shareware

disk with DANCAM v2.6 and DANPLOT v2.6) under DANCAD3D v2.6, schmatics

and printed circuit board foil patterns will be produced that you  can

use to build low cost Bi-Polar stepper motor translator driver circuit

boards  to automate your machine.  The circuit of the BIPOLAR2 circuit

board was developed to work well with DANCAM and DANPLOT,  have a  low

cost,  work  with most commonly available stepper motors,  and be made

from parts that are generally available in most countries.

Other circuits are described in the DANCAD3D user outfit.

Remember to double check any connections before you turn the power

on.  I assume that you have some experience building circuits,  if you

have no experience you might do better to buy ready built modules,  or

find a friend that builds circuits to help you.  Making changes to the

design of the circuit will probably reduce  performance,  so  try  the

circuit as given first.

STEPPER MOTOR TRANSLATOR MODULES

The  stepper  motor translator module takes the step and direction

[68]

[               E L E C T R O N I C   C I R C U I T S                ]

signals from the parallel port and switches the coils in  the  stepper

motor  on and off (or positive and negative) to make the stepper motor

turn.  The translator is really a counter that counts the pulses  from

the  parallel port.  When the direction pin is low (D < 0.3 volts) the

counter counts up,  i.e.  1-2-3-4-1-2-3-4 and  so  on,  and  when  the

direction  pin  is high (D > 2.8 volts) the counter counts down,  i.e.

1-4-3-2-1-4-3-2.  Counting up turns the motor clockwise, counting down

makes the motor turn counter clockwise.

To reduce the step angle of the motor shaft from 90 degrees to 1.8

degrees most stepper motors have 50 groves cut in the surface  of  the

rotor.  By  also  grooving  the poles of the four coils the motor will

give 200 steps per revolution, i.e.  every fourth step moves one grove

width  because  the groves on the successive poles are 1/4 of 50 parts

per revolution advanced.  So step 1's pole is 0/200 advanced, step 2's

pole is 1/200 advanced, step 3's pole is 2/200 advanced,  and step 4's

pole  is  3/200 advanced (step 5 lines up at pole 1 again but with the

rotor one groove ahead.)

FULL STEP MODES

There are two ways (for  unipolar  drive)  that  you  can  make  a

stepper  motor  advance  a  "full"  step.  You  can sequence the motor

windings one at a time, i.e.  1-2-3-4 or you can sequence two windings

at  a time,  i.e.  1&2-2&3-3&4-4&1.  When you sequence two windings on

you get more torque and fewer lost steps.

HALF STEPPING THE MOTORS

If you alternate between the one coil on and two coil on full  step

modes  you  can  make  the motor half step and get twice the number of

steps per revolution,  e.g.  400 in place of 200.  Half stepping gives

as much torque as the one coil on full step mode.  The counting 8 step

sequence for half stepping goes like this: 1-1&2-2-2&3-3-3&4-4-4&1.

UNIPOLAR AND BIPOLAR

The  counter  portion  of  the  steeper translator module needs to

connect to a power amplifier  to  supply  the  current  to  the  motor

windings.  There  are  two  types  of  amplifiers you can use to drive

stepper motors.

A unipolar amplifier will supply the coils with  positive  current

when  the  coil  is to be on,  or high impedance when the coil is off.

Since the stepper motor's coils  that  are  off  will  "pick  up"  the

generated  power  from the rotors movement the unipolar amplifier must

divert the stray current from the off windings  without  reducing  the

impedance  to  a  level  which  would  act as a brake on the motor and

prevent the motor from running at high RPM.  Unipolar  amplifiers  are

the simplest to build and operate from a single positive power supply.

[69]

[               E L E C T R O N I C   C I R C U I T S                ]

Bipolar amplifiers generate more torque from the stepper motors by

applying both positive pulling power and negative pushing power to the

motor windings.  Bipolar amplifiers generally require more transistors

than  unipolar  amplifiers,  but  since transistors are less expensive

now, most commercially built drivers now use bipolar drive.

Since I only had room for one circuit board on the shareware disks

I have designed a Bi-Polar full step  board.  This  board  gives  good

speed and torque on medium size motors.

CONSTANT CURRENT

Because  the  motor  windings are a coil (an inductor) the current

moving through the coil decreases as  the  frequency  (speed)  of  the

motor  steps  increases  (when  a  constant  voltage  source is used.)

Because the torque  is  roughly  proportional  to  the  current  going

through  the motor coils you will want to have the voltage supplied to

the motor increase as the motor step frequency increases in  order  to

hold the motor coil current constant (Ohm's law, E/(I*R).) If you used

a  raised  the  voltage  all  the time the motor would burn up when it

slowed down or stopped.  The solution to getting fairly constant  coil

current  is  to put a resistor in series with the motor coil such that

the resistor limits the current when  the  motor  is  stopped  to  the

motor's rated current, but lets you use a higher supply voltage (about

6  times  the  motors  rated  voltage) so that the motor will get more

voltage when the motor steps quickly.

The reason the resistor gimmick works is that the resistor and the

motor coil form a  voltage  divider.  When  the  translator  amplifier

switches  the  power  on  to a coil the inductance of the coil is very

high,  so no mater how big the series resistor is,  (nearly)  all  the

voltage  is dropped across the motor's coil.  As time goes on (about a

hundredth of a second) the impedance (resistance) of  the  coil  drops

and  more,  and  more,  of the voltage is dropped across the resistor.

After about a tenth of a second the impedance of the  coil  stabilizes

and  the  coil acts like a resistor in series with the series resistor

dividing the voltage to the value required  for  the  motor's  holding

torque current rating.

To  improve  the  high  speed  torque further the BIPOLAR2 circuit

board has  some  transistors  arranged  in  parallel  to  the  current

limiting  resistors  that  "short  circuit" the motor current limiting

resistors for about a thousandth of a second each time the polarity of

the power through the resistors reverses.  The value of the capacitors

between the collector and base of those transistors controls how  long

the "short circuit" lasts.

To  calculate  the  value  of the power resistors to use in series

with the stepper motor winding, to limit the current, you can use this

equation:

[70]

[               E L E C T R O N I C   C I R C U I T S                ]

Ro = ((Vs - Vd) - Vm) / Im

WHERE:   Ro = Series resistor value in ohms.

Vs = Motor coil supply voltage, larger than Vd + Vm.

Vd = Voltage drop from transistor & diode, about 1 to 2 volt.

Vm = Rated voltage for motor when stopped.

Im = Rated current (amperes) for motor when stopped.

To calculate the wattage rating for the series  resistor  use  the

following equation:

Rw = ((Vs - Vd) - Vm) * Im

WHERE:   Rw = Series resistor value in watts.

Vs = Motor coil supply voltage larger than Vd + Vm.

Vd = Voltage drop from transistor & diode, about 1 to 2 volt.

Vm = Rated voltage for motor when stopped.

Im = Rated current (amperes) for motor when stopped.

In  practice  Vs will be about six times the motors rated voltage.

Vd would only be significant when Vs is less than about 20 volts.  The

motor and series resistors get hottest when the motor is stopped.

An example may help you work out the values for the resistors that

go with your motors.  Assume we have a motor rated at  9.6  volts  and

2.1  amperes.  You  are  going to use a supply voltage of +/-45 volts,

and your translator power amplifier circuit drops  1.4  volts  in  its

semiconductors.

EXAMPLE: Ro = ((45 - 1.4) - 9.6) / 2.1

Ro = (43.6 - 9.6) / 2.1

Ro = 34 / 2.1

Ro = 16 ohms

Rw = ((45 -1.4) -9.6) * 2.1

Rw = (43.6 - 9.6) * 2.1

Rw = 34 * 2.1

Rw = 71.4 watts

To assemble the BIPOLAR2 circuit board use a low wattage, 27 to 30

Watt, pencil type soldering iron, and rosin core tin/lead alloy solder

to make the connections.

To  transfer the foil pattern (created by BIPOLAR2.MAC) to a blank

copper clad circuit board,  you can  use  a  laser  printer,  or  copy

machine,  to print the pattern onto the special dry transfer materials

now available.  Then iron the toner from the transfer material to  the

[71]

[               E L E C T R O N I C   C I R C U I T S                ]

board, and etch and drill in the usual way.

The  macro  BIPOLAR2.MAC  makes  five  DANCAD3D ASCII type drawing

files:

BP2FOILS.ASC Foil patterns for both sides of circuit board.

BP2PARTS.ASC Parts placement and identification drawing.

BP2SHOLE.ASC Small (0.047", 1.2mm) hole placement.

BP2MHOLE.ASC Medium (0.0625", 1.6mm) hole placement.

BP2LHOLE.ASC Large (0.128", 3.25mm) hole placement.

BP2SUPLY.ASC Schmatic for two types of power supply.

FOIL PATTERNS

The foil patterns (file BP2FOILS.ASC) for the component  and  back

of  the  board  are  included as mirror images so you can iron them on

without having to flip them.  Special attention was made to  have  the

points  where the traces connect from the component to the back have a

sizable tolerance for misregistration.  Just solder a  piece  of  wire

from  the  component trace to the back trace through the hole.  If you

want to make the board "single sided" you can  replace  the  component

side  traces  with jumpers.  Several registration marks are located at

the corners so you can use different size blank boards: 4" by 6", 4.5"

by 6", 4.5" by 6.25", and the recommended size of 4.5" by 6.334".  The

meaning of the values indicated by the abbreviations at the  terminals

is:

STEP.....Step signal from parallel port.

DIR......Direction signal from parallel port.

S-COM....Signal common to parallel port, and the +5 volt supply.

+5V......Positive five volt supply for logic.

COM......Common for motor coil power supply.

+V1......+45 volts from motor coil supply (actually supply center).

+V2......+90 volts from motor coil supply.

RED......"Red" coil lead from the stepper motor.

R/W......"Red & White" lead from the red coil of the stepper motor.

GRE......"Green" coil lead from the stepper motor.

G/W......"Green & White" lead from the green coil of the stepper motor.

R2B......One end of power resistor R2.

R2A......The other end of power resistor R2.

R1B......One end of power resistor R1.

R1A......The other end of power resistor R1.

FOUR LEAD MOTORS

Four  lead  stepper  motors  have two coils,  one coil goes to the

"RED" and "R/W" terminals,  and the other coil goes to the  "GRE"  and

"G/W"  terminals.  Three,  five,  six,  and  eight  lead motors can be

connected as follows:

[72]

[               E L E C T R O N I C   C I R C U I T S                ]

THREE LEAD MOTORS

"RED" = RED lead.

"R/W" = WHITE lead (center tap of coils.)

"GRE" = BLACK lead.

FIVE LEAD MOTORS

"RED" = RED lead.

"R/W" = WHITE lead (common of all four coils.)

"GRE" = GREEN lead.

The GREEN & WHITE,  and RED & WHITE leads are not  connected  when

five lead motors are used.

SIX LEAD MOTORS

"RED" = RED lead.

"R/W" = BLACK lead (center tap for red coil.)

"GRE" = GREEN lead.

"G/W" = WHITE lead (center tap for green coil.)

The GREEN & WHITE,  and RED & WHITE leads are not  connected  when

six  lead  motors are used.  It is possible to get more torque on some

six lead motors by using the full coil, but the motor will turn faster

if the half coil connections are used.

EIGHT LEAD MOTORS

Eight lead motors can be hooked up with the  coils  hooked  up  in

parallel or series, you will need to look at the manufactures drawings

and  figure  out  the  voltage  and  current for different connections

(parallel draws four times the current of series connection.)

PARTS

The drawing in file BP2PARTS.ASC shows the placement of the of the

components on the board.  Where you buy the parts will  make  a  large

difference  in  the  final  cost  of building this board.  If you shop

carefully the board can be build for less than about $30.  Mail  order

surplus  electronic suppliers are generally best,  but even among them

there is a large price spread,  the TIP122's for instance can be found

at prices ranging from $0.35 to $0.75, which can add up since you will

need 24 of them to build three boards (.75*24)-(.35*24) = $9.60.

[73]

[               E L E C T R O N I C   C I R C U I T S                ]

PARTS LIST FOR CIRCUIT BIPOLAR2

PART   QUANTITY  DESCRIPTION

R1,R2     2      Power resistors to limit motor current, see text.

R3,R4     2      100 ohm 0.25 watt.

R5,R6,

R7        3      2.2K ohm 0.25 watt.

R8,R9,

R10,R11,

R12,R13,

R14       7      1K ohm 0.25 watt.

R15,R16   2      2.2K ohm 5 watt.

R17,R18,

R19,R20   4      300 ohm 0.25 watt, see text.

C1,C2     2      1000 pf 10 volt ceramic capacitor.

C3,C4     2      100 mf 6 volt electrolytic capacitor.

C5        1      0.1mf 10 volt ceramic capacitor.

C6,C7,

C8,C9     4      1 mf 63 volt electrolytic capacitor, see text.

D1,D2,

D3,D4     4      LED type XC556R

D5,D6,

D7,D8     4      1N4004 rectifier

D9,D10,

D11,D12   4      1N5404 rectifier

Q1,Q2     2      TIP127 100 volt PNP Darlington transistor.

Q3,Q4,

Q5,Q6,

Q7,Q8,

Q9,Q10    8      TIP122 100 volt NPN Darlington transistor.

U1        1      7414 TTL IC.

U2        1      7486 TTL IC.

U3        1      7476 TTL IC.

J1        1      4 screw terminal block 0.2" spaced PCB mount.

J2        1      12 screw terminal block 0.2" spaced PCB mount.

In addition you can use two 14 pin and one 16 pin IC sockets.  All

the transistors need large heat sinks,  except for Q3 and Q4.  The LED

D1 blinks for the step,  D2 is the direction,  and D3 &  D4  show  the

polarity of the power going out to the two motor coils.

With  the parts indicated motors rated up to 45 volts at 3 amperes

can be driven.  A cooling fan should be used on the heat sinks if  the

current  is  over  1 ampere.  Higher power parts can be substituted to

improve the power capacity.  High gain Darlington transistors must  be

used for the circuit to work.

The  value  of  R17-R20  affects  the  amount of high speed torque

boost.  Values form 1K ohm to 100  ohm  are  useful.  The  higher  the

value the more current the motor gets at high speed.  Start with small

values  and  increase  until the motor has sufficient torque.  Setting

[74]

[               E L E C T R O N I C   C I R C U I T S                ]

the high speed torque to high will make the motor over heat,  and  may

also cause lost steps due to over shooting.  C6-C9 can also be changed

to affect the amount of boost,  values from 2 mf to 0.5 mf are useful,

the larger value giving more boost.  A simple way to tell if the boost

transistors are working is to run the motor at  high  speed  (the  RPM

test)  and then see if the heat sinks on Q7-Q10 get warm.  If you have

an oscilloscope you can put a 0.1 ohm 10 watt resistor in series  with

the motor winding and put the oscilloscope across the 0.1 ohm resistor

to  measure  the  current  through the motor coil at different speeds.

Without the boost,  the current will fall off as the speed  increases,

and,  with  too much boost,  the current will more than double at full

speed.

The holding torque current is set by the power  resistors  R1  and

R2.  Use  the equations given above to find the correct resistance and

wattage (generally the values will be between 10 and 100 ohms at 30 to

200 watts depending on the motor you are using.)

Be sure you have a good signal  ground  wire  going  back  to  the

parallel  port,  otherwise  you will get spurious motor steps.  If you

get spurious steps try turning off the motor coil power  and  look  at

the  LED's  to  see  if the signals are getting to the board properly.

Electrical noise can "feedback" from the motor  coil  wires  into  the

step and direction inputs through the ground wire if you have not been

careful  about  using  separate  ground  wires for the signal and coil

power.

Bipolar drive of  a  stepper  motor  applies  either  positive  or

negative  current  to the two coils in the stepper motor all the time.

The sequence for bipolar drive of a stepper motor is:

--------------------------------------

|  BIPOLAR    COIL "A"  |  COIL "B"  |

--------------------------------------

|  STEP 1  |  POSITIVE  |  POSITIVE  |

|  STEP 2  |  NEGATIVE  |  POSITIVE  |

|  STEP 3  |  NEGATIVE  |  NEGATIVE  |

|  STEP 4  |  POSITIVE  |  NEGATIVE  |

--------------------------------------

Using bipolar drive has the advantage of giving more  torque  when

three  or  four  lead  stepper motors are used in part because all the

motor windings are constantly in use (in unipolar drive half to  three

quarters  of  the  windings are always off!) Five and six lead stepper

motors  can  also  be  driven  from  the  bipolar  drive   with   good

performance.

To  get  negative  power  from  two  positive supplies the circuit

BIPOLAR2 "floats" the common end of the motor coils at a voltage  half

way  between the maximum voltage and common.  When the switched end of

the motor coil  goes  high  the  voltage  flow  through  the  coil  is

[75]

[               E L E C T R O N I C   C I R C U I T S                ]

positive,  and  when  the  switched end of the motor coil goes low the

voltage flow through the coil is negative.  Two amplifiers are used in

the BIPOLAR2 circuit to operate the two motor coils.

HOLES

The files BP2SHOLE.ASC,  BP2MHOLE.ASC,  and  BP2LHOLE.ASC  can  be

taped  on  the  board to help you see where to drill the holes.  These

files can also be used with DANPLOT to automatically drill the boards.

POWER SUPPLY

The BIPOLAR2 circuit requires a  "split"  power  supply.  You  can

wire  two  commercially  built 45 volt power supplies in series to get

the +45, +90 volts required.  Really the +V1 terminal is the common of

a +/- 45 volt supply,  but I have connected the -45 volt  terminal  to

common,  and  "floated"  the  supply  common to +45 volts to allow the

circuit to drive three and five lead motors.  Be very careful that you

do not connect the supply incorrectly.

To avoid problems with commercially built supplies,  you can build

the  supply  circuits shown in the drawings SUPPLY2A and SUPPLY3A that

are in the file BP2SUPLY.ASC.  They are marked for:  COM, +V1, and +V2

to correspond to the markings on the BIPOLAR2 board.

AC  power  is  transmitted  in  the  form of a sine wave form at a

frequency of 50 Hz or 60 Hz.  To make DC  power  you  need  to  use  a

rectifier  and capacitor.  AC voltage is measured in RMS value usually

which means that the peek of the wave crest is higher in voltage  than

the  RMS value.  Because,  the rectified DC will have a higher voltage

than  the  stated  RMS  secondary  voltage,   you  need  to  select  a

transformer  that  has a RMS output less than the required DC voltage.

Generally you will want an amperage rating twice the required  current

to prevent excessive voltage drop under full load.

Vo = Vs * 1.414

WHERE:   Vo = Approximate output DC voltage.

Vs = Transformer secondary rated RMS AC voltage.

Power  supply  SUPPLY2A uses a 62 volt center taped transformer to

supply about +45 & +90 volts DC from terminal COM.  The rating of  the

transformer,  diodes  and  capacitors  needs  to  be  adjusted to your

amperage requirements.

[76]

[               E L E C T R O N I C   C I R C U I T S                ]

PARTS LIST FOR CIRCUIT SUPPLY2A

PART   QUANTITY  DESCRIPTION

1      AC line cord.

T1        1      62 volt RMS, center tap secondary, transformer.

D1-D4     4      Rectifier, e.g. 25 ampere at 400 piv.

C1,C2     2      4000 mf per ampere, 75 volt.

R1,R2     2      47K ohm at 1 watt resistor.

Power supply SUPPLY3A uses a 31 volt transformer to  supply  about

+45  & +90 volts DC from terminal COM.  Only two diodes are used.  The

value of the capacitors needs to  be  increased  since  SUPPLY3A  uses

"half wave" rectification.  The rating of the transformer,  diodes and

capacitors needs to be adjusted to your amperage requirements.

PARTS LIST FOR CIRCUIT SUPPLY3A

PART   QUANTITY  DESCRIPTION

1      AC line cord.

T1        1      31 volt RMS transformer.

D1-D2     2      Rectifier, e.g. 25 ampere at 400 piv.

C1,C2     2      10000 mf per ampere, 75 volt.

R1,R2     2      47K ohm at 1 watt resistor.

USED AND SURPLUS STEPPER MOTORS

Used and surplus stepper motors can be found for prices much  less

than new motors.  Since stepper motors do not have brushes or contacts

they  frequently  "out  live" the machine they were installed in.  The

usual failure mode is for the bearings to get sloppy with wear, making

the rotor rub, and the motor to stall.

If you do not know the stepper motor's rated  current  or  voltage

you can figure them out by experiment.  Use a power supply of about 24

volts  at  5 amperes.  Connect one of the motor coils in series with a

large power resistor,  about 200 ohms  at  100  watts,  to  the  power

supply.  Try  turning  the  motor shaft.  If the shaft is hard to turn

leave the resistance  at  its  correct  value,  otherwise  reduce  the

resistance until you have some difficulty turning the motor shaft with

your  fingers.  Put  some  small  wooden blocks on the jaws of locking

pliers (e.g.  Vice-Grips (tm)) and lock the pliers on the motor shaft.

Use a spring type fish scale (the kind that has a hook on one  end  to

weigh  a  fish  you  have caught) to pull on end of the locking pliers

(with the motor shaft pointing up and the fish scale  gong  away  from

you)  to  measure  the  torque  required to turn the motor shaft.  The

weight you measure when the  motor  shaft  begins  to  move,  and  the

distance  that  the  scale  is  hooked on from the center of the motor

shaft give you  the  motors  holding  torque.  For  instance  if,  you

[77]

[               E L E C T R O N I C   C I R C U I T S                ]

measure  0.5  pound  at a distance of 6 inches the motor torque at the

applied current is:  (0.5*16)*6=48 oz-in.  You can estimate the amount

of  holding  torque  a given motor was meant to produce from the shaft

size:

-------------------------------------------------

|   SHAFT SIZE   |   ESTIMATED HOLDING TORQUE   |

|----------------|------------------------------|

|     0.250"     |        20 -  50 oz-in        |

|     0.375"     |        50 - 150 oz-in        |

|     0.500"     |       150 - 500 oz-in        |

|     0.750"     |       500 - 900 oz-in        |

-------------------------------------------------

If you seem to be getting about half of the lower expected  torque

value  leave  the  series  resistance  where  you  have it,  otherwise

decrease the series resistance somewhat  and  re-measure  the  holding

torque.  When you get the holding torque up to half the expected value

leave  the  motor  on  for  a  while  and  monitor  the  motor's  case

temperature.  The motor should get hot only after it has been  on  for

two  hours.  If  the case temperature gets over 75C,  or you smell the

motor burning inside,  turn off the  power  and  increase  the  series

resistance.  If  the  motor  does  not  get hot after being on for two

hours decrease the series resistance and monitor the temperature rise.

Once you find the resistance that lets the motor rise to about 75C

and level off at that temperature you can re-measure the motor torque,

and also measure the voltage being applied to the  motor.  Use  a  VOM

meter  to  measure the voltage across (in parallel to) the motor leads

of the coil you have energized.  By using ohms  law,  E/R=I,  you  can

figure  the  motor coil current by dividing the voltage on the coil by

the resistance of the coil (measure the resistance, of the coil,  with

the  coil  disconnected!) You can also measure the current directly if

your meter has a 10 ampere scale,  and you place the meter  in  series

with the motor coil and resistor while the power is applied.

SERVO MOTORS

The  precision  of  positing  of servo systems is a complex issue.

Many manufactures state misleading and exaggerated  specifications  of

the  positioning  accuracy  (do not confuse the position at the end of

movement with the instantaneous position during the actual  movement!)

Do not confuse resolution with accuracy, resolution has to do with the

number  of encoder lines,  accuracy includes all mechanical and timing

errors that occur.  To truly measure the position of the tool  at  any

given  instant  you need to use a stroboscope precisely timed relative

to a reference.  The instantaneous positioning accuracy generally gets

worse as the motor is run faster (as opposed to stepper  motors  where

the accuracy is constant up to the point where the motor fails to step

accurately.)  Since  servo  motors can "catch up" after falling behind

[78]

[               E L E C T R O N I C   C I R C U I T S                ]

slightly most users are never aware of the degree to  which  the  tool

was out of position.

One  major  advantage  of  using  a servo motor over using stepper

motors is the ability of the servo  motor  to  recover  from  overload

conditions  and  continue  to  stop  at  the  proper  destination (the

electronics always know where the servo motor's shaft is  and  try  to

correct possitional errors.) Stepper motors once they have lost a step

will  be  off position for the rest of the job.  The ability of the DC

servo to not accumulate errors the way that a stepper motor will  when

overloaded and run too quickly allows servo motors to be operated with

a  smaller  margin  of  safety  (and therefore servo motors can be run

faster.)

I recommend that you use stepper motors on the first machine  that

you  automate  for  use  with  DANCAM  or DANPLOT since your chance of

success and satisfaction are higher.  Servo motors  should  mostly  be

used for these applications:

* Where only one axis is moving while the cut is being performed, e.g.

operations where many holes need to be drilled quickly.

* Where the tool will need to move more than three  feet  rapidly  and

frequently,  e.g.  in a large torch cutting machine, or when an over

head crane is being automated.

* Where high speed is absolutely necessary for the use of the machine,

such as in spray painting or other  applications  where  possitional

accuracy of +/-0.02" would be acceptable at high speed.

* Where   cumulative positioning errors are not acceptable,  but brief

intermittent  lead or lag in position is acceptable,  e.g.  in heavy

machine tools were the load is heavy and varies  during  rough  cuts

but is light and constant during the final cut at slower speeds.

For  most  other applications it is better to have the machine use

stepper motors and run a little slower.  Generally you will get better

parts (products) if you have two machines making parts slowly than  if

you  have  one  machine  making  parts twice as fast.  It is simple to

connect two or more stepper driver modules to run two or more machines

from one computer (use a 7407 buffer chip and some pull-up resistors.)

HOW CIRCUIT SERVO CIRCUITS WORK

Servo circuits have two large counters  that  keep  track  of  the

destination  position  and current motor shaft position.  The step and

direction signals from the computer make the destination counter count

up or down to record the desired  destination  position.  The  encoder

channel  "A"  and  "B" signals from the optical encoder mounted on the

servo motors rear shaft make the encoder position counter count up and

down to record the actual current position of the motor's  shaft.  The

count  recorded in the two large counters is continuously compared and

[79]

[               E L E C T R O N I C   C I R C U I T S                ]

if the destination is found to be more cw than the shaft position  the

motor  coils  are given positive current.  If the destination is found

to be less cw (higher values are more cw) than  the  shaft's  position

negative current is sent to the motor windings.

To  make a stable servo circuit the amount of positive or negative

current sent to the motor windings needs to  be  proportional  to  the

amount  of  error (difference) in the motor shaft position,  i.e.  the

larger the difference between the position counters the  more  current

that is sent to the motor.

Additionally  to  make  the  servo  circuit stable there must be a

control in the rate with which the amount of current sent to the motor

changes,  such that the rate of current change is slow when the amount

of position error small, and the rate of current change is faster when

the amount of position error is large.

If the possitional error becomes very large the servo circuit will

need  to put the pause/hold line on the parallel port high to stop the

computer from sending any more step pulses so that the motor shaft can

catch up with the destination counter.  When the motor can "talk back"

to the computer in this way the motor is  said  to  be  working  in  a

"closed loop" operation.  Closed loop operation allows the computer to

respond  to  the  load the motor is trying to move by slowing down the

rate  of  movement  requested  for  heavy  loads   (this   is   called

"sensitivity to load.")

The  motor  speed ramping feature should be used when servo motors

are driven since servo motors do not start  and  stop  as  rapidly  as

stepper motors.  The rate multiplier should be used to set the minimum

resolution  to the largest acceptable quantity to insure the best high

speed operation, e.g.  if your servo circuit has a resolution of 10000

steps  to the inch,  and you only need 0.002 resolution,  set the rate

multiplier to 20 to make the top feed rate 20 times faster.

There is not enough room on these shareware disks to go into  more

detail  about  using  servo motors.  Commercially built servo circuits

are available that read step and direction signals.  Plans  for  a  DC

brush  type  servo  drive circuit are in the CAM manual and video that

come with the DANCAD3D user outfit.  I plan  on  developing  some  new

servo  circuits  in  the  coming  years (I am writing this in 1994) so

write to me if you have an interest in driving servo motors.

[80]

[                 W H E R E   T O   G E T   P A R T S                ]

WHERE TO GET PARTS

Where you buy your parts for building your automation project  can

have  a huge impact on the cost of your project.  I recently purchased

some 750 oz-in stepper motors for $20  each  that  were  removed  from

photo copy machines, if I had purchased motors of comparable torque as

new  through  a  manufactures representative I might have paid several

hundred dollars each!

Go to your local  magazine  store  and  buy  the  electronic,  and

mechanical  hobby  magazines (or just go to the library and photo copy

the useful pages.) In the back of such  magazines  are  advertisements

for  companies  selling surplus and used parts.  Write to the discount

parts companies and request their catalogs.

The  DANCAD3D  user  outfit  includes  a  useful  list  of  names,

addresses, and phone numbers, of parts and tooling sources that relate

to using DANCAM and DANPLOT.

Look  in  your  local  phone  book  under "Power transmission" and

"Motion control" as well as "Motors", "Electronic,  Resale",  "Machine

Tools",  "Automation",  "Metal",  "Salvage",  "Surplus",  and  similar

categories.

Amateur radio operators have "swap meets"  around  the  U.S.  from

time  to time,  try asking your local A.R.R.L.  representative if they

have a "swap meet" in your state.

[81]

[                R E C O M M E N D E D   R E A D I N G               ]

RECOMMENDED READING

I am offering this list of books to supplement the information  in

this  document.  Doing  machine  work  is  a lot of fun,  and building

electronic circuits yourself helps you understand how  to  repair  the

circuits  when  they  fail.  Also  you  can save yourself thousands of

dollars by doing the work yourself.  I hope you enjoy  learning  about

automation and automated manufacturing.

MACHINE TOOLS

Argus Books Limited,  1 Golden Square,  London,  W1R 3AB, England,

prints a series of books for the home workshop that are very clear and

well written.  You can write to them for a full list.  These  are  two

books I found useful in learning about milling operations:

Workshop Practice Series #2

"Vertical Milling in the Home Workshop"

By Arnold Throp

(c) Argus Books Ltd. 1977

ISBN 0 85242 843 X

Workshop Practice Series #5

"Milling Operations in the Lathe"

By Tubal Cain

(c) Argus Books Ltd. 1984

ISBN 0 85242 840 5

ELECTRONICS

For  electronics  knowledge Radio Shack stores have a selection of

books that show you  how  to  build  electronic  circuits.  Since  the

parallel  port  has  TTL compatible outputs,  and the BIPOLAR2 circuit

uses TTL integrated circuits you should read the TTL Cookbook.

SAMS 21035

"TTL Cookbook"

By Don Lancaster

(c) 1974 by Howard W. Sams & Co.

ISBN 0-672-21035-5

While a detailed understanding of electronics is not  required  to

build  electronic  circuits from a drawing such knowledge is a help in

figuring out why the circuit does not work properly!  The best book  I

know  about  as a general introduction to electronics is the ARRL "The

Radio Amateur's Hand Book." The ARRL (American Radio Relay League)  is

a group of HAM radio operators that: negotiate with the FCC, on behalf

of  radio amateurs,  for use of space in the Radio Frequency Spectrum,

and  publish  material  of  interest  to  radio  amateurs.  The  "ARRL

Handbook"  is  updated  yearly  to  cover advancements in electronics.

[82]

[                R E C O M M E N D E D   R E A D I N G               ]

There are sections on electronic theory,  how to build  circuits,  and

example  radio  circuits.  You  should  be  able to find a copy of the

handbook at your local library.

The catalogs published by motion control companies are also a good

source  of  information.   Most  manufactures  have   free   technical

information  available on their products that they will send to you if

you ask.

Best wishes for success in your project.  Feel free to write to me

for information about product improvements and related matters.

[83]