X-10 Professional Developers Library

For

C

=========================================

|                                       |

|               HINTS.DOC               |

|                                       |

=========================================

Version 3.1

Manual and Software Copyright 1993, 1994

Smart Home Technology

24123 94th Ave. S.

Kent,  WA  98031

(206) 813-8410

ALL RIGHTS RESERVED

QUESTIONS, ANSWERS, AND HINTS

Q. When I dim a lamp with a controller that has the same address as the

module, the library does not seem to keep track of its level.  What's

wrong?

A. Nothing is wrong.  Dim commands from a controller come out in a

continuous stream, yet the TW523 interface module was designed only to

tell the computer about every third dim command in a continuous

stream.  So if you hold the controller dim button down continuously

then the library will only recognize every third command and adjust

its values as such.  One way to get around this is to not hold the dim

button down continuously.  If you press it once it sends out one dim

command.  Press it again it sends out another. Press it again it sends

out another.  Do this until the lamp is at the level that you want. If

you do it this way the library can accurately keep track of the level.

Q. What actually happens when the library recovers from a power outage?

A. The power outage recovery is a very complicated item.  I will try and

explain it the best I can so you won't get confused.

One of the first things you'll need to know and change on your

computer is to have the name of your automation program the last

called program in your autoexec.bat file.  That way when power is

restored and the computer starts, your program will automatically

start itself and proceed with the recovery.  All this will happen

automatically if its in your autoexec.bat file.

When you call the function open_tw523("file.sta"), that function will

see if that file is located on disk.  If the file is there it

identifies that the program was terminated abnormally, which for all

purposes, was a power outage.  The function will then set an internal

flag, PowerOutage, to TRUE.  If the file was not there then the flag

would be set to FALSE.

Now since we have a flag telling us that there was an outage we can do

an if statement, if (PowerOutage), to do a catchup.  Our shell program

gives you an example of this.  In that if block we call a function

play_catchup("file.sta"),  this function sets more internal flags,

disables the com port from transmitting or receiving x10 commands, and

loads the status and timer tables located in the file "file.sta".  It

will also reset the internal X10Time, X10Date and X10Dow variables

back to the time of the last saved status tables.

Since flags and dates and times have been reset, the function

get_x10time() has all of the workings of accelerating through the

program.  It will quickly go through the schedule in minute

increments.  No X10 commands will be sent out as we disabled that

function earlier.

After the internal X10Time and X10Date catches up to the current time

reported by the computer we flush all transmit and receive buffers,

enable the com port to transmit and receive X10 commands, and set all

internal flags to show there is no power outage and we are not

catching up.  Then we do one of two things, if your program has not

identified a custom refresh for outages, by using the function

set_catchup_refresh(), we will do a call to the library function

refresh_entire_system().  If you did identify a custom refresh we

would call your function only.

If you are using the default refresh_entire_system(), here is what is

basically going to happen.  The library will go thru each module in

the status table, starting at A1 going thru P16.  If there is a module

identified and its not a controller and its refreshable, we refresh

the module to where it should be according to the status tables.  If

the module is identified and its not a controller and its not

refreshable, we refresh the module according to what you had put as

the initial level in the function init_modules() in your startup

routine.

It all sounds pretty complicated and hard to follow but it really

isn't to bad.  It works.  And you really don't need to know anything

about it to have it work for you.

H. When you want to dim a lamp from a controller press the on button for

that particular lamp first and then press the dim button repeatedly

until the level is reached.  This will make sure that the lamp you

want is really the currently selected one.  If you don't do this funny

things might happen since you really do not know what modules are

waiting for that dim command.  So play it safe and always press the on

button first.  This is mainly if you have controllers that are

directly controlling modules.

H. Try to use controllers to turn things on and off.  It takes a little

while to get in the habit of doing this.  But modules themselves do

not send commands out telling what is happening to them.  So if you

turn on a module with its switch no commands are sent and the library

will not know what has happened.  Of course if your just going into a

room and you want the light on for a while then turn it on when you

enter and turn it off when you leave.  But if that module will be

controlled at some point then the library will need to keep track of

its status.  One way to reset things is to turn the module off from a

controller after you have finished with it.  This will accurately set

its status within the library.

H. You can make your program more efficient by using if statements in

conjunction with the functions get_x10cmd() and get_x10time().  Here

are a couple of examples:

if (get_x10cmd(&X10Cmd))

chk_ctrler_events();

With this example the function chk_ctrler_events() will only be called

if a new x10 command was received.  You would then create the

chk_ctrler_events() function where you would have all the code to

respond to x10 controller buttons.  This way the program does not have

to continually go thru all of the if statements whether there was a

new x10 command or not.

Another example is to do the same type of thing only with the

get_x10time() function.  For example:

if (get_x10time())

chk_time_events();

With this example the function chk_time_events() will only be called

when a new minute has arrived.  You would then create the function

that would have the code to respond to time of day events.  Again this

way the program does not have to continually go thru all of the time

if statements whether its a new minute or not.