Subject: DosIps (vs 1.6)  CPU Analyzer for 1996 PC Processors

Date: January 1996

From: 103644.235@CompuServe.Com

ASP ID#: 9902

File: DosIps.Com   (792 bytes)

Author: A.DePaul

Equipt: PC Compatible, MsDos 4 or later

FileType: Zipped .COM & .DOC

PgmType: Copyrighted ShareWare

1996 is here, bringing some exciting new CPU's to the PC compatible arena.

All of which are optimized for 32 bit Operating Systems with improvements

in pre-fetching, cacheing and pipelining designed to make these new

CPU's virtual data scoops. Along with the 32 bit optimizations is the

question of 16 bit performance relative to Dos and Win3.x users, who

make up a large segment of the PC world.

I for one, am a "Dosser" and Win3.x user who will most likely be happy

with the 16 bit environment right into the year 2000 and beyond.

I thought a "shirt pocket" Super CPU evaluation program would be a handy

tool for evaluating these new processors relative to their performance

at the 16 bit level. For that evaluation I chose two methods of collating

the execution speed index:

First and foremost is the number of Dos 16 bit instructions executed per

second, which translates to MIPS or Millions of Instructions executed per

second. To scale the resulting MIPS values to  plain English, I chose to

cross reference the MIPS value to a theoretical or Virtual 486 processor

clock rate, required to produce the same results as the machine under test.

The 486 25Mhz processor was chosen as the origin of the scale, or reference

CPU being the bedrock processor of the current offerings, which turns 11

mips at 25 Mhz.

The 1996 150 Mhz and faster CPU's are for the most part combinatorial

486's intended for concurrent processing of larger instruction streams.

Boiling this all down, points to the 486 as being the logical choice for

a standard by which to judge execution speed of evolving CPU's decendant

from that base.

The test eliminates effects of cache coherrency, wait states and

pipelining to focus on the internal register execution times as the

prime point of interest, as register operations underly all speed

enhancements to any processor thru pipelining, cacheing, etc.

The program is under 1,000 bytes and is intended to be launched from a

carry-around 3 1/2 inch disk, which can serve as a test probe for anyone

considering a given machine for purchase, by testing several machines

and comparing the results with your current machine. The program runs

it's test in microseconds and does not need to be installed on the

machine you're checking, so sales personnel should not have any objection

to test driving a machine under consideration for purchase.

Readout is on screen using standard Dos calls, so any PC compatible

platform running under Msdos 4 or later will support DosIps.

If you are considering a cross platform system that emulates Dos, DosIps

will give you an exact picture of the emulator's performance.

An additional use is the verification of system integrity on boot-up

by placing the file in your Dos directory and including

DosIps

in your autoexec file. As a troubleshooting and catalogging tool, it can

greatly help service personnel in quick evaluation of a system's state

of operation at the base level.

DosIps is not claimed to be the final word in CPU testing, but rather a

portable and accurate gauge by which to test existing and future CPU's

bred from the 486 mold. DosIps will test 486 and later CPU's running from

25Mhz thru 500Mhz clock rates.

DosIps.COM is a copyrighted work. No warranty other than replacement of

defective copy is attached to this software product.

Registration for this program is thru Compuserve on-line Shareware services.

Registration entitles registrant to a single user license. Support is via

CompuServe Email @ 103644.235. Price is $19.95.

or

U.S. Mail check/money order to:

A.DePaul

410 West 53rd st./203

NY NY 10019

Include name&address for mailings

Thank you for your purchase and for supporting the Shareware concept.