***********************  FLOATING-POINT DATA  *******************************

ASM32 floating-point data subroutines (C) Copyright 1994 Douglas Herr

all rights reserved

ASM32 recognizes two floating point formats: IEEE-standard single-

precision, and IEEE-standard double-precision.  Single-precision

floating point numbers are 4 bytes long, and are referred to as float4

or simply f4, while double-precision floating point numbers are 8 bytes

long, which I describe as float8 or f8.  This is the format used by the

Intel 80x87 math coprocessors.

ABSF4:     absolute value of float4 number

Source:    absf4.asm

Call with: DS:[EBX] pointing to 4-byte real number

Returns:   number converted to absolute value

Uses:      flags

Example:

include model.inc

extrn   absf4:near

include dataseg.inc

f4      dd -123.456

@curseg ends

include codeseg.inc

.

.

.

lea     ebx,f4           ; point to 4-byte real number

call    absf4            ; convert to absolute value

ABSF8:     absolute value of float8 number

Source:    absf8.asm

Call with: DS:[EBX] pointing to 8-byte real number

Returns:   number converted to absolute value

Uses:      flags

Example:

include model.inc

extrn   absf8:near

include dataseg.inc

f8      dd -123.456789

@curseg ends

include codeseg.inc

.

.

.

lea     bx,f8            ; point to 8-byte real number

call    absf8            ; convert to absolute value

ADDF4:     add two float4 numbers

Source:    addf4.asm

80x87 not required; no OS dependancies

Call with: DS:[ESI] pointing to number0, ES:[EDI] pointing to number1

both numbers must be float4 numbers in IEEE format

Returns:   if CF = 0, number0 is replaced by the sum

if CF = 1, the sum would result in an overflow or underflow;

neither number is changed

Uses:      flags

Example:

include model.inc

extrn addf4:near

include dataseg.inc

number0 dd 123.456

number1 dd -67.4

include codeseg.inc

.

.

.

push    ds

pop     es           ; ES = DS

lea     esi,number0

lea     edi,number1

call    addf4

; note that this can be used for subtraction by changing the sign

; of one of the numbers.

;

; changing the sign is trivial: if DS:[ESI] points to a float4 number,

; the sign is changed with

;

;      xor    byte ptr 3[esi],10000000b

ADDF8:     add two float8 numbers

Source:    addf8.asm

80x87 not required; no OS dependancies

Call with: DS:[ESI] pointing to number0, ES:[EDI] pointing to number1

both numbers must be float8 numbers in IEEE format

Returns:   if CF = 0, number0 is replaced by the sum

if CF = 1, the sum would result in an overflow or underflow;

neither number is changed

Uses:      CF

Example:

include model.inc

extrn addf8:near

include dataseg.inc

number0 dq 123.456e16

number1 dq -67.407865

include codeseg.inc

.

.

.

push    ds

pop     es           ; ES = DS

lea     esi,number0

lea     edi,number1

call    addf4

; note that this can be used for subtraction by changing the sign

; of one of the numbers.

;

; changing the sign is trivial: if DS:[ESI] points to a float8 number,

; the sign is changed with

;

;      xor    byte ptr 7[esi],10000000b

CMPF4:      compare two float4 numbers

Source:     cmpf4.asm

CMPF8:      compare two float8 numbers

Source:     cmpf8.asm

80x87 NOT required

Call with:  ESI pointing to number 0

EDI pointing to number 1

Returns:    if ZF = 1, numbers are equal

if SF = 1, number 1 is larger

Uses:       zero flag (ZF), sign flag (SF)

Example:

extrn   cmpf4:near

include dataseg.inc

v0      dd 12.345

v1      dd 17.04

@curseg ends

include codeseg.inc

.

.

.

lea    esi,v0            ; point ESI to number 0

lea    edi,v1            ; point EDI to number 1

call   cmpf4             ; compare

js     number1           ; v1 is larger if SF = 1

je     equal             ; numbers identical if ZF = 1

; otherwise v0 is larger

F4TOI2:     copies the integer portion of a float4 number to AX

Source:     f4toi2.asm

80x87 not required

Note:       only the integer portion of the number is converted; decimals

are truncated

Call with:  ESI pointing to a float4 number

Returns:    if CF = 0, AX = integer number

if CF = 1, float4 number is too large (if positive)

or is too small (if negtive)

Uses:       AX, CF; all other flags and registers are saved

Example:

extrn   f4toi2:near

include dataseg.inc

float4  dd  1234.567

@curseg ends

include codeseg.inc

.

.

.

lea  esi,float4

call f4toi2

jc   oops                     ; gotta fix something if error

F4TOI4:     copies the long integer portion of a float4 number to EAX

Source:     f4toi4.asm

80x87 not required

Note:       only the integer portion of the number is converted; decimals

are truncated

Call with:  ESI pointing to a float4 number

Returns:    if CF = 0, EAX = long integer number

if CF = 1, float4 number is too large (if positive)

or is too small (if negtive)

Uses:       EAX, CF; all other flags and registers are saved

Example:

extrn   f4toi4:near

include dataseg.inc

float4  dd  1234.567

@curseg ends

include codeseg.inc

.

.

.

lea    esi,float4

call   f4toi4

jc     oops                   ; gotta fix something if error

I4TOF4:     convert 4-byte integer to float4 IEEE format

Source:     i4tof4.asm

80x87 not required

Call with:  DS:[EBX] pointing to 4-byte buffer for float4 number

EAX = signed integer number to convert

Returns:    converted number in float4 IEEE format at DS:[EBX]

Uses:       nothing; all registers and flags are saved

Example:

include model.inc

extrn   i4tof4:near

include dataseg.inc

float4  dd ?                          ; initial number undefined

longint dd -123456789

include codeseg.inc

.

.

.

mov    eax,longint       ; signed integer

lea    ebx,float4        ; point to buffer for converted number

call   i4tof4

MAXF4, MAXF4b: find maximum value in single-precision real-number array

Source:        maxf4.asm

MINF4, MINF4b: find minimum value in single-precision real-number array

Source:        minf4.asm

80x87 not required

Call with:  EDI pointing to array element at start of search

ECX = number of array elements to search

For maxf4b or minf4b, call with EBX = byte increment between

array elements.  Maxf4 and minf4 assume byte increment = 4.

Returns:    EAX = array element number with maximum or minimum value

note that the offset of that number is EDI + (EAX shl 2).

With maxf4b and minf4b, the offset of the number is

EDI + (EAX * EBX).  CF = 1 if called with ECX = 0.

Uses:       EAX, CF

Example:

extrn  maxf4:near

include dataseg.inc

floatdata   dd 1500 dup(0)

@curseg ends

include codeseg.inc

; program provides values for the array

.

.

.

lea    edi,floatdata      ; EDI points to the data array

mov    ecx,1500           ; search entire array

call   maxf4              ; returns with EAX = array element

MAXF8, MAXF8b: find maximum value in double-precision real-number array

Source:        maxf8.asm

MINF8, MINF8b: find minimum value in double-precision real-number array

Source:        minf8.asm

80x87 not required

Call with:  EDI pointing to array element at start of search

ECX = number of array elements to search

For maxf8b or minf8b, call with EBX = byte increment between

array elements.  Maxf8 and minf8 assume byte increment = 8.

Returns:    EAX = array element number with maximum or minimum value

note that the offset of that number is EDI + (EAX shl 3)

With maxf8b and minf8b, the offset of the number is

EDI + (EAX * EBX).  CF = 1 if called with ECX = 0.

Uses:       EAX, CF

Example:

extrn  maxf8:near

include dataseg.inc

floatdata   dq 1500 dup(0)

@curseg ends

include codeseg.inc

; program provides values for the array

.

.

.

lea    edi,floatdata      ; EDI points to the data array

mov    ecx,1500           ; search entire array

call   maxf8              ; returns with EAX = array element

shl    eax,3

add    edi,eax            ; EDI points to maximum value

MULF4:      multiplies two float4 numbers

Source:     mulf4.asm

80x87 not required

Call with:  DS:[ESI] pointing to number0, ES:[EDI] pointing to number1

Returns:    if CF = 0, no error; product replaces number0 at DS:[ESI]

if CF = 1, the product is either zero or infinity (number0 is lost)

Uses:       CF; all other flags and registers are saved

Example:

include model.inc

extrn   mulf4:near

include dataseg.inc

number0 dd 1.234

number1 dd 0.5

@curseg ends

include codeseg.inc

.

.

.

push   ds

pop    es

lea    esi,number0

lea    edi,number1

call   mulf4

SCALEF4:    scale a float4 number by an integer power of 2

Source:     scalef4.asm

SCALEF8:    scale a float8 number by an integer power of 2

Source:     scalef8.asm

80x87 not required; no OS dependancies

Call with:  DS:[ESI] pointing to the float4 data

AL = scale factor (may be positive or negative)

Returns:    CF = 0 if success: number was properly scaled

CF = 1 if error: resulting number would have overflowed

number unchanged in this case

Uses:       CF; all other flags and all registers saved

Example:

include model.inc

include codeseg.inc

.

.

.

lea   esi,float4data

mov   al,3             ; scale by (2^3) effectively multiplies by 8

call  scalef4

jc    oops             ; take remedial action if error

SORTF4HI:    sort an array of single-precision real numbers, highest first

Source:      sortf4hi.asm

SORTF4LO:    sort an array of single-precision real numbers, lowest first

Source:      sortf4lo.asm

80x87 not required

Call with:  ES:[EDI] pointing to the first of the array elements to be

sorted, ECX = number of array elements

Returns:    nothing

Uses:       nothing; all registers and flags are saved

Example:

extrn   sortf4hi:near

include dataseg.inc

floatdata   dd 1500 dup(0)

@curseg	ends

include codeseg.inc

; program provides values for the array

.

.

lea     edi,floatdata

push    ds

pop     es

mov     ecx,1500           ; sort entire array

call    sortf4hi           ; highest number first

SORTF8HI:    sort an array of double-precision real numbers, highest first

Source:      sortf8hi.asm

SORTF8LO:    sort an array of double-precision real numbers, lowest first

Source:      sortf8lo.asm

80x87 not required

Call with:  ES:[EDI] pointing to the first of the array elements to be

sorted, ECX = number of array elements

Returns:    nothing

Uses:       nothing; all registers and flags are saved

Example:

extrn   sortf8hi:near

include dataseg.inc

floatdata   dq 1500 dup(?)

@curseg	ends

include codeseg.inc

; program provides values for the array

.

.

lea     edi,floatdata

push    ds

pop     es

mov     ecx,1500           ; sort entire array

call    sortf8hi           ; highest value first