IOSPY Copyright (c) Stephen L.Briggs 1996

IOSPY DOC

Have you ever wondered what control commands were being sent to or

recieved from the Peripherals installed inside your computer, such as

Video Processor, Network Adapter, Software Locks, Telephone Modem,

even Sound Cards...  via Input Output instructions?

Since these instructions are at the very heart of hardware control,

hours of hair pulling can go into the simplest hardware manipulation.

I started writing this program of couple of years ago because I wanted

to find out what I/O port range was being used by a device driver to

access an old network card.

Since then, I have added many features, such as incorporating the

internal debug registers of the 80386,486,586, so we could check out

what I/O was being performed inside selected interrupts.

The IOSPY program is a TSR that requires less than 17k of conventional

memory, yet can recall over 7000 of the I/O port numbers, Data, and

direction of I/O statements gone by. This is accomplished by the use of

a monochrome monitor used in conjunction with your primary CGA,EGA,VGA

monitor. The monochrome monitor makes it possible to view I/O data on the

fly. Even when your application program has placed your color monitor in

graphics mode. IOSPY accomplishes this by storing data to the portion of

monochrome memory not used to display text.

OVERVIEW

IOSPY examines each microprocessor instruction that is executed by your

application program for an I/O instruction. When one is executed, IOSPY

will display the I/O port number, the direction of data flow, and Data sent

or received on the secondary monochrome monitor. Then IOSPY will store the

same information to upper mono video memory for later review.

When you select the Interrupt mode from the IOSPY command menu, you will

be prompted for an Interrupt number. This mode is essential for spying on

applications like device drivers and software keys. It also opened a new can

of worms from my perspective, because some interrupts call other interrupts,

and still others call themselves. I don't think this program would have been

possible without the debug registers available inside microprocessors

beginning with the 80386. After you select an Interrupt, IOSPY sets a

debug register to point to the first instruction of the selected interrupt.

Then promptly goes into Disable mode. Your application program will

execute at full speed, until the first instruction of the selected interrupt

is executed. When that occurs, IOSPY wakes up and data collection begins.

As I mentioned earlier, some interrupts call other interrupts and even

occasionally themselves. The IOSPY will continue to report any and all I/O

that occurs during the various levels of interrupts that occur.

Take for instance interrupt 5 the Print Screen service. Print screen will

print all 2000 characters of any single video screen. The rub, is that int 5

will call interrupt 17, the printer services interrupt, 2000 times! At an

average of 8 I/O instructions per printed character, we would end up with

over 10,000 I/O instructions to print one screen. One consolation is that

we don't need to review all 10,000 instructions to grasp how a single

character is sent to the printer.

INSTRUCTIONS

Install the IOSPY by executing the IOSPY program. No Command line switches

are used. IOSPY will check your hardware configuration for a compatible

monochrome monitor adapter.  If successful, you will then see the program

sign-on.

At this point you will be asked to press the Hot-Key you wish to use to

activate the IOSPY program. You will want to select a hot-key that isn't

used by your application. I like the Scroll-Lock key for most applications.

Don't worry if you select the wrong one, you can always select a different

hot-key from the command menu.

All IOSPY commands are single key commands and are listed below.

C    Clear all Stored Data

This command just erases any collected data from the IOSPY

program data area to give you a new frame of reference for

some upcoming session.

D    Disable I/O Spy

This command removes the IOSPY software overhead. So your

applications will execute at full speed. And Hot-key is

still active but may not be accessible if an application has

hooked the keyboard services (int 9). If this is a problem,

just leave the IOSPY in the 'Go' Mode. Although this will

slow things up a bit, it is almost impossible for an

application to take the keyboard away from you.

F    Filter Selected Ports

This command will prevent data collection of up to 16

selected ports. Port 201 for example, is the joy-stick port.

Some game software reads this port a million times. Making

meaningful data collection impossible. After choosing the

Filter option you'll be prompted to enter from a sub-menu.

Sub-menu options are Include a port, Exclude a port and Go.

After selecting Exclude you'll be prompted for a port number.

After selecting Include, data collection will allowed for

all ports. Use the Go command to exit the port filter

Sub-menu.

H    Help

A short explanation of each command.

I    Interrupt Mode

After selecting Interrupt mode you will be prompted for

a two digit hex interrupt number. The IOSPY program

calculates the beginning address for that interrupt service

and sets a debug register trap. Then disables itself.

Data collection and individual instruction examination

halts. The Hot-Key is still active in this mode.

If an application program or DOS executes the instruction

at the beginning of the selected interrupt, data collection

resumes. Data collection does not stop until the IRET

instruction is executed for which the interrupt the trap

was set. Not subsequent IRETs performed by any calls during

the process.

Sorry, this command is not available in the DEMO version.

N    New Hot-Key

This command allows selection of a different Hot-Key.

After selecting the New hot-key command simply press the

Key you wish to use to activate the IOSPY program.

P    Print Screen

This command will print any data currently being displayed

on the Monochrome Monitor.

R    Report Collected Data

This command initiates a sub-menu of single key commands

that allow you to scroll through any collected data stored

by IOSPY. This can be as much as 7000 individual I/O

instructions.

Sub-menu options are the up / dn arrow keys, pg up, pg dn,

(P)rint Quit or Esc.

Sorry, this command is not available in the Demo version.

G    Go

This command starts IOSPY data collection. Remember that

every instruction executed by an application program or

DOS is examined for I/O statements. This 'Software Overhead'

will cause any application to execute slowly. So be patient.

V    Hooked Vector Reset

During IOSPY installation, this program takes a snap-shot

of the Interrupt Vector Table. When the IOSPY is active,

it occasionally compares the current content of the vector

table with the snap-shot in memory. The address of the

differences between the two (divided by 4)  provide the

interrupt number of the hooked interrupts.

By pressing the 'V' key at the command prompt the IOSPY

takes a new snap-shot to give you a new frame of reference.

Typical Output Screen

.----------------------------------------------------------------------------.

| Hooked Vectors 08 22 23 21                                                 |

| 2e > 03d4                                                                  |

| 00 < 03c5                                                                  |

| 0b > 03c4                                                                  |

| 04 > 03c5                                                                  |

| 05 > 03c4                                                                  |

| ff < 03c5                                                                  |

| 05 > 03c4                                                                  |

| 04 > 03c5                                                                  |

| 05 > 03c4                                                                  |

| ff < 03c5                                                                  |

| 05 > 03c4                                                                  |

| 04 > 03c5                                                                  |

| 05 > 03c4                                                                  |

| ff < 03c5                                                                  |

| 05 > 03c4                                                                  |

| 90 < 0040                                                                  |

| f1 < 0040                                                                  |

| 00 > 0043                                                                  |

| a0 < 0040                                                                  |

| b6 < 0040                                                                  |

| 00 > 0043                                                                  |

| 3c > 0061                                                                  |

| Enter Hot-Key for IOSPY command menu                                       |

`----------------------------------------------------------------------------'

The Hooked Vectors display reports any Interrupts taken over by an

application program or DOS.

The left most byte value displayed on each line is the Data being

transferred to an I/O Port displayed as a hex word to the right of the

'<'  or  '>'. The greater than sign '>' indicates that the Data was

output to a port. The less than '<' indicates that the data was input from

a port.

The newest data is displayed on the top line of the monochrome monitor

the oldest data (on the screen) is on the bottom.

Remember, there are nearly 7000 pieces of collected data that may be

reviewed with the 'R' Report command.

Other INDUSTRIAL SPYWARE Products.

The ISPY386 program is the Flagship of the Industrial Spyware line.

A full-featured system level debugger that spies on any real-mode

program (8088-P5) one instruction at a time or stores thousands of

instructions into memory for later review, even after a crash. Other

features include an interrupt review for the last 256 interrupts gone

by as well as register contents both coming and going. File activity

review, Memory watch window, Memory Editor, Instruction Editor, an 80386

debug register mode that allows trapping memory reads/writes or instruction

fetches, a register content trap that allows trapping on specific register

contents, even a mini IOSPY that records 128 of the last I/O commands.

This program is highly optimized and uses less than 25k of conventional

memory and 0k extended / expanded memory.

The beauty of this debugger is that it operates in the background and on

the secondary monochrome monitor. So as your application is running on the

primary CGA/EGA/VGA you can see the instructions as they are being executed,

(and even before they execute) on the the monochrome monitor. If something

interesting happens on the VGA, just press the hot-key and review the

previous 1024 assembler instructions that just occured.

Since IBM designated seperate memory ranges for color and monochrome

video adapters ( b800: & b000: respectively ),  I feel that the use of two

monitors is the most powerful and least used debugging techinques available.

There are other debuggers that use two monitors, but they're so bloated

with code, that they're almost useless.

Yet More Utilities

( Included with purchase of Industrial Spyware Package.  only $99.00 )

ISPY       8088 & 80286 version of Industrial Spyware

ISPY386    8088 - P5 Industrial Spyware Pro

FILESPY    Reports real time file activity

INTUSAGE   Reports real time interrupt activity

KEYSPY     Reports real time keyboard activity

MSPY       Reports real time memory activity

ROM_VIEW   Reports real time rom-bios activity

SPY.DOC    Vital PC Statistics Document (300k)

VSPY       Reports Interrupts and Register Usage Both Coming and Going

V_HOOK     Reports Hostile Interrupt Take-overs

V_REPORT   Reports Real Time Interrupt Frequency

IOSPY      Reports latest 7000 I/O Instructions (requires 386 up)

IOSKEL     A shell program for telling lies to I/O Ports (i.e. Keys)

NO LIABILITY  FOR CONSEQUENTIAL DAMAGES.  IN NO EVENT SHALL Mr. BRIGGS

BE LIABLE FOR ANY DAMAGES WHATSOEVER. INCLUDING, WITHOUT LIMITATION,

DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF BUSINESS

INFORMATION, OR OTHER LOSS ARISING OUT OF THE USE  OF OR THE INABILITY TO

USE THIS SOFTWARE PRODUCT. EVEN IF Mr. BRIGGS HAS BEEN ADVISED OF THE

POSSIBILITY OF SUCH DAMAGES.  BECAUSE SOME STATES DO NOT ALLOW THE EXCLUSION

OR LIMITATION OF LIABILITY FOR CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE

ABOVE LIMITATION MAY NOT APPLY TO YOU.

IOSPY  System Requirements

This Software requires  Two Monitors.

The primary monitor must be color ( RGB, EGA, or VGA) and the secondary

monitor Must be a Hercules Graphics compatible Monochrome monitor with

at least 32k of memory.

A full sized IBM AT or 100% Compatible. With an 80386, 80486 or P-5

(80586) Microprocessor running an MS/PC-DOS based operating system

version 3.0 and later with at least 64k of available system memory.

IOSPY Order Form

______ IOSPY      single user version .................. $  39.00  $ ______

______ Industrial Spyware Package single user version .. $  99.00  $ ______

California orders please add Sales Tax .................... 7.75%  $ ______

Shipping & Handling  $   5.00

Overseas Shipping & Handling  $  15.00

Total  $ ______

Name:    _____________________________________________________

Company: ____________________________________________________________

Address: ____________________________________________________________

: ____________________________________________________________

: ____________________________________________________________

Phone: ______________________________ FAX: ________________________

(Sorry No credit card orders Please)

Please complete and mail with payment to:

Steve Briggs

1035 Minnesota Avenue

Kansas City Kansas

66101