AWACS 1.2 User Guide

Copyright (C)1996 Cornel Huth

21-Mar-1996

This guide covers:

Overview and changes

Section 1.  Hardware and software requirements

Section 2.  Installation of supporting software

Section 3.  Installation of AWACS

Section 4.  Controlling AWACS

Section 5.  Troubleshooting

Section 6.  Ordering AWACS, License Agreement, and order form

AWACS is a multi-part package.  Included is a PM program, called AWACS,

that controls the swap, system performance analyzer (T2), the com

device and caller ID monitor (COM40), and displays this to the second

monitor, in real-time, using the second-monitor driver (MONY).

First, a screen shot, similar to how it displays on the second monitor:

������������������������������������������������������������������������������Ŀ

�Swap: 32768 *****�Swp  Sys  CID��COM2:02F8  57600 8N1 DCD RNG DSR CTS RTS DTR۳

�used:  3756 � 11%�5.0  2.0  1.0� FOYT A J        (281)555-1212 02/16/96 15:30 �

�����������������ĴLOW  MID  CSD�                                              �

� RAM: 20332 �    �������������Ĵ                                              �

�free:  2008 �  9%�    .:.      �                                              �

�idle:   288 �  1 �  .:::::    .�                                              �

�lckd:   256 �  1 �.::::::::..::�                                              �

�rsdt:  5800 � 28 �������������Ĵ                                              �

�����������������Ĵ�Ŀ�Ŀ �Ŀ�Ŀ�                                              �

�wMin:  6820 � 34 �� �� �:� �� ��                                              �

�wSet:  7120 � 34 ������� �����ٳ                                              �

������������������������������������������������������������������������������Ĵ

�20                                                                          20�

�18                                                                          18�

�16                                                                          16�

�14                                                                          14�

�12                                                                          12�

�10                                             ���                          10�

�08                                            ������      ���               08�

�06   ��������������������������������������������������  ������������������ 06�

�04������������������������������������������������������������������������� 04�

�02������������������������������������������������������������������������� 02�

��������������������������������������������������������������������������������

AWACS 1.2                   Not For Corporate Use                  Eval-U-Ware

The ***** to the right of Swap: 32768 indicates where any 'fast swap'

anount would be indicated, with a leading + sign.  The RAM: figure is

the physical amount in any case, though percent calculations are based

on useable RAM (i.e., not counting fast swap).  Free RAM is RAM not being

used.  Idle RAM is memory not recently used (prime to be swapped out and

made available for something else).  Locked RAM is the total amount of

locked RAM, all types.  Resident RAM is the amount of memory that cannot be

swapped out.  wMin and wSet are the minimum and recommended memory needs

to run the system, given the current state.  It should not be used to

compare dissimilar systems, rather, it should be used to determine the

state of the current system only.  If the reading exceeds 87%, results

are not meaningful due to memory contention (usually, it stays pretty low!).

That's why AWACS exists:  to output real-time system status right

at you, not hidden two levels down, or tucked away up in a corner

-- it's right out there.  The next six sections cover the basics

on installation and use.  Be sure to read it all before going to

AWACS - A Wonderful and Cool State.

Any questions or problems you encountered will likely be answered by

the information in this User Guide.  Read it and save yourself time.

There is no 'Fast Start' section because setup needs are explicit.

Reading this Guide from top to bottom is the only way to fly with

AWACS.

Changes:

1.20  21-Mar-96 - -page:n option works for pages other than 0.

- The included Mony driver has been updated to version 1.20 and is required

for AWACS 1.20.

- T2ERR:00003 in the Pulse box has been removed.  T2 gives this error when a page

it was tracking is no longer valid.  This error is now disregarded (3, only).

- "RESET" no longer put in Pulse box.

- Changed any fast swap indication to be preceded by a + rather than a -.

With the new swap monitoring, fast swap is not added in with the Swap:

amount (Swap: is the physical size of the swap file).  Effective swap

space, then, is Swap: PLUS any fast swap RAM.

1.10: 19-Feb-96 - No longer using T2 to monitor swap file state since

the T2 swap analyzer call would block-lock under heavy disk swapping

blocking all swap file monitoring.  The new algorithm is much faster,

though not as accurate (I believe).  However, it does not block-lock.

The -noT2 switch has been replaced by -noSwap and -noSys; T2 is used

on for system monitoring now.

1.01: 18-Feb-96 - Attempted to work around T2 swap analyzer block-lock.

Results required re-write of swap monitoring (1.10).

1.00: 17-Feb-96 - AWACS unleashed.

--------------------------------------------------------------------------

Section 1. Hardware and Software Requirements

--------------------------------------------------------------------------

o Hardware required

- Monochrome monitor as second display (digital/TTL)

- Monochrome display adapter (MDA, MPA, MGP, Herc card, etc.)

o Optional

- Modem, or modem with caller ID support

AWACS information is displayed on a second monitor.  It will not

operate at all on single-monitor systems.  See MONY installation,

below, for more information on the second monitor.  AWACS will

not operate without a second monitor available - no exceptions.

Any type of VGA is -not- a suitable second monitor.

A modem is not required, but if one is available, its status may

be monitored in real-time.  All modem control and status signals

are displayed, as is the port baud and line control settings.

This status is available to any communications package running,

not just when AWACS is using the port.  For example:

COM2:02F8  57600 8N1 DCD RNG DSR CTS RTS DTR

Caller ID, if supported by the modem, and the service is available

from your telephone company, is provided, showing the names, numbers,

and times of the last 10 callers.  For example:

FOYT A J        (281)555-1212 02/16/96 15:30

If someone calls hiding their number information, SNEAK/NOT AVAIL

is shown instead, and 9's as the number.  If no CID info is available,

NOT AVAILABLE is shown, again with 9's as the number.  The date and

time are always shown.  Most, but not all, long distance numbers are

available, depending on the caller's telephone company's abilities

to transmit the required data (it's the law! says Box).  During a

CID session, the space before the COM2:02F8 line (see above) is

replaced with a '1' when RING is being scanned, '3' when scanning

for the CID data, '5' when valid CID data has been detected, and

'9' when reading the actual CID data, then back to SPACE.  This

may occur very fast, and so all you may see is 1, then 9.  If you

only see 1 or 3, then no CID data has been detected.  See the

Troubleshooting Section, below, for more.

Included in the AWACS package are these files:

awacs.exe    - PM control program

awacs.doc    - this document

mony.sys     - monochrome display driver

com40.sys    - com status and caller ID driver

allocmem.exe - works-out the swap/virtual memory system

Not included are these files:

theseus2.sys  - system performance monitoring driver

theseus0.dll  - system performance monitoring interface

o Software required

- MONY.SYS    Second monitor display driver, v1.1 or later (required)

o Optional:

- COM40.SYS   Com port device driver

- THESEUS2.SYS and THESEUS0.DLL  Performance monitoring software

Mony is the driver that enables AWACS to read and write to the display

memory of the mono video card.

COM40.SYS completely replaces the COM.SYS driver of OS/2.  Most of the

functionality of COM.SYS is available, with added support for Caller ID

and COM status monitoring, when used with AWACS.  The current maximum

baud setting is 57600.  You may notice that some com programs let you

set 115200, and then say 115200, but 57600 is the real limit (currently).

Theseus is the performance monitoring software AWACS calls to get swap,

memory, and working set analysis data.  AWACS operates without it, but

lots of useful data is not available unless this software is installed.

Theseus files are not included with AWACS, but can be retrieved via FTP

from the Internet, and also downloaded from Compuserve or the AWACS

support BBS.  These sites and filenames are listed under "Installing

THESEUS2.SYS", below.

OS/2 Warp or later is required.

IMPORTANT NOTE

--------------

Unless specifically mentioned as optional, all items in the

"Required" sections are absolutely required to run AWACS.

--------------------------------------------------------------------------

Section 2. Installation of Supporting Software

--------------------------------------------------------------------------

Given that the hardware requirements have been met, software installation

is as simple as adding a few lines to the config.sys file to install Mony,

Com40, and T2.

o Installing MONY.SYS

MONY.SYS, version 1.1 or later, is required to use AWACS.  It must be

installed in config.sys.  Add the line:

DEVICE=G:\MONY.SYS

anywhere in the config.sys file (the last line should do fine).

Include any needed path.

During boot-up, a sign-on banner is shown, first on the main display,

then shortly after, on the second display.  All is well when the

second display shows the Mony banner.  If the banners do not display,

check your config.sys line.  An option switch for Mony is -q, which

quiets the banner on the main display.

LICENSE NOTE:  An attachment-use license is granted to licensed

users of AWACS for using MONY.SYS with their AWACS license, without

the need to also license MONY.  However, this license does not grant

the use of MONY outside of AWACS.  For that, a separate MONY license

is required.

o Installing COM40.SYS

COM40.SYS, version 1.1 or later, is optional.  It's used to enable

com port monitoring, and to enable caller ID support.  Installation

is exactly as the standard COM.SYS for OS/2 Warp (refer to the OS/2

online help if needed).  Locate the DEVICE=COM.SYS line in config.sys

and change it as follows:

rem DEVICE=G:\COM.SYS

DEVICE=G:\COM40.SYS

If there are any arguments on the COM.SYS line, include those on

the COM40.SYS line.  COM40.SYS is not compatible with SIO drivers.

o Installing THESEUS2.SYS

THESEUS2.SYS and THESEUS0.DLL are part of the System Performance Monitor/2

package (SPM/2).  Available for download from many sources is the THESEUS2

package, but not the entire SPM/2 package.  That's okay, since all that is

required is the T2 package (THESEUS2 package) specifically for Warp or

later.  For downloading the T2 package, see below.

In config.sys, add the line:

DEVICE=G:\THESEUS2.SYS

o Installing THESEUS0.DLL

Add this file to any real directory that is pointed to by LIBPATH=.

For example, if LIBPATH= in config.sys reads

LIBPATH=.;H:\EPM;G:\OS2\DLL; ...

you may place THESEUS0.DLL into the G:\OS2\DLL directory.

o Downloading THESEUS2.SYS and THESEUS0.DLL

If you don't already have a version of T2 for your system, you can

easily download these two files by getting the T2ZIP_K.ZIP file, for Warp.

For Internet FTP:

site: FTP.PCCO.IBM.COM

dir: corrective_service/

file: t2zip_k.zip

For Compuserve:

forum: OS2DF2

area: System Performance Monitor/2  (9)

file: t2zipe.zip (there may be a latter version available as CIS)

To ensure you find the same copy I use, visit either of the following:

Home sites:

site: ftp.crl.com

dir: /users/co/cornel/os2/util

file: t2update.zip

site: 40th Floor BBS

tele: 1(210)684-8065 (hours are 5pm to 9am, US Central, 24hrs weekends)

dir: OS/2 Programmer Toolkits

file: t2update.zip

These are the versions I tested, using Warp at Fixpack #17:

12-19-94  10:51a   3136  THESEUS2.SYS

11-23-94   8:58a  66632  THESEUS0.DLL

The Home sites listed above will always have the latest version of the

AWACS package.  If you would like to check for updates, those are the

places to check.  Once registered, your license carries forward at

least through any and all 1.x versions (i.e., free updates).

--------------------------------------------------------------------------

Section 3. Installation of AWACS

--------------------------------------------------------------------------

AWACS consists of a single file, AWACS.EXE.  It is a Presentation

Manager program, and requires OS/2 Warp or later.  It may be started

from the command line or from a program object.  For example:

[G:\]awacs  [-options]

or

[G:\]start awacs [-options]

This line may be placed in startup.cmd, too.

Or, a program object can easily be made and placed on the launchpad,

or placed in the startup folder.

Only one instance of AWACS can be running at any one time.  Trying

to start a second copy of AWACS does nothing.

When installing an update, and you are using a program object, make sure

that the new EXE is being used.  Compare the AWACS version to what is

displayed on the screen.

o Option switches

AWACS itself presents a very simple control panel, with just six buttons

and three fields for controlling monitoring priorities and times.  AWACS

configuration is done at program start, via command-line option switches.

All switches are optional, though some are likely to always be used.

These are:

-com#          com device name

-istr:aaa      caller ID string

-bell          bell sound at incoming CID

-noCID         disable CID monitoring

-cid:TP        CID monitor rate and priority

-noSwap        disable swap monitoring

-swap:TP       swap file monitor rate and priority

-noSys         disable system monitoring

-sys:TP        system monitor rate and priority

-rst:ddd       system monitor reinit rate

-gc:ddd        working set graphics character

-noPulse       disable pulse

-cls           clear screen page on startup

-page:n        use this page for display

-noAbout       do not show driver versions found at startup

-com#

Where -com# indicates the device to monitor for status and caller ID.

For example, if your modem is on COM2, use -com2.  The modem needs to

recognize AT, and the -istr: string (nearly all do).  Caller ID support

from the modem is not required except if you expect to get caller ID

info.

The default is -com0, or com port and CID monitoring disabled.  You must

supply a valid modem device (named 'com1' to 'com8') for monitoring to

be enabled, and this must be done using -com#.

-istr:aaa

Where aaa is the modem command string to enable caller ID, raw format,

from your modem.  For example, most modems use AT#CID=2, which is the

default (-istr:AT#CID=2) if no -istr: is used.  Do not use the command

for formatted out, generally=1;- only raw output can be used by AWACS.

-bell

If you want an audio indication of a CID call coming in, use -bell.

-noCID

If you don't have caller ID available (either the modem, or the service),

use this option to disable CID monitoring.  This option also forces the

working set analysis graphics to display in the CID box, with the bottom

half of the screen unused by AWACS.  Status monitoring is still available,

so long as -com# is used.

-cid:TP

By default, CID monitoring is in a CLOSED state on start up.  The T

is the monitoring time, in seconds (1-999), and P is the priority. P

may be H for hold (CLOSED in the case of CID), L for low, M for middle,

and X for time-critical.  For example: -cid:5L  sets the monitor to

read the CID queue inside COM40.SYS every 5 seconds, and to run that

monitoring thread at a low priority (low = idle class, level 31).

Seconds must be integer, from 1 to 999.  The time and priority may

be changed from the AWACS control panel, and the time adjusted to

finer periods.

While CLOSED, any other program may use the modem.  Otherwise, you

must first CLOSE the CID monitor, then start the other program that

needs the modem.

-noSwap

If you do not want swap file analysis done use this.  Even if disabled,

the amount of 'Fast Swap' RAM, -if any- is shown.  Fast Swap is that

memory above 16MB that OS/2 cannot use directly, but must use as a

fast swap file addition, due to hardware limitations (using an ISA

bus-master disk controller is the only reason I'm aware of).  If fast

swap is a factor, the fast swap amount is shown on the Swap: line, to

the right of where the swap file amount is shown (or would be shown).

See the Troubleshooting Section for more.

I recommend running the swap analyzer at all times.  The information is

more or less accurate, though if anything it overstates the amount of

swap file being used, sometimes as much as 10MB.  It's still useful.

-swap:TP

By default, swap file monitoring is in a HOLD state on start up.

The T is the monitoring time, in seconds (1-999), and P is the priority.

P may be H for hold, L for low, M for middle, and X for time-critical.

For example: -swap:10L sets the monitor to check the swap file state

every 10 seconds, and to run that monitoring thread at a low priority

(low = idle class, level 31).  Seconds must be integer, from 1 to 999.

The time and priority may be changed from the AWACS control panel after

start-up, and the time adjusted to finer periods.

-noSys

If you do not want performance analysis done (memory and working set

analysis, including WS graphics), use this.  If T2 is not available,

this is the default.  Even if disabled, the amount of physical RAM is

shown.

I recommend running the performance analyzer at all times.  The

information is truly an OS nut's dream come true (almost?).  Since

running at idle priority means it uses CPU power only when the CPU

isn't doing anything anyway, it's comes at no cost, either in terms

of CPU, or in terms of you having to dig through layers of windows

looking for it.

-sys:TP

System monitoring is as swap file monitoring with regard to T and P.

The system monitoring displays a graphical result of the real-time

working set analysis in the lower half of the screen.  If -noCID is

used, the graph is shown where the CID box normally is, leaving the

lower half of the screen unused.

-rst:ddd

The working set analysis is reset every 100 calls.  You can change this

to any number 20 or greater.  A reset may also be done from the AWACS

control panel by pressing the RESET button in the System Monitor box.

After a reset, the wMin: and wSet: values are at their lowest, typically.

-gc:ddd

The working set analysis graphics display uses character 176 as the

bar graphic.  This can be changed to any character above 13 (14-254).

For example, to replace the default bar character with the letter A,

use -gc:65.  Using -gc:0 turns off working set graphics output.  Also,

the high-byte of the -gc:n value is used as the background character

in the graph.  For example, to change the SPACE background to the

reverse space, use (176 + 219*256), or -gc:56240 (the default is SPACE).

-noPulse

Monitoring CPU usage -- the pulse -- may be disabled by using this

option.  Otherwise, every second, a pulse graph is scrolled in the

Pulse box.  Each dot represents another 15% CPU utilization (6 dots),

and no dot indicates < 15% CPU use.  There are 13 positions (13 seconds)

of graph data.  Generally, unless the CPU graph is maxed out (6 dots),

you are not CPU-bound and so even if you had more CPU power, it wouldn't

make a difference (wouldn't hurt!).  However, even if it is maxed out,

it still does not necessarily mean that you are CPU-bound; process(es)

may simply be 'hogging' resources unncessarily, in which case the more

CPU power you have, the more it will be hogged.

6: >= 90%

5: 75-89%

4: 60-74%

3: 45-59%

2: 30-44%

1: 15-29%

no dot: <= 14%

Note that since AWACS generally runs its threads at idle priority,

any CPU use from AWACS will completely disappear as soon as some other

task needs to use the CPU.

-cls

This clears the Mony screen on startup.  This is useful if you first

run AWACS using all the screen, have it keep the screen data up

when you close, and then restart AWACS in half-screen mode (-noCID).

-page:n

For Mony users, this lets you tell AWACS to use a Mony video page

other than page 0.  For example, if you already are using page 0

for your own programs, you can tell AWACS to use another page.  For

example, to have AWACS display all data to the second page, use:

-page:1.  The page must be from 0-7 (0=default), and Mony must

have been configured to support that page.  Whenever AWACS is brought

to the foreground, its page is redrawn (you have to redraw your own

page, as required).

-noAbout

On startup, AWACS shows a message box with the versions of the

drivers in use, and waits for an OK click.  Use this option to

bypass that compeletly.

Note:  If a driver was not found, or was not loaded, or is not the

correct version, n/a is shown instead of a version number.  T2, if

loaded okay, is always shown as version 2.000, because I have no

idea on how to determine anything about it from its header file.

Do not use -noAbout until you know the drivers are loaded/loading okay.

The large-font clock is always displayed.  Its display is updated

with the priority that the System Monitor is using, twice per minute.

A useful start line might look like this:

[G:\]start awacs -com2 -bell -swap:3L -sys:2L -cid:1L -noAbout

--------------------------------------------------------------------------

Section 4. Controlling AWACS

--------------------------------------------------------------------------

Once AWACS has been started (see section 3) and the about box is

cleared, the AWACS control panel is ready for use.  Three monitors

are available:  Swap, System, and CID.  Those monitors that do not

have the necessary drivers loaded, or were disabled with an option

switch, are disabled.

To exit and shutdown AWACS, use Alt-F4, or click the OS/2 system menu

at the upper-left and choose CLOSE.  If all monitors are in the HOLD

state, the display screen on the Mony is left as-is.  If any is not,

the screen is cleared.  AWACS may also be shutdown from the task list,

or during a PM shutdown, in which case the screen is always cleared

(bye!).

Now that that's out of the way...  Each time AWACS is brought to the

foreground, the display screen is redrawn.  If there was some other

task using the Mony screen, or perhaps another Mony application running,

simply bringing AWACS to the foreground redraws the screen anew.

Selecting AWACS from the task list also redraws the display.

Each of the three monitors has three controls: a priority button,

a rate field, and a reset button.  The priority button moves through

the priority states.

o SWAP MONITOR

The priority button varies from HOLD, LOW, MID, and HIGH.  The default

montoring RATE is every 5 seconds, and the range is 1.0 to 9.9, and

10 to 999 seconds.  The RESET button is not currently used for the

Swap Monitor.  When in the HOLD state, no swap monitoring takes place.

o SYSTEM MONITOR

The priority button varies from HOLD, LOW, MID, and HIGH.  The default

montoring RATE is every 2 seconds, and the range is 1.0 to 9.9, and

10 to 999 seconds.  The RESET button re-initializes the working set

analyzer.  When in the HOLD state, no system monitoring takes place.

o CID MONITOR

The priority button varies from CLOSED, LOW, MID, and HIGH.  The

default montoring RATE is every second, and the range is 1.0 to 9.9,

and 10 to 999 seconds.  This is the period after which AWACS checks

the COM40 CID queue for new entries.  It is not intrusive and so can

be easily run every second, so that you can see within a second who

is calling.  Caller ID data is transmitted between the first and

second RINGs.

The RESET button sends the caller ID init string to the modem.  When

in the CLOSED state for -more than 1.5 seconds-, the com device is

closed and made available for another program, such as your comm

program (CIM, Zoc, etc.).

Caller ID data is handled by the COM40.SYS driver so long as the

modem/com device is open and so long as the modem is told to accept

caller ID data.  This can be done in other programs by using the

same caller ID init string used by AWACS (in -istr:, default AT#CID=2)

as part of the init string of your comm package.  In other words,

AWACS does not have to have control of the modem for caller ID data

to be logged.  COM40.SYS queues and holds for AWACS up to 14 caller

ID data packs.  However, AWACS must be still be running, though it

may be in the CLOSED state, for COM40 to queue CID data.

The CID reset button should be used if the modem is not picking up

caller ID data.  You may want to press it once after startup, too,

just to ensure that the modem is ready for CID data.  The reset

button also clears previous CID entry data, and clears the CID box.

If it is in the CLOSED state, only the CID entries and CID box is

cleared; nothing can be sent to the modem by AWACS when it is CLOSED.

--------------------------------------------------------------------------

Section 5. Troubleshooting

--------------------------------------------------------------------------

1: I cannot get anything to happen.

A: Check the hardware and software requirements section above.  If

still nothing, feel free to contact me by e-mail (cornel@crl.com)

or at the BBS (listed above).  This is for installation problems

only.  If you aren't a licenced AWACS user, and have any other

problem, please do not contact me until you do register AWACS.

2: The program flashes 'evaluation over'.

A: This Eval-U-Ware version is time-limited.  The evaluation period

should be long enough to determine if AWACS is worth the cost to you.

See the license agreement and order form for an easy remedy.

3: I don't get any COM status monitoring.

A: COM40.SYS must be used in place of any other COM device driver.  Also

make sure -com# is set to the modem, and that the modem is on.

4: I don't get any caller ID data.

A: COM40.SYS must be used in place of any other COM device driver.  Also

make sure -com# is set to the modem, and that the modem is on.  You

also must have a CID-capable modem, and must have CID service from

the telephone company (about $80/year from SWB).  If still nothing,

see #1.

Also, check if AT#CID=2 is the required init string to enable raw,

unformatted caller ID info for your modem.  If you only ever see a

'1' or a SPACE preceding "COM2:02F8  57600..." (right before the

COM word) on the status line, then your modem is not sending out

the required raw data.  Normally, with valid CID data, it goes 1,

3, 5, 9 after the first ring (rather fast, so you may not see the

5), then just 1, and perhaps 3 on the second and later rings.  If

you don't see a 9, COM40 is not getting the expected CID data.

5: AWACS was exited, yet there's still a COM status line on the monitor.

A: To fully shutdown the status monitor you must have the com device

open when you quit AWACS.  Start AWACS again, change the CID Monitoring

from CLOSED to LOW, then shutdown AWACS.  AWACS cannot communicate with

COM40.SYS unless AWACS has it open (i.e., not CLOSED).

6: I can't get my other com packages to work when AWACS is running.

A: Push the large CID Monitor push button until CLOSED appears.  After

1.5 seconds the com device is closed and may be opened by another

program.  Also see #3 and #4.

7: I don't get Swap monitoring.

A: Make sure that config.sys is available for read access on the

boot drive at AWACS start up, and that SWAPPATH= is valid.

8: I don't get System monitoring.

A: Make sure you have the Theseus2 support files installed, as

indicated in the text.

9: I have a mono VGA system and want to use it as a second monitor.

A: It won't work.  See the Hardware Requirements Section.

10: Everything appears fine, but not much is going on as far as monitoring.

A:  If the monitors are running at LOW priority, then "heavy' activity

takes the monitors out of the CPU loop.  A database sort, for example,

removes just about all of the idle processing.  If you need to have

constant monitoring displayed, use MID or HIGH priority.

11: When running most MS-DOS programs, not much happens at LOW priority.

A:  DOS programs typically 'hog' the CPU.  Either adjust their idle

detection settings from the VDM's DOS SETTINGS (usually all it takes),

or increase the monitor priorities from LOW to MID.

12. What is fast swap space?

A:  Fast Swap space is RAM added to the swap file manager; sort of like

a RAM disk, only added as part of the swap file subsystem.  It is

only used if each of the following is true:  a) you have an ISA-bus

SCSI card using DMA with a fixed disk attached to it; b) you

have more than 16MB of memory.  If these are both true, your RAM

available for code/data is limited to the first 16MB of your RAM,

and the rest is given to the swap file system.  AWACS indicates this

by adding "+3948" (whatever the amount is over 16MB, though the swap

file figure minus the fast swap size may not exactly equal 16MB)

directly after the Swap: amount at the upper-left of the monitor

screen.  The RAM: amount is the total RAM in any case, though the

percent calculations are based on the useable RAM (16MB if you meet

A and B above).  The Swap: amount includes the physical swap size

and any added fast swap RAM.  If there is nothing after Swap: then

you do not have a ISA-bus SCSI DMA card with a fixed disk attached

to it.  Note that if you meet A, but do not have more than 16MB of

RAM, then -0000 is shown, indicating that if you were to add memory

for more than 16MB, that memory over 16MB would be used as fast

swap space.  This is for ISA SCSI, using DMA, disk cards only;

VLB or PCI cards do not apply.

13: How can I get Mony, or COM40?

A:  Mony is available as a separate product.  Visit the Home sites

listed above.  COM40 is not available as a separate product at

this time.  Theseus2 was part of the IBM SPM/2 package.  Contact

IBM for availability.

--------------------------------------------------------------------------

Section 6. Ordering AWACS

--------------------------------------------------------------------------

AWACS is available for individuals at $14.99.  Corporate use requires

a site license, available at $249.99.  Corporate use is defined as

any business entity employing more than just yourself, in any capacity.

o Ordering Information for AWACS

To order AWACS by check, bank check, money-order, or cash, use the

form at the bottom of this documented.  Credit card orders are not

accepted.  See the License Agreement below for license restrictions.

o Payment Options

Check, money-order, cash

Funds must be in US Dollars and must be drawn on a US bank (complete

with bank routing numbers).  If sending currency, use Registered

AirMail.  Personal checks require 10 working days to clear.  Most

major foreign banks have 'branch' banks in the US and this can be

used for US Dollars/US bank requirements.  Direct wire-transfer is

not available.

Send payment and order form to:

Cornel Huth

6402 Ingram Rd

San Antonio, Texas 78238-3915

USA

o Order Options

Price for your AWACS license is based on this schedule:

Personal License:  $14.99 (per machine)

Corporate Site License: $249.99 (per site)

All orders must be pre-paid.  No exceptions.  Site licenses are

customized to include the name of the corporation.  Any corporate

use requires a site license.  Refer to the License Agreement for details.

o Delivery Options

Standard AirMail is used to deliver the license pack.  By request,

delivery by e-mail may also be used.  Allow 2 weeks for processing.

o License Agreement

Before using this software you must agree to the following:

1. An AWACS license grants you the right to use the AWACS product on one

computer.  One license is required for each machine using AWACS.

2. You are not permitted to operate more than one copy of this software

package at one time per license.  For example, if you have ten machines

that have access to the AWACS package at the same time, you are required

to have ten AWACS licenses.

3. You are not permitted to distribute any part of the AWACS package

without written permission from the copyright holder.

4. Evaluation use is limited to 5 minutes after each start, and for

no more than 28 days, and is for the sole purpose of evaluating the

software.

5. Use of the evaluation version (i.e., without a license) within any

Corporation is strictly forbidden.  The evaluation version is for

personal evaluation only.  Any Corporate use requires a site-license

purchase before installing AWACS on any Corporate machine.  Corporate

use of AWACS without a site license constitutes a violation of the

AWACS copyright.  Corporate use is defined as any business entity

employing more than just yourself, in any capacity.

6. AWACS is owned by the author, Cornel Huth, and is protected by United

States copyright laws and international treaty provisions.  You are not

permitted to make copies of this software except for archival purposes.

7. You may not rent or lease AWACS. You may not transfer this license

without the written permission of the author.  If this software is an update

or upgrade, you may not sell or give away previous versions.

8. You may not reverse engineer, decompile, or disassemble this software if

the intent or result is to alter the software.

9. There are no expressed or implied warranties with this software.

10. All liabilities in the use of this software rest with the user.

11. U.S. Government Restricted Rights.  This software is provided with

restricted rights.  Use, duplication, or disclosure by the Government is

subject to restrictions as set forth in subparagraph (c)(1)(ii) of the

Rights in Technical Data and Computer Software clause at 52.227-7013.

The software is owned by Cornel Huth/6402 Ingram Rd/San Antonio Texas

78238/USA.  This agreement is governed by the laws of the Great State

of Texas, the United States of America, and all other countries of Earth.

Any questions concerning this License Agreement should be directed to

me, Cornel Huth.

Failure to comply with any part of this License Agreement immediately

terminates any and all licenses that you may have to use AWACS.

o Order Form (cut at line below)

---------------------------------------------------------------------------

AWACS for OS/2 Order Form for Check, Money-order, or Cash

Cost Per    Number of

License     Licenses    Extended

--------------------------------------------------------------------

Personal License, One license required for each machine installed

$  14.99  x          =

------------------------------------------------------     ---------

Corporate Site License, required for any Corporate use

$ 249.99  x  site    =

------------------------------------------------------     ---------

For Texas shipping destinations only, add 7.75%  =

---------

TOTAL =

=========

Send TOTAL payment on check (US Dollars/US bank), money-order, or cash to

CORNEL HUTH                       Always endorse your letter

6402 INGRAM RD                             AirMail

SAN ANTONIO TX 78238-3915          if you are not in the USA

USA

PLEASE PRINT - For comments use back and mark here [  ]

Your Name>

-------------------------------------------------- (required)

Company>

-------------------------------------------------- (optional)

Address>

-------------------------------------------------------------

-------------------------------------------------------------

-------------------------------------------------------------

Telephone                               Fax

Number> ----------------------------- No.> --------------------------

e-mail>                                    Today's Date>

----------------------------------               ---/----/---

<end>