TRAJECTORY MAKER AND TRAJECTORY SCAPE

USER'S GUIDE

Version  1.1

By

H.B. Reynolds

ORBIT HORIZONS

94 Promenade

Irvine, California 92715

(714) 552-8692

(C) Copyright 1991.  All rights Reserved.

Page ii

"WELCOME TO TRAJECTORY MAKER"

_____________________________

Thank you for trying Trajectory Maker and its companion program

Trajectory Scape.  Whether you are an experienced orbital mechanic,

relatively new to the field or just want to learn about it, I am

confident that you will enjoy using these tools.

It is the author's contention that complex simulations need not

burden the user with complex input requirements.  I have endeavored

to keep the input simple without sacrificing flexibility, while

providing output formats that have proved to be useful for many

applications.

If there are features that you would like to see in a future

version of these programs, please let me hear from you.  Address your

comments to:

H. B. Reynolds

President

ORBIT HORIZONS

94 Promenade

Irvine, CA 92715

Page iii

TRAJECTORY MAKER FEATURES

Trajectory Maker is a professional simulation and design tool for

scientists and engineers involved in mission analysis and orbital

operations.

1. Trajectory Maker generates precision trajectories for

o Earth orbiting satellites.

o Escape probes.

o Ballistic missiles.

2. Four Acceleration Models

o Keplerian two-body motion.

o Gravity perturbations.

o Atmospheric drag perturbations.

o Gravity perturbations with atmospheric drag effects.

3. Graphics Visualization

o Displays trajectory ground traces as they are generated.

o Displays three conformal map projections.

o Displays altitude history trend.

o Displays time and altitude numerical data.

4. Four Numeric Output Formats

o Geographic coordinates: geodetic latitude, longitude and

altitude.

o Geospherical inertial coordinates:  radius vector magnitude,

right ascension, declination ( geocentric latitude ), velocity

magnitude, velocity azimuth and flight path angle.

o Earth centered inertial coordinates: rectangular position,

velocity and acceleration.

o Echoed input data with period, Kepler flight time, nodal

regression rate and line of apsides rotation rate, initial true

anomaly, eccentric anomaly, mean anomaly, energy, altitude

bounds, etc.

5. User Controlled Time Parameters

o Trajectory start and end times.

o Granularity of computations.

o Output sampling frequency.

6. Initial State Vector Options

o Classical orbital elements.

o Earth-centered-inertial coordinates.

7. English or Metric Units

Page iv

TRAJECTORY SCAPE FEATURES

1. Views Multiple Trajectories

2. Five Geographic Data Presentations

o Mercator projection.

o Stereographic projection of the northern hemisphere.

o Stereographic projection of the southern hemisphere.

o Hammer-Aitoff equal area projection.

o Mollweide equal area projection.

3. Three Map Resolutions

o Fine resolution.

o Medium resolution.

o Coarse resolution.

4. Three Central Viewing Regions

o Greenwich meridian.

o Americas.

o USSR.

5. Both Programs are easy to install and use.

Page v

LICENSE

"TRAJECTORY MAKER" AND ITS COMPANION PROGRAM "TRAJECTORY SCAPE"

ARE COPYRIGHTED.  ALL RIGHTS ARE RESERVED.  YOU HAVE PURCHASED A

LICENSE TO USE THIS SOFTWARE ON ONE MACHINE AT A TIME.  YOU ARE

AUTHORIZED TO MAKE COPIES OF THESE PROGRAMS FOR THE SOLE PURPOSE OF

BACKING UP YOUR SOFTWARE AND PROTECTING YOUR INVESTMENT FROM LOSS.

WARRANTY

"TRAJECTORY MAKER" AND ITS COMPANION PROGRAM "TRAJECTORY SCAPE"

ARE LICENSED "AS-IS".  ORBIT HORIZONS MAKES NO WARRANTIES, EITHER

EXPRESSED OR IMPLIED, WITH RESPECT TO THESE PROGRAMS, THEIR QUALITY,

PERFORMANCE, MERCHANTABILITY, OR FITNESS FOR ANY PARTICULAR PURPOSE.

IN PARTICULAR, IT IS NOT GUARANTEED TO PREVENT OR DETECT DAMAGE TO

YOUR DATA OR PROGRAMS.  IN NO EVENT SHALL ORBIT HORIZONS BE LIABLE

FOR ANY CLAIMS FOR LOST PROFITS OR DAMAGE, INCLUDING, BUT NOT LIMITED

TO, SPECIAL, INCIDENTAL, CONSEQUENTIAL OR OTHER DAMAGE.

SOME STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION OF INCIDENTAL

OR CONSEQUENTIAL DAMAGES, SO THE ABOVE LIMITATION OR EXCLUSION MAY

NOT APPLY TO YOU. IN NO CASE SHALL ORBIT HORIZONS LIABILITY EXCEED

THE LICENSE FEES PAID FOR THE RIGHT TO USE THE LICENSED SOFTWARE. THE

LICENSE AGREEMENT AND WARRANTY SHALL BE CONSTRUED, INTERPRETED AND

GOVERNED BY LAWS OF THE STATE OF CALIFORNIA.

YOU AGREE TO THE TERMS OF THIS LICENSE BY YOUR DECISION TO USE

THIS SOFTWARE.

TABLE OF CONTENTS

_________________

Page

____

WELCOME TO TRAJECTORY MAKER .................................... ii

TRAJECTORY MAKER FEATURES ..................................... iii

TRAJECTORY SCAPE FEATURES ...................................... iv

INVOICE ......................................................... v

LICENSE AND WARRANTY ........................................... vi

1  INTRODUCTION ................................................. 1

2  INSTALLATION ................................................. 2

3  TRAJECTORY MAKER INPUT DESCRIPTION ........................... 5

3.1  Unit Convention Menu .................................. 5

3.2  Perturbation Model Option Menu ........................ 5

3.2.1  Unperturbed Two-Body Option .................. 5

3.2.2  Gravity Harmonic Effects Option .............. 5

3.2.3  Atmospheric Drag Effects Option .............. 7

3.2.4  Gravitational and Drag Effects Option ........ 7

3.3  Initial State Vector Option Menu ...................... 7

3.3.1  Orbital Elements ............................. 7

3.3.2  Initial Position and Velocity ............... 13

3.4  Time Control Parameters Menu ......................... 16

3.5  Output Format Options Menu ........................... 18

3.5.1  Geodetic Rotating Coordinates ............... 18

3.5.2  Geospherical Inertial Coordinates ........... 18

3.5.3  Earth-Centered-Inertial Coordinates ......... 21

4  TRAJECTORY MAKER OUTPUT DESCRIPTION ......................... 22

5  EXAMPLE ORBITS AND TRAJECTORIES ............................. 24

5.1  Space Shuttle Atlantis ............................... 24

5.2  Repeating Ground Trace ............................... 24

5.3  Sun-synchronous Orbit ................................ 25

5.4  Candidate Surveillance Orbits ........................ 25

TABLE OF CONTENTS ( continued )

_______________________________

Page

____

5.5  An Example Illustrating Orbit Decay .................. 26

5.6  Orbits Near Transition Region ........................ 27

5.7  Ballistic Missile Trajectories ....................... 27

5.7.1  Western Test Range Trajectories ............. 27

5.7.2  Hypothetical Threat Trajectories ............ 28

6  TRAJECTORY SCAPE INPUT DESCRIPTION .......................... 29

6.1  World Map Projections Options Menu.................... 29

6.1.1  Mercator with stereographic projection....... 29

6.1.2  Hammer-Aitoff Equal Area Projection ......... 29

6.1.3  Mollweide Equal Area Projection ............. 29

6.2  Map Resolution Options Menu .......................... 31

6.2.1  Fine Option ................................. 31

6.2.2  Medium Option ............................... 31

6.2.3  Coarse Option ............................... 31

6.3  World Map Focus Options Menu ......................... 31

6.3.1  Greenwich Meridian .......................... 31

6.3.2  American Region ............................. 31

6.3.3  Soviet Region ............................... 31

7  TRAJECTORY SCAPE OUTPUT DESCRIPTION ......................... 33

APPENDIX A.  HISTORICAL NOTES

LIST OF FIGURES

_______________

Figure                                                          Page

______                                                          ____

1  Unit Convention Menu ....................................... 6

2  Perturbation Model Option Menu ............................. 6

3  Initial State Vector Option Menu ........................... 8

4  Initial Orbital Elements ................................... 8

5  Semimajor Axis and Eccentricity ............................ 9

6  Orbit Inclination ......................................... 11

7  Longitude of the Ascending Node and Argument of Perigee ... 12

8  Initial Position and Velocity ............................. 14

9  Earth-Centered-Inertial Coordinates ....................... 15

10  Time Control Parameters ................................... 17

11  Output Format Option Menu ................................. 17

12  Geodetic Rotating Coordinates ............................. 19

13  Geospherical Inertial Coordinates ......................... 20

14  Sample Trajectory Maker Graphics Display .................. 23

15  World Map Projections Menu ................................ 30

16  Map Resolution Options Menu ............................... 32

17  World Map Focus Menu ...................................... 32

18  Four Tundra Orbits on Mercator and Stereographic

Projections ............................................... 34

19  Circular, 45 Degree Inclined Orbit on an

Hammer-Aitoff Projection .................................. 35

20  Circular, 45 Degree Inclined Orbit on a

Mollweide Projection ...................................... 36

Page 1

1.  INTRODUCTION

Trajectory Maker does just what its name implies - it makes

trajectories.  In particular, it makes trajectories for objects under

the influence of the earth's gravitational field, whose mass is small

relative to the earth.  Such an object may be a manmade satellite

continually revolving around the earth, it may be an interplanetary

spacecraft having achieved escape velocity or it may be a ballistic

missile on a destination towards its target.

In any case, the type of trajectory an object attains depends on

the energy manifested by its initial position and velocity.

Trajectory Maker really doesn't care because it integrates the

object's equations of motion directly, rather than using the

classical universal variables approach implied by two-body theory.

Because the figure of the earth is more accurately represented by

an ellipsoid of revolution than it is a sphere, trajectories deviate

from idealized conic sections implied by the universal variables.

Moreover, depending on the objects altitude, the earth's

atmosphere tends to retard the objects motion, causing it to decay,

ultimately colliding with the earth.  In the case of ballistic

missile trajectories, the retarding force becomes much greater than

the gravitational force as the object rips through the atmosphere

towards its target, and if its mission is aborted, atmospheric winds

may cause its fragments to drift.

By integrating the equations of motion directly, these

perturbation effects may be treated without much difficulty.

Specifically, Trajectory Maker's force model includes both

atmospheric drag and gravitational perturbation models.  The

differential equations of motion are integrated with the Runge-Kutta-

Shank's eighth-order, fixed-step numerical scheme having twelve

stages.  Its speed is moderate, depending on the user selected

step-size, but it is stable and very accurate.

Trajectory maker also contains an innovative algorithm for rapid,

high precision altitude computations.  All computations are performed

with double precision arithmetic.  For the interested reader, some

historical background is given in Appendix A.

However, you don't need to know or be concerned with the technical

details to use the program - that's one reason for the program !

Page 2

2.  INSTALLATION

"Trajectory Maker" and "Trajectory Scape" are designed for use on

IBM PC XT/AT and compatibles with a hard drive.  Numeric data is

available on any monitor.  Graphics visualization requires an EGA or

VGA card.  A math co-processor is optional and used if present.

Make a sub-directory and copy all of the files on the distribution

disks from your floppy drive.  For example,

C:>MD TMAKER

C:>CD TMAKER

Insert the distribution Disk 1 in Drive A: then,

C:>COPY A:*.*

Repeat this process for Disk 2.  That's it for installation.  You

should have the following files in your subdirectory:

TMAKER.EXE ... Trajectory Maker Executable Program.

TSCAPE.EXE ... Trajectory Scape Executable Program.

TMSRMN.FON ... Graphics Fonts.

WORLDB.LGE ... Large World Map.

WORLDB.MED ... Medium World Map.

WORLDB.SML ... Small World Map.

USER.DOC ..... User Guide ( This File )

INVOICE.DOC .. Invoice.

Running Trajectory Maker

________________________

Before giving a detailed description of the input options and

data, let's go ahead and run an example to familiarize you with the

operation of the program.

Type TMAKER at the DOS prompt.  After the opening banner, a menu

will appear requesting your choice of units.  Select the Nautical Miles

option. ( You may abort the job by pressing the escape key. )

* Type 1 and press <Enter>

Next, a menu appears requesting your choice of models.  Select the

Unperturbed two-body model.

* Type 1 and press <Enter>

Next, a menu appears requesting your choice of input for the initial

conditions.

* Type 1 and press <Enter>

Page 3

The next menu requests the orbital elements of the satellite.

These elements characterize the orbit.  For this example, we will

assume the satellite will have a circular orbit at an altitude of 400

nautical miles (nm), in a plane inclined 45 degrees with respect to

the equatorial plane.

The radius of the earth is 3444 nm at the equator, so the semimajor

axis is approximately: a = 3844 nm.

* Type 3844 and press <Enter>

Since the orbit is circular, its eccentricity, e = 0.

* Type 0 and press <Enter>

The satellite's orbit plane has inclination, i = 45.

* Type 45 and press <Enter>

The next two entries are orientation angles which are described in

Section 3.3.1.  Assume   = 0, and   = 0.

* Type 0 and press <Enter>

* Type 0 and press <Enter>

Notice that the period of the satellite, about 6000 seconds, is

displayed.  The period is the time it takes the satellite to make one

revolution about the earth.  It is displayed to aid you in selecting

the next parameter, t, which is the time past perigee, when you wish

to start the orbit.  We will use t = 0.

* Type 0 and press <Enter>

That completes the orbit description.  The next menu is for

specifying the time span of the orbit.  The parameters t0 and tf are the

initial and final times.  We will do one day, about 86400 seconds, which

is a little over 14 orbits.

* Type 0 and press <Enter>

* Type 86400 and press <Enter>

The next parameter, dt, is the time increment.  We will use one

minute, or 60 seconds.

* Type 60 and press <Enter>

The final parameter, pt, is the sampling frequency for the output data

file.  We will use the same value as the time increment.

* Type 60 and press <Enter>

Page 4

The next menu requests your choice of output formats. We will

select the geodetic coordinates which are the most common.

* Type 1 and press <Enter>

You are given the option of text or graphics output.  Select

graphics.

* Type y and press <Enter>

The program begins by displaying the world on three projections

which are described in Section 4.  When the maps are drawn, you will

be prompted to press a key to start the orbit computations.  When the

orbit propagation is complete, your screen should have a figure

resembling Figure 14 in Section 4.  You may select a black and white

palette if you want a screen dump.  Then you may press any key to

end.

Section 5 contains several examples, for you to try, that

illustrate the various trajectories that you can generate with

Trajectory Maker.

Running Trajectory Scape

________________________

To run Trajectory Scape, type TSCAPE at the DOS prompt.  After the

opening banner, a menu appears requesting your choice of world map

projections.

* Type 2 and press <Enter>

Next, a menu appears requesting your choice of map resolution.

* Type 3 and press <Enter>

Next, a menu apppears requesting your choice of world map focus.

* Type 2 and press <Enter>

You will be prompted, in the command window, for an orbit file name.

The output file that was just created by Trajectory Maker is

HOST.DAT.  To display this file,

* Type HOST.DAT and press <Enter>

Your screen should have a figure resembling Figure 19 in Section 7.

You may select a black and white palette if you want a screen dump.

Then you may press any key to end.

The next section describes the input parameters in detail.

Page 5

3.  TRAJECTORY MAKER INPUT DESCRIPTION

Trajectory Maker is very easy to use.  Just go to the subdirectory

where you installed the programs and type TMAKER at the DOS command

line.  You will be presented with a series of five principle menus

requesting the following information (to abort the job at any time,

just press the escape key):

3.1  Unit Convention Menu

_________________________

You are given the option of working in either english or metric

units.  In either case all time units must be input in seconds.  Both

input and output for angles are measured in decimal degrees.

If you choose the English Units Option, then length measurements

must be input in nautical miles, and velocity measurements in

nautical miles per second.

If you choose the Metric Units Option, then length measurements

must be input in kilometers, and velocity measurements in kilometers

per second.

The menu for this option is illustrated in Figure 1.

3.2  Perturbation Model Option Menu

____________________________________

You are given four force model options for tailoring your

particular application.  The menu for this option is illustrated in

Figure 2.

3.2.1  Unperturbed Two-body Option.

This option assumes that the earth's gravity field is modeled with

an homogeneous sphere.  The orbit motion will be along a conic

section ( ellipse, parabola or hyperbola depending on the initial

conditions ).  However, altitude is based on the earth having the

figure of an oblate spheroid.  The altitude of a circular orbit will

therefore not normally be a constant.

3.2.2  Gravitational Harmonic Effects Option.

This option assumes that the earth's gravity field is modeled with

an oblate spheroid ( A surface of revolution generated by rotating an

ellipse about its minor axis ).

If you choose this option, you will be asked to specify a positive

number between 1 and 23.  The larger the number, the more precise the

gravity model.  ( This number corresponds to the order of the gravity

potential function. )  For example, use order 2 for Sun-synchronous

orbits.

Page 6

Figure 1.  Unit Convention Menu.

Figure 2.  Perturbation Model Option.

Page 7

3.2.3  Atmospheric Drag Effects Option.

This option takes into account the drag force created by the

earth's atmosphere.  This force varies with the mass of the object,

its effective cross-sectional area and its drag coefficient.  These

three parameters are commonly lumped together into a single

parameter, which you will ask to supply, if you choose this option.

This parameter is called the ballistic coefficient (BC), and

conveniently has units measured in lbs/sq-ft, or kg/sq-m, sort of

like density.

Heavy masses tend to have larger BCs than lighter masses, but

large areas have smaller BC's than small areas.  A space based radar

antenna may have a BC of 0.5 lb/sq-ft, whereas a reentry vehicle may

have a BC varying from 25 to 1000 lb/sq-ft, for example. The

definition used is consistent with that used at the Western Test

Range.

3.3.4  Gravitational and Atmospheric Drag Effects Option.

You may choose either or both gravity and drag perturbations.  For

reentry trajectories, you should consider this option, with the

gravity model having Order 2 or 4.

3.3  Initial State Vector Option Menu

_____________________________________

You may input the initial state vector in two ways.  Either with

Earth-Centered-Inertial (ECI) coordinates or classical orbital elements.

The menu for this option is illustrated in Figure 3.

Both vector options are aligned with respect to the Greenwich

meridian at the user specified initial time. ( That is, the Greenwich

hour angle is zero at t0. )

3.3.1  Orbital Elements

Orbital elements are classical, fundamental parameters that

characterize an orbit.  They are defined in almost any elementary

book on astronomy, celestial mechanics or astrodynamics.

The input format for the orbital elements is illustrated in

Figure 4.

a - Semimajor axis

__________________

This element defines the size of the orbit.  The semimajor axis of

an ellipse or hyperbola is the distance from their centers to their

respective vertices.  As used herein, the semimajor axis of a

parabola is the distance from its focus to its vertex.  A circular

orbit is a special case of an ellipse.  Its semimajor axis is,

simply, its radius.  Input this parameter with the units selected in

the units option menu.  This element is illustrated in Figure 5.

Figure 3.  Initial State Vector Option Menu.      Page 8

Figure 4.  Initial Orbital Elements.

Figure 5.  Semimajor Axis and Eccentricity.       Page 9

Page 10

e - Eccentricity

________________

This element defines the shape of the orbit.  It is a fundamental

parameter in the definition of conic sections.  It is dimensionless

and can be any nonnegative number.  If it is zero, the orbit is

circular; if it is between zero and one, the orbit is an ellipse; if

it is one the orbit is a parabola; and if it is greater than one the

orbit is an hyperbola.  These configurations are illustrated in Figure 5.

Remark: If the orbit is an ellipse and the minimum and maximum

distance from the earth's center is known ( i.e., its perigee and

apogee radius ) then its semimajor axis is given by their mean and

the eccentricity is their difference divided by their sum.

i - Inclination

_______________

This element defines the orientation of the orbit plane relative

to the equatorial plane.  It may be in the range, 0 <= i <= 180

degrees.  If i <= 90 degrees, the maximum latitude of the orbits

ground trace will be equivalent to i, otherwise the maximum latitude

will be its complement, 90 - i.  Equatorial orbits have zero

inclination, and polar orbits have 90 degree inclination.  This

element is illustrated in Figure 6.

- Longitude of the ascending node

___________________________________

This element defines the orientation of the orbit plane relative

to the Greenwich meridian.  Inclined orbits intersect the equatorial

plane in two points.  The point where the orbiting object intersects

the equatorial plane on its journey from south to north is the

orbit's ascending node. The other point is its descending node.  The

line connecting the two nodes is the line of nodes.  The longitude of

the ascending node is the angle measured in the equatorial plane

between the initial Greenwich meridian and the line of nodes. It may

be in the range,  0 <= W < 360 degrees.  This element is illustrated

in Figure 7.

- Argument of perigee

_______________________

This element defines the orientation of the object's perigee in

the orbit plane relative to the ascending node.  It is the angle

measured in the orbit plane between the line of nodes and the line

connecting the center of the earth and the objects perigee. It may be

in the range, 0 <= w < 360 degrees.  This element is also illustrated

in Figure 7.

Figure 6.  Orbit Inclination.             Page 11

Page 12

Figure 7.  Longitude of the Ascending Node and Argument of Perigee.

Page 13

- Time past perigee

_____________________

This element defines the object's initial location in the orbit

plane relative to the argument of perigee.  It is measured in

seconds, and for elliptical orbits is normally not greater than the

orbital period.  If the orbit is, indeed, an ellipse, the program

computes the orbital period and displays the value on your screen.

This is useful in making a choice regarding where to start the orbit,

and in the Time Options Menu, where to end the orbit.

3.3.2  Initial Position and Velocity

Rectangular, Earth-Centered-Inertial (ECI) coordinates are the

fundamental coordinates used in Trajectory Maker for measuring an

objects motion, i.e., its position and velocity.  Indeed, the

equations that govern the motion are propagated relative to this

reference frame.  If the orbital elements option is selected, they

are transformed into ECI coordinates prior to propagating the orbit.

The input format for these coordinates is illustrated in Figure 8.

As used herein, the principle X-axis lies in the equatorial plane

and is in the direction of the Greenwich meridian at the time of

orbit insertion, or the initial time, and remains fixed in that

direction in space for the duration of the trajectory.  The Greenwich

meridian, of course, rotates relative to this frame.

The principle axis is not in the direction of the vernal equinox

for a couple of reasons.  It requires additional input data.  Namely,

the date, and time in hours, minutes, and seconds.  A simple

longitude shift will align the ECI system relative to a predefined

launch site for ballistic trajectories, e.g., Vandenberg Air Force

Base.  In fact, this ECI option is primarily intended for, but not

limited to, ballistic missile trajectories.  These coordinates are

illustrated in Figure 9.

X - Coordinate in the equatorial plane, positive

in the direction of the Greenwich meridian at

the initial time t0.

Y - Coordinate in the equatorial plane orthogonal

to X.  Approximately in the direction of

central Russia.

Z - Coordinate coincident with the earth's spin axis,

positive northwards.

XD - Velocity component in the X-direction.

YD - Velocity component in the Y-direction.

ZD - Velocity component in the Z-direction.

Figure 8.  Initial Position and Velocity.       Page 14

Figure 9.  Earth-Centered-Inertial Coordinates.    Page 15

Page 16

3.4  Time Control Parameters

____________________________

These parameters control the time span and precision of the

trajectory.  Once the initial state vector has been entered, a

message is displayed relating the "period" of the orbit, an escape

condition or the "Kepler flight time" ( no drag or oblateness ).  The

input format for these parameters is illustrated in Figure 10.

This latter condition occurs if the trajectory is going to impact

with the earth.  These parameters are provided to aid you in your

choice of the following time control parameters.  See also the

remarks below on selection criteria.

t0 - Initial time (seconds).

Time to start the trajectory.

tf - Final time (seconds).

Time to end the trajectory.  If your application is a

reentry trajectory and you are using a perturbation model,

the input value for final time should be larger than the

"Kepler flight time."

dt - Propagation step-size (seconds).

Controls the precision of the computations, and

should be a multiple of tf.

pt - Output frequency (seconds).

You may want to propagate the trajectory at one step-size,

dt, to maintain accuracy, but output the data at some

larger multiple of dt.  If you just press the return key,

with no data entry, the program will default this parameter

to dt.

Numerical experience has shown that for non-drag perturbed

trajectories, a step size of 300 sec will give seven significant

figures accuracy for tf = 86400 sec (i.e. one day), six figures for

14 days, and five figures for 28 days.  This latter case means that

your results will be accurate to 1/10 of a nautical mile after 8064

integration steps.

To maintain stability and numerical integrity for reentry

trajectories, you will have to cut the step-size down to 10, or even

5, seconds, depending on the ballistic coefficient.  You should not

have to go below 1 second, unless you are dealing with very light

masses with ballistic coefficients of the order of 1 lb/sq-ft or

less.

Program execution time will vary depending on your choice of the

above time parameters.  On a 12 MHz AT having a math coprocessor, for

unperturbed orbits, assume 77 msec per step, add 20 msec for each

gravity harmonic, and add 62 msec per step for drag effects.

Figure 10.  Time Control Parameters.         Page 17

Figure 11.  Output Format Option Menu.

Page 18

3.5  Output Format Options Menu

_______________________________

You may choose one of the three output format options from the

menu illustrated in Figure 11.

3.5.1  Geodetic rotating coordinates.

These coordinates are the standard geographic coordinates that are

used for maps: latitude, longitude and altitude.  Geodetic altitude,

h, and latitude,  , are illustrated in Figure 12 .

The altitude of an object is its instantaneous height above the

earth's surface.  Since the earth is modeled as an oblate spheroid,

the polar and equatorial radii differ by about 10 nautical miles,

thus, circular orbits usually will not have constant altitude.

Latitude is the instantaneous angle between the equatorial plane,

and the line that contains the object and is normal to the earth's

surface.

Longitude is the instantaneous angle between the Greenwich meridian

and the projection of the line connecting the object with the earth's

center.

3.5.2  Geospherical inertial coordinates.

This output format contains the magnitude of the object's radius

vector, its right ascension (relative to Greenwich) and declination.

It also contains its inertial velocity magnitude, velocity azimuth and

flight-path angle.  These parameters are illustrated in Figure 13.

The magnitude of the radius vector is, simply, the distance from

the earth's center to the orbiting object.

As used herein, the right ascension of the object is the angle,  ,

measured in the equatorial plane between the initial Greenwich meridian

and the projection of the radius vector onto the equatorial plane.

The declination of the object,  , is the angle between the

equatorial plane and the radius vector.  It is equivalent to

geocentric latitude.

The magnitude of the velocity vector is the speed of the object.

The velocity azimuth angle,  , is the angle measured clockwise from

the northern direction to the projection of the velocity vector on

the local horizontal plane ( the local horizontal plane is the plane

normal to the radius vector).

The flight-path angle,  , is the angle between the velocity vector

and the local horizontal plane.

Figure 12.  Geodetic Rotating Coordinates.      Page 19

Figure 13.  Geospherical Inertial Coordinates.    Page 20

Page 21

3.5.3  Earth-Centered-Inertial coordinates.

This output format contains the object's rectangular, earth-

centered-inertial position, velocity and acceleration components in

double precision.  Their definition is the same as in Section 3.3.

Once you have made your choice for output format, you are given

the choice of either text or graphics output.  If you just want the

data file, choose text format.  If you choose graphics format, you

will get both.  The graphics display is in geographic coordinates

regardless of the output format.

Page 22

4.  TRAJECTORY MAKER OUTPUT DESCRIPTION

The output data is in ASCII format and is contained in two files.

For convenience, one file, HOST.HED, echoes your input data.  The

other file, HOST.DAT, contains your selected output format from the

Output Format Options Menu.  You may wish to rename these files, and

save them for use in the companion program, "Trajectory Scape."

The first record in HOST.DAT contains a flag used in Trajectory

Scape.  Just delete this record, and you can plot any desired data

items with your favorite plotting package.

If you selected Geodetic Rotating Coordinates, HOST.DAT contains

four data columns: time, latitude, longitude and altitude, in that

order.

If you selected Geospherical Inertial Coordinates, HOST.DAT

contains seven data columns: time, radius, right ascension,

declination, speed, velocity azimuth and flight path angle, in that

order.

If you selected Earth-Centered-Inertial coordinates, HOST.DAT

contains ten data columns: time, three position, three velocity and

three acceleration components, in that order.  The three components

represent the X, Y, Z directions.

In addition to the numeric data, you have the option of displaying

the trajectory's ground trace on your monitor while it is being

computed.  The graphics format displays three conformal views of the

trajectory.

(1)  A Mercator projection.

(2)  A stereographic projection of the northern hemisphere.

(3)  A stereographic projection of the southern hemisphere.

Conformal projections are useful because angles are preserved

under the mapping.  Therefore, the relative shapes of objects are

preserved.  Other projections may be viewed with Trajectory Scape

( see Section 6 ).  A sample of the graphics display is shown in

Figure 14.

At the top of the screen there is a chart that displays the

altitude trend as it is being computed.  This is a very useful

display, for you can immediately correlate altitude with location.

The actual numeric altitude values that are being computed are

displayed in a text window at the bottom of the screen.  Time is also

displayed in this window in sexagesimal format, that is,

days:hrs:mins:secs:

When the computations have been completed you may change the color

palette to black and white.  This option is included so that a

readable screen dump may be obtained on a black and white printer.

It should be noted that standard screen dumps give a distorted image

on some printers.  However, there are several commercial products on

the market that will provide color and distortion-free prints.

Figure 14.  Sample Trajectory Maker Graphics Display.     Page 23

Page 24

5.  EXAMPLE ORBITS AND TRAJECTORIES

This section provides initial conditions for various trajectories

that Trajectory Maker is capable of predicting.

5.1 Space Shuttle Atlantis OV-104 Mission STS 44

_______________________________________________

The space shuttle Atlantis was launched on Tuesday, November 26,

1991.  The following orbital elements were obtained from Rockwell

International in Downey, California.

a = 3642.9363 nm      W = 0 deg (Assume)

e = 0.00121           w = 51.0262 deg

i = 28.36957 deg      t = 0 sec (Assume)

Try an unperturbed gravity model (Model 1) with these elements and

the following time parameters:

t0 = 0 sec

tf = 86160 sec ( One sidereal day )

dt = 60 sec

pt = 60 sec

5.2  Repeating Ground Trace

___________________________

If a satellite's period is a multiple of the time it takes for the

earth to make a complete rotation on its axis ( 23 hrs 56 min or

86160 seconds), then the satellite will retrace the same path over

the earth as it did initially.

The following parameters define an orbit whose ground trace

repeats after 15 revolutions.  The period is 5743.84 seconds.

a = 3743 nm       W = 0 deg

e = 0             w = 0 deg

i = 60 deg        t = 0 sec

Try an unperturbed gravity model (Model 1) with these elements and

the following time parameters:

t0 = 0 sec

tf = 172320 sec ( Two sidereal days )

dt = 60 sec

pt = 60 sec

Repeat this case with gravity model of order two (Model 2).  The

ground trace will be displaced a few degrees from the unperturbed

case, demonstrating the effect of a nonspherical earth.

Page 25

5.3  Sun-synchronous Orbit

__________________________

An orbit in which a satellite passes over the same part of the

earth at the same time each day is a Sun-synchronous orbit. The

orientation of the orbit plane will remain nearly fixed relative to

the sun as the earth moves in its orbit.

Such an orbit is achieved by adjusting its orbital elements so

that the regression rate of the line of nodes is just that of the

earth's angular rate about the sun, which is about 1 degree each day

( 0.9856 deg/day ).

The above orbit becomes sun-synchronous if the inclination is

elevated from 60 degrees to 97.61 degrees. The regression rate may be

inspected in the output file, HOST.HED.  Also note that the orbit is

a retrograde orbit.

5.4  Candidate Surveillance Orbits

____________________________________

The next few examples generate repeating ground traces that are

especially good candidates for surveillance and/or communication. The

orbital elements are adjusted such that apogee is in the northern

hemisphere ( w = 270 deg ), where they will hang the longest.

Their inclination is critical (63.44 deg) so that gravity will not

cause their line of apsides to rotate (so that apogee will stay put).

This can be verified by inspecting the file HOST.HED.  A value of

zero, or nearly so, should be given for the argument of perigee rate.

Also, the longitude of the ascending node and time past perigee

can be adjusted to synchronize multiple orbits of these types for

various satellite constellation considerations.  For example, you

could try W = 0, 90, 180, 270 degrees.

Tundra Orbits ( 24 hour period )

________________________________

a =   22766      22766         22766          22766  nm

e =   0.33       0.69          0.69           0.69

i =   63.44      63.44         63.44          63.44  deg

W =   0.00       0.00          0.00           0.00   deg

w =   270        270           290            300    deg

t =   0          0             0              0      sec

Model 1

Unperturbed Gravity Model

t0 = 0,   tf = 86160,   dt = 300,   pt = 300  (sec)

Page 26

Molniya Orbits  ( 12 hour period )

__________________________________

a =  14342      14342  nm

e =   0.69       0.69

i =  63.44      63.44  deg

W =     0          0   deg

w =    270        90   deg

t =     0          0   sec

Model 1

Unperturbed Gravity Model

t0 = 0,   tf = 86160,   dt = 120,   pt = 300  (sec)

An Eight-hour orbit

___________________

a = 10945   nm

e =  0.594

i =  63.44  deg

W =   0     deg

w =  270    deg

t =   0     sec

Model 1

Unperturbed Gravity Model

t0 = 0,   tf = 86160,   dt = 120   pt = 120  (sec)

5.5  An Example Illustrating Orbit Decay

________________________________________

a = 6478 km

e =  0

i = 45  deg

W =  0  deg

w =  0  deg

t =  0  sec

Drag perturbation model (Model 3), bc = 100 kg/sq-m

to = 0, tf = 2000 sec, dt= 5 sec, pt= 5 sec

Page 27

5.6  Orbits Near Transition Region

__________________________________

The next three examples illustrate orbits all having the same

radius of perigee near the transition region from a closed to an open

orbit.

Elliptical Orbit     Parabolic Orbit     Hyperbolic Orbit

_________________     _______________     ________________

a = 40000 nm          a = 4000 nm          a = 40000 nm

e = 0.9               e = 1.0              e = 1.1

i = 80   deg          i = 80   deg         i = 80   deg

W = 180  deg          W = 180  deg         W = 180  deg

w = 0    deg          w = 0    deg         w = 0    deg

t = 0    sec          t = 0    sec         t = 0    sec

Model 1

Unperturbed Gravity Model

t0 = 0,   tf = 200660,   dt = 120   pt = 120  (sec)

5.7  Ballistic Missile Trajectories

_____________________________________

The renaining set of trajectories are defined with initial state

vectors given in ECI coordinates (Initial State Vector Option 2).

These trajectories exit and reenter the earth's atmosphere on a

predefined target destination.

5.7.1  Western Test Range Examples

These two hypothetical trajectories are launched from Vandenberg

Air Force Base with destination in the Kwajalein Atoll.

X = -1705.1574 nm           Model 4

Y = -2509.1712 nm           Gravity model order 2

Z =  1926.6321 nm           bc= 25 #/sq ft

XD=    -3.1022 nm/sec       t0= 200 sec,  tf= 2000 sec

YD=    -0.5247 nm/sec       dt= 5 sec,    pt = 5 sec

ZD=    -0.4826 nm/sec

X =  -2525.03 nm            Model 4

Y =  -2146.25 nm            Gravity model order 2

Z =   2012.86 nm            bc= 75 #/sq ft

XD=     -2.608 nm/sec       t0= 475 sec,  tf= 2000 sec

YD=      1.351 nm/sec       dt= 5 sec,    pt= 5 sec

ZD=     -0.318 nm/sec

Page 28

5.7.2  Hypothetical Threat Trajectories

The next three trajectories are hypothetical Soviet launched ICBMs

with Continental USA destination targets.

A Nominal Trajectory (Reentry angle 24 degrees)

_______________________________________________

X = 1404.72 nm              Model 4

Y =  845.78 nm              Gravity model order 2

Z = 3242.26 nm              bc= 1200 lbs/sq ft

XD = - 0.959 nm/sec         t0= 0,   tf= 1600

YD = - 2.244 nm/sec         dt= 10,  pt= 10 sec.

ZD =   2.436 nm/sec

A Depressed Trajectory (Reentry angle 20.17 degrees)

____________________________________________________

X = 1149.67 nm              Model 4

Y = 1075.78 nm              Gravity model order 2

Z = 3348.70 nm              bc= 1200 lbs/sq ft

XD =  1.007 nm/sec          t0= 0,   tf= 1600

YD = -2.889 nm/sec          dt= 10,  pt= 10 sec

ZD =  1.719 nm/sec

A Lofted Trajectory (Reentry angle 34.91 degrees)

_________________________________________________

X =  980.98 nm              Model 4

Y = 1218.02 nm              Gravity model order 2

Z = 3523.47 nm              bc= 1200 lbs/sq ft

XD = -0.393 nm/sec          t0= 0,   tf= 2400

YD = -1.939 nm/sec          dt= 5,   pt= 5 sec

ZD =  2.787 nm/sec

Page 29

6.  TRAJECTORY SCAPE INPUT DESCRIPTION

Trajectory Scape is a multiple trajectory viewing tool.  Once you

have created one or more geographic output files with trajectory

Maker ( Output option 1 ), you may view any or all of them with

Trajectory Scape.  At the DOS prompt type TSCAPE.  You will be

presented with a series of three menus that simply request choices

for map projections, their resolution and central focus region.

6.1  Word Map Projections Options Menu

______________________________________

You are given the option of three world map projections.

Actually, there are five, but three projections are combined in one

display.  The menu for this option is illustrated in Figure 15.

6.1.1  Mercator with stereographic projections.

A Mercator projection is displayed with stereographic projections

of both the northern and southern hemisphere.  These projections are

perhaps the most useful of all map projections for navigation.  They

have borne the test of time.

They are conformal projections, that is they preserve angles, so

that relative shapes remain the same.  Rhumb lines are straight on

the Mercator projection.  Distortions are maximal in the polar

regions of the Mercator projection, but minimal with the

stereographic projections.  The combination compliments one another.

Examples of these projections are illustrated in Figure 18, Section 7.

6.1.2  Hammer-Aitoff equal area projection.

This projection preserves areas and is best near the center of the

map.  It is similar to the Mollweide projection described next, but

it has less distortion in the polar regions.  Far east and west are

not as distorted. This projection is useful for atlases on physical

geography or for statistical distribution purposes.  It is used in

astronomical work for showing the distribution of stars.  An example

of this projection is illustrated in Figure 19, Section 7.

6.1.3  Mollweide equal area projection.

This projection also preserves areas.  It is also known as

Babinet's equal surface projection.  The distortions increase as you

move away from the center, horizontally or vertically.  Its major use

is for geographic illustrations relating to area such as density and

populations or extent of forests, etc.  It serves somewhat the same

purpose as the Hammer-Aitoff projection.  An example of this

projection is illustrated in Figure 20, Section 7.

Figure 15.  World Map Projections Menu.        Page 30

Page 31

6.2  Map Resolution Options Menu

________________________________

You are given three map resolution options illustrated in Figure 15.

Your tradeoff is between speed and detail.

6.2.1  Fine Option.

This option gives the best detail but is the slowest of the three.

The data file contains approximately 4500 latitude longitude pairs.

6.2.2  Medium Option.

This option gives a good presentation and is about twice as fast

as the fine option.  The data file contains approximately 2000

latitude/longitude pairs.  This option is used in Trajectory Maker.

6.2.3  Coarse Option.

This option gives fair presentation and is the fastest - about

twice as fast as the medium option.  The data file contains

approximately 1000 latitude/longitude pairs.

6.3  World Map Focus Options Menu

_________________________________

Ths final menu, illustrated in Figure 16, allows you to focus on

one of three regions of the world, provided that you selected one of

the equal area projections.

6.3.1  Greenwich Meridian.

The central meridian is the Greenwich meridian.  The center of the

map is at 0 degrees longitude.

6.3.2  American Region.

The central meridian is at 80 degrees west longitude which puts

the focus of the map between North and South America.

6.3.3  Soviet Region.

The central meridian is at 80 degrees east longitude which puts

the focus of the map in central Russia.

Figure 16.  Map Resolution Options Menu.       Page 32

Figure 17.  World Map Focus.

Page 33

7.  TRAJECTORY SCAPE OUTPUT DESCRIPTION

If either the American or Soviet region is selected, the displayed

map will have a yellow and a red meridian.  The yellow meridian

represents the Greenwich meridian and the red meridian represents the

international date line.  These two meridians provide a reference

base.  The graticule is 20 by 20 degrees and is not labeled to avoid

clutter.

Once the map has been generated, the command window at the bottom

of the display asks for the name of a geographic trajectory file to

display.  Simply type the name of your file, and the program will

display the orbit or trajectory.  If you store your data files in

another directory, give the complete path specification.  Once plotted,

you will be asked if you would like to view another trajectory.  The

program will continue until you answer no.

To aid in discrimination, multiple trajectories are displayed in

different colors, a maximum of five.  You may display more

trajectories, however, you are limited to five colors.

Examples of the various projections are illustrated in Figures 18,

19 and 20.  Figure 18 illustrates four "tundra" orbits (Section 5.4)

on the conformal projections.  Figures 19 and 20 illustrate the

circular, 45 degree orbit that you generated in Section 2, on the

Hammer-Aitoff and Mollweide projections, respectively.

Page A-1

APPENDIX A.  HISTORICAL NOTES

In early 1976 while employed under the auspices of the Western

Test Range at Vandenberg Air Force Base, California, the author was

assigned the task of researching the existing trajectory related

computer programs in use on the range, and assess the relative

accuracy of their predicted impact points based on the integrity of

their numerical integration schemes.  Initially, the study was not to

be concerned with the force model, but limited to the integration

process itself.

At that time, the author found that there existed at least eleven

distinct programs lying around for propagating reentry trajectories.

Namely, VPAR/BIPS, MIPS/AMARKIN, FFTRG2, RIP, F&G, ARMS, OASYS,

FIT500, TGP, NITE and TAPP (FIAT).  These programs, all written in

FORTRAN, the following computers: IBM 7094, IBM 360/65, SIGMA 5/7,

CDC 6400/6600, UNIVAC 1108, GE 635.

Their numerical integration schemes covered a fair gamut:

trapezoidal, Taylor series, fixed and variable step fourth-order

Runge-Kutta and Runge-Kutta-Gill, fourth order Adams-Moulton,

eighth-order Gauss-Jackson, variation of parameters, and f & g

series.  Moreover, the force models of the various programs varied.

For example, all of the models included at least two zonal

harmonic coefficients, in either Jeffrey's or Vinti's, potential

function.  One potential function included six zonal harmonics, and

another included nine zonal harmonics and thirty-six tesseral

harmonics.  All models included atmospheric drag but one used the US

1962 Standard Atmosphere, while another used ARDC 1959.  Then there

were the earth constants.

All of the models have their merits and limitations, but when it

comes to computational precision, each of the things above must be

considered.  A tenth of a foot difference in the earth's equatorial

radius may cause unknown output differences.  For example, when

comparing the output of two models, one may observe differences in

the ninth or tenth decimal place, which could be due to computational

precision, or some parameter difference, among other things.  One

must be able to isolate the source of any discrepancy.

But the primary thrust of the study was to evaluate the numerical

integration process as applied to reentry trajectories. Consequently,

only a representative force model was required.  One that would

exhibit the underlying behavior, when the differential equations of

motion were integrated. This model should at least contain a few

gravity harmonics, and more importantly, a representative atmospheric

drag model, which is what really taxes the integrator.

The problem then, became one of isolation.  A baseline integrator

was required, whose precision was known with certainty, in order to

compare the integrators.  Also, a representative force model was

Page A-2

required that could, without too much difficulty, be substituted into

the various trajectory programs, including the baseline model.  The

force model didn't need to be all that exotic, just representative of

the kinds of forces the differential equations would experience.

The gravity model chosen for the baseline model was the one

developed by R.M.L. Baker at CSC for use in their f & g series

approach.  The reason for this was that their program was one of

interest to some range people, and their f & g series approach was

based on high order derivatives of the gravity potential function,

which were embedded in the program code.  This model included the

zonal gravity harmonics of the Vinti-potential of order six.

The drag force model selected was based on the abbreviated,

preliminary tables for the U. S. 1976 Standard Earth Atmosphere.  The

reason for this was that most of the atmospheres used in the various

models were lengthy tables.  The abbreviated tables contained just 21

pairs of altitude versus density, which were more manageable, and

could be accurately modeled with cubic spline functions.  In

addition, the model was the latest and greatest.

The numerical integration scheme chosen for the baseline

integrator was the eighth-order, fixed-step, Runge-Kutta-Shanks'

technique having twelve stages.  This scheme is stable, and was the

most accurate scheme that the author knew of at the time.

A fixed-step scheme was desirable, because one could determine

precisely the accuracy of the computations.  This could be done by

selecting a step-size, integrating the differential equations to a

predefined time, cutting the step-size in half, repeating the

integration and comparing the two trajectories.  This process could

be continued until two successive trajectories matched in their

fifteenth or sixteenth significant figures ( the precision of double

precision arithmetic on the IBM 360/65 ).  Clearly, computation was

of no concern - just accuracy.

Once the baseline force model together with all the relevant

constants was substituted into the other trajectory programs, the

accuracy of their integrators could be determined with a fair amount

of certainty.  Certainty could be claimed if it wasn't for the fact

that the various hardware on which the software resided had some

differences in precision.

It was soon discovered that the baseline model was producing

rather accurate trajectories.  In fact, by shifting density models it

was proven that the USAF "certified" program, FFTRG2, could miss

impact points by as much as two nautical miles, just because of the

manner in which it was interpolating the density profile.  Moreover,

its interpolation scheme, used presumably for computation speed,

actually slowed the integrator down by causing the step-control to

artificially decrease the step-size.

Page A-3

Considerations such as these led to many enhancements of the force

model in the baseline program.  The final version included the

following:

o Six zonal gravity harmonic coefficients.

( Smithsonian 1973 II values )

o U. S. 1976 Standard Atmosphere.

Fitted with cubic splines to the logarithm of the density

profile.

o Drag coefficient as a function of either altitude, or Mach

number.  Fitted with cubic splines or linear depending on the

profile.

o Accurate speed of sound computation based on the 1976

temperature profile.

o Atmospheric wind profiles.  Fitted with cubic splines.

o Innovative altitude computation which produces trigonometric

functions for wind effects, and the topocentric rotation matrix

as a by-product.

The baseline integrator, i.e. the Runge-Kutta-Shanks' fixed-step

scheme, proved to be very accurate.  To speed things up, a dual mesh

step control option was added.

What started out to be an accuracy study for numerical integration

schemes of various Western Test Range trajectory programs ended up

with another trajectory program - a rapid, high precision reentry

trajectory generator.  And that program is the basis of "Trajectory

Maker."

Later, the author joined another aerospace company, and shortly

thereafter, an assignment required a trajectory program.  The

company's program ( actually, their program, RIPR, a variation of

RIP, was bootlegged from Vandenberg) would go unstable for certain

trajectories when they entered the atmosphere.  This led to a

conversion of the baseline program to run on their in-house PDP-11.

The baseline program was used for validating Space Shuttle

ephemerides, and with its use an error in launch azimuth was

uncovered in a NASA simulation program.

Another aerospace company had spacecraft computers in-house for

use in verifying, validating, and testing of the flight software for

a particular USAF satellite system.  In addition they had a separate

computer for simulating the spacecraft's environment.

Page A-4

Several problems were manifested that required the use of an

independent trajectory simulation.  The baseline program was again

modified for use on their in-house Perkin Elmer 8/32 computer.  This

time, all of the drag stuff was stripped out of the program, and

emphasis was placed on the integrator and gravity model.  Clever

closed for expressions were developed and implemented for the gravity

model so that it would include an arbitrary number of harmonics with

rapid computation times.

The program was used for several applications including another

satellite program.  The baseline program was further vindicated when

it matched orbits generated with the Advanced Orbit Ephemeris

Subsystem in use at the Satellite Control Facility at Sunnyvale,

California.

The program was also used for Strategic Defense Initiative

applications.  In particular, it was used to propagate thousands of

missile threat trajectories into the field of view of special

sensors.  It was also used to study the following:

o A Space based kinetic energy weapon interceptor system.

o Evasive enemy spacecraft orbit maneuvers in the Earth's shadow.

o Multi-satellite constellation design for a special sensor.

o Drag effects on a space based rotating radar antenna system.

o Relative motion for a space laser targeting experiment.

Having reviewed the merits and limitations of the "baseline

program" relative to the many applications in which it has been

involved, and modifications it has undergone, the author decided to

redo the entire program from a different perspective, and in a

different language.

There exists a number of software tools available in the market

today that are neat programs for orbital mechanics and astrodynamics

applications, and several were purchased at one of the recent

companies where the author was an employee.  But the fact of the

matter is, all of them were looked at briefly, and put on a shelf,

for none of them did what was needed to be done.  Moreover, the

packages were all in the $2000.00 range.

Invariably, it was the case, that a special tool needed to be

developed to solve the problem at hand.  Moreover, it was usually the

case that individual trajectories needed to be generated and plotted

(usually plotted by hand) in order to shape the underlying problem.

The most useful tool has been the capability to generate a

trajectory, of varying degrees of complexity, depending in the

application, and see what it looked like.

Trajectory Maker is the result of selecting and adding those

features that the author has found to be the most useful in many

aerospace applications requiring the use of a trajectory simulation.