FATman

Disk Utilities for F.A.T. Drives

(FATMAN.EXE)

Copyright 1997 Sound Expressions

5361 Browntonw Rd.

Sawyer, MI  49125

(616) 426 - 4685

Table of contents:

Advanced Features                                         10

Automated File Recovery                                    5

Basic disk structure terminology definitions               3

Before you begin                                           3

Directory Editing                                          8

Directory Sorting                                          9

Disk / System Parameter Display                            9

Disk Usage Map                                             9

Editing Logical Sectors                                    8

Examine File in HEX and/or ASCII format                    8

FATman Error Message List and Explainations               11

FATman License Agreement                                  21

Fees                                                      22

Hidden Features of FATman (for advanced+ users)           18

Manual File Recovery                                       6

Order Form                                                23

Physical File Location                                     9

Purpose, Entry form and command line parameters            2

Recovering Directories                                     8

Searching the disk for data                                8

Understanding File Recovery                                5

Page 2

Purpose:

FATman  Disk  Utilities (FATMAN) provide a  convenient   method

for  manipulating  data   stored  on  fixed  or  floppy  disks.

FATMAN   allows   recovery  of  accidentally   deleted   files,

rebuilding  of  files  using erased data clusters,  editing  of

logical  sectors  and examining and editing any  part   of  the

disk.  FATman  is intended to run under DOS (it should  not  be

executed in a DOS box under Windows).

Target Users:

FATman  was  designed  for users who are fairly  familiar  with

their  system.   However, standard file recovery  features  are

easy  to  use.  Advanced users who understand the  F.A.T.  file

system  (or  who want to explore and learn it) will  find  that

this  utility allows them a tool to cut through the 'red  tape'

of  most  other  utilities  and directly  manipulate  the  file

system.   This tools is powerful in that regard but along  with

that power, you can cause a loss of data if used improperly.

Entry Form:

FATMAN [d:<path/filename>] [/t] [/f] [/d] [/r] [/b] [/c] [/v]

For a quick file undeletion:

FATMAN  [d:path/filename] will bring up FATMAN  to  attempt  to

undelete  a  file from the command line without having  to  use

the  main  menu.  This will look for an erased  file  with  the

same  name  (except for the first character) and  unerease  it.

After doing so you will be returned to the command line.

/T[tcneds][ad]  Invokes  the directory sort  from  the  command

line.   (Note: there is no space allowed between the  T  and

the  2  optional  parameters.) This is useful  for  doing  a

directory  sort from within a batch file.  The default  sort

is  ascending by file type.  The options for the 2nd and 3rd

parameters are:

2nd:

t - sort by type

c - sort by complete file name (including extension)

n - sort by file name (w/o extension)

e - sort by file extension only

d - sort by file date

s - sort by file size

3rd:

a - sort in ascending order

d - sort in descending order

For  example, to sort by file date in descending  order  you

would use:

FATMAN /Tdd

/Fbrings  up  FATman  but  ignores the file  allocation  tables

(FAT).   If  you  have  determined that  the  FAT  has  been

corrupted  it  is sometimes better to ignore it  completely.

In  this  case you may want to try to locate files a cluster

at a time to copy to another disk

Page 3

/Dswitch  instructs  FATman to ignore the root  directory.   If

the  root  directory has been corrupted, on occasion  it  is

better  to  ignore it completely while you  search  for  the

subdirectory clusters to recover most of your files.

/Rinvokes  read-only mode.  This is a safe way to examine  your

hard  disk without any chance of writing to your  disk.   If

you  are new to the structure of the disk on a F.A.T. drive,

this  is  a  safe  way to browse the disk  without  risk  of

accidentally corrupting it.  A second reason for  using  the

/R  switch  is when running under on operating  system  that

protects  the disk from programs the try to access the  disk

directly (without going through the operating system).   For

example  to examine a FAT drive under a DOS box in OS/2  you

would  need  to  use /R to avoid OS/2 error  messages  while

using FATman to look at the drive.

/Vswitch  will  use  BIOS  to write to  the  video  instead  of

directly  accessing video memory.  Only very  old  computers

would need this feature.

/C /B    To  set Color or B&W mode.  Black & white mode may  be

useful  on  some laptop machines to be easier  to  read  the

screen.

Using FATMAN:

Because of the complexity of FATMAN, you should consider the

following before proceeding:

>Make a backup copy of your disk before using FATMAN.

>Do  not write to your disk if you are attempting to recover

an   accidentally erased file. Writing to  your  disk  may

cause data in  deleted files to be permanently lost.

FATman  Disk  Utilities are accessed through  a  menu-driven

interface  with on-screen prompts and helps.  Users  with  a

limited  knowledge  of the MS-DOS file  system  can  recover

accidentally  deleted files. The more advanced  features  of

FATMAN  allow  the  experienced user to make  detailed  data

corrections, edits or general disk snooping.

To make selections in FATMAN, use the arrow keys to position

the highlighted bar over the FATMAN menu you wish to execute

and press ENTER. The HOME key returns you to the top line of

a menu and END key takes you to the last line of a menu. For

most  menus,  the  ESC key will return  you  to  either  the

previous menu or the main menu.

Definitions:

Before  you begin to use FATMAN, be sure you understand  the

following definitions.

Page 4

Sector:  Smallest unit of storage accessible on  a  disk  or

diskette.   Usually a sector is 512 bytes  (characters)

of information.

Cluster:   A  cluster consists of one or more sectors.   The

number  of  sectors per cluster is fixed and determined

when the disk is formatted and is always a multiple  of

two.   Think of a sector as a folder in a file  cabinet

drawer for all the ``S'' Information, and  your program

needs  only  two folders in this drawer.  Your  program

may  need only two files or sectors, but it would still

reserve  the entire drawer or cluster because a cluster

is  the  smallest amount of disk storage  that  can  be

allocated to a file.  All files consist of one or  more

clusters.

If   a  cluster  is  active,  it  contains  valid  data

allocated  to a file.  If it is erased,  the file  that

used  the data in this cluster has been deleted or  the

cluster  has never been allocated to a file.   Clusters

may  still  have data in them even though they  are  no

longer allocated to a file.  The data stays there until

the cluster is reused by a new file.

Boot Sector:   Sector 0 (the boot sector) on  your  disk  is

reserved  for the disk information necessary  for  your

computer  to boot.  Altering information here may  make

your system unbootable. FATMAN will caution you if  you

attempt  to  edit  the  boot  sector.   If  you  change

anything in sector 0, the computer may not be  able  to

properly access the disk.  Note that FATman will  allow

you  to edit the boot sector so advanced users who  are

very  knowledgeable about the system  can  edit  it  if

needed.

Logical  Sector: MS-DOS treats a disk as if it were made  up

of  linearly arranged sectors numbered 0, 1, 2, and  so

on, up to the size of the disk, although physically the

disk sectors spiral around the disk. Because of the way

MS-DOS  accesses a physical sector (linearly) we  refer

to the sector as a logical sector.

File Allocation  Table  (F.A.T. ... hence the  name  FATman)

stores  locations  of clusters that a  particular  file

might  use. For instance if your program uses  clusters

11, 12, 14, 25, 26, and 45, this information is kept in

the FAT. When your computer wants to load your program,

the  FAT tells it which clusters contain your program's

data.

HEX/ASCII:   FATMAN   displays  disk  data   in   both   HEX

(Hexadecimal) and ASCII format. Hexadecimal refers to a

numerical  representation of the disk data  using  base

16. ASCII is a displayable character representation  of

the Hexadecimal value. As an example, 61 HEX translates

Page 5

to an "a" when displayed in ASCII form.

Data Display Format:

FATMAN simultaneously displays data in both Hexadecimal

and  ASCII  form. Any of these formats can be  selected

(or  reselected)  by the user at any  time  for  easier

interpretation of the data. The F2 key  allows  you  to

make these selections from the View Selection menu.

>Hexadecimal:    Useful    for   systems-level    file

examination.

>8-Bit  ASCII:  Displays  all  ASCII  and  PC  graphic

characters.

>7-Bit  ASCII:  Strips  the  high  bit  to  make  word

processor files more readable.

>80-Column  Text (8 bit): Display data in  full-screen

text format in 80-column text.

>80-Column  Text (7-Bit): Display data in  full-screen

text  format  after stripping the high bit  (useful

for word processor file formats)

>Directory  Format:  The  data  can  be  displayed  in

directory  format so the file information  such  as

size  and date can be easily deciphered if you  are

looking  at a directory cluster.  You will be  able

to  recognize directory clusters if you have become

familiar with disk format.

Understanding File Recovery:

It  is important to realize that when you delete a file, you do

not  actually destroy the data in that file; you just tell  the

computer  to  free the space the file occupied so  other  files

can  use  it. Specifically, the computer changes the values  in

the   FAT   to   zero  and  puts  the  special  character   (E5

Hexadecimal)  in the directory's file name. The next  time  the

computer  needs space for data storage, it first  goes  to  the

FAT  and  finds enough free clusters (those labeled as  unused)

and  then writes the data in them. After you delete a file  the

data is still there; it is possible to recover the data in  the

deleted file unless you write over it by writing to the disk.

Automated File Undeletion/File Recovery:

If  you accidentally delete a file, you still have an excellent

chance  for  complete file recovery using one of  two  powerful

options for automatically recovering accidentally erased  files

can be selected with Undelete a File.

>Cluster  Analysis  Priority option. (This  is  the  default

option.)

>Cluster Sequence Priority option.

Cluster Analysis Priority:

Cluster Analysis Priority (the default option) analyzes  the

data type for the file being recovered. For example, program

files  are in binary form, and word processing files are  in

Page 6

text form. When the disk is searched to locate the data  for

the  deleted file, each cluster suspected to belong  to  the

deleted file is analyzed to see if it is the proper type.

This   option   will   recover   files   successfully    and

automatically where other file recovery utilities fail.  The

quick  undelete  from the command line  uses  this  type  of

recovery.

Cluster Sequence Priority Option:

The Cluster Sequence Priority option uses a cluster sequence

type   of   recovery  similar  to  what  other  commercially

available  recovery utilities use. Beginning with the  first

cluster  of  the  file  (which is still  maintained  in  the

directory)  the next sequence of erased clusters is  assumed

to  belong to the deleted file. On occasion, this option may

be more successful than the Cluster Analysis Priority option

especially when recovering .COM or .EXE files.

If Automated Recovery Fails:

If  one  of  the  automated options fails to recover  the  data

correctly,  the file created when you attempted to  recover  it

can  again be deleted using the DEL command, or the Delete File

selection  from FATMAN's main menu. Deleting the  file  returns

it  to  the  same  state  as before the  recovery  attempt  and

another  option may be tried. In the unlikely event  that  none

of  the  automated undeletion options provides  100%  recovery,

the  manual recovery feature Rebuild File can be used to search

the disk for missing pieces of the file.

Manual File Recovery:

If  automated  options for undeleting a file were unsuccessful,

the  file  may  be  recovered manually  with  Rebuild  File  or

Directory.  You will first be asked to select a search  option.

Select  the  appropriate option corresponding to  the  type  of

file  you  are  recovering.  For  example,  use  Search  Erased

ASCII/Text  Clusters  to search for an  erased  word  processor

file.

To  select  the  file  you wish to recover  from  the  list  of

deleted  files, use the arrow keys to highlight your  selection

and  press  ENTER. Then press the missing first letter  of  the

filename.  If your file does not appear in the list, press  INS

and  enter the file name. If there are no recoverable files  in

this  directory,  the INS option will be invoked  automatically

and  you  will be prompted to enter a filename. Since  you  may

also  recover  a directory, you will be asked whether  you  are

recovering a file or directory. Also note that if the file  was

not  in the list, the file size will initially show zero  since

you will determine the file size by how much data you recover.

FATMAN  will  search for the first erased cluster suspected  to

be  part  of your file and display the data in standard  FATMAN

Hexadecimal  and ASCII format. Look at the data  displayed  and

determine  if it is part of your file. Press F6 to  accept  the

Page 7

cluster.  Include or Exclude will be highlighted at the  bottom

of  the screen, indicating whether the currently displayed data

has  been  accepted. The View Menu selected  with  F2  presents

choices  for  viewing  the data which may  make  it  easier  to

determine  whether  the cluster was part of your  file.  Notice

that  the number of clusters recovered is indicated at the  top

right of the screen.

Any  of  the following options can be used to select  the  next

cluster for consideration:

Next  erased  cluster  (F9) - selects the  next  cluster  in

sequence,  regardless of data type. Once  you  have  found

one  cluster  belonging  to your file,  there  is  a  good

chance  that  some  of the file's other  clusters  are  in

sequential order.

Current  search option (F7) - locates the next cluster using

the option displayed on the third line from the bottom  of

the  screen. You may change this option at any  time  with

the F10 key.

String Search Option (F9) - prompts you for a search string.

This  is  the most useful and powerful option for locating

data.  If you are rebuilding a text file (such as  a  word

processor  file), type a word or phrase you  remember  was

part  of  the  file. It is recommended that you  set  your

View  Menu  option  (with F2) to 7-bit ASCII  for  a  text

file.  After  you  enter  a search  string,  you  will  be

prompted   to  select  case  sensitivity.  If  you   don't

remember   if  the  words  you  are  searching  for   were

capitalized, set case sensitivity to OFF.

When  your  file recovery is complete, press the F4  key  to

save  the  file.  If, while performing recovery  you  cannot

remember what data you have already recovered, press  F1  to

review  this data. When you are ready to return to  recovery

mode, press ESC. Remember that the last cluster of the  file

will  likely have unrelated data at the end. You can  remove

this  extra data using your word processor or a text  editor

after you have completed the recovery.

Following  is a summary of the actions of the function  keys

used  during  manual file recovery with Rebuild  a  File  or

Directory:

>F1  Review the clusters previously selected for inclusion

in the rebuilt file.

>F2  View  clusters in the format selected from this  View

Menu.

>F3 Cancel the recovery attempt.

>F4 Complete and execute the recovery.

>F5 Select the next erased cluster.

>F6  Mark  a  cluster  for inclusion or exclusion  in  the

recovered file.

Page 8

>F6  is  a  toggle function, allowing you to switch easily

between include and exclude.

>F7  Select  next cluster according to the current  search

option set with the F10 key.

>F9 Select the String Search function.

>F10   Show   the   Options  menu,  listing  choices   for

searching.

Recovering a Subdirectory:

Removed  subdirectories may be recovered with the Rebuild  File

or   Directory  feature.  Erased  directory  clusters  can   be

searched  for  the  name  of a file  contained  in  the  erased

directory. To search for a file in a deleted subdirectory,  use

the   file  name  without  the  extension,  remove  the   first

character  and use no "." after the file name. For example,  to

look   for   a  removed  directory  that  contained  the   file

LETTER.DOC  the search string should be ETTER.  Since  you  are

rebuilding  a  directory,  set  the  search  option  to  search

directory  clusters. The cluster(s) found will be made  into  a

subdirectory rather than a file.

Disk Searching:

You  may  search for text, binary or a combination of text  and

binary  data.  Searching  options  can  be  accessed  while  in

List/Edit  mode,  Rebuild  File mode,  or  Search/Display  Data

Clusters  mode. You can search the entire disk or only selected

areas by using the following search options.

List/Edit - Allows you to search a particular file only.

Rebuild  File - Allows you to search erased binary, text, or

directory clusters.

Search/Display Data Clusters - Allows searching as follows:

>search entire disk (including erased & unused areas)

>search selected file

>search only text clusters

>search only binary clusters

>search only directory clusters

>search only erased clusters

>search only active clusters

List/Edit in Hex and ASCII:

Files  can  be edited in either ASCII or Hexadecimal mode.  All

MS-DOS  files  are  allocated storage space one  cluster  at  a

time.  Therefore,  the  last cluster  of  most  files  is  only

partially  used. If the end of the file occurs before  the  end

of  the last cluster, List/Edit File marks the end of the  file

with  a blinking "<". (Note that there will likely be unrelated

data  beyond the "<" which was left there by another file  that

previously used the cluster.)

Editing Logical Sectors:

All  sectors  on  the  disk (or fixed disk  partition)  can  be

Page 9

edited   with   the   Display/Edit  Logical  Sectors   feature,

including  system  areas of the disk such as the  boot  sector,

file  allocation table, sectors, and directory sectors. Do  not

attempt  to  edit the system areas of the disk unless  you  are

very  familiar with MS-DOS disk structure because of the chance

of  making  your disk unusable. Sector editing can be performed

in either Hexadecimal or ASCII mode.

Physical File Location:

The  cluster  numbers used by each file are  displayed  by  the

List/Edit  File  and Display FAT Cluster Chain  features.  This

information  indicates  where a file is physically  located  on

the  disk, whether or not the file is fragmented, and what type

of data is contained in each cluster.

Directory Display and Editing:

The  Display/Edit Directory feature displays all files  in  the

directory.  This  menu  displays all directory  entries  rather

than  the  limited  list  shown  by  the  standard  MS-DOS  DIR

command.  The  complete list of directory entries includes  the

following:

valid file names

erased file names

directory names

erased directory names

hidden directory names

system file names

hidden file names

read-only file names

Activate  the Directory Edit Mode by selecting a file from  the

names  displayed. In this mode, entries in the directories  can

be  edited  to change file names, attributes, times, and  dates

as noted in the on-screen help.

Directory Sorting:

Individual  directories can be sorted into a logical  order  to

make   locating  files  in  a  directory  much   easier.   Once

directories   are  sorted  they  remain  sorted  after   FATMAN

terminates.  Directories  can  be  sorted  with  one   of   the

following options in either ascending or descending order:

file type (.EXE, .COM etc.)

complete file name (alphabetically)

file name only (excluding extension)

file name extension only

file date & time

file size (in bytes)

Disk Usage Map:

Displays  a visual map of the disk which indicates the physical

location  of  unused  clusters as well as  bad  clusters.  This

visual  map give you an indication of the status of  your  disk

space usage.

Display System Parameters:

System   Parameters  Contains  three  menu  screens  of  system

Page 10

information

Page 1  -  Logical  Drive  Parameters  Displays  detailed  disk

parameter  data such as data capacity, sectors per  track,

sectors  per  cluster, system ID, number of heads  and  so

on.

Page 2  - System Parameters Displays operating system version #

fixed  drives  # floppy drives # serial ports  #  parallel

ports  CPU  type (e.g. 8088/80286/80386) math  coprocessor

(y/n)   total  memory  extended  memory  expanded   memory

display  adapter  type  logical  drive  letters  networked

drive letters

Page 3  -  Fixed Disk Parameters Displays Total Capacity, Heads

(sides) Cylinders (tracks) Sectors per cylinder.

Advanced Features:

Advanced  features  is a sub-menu of additional  Utilities  for

Advanced Users. The following features are provided for use  by

advanced users or systems programmers:

Identify  file  from  sector number - Searches  the  directory

tree  to determine what file or directory is using a given

sector  number. This could be useful if, for example,  you

search  the disk to locate a given text string in a sector

(see Disk Searching) then want to locate the file that  is

using that sector.

Identify  file  from  cluster number - Searches  the  directory

tree  to  determine what file or directory  is  using  the

cluster  you  entered.  In fixing disk  problems  you  may

find  the  information in a cluster by searching the  disk

(see  Disk  Searching).  If you FAT  indicates  that  this

cluster  is being used by a file, you may want  to  search

the  disk to determine what file is using it.  If no  file

is  using  the cluster, yet it is marked as in  use,  then

the  cluster may be a 'lost cluster' (marked is in use but

not 'claimed' by a file).

Edit/Search the File Allocation Tables (FAT) - provides a  very

convenient  method for advanced users  to  edit  the  FAT.

Manipulating the FAT by hand may be necessary  to  recover

a  badly  corrupted disk.  However, this is risky  if  you

don't  know  what you are doing.  Use these features  with

caution.  One example of needing to manipulate the FAT  by

hand  is to correct cross-linked files where several files

clam  to  be  using  the  same cluster.   This  is  tricky

because  first you must determine whose cluster it  really

is.   Then  the FAT chain can be edited with this  feature

to  correct it.  You can also search the FAT for  a  given

entry (see on-screen help).

Display  fixed  disk boot sector - displays the first  physical

sector on a fixed disk in binary form.

Page 11

Display/Edit/Copy  Logical  Sectors  -  Allows  sectors  to  be

edited  and  displayed,  and when  you  are  finished  the

sector  may be copied back to the same sector, a different

sector, or a different logical drive.

FATMAN Error Messages

FATman Disk Utilities (FATMAN.EXE)

Error Message Listing and Documentation

Compiled for FATMAN Version 6.00

Following is a list of messages which could appear while  running

these  utilities.  Following each message is an  explanation  and

possible causes.

" Not Enough Memory Error "

When  FATMAN  is booted space for several large  buffers  is

requested of the system. If there is insufficient memory  to

fill  the  request  this message will appear  before  FATMAN

terminates back to DOS.

" Path Name Error "

Under normal operation on a valid disk (one for which CHKDSK

shows  no errors) this message is unlikely. It is caused  by

attempting  to  read  or change to an invalid  subdirectory.

Note  that  it  is not too difficult to accidentally  induce

this error if you are editing a disk directory. For example,

log  FATMAN  onto  a subdirectory so that the  name  of  the

subdirectory appears on the bottom of the main menu  screen.

Then  bring up the root directory sector under "Display/Edit

Logical  Sectors"  which contains the name  of  our  current

default  directory.  Now press F8 to  edit  the  sector  and

change  the  name of our subdirectory. When editing  sectors

FATMAN  must  always assume that the user may  have  made  a

major  change to the system during his/her editing  session.

Therefore, when you press F8 again to write the changes back

to the disk, FATMAN reinitializes the system and attempts to

read our current default directory again. However, since  we

just  changed the name (by brute-force) FATMAN  cannot  find

the  path  name it is looking for. FATMAN has no  option  at

this point but to terminate back to DOS with the above error

message.

" Unable to determine disk media type! "

This  message  is most likely caused by a bad disk  with  an

invalid logical boot sector.

" Invalid Command Line Parameter "

An  unrecognized command line switch or invalid data on  the

command line.

Page 12

"Error  in  reading File Allocation Table sectors!  Returning  to

DOS..."

If   FATMAN  encounters  a  bad  sector  in  the   FAT   the

organization  of  the  disk cannot  be  determined  and  the

program must terminate.

" Drive not responding "

User  has  probably finished working on a floppy and  pulled

the  disk out while FATMAN was logged onto that drive.  When

the user tried to return to the main menu FATMAN checked and

determined  that  the disk wasn't there.  When  any  key  is

pressed  to continue the user is prompted to select  another

drive.

" DOS     3.0+ required! "

A version of DOS prior to 3.0 will not work with FATMAN.

" Directory Too Large for Available Memory "

Early release of FATMAN would handle a directory with up  to

512  entries. Current release will handle up to 1024 entries

in  a subdirectory. The user will not be able to operate  on

this particular directory due to it's large size.

"  Number of clusters does not match the directory file size.  An

error  is apparent in either the directory or the File Allocation

Table. Press ESC to quit, any other key to continue. "

This  message is invoked by the "Display FAT Cluster  Chain"

utility.  The number of clusters in the FAT chain  does  not

correspond  to  the  size of the file as  indicated  by  the

directory entry for this file. After pressing any key  while

this message is displayed one of two things will happen.

1. The  message  window  will clear  and  no  more  cluster

numbers will appear. This indicates that the file  size

indicated  by the directory is greater than  the  space

actually allocated by the FAT.

2. The  message  window will clear and 1 or more additional

cluster numbers will be displayed. This indicates  that

more  space  is  allocated by  the  FAT  than  what  is

indicated by the directory's file size entry.

Either of these conditions may easily be corrected by  using

"CHKDSK /F" which will report the message :

DRIVE:\<path>

Allocation error, size adjusted.

" There are no clusters allocated for this file. "

It  is possible to have a file in the directory which has  a

valid  file  size  of  0. In other cases  a  prior  disk  or

software crash could have caused the first cluster number in

the  directory  to  be  written with a  zero.  If  the  user

displays the FAT chain for this file this error message will

be displayed. Note that in the event of a disk problem, such

as  a  file size mismatch, a file could indicate a  non-zero

file size in the directory but display this message when  an

attempt is made to show the FAT chain or list the file.

Page 13

" Invalid file name! "

The user has entered an unacceptable file name.

" Invalid Search String "

This  will display if the user enters a null string  at  the

"Enter search string" prompt.

"Clusters  used  by  erased files have been  allocated  to  other

files.  You may still be able to recover some lost data by  using

the REBUILD FILE feature."

I suspect that this may prompt numerous questions from users

of  FATMAN who are attempting to recover deleted files. This

message  appears  after the user has  selected  "Undelete  a

File"  from  the  main menu. This message  may  appear  even

though there are erased file names which appear in the  main

menu  selection "Display Total Directory". This  is  because

the undelete feature of FATMAN has checked every erased file

in  the  current  directory and determined  that  the  first

cluster  for  each  of these files has  been  reclaimed  and

allocated to other files. (Obviously in this case  the  disk

has  been  written  to after the files  were  deleted!)  The

purpose of this message is to inform the user that this does

not  necessarily  indicate that ALL  of  the  data  is  lost

(although  that may, in fact, be the case).  Let's  take  an

example:

Assume  I  have a document in a text file on a  floppy  disk

which  allocates one sector per cluster and this file  takes

up  20  clusters. I delete this file and then write  a  very

small  file  to  the disk which only takes up  one  cluster.

Let's  also assume that when I wrote the small file  to  the

disk,  the new file used what was the very first cluster  of

my  old file. Now I realize I didn't want to delete the file

so  I attempt an undeletion using FATMAN. Either the name of

the  deleted  file does not appear on the list of  files  to

select  from  or  the  above message pops  up.  The  message

correctly  informs me that all may not be lost and  that  by

using the "Rebuild File" feature I may be able to search the

erased  areas  of the disk and find some of my file's  data.

However, FATMAN doesn't know for sure where to start looking

for the file's data since the first cluster in the FAT chain

of  the old file was re-used by a new file. As you can  see,

it  would still be possible to recover 19 out of 20 clusters

of  my file since only one was written over by the new file.

(This  could  still save countless hours  of  retyping  even

though some of the text will still be missing.)

"Not  enough erased clusters were found to reconstruct the  file.

If you are recovering a program file such as a .COM or .EXE file,

the other undelete method option may be more successful. For text

files (such as word processing files) the clusters recovered  may

still  represent  a  large enough portion of  the  lost  file  to

warrant  using the recovered file. To discard the recovered  file

and/or  retry  using  the  other  undelete  option,  press   ESC.

Otherwise,  the  file will be reconstructed  using  the  clusters

found."

Page 14

This  message appears after an undeletion attempt in which  not

enough clusters were found to completely reconstruct the  file.

This  is  a  very  important  message  which  should  be  fully

understood  before deciding whether to press  the  ESC  key  to

cancel  the  file recovery or any other key to go ahead.  First

we'll  look  at what this message is really saying.  FATMAN  is

telling us this:

Using  the  method  of  undeletion  you  selected,  I   have

sequentially searched the disk for clusters which could have

belonged  to  this file. I did not find enough  clusters  to

make  the file as large as the directory entry says  it  was

before it's deletion. Therefore, I need to know what  to  do

from this point. There are two options.

1)  I can go ahead and make a file out of all the clusters I

did  find.  This file will be smaller than  the  file  was

before  you deleted it. This means I will have  to  change

the  file  size  in  the directory to  correspond  to  the

actual  size the file will now be. If we do that, we  will

now  longer know what the size of the file used to be.  If

the  file  your are attempting to undelete is a text  file

(e.g.  a  file from your word processor as opposed  to  an

executable file with the extension ".COM" or ".EXE")  then

the  data  I  have located represents all the erased  text

data  which was on the disk and you might as well  make  a

file  out  of it. If this is you decision then just  press

any key and I will proceed.

2)  I can cancel this undeletion, leave all the data I found

alone and return to exactly where we started from. If  the

file  you  are attempting to recover is not  a  text  file

(e.g.  it has a filename extension such as ".COM", ".EXE",

".OVL",  ".OBJ")  then  might I  recommend  we  take  this

option.  If  you  used  the  "Cluster  Analysis  Priority"

method  then  you  will probably want  to  try  using  the

"Cluster  Sequence Priority" method before doing  anything

else.  If you want me to take this action, just press  the

ESC key."

In  case  you  need  more detail on exactly what  is  going  on

inside  of  FATMAN, let's discuss the undeletion  options.  The

cluster analyses priority method looks at the first cluster  of

the  file  (which  is still indicated in the  erased  directory

entry)  which  we can assume at this point still  contains  the

data  from  the erased file. (However, note that  there  is  no

guarantee  of  this if the disk has been written to  since  the

file  was erased.) From here on any erased cluster on the  disk

could,  in  theory  at least, have been a part  of  the  erased

file.  The  clusters could have been chained  together  in  any

order  imaginable.  However, using information  about  the  way

MSDOS  allocates  files to begin with, we can  examine  suspect

clusters  in a logical ordering which assures us accurate  file

undeletions for a vast majority of the cases.

Before  proceeding,  this first cluster is  processed  using  a

proprietary  Zenith-developed algorithm  to  determine  whether

Page 15

the  data  was part of a text file or a binary file.  (I  would

say  that  this algorithm is correct about 99% of the time.  In

the  event the file was a ".COM" file and the programmer  coded

all  of the text messages at the front of the source file there

is  a  small  possibility that the algorithm could  be  tricked

into  thinking this is a text file.) The reason for  this  data

analysis is that if we are undeleting a text file we should  be

able  to assume that no clusters containing binary data were  a

part  of  the original file. Therefore, in the cluster analysis

priority   method,  each  cluster  is  analyzed  and   selected

accordingly.

On  the  other hand, the "Sequence Priority" does no  analysis.

It  simply  searches sequentially for the next  erased  cluster

and  assumes  that it is the next cluster in the file.  Now  we

can  see  why the sequence priority method may not  select  the

correct  clusters any time the deleted file was fragmented.  On

the  other hand, experience has shown us that in a small number

of  cases  there are large portions of some ".COM" files  which

contain  only  text data for the messages in  the  program.  If

these  text  message contain no executable code in between  and

take  up  at  least  one contiguous cluster, then  the  cluster

analysis priority undeletion will fail to include this  cluster

since  it  looks  like  a text file (and we  are  undeleting  a

binary  file).  This is why the error message  recommends  that

the other recovery option be attempted for binary files.

Note  that  a  sequence priority undeletion is the only  method

that  other  file undeletion utilities use. This is  why  there

have  been numerous cases where FATMAN "Analysis Priority" will

successfully undelete a file where others fail.

" --- Duplicate File Name --- "

The user has attempted to re-use an existing file name.

" --- Improper file name --- "

The  file  name in question has unacceptable  filename

characters.

" --- Erased cluster not found --- "

" --- Active cluster not found --- "

" --- Directory cluster not found --- "

" --- Erased text cluster not found --- "

" --- Text cluster not found --- "

" --- Active text cluster not found --- "

" --- Erased binary cluster not found --- "

" --- Active binary cluster not found --- "

" --- Binary cluster not found --- "

" --- Cluster NOT Found (using current search option) --- "

These messages (which really aren't necessarily errors)  may

appear when searching for data in erased clusters of a disk.

In   the   utilities   "Rebuild  File  or   Directory"   and

"Search/Display Data Clusters" the user may select what type

of  clusters he/she wants to look at. If he/she is trying to

rebuild  a  text file then it's no use looking  at  clusters

Page 16

containing binary data. If the user has selected the  option

to  only  display  text  clusters,  (using  the  F10  option

selection)  when  he/she presses the  F7  key  (Next  Search

Option)  FATMAN  will  search the disk  for  the  next  text

cluster. Note that these searches will search to the end  of

the  disk  and  then  wrap around  to  the  front  and  keep

searching  up until just before the point where  the  search

started.  If the type of cluster we are looking for  is  not

found  by  this time then the appropriate message  from  the

above list will be displayed.

" --- Cluster containing string not found --- "

This message is displayed by the string search function (F9)

which  is part of the utilities "Rebuild File or Directory",

"List/Edit  in  Hex  and  ASCII"  and  "Search/Display  Data

Clusters".  If  the entire disk is searched (searching  only

the  type  of  clusters the user has selected with  the  F10

search  option)  and  the string is  not  found,  then  this

message is displayed.

" --- ESC cannot cancel this procedure --- "

" Press: "

"    F3 - to discard work and exit"

"    F4 - to save work and exit"

In  the "Rebuild File or Directory" utility the ESC key will

not cancel the utility and return to the main menu (which is

the  standard use of the ESC key in the FATMAN  package)  if

the  user  has already located some of his lost data.  Since

the  user  may be tediously rebuilding a file by hand,  care

has  been  taken in the design of FATMAN so the user  cannot

accidentally loose his/her work by accidentally hitting  the

ESC  key. Therefore, when the ESC key is pressed after  some

erased clusters have already been selected, this message  is

displayed  to give the user the opportunity clarify  his/her

decision.

FATMAN   needs  several  important  buffers  in   order   to

successfully implement these utilities. In order  to  reduce

the  amount of necessary memory some buffers are implemented

as  virtual buffers. Therefore, on large disks there is  the

possibility that part of the FAT may have been written  back

to  disk while the user is still searching for more data. If

this has happened, the message below will appear rather than

the  one discussed above when the ESC key or the F3  key  is

pressed.

" --- You cannot cancel this procedure --- "

"  Part of the File Allocation Table has been written to disk. To

discard work you must:"

"     F4 - to save work and exit"

"     Delete the file you've created"

If  the  F3 key is pressed while rebuilding a file  by  hand

and,  due to the buffering of the FAT information,  some  of

the  FAT has been already written to disk then the file must

be  saved then deleted to cancel the rebuild operation which

Page 17

was  in progress. Note that this will still leave the status

of  the  entire disk exactly like it was before the  rebuild

operation was invoked.

"  You  are rebuilding a directory. However, the cluster you  are

trying  to  include is NOT A DIRECTORY CLUSTER!  Select  the  F10

Option - Search Erased Directory Clusters, and press F7 to locate

the next acceptable cluster."

When  a  user  must undelete a directory using the  "Rebuild

File  or  Directory" utility, FATMAN knows that a  directory

undeletion is in progress. When the user presses the F6  key

to  include  a  particular cluster in the  directory  FATMAN

checks the cluster to see that the data about to be included

in  the  directory is valid directory data. (The user  could

make  a real mess out of the disk by making the system think

that  some other cluster is a directory!) If the cluster  is

not   a  directory  cluster  then  this  error  message   is

displayed.

"Error: Cannot QUICK UNERASE a directory!"

"     Use "Rebuild File or Directory" from FATMAN main menu."

This message appears only when the user has tried to use the

"Quick Undelete" method by typing FATMAN <d:filename> at the

DOS command line to unerase a directory.

"You  have  selected an empty file which has  no  allocated  data

clusters."

FATMAN was asked to list a file of size 0 or one that has no

data clusters assigned to it.

" -- File Allocation Table Error -- "

This  is  a very unlikely error message. A bad directory  or

file  size  error  could possibly cause this  under  certain

circumstances.

"The sector about to be accessed is marked as a bad sector in the

File  Allocation Table! An attempt to read/write this sector  may

result in read/write error or invalid sector data! --- Press  ESC

to cancel this read/write --- "

The  user could be attempting to edit a sector which is part

of  a  cluster marked as bad. This is simply warning him/her

that  there  is no guarantee regarding the validity  of  the

data in this particular sector or cluster if they decide  to

proceed.

"    FATman Disk Utilities may not be used on a network drive. "

This  type  of utility would be a disaster if someone  could

use  it over a network. (It's also not possible since FATMAN

bypasses  DOS  to  manipulate the  disk.  As  a  rule  these

instructions are not implemented over networks.)

" Write Protected Disk "

" Invalid Drive Number "

" Drive Not Ready "

" Unknown Command "

Page 18

" Disk Parity Error "

" Bad Request Structure "

" Disk Seek Error "

" Unknown Media Type "

" Sector Not Found "

" Printer Out of Paper "

" Disk Write Error "

" Disk Read Error "

" General Disk Failure "

These  are standard MSDOS disk errors as documented in MSDOS

manuals.  If one of these error codes is detected by  FATMAN

the appropriate message is passed on to the user.

"Root  directory  appears  to have been  corrupted!  Subdirectory

appears to have been corrupted! "

When  a  directory is selected to become the current default

directory, FATMAN tests for obvious errors (such as  garbage

in  the  active part of the directory) and if  problems  are

found one of these messages reports it. Keep in mind that it

is  possible  for certain bytes of directory entries  to  be

corrupt   with   no   ill   effects   whatsoever.   However,

notification  of  such potential problems seems  appropriate

when  running this type of utility. This message is  only  a

warning  and  normal operation of FATMAN resumes  after  the

prompt is cleared.

" File is marked as read-only and may not be deleted. "

The  user attempted to delete a file marked in the directory

as read-only.

"  File  is marked as a Hidden or System file. Are you  sure  you

want to delete it? (Press ESC to cancel deletion)"

FATMAN  will let hidden or system files be deleted but  only

after this warning.

" Memory Error #1 "

" Memory Error #2 "

" Memory Error #3 "

Once  FATMAN  is running and the display has been  setup  by

FATMAN,  there  are various points where  small  amounts  of

memory  are  requested of the system. In the  very  unlikely

event  that  there  is not enough memory available,  one  of

these messages would appear.  These memory allocations occur

only  after  FATMAN has successfully allocated all  of  it's

main buffers (e.g. FAT buffers, directory buffers etc.)

Other Poop on FATman:

HIDDEN CAPABILITIES OF FATman  6.00

Confidential Internal FATman Information

By Gale R. Horst, FATman Author

Zeroing entire sector:

Page 19

Zeroing entire cluster:

When editing logical sectors or clusters many times we want

to zero everything.  Rather than typing 0 512 or 4,096 times:

1.   Get ready to edit the sector or cluster as you normally

would by:  Select Display/Edit Logical Sector or

List/Edit in Hex and ASCII.  Press F8 to go into edit

mode

2.    Now press F2 to move the edit cursor over to the ASCII

side of the screen (if it's not already there).

3.   Press Alt-S to zero the entire sector.  In List/Edit in

Hex  and  ASCII you may also use Alt-C to zero  an  entire

cluster.

Note  that  the  zeros are not written to the  disk  until  you

press  F8  again  to write the edited data  back  to  disk  and

terminate  edit mode.  After pressing Alt-S or  Alt-C  you  may

still  press  ESC to change your mind and restore the  data  to

what you started with.

Editing physical sector(s) by H/T/S (Advanced Menu):

1)  You  may  edit any sector on the physical drive  using  the

selection  Display  by  Head/Sec/Cyl  under  of  Utilities  for

Advanced  Users.  After selection the H/S/C you want  to  edit,

press Ctrl-F8 then press Alt-F8.  Now you are in the same  edit

mode  as  when editing logical sectors.  Note that  this  is  a

hidden  feature  intended  for our internal  use  only.   Every

other  feature and hidden feature of FATman limit the  user  to

editing   (or  in  some  cases,  damaging)  a  single   logical

partition.   Using  this hidden H/T/S feature  the  user  (you)

will  be able to damage any part of the physical disk from  the

master   boot   record  up  through  the  bad   sector   table.

Therefore, use with caution!

2)  When  selecting the H/T/S note that the minimum and maximum

values  accepted  are  displayed for  each  H/T/C  entry.   The

maximum   Cylinder  number  displayed  is  the  maximum   user-

cylinder.   This  entry will accept up to the maximum  physical

cylinder  number.   Therefore, you  may  examine  the  reserved

cylinders with this feature.

Hidden features of Display/Edit Directory:

There  are  very few (if any) of our end users who  would  ever

have  any  reason to edit the file size.  Allowing our  general

users  this  ability  would be asking for unnecessary  problems

and  therefore this is not allowed.  However, if you do have  a

valid  need  to edit the file size (or are over-confident  that

you  know  how and have reason to do this), select Display/Edit

Directory  from the main menu, move the select  bar  your  file

and  press ENTER to go into directory edit mode.  Now move  the

edit  select  bar to the year entry of file date by  using  the

right  arrow  on the cursor key pad.  (Pressing the  -->  again

would wrap around back to the file name again)  Now press  Ctrl

and  the  right arrow key to position the edit cursor  on  file

size  and  allow  it  to  be edited.   The  file  size  in  the

directory will be updated when you press ENTER.

Page 20

To  determine  (and/or edit) the first cluster pointer  in  the

directory  entry  (a  good way to mess up your  disk  a  create

cross-links if you don't know what you are doing ... and  why),

position  the  cursor  on the file size (as  described  above).

Then,  press  Ctrl  and  --> again to  display/edit  the  first

cluster  number  for this file.  The modified starting  cluster

number  will be written to the directory when you press  ENTER.

USE  WITH CAUTION!  Make sure you know what (and why)  you  are

doing this.

Display FAT Chain:

When  using  this feature each cluster is analyzed  to  display

whether   it   is   a  binary  or  ASCII/test  cluster.    This

information is displayed for each cluster as the link-chain  is

built  and  displayed.  When looking at a large  file  you  may

want  to  speed  up this display an NOT check and  display  the

cluster   data-type  analysis.   Once  the  first   screen   is

displayed,  press the letter 'D' to go to the  next  screen  of

information an it will display much faster (although data  type

will not be shown).

Useful Hints for FATman:

Sometimes  in  our testing there is a need to either  create  a

very  large  file or use up all of the disk in DOS files.   You

may easily do this with FATMAN as follows:

1.   Delete a junk file (one you don't care about)

2.    Edit  the file size (as described above) and make  the

file   size   of  the  deleted  file  very  large   (e.g.

999999999).

3.   Select Undelete File(s) from main menu.

4.    Select Cluster Sequence Priority from undelete options

menu.

5.   Now select your erased junk file for recovery.

6.    After  using all the available clusters for your  file

(assuming you edited the file size to need more  clusters

than  are  available) you will see a large error  message

stating  that  not  enough erased  clusters  were  found.

Press  any  key (except ESC) to tell FATMAN to  go  ahead

and  create  the  file anyway.  You will  see  the  error

message  at  the  bottom of the screen saying  that  your

file did NOT undelete correctly since it was not able  to

find as much space as your file needed.  However, it  did

create  a file containing all the unused clusters and  it

also  changed  the  file  size to  match  the  number  of

clusters that it actually used.

To examine the "raw" directory data for a subdirectory:

1.   Change to the parent directory of the directory you

want to examine.  (Press C from the main menu)

2.    Press  H  (for  List/Edit file in Hex and  ASCII)  and

select  your subdirectory.  This will display  the  "raw"

subdirectory  data clusters as a file.   Note  that  with

Page 21

FATman  6.0  you may examine the data as a  directory  by

using F2 View Menu feature.

FATman

Copyright 1997 Sound Expressions

LIMITED SOFTWARE LICENSE AGREEMENT

You  must  read the following terms and conditions  before  using

FATman (SOFTWARE).  By using this SOFTWARE, you acknowledge  that

you  have  read, understood, and accpted this agreement  in  it's

entirety.   If you do NOT to accept all the following  terms  and

conditions,  you  must  remove the SOFTWARE  from  your  computer

system and destroy any copies of the SOFTWARE.

This  SOFTWARE  is  a  proprietary product and  is  protected  by

copyright  laws. It is  licensed (not sold) for use on  a  single

machine, and is licensed only on the condition that you agree  to

the terms of this LIMITED SOFTWARE LICENSE AGREEMENT.

Sound  Expressions  (hereinafter  termed  THE  AUTHOR)  does  NOT

warrant  that the functions contained in the SOFTWARE  will  meet

your  requirements or that the operation of the SOFTWARE will  be

uninterrupted or error free.

LICENSE

1. USE. You and only you may use the SOFTWARE on a single machine

(a single CPU) at any given time.

2. COPY. You may NOT copy this SOFTWARE.

3. TRANSFER.  You may NOT transfer this license to another party.

4. OTHER RESTRICTIONS. You may not reverse engineer, decompile or

disassemble  the  SOFTWARE. Any modification of the  SOFTWARE  is

prohibited.

TERM

You  are hereby granted a limited license for a period of  thirty

(30)  days  (hereinafter termed EVALUATION PERIOD).  The  license

term may be extended by your written request at the option of THE

AUTHOR.

Use   of   TMONEY  beyond  your  EVALUATION  PERIOD  is  strictly

prohibited.

Distribution of any printed output generated by the  SOFTWARE  to

any   third  party  during  the  EVALUATION  PERIOD  is  strictly

prohibited.

LIMITED WARRANTY

The  software is provided "AS IS" without warranty of  any  kind,

either  expressed or implied, including, but not limited  to  the

implied   warranties  of  merchantability  and  fitness   for   a

Page 22

particular  purpose.  The  entire risk  as  to  the  quality  and

performance  of  the  software is with you. Should  the  software

prove defective, you (and not THE AUTHOR) assume the entire  cost

of all necessary correction and legal damages.

In  no event will THE AUTHOR be liable for any damages, including

any   lost   profits,  lost  savings  or  other   incidental   or

consequential damages arising out of the use or inability to  use

the SOFTWARE or for any claim by any other party.

Nothing  in  this LICENSE AGREEMENT constitutes a waiver  of  THE

AUTHOR's rights under the U. S. copyright laws or any other law.

You acknowledge that you have read this agreement, understand it,

and agree to be bound by its terms and conditions.

--- Ordering, site licenses, and quantity discounts ---

Licensing fees (all fees are in U.S. dollars):

Single user / single CPU license. . . . . . . . . . . . $20.00

This is a single user LICENSE. The user is licensed to use

FATMAN on a single machine at any given time. The user

must make certain that there is no chance of their copy of

FATMAN being used by anyone else. If more than one person

has access to the machine on which FATMAN has been

installed, the multiple users license must be requested.

In the case of a user who will ONLY be using FATMAN at home

for personal (non-business or professional) reasons, the

single user license definition will be extended to include

members of the immediate family or household.

Under the single user license, printed output (other than

the FATMAN order form) generated by FATMAN may NOT be given

to a third party.

Quantity discount / site license. . . . . . . percentage based

Purchases of multiple FATMAN licenses are based on the

following discounts:

# licenses        discount

----------        --------

2 - 10         10%

11 - 25        15%

26 - 100       25%

101 - 500      35%

501 - ?        45%

Page 23

-------------------------------------------------------

Application for software license:  FATMAN  Version 6.0

-------------------------------------------------------

Name:_______________________________________________________

Address:_____________________________________________________

City,ST,Zip:___________________________________________________

Phone:______________________________  Date:____________________

E-Mail Address:_______________________________________________

If this is a company, give the name of the contact person:

Name:______________________________ Dept:_____________________

Make checks payable to:

Sound Expressions

5361 Browntown Rd

Sawyer, Michigan  49125

License type applied for:

Single-User/Single CPU ______  ($20 each)

Number of copies: _______          Price each: $_____________

Total list price: $________________

Quantity discount: _____%     (see quantity discount chart)

Final Purchase Price: $______________

What disk format do you use?    3 1/2" _____     5 1/4" _____

Where or how did you learn about FATMAN? :_________________

_________________________________________________________

_________________________________________________________

Comments & Suggestions: ___________________________________

_________________________________________________________

_________________________________________________________