MOZART.SYS, Mozart Device Driver -  Copyright (c) 1995 Robert Muchsel

Release 007

Initializes Mozart Sound cards for OS/2

---------------------------------------

Shareware: DM 20.-/US$ 12.50 (see end of this file).

This OS/2 device driver initializes your Mozart sound card so that

the OS/2 Soundblaster driver can be loaded. CD-ROM and Game Ports

can be enabled.

Installation

============

The Mozart driver loads before the MMOS/2 driver and configures

the Mozart sound card.

Copy MOZART.SYS to your \OS2 or \ direcory on the OS/2 boot drive.

If the driver finds a Mozart card and initialization is successful,

you'll hear a cuckoo (else an error message is displayed).

(NB: Registered users can turn of the cuckoo).

Some noise and hissing before the cuckoo is normal.

Command Line Arguments

----------------------

The default configuration is:

-        Version 2 Sound Card

/A:220   I/O Address 220

/D:1     DMA channel 1

/I:5     Interrupt 5

-        CD-ROM disabled

-        Game Port disabled

The driver understands several command line arguments:

BASEDEV=MOZART.SYS [/1] [/A:Addr] [/D:DMA] [/G] [/I:Int]

[/E:x or /M or /P or /S [/CA:CDAddr] [/CI:CDInt]]

where:

/1  Initialize v1 Sound Card (default: v2)

/A: I/O Address: 220 (default), 240

/D: DMA Channel: 1 (default), 3

/G  Game Port enabled (default: disabled)

/I: Interrupt:   5 (default), 7, 11

CD-ROM Interface (default: disabled):

/E: ATAPI CD-ROM: 1 (=primary), 2 (=secondary)

/M  Mitsumi CD-ROM

/P  Panasonic CD-ROM

/S  Sony CD-ROM

/Cx CD-ROM Parameter, where:

/CA: I/O Address: 320, 330, 340 (default)

/CI: Interrupt:   9, 10, 11 (default: none)

Examples:

BASEDEV=MOZART.SYS /A:220 /I:5 /D:1 /S /CA:320 /CI:11 /G

BASEDEV=MOZART.SYS /A:240 /I:11 /D:1 /M /CA:340 /CI:10

BASEDEV=MOZART.SYS /G

BASEDEV=MOZART.SYS /1 /G

BASEDEV=MOZART.SYS /E:2 /G

Configuring OS/2 - CONFIG.SYS:

------------------------------

BASEDEV=MOZART.SYS /G

REM Following lines added by OS/2 installation:

DEVICE=x:\MMOS2\SBP2D2.SYS 1 1 5 220 4 /N:SBAUD1$

DEVICE=x:\MMOS2\AUDIOVDD.SYS SBAUD1$

Configuring OS/2 - AUTOEXEC.BAT:

--------------------------------

SET BLASTER=A220 I5 D1

SET SOUND=x:\MOZART

Notes:

* You must __NOT__ load the DOS device driver MZTINIT.SYS under OS/2

(well, you can, but you have been warned). This also applies to

MZTMODE and MZTVOL.

* If the MMPM/2 driver won't load ("Soundblaster not found"), try

another version of the MMPM/2 driver (e.g. from TBOS2.ZIP).

* MIDI doesn't seem to work properly on OPL4 cards (this is a

problem of the OS/2 Soundblaster driver).

* If OS/2 traps when loading the ATAPI CDROM driver, get a fixed

version of the ATAPI driver from IBM.

* Under WinOS/2, you must use the Soundblaster drivers provided by

OS/2. Mozart specific drivers ans programs will not work.

* OS/2 Warp: When losing sound in seamless WinOS/2 sessions, install

the fix from the SNDWORKS.ZIP package by Michael K. Schumell.

* MOZART.SYS loads before all other BASEDEVs which don't have a

.SYS extension. If your CD-ROM driver has a .SYS extension, ensure

MOZART.SYS comes first in CONFIG.SYS.

* There are different versions of the Mozart sound card. The driver

defaults to v2 cards (OTI-601E, Mozart OPL4, BTC Mozart A-16), but

can be configured to support v1 cards (Raysound, Mozart OPL3,

Sonicsound).

The suggested method is to try the driver without '/1' first.

* OEM/Custom versions available! I'm substituting the cuckoo with

any other sound (e.g. advertisment) for anybody ordering more than

20 licences. Call for techincal details.

New in Rel. 007

===============

CD-ROM Interrupt now works in /1 mode.

New in Rel. 006

===============

Registered users can turn off the cuckoo.

New in Rel. 005

===============

MOZART.SYS now supports ATAPI CDROM drives.

-> New switches /E:1 and /E:2.

New in Rel. 004

===============

MOZART.SYS should now support all Mozart based sound cards available.

-> Try the /1 switch if your card is not found.

New in Rel. 003

===============

MOZART.SYS is now a BASEDEV driver.

-> The CD-ROM interface will now be initialized before the CD-ROM

driver is loaded.

New in Rel. 002

===============

Command line arguments are now supported.

-> Game Port can be enabled

-> CD-ROM interface can be enabled

-> Interrupt setting can be modified

Licence, Registering

====================

Developing this driver has taken a considerable amount of time. I'm

sure you'll honor my work, so there are no restrictions in the

shareware version!

If you register, you will be able to turn off the cuckoo.

You may try this program for 30 days. If you  want to continue to use

it, you must register. If you don't like the driver, simply DEL it.

!! Warning: If you are using shareware without registering, you'll

!! certainly descend into Hell some day. Even if you don't bother,

!! please register.

Please use the registration form supplied.

Optionally, you can order a disk with the driver.

Price (subject to change w/o notice):

Country     Registration   (optional) Diskette

----------------------------------------------

Europe      DM 20.-        DM 5.-

or US$ 12.50      US$ 3.20

World       DM 20.-        DM 11.-

or US$ 12.50      US$ 6.50

DM=Deutsche Mark        more than 5: please call!

The Author

----------

Robert Muchsel, Hegaublick 2, D-78465 Konstanz, Germany

Fax:    07533-3151 (intl. +49-7533-3151)

e-Mail: rmuchsel@iiic.ethz.ch (until end of 1996)

Fidonet 2:246/8100.11

I'm studying computer science at the Swiss Federal Institute of

Technology, Zurich.

You might know some of my other shareware tools:

AMIsetup - External Setup for AMI-BIOS

HPFSDOS  - HPFS Driver for DOS (Read only)

CD800X   - Audio Support for Apple CD-150/SC-CD under OS/2

EFDD     - Use up to 20 diskette drives under DOS

Important

---------

The author disclaims all warranties as to this software, whether

express or implied, including without limitation any implied

warranties of merchantability, fitness for a particular purpose,

functionality, data integrity or protection, in so far as permitted by

applicable legislation.

All trademarks are property of their respective owners and appear for

identification purposes only.

You are encouraged to copy and pass on this program. However,

- the program must be distributed in its original form

(complete, unmodified)

- the distribution must not exceed the equivalent of DM 15.-

(US$ 8.50).

11 Feb 1995