S E R I A L     P O R T S     M A N A G E R     v 2.23

------------------------------------------------------

S.T.A.R.COMM     Serial Turbo Async. Resident COMMs

------------------------------------------------------

09/18/96

��������������(C)opyRight HETRU 1991-1995�������������

13, chemin de la croix St Vincent

94430 CHENNEVIERES SUR MARNE

FRANCE

tel: (16-1) 45-93-27-38

CompuServe address: 100414,1524

Internet: 100414.1524@Compuserve.com

U S E R    G U I D E

REMARK: In all this document, STARCOMM.EXE could be replaced by

STARCOM8.EXE. STARCOMM.EXE is interfacing no more than 4 ports, whereas

STARCOM8.EXE could support up to 8 ports ! (All the values limits

specified in the following text are for STARCOM8: we refer to "COM8:"

or to "8" ports or buffers. Just change these values to "COM4": or "4"

if you consider STARCOMM !)

TABLE OF CONTENTS

Introduction........................................................ 1

I- Functional features of STARCOMM.................................. 2

II- Respective advantages between "BIOS" and "DEVICE DRIVER" installations

1- Used in the BIOS (T.S.R.) mode............................... 4

2- Used in the DEVICE DRIVER mode............................... 4

3- When using the library (SCLIB.OBJ)........................... 4

III- STARCOMM.EXE COMMAND SYNTAX

1- To load the driver in its T.S.R. BIOS mode................... 5

2- To load the driver in its DOS DEVICE DRIVER mode............. 5

3- Commands and options known by STARCOMM.EXE................... 6

IV- USING SPECIFIC(S) DEVICE(S) ON SERIAL PORT(S)

1- Using a mouse in 'competition' with STARCOMM................. 13

2- Accessing to an internal or external modem................... 13

3- All type of devices knowing a protocol....................... 13

V- SETPORT.COM UTILITY

1- Syntax of SETPORT.COM........................................ 14

2- Command options of SETPORT.COM............................... 14

ANNEX A:  Principle of the null-modem cable......................... 17

ANNEX B:  Types of UART chips on PC and compatibles................. 19

ANNEX C:  Why a "BIOS emulation" ?.................................. 20

ANNEX D:  Files making SERIAL PORTS MANAGER v2.23................... 21

1

INTRODUCTION

This document exposes the main features of the serial driver, and the

different ways STARCOMM.EXE can be used.

This driver was developed in 8086 assembler language on IBM-PC and works

under MS-PC/DOS and all compatibles operating systems (DR-DOS, DOS "task-

windows" under WINDOWS and OS/2, etc.).

The SETPORT.COM utility allows to modify the mode of working and the

initialization of STARCOMM by nominative port, as well as visualizing the

states of all the serial ports recognized by this driver (when found

resident in memory...). It also presents the advantage, over the

command-line interfacing offered by STARCOMM.EXE, that it doesn't

reactivate the default values of the different driver parameters.

The TESTSTAR.EXE demonstration (source *.PAS) permits to visualize

the efficient working of STARCOMM, and permits to test locals serial links

(TESTSTAR is a multi-ports processus).

The TERMINAL.COM program (developed in C) demonstrates the utilization

of STARCOMM when loaded in its DEVICE DRIVER mode. It creates a communica-

-tion active on one channel at a time (it is a simple TTY emulator...).

The REMOTE.EXE utility allows you to exploit a second IBM computer

from a computer called "local". The "local" IBM then acts like the normal

console of the distant IBM (called the "REMOTE" !). It is a small

application gracefully offered with SERIAL PORTS MANAGER in order to give

an example of what this package can do for you.

The "EXISTENCE" of a port answers to the double following condition:

the address that it wants to use correspond to a physically present UART,

AND the IRQ it is assigned to could be installed (ATTENTION: if an other

program comes to reroute this IRQ after the installation of STARCOMM, the

port will be un-usefull to establish any serial link to the means of

STARCOMM whereas the port will always remain "EXISTING", and this until

restitution of our vector by this other application...).

A port is said "OPENED" when the electric logical signals of the PC

are set in order to activate the interrupts that are physically going

to allow a completely functional link. Otherwise, this port will be

said "CLOSED" !

2

I  - Functional features of STARCOMM:

�������������������������������������

� STARCOMM.EXE could be loaded from the DOS prompt (like any standard

program) or from the CONFIG.SYS (DEVICE=STARCOMM.EXE...).

� In order to get an information page and a brief help, use the "?"

character (the "/" character is not compulsory):

x> STARCOMM /?

� ErrorLevel codes returned by the installation module in BIOS mode:

0 --> No error: requested action correctly done,

1 --> End of program on impossibility to un-install,

2 --> End of program on an un-acceptation to install itself.

� This driver manages, under hardware interrupt(s), the serial ports COM1:

to COM8: of compatible PC/AT in using buffers in RAM memory. There are

1 receipt buffer AND 1 transmit buffer for each port recognized at the

installation time (except orders making a more explicit limitation in the

command-line). The receipt buffer is 1 kilo-byte in size, whereas the

transmit buffer is reduced to 1/2 kilo-byte by default. It is possible to

fix each buffer size between � and 55 kilo-bytes. It is also possible,

for each application, to define its own buffers of any sizes in the limit

of ONE segment (of the CONVENTIONAL MEMORY) for each serial port used:

every acknowledged port can thus dispose of up to 64 Kilo-byte of I/O

buffers if needed !

� STARCOMM manages asynchronous communication ports in exploiting the

availables UART on the IBM. These UART could be a 8250, 16450 or 16550

type. If an 16550 UART is available on a given serial port, its FIFO mode

is automatically exploited (excepted if forbidden from the command-line !).

� The Request_To_Send(RTS)/Clear_To_Send(CTS) and Data_Terminal_Ready

(DTR)/Data_Set_Ready(DSR) signals can be managed by the STARCOMM communi-

cation processes, as well as RI, DCD and OUT1 ! The software hand-shaking

Xon/Xoff is also acknowledged, and can be activated and dis activated, as

can be RTS/CTS and DTR/DSR. This can be ordered from the DOS prompt, or

using the software interrupt giving access to the serial driver functions

(Int 14h). The hexa. codes to use for Xon and Xoff can be define as needed.

� All group of 10 consecutives received errors provokes a complete

initialization of the controller associated to the serial port used, this to

avoid any possible interference with any other programs reaching the port

in the local machine. To this effect also, the BIOS software interrupt 14h

is filtered by STARCOMM (case of the programs using the serial ports via the

BIOS: in this case,they provoke a communication format resetting to the level

of our driver). As a consequence, for example, this implies that any

initialization made using the DOS "MODE" utility will be acknowledged by the

STARCOMM driver...

Remarks:     1- Any initialization request on an inhibited port (Cf. III-3)

will be rejected by our filtering of the BIOS interrupt 14h.

2- The BIOS variables addressing the UART chips are up-dated by

the driver. They are reseted in their initial state at the time of the

STARCOMM uninstallation.

3- The time-out BIOS variables are not, in any case, altered

by STARCOMM.

3

� STARCOMM filters the basics interrupt 14H BIOS functions: it redirects

to the driver any commands sent to an open serial port. From then on, it

is the addresses and all the technical features of STARCOMM that are stakes

to disposition of ALL the DOS comm. applications based on the BIOS level.

These applications will therefore have their performances distinctly

improved, and they will be able to gain the mapping benefit offered by

STARCOMM.EXE (see "/A..." and "/I..." options...).

Example: suppose D_COMM.EXE only uses the COM1 (via the BIOS), and your

modem is installed as a COM3:, using the IRQ 7.

execute:

x> STARCOMM /A1=03E8 /I1=7

x> SETPORT COM1: /O /Inform

...and then:

x> D_COMM              And that's all !...

REM: Thanks to the "/EB" option that activate the STARCOMM BIOS emulator,

it could be avoided having to open the port to use (Cf. ANNEX C) !

However, no buffer will be used by the driver in this last case.

� ATTENTION: After the installation and/or after any parameter(s)

modification from the driver command-line, the ports are reseted in their

UNACTIVE state (except using option "/O..."). Any application wanting to use

port(s) in order to communicate will therefore have to initially take

carefulness of reopening the appropriate(s) port(s)...

� STARCOMM can generate 2 kinds of transmitting: a slow transmission (18

bytes by seconds), and a fast transmission (continuous flow only limited by

the capacity of the UART to transcode the bytes). On 8 simultaneous opened

ports, each one could dispose of its own transmitting mode. The choice of

the mode to activate only depends on the device with which it is necessary

to converse. For example, to make a direct local link with a Minitel, it IS

necessary to activate the slow transmission because this terminal doesn't

presumably work in an interrupt way: if too much bytes arrives between 2

Minitel scans, there is overrun, and therefore loss of information to the

Minitel level. In general, for all device working in a scanning mode, it is

necessary to activate the slow transmission of STARCOMM (thus creating an

everage transmission of 18 bytes a second...).

� IMPORTANT REMARK ON HARDWARE INTERRUPTS SHARING:

Only the specials multi-serial ports boards and the IBM-PS/2 computers

permits several serial ports to share a same IRQ. If you joined several

ports on a same IRQ with any other material (PC, XT or AT), know that you

must NOT open at the same time all these ports. It will then be necessary to

always open only ONE of these ports at a time for it to be fully working !

Please report to the "/MC=..." option of SETPORT.COM to know how to use

such a material with non-MCA local bus by using SERIAL PORTS MANAGER...

(! THIS IS A HARDWARE LIMITATION NOT SURPASSABLE BY ANY SOFTWARE !)

4

II- Respective advantages BETWEEN "BIOS" and "DEVICE DRIVER" installations

��������������������������������������������������������������������������

1- Used in the BIOS (T.S.R.) mode:

����������������������������������

� Uses less RAM memory (the DOS normalized interface doesn't remain

in RAM memory after the driver installation !).

� Ability to un-install the driver when becoming useless in RAM, without

having to fully reset the computer;

� Complete relative independence to the operating system, so that:

. considerable increase on execution speed of the services offered

by the driver: indeed, the DOS layer being absent in the case of an

Int 14h, every logical access to the driver corresponds to an unique

physical call to this driver (where a DOS interface function could

easily achieve it by at least 4 DOS internals calls !),

. and absence of any trouble on EOF code (^Z) management that is merely

unknown to the interrupt 14h.

2- Used in the DEVICE DRIVER mode:

����������������������������������

The driver is here-by known by the DOS. It opens the access to a new

logical device (referred as "COMM:" by default) to which it is possible

to access with no more programming using the DOS shell (COPY CON: COMM:)

OR, with programming, using any computer language (use the DOS standard

files access functions integrated to all programming languages !).

3- When using the library (SCLIB.OBJ):

��������������������������������������

This serial port driver format is not usable from the DOS prompt.

It is only usable by the programers in order to integrate the kernel

of SERIAL PORTS MANAGER directly to their creations,this to avoid any de-

-pendence between their programs and any external driver if he wants to !

The application thus developed will only benefit the BIOS mode, with

the advantages associated (report to part 1-...) PLUS the advantage to

be able to call the driver services without transitting through the

interrupt 14h (Cf. STAR-REF.DOC on this...).

The ability, for each application, to define its own external buffers

for each serial port negates the fact that the library only integrates

2 very small buffers for each port by default (1/2 kilo-byte each...).

5

III- STARCOMM.EXE COMMAND SYNTAX

��������������������������������

STARCOMM.EXE can be loaded as an external driver to the DOS (in its

"BIOS" mode) OR as a DEVICE DRIVER (thus creating a "character" type file

named "COMM:" by default).

The SETPILOT.EXE utility can help you to get in touch with the

numerous options that STARCOMM.EXE knows !

1- To load the driver in its T.S.R. BIOS mode:

����������������������������������������������

From the DOS command line (the "prompt"...), just order:

d>[<drive>:][<\path\>]STARCOMM [COM:][speed][,parity][,bits][/<options>]

The only software interrupt to access the STARCOMM services will

then be the interruption number 14h.

2- To load the driver in its DOS DEVICE DRIVER mode:

����������������������������������������������������

2 differents ways are usable to install the DEVICE interface. They can

be used at the same time:

A/  Integrate the following order in your CONFIG.SYS:

DEVICE=[<drive>:][<\path\>] STARCOMM

[COMi:][speed][,parity][,bits][/<options>]

In addition to the interrupt 14h, STARCOMM will thus be also accessible

by using the DOS interrupt 21h FILES functions; and you will even have

access to the "COMM:" file at any moment from the DOS prompt.

B/       DEVICE=[<drive>:][<\path\>]COM0X.SYS [COMi:]

or     DEVICE=[<drive>:][<\path\>]COM0X.SYS [COM0i:]

also allow to interface, in DEVICE DRIVER mode, a complete (i.e. using

DEVICE=STARCOMM.EXE) or partial (i.e. with the BIOS mode) installation of

the driver, BUT to give access to THE ONLY port specified with the command,

without any possible change...: COM01: won't give access to any other port

than the number 1, COM02: to the number 2, etc...

COM01: is always used by default. It is impossible to load more than once

this driver for a same port. If the serial driver is NOT accessible through

the int 14h a "General failure" error is generated: REMEMBER to load the

serial ports driver using a BATCH or from the DOS command line, to make

usable the COM0X.SYS loaded from the CONFIG.SYS when you are using this

DEVICE DRIVER with a partial installation of the driver !

The advantage for this loading is to allow ONE DOS acces for EACH serial

port supported,without having to commute the used port each time it becomes

necessary. It also allow to directly redirects data flow between serial

ports from the very DOS prompt...

To use this release of the DEVICE DRIVER interface from a personnal

programming, you may just use the DOS functions documented in STAR_REF.DOC,

changing the default DEVICE name "COMM" ("COM01", or "COM2", etc...).

Please NOTE that you HAVE TO give the port number associated to the driver

used only (0 for "COM01:", 1 for "COM02:", etc...) with the "IO-Ctrl

write" function OR you'll get an error AND the requested job won't be made:

once more, each installed COM0X.SYS is only able to interface ONE serial

port...the one SPECIFIED as it was INSTALLED !

6

3- Commands and options known by STARCOMM.EXE:

����������������������������������������������

The initialization format explained below takes effect for

the whole acknowledged serial ports of the driver.

[COM:]    = Announces the definition of the format initializations.

When STARCOMM has been (or is) loaded as a

DEVICE DRIVER, specifying COM1: (or COM2:, etc...) allow

to orientate the logical file ("COMM:" by default) on the

serial port thus specified.

[speed]   = {110,150,300,600,1200,2400,4800,9600,19200,28800,38400,

57600, or 115200} bits a second (9600 by default).

[,parity] = {N, P, I, T, or R} for 'N' no, 'P' even, 'I' odd,

'T' work, or 'R' rest (N by default).

[,bits]   = {5,6,7 or 8} for a coding on 5,6,7 or 8 bits (8 default).

[,stop]   = {1 or 2} for 1 or 2 stop-bit(s) (1 by default).

The working-control options recognized by STARCOMM are:

[/A<N>=<address>[,<N>=<address>[,...]] =

Use <address> (in hexa.) for the port <N>, then... For

each specification, give the address of the first associate

UART register for the indicated port number. At this

address, if no UART is detected, the associate port will

not be functional and treated as INEXISTANT by the driver.

Every specified address must be made of 4 numbers: write

/A3=0370 and not /A3=370 !...

Note that in permuting some addresses between different

existing ports, you could redirect the standard DOS ports

(attention to the IRQ linkage !...).

ATTENTION: This option is unknown for the ports of which

the IRQ seems to be rerouted by a program loaded after

STARCOMM.EXE (in order to respect the consistency of the

IRQ chain in the computer !).

[/I<N>=<num>[,...]] = Use the <I>RQ number <num> for COM<N>:.

Only the IRQ 2, 3, 4, 5, 7, 10, 11, 12 and 15 are admitted.

Any indicating sequence can be used with no importance. The

erroneous or forbidden indications will be unknown. If a

program installed after STARCOMM.EXE has rerouted some

interrupts used by STARCOMM, it won't be possible to modify

these IRQ for the associated port(s), and this until

un-installation of that post-loaded program. The Order is

then unknown (no apparition of new IRQ number in the driver

message !). In case of repetitions for a same port, only

the last specification will be considered as consistant.

REM: Don't use IRQ2 on AT and PS/2, this IRQ being

reserved to a second 8259 (interrupt controller)

installed in cascade on these machines ! Don't use

IRQ5 on the XT machines (providing hard disks). At the

time of the installation (as well as at the time of

any parameters modifications), a demand for IRQ2

on an AT or a PS/2 or a demand for IRQ5 on an XT will

be rejected. The IRQ10 and above could not be accepted

on AT or PS/2 equivalents !

7

[/G<num>[,<num>...]] = I<G>nore the serial ports <num>. Permit

the cohabitation of STARCOMM with other drivers working on

serial port(s) too (we can explicitly dedicate them one

or several ports: report to the paragraph IV about

this topic...). This option also permits to manage the

conflictual problem of sharing hardware interrupts on the

materials that don't support these sharings (ex: if you

have a COM2: and a COM4: both linked to the IRQ3 on an AT,

forbid either before executing TESTSTAR.EXE; because this

program opens all the marked present ports, it would result

of it, in the contrary case, a conflict that would block

either port COM2: or COM4:, in truth the two !...).

Forbidding the access to all the ports won't forbid to load

STARCOMM.EXE; but there won't be any possible communication

in this case (except utilization of the function 0Ah of the

services interrupt 14h...) !

ATTENTION: This option is never USEFUL when used to

modify an already RAM-installed STARCOMM setup whereas it

tries to act on a port of which the interrupt seems to be

redirected by an other application than STARCOMM ! Allowing

such a maneuver would set the logical existence statutes of

the serial ports unconsistant with the real state of the

machine's vectors diverted by the asynchronous ports

driver; and then, some declared "EXISTING" ports could

not be functional. Besides, it would become impossible to

uninstall STARCOMM.EXE, even after all competitor

applications (to the level of the used interrupts...) have

been themselves uninstalled !

[/V<n�>[,<n�>,...]] = Locks the speed for the specified comm ports.

Any new speed would be substituted by the locked one !

[/F<n�>[,<n�>,...]] = Locks the comm <F>ormat. Same as "/S"...

[/EL<num>[,<num>...]] = s<E>nd s<L>ow on the port <num>. Permit

to activate the transmission in its reduced flow for the

specified ports. It will be the clock interrupt (and not

the UART one) that will order them to send out some bytes

contained in the transmit buffers of the specified ports.

It will therefore be 18 bytes a second (maximum...) that

will be transmitted toward the outside.

[/O<num>[,<num>...]] = <O>pen the serial ports <num> as soon as

the installation is finished. Using the device attached to

"COM<num>:" is then possible as soon as the driver is

installed.

[/EB]       <E>mulator <B>IOS permit not to redirect the calls of the

interrupt functions 1 to 5 toward the original BIOS int.

14h. A BIOS emulation code is then activated. It presents

the advantage of using the addresses, the time-out values,

and the respect of the hardware hand-shaking signals that

are all imposed to the STARCOMM driver. This code executes

its inputs/outputs directly on the physical specified

port (buffers are not used...).

8

Each of the following options take value for all the ports known as

existant by the installed driver. In order to act only on one port, it is

necessary to act in using the software interrupt of the driver interface

(this results of the will to preserve the driver installation process

from being too complex; use SETPORT.COM in order to control the ports

separately from one another...).

[/PC]     = Knows the used machine as a <PC>. The IRQ2, 3, 4, 5 and 7

will be all authorized for utilization. This command is to

reserve for the old computers (BIOS of 1985 and before

generally) or for those that declares of erroneous way

(compatible PC to the BIOS little compatible (!), or

advanced machine "under-configured": ex. an XT without

hard disk --> its IRQ5 is therefore free !...).

[/XT]     = Knows the used machine as an <XT>. The IRQ2, 3, 4 and 7

will be the only IRQ authorized.

[/AT]     = Knows the used machine as an <AT>. The IRQ3, 4, 5, 7, 10,

11, 12 and 15 will be the only authorized.

[/PF]     = Leave <P>orts <F>IFOs un-activated on the found 16550.

This option surpasses the following one.

[/F=<Nb. of bytes>] = Number of bytes of the 16550 UART buffers to use

at the transmit process level. "<Nb. of bytes>" must be

included between 2 and 15, otherwise the option is merely

unknown, and the default value of 14 is then used.

REMARKS: Indeed,16 bytes do constitute this physical buffer

of the 16550 UART. STARCOMM only allows a maximum of 15

because it seems that some kinds of these UART loses

some informations if the software tries to benefit from the

transmit buffer in its entirety !

In receipt,the FIFO is always set to activate the receive

interrupt as soon as 4 bytes are hold in the UART buffer

(the "trigger level" is frozen to 4 by STARCOMM). This

allow to avoid as many overrun errors as possible, unless

it gives a little more work to the CPU, when the commu-

-nications flows are heavy !

[/AI]     = <A>utoscan <I>RQ... The driver automatically research on

which IRQ lines are installed the serial ports found at the

specified addresses. The "/I=..." option has priority on

this "/AI" option that can reveal itself not reliable on

some hardware (particular modems OR very fast computers...)

NEVER USE OPTION "/AI" UNDER WINDOWS OR THE OPERATING

SYSTEM WILL CRASH ! ! ! .

[/S[=<hexa_code>]] = Activate the <S>ubstitution process of the bytes

specified as deficient upon receipt by the specified code.

This code could be specified in the room of term

<hexa_code>. The default value used is 0FEh.

/S=NULL (or "/S=N" because only the first letter of

"Null" is really necessary !) permit to order the

voluntary loss of any erroneous byte received.

[/K[=<code_hexa>]] = Activate the function of representing, in the

receive buffers, the received BREAKs. <code_hexa> is the

code that will represents the BREAKs. /K without any more

specification will force the use of 03h to represent a

BREAK in the buffer. /K=Null stops this function.

9

[/<nb>] = Permits to limit the number of buffers pairs (transmit/

receive) that the driver could open in RAM. By default,

STARCOMM reserves 1 pair of buffers by port physically

acknowledged during the installation process.

Attention: limiting the number of buffers pairs also

limits the number of serial ports that it will be possible

to open at the same time ! <nb> is limited between 1 (at

least 1 buffer to create !) and 8 (i.e. the biggest

possible number of usable ports). This option could only

be acknowledged by the installation process of STARCOMM.

Otherwise, it is ignored, and the correspondent message

displayed will only be a recall of the number of buffers

the driver knows it can create in RAM.

REMARK: In order to save room in memory, it is better

to load "STARCOMM/1" when it is going to be used with

mono-link applications (some emulators generally...),

whereas several serial ports exists on the used machine.

[/BR=<size>] = Specification of the memory size to allocate to the

<R>eceipt <B>uffer(s). <size> must be included between

� and 55 Ko., and won't be acknowledged but at the time

of the first installation of driver in memory. Thereafter,

the driver will satisfy of recalling the buffers used

due to the initial installation. In order to modify

the size of the receive buffers, uninstall then install

again the driver with a new <size> value !

[/BE=<size>] = Specification of the memory size to allocate to the

s<E>nd <B>uffer(s). The notes on the reach of this option

are identicals to those of the option above. NOTE that if

the added sizes of all the buffers to create passes the re-

maining free room in the driver segment, the sizes of these

buffers will be automatically reduced in order to contain

the buffers in the limit of this segment ! The sizes

finally restrained will always be a multiple of 1024 bytes.

[/R=<tps>] = Definition of the time limit to the tip of which the

impossibility of <R>eceiving must abort on a time-out

error... The <tps> parameter, that must be included

between 1 and 255, explicit the maximum number of half a

second to wait before generation of a receiving time-out.

[/E=<tps>] = Definition of the time limit to the tip of which the

impossibility of s<E>nding must abort on a time-out

error... The <tps> parameter, that must be included

between 1 and 255, explicit the maximum number of half a

second to wait before generation of a transmitting

time-out.

[/LC]     = "<L>ocal <C>lear_to_send": The local site must recognize

the RTS/CTS hand-shaking: it checks CTS=TRUE before any

send, but it sets RTS in order to control the distant site

that is supposed deaf to this signal.

[/EC]     = "r<E>mote <C>lear_to_send": The distant site is supposed to

check its CTS signal before its TRANSMITTING, this to

control the speed of its transmit flow upon the order of

the local site; however, the local site must not interpret

the local CTS before its own TRANSMITTING. The local site

sets RTS in order to up-date the CTS signal of the distant

site.

10

[/BC]     = Use the RTS/CTS hardware hand-shaking in bilateral.

(i.e. "<B>ilateral <C>lear_to_send")

[/LD]     = "<L>ocal <D>ata_terminal_ready": The local site must

recognize the DTR/DSR hand-shaking: it checks DSR=TRUE

before any send, but it sets DTR in order to control the

distant site that is supposed deaf to this signal.

[/ED]     = "r<E>mote <D>ata_terminal_ready": The distant site is

supposed to check its DSR signal before TRANSMITTING,

this to control the speed of its transmit flow upon the

order of local site; however, the local site must not

interpret the local DSR before its own TRANSMITTING. The

local site sets DTR in order to up-date the DSR signal

of the distant site.

[/BD]     = Use the DTR/DSR hardware hand-shaking in bilateral.

(i.e. "<B>ilateral <D>ata_set_ready").

[/RI]     = The local site must check that RI=TRUE before TRANSMITTING.

REMARK: Check the remark below !

[/CD]     = The local site must check that DCD=TRUE before TRANSMITTING.

REMARK: In fact, it goes of it precisely in the same way

for the options [/LC] (with the CTS signal) and [/LD]

(with DSR) that are explained,higher, in the context of the

normalized protocols RTS/CTS and DTR/DSR. (There is no

obligation in the act that local CTS is hardback to remote

RTS; or that local DSR has connected to distant DTR !...).

[/OT]     = The local site set OUT1=FALSE as soon as it is not able

to receive anymore data. Otherwise, it sets this signal

to TRUE.

Analogous REMARK to the previous for [/EC] (RTS signal)

and [/ED] (DTR signal).

[/LX]     = "<L>ocal <X>on/Xoff": The local site knows the Xon/Xoff

hand-shaking. But no Xon or Xoff code will be transmitted

toward the distant site, supposed deaf to this protocol.

[/EX]     = "r<E>mote <X>on/Xoff": The distant site is supposed to

recognize Xon/Xoff to manage its transmissions on order

of the local site; however, the local site doesn't

interpret the Xon and Xoff codes that it receives, but

must manage them like any other data.

[/BX]     = Use the software Xon/Xoff hand-shaking in bilateral.

(i.e. "<B>ilateral <X>on/Xoff").

[/X=[<Xon>][,< Xoff>]] = Use the hexa. values specified for

the Xon codes (before the comma) and Xoff (after the

comma). The values kept by default are 11h (for Xon) and

13h (for Xoff). Every hexa. value specified must be

made of 2 numbers included between the values 0 and F !

The following option doesn't have any sense except when STARCOMM is used

as a DEVICE DRIVER:

[/ND=xxxxxxxx] = Logical <N>ame of the <D>evice. Permits to assign a

different name than "COMM:" to the unit created by STARCOMM

when it is loaded from the CONFIG.SYS. It is possible to

define this name with this option at any moment using

STARCOMM.EXE from the DOS prompt, under the simple

condition that STARCOMM resides in RAM as a DOS DEVICE

DRIVER. Only the characters 'A'..'Z' and '0.'..'9' are

admitted in order to constitute the logical name of

STARCOMM used as a DEVICE DRIVER ! This name could be

constituted of a number of any active characters included

between 2 and 8.

11

The following option is special to a specific type of multi-comm boards:

[/AP<adress>=<val>] = The <val> byte (hexa.) is sent to the <adress>

adresse (hexa.). This permit to activate more than 2 serial

ports on specifics multi-ports asynchronous I/O boards.

<adress> must be 4 bytes length, whereas <val> must be 2...

For example:  /AP03FF=80 is OK,   /AP3FF,80 is NOT !

This option may be used more than once (always with the

same adress or with differents adresses each time...).

ATTENTION: There is no check on the neutral and not dange-

-rous type of accessing the specified I/O adress...

Only use this option IF YOU KNOW WHAT YOU ARE DOING ! ...

Please report to the "/MC=..." option of SETPORT.COM for

using multi-comm board with non-MCA CPU bus systems.

The 2 following options surpass and nullify all the others options;

on the other hand, they are not executable by STARCOMM when it is used

as a DEVICE DRIVER:

[/D]      = <D>esactivate AND UNINSTALL the driver from the RAM memory.

If a program executed after STARCOMM rerouted at least one

of the STARCOMM used interrupts (anyone, hard OR soft...)

this option won't be treated. A message of impossibility

will appear, and it will reacts the same if STARCOMM

is used as a DEVICE DRIVER, or if one (or several)

program(s) forbade STARCOMM to leave the memory.

[/?]      = Demand a display of the helping and informations page.

This page will be displayed only if STARCOMM is executed

from the DOS prompt.

Finally, the [/PM] (No <M>essage) option permits to forbid the writing

of the initializations control messages that finally took place. This

option is especially usefull when we want to avoid the proliferation of the

messages at the boot time (loading of STARCOMM from the CONFIG.SYS or the

AUTOEXEC.BAT) ! Only the essential messages of success to install or of

impossibility to install (or to uninstall) could appear despite this "/PM"

option. To the contrary, the [/PA[=<length>]] option permits to force

a <PA>use in order to allow the reading of the driver installation

report. (<length> admitted from 0 to 60 seconds, 0 for an infinite

expectation of hitting a key. 0 is the default used value...).

NOTE that, even though STARCOMM is loaded as a DEVICE DRIVER, it

remains possible to execute STARCOMM.EXE at any time from the DOS prompt

in order to command somme new initializations or new working modes

to the driver installed in memory. All the options, "/D" excluded,

are perfectly usable in this case.

All these options could be enumerated in an any order.

It is possible to modify the status of the driver thanks to these

commands even though it is already installed in RAM memory.

The spaces introduced in a command-line don't have any particular

meaning to the "eyes" of the commands sensor inserted in the serial

driver. They will always be unknown: you can therefore use spaces at your

own discretion in order to give your line a more explicit meaning

to YOUR eyes.

A request for an helping page provokes the interruption of the program

immediately after the information is displayed: it will be necessary to

execute again the driver with the needed options (and, this time, without

the "/?" option) in order to get the appropriate requested installation.

12

If some contradictory requests are enumerated,only the last valid request

of the concerned option will be restraint. Besides, it is always the last

valid declared option (in a given type) that will be served and not its

previous apparitions in the command-line. For example, "/I1=5,2=3 /I2=7"

will provoke the utilization of IRQ5 for the COM1: AND of IRQ7 for the

COM2: !... Exception: the command options of the hand-shaking controls

activity are cumulatives. For example, "/LX/EX" does have the same effect

than "/BX".

The values kept by default are, for each option:

Addresses--> COM1: 03F8 COM2: 02F8 COM3: 03E8 COM4: 02E8

COM5: 01A8 COM6: 01E8 COM7: 01F8 COM8: 02A8 on PC, XT,

AT--> COM1: 03F8 COM2: 02F8 COM3: 3220 COM4: 3228

COM5: 4220 COM6: 4228 COM7: 5220 COM8: 5228 on PS/ 2

IRQ--> COM1: 4 COM2: 3 COM3: 4 COM4: 3

COM5: 3 COM6: 3 COM7: 3 COM8: 3

on PC, XT, AT

except COM3: 3 (instead of 4) on a PS/ 2

All the ports are defined as "standard" (i.e. NOT multi-comm ports !)

Speed/Format            --> 9600 bds, N81, fully UN-locked...

Ports to be unaware of  --> Not any one.

BIOS Emulator           --> Inactive.

Utilization of the FIFO --> YES with 14 bytes.

Substitution process    --> Inactives (Smode0 status; BREAK unknown).

Receipt buffer          --> 1 kilo-byte.

Transmit buffer         --> 1/2 kilo-byte.

Time-out in receipt     --> 10 seconds.

Time-out in transmit    --> 10 seconds.

Hand-shakings activity  --> All inactive (hard and software).

Codes Xon and Xoff      --> Xon=11h and Xoff=13h.

Name of DEVICE          --> COMM:

Physical serial port accessed by COMM:  --> COM1:

STARCOMM.EXE will never accept to use a same physical adress for 2 or

more internals logicals ports (COM1:, COM2:, etc...) !

When an option provided to STARCOMM.EXE is not correct, it uses the

default value of the parameter associated to the deficient control, but

it doesn't write any message of specific mistake. Just by checking all the

inits messages (except utilization of "/PM" !) you can assure yourself

of your command(s) success...

STARCOMM.EXE systematically disactivates the "user interrupt" functions

on all the serial ports, as well as the "error management function" of the

interrupt 14h when it is executed. The BIOS emulator is always DISACTIVATED

too by default.

13

IV- USING SPECIFIC(S) DEVICE(S) ON SERIAL PORT(S)

�������������������������������������������������

1- Using a mouse in 'competition' with STARCOMM:

������������������������������������������������

� Install the serial driver with the parameters you really need,

enumerated in addition to the option "/G<i>" (where 'i' gives the number

of the port to reserve for the mouse).

� Install MOUSE.COM on the serial port your mouse is attached to.

� In case of requiring, you could use STARCOMM.EXE all over again in

order to modify the installation parameters of this driver, but ALWAYS

INCLUDING the "/G<i>" option in your command-line !

Indications: This is due to the dynamic management of the environment

practiced by the driver. In the present case of modifications of the

inits, whereas an other driver set intervenes (the one of the mouse),

it makes necessary to protect the inits associated to this other

driver. This is made by forbidding to our driver the access to

the port(s) dedicated to the other(s) driver(s). In order to

interrupt the utilization of the mouse and recover its port to the

level of our driver, it is sufficient to execute STARCOMM.EXE all

over again, SUPPRESSING the "/G<i>" option.

ATTENTION: The most recents mouse drivers MUST be uninstalled because

they are able to recover their IRQ and inits. dynamically !...

2- Accessing to an internal or external modem:

����������������������������������������������

The STARCOMM driver of SERIAL PORTS MANAGER permits to reach any modem

plugged in any of the serial ports numbered from COM1: to COM8:.

It is sufficient, for that, to normally install the driver in memory.

REM: In order to send some specific commands to the modem connected on

the serial port of your choice, uses the function 1Ah of STARCOMM.

(i.e., in Pascal and in C, the 'WriteCmde' procedure).

ATTENTION: When this function 1Ah uses the DTR signal in order to

indicate the modem that it is going to receive a control, and not a

sequence of bytes to send on the line, it is necessary that the modem

receives (before BEING connected) an order indicating not to interpret

the DTR change to 0 as a connect aborting request ! (For the

Compatible Hayes modem, the command is AT&D1...). The alternative

consists in using the function 1Ah with BH=0 (=> DTR no used), and in

including the prefix of associated command to the modem ("+" by default

for the Compatible Hayes modems). Report to MODEM.DOC and its examples

(in PASCAL and C) to this topic.

3- All type of devices knowing a protocol:

������������������������������������������

It goes of it precisely like for the modems.

Question: Particular case as examples ?

Answer: The MINITEL with its TeleTex commands,

the serial printers, ...

and a lot of other materials (...from laboratories generally !)

14

V- SETPORT.COM UTILITY

�����������������������

The STARCOMM installation module doesn't permit a selective

initialization between the various serial ports acknowledged by the driver.

The reach is global to all the ports supported by the driver... SETPORT

gives this selective interfacing. Further more, the only functional

parameters of STARCOMM that SETPORT modifies are those explicitly (and

correctly) specified in the command-line ! There is not any systematic ini-

-tialization of all the working variables to their default values like the

STARCOMM installation/modification module does. Finally, in case of

contradictory options, it is always the last stated option that carries it

away, except for the hand-shaking command options that are cumulatives

(/LX/EX will have the same effect than /BX for example...).

SETPORT.COM uses the interfacing functions of the driver (interrupt 14h)

in order to communicate with it and thus to modify its working modes and

status. This permits, therefore, to demonstrate the good working of these

service functions of the STARCOMM.EXE serial port driver.

1- Syntax of SETPORT.COM:

�������������������������

A NOT EMPTY command-line is COMPULSORY. The "/?" command option

recalls an helping message on the available options, and interrupts the

utility (all other controls associated to "/?" are ignored):

d>[<drive>:][<\path\>]SETPORT [<parameter(COM1)>][<parameter(COMi)>...][/I]

where [COMi:][,<speed>][,<parity>][,<bits>][,<stop>] [/<options>]

constitutes every group [<parameter(COMi)>] (any space forbidden !).

If STARCOMM is NOT found available in memory, SETPORT.COM protests by an

error message and immediately stops.

If STARCOMM is available and that SETPORT received no command to

execute, it presents some lines explaining the utility of this program.

IF all the commands given to SETPORT are unvalid, there will be a

specific error message; but if at least one command is correct, the

erroneous command(s) will be merely unknown(s) while the correct(s)

command(s) will be served...

2- Command options of SETPORT.COM:

����������������������������������

The 8 following options have a global reach because they don't concern

a port in particular:

[/Inform] (or [/I]) = display an array on the status of the serial

ports acknowledged as "existing" (Cf. the introduction of

this document). Only the first letter of word "Inform"

is sufficient for that option to be acknowledged (ex:

"/Infos" will be also admitted !). The written array allows

you to check the good acceptance of your commands.

[/8]     = Display the infos. on the 8 ports.

[/PC| XT| AT] = Force the recognition of the material architecture.

(Cf. same options made more explicit for STARCOMM.EXE).

[/EB]    = Activate the <E>mulator of the original comm. <B>ios.

(report to the explanation of the option "/EB" in page 7

of present document...).

15

[/PEB]   = <P>lease, set <E>mulation of original Comm. <B>ios OFF...

(that is: desactivation of the BIOS emulator !)

[/COMM=x]= Makes to point the logical port created by STARCOMM (when

it is loaded as a DEVICE DRIVER) on the specified serial port

(/COMM=2 will make to use the COM2: by the DEVICE...).

The following options don't make any sense but when they constitute a

group clearly identified (a group concerns 1 port identified by "COMi":,

and it doesn't contain ANY space !):

[COMi:]  = Announces the definition of the format initializations for the

serial port number i (i from 1 to 8 in order to designate

from COM1: to COM8:).

[speed]  = {110,150,300,600,1200,2400,4800,9600,19200,28800,38400,57600,

or 115200} bits by second (9600 by default).

[,parity]= {N, P, I, T, or R} for 'N' no, 'P' even, 'I' odd, 'T' work,

or 'R' rest (N by default).

[,bits]  = {5,6,7 or 8} for a coding on 5, 6, 7 or 8 bits (8 by default).

[,stop]  = {1 or 2} for 1 or 2 stop-bit(s) (1 by default).

[/L-]    = Where "-" worth from 0 to 3: L0 makes a complet unlocking,

L1 sets the speed locking, L2 the format locking, and L3

locks the speed AND the format for the specified port...

Notice that SETPORT.COM (like STARCOMM.EXE) doesn't

take care of these locking: it may change any speed

and format for any port at any time !

[/A=xxxx]= Definite a new address (HEXA) to associate to the port.

[/I=x]   = Definite a new IRQ to use (2 to 5, 7, 10 to 12, or 15).

[/AI]    = <A>utomatic <I>RQ research of IRQ associated to the ports.

[/MC=aaaa,mm,l]= <M>ulti-<C>omm allows to associate a serial port to

a board supporting several asynchronous ports and that

permits the sharing of an interrupt line. Such a board owns

an IT-Rq status register (at the aaaa Hexa. adress) telling

what UART, from the board, are waiting to be  serviced. It

works using a positive logic (l is worth 1) OR a negative

logic (l is worth 0). To every port is associated a UNIQUE

bit of this register, this permitting to attribute a mask

of hexa. value "mm". This mask must be computed regarding

the logic of the board: in positive logic (l=1), set to 1

the ONLY bit that represents the port being defined; in

negative logic, put this ONLY bit to 0. Give aaaa (in

hexadecimal) using 4 numbers, mm in hexa. 2 numbers, and 0

OR 1 for the l parameter (l is brought back to 1 other-

-wise...). The "/MC=0000" command permit to desassociate

a port from a multi-comm. board.

STARCOMM is NOT able to manage more than 3 specials multi-

comm. boards that are based upon the process that corres-

-ponds to this option. (Some multi-comm. boards are made

for the ISA bus and don't have to be managed to the means

of the mechanism activated by the "/MC..." option).

Every multi-comm. board used is supposed be identified by

the address of its IT-Rq status register: Therefore USE a

UNIQUE address for each map !... It is UN-USEFULL to acti-

-vate this option for any multi-comm. board installed on a

MCA bus type (ex: PS/2 !), as well as for any board's port

NOT sharing the same interrupt line !

All of the /A=, /I= and /AI options do disactivate the

"multi-comm." mode that may be associated to the port they

are seting.

16

ATTENTION: DANGEROUS option ! Declaring anything not

physically real on your hardware could drive the I/O

interrupt of the driver to an infinite loop: NO check

could be led on the values of the given parameters !

ONLY USE THIS OPTION KNOWING WHAT YOU ARE DOING !

[/G]     = Forbade the access to the port: it then becomes INEXISTANT.

This option could not be executed if the IRQ associated

to the concerned port was captured by a program executed

after STARCOMM.

[/PG]    = Permits (if possible) the access to the port that becomes

existing again (same remark than for "/G" on the IRQ).

[/PF]    = <P>ort <F>IFO: if the port's UART is a 16550, it will

be used in its 8250 emulation mode. Useful for some (old)

PS/2 that uses some buggies 16550-A !

[/F=xx]  = Size of FIFO: 0, or 2 to 15 bytes. "/F=0" is equivalent

to "/PF" (see previous...).

ATTENTION: The options /A /L /AL /PF and /F closes the port by

authority !

[/EL]    = Activate the s<L>ow transmits.

[/ER]    = Activate the <R>apid (fast) s<E>nds (transmits).

[/PS]    = No <S>ubstitutions of the bytes received with an error

signalled on their transmission.

[/S]     = Activate the <S>ubstitution of the bytes received with

an error by the code 0FEh (the '�' character...).

[/S=Null]= <S>ubstitutions by the empties: all erroneous byte in

receipt is not placed in the receipt buffer.

[/S=xx]  = <S>ubstitutions by the HEXA code xx.

[/K=xx]  = For each Brea<K> received, make the xx HEXA code appearing

in the receipt buffer. /K=Null stops this function. /K

(alone) will activate it with the standard value of 03h...

[/R=xxx] = Time-out <R>ecept after xxx half a second.

[/E=xxx] = Time-out s<E>nd after xxx half a second.

[/NC]    = Stop the RTS/<C>TS protocol.

[/LC]    = Activate the <L>ocal hand-shaking by RTS/<C>TS.

[/EC]    = Activate the r<E>mote hand-shaking by RTS/<C>TS.

[/BC]    = Activate RTS/<C>TS in <B>ilateral working.

[/ND]    = Stop the DTR/<D>SR protocol.

[/LD]    = Activate the <L>ocal hand-shaking by DTR/<D>SR.

[/ED]    = Activate the r<E>mote hand-shaking by DTR/<D>SR.

[/BD]    = Activate DTR/<D>SR in <B>ilateral working.

[/NRI] [/RI] = Stop/activation of hand-shaking based on RI.

[/NCD] [/CD] = Stop/activation of hand-shaking based on DCD.

[/NOT] [/OT] = Stop/activation of hand-shaking based on OUT1.

[/NX]    = Stop <X>on/Xoff protocol.

[/LX]    = Activate the <L>ocal hand-shaking by <X>on/Xoff.

[/EX]    = Activate the r<E>mote hand-shaking by <X>one/Xoff.

[/BC]    = Activate <X>on/Xoff in <B>ilat�ral working.

[/X=[xx],[yy]]= Definite the HEXA codes to use for Xon ([xx])

and Xoff ([yy]).

[/O]     = <O>pen the port.

[/F]     = Close the port.

EXAMPLE: C:> SETPORT COM1:110,p,7,2/s=Null/El/o  COM3:/BX/R=2/AI  /Info

17

ANNEX A:  Principle of the null-modem cable

The null-modem cable to use in order to exploit the SERIAL PORTS MANAGER

driver as well as the associates demos., must be made following

the minimum installation below:

(3) Rx (receipt)�������Į���������(transmit) Tx (2)

(2) Tx (transmit)��������į��������(receipt) Rx (3)

(7) Gnd (ground)�������������������(ground) Gnd (7)

The following installation allows to neutralize the RTS/CTS and DTR/DSR

signals (allows to verify the neutrality of the hard hand-shaking protocols

implemented in STARCOMM.EXE):

(3) Rx (receipt)�������Į���������(transmit) Tx (2)

(2) Tx (transmit)��������į��������(receipt) Rx (3)

(7) Gnd (ground)�������������������(ground) Gnd (7)

(4) RTS�Ŀ                               �ĮRTS (4)

(5) CTS���                               �įCTS (5)

(20) DTR�Ŀ                               �ĮDTR (20)

(6) DSR���                               �įDSR (6)

In all rigor, the complete scheme for the null-modem cable is:

(1) Chassis ��������������������������� Chassis (1)

(3) Rx (receipt)��������Į��������(transmit) Tx (2)

(2) Tx (transmit)�������į���������(receipt) Rx (3)

(7) Gnd (ground)�������������������(ground) Gnd (7)

(4) RTS ����������������į����������������� CTS (5)

(5) CTS ����������������Į����������������� RTS (4)

(20) DTR �����į���������Ŀ  ������Į������� DTR (20)

(6) DSR ĮĿ             ���������������į� DSR (6)

(8) DCD Į�������������������          �į� DCD (8)

(22) RI ������������������������������������� RI (22)

18

There is, between the number of the DB25 binds (concerned by the above

diagrams) and the number of the DB9 bind, the following corresponding:

Taken DB9                   Taken DB25    Signal

3----------------------------2   <-->   Tx

2----------------------------3   <-->   Rx

5----------------------------7   <-->   Gnd

7----------------------------4   <-->   RTS

8----------------------------5   <-->   CTS

6----------------------------6   <-->   DSR

4----------------------------20  <-->   DTR

1----------------------------8   <-->   DCD

9----------------------------22  <-->   RI

!!!!!!!!!!!!!!!!!!  T A K E    C A R E   ---> Users... !!!!!!!!!!!!!!!!!

!!!    All other type of NULL MODEM CABLE is susceptible of          !!!

!!! provoking some stops of communication between the programs       !!!

!!! using the serial driver STARCOMM.EXE v2.0 and above. The risk    !!!

!!! entirely rest on the nature of the used links (by the cable)     !!!

!!! between the communication control signals...                     !!!

!!! If your cable is not suitable, do not use the options to activate!!!

!!! any of the hand-shaking protocols RTS/CTS, DTR/DSR, RI, etc...   !!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!! Developers... !!!!!!!!!!!!!!!!!!!!!!!!!!

!!!    You could define some particular hard hand-shaking            !!!

!!! (following the developed instructions in STAR_REF.DOC ANNEX...). !!!

!!! Using its personalized management of modem signals, your program !!!

!!! won't be able to work without its cable to the particular        !!!

!!! pattern: you will have earned a good hardware KEY protection for !!!

!!! your program...                                                  !!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

19

ANNEX B:  Types of UART chips on PC and compatibles

This appendix present a fast summary on the types of UART, products by

INTEL C� (tm) and compatible with the 8250, that was successively

implemented on the range of IBM computers and compatibles.

8250 ......... First UART used on the first IBM PC. It is a slow

controller: the accesses to the registers are faster

than the treatments done by the 8250 ! It disposes of

7 registers and is "sold" in order to accept bauds until

56 Kilo-bauds. The receipt and transmit buffer are

both made of only one byte.

8250A ........ It is a correction of 8250, with 1 register in addition:

the scratchPad, that allows to mark the presence of

the UART controller.

16450 ........ It is an improved version of 8250A: its internal

treatments are fast enough in order to avoid some losses

of time at the time of software accesses to the

controller regiters. It doesn't permit however, in

theory, to support more than a 56 kilo-bauds speed of

transmission.

16C451 ....... It is the CMOS version of 16450 (i.e. 16450 with low

electric energy needs).

16550 ........ First version of a new improved 16450: it should permit

the activation of an internal buffer of 16 bytes,

and the support of 256 kilo-bauds. A mistake in

realization made it inoperative. Only some first

PS/2 received this controller (quickly retired from

INTEL C� !). It must NOT be used as a 16450 (i.e.

forbid the activation of its FIFO mode using the /PF

option of STARCOMM.EXE !).

16550A/AF/AFN. It is the first functional version of 16550 ! Its 16

bytes buffers do works and it support up to 256 Kilo-Bds.

16C551 ....... CMOS version of 16550AF.

16C552 ....... Implement 2 16C551 in the same electronic box.

82510 ........ It is a 16450 supplied with buffers of 4 bytes. It is

used in some notepad.

Then.......... It decorated how certain(s) competitor(s) of the INTEL

business is attacking the production of 16550AFN type

controllers, based on the utilization of 32 bytes in

its internals buffers (this since mis-1990...).

To follow !...

To summarize, retain 3 generations in the evolution of this UART

controller essentially (Universal Asynchronous Receiver & Transmiter)

8250A ........ Simple but slow UART,

16450 ........ Simple and fast UART,

16550AF ...... Fast UART, with 16 bytes buffers and reliable !

20

ANNEX C:  Why a "BIOS emulation" ?

Why offering an option to emulate the BIOS interrupt 14h whereas

STARCOMM.EXE seem already be (by nature) such an emulator ? Because the

functions 1 (send) and 2 (receive) of the INT 14h filtered by STARCOMM

could benefit from contributions of this driver (16550 UART FIFO buffers

excepted) only with the OPENED ports of the driver. Indeed, STARCOMM

redirects the calls to these functions for the closed serial ports on

the original BIOS interrupt (it was a logical choice at the time of the

initial program development !...).

In order to benefit from the possible redistribution of some serial

ports addresses, from the access to the ports 1 up to 8, from the time-out

definitions, and from hardware (input/output) hand-shakings, even on

STARCOMM CLOSED ports, it is therefore necessary to activate a special

part of the driver code: its BIOS emulator of the original BIOS

communication functions.

Of course, the I/O serial ports buffers won't be used when accessing

a closed port THROUGH the BIOS emulator (it is necessary and sufficient

to OPEN the ports in order to fully benefit from STARCOMM contributions !)

But this emulation permits to dispose of an improved BIOS (from the today

available UART point of view) as soon as STARCOMM.EXE is installed on

your machine with its BIOS emulator activated...

This improved BIOS also guaranties a security on the used addresses in

order to reach to the UART (it is not anymore the BIOS addresses specified

by [40:00] to [40:06] that is used, but the driver addresses, well

"hidden" in RAM !). It finally permits to enhance all PC compatible BIOS

with the function 05h that doesn't normally exist, except on the PS/2...

21

ANNEX D:  Files making SERIAL PORTS MANAGER v2.23

INFORME  DOC...General informations on the product origin.

SPECIFS  DOC...General technical facts on the pack.

REGISTER DOC...To register SERIAL PORTS MANAGER, send me your

appreciations, or report some noted bugs.

UPDATES  DOC...What's new in this last release of S.P.M. ?...

STARGUID DOC...The user guide for SERIAL PORTS MANAGER v2.23

STAR_REF DOC...The reference manual for programming by using

STARCOM8.EXE, STARCOMM.EXE or SCLIB.OBJ.

SETPILOT  EXE..Small utility to install and setup STARCOMM.EXE

STARCOMM EXE...Serial driver:it is the kernel of SERIAL PORTS MANAGER !

STARCOM8 EXE...Kernel of SERIAL PORTS MANAGER adapted to the management

of 8 serial ports (STARCOMM can only drive up to

4 serial ports...).

SETPORT  COM...An utility developed (using C) in order to offer you

a way to have a 'port by port' control on the installed

driver in memory (whereas it works in its BIOS or

DEVICE DRIVER mode).

COM0X    SYS...The "DEVICE DRIVER" part of the driver itself. It is

adaptated to a multiple loading so that each supported

port may have its OWN DOS based DEVICE interface...

STARINTF PAS...Interface in order to use the driver services from

the (C)Turbo Pascal.

STARINTF C.....Interface in order to use the driver services from

the (C)Turbo C.

SCLIB    OBJ..."Library" release of STARCOM8.EXE (8 ports).

DEMO_LIB ASM...A simple demo. of how to use SCLIB.OBJ.

TESTSTAR PAS...A demo. DEVELOPED in Turbo PASCAL to use STARCOMM

in a multi-communication process.

Require the procedures of STARINTF.PAS

TESTSTAR EXE...Executable file of this demonstration !

TERMINAL C.....A simple TTY terminal based on the use of STARCOMM

loaded as a DEVICE DRIVER: we reache the port by

the means of the logical file (called "COMM:").

Require the procedures of STARINTF.C

TERMINAL COM...Executable file of this demonstration !

REMOTE   EXE...A small application allowing you to exploit a remote

IBM PC/AT from the local computer. We could name it

a "DOS terminal emulator"...

MODEM    DOC...Document containing informations on the procedures

made to access modems, procedures contained in

MODEM.PAS and MODEM.C.

MODEM    PAS...PASCAL procedures to access HAYES modems.

MODEM    C.....C procedures to access HAYES modems.

TESTMODP PAS...Source of the demo. to use MODEM.PAS.

TESTMODC C.....Source of the demo. to use MODEM.C.

CONVERT  DOC...Document for the procedures made to convert data.

CONVERTA ASM...Simple procedures to make ASCII-transcoding: they

CONVERTP PAS...permit to transfer any binary file even when using

CONVERTC C.....the Xon/Xoff software hand-shaking...