The SuperVGA Kit

----------------

Version 5.0

22 August 1994

The SuperVGA Kit is a set of routines for working with SuperVGA

video cards that have a VESA VBE compliant Video BIOS. This library was

an offshoot from the development of the Universal VESA VBE, which is an

installable TSR to provide VESA VBE video BIOS extensions for most

SuperVGA cards. It is intended to show how to program the SuperVGA

cards through the VESA VBE and uses some of the more powerful features

that the latest standard include in your own software.

This library supports many of the extended features of today's SuperVGA

adapters, such as being able to separately set the read and write banks

and support for extended page flipping. This is all done via the

standard VESA VBE programming interface.

NOTE: All source code is formatted with 4 space tabs, including the

assembly language source.

NOTE: This library has '386 specific code in it, so it will _not_ work

with machines without a '286 or lower processor. Some of the code

such as the screen clearing code etc uses fast 32 bit string

instructions to run at full speed on new VESA VLB Local Bus

video cards.

Using it in your own programs

-----------------------------

The SuperVGA test programs all call upon a single C library to access the

SuperVGA using the VESA VBE programming interface. You can use this

same library to provide SuperVGA support in your own applications, or

you can take the source code for the library as a starting point and

expanding on it from there.

Assembling the source files

---------------------------

All of the assembler source files have to be assembled with Borland's TASM,

since they are written using TASM's IDEAL mode. If you do not have TASM,

you can simply link with the pre-assembled object files provided.

Test programs

-------------

The library now has only one single test program, that presents a set of

menus for testing all of the available video modes that the library

supports (and your hardware supports).

Note that the test program requires a VESA VBE compliant SuperVGA in

order to run, so if your video card does not have the VESA VBE in the BIOS,

then you will need to install the Universal VESA VBE before running it.

VBETest

-------

The SuperVGA Kit also includes the executable and source code for the

VBETEST.EXE program. This program is a stress test program for the VESA

VBE interface to check the BIOS of your video card (or the installed version

of UniVBE if you are using this) for conformance. It fully tests all the

standard and advanced functions of the VESA VBE 1.2 interface, and will

report any errors that it detects to the VBETEST.LOG file.

Unfortunately many video BIOS'es pass the conformance test, however the results

that are displayed visually on the screen (such as when doing wide virtual

screens and scrolling) are not correct. If this is the case then you can

consider that the BIOS has failed the conformance test. If you have a BIOS

that does not pass the test, you might like to contact your video card's

manufacturer and report the problems to them.

PM/Lite library

---------------

The SuperVGA Kit uses the PM/Lite library, which should also have come

as pre-compiled library files in the same archive. If you wish to compile

it up for a different extender, you will need to obtain the source code

archive (see the file INSTALL.ME for details).

Global Variables Reference

--------------------------

int     maxx,maxy;          Maximum device coordinate values (res-1)

long    maxcolor,defcolor;  Maximum and default color values

int     maxpage;            Maximum video page number (numpages-1)

int     bytesperline;       Bytes in a logical scanline

int     bytesperpixel;      Bytes in a pixel (if > 1)

bool    twobanks;           True if separate banks available

int     memory;             Video memory available in Kb

short   modeList[];         List of available video modes (USE THIS!)

char    OEMString[];        OEM string from VBE

The following two globals are only valid if the enableSpecialFeatures

flag is set to TRUE when initSuperVGA() is called.

bool    extendedflipping;   True if we have extended flipping

bool    widedac;            True if wide dac is supported

The following globals defint the pixel format information, and is used by the

rgbColor() routine to build the correct pixel for the current display mode.

You can however use it yourself to build scanline information in the desired

format if you need to optimise these routines for speed.

uchar   redMask,greenMask,blueMask;  Color channel mask values

int     redPos,greenPos,bluePos;     Starting position of color channel bits

int     redAdjust,greenAdjust;       Shift factors need to adjust to correct

int     blueAdjust;                     postition in pixel image

Function Reference

------------------

int initSuperVGA(bool enableSpecialFeatures);

Initialise the SuperVGA Kit. The enableSpecialFeatures flag is

used to enable the detection of the 8 bit wide RAMDAC and SuperVGA

page flipping. In order to detect this, a SuperVGA video mode must

be started, so the screen will be cleared. Leaving this flag

false will not change the current video mode.

Returns the packed BCD version number for the VBE interface

detected, or 0 if none is present. For VBE 1.2 this is 0x102, for

VBE 2.0 this will be 0x200.

bool setSuperVGAMode(int mode);

Set the specified video mode, given the internal VBE mode number.

DONT pass old style VBE mode numbers to this routine (ie: 0x101

for 640x480x256), although the will still work, the extensible

method is to search through the list of available video modes

for the one that has the desired resolution and color depth. This

will allow you code to work with all custom video resolutions

provided by different OEM VBE drivers (like UniVBE/Pro). Have a

look at the code in the HELLOVBE.C file that demonstrates how

to start any video mode given a user specified resolution.

The display page is always cleared to black when this routine is

called.

Returns FALSE is the mode could not be set correctly.

void restoreMode(void);

Restores the previous video mode active befor the setSuperVGAMode

routine was called. Also correctly restores the VGA 50 line mode

if it was previously active.

bool getSuperVGAModeInfo(int mode,int *xres,int *yres,int *bytesperline,

int *bitsperpixel,int *memmodel,int *maxpage,long *pagesize);

Returns the video mode information for the specified VBE internal

video mode number. The mode number must be valid, or this routine

will return FALSE. Check the modeList[] array which contains the

list of available video modes that you can use.

bool setSuperVGADisplayStart(int x,int y);

Sets the CRTC display starting address to the specified value. You

can use this routine to implement hardware scrolling (if you wish

to set up a wide virtual screen, you will need to call VBE function

07h yourself, as we dont currently have this function implemented

in the SuperVGA Kit).

To perform double buffered animation, use the setActivePage() and

setVisualPage() routines instead.

Returns FALSE if the value could not be set.

bool set8BitPalette(void);

This function enables the 8 bit wide palette if available and

returns TRUE. If an 8 bit wide palette is not available, this

routine returns FALSE and the palette width is unchanged.

bool set6BitPalette(void);

This function restores the default VGA style 6 bit palette.

void setPalette(int start, int num, palette *palbuf);

This function set the specified palette entries, by calling the

video BIOS. Eventually this will become a native VBE function

in VBE 2.0, and will eliminate all the problems associated with

snow.

The buffer is expected to be in the correct format, which will

be an array of 'palette' structures. If the palette is currently

in the default 6 bit VGA mode, this routine expects the values to

be 6 bits wide. If it is in an 8 bit palette mode, then the values

should be 8 bits wide (and hence you will need to do the

conversion yourself).

void getPalette(int start, int num, palette *palbuf);

This function reads the current value of the paletet and stores

it in the buffer.

long rgbColor(uchar r,uchar g,uchar b);

Packs a set of RGB values into a color value for passing to the

primitive drawing routines that is appropriate for the current

video mode. This routine is intended to work with RGB video modes

such as the 15, 16, 24 and 32 bits per pixel modes (in 8 bit modes

it will packed it into a simple 2:3:2 style pixel, so you can set

up your own pseudo RGB palette if you so desire).

You should use this routine to convert all color values to ensure

that they work correctly on the different types of direct color

video modes available.

void writeText(int x,int y,char *str,long color);

Writes the text sting at the location x,y in the standard VGA

8x16 VGA font, and the specified color. The background between

the text is not erased.

void setActivePage(int page);

Sets the currently active video page for output. Used to implement

SuperVGA double buffering for smooth animation.

void setVisualPage(int page);

Sets the currently visible display page. Used to implement SuperVGA

double buffering for smooth animation.

void setBank(int bank);

Set the current read/write bank values to the same location.

void setReadBank(int bank);

Sets the read bank value to a different location. If you wish to

use separate read/write banks, you will need to first set the

read/write banks to the write bank location with the setBank()

routine above, then call this routine to change the read bank

to a different location.

void putPixel(int x,int y,long color);

Plots a pixel at the specified (x,y) location in the specified

color. The color value MUST be in the correct format for the

current video mode (use rgbColor() to pack HiColor and TrueColor

RGB values). The pixel is plotted on the currently active

display page (which may possibly be hidden from view).

void clear(long color);

Clears the currently active display page to the specified color.

The color value MUST be in the correct format for the current video

mode (use rgbColor() to pack HiColor and TrueColor RGB values).

void line(int x1,int y1,int x2,int y2,long color);

Draws a line from the point (x1,y1) to (x2,y2) in the specified

color. The color value MUST be in the correct format for the

current video mode (use rgbColor() to pack HiColor and TrueColor

RGB values). The line is drawn on the currently active

display page (which may possibly be hidden from view).

Where to get the latest version

-------------------------------

You can probably get the latest version of the package from Simtel20 or

garbo (and their mirrors):

oak.oakland.edu:    pub/msdos/graphics/svgakt??.zip

garbo.uwasa.fi:     pc/graphics/svgakt??.zip

If you have access to CompuServe, you can download the file from the

IBMPRO VESA file library (12) as the file svkt??.zip (CompuServe has

restrictive file naming conventions).

Corrections, updates and queries

--------------------------------

If you have any corrections or updates to the code in this package, or

you have any questions you would like asked, feel free to contact me

through one of the following methods.

Internet:       rcskb@minyos.xx.rmit.edu.au

Compuserve:     100237,2213

Fax:            +61 3 690 2137

Snail Mail:     Kendall Bennett

SciTech Software Australia

Unit 5, 106 Southbank Boulevard

South Melbourne 3205

AUSTRALIA