Note:

If you are upgrading from 1.1.3 or older versions you will need to

perform the 1.1.4 modification before using this release. Contact

TAPR, P.O. Box 12925, Tucson AZ, 85732.  (602) 323-1710

***********************************************************************

November 7, 1988

TNC-2 Software Release 1.1.6 Notes

----------------------------------

This  document  describes  the  differences  between  the  latest

release  1.1.6 and the previous (1.1.5) release of TAPR  software

for the TNC 2 Terminal Node Controller.

The reported checksum on powerup is $2E.

This release include "KISS" capability from K3MC.

NEW COMMANDS

------------

KISS ON|OFF                                          default: OFF

This command is used to enter the "KISS" mode for operation  with

various  experimental protocols.  KISS is used  with  intelligent

host  computers.   The most popular use has been  in  conjunction

with the KA9Q TCP/IP networking software, available from TAPR and

elsewhere.

To  operate  KISS with this release, enter the command  KISS  ON,

then the command RESTART (NOT RESET!).  The CON and STA LEDs will

flash  three times when initially actiated.  This  command  will

then  have effect for subsequent power on/off cycles.  To  return

to normal operation, enter the command PARAM <device> 255 at the

NET>

prompt  if you are running the KA9Q TCP/IP software package.   If

you  cannot send this parameter with the software you  intend  to

use,  you  must  power the TNC off,  then  disconnect  the  bbRAM

battery for several minutes, then reconnect the battery and power

up again.

NOTE:     While  the  TNC  is  in the  "KISS"  mode,  normal  TNC

operation is suspended and the familiar command set  is

inoperative.

TXUIFRAM ON|OFF                                      default: OFF

This  command  deterines  whether the  TNC  will  transmit  most

unconnected information packets.

Setting  TXUIFRAMe  OFF  will  prevent  all  but  BEACON  and  ID

unconnected ("UI") frames from being originated and  transmitted.

This  is  most useful for BBS and other stations  which  tend  to

leave  unacknowledged  data  in the TNC transmit  buffer  when  a

connection is lost.  The normal behavior of a TNC would send  the

buffered data as "UI" frames, adding to channel congestion.

The  behavior  with  TXUIframe  OFF  serves  to  reduce   channel

congestion because the now useless data is never sent.

NOTE:     TXUIFRAM has NO effect on the digipeating of UI frames.

It  only  affects  frames  being  originated  at   this

station.

CHANGES

-------

o -  The  TNC  now requires that the Data  Carrier  Detect  (DCD)

signal  from  the  internal or external  modem  be  asserted

(true) for a packet to be received.  This change is intended

to  ensure  that a modem with a "threshold" control  is  set

properly.

The  result  is that the station operator will set  the  DCD

control  in order to receive data.  This then  ensures  that

the TNC will have the best chance of detecting other station

activity,  reducing collisions that would  otherwise  result

from  jumping  on a channel believed to be open when  it  is

really in use by another station.

This protection feature will be invisible to most operators.

NOTE:     To test with digital loopback, FULLDUP must be ON.  For

normal operation, FULLDUP should be set to OFF.

THE PACKET REVOLUTION REVOLVES!

Howard Goldstein N2WX

TNC-2 Software Design

'n

Damage Control