EED3 LAYOUT ASCII DATABASE FILE FORMAT

Layout ASCII Database records are explained in sequence in which they MUST

appear in the file.

The first parameter in each record is record type. Record type parameter

must ALWAYS be of fixed (3 characters) lenght.

I.  Layout Database Common Data

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Header      |

|

| 000,databasename,l,sunit,cunit,cref,Xmin,Ymin,Xmax,Ymax

|

| where:

|

|  000              record type

|

|  database name    1 to 8 characters of data base name

|

|  l                layout data base flag

|

|                   if L then EED III version up to 2.10/1.30

|                   if L3 then EED III version from 2.20/1.40

|                   if L2 then EED II database conversion file

|

|  sunit            if I then all widths/sizes/diameters in inches

|                   if M then all widths/sizes/diameters in mm

|

|  cunit            if I then all coordinates in inches

|                   if M then all coordinates in mm

|

|  Cref             if B then all coordinates as in layout database

|                   i.e x0,y0 in the center of board outline

|                   if R then all coordinates as displacement from

|                   Xmin,Ymin of board outline

|  Xmin,Ymin        Coordinates of Xmin,Ymin of board outline

|

|  Xmax,Ymax        Coordinates of Xmax,Ymax of board outline

|

| Example:

|

|  000,L,GPOST   ,I,I,B,-2.500,-2.500,2.500,2.500

|

-------------------------------------------------------------------------------

Via hole    |

size info   | 001,dcmask,dclay,dmlay,dslay,dsmask,dhole,holecat

|

| where:

|

|  001              record type

|

|  dcmask           pad diameter for component side mask

|

|  dclay            pad diameter for component side layer

|

|  dmlay            pad diameter for middle layers

|

|  dslay            pad diameter for solder side layer

|

|  dsmask           pad diameter for solder side mask

|

|  dhole            hole diameter

|

|  holecat          hole category

|

|

| Allowed range for pad/hole diameters:

|

|        from .000"/.00mm to .255"/6.47mm

|

| Allowed range for hole catergory:

|

|        from  0 to 7

|

| Example:

|

|  001,.062,.050,.050,.050,.062,.032, 0

-------------------------------------------------------------------------------

Integration |

status      | 002,intgstatus

Only with   |

integrated  | where:

databases   |

|  001              record type

|

|  intgstatus       database integration status

|

|        b3         1 - back annotation required

|        b2         1 - front annotation required

|        b1         1 - layout database exists

|        b0         1 - schematics databse exists

|

| Example:

|

|  002,11

-------------------------------------------------------------------------------

|

Trace       |

width/      | 010,widthcode,tracewidth,dcode

aperture    |

table       | where:

|

For L3 data-|  010              record type

base this   |

table conta-|  widthcode        width code in range from 0 to 15

ins trace   |                   (unused in L3 database)

widths for  |

instant sele|  tracewidth       trace width in range from .000"/.00mm

ctions.     |                                        to   .255"/6.47mm

|

| ONLY for L and L2 data bases:

|

|  dcode            GERBER aperture code Dnn*

|

| Trace width/aperture table consists of 16 records where

| widthcode must appear in asccending sequence.

|

| Example:

|

|  010, 0,.100,D19*

|  010, 1,.008,D11*

|  010, 2,.013,D12*

|       .

|       .

|

-------------------------------------------------------------------------------

|

Trace       |

width/      | 011,number,tracewidth,dcode

aperture    |

table       | where:

|

FOR L3 DATA-|  011              record type

BASE ONLY   |

VERSIONS    |  number           sequence number fom 0 to 31

UP TO 2.30/ |

1.50  |  tracewidth       trace width in range from .000"/.00mm

|                                        to   .255"/6.47mm

|

|  dcode            GERBER aperture code Dnn*

|

| Trace width/aperture table consists of 32 records

| Unused positions must have tracewidth=0 and dcode=D00*.

|

| Example:

|

|  011, 0,.100,D19*

|  011, 1,.008,D11*

|  011, 2,.013,D12*

|       .

|       .

|

-------------------------------------------------------------------------------

Pad         |

aperture    | 020,paddiam,dcode

table       |

| where:

ONLY FOR    |

L DATABASES |  020              record type

|

|  paddiam          pad diameter in range from .000"/.00mm

|                                         to   .255"/6.47mm

|

| dcode             GERBER aperture code Dnn*

|

| Pad aperture table consists of 64 records where

| first 32 records contain Dcodes for round pads

| and last 32 records contain Dcodes for square pads

| Unused positions must have paddiam=0 and dcode=D00*.

|

| Example:

|

|  020,.040,D70*

|  020,.050,D71*

|  020,.055,D20*

|  020,.062,D21*

|       .

|       .

|

-------------------------------------------------------------------------------

Symbol      |

creation    | 021,flag,diameter

pad/hole    |

default     | where:

diameters   |

and         |  021              record type

shapes      |                   always four records containing:

|                   1st record: pad diam. for comp. side mask

|                   2nd record: pad diam. for comp/sold layers

|                   3rd record: pad diam. for middle layers

|                   4th record: hole diameter

|

|  flag             Valid only in 1st record

|        b2         0 - round  1 - square mask layers pads

|        b1         0 - eound  1 - square comp./sold. layer pads

|        b0         0 - round  1 - square middle layer pads

|

|  diameter         pad/hole diameter in range from .000"/.00mm

|                                              to   .255"/6.47mm

|                   record in sequence describes which diameter

|

| Example:

|

|  021, 0,.075

|  021, 0,.062

|  021, 0,.055

|  021, 0,.035

|

-------------------------------------------------------------------------------

Text        |

size/       | 030,sizecode,xsize,ysize,linewidth,1

line width  |

table       | where:

|

|  030              record type

|

|  sizecode         text character size code in range from 1 to 7

|

|  xsize            text character vertical dimension

|

|  ysize            text character horizontal dimension

|

|  linewidth        text line width with GERBER photoplotting

|

| Text size/line width table consists of 7 records. Text character

| x and y sizes must multiplication of .006" which is lowest

| allowed text sizes. If xsize=0 and ysize=0 then EED3 defaults

| will be used.

| Text line width range from .001" to .255"

| Maximal allowed text size is .600".

| Ascending sequence for size codes is required.

|

| Example:

|

|  030, 1,.060,.040,.013,1

|  030, 2,.120,.080,.016,1

|  030, 3,.180,.120,.020,1

|  030, 4,.240,.160,.030,1

|         .

|

-------------------------------------------------------------------------------

Board       |

outline/    | 040,bdwidth,pdwidth,1

Component   |

print       | where:

apertures   |

|  bdwidth          board outline widh with GERBER photoplotting

|

|  pdwidth          silkscreenline width with GERBER photoplotting

|

| Example:

|

|   040,.005,.013,1

-------------------------------------------------------------------------------

II. Symbol Library

All coordinates are expressed as displacement from coordinates

of entry/pad number 1 which are always X=0 and Y=0

Symbol definition consist of several records. Structure is

following:

Record type:   Number:      Contents:

100             1            symbol header

101             1 or more    entry/pad definition

102             0 or more    single line primitive

102             0 or more    single conductor (line)

103             0 or more    single rectangle primitive

103             0 or more    single conductor (solid rect.)

104             0 or more    single circle primitive

105             0 or more    single arc primitive

106             0 or more    single text string

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Library     |

symbol      | 100,smbref,smbname,xname,yname,xdesc,ydesc,entries,xomin,yomin,

header      |      xomax,yomax,xemin,yemin,xemax,yemax

|

| where:

|

|  100              record type

|

|  smbref           symbol reference number within database

|

|  smbname          symbol name 1 to 8 characters

|

|  xname,yname      coordinates for component name text

|

|  xdesc,ydesc      coordinates for component description text

|

|  entries          number of entries/pads in symbol

|

|  xomin,yomin      minimal coordinates for outline/conductors

|

|  xomax,yomax      maximal coordinates for outline/conductors

|

|  xemin,yemin      minimal coordinates for entries

|

|  xemax,yemax      maximal coordinates for entries

|

| Example:

|

|   100,1,R47,.200,.200,.400,.400,14,-.050,-.050,.350,.350,

|                  .000,.000,.600,.300

------------------------------------------------------------------------------

Single      |

entry/pad   | 101,xentry,yentry,dcmask,dclay,dmlay,dslay,dsmask,dhole,hcat,shp

definition  |

| where

|

|  101              record type

|

|  xentry,yentry    entry/pad X,Y coordinates

|

|  dcmask           pad diameter for component side mask

|

|  dclay            pad diameter for component side layer

|

|  dmlay            pad diameter for middle layers

|

|  dslay            pad diameter for solder side layer

|

|  dsmask           pad diameter for solder side mask

|

|  dhole            hole diameter

|

|  hcat             hole category

|

|  shp              8 bit mask

|                   0 - round shape  1 - square shape   X - unused

|        b7         X

|        b6         X

|        b5         sold. side mask pad shape

|        b4         sold. layer pad shape

|        b3         middle layers pad shape

|        b2         comp. layer pad shape

|        b1         comp. side mask pad shape

|        b0         X

|

| Example:

|

| 101,.000,.000,.075,.063,.055,.063,.075,.035,0,0

| 101,.100,.000,.075,.063,.055,.063,.075,.035,0,0

| 101,.200,.000,.075,.063,.055,.063,.075,.035,0,0

|                  .

|                  .

-------------------------------------------------------------------------------

Line        |

primitive/  | 102,xstrt,ystrt,xend,yend,size,side,mask,link

conductor   |

| where:

|

|  102              record type

|

|  xstrt,ystrt      start of line coordinates

|

|  xend,yend        end of line coordinates

|

|  size             trace width code for conductor (L and L2)

|                   trace width for conductor (L3)

|                   0 for line (outlines)

|

|  side             layer

|                   a. outlines:

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|                   b. conductors:

|                   1 component side mask layer

|                   2 component side trace layer

|                   3-12  middle trace layers 10 to 1

|                   13 solder side trace layer

|                   14 solder side mask layer

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         1 - filling line 0 - common line

|        b0         1

|

|  link             0 for outlines

|                   for conductors:

|                   a. negative value of entry number links

|                      conductor to joint trace segments (other

|                      conductors ) from which at least one is

|                      connected to pad/entry in link

|                   b. positive value connects conductor to

|                      joint trace segments (other conductors)

|                      with the same number - no connection to

|                      pad/entry

|                   c: 0 means single unconnected conductor

|                      segement

|

| Example:

|

|   102,.000,.000,.000,.500, 0, 2, 1,-1

|   102,.200,.000,.200,.500, 1, 2, 1,2

|

|   102,-.100,.100,.050,.100, 0, 0, 1, 0  <-- outline

-------------------------------------------------------------------------------

Rectangle   |

primitive   | 103,xstrt,ystrt,xdim,ydim,sizecode,side,mask,link

Solid       |

rectangle   | where:

conductor   |

|  103              record type

|

|  xstrt,ystrt      start corner coordinates

|

|  xdim,ydim        horizontal and vertical dimensions

|

|  sizecode         0

|

|  side             layer:

|                   a. outlines:

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|                   b. conductors (solid rectangle):

|                   1 component side mask layer

|                   2 component side trace layer

|                   3-12  middle trace layers 10 to 1

|                   13 solder side trace layer

|                   14 solder side mask layer

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0 for outlines

|                   for conductors:

|                   a. negative value of entry number links

|                      conductor to joint trace segments (other

|                      conductors ) from which at least one is

|                      connected to pad/entry in link

|                   b. positive value connects conductor to

|                      joint trace segments (other conductors)

|                      with the same number - no connection to

|                      pad/entry

|                   c: 0 means single unconnected conductor

|                      segement

|

|

| Example:

|

|

|  103,-.100,.100,.050,.100, 0, 0, 1, 0

-------------------------------------------------------------------------------

Circle      |

primitive   | 104,xcent,ycent,radius,sizecode,side,mask,link

|

| where:

|

|  104              record type

|

|  xcent,ycent      circle center coordinates

|

|  radius           circle radius

|

|  sizecode         0

|

|  side             layer:

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0

|

| Example:

|

|  104,-.100,.100,.250, 0, 0, 1, 0

-------------------------------------------------------------------------------

Arc         |

primitive   | 105,xcent,ycent,radius,sangle,aangle,sizecode,side,mask,link

|

| where:

|

|  105              record type

|

|  xcent,ycent      circle center coordinates

|

|  radius           circle radius

|

|  sangle           arc start angle 0-360 deg.

|

|  aangle           arc angle 0=360 deg.

|

|  sizecode         0

|

|  side             layer:

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         0

|        b1         0

|        b0         1

|

|  link             0

|

| Example:

|

|  105,-.100,.100,.250, 45, 180, 0, 0, 1, 0

-------------------------------------------------------------------------------

Text        |

string      | 106,xtxt,ytxt,rot,sizecode,side,mask,link,textstring

|

| where:

|

|  106              record type

|

|  xtxt,ytxt        first character in string coordinates

|

|  rot              text rotation rot*90 deg

|

|  sizecode         text size code

|

|  side             layer:

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0 - common text 1 - COMPNAME,COMPDESC,user ref.

|        b2         0

|        b1         0

|        b0         1

|

|  link             0 - common text

|                   1 - COMPNAME

|                   2 - COMPDESC

|                   3 - user ref.

|  textstring       text string

|

| Example:

|

|  106,.200,.200, 0, 1, 0, 8, 1,COMPNAME

|  106,.400,.400, 0, 1, 0, 8, 2,COMPDESC

|  106,-.100,.100,1, 1, 0, 1, 0,Data In

------------------------------------------------------------------------------

III. Ground/Copper plane global symbols

All coordinates are expressed as defined previously in

database header record.

Groundplane definition consists of several records. Structure

is following:

Record type:   Number:      Contents:

120             1            Groundplane header

122             0 or more    single conductor (line)

122             0 or more    single line primitive

123             0 or more    single rectangle primitive

123             0 or more    single conductor (solid rect.)

125             0 or more    single arc primitive

125             0 or more    single conductors (arc)

126             0 or more    single text string

There may be up to 8 different Ground/Copper plane symbols

defined in data base. They are named @GND for symbol refernce

number -10000 and COPPER.A to COPPER.F for refernce symbol

numbers -10003 to -10009.

Every symbol contains text string record (126) with mask=8

and link=1 defining reference net name for this copper plane.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Groundplane | 120,gnumber,gname

header      |

| where:

|  120              record type

|

|  gnumber          reference number for groundplane

|                   global symbols (-10000 or -10003 to -10009)

|

|  gname            for ground/copper plane global symbol

|                   (@GND or COPPER.A to COPPER.F)

-------------------------------------------------------------------------------

Groundplane |

primitives/ | 122               same format as 102 (line)

conductors  |

| except:           link=0

|

| 123               same format as 103 (rectangle)

|

| except:           link=0

|

| 125               same format as 105 (arc)

|

| except:           size=trace width

|                   link=0

|

| 126               same format as 106 (text)

|

| except:           mask=8 and link=1 - reference net name

|                   otherwise link=0

|

| Example:

|

|   122,.000,.000,.000,.500, .20, 2, 1,0

|   122,.200,.000,.200,.500, .25, 2, 1,0

|

------------------------------------------------------------------------------

IV. Dimensioning global symbol

All coordinates are expressed as defined previously in

database header record.

Dimensioning informations consist of several records.

Structure is following:

Record type:   Number:      Contents:

140             1            Dimension header

142             0 or more    single dimensioning line

145             0 or more    single arc (angle dimension)

146             0 or more    dimensioning text strings

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Dimension   | 140,-10002,@DIM

header      |

| where:

|

|  140              record type

|

|  -10002           fixed reference number for dimensioning

|                   global symbol

|

|  @DIM             fixed name for dimensioning global symbol

-------------------------------------------------------------------------------

Dimensioning|

primitives  | 142               same format as 102 (line)

|

| 145               same format as 105 (arc)

|

| 146               same format as 106 (text)

|

|  except:          for all dimensionig primitives:

|

|  sizecode         0 for 142,145 or text size code for 146

|

|  side             0

|

|  mask             8 bit extra info mask

|        b7         X

|        b6         X

|        b5         X

|        b4         X

|        b3         0

|        b2         1 dimension line flag

|        b1         0

|        b0         0

|

|  link             all primitives in one dimensioning

|                   have the same value in link to be

|                   identified as entity

|

| Example:

|

|  142,3.300,2.500,3.300,-2.500, 0, 0, 4, 1

|  142,2.500,2.500,3.350,2.500, 0, 0, 4, 1

|  145,-.100,.100,.250, 45, 180, 0, 0, 4, 2

|  146,3.200,-.100, 1, 1, 0, 4, 1,5mm

------------------------------------------------------------------------------

V. Board Description (Design Notes) global symbol

All coordinates are expressed as defined previously in

database header record.

Symbol definition consist of several records. Structure is

following:

Record type:   Number:      Contents:

160             1            symbol header

162             0 or more    single line primitive

163             0 or more    single rectangle primitive

164             0 or more    single circle primitive

165             0 or more    single arc primitive

166             0 or more    single text string

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Design Notes|

symbol      | 160,-10001,@DES

header      |

| where:

|

|  160              record type

|  -10001           fixed reference number for design notes

|                   global symbol

|

|  @DES             fixed name for design notes global symbol

|

|

------------------------------------------------------------------------------

Design Notes|

primitives  | 162              same format as 102 (line)

|

| 163              same format as 103 (rectangle)

|

| 164              same format as 104 (circle)

|

| 165              same format as 105 (arc)

|

| 166              same format as 106 (text)

|

| except:          for all primitives

|

|  side             layer

|                   0  for component side print (outlines)

|                   15 for solder side print    (outlines)

|

|

|  link             0

|

| Example:

|

|  162,-.100,.100,.050,.100, 0, 0, 1, 0

|  163,-.100,.100,.050,.100, 0, 0, 1, 0

|  164,-.100,.100,.250, 0, 0, 1, 0

|  165,-.100,.100,.250, 45, 180, 0, 0, 1, 0

|  166,-.100,.100,1, 2, 0, 1, 0,DESIGN NOTES

------------------------------------------------------------------------------

VI. Device Description Library.

Every device description consist of several records.

Structure is following:

Record type:   Number:      Contents:

200             1            device header

201             1 or more    Voltage&Gnd pinout

202             1 or more    Group header

203             1 or more    group pinout

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Device      |

header      | 200,devref,devname,prefix,laysmb,no.groups,status,laydesc

|

| where:

|  200              record type

|

|  devref           device reference number within database

|

|  devname          device name 1 to 8 characters

|

|  prefix           layout component name prefix 1 to 4 characters

|

|  laysymb          name of layout symbol to represent device  in

|                   layout database

|

|  no.goups         namber of groups within device

|

|  status           XXXXXXXX - internal 8 bit mask (0 default)

|

|  laydesc          text string to appear with layout componet

|                   referencing this device

|                   "none" stands for "no text condition"

|

|  Example:

|

|   200, 1,7400    ,IC,R47     , 4, 0,none

------------------------------------------------------------------------------

Voltage     |

Ground      | 201,volt,pinnumber

pinout      |

| where:

|  201              record type

|

|  volt             voltage designator in form:

|          VCn      n from 1 to 9 for Vcc1 to Vcc9

|          GN0      for GND

|

|  pinnumber        pin number (layout symbol) where voltage/GND

|                   is connected

|

| Example:

|

|  201,VC1, 14

|  201,GN0, 7

------------------------------------------------------------------------------

Group       |

header      | 202,schsymb,simcode,rpower,rdelay,fpower,fdelay,nparams,schdesc

|

| where:

|  202              record type

|

|  schsymb          schematic symbol to represent this group in

|                   schematic data base

|

|  simcode          simulation model ID number

|

|  rpower           power level for logic high

|           S       Supply -         priority level 1

|           D       Drive  -         priority level 2

|           R       Resisitive -     priority level 3

|           Z       High impedence - priority level 4

|

|  rdelay           propagation Low->High in simulator time steps

|

|  fpower           power level for logic low

|           S       Supply -         priority level 1

|           D       Drive  -         priority level 2

|           R       Resisitive -     priority level 3

|           Z       High impedence - priority level 4

|

|  fdelay           propagation High->Low in simulator time steps

|

|  nparams          no. of parameters required to define simulation

|                   model

|

|  schdesc          text string to appear with schematics componet

|                   referencing this device

|                   "none" stands for "no text condition"

|

|  Example

|

|   202,G00     , 11,D, 1,D, 1, 0,none

------------------------------------------------------------------------------

Group       |

pinout      | 203,pinnumber,dir,simentry,vcode,swaplevel

|

| where:

|  203              record type

|

|  pinnumber        layout symbol pin number

|

|  dir              direction

|            I      input

|            O      output

|            T      tristate

|

|  simentry         logic entry number for simulation model

|

|  vcode            pin packaging no visibility on schematics

|            v      pin number visible

|            i      pin number invisible

|

|  swaplevel        all pins with the same number are ligically

|                   swapable

|                   -1 designates unswapable pin

|

| Example:

|

|  203, 3,O, 0,v,-1

|  203, 1,I, 0,v,1

|  203, 2,I, 1,v,1

------------------------------------------------------------------------------

VII. Netlist

Every nete description consist of several records.

Structure is following:

Record type:   Number:      Contents:

300             1            net header

301             1 or more    node

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Net         |

header      | 300,netref,netname,status,busref,busmemb

|

| where

|

|  300              record type

|

|  netref           net reference number within database

|

|  status           net 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      1 if SPL  otherwise 0

|           b3      1 if named  otherwise 0

|           b2      1 if bus othervise 0

|           b1      1 if in bus otherwise 0

|           b0      1 if active otherwise 0

|

|  busref           if net in bus then bus net refernce number

|

|  busmemb          if net in bus then bus member number

|

|  Example:

|

|   300, 1,SPL1    , 25, 0, 0

|   300, 2,UN1     , 73, 0, 0

------------------------------------------------------------------------------

Node record |

| 301,status,compref,entry

|

| where:

|

|  301              record type

|

|  status           node 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      X

|           b0      1 if physically connected otherwise 0

|

|  compref          component reference number

|

|  entry            entry number for node

|

|  Example:

|

|   301, 1, 2, 2

|   301, 1, 1, 13

------------------------------------------------------------------------------

VIII. Page information

Every page description consist of several records.

Structure is following:

Record type:   Number:      Contents:

400             1            page header

401             5 or more    board outlines corners

Coordinates for board outline corners are expressed as defined

previously in database header record.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Page header |

| 400,pageref,pagename,status,Xzoom,Yzoom,scal,Xmin,Ymin,Xmax,Ymax

|

| where:

|

|  400              record type

|

|  pageref          page reference number within data base

|

|  pagename         1 to 8 characters of page name

|                   MAINPAGE for page no. 1

|

|  status           page status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      1 if page active otherwise 0

|           b0      1

|

|  Xzoom,Yzoom      Coordinates of last ZOOM center point

|

|  scal             Scale factor in last ZOOM

|            1      scale 10:1

|            2      scale  5:1

|            5      scale  2:1

|           10      scale  1:1

|           20      scale  1:2

|           50      scale  1:5

|          100      scale  1:10

|

|  Xmin,Ymin        Coordinates of Xmin,Ymin of board outline

|

|  Xmax,Ymax        Coordinates of Xmax,Ymax of board outline

|

| Example:

|

|  400, 1,MAINPAGE, 3,.000,.000, 10,-2.500,-2.500,2.500,2.500

------------------------------------------------------------------------------

Board       |

outline     | 401,xcorn,ycorn

corner      |

| where:

|

|  401              record type

|

|  xcorn,ycorn      Outline corner X/Y coordinates

|

| Example:

|

|  401,-2.500,-2.500

|  401,-2.500,2.500

|  401,2.500,2.500

|  401,2.500,-2.500

|  401,-2.500,-2.500

------------------------------------------------------------------------------

IX. List of via holes

Coordinates for via holes are expressed as defined

previously in database header record.

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Via hole    |

| 500,viaref,pageref,status,laymask,xvia,yvia

|

| where:

|

|  500              record type

|

|  viaref           via hole reference number within data base

|

|  pageref          page reference number to which via hole is

|                   attached

|

|  status           via 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      1 if burried via otherwise 0

|           b0      1

|

|  laymask          16 bit mask defining layer on which

|                   entry is active

|

|  xvia,yvia        via hole coordinates

|

| Example:

|

|  500, 1, 1, 1, 32766,-2.200,-1.500

|

------------------------------------------------------------------------------

X. List of components

Coordinates for component placement are expressed as defined

previously in database header record.

Stucture of layout component definition is following:

Record type:   Number:      Contents:

600             1            component definition

601             0 or more    packaging information

602             1            component value

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Component   |

| 600,compref,cmpname,pageref,status,symbref,devref,xcomp,ycomp,

|        cscale,crot,cmirror,cmask,xname,yname,cnside,cnsize,cnrot,

|        xuser,yuser

| where:

|

|  600              record type

|

|  compref          component reference number within database

|

|  compname         component name 1 to 8 characters

|

|  pageref          reference number for page to which copmponent is

|                   attached

|

|  status          component 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      0 - unlocked   1 - locked

|           b2      0 - name invisible   1 - name visible

|           b1      1 - display combined symbol

|                   0 - no combined symbol displayed

|                   (combined symbol is secondary symbol with

|                    reference no. = -compref)

|           b0      1

|

|  symbref          reference no. of symbol used to display

|                   component

|

|  devref           reference no. of component device

|

|  xcomp,ycomp      coordinates for component entry 1

|                   (component placement coordinates)

|

|  cscale           component display scale = 4 ( 1:1)

|

|  crot             component rotation crot*90 deg

|

|  cmirror          0 - unmirrored display

|                   1 - mirrored display

|

|  cmask            reference symbol display mask

|

|  xname,yname      coordinates of component name text string in

|                   reference to coordinates of entry 1

|

|  cnside           component name placement layer

|

|  cnsize           component name text size code

|

|  cnrot            component text name rotation cnrot*90 deg

|

|  xuser,yuser      coordinates of <user ref. text in symbol

|                   referenced by the component, translated to

|                   component location, rotation and mirroring.

|                   (Additional user defined reference point for

|                    ex. Pick&Place machines)

|

| Example:

|

|  600,1,IC1,1,5,1,1,-2.300,-2.300,4,0,0,7,.200,.200,0,1,0,

|  -2.300,-2.300

------------------------------------------------------------------------------

Packaging   |

information | 601,group,scompref

|

| where:

|

|  601              record type

|

|  group            sequential group number within device used

|                   by component

|

|  scompref         reference number of schematic component

|                   assigned to group

|

| Example:

|

|   601, 1, 0

|   601, 2, 0

|   601, 3, 4

|   601, 4, 2

------------------------------------------------------------------------------

Component   |

value       | 602,compref,textstring

|

| where:

|

|  602              record type

|

|  compref          component reference number within data base

|

|  textstring       component value text

|

| Example:

|

|   602,1,200pF

-------------------------------------------------------------------------------

XI. List of references schematic vs layout components

Stucture of layout component definition is following:

Record type:   Number:      Contents:

650             1            schematic component definition

651             1            packaging information

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Schematic   |

component   | 650,scompref,scompname,status,devref,group

|

| where:

|

|  650              record type

|

|  scompref         schematic component reference number within

|                   database

|

|  scompname        schematic component name 1 to 8 characters

|

|  status           8 bit status: XXXXXXX1

|

|  devref           reference no. for device used by component

|

|  group            group within device used by component

|

| Example:

|

|  650, 1,L1      , 65, 1, 1

|

------------------------------------------------------------------------------

Packaging   |

information | 651,1,compref,group

|

| where:

|

|  651              record type

|

|  1                sequence number (=1)

|

|  compref          layout component reference no.

|

|  group            group occupied by schematics component within

|                   layout component

|

| Example:

|

|  651, 1, 2, 1

------------------------------------------------------------------------------

XII. List of traces

Coordinates for trace bending points are expressed as defined

previously in database header record.

Structure of trace description is following:

Record type:   Number:      Contents:

700             1            trace header

701             2 or more    trace segments

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Trace header|

| 700,trcref,netref,pageref,status

|

| where:

|

|  700              record type

|

|  trcref           trace reference number within database

|

|  netref           net reference number to which trace belongs

|

|  pageref          page reference number to which trace is attached

|

|  status           trace 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      X

|           b1      1 - bus oterwise 0

|           b0      1

|

| Example:

|

|  700, 1, 1, 1, 193

------------------------------------------------------------------------------

Trace       |

segment     | a. 701,status,side,size,compref,entry

|   or

| b. 701,status,side,size,viaref

|   or

| c. 701,status,side,size,xbpoint,xbpoint

|

| where:

|  700              record type

|

|  status           segment end 8 bit status

|           b7      X

|           b6      X

|           b5      X

|           b4      X

|           b3      X

|           b2      1 component otherwise   0

|           b1      1 via hole    otherwise 0

|           b0      1 bending point absolute otherwise 0

|

|  side             layer for trace segment

|                   1 component side mask layer

|                   2 component side trace layer

|                   3-12  middle trace layers 10 to 1

|                   13 solder side trace layer

|                   14 solder side mask layer

|

|  size             trace width

|

| a. variant (b2b1b0 in status = 100)

|

|  compref          component reference no. to which segment

|                   is connected

|

|  entry            entry number within component to which segement

|                   is connected

|

| b. variant (b2b1b0 in status = 010)

|

|  viaref           via hole reference no. to which segment is

|                   connected

|

| c. variant (b2b1b0 in status = 001)

|

|  xbpoint,ybpoint  bending point coordinates

------------------------------------------------------------------------------

XII. Text strings

Coordinates for text strings are expressed as defined

previously in database header record.

Structure of trace description is following:

Record type:   Number:      Contents:

800             0 or more    text string

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

Text string |

| 800,link,pageref,status,xtxt,ytxt,side,size,rot,textstring

|

| where:

|

|  800              record type

|

|  link             0

|

|  pageref          page reference number to which text is

|                   attached

|

|  status           text 8 bit status =XXXXXXX1

|

|  xtxt,ytxt        coordinates of first character in text string

|

|  side             text placement layer

|                   0  for component side print

|                   15 for solder side print

|                   1 component side mask layer

|                   2 component side trace layer

|                   3-12  middle trace layers 10 to 1

|                   13 solder side trace layer

|                   14 solder side mask layer

|

|  size             text size code

|

|  rot              text rotation rot*90

|

|  textstring       text string

|

| Example:

|

|  800, 0, 1, 1,-1.400,.300, 0, 1, 0,COMP.PRINT

------------------------------------------------------------------------------

XIII. End of File record

===============================================================================

RECORD      | RECORD FORMAT AND CONTENTS DESCRIPTION

MEANING     |

===============================================================================

End of File |

| 999

|

| where:

|

|  999              record type

------------------------------------------------------------------------------