MSYS

(C) Copyright 1993 By HUB COMPUTERS, INC.

All Rights Reserved

INTRODUCTION

MSYS is a Copyrighted program that I am making available to the amateur

radio community free of charge.  Source code is not generally available

(if you want it you will have to pay dearly for it!).

If you happen to see my system on the air don't get excited when you

see that I am running the next numbered version of MSYS.  It doesn't

mean its ready for distribution!  I generally use the next number on my

system as soon as I release a version as there are always changes to be

made.

KA-NODE is a trademark of Kantronics Inc.  The KANODe component of MSYS

is compatible with the Kantronics KA-NODe but is completely different

code from that used in the Kantronics tncs.

NET/ROM is a trademark of SOFTWARE 2000.  The network NODe component of

MSYS is compatible with NET/ROMs but is completely different code from

that sold by SOFTWARE 2000.

The network NODe component of MSYS is completely different from that in

the KA9Q-NOS program and G8BPQ Node software.

IBM is a trademark of International Business Machines Corp.

SAM is a trademark of RT Systems

j-comm is a trademark of j-comm Inc.

MSYS is available from the Cleveland Hamnet phone BBS.  Numbers are

216-942-6382 and 216-942-7516.  Both are 300 to 9600 baud.  This is a

free BBS (you pay for the call).  MSYS may also appear on other phone

BBS systems, but this is the only one I put it on directly.

You may also obtain MSYS from me directly on disk (this may be the

cheapest way really and the fastest when a new version comes out).  For

US addresses the cost is $5 per disk which includes the disk, mailer

and postage.  You may order several versions at a time, I will hold

your request until the versions are released.  This way you get to be

the first kid on the block with the new toy!  When ordering from me,

please specify which version you already have or which one you want.

Distribution is on 5.25 inch disks.  I keep records with the following

information for people who order MSYS, so you may want to give me this

info.  Also please indicate if you want me to call you (collect) if it

would be helpful to answer questions you send me about MSYS.

CALL:          Your callsign.  This is the key I use to record all your

information.

NAME:          Where to send the disk to!

ADDR1:

ADDR2:

CITY:

STATE:

ZIP:

COUNTRY:

TELEPHONE:     If you want me to call you back (collect)

VERSION:       The version you have now

OWED:          How many you have prepaid

COMPUTER:      Speed, memory, disk space, CPU chip etc.

TNC:           Model & ROM versions

DOS:           Which version?

BBS:           BBS callsign to send you msgs .

IP:            Your IP address, if you have one

Mail requests for MSYS on disk to:

Michael Pechura

10809 Beechwood Drive

Chesterland, OH 44026

You may call me with questions/comments at   216-256-1588.

Canadian stations may want to contact VE3WZL for distribution info in

Canada.

Address:

J. B. Weazle McCreath

Aerial Acres

R.R. No. 4,

Goderich, Ontario, Canada

N7A3Y1

Ordered from me the cost is $7.50 per disk.

Other  foreign  stations:  Price is $10 (US) per disk which includes

Air Mail postage.

A stripped down version of MSYS that supports only the BBS functions is

also available.  There is no network NODe, no amtor, no tcp/ip in this

version.  You can still forward using external network nodes, but the

bbs is NOT a netrom network node!  The KANODe is still there.  All of

the BBS functions remain, along with modem support.  Expanded memory is

also still supported.  This version is called MSYSB and is available on

a separate disk or on Cleveland Hamnet.  If want to get this disk from

me, it is the same price as the others: $5 to US addresses, $7.50 to

Canada and Mexico, and $10 to other countries.

MSYS 1.14                                                   Page 2

A Multi-User, Multi-Port, Multi-Function

Amateur Packet Radio BBS

By Mike Pechura, WA8BXN

(C) Copyright 1993 By HUB COMPUTERS, INC.

All rights reserved.

Version 1.14  APRIL 1993

MSYS Documentation Table of Contents

INTRODUCTION...............................................1

MSYS Documentation Table of Contents.......................3

MSYS INITIAL INSTALLATION..................................4

MSYS UPDATE INSTALLATION INSTRUCTIONS......................6

SETUP CHANGES.........................................6

ADDITIONS & CHANGES...................................6

COMMENTS ABOUT CHANGES................................7

MSYS COMMAND LINE.....................................8

ABBREVIATIONS/CONVENTIONS.............................8

FREQUENT PROBLEMS AND NOTES FROM THE AUTHOR................9

The system will not forward...........................9

The system forwards messages to improper locations....9

Nothing is on the screen!.............................9

The system comes up and appears to hang..............10

SYSTEM REBOOTS AFTER FORWARDING......................10

PERFORMANCE HINTS....................................10

"I Haven't tried it, but maybe you will want to".....10

ADDITIONAL NOTES.....................................11

MSYS DIRECTORY LAYOUT DESCRIPTION.........................12

BIT PATTERN CHART.........................................21

DRSI CARDS................................................23

MSYS BBS USER Commands....................................63

REQxxx SERVERS............................................72

CALLSIGN SERVER...........................................74

MODEM SUPPORT........................................76

MSYS modem forwarding protocol.......................76

Incoming Call Protocol...............................76

MSYS.OPT FILES.......................................77

NETNode USAGE........................................78

FORWARDING ON THE MODEM PORT.........................79

FBB ON THE PHONE.....................................79

ANONYMOUS CHECK-INS..................................80

ACOUSTIC COUPLED MODEMS..............................81

MISCELLANEOUS NOTES..................................82

DATABASE HELP.............................................83

FORWARD FILE DESCRIPTION..................................85

NODe Scripts.........................................88

MSYSBBSB.DAT.........................................90

(callsign).FWD/.REV Files............................90

MERge: (MUST DO THIS IN ORDER TO FORWARD MAIL).......91

REVERSE FORWARDING...................................92

SELECTIVE FORWARDING:................................94

MSYS 1.14                                                   Page 3

TCP/IP FORWARDING:...................................94

SMTP FORWARDING......................................95

FORWARDING NOTES..........................................96

BID:.................................................96

FORWARDING DISPLAY...................................97

HF Forwarding........................................98

MSYS NODe Forwarding.................................98

MISSING TEXT TO Message..............................98

FBB COMPATIBLE FORWARDING............................99

FBB COMPATIBLE COMPRESSED FORWARDING................100

HIERARCHICAL FORWARDING and MSYS.........................102

Creating the BBSTONTS.BIN file......................104

HOUSECLEANING (HCL command)..............................105

MSYS.HCL............................................105

MSYS Network NODe Introduction...........................107

Routing.............................................107

Basic Decisions About the Network NODe..............108

Network NODe Quickstart.............................108

Manual Operations on the Network NODe Database......109

How I Run My Network NODe...........................110

NODE RELATED PARMS in MSYS.DO.......................111

Miscellaneous Comments on Network NODe..............113

NET NODe Parameters Summary.........................114

MSYS GENERAL INFORMATION.................................115

MEMORY MANAGEMENT...................................115

EXPANDED MEMORY USAGE...............................116

REMOTE SYSOP........................................117

Automatic Password Reply............................117

MSYS.DO.............................................118

MSYSTODO.DAT........................................118

***LINKED...........................................119

KEYBOARD............................................119

Packet size.........................................119

STACK Requirement...................................119

Device Errors.......................................119

Disk Full Message...................................120

MSYSTNC.#...........................................120

MAIL BEACON.........................................120

LOG.................................................120

LOCKUPS.............................................121

CHANNEL NUMBERS.....................................121

USER BIT SETTINGS:..................................121

USER PASSWORDS......................................121

#...................................................122

MAX channels AND PORTS..............................122

END OF LINE BELLS:..................................122

WP SERVER:..........................................122

TESTING.............................................122

Automatic Back-up...................................122

SMTP................................................123

Titles Truncated....................................123

MAIL.BAK............................................123

TheNet & NetRom RS-232 Connections..................123

Type of Message.....................................124

Automatic Message Holding...........................126

MSYS 1.14                                                   Page 4

DOS.................................................128

HEADERS.............................................128

KANTRONICS 3.0 EPROM Upgrade users:.................128

BBS ONLY ACCESS.....................................128

CONNECTS FROM NON-AMATEUR CALLSIGNS.................128

PASSWORDS...........................................129

MSYS Utility Support Programs............................130

MUTIL.EXE...........................................130

MUTIL Function 1 - Change max number of messages....131

MUTIL Function 2 - Restore Messages.................132

MUTIL Function 3 - Print Remote Sysop Phrase........133

MUTIL Function 4 - Check Mail Index File Integrity..134

MUTIL Function 5 - Test serial ports................135

MUTIL Function 6 - Print forwarding tree............136

MUTIL Function 8 - Run Log File Analyzer............137

MUTIL Function 9 - Run Setup Program................139

MUTIL Function 10 - Scan Message Headers............140

MUTIL Function 11 - Deletes Old Entries.............141

MUTIL Function 12 - Fix Mail Index File.............141

- RENUMBERING MESSAGES............142

MUTIL Function 13 - BBSLIST.DAT to BBSTONTS.DAT.....143

MUTIL Function 14 - BBSTONTS.DAT to BBSTONTS.BIN....145

MUTIL Function 15 - Build List of Archived Messages.145

MUTIL Function 16 - Printer Status..................145

MUTIL Function 17 - MERge BBSLIST.DAT Files.........146

MUTIL Function 18   Check MSYSNODE.DAT Integrity....146

MUTIL Function 19 - Convert User Files..............146

MUTIL Function 20 - Sort BBSLIST.DAT File...........147

MUTIL Function 21 - Delete Old BIDs (and MIDs)......147

MUTIL Function 22 - Specify Printer File Name.......147

SAMPLES OF MSYS FILES....................................148

MSYS.OPT:...........................................148

MSYS.DO:............................................149

MSYS.OTD:...........................................152

MSYSK.OTD...........................................152

MSYS.REP:...........................................152

MSYS.RMT:...........................................152

MSYSBBSB.DAT:.......................................152

MSYSFWD.DEF:........................................153

MSYSHOST.NET:.......................................154

MSYSPASS.DAT:.......................................154

Typical MSYSTNC.1...................................154

(AEA PK-232)........................................154

INFORMATION FOR ADVANCED USERS...........................155

REPLACEABLE DIRECTORY NAMES.........................155

Replaceable File Names..............................156

MSYS MSG FILE FORMAT:(For the adventuresome only!)..158

ROUTES.DAT FILE FORMAT:.............................159

MSYS under WINDOWS 3.1..............................160

MSYS under DesqView.................................161

SYSOP COMMANDS RELATED TO TCP/IP....................162

Files related to TCP/IP.............................163

USER ACCESS level chart.............................164

FTP COMMANDS:.......................................165

IP Routing:.........................................166

MSYS 1.14                                                   Page 5

EDITVIEW (ED Command)...............................167

EDITFILE (EF Command)...............................168

Epilog..............................................170

INDEX....................................................171

MSYS 1.14                                                   Page 6

MSYS INITIAL INSTALLATION

Copyright 1993 by HUB COMPUTERS, INC.

1. Installation of MSYS is not particularly difficult but you should

follow the following steps in order.

2. If you are upgrading from MSYS 1.13, please see the UPDATE

Instructions.

3. Make a backup copy of the distribution disk.  You can copy the

entire disk with Diskcopy.

4. Make a default directory on your hard disk.  I call mine MSYS.

Use the command:

md c:\msys

5. NOTE: You may use any drive letter.  All references to the C: drive

apply to your selected drive.

6. Go to the default directory: cd c:\msys.

7. Make required sub directories:

md c:\msys\mail

md c:\msys\files

md c:\msys\help

md c:\msys\mail.bak

md c:\msys\mail.bin             ***NEW VERSION 1.14

8. Move the file MSYS114.exe into the c:\MSYS directory

9. Un-arc the distribution disk by running the program MSYS114.EXE.

10.To move the help files to the help sub-directory.  Type MOVEFILE

11.NOTE:  REQUIRED FILE:  C:\MSYS\HELP\MSYSMSGS.DAT

This file contains a number of the seldom used messages used by

MSYS.

12.Change Directory to SAMPLES.  Edit the file

C:\MSYS\SAMPLES\MSYS.OPT with your favorite program editor.  See

documentation on MSYS.OPT for details.  After editing move MSYS.OPT

file to the default directory.

13.RUN MUTIL.EXE Function 9 to create C:\MSYS\MSYS.DEF from your

edited MSYS.OPT

14.Make sure you have at least:

files=40                        ***CHANGE VERSION 1.14

buffers=20

in your config.sys file in the root directory of your boot disk.

Don't forget to reboot after changing this!

MSYS 1.14                                                   Page 7

15.Using any ASCII editor, create MSYSFWD.DEF.  If you don't know what

to put it in now, put the following (to be replaced later):

F0 WA8BXN

WA8BXN

-----

16.Type MSYS and it should begin to run!

17.Type MERge. You can ignore any messages about MSYSBBSB.DAT at this

time.

SPECIAL NOTE

If you have a color monitor, the default is OK.  If you have a

monochrome monitor that uses and RCA phono pin plug then you are

using a Color Graphics Adapter interface card and the default

should be OK.  If you can do the command MODE MONO (this is a DOS

command) and not get any error messages and the computer continues

to work, you probably have a monochrome (TTL) interface card.  You

will need to specify WINDOW DISPLAY MONOCHROME in the MSYS.OPT

file.

MSYS 1.14                                                   Page 8

MSYS UPDATE INSTALLATION INSTRUCTIONS

SETUP CHANGES

There are two new directories that are required to update MSYS Ver.

1.13 to Ver. 1.14  One is the MSYS\MAIL.BIN directory that is used to

store the compressed messages as they are received and sent, and the

other is MSYS\YAPP which is used to store files that have been

transferred via YAPP compressed protocol.  Please see the directory

tree listed on page 15 for further details.  Use DOS commands to make

the new sub-directories.

Increase your FILES statement in CONFIG.SYS to files=40.  BUFFERS can

remain at 20.

It is suggested that a program similar to the Quarterdeck Systems:

VIDRAM ON be run prior to running MSYS to create as much working memory

as possible.  MSYS does not require graphic capability.  Of course, if

other background applications require graphics, this option is not

available.  VIDRAM works with EGA or VGA video adapters, and should not

be loaded into upper memory.

Run MUTIL 9 (setup) and of course, MUTIL 4 before restarting the new

version of MSYS.

ADDITIONS & CHANGES

There have been many changes to MSYS, some major and many minor.  Here

are some of the "major" changes:

1.   MSYS now supports compressed forwarding and is compatible with FBB

5.15 systems via radio.

2.   Either EXPANDED or EXTENDED memory is now detected and used.

3.   The DXNode component of MSYS has been removed.

4.   The AMTOR component of MSYS has been removed.

5.   The CONFERENCE node has been restored to the previous style.

6.   The modem forwarding and control problems have been resolved.  See

section on MODEMS.

7.   Many of the "little" things that have caused SYSOPS to pull their

hair out have been resolved, such as crashes on a restart in a

network environment.

8.   The R: lines have been revised to the new minimum standards to

eliminate the unnecessary clutter.

9.   The PING routine in TCP/IP has been fixed.

10.  The NETNODe now supports full binary data transfers.

11.  The number of unique forward entries has been increased from 300

to 400.

12.  The limit to the number of active messages has been increased to

2000.  But, there is a penalty of some loss of available memory

for user connects, based upon the number of active messages.

13.  YAPP protocol is now supported for file transfers to and from MSYS

using radio or modem

MSYS 1.14                                                   Page 9

COMMENTS ABOUT CHANGES

MUTIL 2 has been replaced by the /RESTore function in the main BBS, so

it is no longer necessary to exit MSYS to "bring back" those files

deleted in error.  When a message is restored, it will have a new

MESSAGE NUMBER in MSYS, but will maintain the existing BID number (if

any).  /REST ? from the bbs brings full help.

The major change in Ver. 1.14 is the addition of FBB compatible

protocol, both with  or without compression.  There are 4 new SYSOP

commands that need to be exercised to implement FBB style forwarding.

They are FBBC, NETFBBC, FBBG, and NETFBBG.  Please refer to the SYSOP

Command section for full details.  FBBC 0 turns off the special FBB

format.  FBBC 1 causes MSYS to respond to FBB non-compressed group

forwarding.  FBBC 2 enables full FBB-style compression forwarding.  The

MSYS\MAIL.BIN directory MUST be created prior to attempting compressed

forwarding.

A note about editing of compressed messages.  If you make a change for

any reason in the ASCII text BEFORE it has been forwarded out the first

time, the change will  be made in the compressed text.  If the bulletin

has been forwarded once, any changes in the ASCII text will NOT be

reflected in the forwarded bulletin.  If you find that a change is

necessary in a bulletin after it has been forwarded once, you must

delete the corresponding MSGxxxx.DAT file in the MAIL.BIN directory.

Probably the easy way to resolve that whole issue is to:  A) not make

changes in the text of messages or bulletins, or B) If you must, do it

before they have been forwarded anywhere, or C) RESTore the message,

make the ASCII text changes, and Kill the "old" message.

Some commands have had their abbreviations lengthened to accommodate

additional commands.  COpy is now COPy to make room for the CONference

(ON/off) command.  Conferencing can take considerable "bandwidth" if

large numbers of users utilize this feature.

MSYS 1.14                                                   Page 10

MSYS COMMAND LINE

The MSYS command typed at the DOS prompt may have several switches or

modifiers.

NOHEARD   Disables JK,JM,JD,JG,JB and JT recording.  Saves 26,880

bytes

NONODE    Disables network node, saves 28,104 bytes

NORAMHDRS Stores msg headers on disk only, saves 136 bytes per

possible msg.  This can be quite slow if you have lots

of msgs and no disk cache. Could be useful on 286

systems with cache in EXTENDED memory.

NOEMM     Disables Expanded Memory.

NOEXMEM   Disables Extended Memory.

Example:

MSYS NOHEARD   Starts MSYS but will not log any stations it hears.

MSYS NONODE    Starts MSYS without the NETNODe feature.

MSYS NORAMHDRS  Starts MSYS and stores all headers on disk - not

in RAM

MSYS NOEMM      Starts MSYS and forces MSYS to use EXTENDED memory

(if any).

MSYS            Starts MSYS, permitting it to use EXPANDED memory,

(if any) and all other features that have been

optioned.

MSYS has the ability to determine if there is an EXTENDED or EXPANDED

memory manager in use.  It will attempt to use the EXPANDED memory

unless the NOEMM option is specified, in which case it will attempt to

use the EXTENDED memory.

While MSYS may be run on an 8088 or 8086 system, much better

performance will be achieved by using a 286 or higher system.

ABBREVIATIONS/CONVENTIONS

Just a word about this manual.  Most commands are not case sensitive,

with the notable exception of the /Sysop command.  Many commands may be

abbreviated.  In this manual, we have attempted to show the minimum

abbreviation with the UPPER CASE letters with the balance of the

command word in lower case.  For example, the command ADDUser may be

typed as:  addu  or  ADDU  or  adduser  or  ADDUSER  with the same

results.  Where there is an argument shown as [on/OFF], the UPPER CASE

will be the default.  For example, in the case of the command

PC [on/OFF],  the call server lookup defaults to OFF.

MSYS 1.14                                                   Page 11

FREQUENT PROBLEMS AND NOTES FROM THE AUTHOR

IMPORTANT:  You must use the new version of MUTIL with this version of

MSYS (i.e., run setup, function 9).  It is probably also a good idea to

delete MSYS.HRD when going to a different version of MSYS or changing

the port statements in MSYS.OPT!

If you have problems getting MSYS going here are some things to try

that have helped many who have called:

1.  Make sure you have FILES=40 and BUFFERS=20 in CONFIG.SYS

2.  Try adding STACKS=0,0 to CONFIG.SYS

3.  Get rid of anything resident (check AUTOEXEC.BAT and CONFIG.SYS)

4.  Run MUTIL 4 (do this EVERY time you start MSYS)

5.  Simplify MSYS.OPT as much as possible; remove all PORT statements

6.  Delete your MSYS.DO file (after making a backup!)

These items should get you to the point where MSYS will work.  From

here try putting things back and see what causes failure.

The system will not forward.

Make sure you executed the MERge command.  Without doing this the

system will never forward.

NOTE:  You must have the file MSYSBBSB.DAT to be able to forward

bulletins.

The system forwards messages to improper locations.

Check to make sure that your MSYSFWD.DEF or MSYS.FWD files do not have

improperly placed "*" characters or have imbedded empty lines at the

end of the sections.  These "empty lines" are often found at the end of

the xxx.FWD or xxx.REV files.

The system appears to work but:

Nothing is on the screen!

Make sure you set the display type correctly in  the MSYS.OPT file.

The command is WINDOW DISPLAY COLOR for color monitors  or

WINDOW DISPLAY MONOCHROME  for monochrome monitors.

MSYS 1.14                                                   Page 12

The system comes up and appears to hang.

This may be caused by a non-existent com port.  Check the lines in your

MSYS.OPT to make sure the port addresses are correct.  Also, Rerun

MUTIL 4 to insure the mail files are properly indexed.

SYSTEM REBOOTS AFTER FORWARDING

This may be caused by having too many entries in your forwarding file.

Remember you may only have up to 400 different bbs names in your file!

PERFORMANCE HINTS

Don't let too many messages pile up in MAIL.BAK <DIR> before you delete

them (after archiving if you wish) so the hard disk doesn't fill up.

From time to time do a BID # command and see how many bids are active.

If its over 3 or 4 thousand its probably time to run MUTIL function 21

to get rid of the older bids.

Delete old log files.  It is probably wise to put a statement: ??D10000

log in your MSYSTODO.DAT file to keep the log file from expanding to

extremes.  It is not uncommon to have a log file exceed 1 megabyte in a

week's time!  The "log" statement will convert the MSYS.LOG file to a

yymmdd.log, which then can be archived off line.

While you are creating the MSYSTODO.DAT file, it is also not a bad idea

to add a: ????0001 NODe SAve  just to keep your current node list in

case of a reboot.

Periodically you should also do the following:

1.  Delete old BIDs using MUTIL 21

2.  Delete old backup messages. (after running Mutil 10)

"I Haven't tried it, but maybe you will want to"

Here are some suggestions from users of MSYS that they claim work.  Use

at your own risk!

To cure the problem of certain tncs that don't transmit properly on HF

use radiospeed of 1200 in place of 300 in MSYS.OPT.  I can't imagine

how this could help, but someone says it did for them (they weren't

using MFJ tncs which seem to have this kind of problem).  One thing you

might want to try if you are having problems with a tnc on HF is to get

a copy of KA9Q's net.exe program and see if you can use its AX.25

connect command with your  tnc on HF.  If that fails as well the

problem definitely is in the tnc.

MSYS 1.14                                                   Page 13

MFJ KISS MODE

To put a MFJ TNC in kiss mode, try this in your msystnc.# file

ax off

parity 0

awlen 8

conmode trans

kiss on

restart

Paccom ROM version 1.1.5 may work better than 1.1.662

If you are just bring up MSYS for the first time and experience crashes

when mail is forwarded to you, or the size of BIDLIST.DAT  is 0 bytes,

do the following to solve the problem:  Shut down MSYS (quit), delete

BIDLIST.DAT, start MSYS, shut it down (the DOS dir command should now

show a size of 130000 for  BIDLIST.DAT).  You should now be ready for

normal operation.

ADDITIONAL NOTES

If you are using DOS 3.3, you may need to put  STACKS=0,0 in your

config.sys file.

If you find that the displayed time is different from what you have set

the time to be in DOS, put the following in AUTOEXEC.BAT and reboot:

SET TZ=EST5  (Obviously, set it to YOUR time zone!)  One more thing -

the time can (and does) often get hosed up around the first of April

and the end of October because of certain software language routines

that attempt to account for the change in Daylight Savings time.  Not

to worry (too much) because it will eventually make itself right (with

maybe a little help from you!)

Home BBS replacement occurs only if @BBS is blank or the call of your

bbs.

Note:  If you run into problems with newer versions of QEMM and MSYS,

try turning off the stealth option in QEMM.

IF MSYS DOESN'T SEEM TO BE WORKING PROPERLY, REMOVE ALL

(EVERY LAST ONE!) RESIDENT PROGRAMS INCLUDING YOUR ENTIRE

CONFIG.SYS (keeping files=40 and buffers=20) AND AUTOEXEC.BAT

FILES AND SEE IF IT WORKS THAT WAY.  IF IT DOES, PUT THINGS

BACK IN ONE AT A TIME TO SEE WHERE THE PROBLEM IS!

MSYS 1.14                                                   Page 14

MSYS DIRECTORY LAYOUT DESCRIPTION

The following chart is the recommended organization for your

directories and files.  NOTE: Some of these files are created at time

of installation and others are operating files that MSYS creates while

operating.

C:\MSYS  Contains all of the MSYS operating programs and configuration

|       files

|  MSYS.EXE $     BBSTONTS.DAT # MSYS.MSG #

|  MUTIL.EXE $    MSYSNODE.DAT # MSYSPASS.MOD @

|  ROUTES.DAT $   MSYS.DEF #  MSYSBBSB.DAT @  MSYS.HCL @

|  STATES.DAT $   MSYS.FWD #  MSYSTODO.DAT @  MSYSHCL.DO @

|  ZIPCODES.DAT $ MSYS.HRD #  MSYSHOLD.DAT @  MSYS.OPT @

|  CALLSRV3.EXE $ MSYS.LOG #  MSYSFWD.DEF @   MSYS.OTD @

|  MSYS.BBS #     MSYS.USR #  MSYSHOST.NET @  MSYS.REP @

|  BBSTONTS.BIN # MSYS.BBS #  BADWORDS.DAT @  MSYS.RMT @

|       Files marked with a: $ are Distribution files.

|                       # are created by MUTIL or MSYS

|                       @ are SYSOP created files

|

|--------FWD

|   Directory which may contain forwarding files (.FWD)

|   the reverse forwarding  files (.REV) and the

|   PASSWORD.RMT file and is used by MERge command

|

|--------HELP

|   Directory which contains HELP messages including

|   MSYSMSGS.DAT

|

|--------MAIL

|   Directory which contains ACTIVE messages

|

|--------MAIL.BAK ***Optional

|   Directory which may contain BACKUP messages  See ADVANCED

|   USER section for suggestion!

|

|--------MAIL.BIN

|   Directory which contains COMPRESSED messages

|

|--------PASSWORD ***Optional

|   Directory which may contain user PASSWORD files

|

|--------PUBLIC   ***Optional

|   Directory which may contain uploaded FTP/SMTP messages

|

|--------YAPP     ***Optional

|   Directory which may contain YAPP files

MSYS 1.14                                                   Page 15

MSYS.OPT

Before MSYS can be used a configuration file (MSYS.OPT) must be created

and the program SETUP (MUTIL.EXE Function 9) must be run to create the

file MSYS.DEF. The file MSYS.OPT is a  standard ASCII file and may be

created with any text editor.  The basic format of the commands in the

file are:

PORT number AT number INT number SPEED number [NAME description]

PORT number SUBPORT number OF [PORT] number [NAME description]

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

NUMCHANS number

BBS CALL callsign

BBS ALIAS callsign

KANODe CALL callsign

ANSWER CALL callsign

FORWARD CALL callsign

ID CALL callsign

ID EVERY number

AX25 FRACK number

AX25 MAXFRAMES number

AX25 RETRIES number

AX25 CHECK number

MONITOR IFRAMES on/off

MONITOR UFRAMES on/off

MONITOR SFRAMES on/off

MONITOR BFRAMES on/off

MONITOR OUTGOING on/off

MONITOR PORTS [MASK]

WINDOW number SIZE number number number number

WINDOW number BACKGROUND number

WINDOW number FOREGROUND number

WINDOW number PAGE number

WINDOW UNUSED BACKGROUND number

WINDOW UNUSED FOREGROUND number

WINDOW DISPLAY color/monochrome

PROCESS processname [WINDOW] number

STACK processname number ***NOT USED IF USING EXT/EXP MEMORY

(SEE TEXT)

SYSOP NAME name

SYSOP QTH location

SYSOP ZIP zip code

MAKE

MSYS 1.14                                                   Page 16

Notes:

1.Items enclosed in square brackets ([...]) are optional

2.Numbers assumed in decimal unless prefixed by $ then hex is

assumed

3.on/off means the word ON or OFF

4.Where more than 1 space is shown, 1 or more space are OK

5.Callsign can include SSID (-number)

6.Ordering of the statements is generally not important

7.If there is an * at the beginning of a line it is ignored

8.The BBS alias, KANODe, Answer and Forward calls must be

different calls or SSID's.

9.MASK is described on page 24

Output from the execution of SETUP is the file MSYS.DEF (a binary file)

and a listing to the display console.  The listing consists of two

parts: the original input statements followed by a complete list of all

options generated (including defaults).

If any errors are detected, the MSYS.DEF file will not be created.

A sample of a MSYS.OPT file is found in MSYS.OPT on the distribution

disk.

A minimal MSYS.OPT file will have similar statements to these:

port 0 at $3e8 int 4 speed 1200 name 145.01

bbs call wa8bxn-13

answer  call wa8bxn-14

sysop name Mike

sysop qth Kirtland, OH

sysop zip 44026

id call wa8bxn

make

EXPLANATION OF EACH STATEMENT TYPE

PORT number at number INT number SPEED number [NAME Description]

[RADIOSPEED #][TNCtype#]   (ALL ON ONE LINE!)

PORT number SUBPORT number OF [PORT] number [NAME description]

The PORT statement has the above two variations.  The number following

the word PORT is the port number.  It must be between 0 and 6.  The

version containing the word 'at' defines an actual serial interface,

while the SUBPORT version defines a second logical port for multiport

TNCs such as the KPC-4.  There must  be one PORT statement for each

RADIO port to be used AND each MODEM port to be used.

Assign port numbers beginning with 0 in increments of 1.

Following the word AT is the base I/O address of the 8250 in the serial

port, such as $3F8. Following the word  INT is the interrupt number for

the 8250, such as 4.

MSYS 1.14                                                   Page 17

After SPEED is the baud rate to be used, such as 1200.  This is the

speed between the computer and the TNC or the MODEM.

Optionally the word NAME followed by some description of the port can

be given. A good description would be the corresponding radio

frequency.

If MSYS is interfacing a NET/ROM matrix, or a TNC using the NetRom

Serial protocol (NRS), precede the NAME with a ">"  as in:  >MATRIX

If MSYS is interfacing a modem, the NAME must be: MODEM

For the other form of the PORT statement, the number following the word

SUBPORT is the logical channel number, typically 1 for the KPC-4.  Next

comes the word  OF  in the statement optionally followed by the word

PORT.  Next comes the port number of a port statement defining an

actual port (i.e., contains  the word AT in its definition).  The

optional NAME and description is the same as above.

PORT description lines can also have two additional parameters:

RADIOSPEED bitrate# TNCTYPE #

RADIOSPEED specifies the bitrate used by the TNC to the radio (300 or

1200 probably).  If 300 bps is selected, shortened prompts are used in

the BBS and perhaps elsewhere.

TNCTYPE  is a number from 0 to 999.  TNC type 0 is a KPC-x or a KAM.

Any other number causes a file MSYSTNC.# to be read for initialization

statements to be sent to the TNC.  These parameters would be used for a

TNC other than a KPC-X or a KAM.

NOTE: You must specify RADIOSPEED if you use TNCTYPE#

For example:

PORT 2 AT $2F8 INT3 SPEED 4800 NAME 145.01 RADIOSPEED 1200 TNCTYPE 1

IN THE MSYSTNC.1 FILE YOU WOULD HAVE THE COMMANDS THAT  WOULD

PUT YOUR TNC INTO KISS MODE. (see note in general info section)

DIGI callsign FROM [PORT] number TO [PORT] number [AS callsign]

The DIGI statement defines a digipeater callsign.  Repeat this

statement type up to a total of 25 times as desired.  The first

callsign is the callsign that would be used in the VIA list by a

station trying to use the digipeater.  The port number following FROM

indicates the port that will be using this call.  The TO port number

indicated the port on which packets having this the first callsign as

digipeater should be output on.  For a normal digipeater the two port

numbers will be the same;  for a gateway they will be different.  The

optional AS callsign specifies a replacement for the first callsign in

the digipeater list when the packet is sent out the TO port.  If

omitted the first callsign is assumed (i.e., no change).  Consider the

following examples:

MSYS 1.14                                                   Page 18

DIGI MYDIGI FROM 0 TO 0

DIGI MYDIGI FROM 1 TO 1

DIGI W1XX-3 FROM 0 to 0

This would allow the callsign MYDIGI to be used in the via list on

either port 0 or 1.  The packet is retransmitted on the same port was

heard on.  The call W1XX-3 can also be used to digipeat on port 0.

DIGI MYGATE FROM 0 TO 1

DIGI MYGATE FROM 1 TO 0

This pair would define a gateway between ports 0 and 1 with the

callsign MYGATE used in both directions.

DIGI TOF1 FROM 0 TO 1 AS TOF2

DIGI TOF2 FROM 1 TO 0 AS TOF1

This interesting pair allows TOF1 to be used as a gateway digipeater

call on port 0 (no effect if used on port 1).  When the packet is

repeated it will have TOF2 in the digipeater list where TOF1 originally

was.  The opposite is true on port 1: When TOF2 is used as a digipeater

call the packet is repeated on port 0 with

TOF1 being substituted for TOF2.  The substitution is transparent to

the users.

NUMCHANS number

NUMCHANS is used to specify the number of logical channels that will be

allocated.  Each connection (while connected) uses one logical channel.

When the KANODe is being used a second logical channel is used for

outgoing connects.  Each logical channel requires about 7K of memory

when in use.  They are available for reuse when a station disconnects.

BBS CALL callsign

This statement gives the callsign that can be connected to for the

BBS.  The callsign can contain an SSID.  This statement is also

optional; if omitted stations will not be able to connect to the BBS

unless a BBS alias is given.

BBS ALIAS callsign

This statement gives the callsign that can be connected to for the BBS.

The callsign can contain an SSID.  This statement is also optional; if

omitted stations will not be able to connect to the BBS unless a BBS

CALL is given.

KANODE CALL callsign

This statement gives the callsign of the KANODe which is similar to the

KANODe provided in the Kantronics tncs.  The callsign can (and probably

should) contain an SSID.  This statement is optional; if omitted

stations will not be able to connect to the KANODe.

MSYS 1.14                                                   Page 19

ANSWER CALL callsign

This statement gives the callsign that can be connected to by stations

wanting to talk to the SYSOP.  It is also used as the callsign when the

Connect command is used.  The callsign can (and maybe should) contain

an SSID.  This statement should not be omitted.

FORWARD CALL callsign

This statement gives the callsign used when the BBS is forwarding.  It

can (and probably should) contain and SSID.  This statement should not

be omitted if the BBS is to do forwarding.

ID CALL callsign

This statement gives the callsign that will be used for identification

purposes.  It must be your actual amateur callsign and should NOT have

an SSID specified.  This statement MUST be used.

ID EVERY number

This statement specifies the number of minutes between IDs.  The list

of stations for which there is mail is generated with every ID.

AX25 FRACK number

Number is the number of seconds to wait before retransmitting a packet.

All of the AX25 statement forms are

optional.

AX25 MAXFRAMES number

Number specifies the maximum number of frames that can be outstanding

before waiting for an ACK.  Must be between 1 and 7.

AX25 RETRIES number

This is the number of retries that will be made before abandoning a

connection.

AX25 CHECK number

If there is no activity on a logical channel for this number of seconds

there will be an automatic disconnect.

MONITOR IFRAMES on/off

MSYS 1.14                                                   Page 20

Turns on or off the displaying of Information frames (SABM, DISC, UA).

All forms of the MONITOR statement are optional.

MONITOR SFRAMES on/off

Turns on or off the displaying of System (RR, RNR, REJ) frames.

MONITOR BFRAMES on/off

Turns on or off the displaying of Beacon frames (Un-numbered

information).

MONITOR OUTGOING on/off

Turns on or off the displaying of all outgoing packets.

MONITOR PORTS  [MASK]

Determines which ports will be monitored.  The rightmost bit of this

number corresponds to port 0, the next bit to the left port 1 and so

on.  Ports with 1 bits are displayed (as selected above).  A value of

13 (decimal, $D in hex) would enable ports 0, 2 and 3, for example.

SEE MASK on page 24 for further details.

WINDOW number SIZE number number number number

This form of the window statement gives the bounds of the window whose

number is given after the word WINDOW.  The four numbers after the word

SIZE are the upper left column (1 to 80), the upper left row (1 to 25),

the lower right column, and the lower right row respectively.  All of

the WINDOW statement forms are optional and should be used with great

caution.

WINDOW number BACKGROUND number

WINDOW number FOREGROUND number

These two forms of the WINDOW statement give the background and

foreground colors of the window specified after the word WINDOW.  The

rightmost number is a color number from 0 to 15 for foregrounds and 0

to 7 for backgrounds.  The corresponding colors are:

Black - 0                Magenta - 5              Lt. Cyan - 11

Blue - 1                 Brown - 6                Lt. Red - 12

Green - 2                Lt. Gray - 7             Lt. Mag - 13

Cyan - 3                 Gray - 8                 Yellow - 14

Red - 4                  Lt. Blue - 9             White - 15

Lt. Green - 10

MSYS 1.14                                                   Page 21

WINDOW number PAGE number

This statement assigns a window to the specified page (0-3) which

corresponds to the F!, F2, F3, and F4 keys.

WINDOW UNUSED BACKGROUND number

WINDOW UNUSED FOREGROUND number

These two statements give background and foreground colors to the

places on the screen not assigned to any window.

WINDOW DISPLAY  color/monochrome

SEE SPECIAL NOTE on PAGE 8.

This statement selects the kind of display adapter you have, COLOR or

MONOCHROME.  If you have a CGA card, select COLOR.

PROCESS processname [WINDOW] number

This statement specifies the window that is to be used for output from

the allowable process names .  Be extremely careful if you use this

statement.

STACK processname number

This statement specifies the number of bytes to be allocated for the

stack for the allowable process names .  You probably shouldn't mess

with the defaults!  ***THIS OPTION IS NOT USABLE ON A 286/386 equipped

with EMM or EMS.

DISCUSSION:

First, if using extended or expanded memory, you have no control over

stack size, they are all 8K. Only when just 640K (or less if that is

possible) regular memory alone is used can adjustments be made to stack

sizes.  Normally these should not be necessary.  The default values set

by MUTIL 9 should be correct.  This means that normally there should be

no process stack statements in MSYS.OPT.

How to tell when you need to adjust stack sizes --- If you get

unexpected crashes that can't be solved by other means, do somewhat

frequent process commands when conditions are similar to when crashes

occur (like when lots of channels are in use, when forwarding is

happening, etc.). If the SPmin value for any process gets below a few

hundred an increase in the stack allocation for that kind of process

may be needed.  In the event SPmin goes to 0 or negative, an adjustment

is required.

MSYS 1.14                                                   Page 22

How to adjust stack sizes --- Get a printed copy of the output from

MUTIL 9. You will see there a list of the process type names and

current stack sizes for each.  Add a process stack line in MSYS.OPT for

the one you need to increase. Any increases should probably be done in

1K increments.

Stack sizes should probably not be adjust downward greatly from the

default values even though it appears on most process command lists

that the SPmin is always very high. How much stack space is needed is

dependent on the particular activities that are occurring.  Values that

appear excessive may be required for infrequently performed operations

that if done with small stacks would result in unpredictable results.

(Can you say "CRASH"?  How about "HANG"?)

SYSOP NAME name

You must use this statement to specify your first name.

SYSOP QTH location

You must use this statement to specify your location.

SYSOP ZIP postal code

You must use this statement to specify your zip or postal code.

MAKE

Use this statement if you want to produce the new MSYS.DEF file if

there are no errors.

Here is a sample MSYS.OPT file for you to figure out!

port 1 at $3e8 int 4 speed 1200 name 223.70

port 0 at $2f8 int 3 speed 1200 name 145.01

digi bxng from 0 to 0 as bxng

digi bxng from 1 to 1 as bxng

digi bxn501 from 0 to 0 as bxn501

digi bxn501 from 1 to 0 as bxn370

digi bxn370 from 0 to 1 as bxn501

digi bxn370 from port 1 to port 1 as bxn370

digi test-4 from port 0 to port 1 as test-5

digi wa8bxn-3 from 0 to 0

bbs call wa8bxn-13

kanode call wa8bxn-1

answer  call wa8bxn-14

forward call wa8bxn-12

sysop name Mike

sysop qth Kirtland, OH

sysop zip 44026

id call wa8bxn

window display color

make

MSYS 1.14                                                   Page 23

MASK INFORMATION

Several commands require the use of a MASK to determine the exact ports

you wish to perform the function.  For example, you.  You may wish to

only permit NetRom connects on port 2, or you may wish to have bbs

connects only on port 3, or you may wish to monitor only ports 2 , 3

and 8 on the F2 screen.  The MASK is merely a hexadecimal

representation of the ports.  The BIT PATTERN CHART is the way to

determine what the MASK value will be.

BIT PATTERN CHART

To be used for DPorts, KPorts, MONPorts, NPorts and TPorts

MASK VALUE         PORT NUMBER          CONDITION

6    5    4    3    2    1    0    1=ON, 0=OFF

0    0    0    0    0    0    0    0    All Off

1    0    0    0    0    0    0    1    0 - ON

2    0    0    0    0    0    1    0    1 - ON

3    0    0    0    0    0    1    1    1,0 - ON

4    0    0    0    0    1    0    0    2 - ON

5    0    0    0    0    1    0    1    0,2 - ON

6    0    0    0    0    1    1    0    2,1 - ON

7    0    0    0    0    1    1    1    0,1,2 - ON

8    0    0    0    1    0    0    0    3 - ON

9    0    0    0    1    0    0    1    3,0 - ON

A    0    0    0    1    0    1    0    3,1 - ON

B    0    0    0    1    0    1    1    0,1,3 - ON

C    0    0    0    1    1    0    0    3,2 - ON

D    0    0    0    1    1    0    1    0,2,3 - ON

E    0    0    0    1    1    1    0    1,2,3 - ON

F    0    0    0    1    1    1    1    0,1,2,3 - ON

--------------------------------------------------------------

MSYS 1.14                                                   Page 24

When you have more than 4 ports, a 2 digit hexadecimal value is

required.  The following table represents the left-hand digit for ports

4,5, and 6.

MASK VALUE  PORT NUMBER  CONDITION

6    5    4    1=ON, 0=OFF

0    0    0    0    4,5,6 OFF

1    0    0    1    4 - ON

2    0    1    0    5 - ON

3    0    1    1    4,5 - ON

4    1    0    0    6 - ON

5    1    0    1    4,6 - ON

6    1    1    0    5,6 - ON

7    1    1    1    4,5,6 - ON

For ALL ports to be turned on, the MASK value would be 7F, and is

written as $7F.

For Ports 0 and 1 on, the MASK value would be 3, written as $3.

For Ports 2 and 4 on, the MASK value would be 14, written as $14.

For Ports 3 and 5 on, the MASK value would be 28, written as $28.

MSYS 1.14                                                   Page 25

DRSI CARDS

Minimum things that must be done to use DRSI cards:

Put HAPNDRSI.SYS in root (C:\) directory

Put HAPNKISS.COM where it can be found to be executed.

Put DEVICE=HAPNDRSI.SYS IRQ=7 (or whatever interrupt you have the board

set up for) in CONFIG.SYS.

Reboot system.

Put:

PORT 1 AT $0 INT 7 SPEED 1200 NAME *DRSI-0

PORT 2 AT $1 INT 7 SPEED 1200 NAME *DRSI-1

in MSYS.OPT and run MUTIL function 9.

Notes: Change the port numbers to what is appropriate to your system .

The ports on the

DRSI card are referenced by the numbers after the AT $ .  The INT 7 is

taken as comments only (but INT something is required).

SPEED  of the DRSI ports is really set on the DEVICE=HAPNDRSI.SYS

statement in CONFIG.SYS .  SPEED value given on port statement is more

or less only comments.

NAME must begin with *, can be something like *145.01 if you prefer .

Before you start MSYS, you must run HAPNKISS.

That should get you started.  For other configurations and options,

read the appropriate documentation.

MSYS 1.14                                                   Page 26

MSYS COMMAND SUMMARY

The following is a SUMMARY of the commands available at the LOCAL

keyboard for MSYS.  The commands may be abbreviated to the capital

letters.  This list does NOT include the BBS commands that are

available only from the BBS.  Most "LOCAL" commands may be executed

from the BBS by prefixing them with a "/".  Please refer to the MSYS

SYSOP section for more detailed explanation of the commands.

*comment  Enables comment line from user to SYSOP

#msgs/use Displays total number of active msgs and known user on BBS

@home     Designates @ home bbs option

ABortfwd  Aborts forwarding as soon as possible

ACceptmid Accept Message Identifiers

ADDUser   Allows sysop to add a user to bbs

APpend    Used to append a file to a message and other general uses

ARChive   Puts given @BBS in specified sub directory

ARP       Lists the known callsign/IP address

ARP add   Adds digipeaters to TCP/IP stations

ARP del   Removes TCP/IP stations from arp list

ATtended  Turn HF forwarding on and off

AUTODest  Add new DESts for known neighbors

AUTOHold  Hold messages that contain own call in R: lines

AUTOKill  Kill all messages after forwarding

AUTONode  Add new neighbors heard

AX25Backoff [MASK] Provides exponential delay to RNR messages.

AX25L2V2 [port#]  Turns on Version 2 of the AX.25 protocol BY PORT #

BACkuptime     Specifies time to make backup

BADuser   Enables the bad user list

BBs       Enter the BBS as a local user

BBSNode   If on and the node is enabled, the bbs will appear as a node

to the network

BBSTimeout#    Number of seconds of inactivity on bbs to disconnect

user

BDigi     Specifies digipeaters for beacons

BElloff   Turns bells on or off (on turns bell off.  Off turns bell on)

BIDall    Manipulates bids in the BIDLIST.DAT file.

BINterval (seconds) Nodes Broadcast interval

BMaxcon [port#]     Maximum BBS connects

BOOT      Reboots computer. (Remote sysop only)

BPorts    Used to indicate BBS ports only

BText     Specifies one line of beacon text

BUDCalls  Set or display the list of calls with SSID to be monitored

CHAnstat  Displays status of a logical channel whose number is given

CHEck     Number of 1 second intervals of no activity to disconnect

CL B #    Change border color

CLrscrn   Clear window number given or * for clear all windows

CMdwindow Specifies the window # to be used for commands (CM 4)

COPy sourcepathname destpathname   (include drive: if needed)

Cscript   Another form of the Connect command:

CTS       Allows specification for each port if hardware hand shaking

should be used

MSYS 1.14                                                   Page 27

CRetries  Sets number of retries for forward connect attempt

C#        Attempt to connect to station via digi list given on port #

DEADletter     Used as default for unknown @BBS

DEBug          Used to display or modify memory

DELEfile       Specifies file name to delete

DELUDays       Specifies the number of days after which a user that has

not connected will automatically be deleted from the

system.

DELUSer        Deletes a user from the user list

DESt ADD, REP, DEL   Adds replaces or deletes destinations in node

list.

DFree          Displays amount of disk space free

DIGipeats      Displays digipeater/gateway list

DIRectory      Displays directory for path specified Example:

DIR mail\*.*

DISconnec      Forces a disconnect on logical channel specified

DOcmdfile      To execute a command file

DPorts         Allows sysop to selectively disable digipeater

DRoute         Callsign port# [digi1...digi8]

DTimedate      Displays current date and time

DUmpbbs        Disconnect any bbs that has connected and uses a command

other than S or F.

EDit      Screen editor for files

EFile     Edits the specified file

EMsghdr   Edits the header of the specified msg number in BBS

EUser     Edits specified user

F2 time   Number of seconds to continue displaying monitored packets

after F2 (function key) was last pressed.

FBbStimes Allows specification of hours bbs is full function

FBBC      Sets the FBB Compatibility mode

FBBG      Sets the FBB forwarding group size

FCall     Finds the logical channel(s) in use by given callsign

FFile     (find file) FF *.DAT C:\

FIle      Converts specified msg number into file name given.

FMsg      Searches all messages in MAIL.BAK for given pattern string.

FOrward   Initiates forwarding immediately

FRAck     Displays or sets time in seconds to retransmit a packet

FREE      Deallocates the logical channel specified

FSize     Displays the file size for the pathname specified

FTIme     Forward time

FTPgm     File Transfer Protocol

FUlldup   Used on the specified ports if set to ON.

GEtmsgotd Reads and displays Message of the day file

GRep      Will search the given file for the specified pattern

HApnstat  Displays HAPN/DRSI statistics.

HCAll     Allows you to properly specify your bbs hierarchical

callsign.

HCLean    This command automatically deletes specified messages.

HEard #   The J heard list recording is turned off if # > Dispatches

Help      Lists the names of the commands

MSYS 1.14                                                   Page 28

HOLdlevel Parameter is a number which if the sum of word weights

matched in message exceeds the value the message is held.

HOStname  Hosts id name TCP/IP

HReplace  Selects adding to end of existing hierarchical @BBS field or

replace.

ID *      Forces transmission of an ID immediately (also mail for list)

IGnore    Calls in budcalls list will not be monitored

IPNCheck #     Inactivity timeout in seconds.

IPNFrack #     Retry timer in seconds.

IPNRetry #     Retries, default 5.

IPTtl #   TCP/IP Time to live

INitpr    Allows you to specify if printer should be initialized before

each file is printed.

JBbs      Lists (P)BBSs heard and their paths

JDigipeat Lists Digipeaters heard and their paths

JGateways Lists Gateways heard and their paths

JHeard    Lists stations heard and those connected to BBS

JKanodes  Lists KA Nodes heard and their paths

JMsys     List other MSYS systems heard

JNetrom   Lists NET/ROM nodes heard and their paths

JTcp/ip   Lists TCP/IP stations and their addresses

KEyboard  Assign keyboard to channel number specified

KILLproc  Abort specified process number (Can be hazardous!)

KPorts    Allows sysop to selectively disable k NODe

LCount    Counts number of lines, characters, and words in file. (LC

Filename)

LOCKkbd   Locks the keyboard.  OR unLOCKs keyboard if locked.

LOGclose  This command closes the log file, renames it to form of

yymmdd.LOG and then opens a new log file.

LUsers    List known user calls that have connected to BBS

LU xxx    Selects: BADusers, BBSystems, EXPert, LIMited, NEW, LOCked,

RESet, XFAst_set

NOTE: Use only the first 3 letters of the item from the list

for xxx!

MAKEPrivate    Sets/resets making messages private by default

MAStersrv      Specifies the master White Pages server (default is

AD8I)

MAXBullage     Used to discard bulletins older than a specified amount

MAxFrames      Display or change number of frames sent before ACK rcvd

MAXRead        Specifies maximum size message that can be read or file

that can be downloaded

MCForward [MASK]  Ports where forwarding is counted as bbs connects.

MCOn      Number of connects allowed to sysop keyboard

MDir      Creates given directory

MEMory    Display amount of memory free

MERge     Reads MSYSFWD.DEF and copies it to MSYS.FWD

MEXpert   [On/OFF] If on, display the message of the day

MHclear   Clears various J heard lists

MIDchar   Message ID indicator (max 4 chars)

MSYS 1.14                                                   Page 29

MINmem    Allows specification of the minimum free memory

MISsing   Reports number of times forwarding aborted due to missing

text file

MONBframe      Enable/disable monitoring of Beacon (UI) frames

MONCC          Enables/disables monitoring of TCP/IP packets

MONCF          Enables/disables monitoring of NetRom packets

MONIFrame      Enable/disable monitoring of Information frames

MONITor        Enable/disable monitoring of incoming frames

MONOutgoing    Enable/disable display of outgoing frames

MONPorts       Select which ports to monitor [MASK]

MONSFrame      Enable/disable monitoring of Supervisory frames

MONUFrame      Enable/disable monitoring of Un-numbered frames

MONxx          Enable/disable monitoring of PIDs on screen F2

monitoring

MSgtrailer     Enable/disable message trailer on messages read from

bbs.

MYAlias        Display or change bbs alias callsign

MYBbs          Display or change bbs primary callsign

MYCall         Display or change Call/Answer callsign

MYIpaddr       Sets IP address that the system will respond to.

MYKnode        Display or change K NODe callsign

MYTcpcall      Callsign used when sending arp and TCP/IP frames

NDwindow #     Net default window size (#=frames 1-7)

NETBbs         Enable BBS cmd in net NODe

NETCon         Enable connect cmd in net NODe

NETFbbstime    Allows specification of hours NET is full function

NETFBBC #      NET/ROM FBB Compatibility Flag ***NEW VERSION 1.14

0=No FBB Compatibility (Default)

1=FBB Non-Compressed Compatibility

2=FBB Compression Compatibility

NETFBBG        NetRom FBB forwarding group size (0-5) **NEW VERSION

1.14

NETNode        Enable the Net Node

NETOnly        If on, only BBSs can connect to bbs via network

NETRead        Specifies maximum size message that can be read or file

that can be downloaded through the Netnode

NETTalk        Enable Talk cmd in net NODe

NEWUser #      Allows you to set the initial user flags

NInfo          Gives net NODe status

NNode          Takes you into the NODe as if you were connected to it.

NOBEacons [MASK]   If a given port is set to 1 then there will be no

ID, mail for, or BText beacons.

NOBUlls        When on, no bulletins will be forwarded during normal

forwarding.

NODe SEnd, SAve, REStore, ADD, REP, DEL

NODETimeout #     Number of seconds of inactivity on network node

NPorts [MASK]     Ports to which nodes broadcast

NRCall callsign   Sets net NODe callsign

NRId mnemonic     Sets net NODe mnemonic identifier

NTtl #            Network time to live (max hops)

OCInit #          Initial obsolescence count

OCMin #           Minimum obsolescence to broadcast

MSYS 1.14                                                   Page 30

PAClen    Specifies maximum number of bytes that are put in a packet

PATh      Display information about given user callsign

PForward  Display information about given BBS callsign

PIng <Destination IP address or a mnemonic> Determines Round Trip Time

to destination

PMessage (Message#)    Prints specified message

PC        Enables/disables callsign server ***NEW VERSION 1.14

POrts     Display information about ports

PPersist  [port#] value (255 to 0)

PQuality  [port#] # Quality for nodes heard on port

PRInt     Prints a file (on printer)

PROcesses Display status of processes

PRTmsgs   Enables/disables printing of messages

PWindow   Sets window to be used by a process

QUIT      Terminates execution of MSYS gracefully (more or less)  Also

done with Ctrl F4.

RDir      Removes the given directory

REadbackup #   Allows you to read a message from the /MAIL.BAK

directory

REMove DESt callsign     Deletes DESt through all neighbors

REName    Oldpathname newpathname (must both be on same drive)

REPlaces  Replaces incoming @BBS calls

RESPonstime #  Number of seconds to wait before sending out a response

RESTore <msg#> Restores a message from the mail.bak directory

RETries   Displays or sets number of retries before disconnecting

REQDIR    Enables/Disables Request directory server

REQFIL    Enables/Disables Request File server

REQMaxfile     Used to set a limit on the size of files that may be

requested from the REQFIL server

REQQTH    (ON/OFF) enables Request QTH server

RUn       Allows you a DOS program with many limitations.

SCreensave #   Enables/Disables screen saver.

SEnd      Sends a line of text to specified channel: SE 1 <The msg>

SLottime [port#]   value (0 to 255)

SMTP      Simple Mail Transfer Protocol

SNow      Enables/disables changing screen only during retrace

SOrt      Will sort up to 200 eighty byte records

STatus    Displays some status information

SUspendtrace #    Number of free bytes in input buffers below which

monitoring is turned off.

TABs      Enables/Disables tabs in the bbs.

TACk      Transport ACK delay (RESPonsetime)

TBdelay   Transport busy delay

TElnet    Used to establish keyboard to keyboard connection

TFrames   Send test frames

TImezone  Allows entry of your time zone

TNatime   Transport no activity timeout time

TOdo      Causes the MSYSTODO.DAT file to be scanned immediately

TPorts    Enables/disables ports for TCP/IP

Trace25   Enables/disables trace of AX.25 state changes

TraceIP   Enables/disables trace of TCP/IP state changes

MSYS 1.14                                                   Page 31

TREtries  Transport retries

TRUncate  filename Adds CR/LF to files as needed

TS (hhmmss)    TimeSet command - sets DOS time to given value (must be

exactly 6 digits)

TTimeout  Transport timeout (Net FRACK)

TXDelay   [port#] value (0 to 255)

TXTail    [port#] value (0 to 255)

TYpe      Allows you to type a file

Users <file_name> Writes out all users to file <file_name>

VErifysysop Enables/Disables Sysop verification when connected to BBS

VIdeo     Displays or changes video monitor in use

WAtch     Toggles watching of specified channel number

WBackgrnd Sets background color for specified window

WColors   Gives list of colors and their numbers

WForegrnd Sets foreground color for specified window

WHO       Displays current users/messages

WHY msg#  Displays why message was held.

Window    Displays current window definitions

WPAge     Sets page number to display specified window

WPEvery   Specifies the number of days between sending out WP

information.

WQupdate  Worst quality DESt route to record

WRecord #  (window record) which sends all output from specified window

to a disk file.  Useful for debugging.

WSize     Specifies coordinates for specified window

W?        Displays window number in upper left of each window

<Esc>     Cancel partially typed command

Function key usage:

F1 through F4   Select page to display (0 to 3 respectively)

F6        Causes the following characters to the next return to be

treated as a command even if keyboard currently connected to

some other process (such as local bbs, etc.).

F8 key    Used to blank the screen.

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands to

retrieve/edit last sysop command typed.

Ctrl/F2   Suspends activity on the monitor screen (F2).

Ctrl/F4   Terminates MSYS as if QUIT command was used

Ctrl/F5   Same as the PRocess command

Ctrl/F6   Assigns keyboard to command processor

MOST COMMANDS GIVE INFORMATION ON THEIR USAGE IF

YOU TYPE THE COMMAND NAME FOLLOWED BY A QUESTION

MARK.

Example: WS ?

MSYS 1.14                                                   Page 32

MSYS SYSOP Commands

*comment

Not really a command as such, * may be used at the beginning of a

line making anything else you type on the line a comment. Useful

in MSYS.DO to temporarily remove commands.

#msgs/use

Displays total number of active msgs and known users on BBS . The

number of messages that can be on the bbs is set by the Mutil

Function 1 program.  The maximum number of users is 500.

@home (ON/off)

If On, causes @bbs field of messages for registered users of the

system to be replaced with the home BBS they gave with the NH

command (the way it has been done in the past) . If Off, the @BBS

field is not modified.

ABortfwd

Aborts forwarding as soon as possible (not always immediately) .

You may want to DISC the channel forwarding is using (particularly

if its trying to connect) to speed things up.

ACceptmid (On\OFF)

If ON message ID string found on S command will be used if present

. If OFF or MID is missing, one will be generated if BIDall is on.

Applies only to non-bulletins.

ADDUser callsign

Adds a user callsign to the user file without that station having

to connect to the BBS . This can be useful in conjunction with the

automatic @BBS generation . After the new user has been added you

automatically will EUsers for the callsign at which time you can

fill in the desired fields.

APpend Sourcefile Destfile

This command causes the source file to be appended to the

destination file.  This command may be used to append a file to a

message file, among other more general uses.

ARChive @BBS filename

This command will concatenate all current msgs with @BBS given to

FILES/filename . No compression is done . Example: ARC RLIBBS

RLINOTES/MSGS   (This assumes you have a RLINOTES subdirectory in

the FILES directory)

ARP

Lists known callsign/IP address correspondence used for TCP/IP

connects.  ARP stands for Address Resolution Protocol.

ARP CLEAR

Erase all of the ARP entries.

MSYS 1.14                                                   Page 33

ARP ADD callsign port# ip-address[/#] [digi1 [digi2 ...[digi8]]]

Allows you to manually add (or put it in MSYS.DO) a station (and

up to eight digipeaters) to the ARP list.  Periods are optional in

the IP address.

Examples:

arp add WA8BXN 0 44 70 4 8 NO8M

arp add N8HSP 2 44.70.4.10 WB8CQR-5 WB8APD

You can put a / and a number after the IP address . For example,

you can say:

arp add W3AAA 1 44 80 1 1/24 K8AAA

This says that any IP frames going to an IP address with 44 80 1

in the left 24 bits should go to W3AAA (through K8AAA as a digi).

Thus the number after the / is the number of significant bits to

keep in comparisons.

ARP DELete callsign

Removes a single entry from the ARP table.

Example: ARP DEL K8EIW

ARP REP callsign port# ip-addr[/#] [digi1 [digi2 ... [digi8]]]

Replaces existing arp entry for given callsign with new

information specified.

ATtended [ON/off]

When set to ON all forwarding is allowed.  If off, ports with

radio speed of 300 will forward only those messages in which the

from call is the idcall (messages that you originate) or the to

station in the message is the station you would directly connect

to for forwarding.  HF reverse forwarding is also inhibited if

ATtended is OFF.  Note that the regulations appear to allow you to

receive messages unattended .  So if you don't have a STA for HF

turn AT OFF when you aren't there and remember to turn it ON when

you are .  To be on the safe side, put ATtended OFF in your

MSYS.DO file should the system reboot when you aren't  present.

AUTODESt (on/OFF)

When on and network node is enabled allows automatic updating of

network node destination tables when node broadcasts are heard

from neighbor nodes.

AUTOHold (ON/off)

This command specifies what should be  done with messages that

contain in them R: lines with your  Hierarchical BBS callsign (see

HCA command).  This normally  indicates that the message has

passed through your system before and  probably is  in a routing

loop .  Currently such messages are held .  AUTO- Hold ON means to

do this .  If AUTOHold is OFF the messages will not be held.

AUTOKill [on/OFF]

When on, non-bulletin messages are automatically killed after they

have been forwarded.

MSYS 1.14                                                   Page 34

AUTONode [on/OFF]

If on and network node is enabled, node broadcasts heard from new

neighbor nodes are automatically added to the list of neighbor

nodes.

AX25Backoff [MASK]

A 1 bit in a given port position in the mask causes the FRACK

value for the port to be multiplied by the retry number to

determine how long to wait before retransmitting packets that

haven't been acked.

AX25L2v2 [port#] [on/OFF]

AX.25 version 2 protocol: Version 2 can be enabled for all ports

or specified ports.  Version 1, the default, may be better for HF

operation.

BADuser [on/OFF]

When ON, the calls in the BUDCall list will be totally ignored by

the system.  Such stations will not be able  to connect to the

system in any way (AX.25 or TCP/IP) and will not be digipeated.

Do NOT use SSIDs in  this list so that the listed calls can be

used to match any incoming SSID the station may use.  Try to avoid

using this facility unless really necessary as it does add some

overhead in the processing of every packet received.  Hopefully

stations that misbehave will realize that they can be turned off

and not persist in their activity.

The BADuser command is more inclusive than the Bad User Bit which

can be set for a given user. Setting the bad user bit only

prevents that user from using the BBS.  There is relatively little

overhead in supporting the bad user bit since it is only checked

when connects attempts to the BBS are made.

Putting calls in the BUDCall list and turning on BADuser has a lot

of overhead since every incoming packet has to be compared with

each call in the BUDCall list.

BBs [window#]

Enter the BBS as a local user this command gets you into the BBS

as if you had connected to the BBS, with a few minor differences.

Of course while you are using the BBS other stations can still

connect as usual. When in the BBS you use the BBS command set

(A,B,C,D,G,H,I,J,K,L,M,N,P,R,S,U,V,W,X,Y,?,*).  If you use the L

command you will see all messages including the private ones.

Likewise you can Read and Kill all private  messages.

MSYS 1.14                                                   Page 35

You can include the contents of a file in a message you are

sending by typing:

\+\ filename

at the point you wish to include the file. If you want to include

the contents of a message file from the MAIL.BAK directory, type:

\+\#msgnumber

at the point you want to include the file. Example:

\+\#1538

You can specify a window number as an argument to the BB command.

BB 0 will use the top half of the F1 screen for example.  If you

were in the BBS locally and leave it for some reason with it still

active and use the BB command you will return to your previous

session rather than starting a new one.

BBSNode [ON/off]

If on and the node is enabled, the BBS will appear as a node to

the network (using the alias as the identifier and the BBS call as

the call).  This way connects may be made to the BBS directly from

the network.

BBSTimeout #

Number of seconds of inactivity on BBS to disconnect user, default

300 seconds.  Note that when version 2 of the AX.25 protocol is

enabled, no activity for CHEck time does not causes a disconnect.

BDigi [digi1 [digi2 ... [digi8]]]

Lets you specify up to 8 digipeaters for beacons (id, mail,

btext). The list of digipeaters can be different for each port. If

only a port number is given, the digipeater list for that port is

cleared.

Examples:

bd 4 digi1 digi2  sets digi1 and digi2 as digipeaters for

port 4 only

bd digia digib    sets digia and digib for all ports

bd 5              clears digipeaters for port 5

BElloff (ON/off)

On makes it quiet, off is default which enables all speaker

sounds.

If BElloff is negative, keyboard connects and normal bell still

work.  If BElloff is positive, no bells will sound.  If BElloff is

zero, all bells work.

BIDall

Manipulates bids in the BIDLIST.DAT file.  The following operands

may be specified:

+ bidstring    Adds given bidstring to bid file

- bidstring    Deletes given bidstring

= bidstring    Tells if given bidstring is present

#                   Tells number of active bids/mids

ON             Enables bids for non-bulletins

OFF            Disables bids for non-bulletins

MSYS 1.14                                                   Page 36

BINterval number

Network node broadcast of known destination nodes is made every

number seconds.

BMaxcon [port#] #

This command limits the maximum number of connects to the BBS on

each port.  BBS connects are exempted from the limit but count for

other users.  Normal users get a BUSY response when over limit.

BM 1 allows 1 user  per port. If a port is marked BBS ONLY (see

BPorts) the BMax value  specified limits the number of BBS

connects.  For non-BBS only ports, the BMax value does not effect

number of BBS connects. See also the MCF command.

BOOT

Boots computer. You should have an autoexec.bat file set up that

runs MUTIL 4 and then MSYS so that after you reboot the computer

MSYS will be restarted. Note that this is an abrupt termination of

MSYS. Files azre not closed for example. The correct reply to the

question asked by the boot command is Yes (CASE IS IMPORTANT).

BPorts [MASK]

This command is used to indicate which ports are BBS ONLY

connects.  A One in the corresponding bit  position makes a port

BBS ONLY.  For example, BP 5 would only allow BBS connects on

ports 0 and 2.   The default is no BBS-only ports.  If you do use

this command to make BBS only ports, you better be sure to mark

all the BBS stations that you want to forward to you on BBS-only

ports with user flag of 2!

BText [port#] text

Lets you specify 1 line of up to 80 characters of your own beacon

text.  Can be different for each port. Use with great restraint!

BUDCalls [callsign1 [callsign2 ... [callsign10]]]

Set or display the list of calls with SSID to be monitored or

ignored. This command is used in conjunction with the  BUDLIST or

BADuser commands.  As operands you give a list of callsigns, with

SSIDs, that are to be monitored or ignored.  The calls should be

separated by blanks.  To list the current setting of BUDCalls,

type the command with no operands.

Examples:

budc w1xx w2xx-1 w2xx

budcalls K9XXX

budc

Note: there can be up to 10 calls.

BUDList [on/OFF]

To enable only those packets with calls in the BUDCalls list to be

monitored use the command BUDL ON.   To monitor packets for all

calls, use the command BUDL off.  OFF is the default setting.

MSYS 1.14                                                   Page 37

CHAnstat channel#

Displays status of a logical channel whose number is given.  This

command requires an active channel  number as a parameter.

Information about the channel will be displayed in response to

this command.  The  information includes the Port number used, the

call of the station connected to and a status byte in hex.

CHEck [number]

Number of 1 second intervals of no activity to disconnect if

Version 1 of the AX.25 protocol is in use, or to send an RR Poll

if Version 2 is enables (see AX25L command). To see what value is

currently set, give the command with no parameters.

CL B number

Allows you to set the border color (used on all screens).  Use the

WC command to see the color list.   Default color is black for the

border.  Any of the 16 color numbers (0-15) may be used.

CLrscrn [number/*]

Clear window number given or use * to clear all windows. If no

parameter is given, then the command processor window is cleared.

CMdwindow number

This command is used to change which window is used by the command

processor.  The default command  window is number 0 which by

default is the upper half of display page 0.  This command is

useful when you  want a larger window (to type a file for example)

to be used by some command.  The new window is cleared

automatically by this command.  You will then get the cmd: prompt.

(page 0 =F1 page 1 = F2 etc.) A good windowk to use is 4 (CM 4)

which is the one you see when you press F4.

COPy

Sourcepathname Destpathname (include drive: if needed) The copy

command copies the specified files and will return an error

message if it isn't able to copy all the records.  Directories and

wildcards can be used.

CRetries number

Sets number of retries that there will be for forward connect

attempt. This value should probably be less than RETries.

MSYS 1.14                                                   Page 38

C script-filename

In addition to the usual C# form (like C0) there is now a connect

command in which you do not specify a port number with the C, nor

do you give the call of the station to which you want to connect,

but rather give the name of a file that contains a connect script

to reach that station.  The files must be in the FWD directory and

can have any name.  The connect script is the same format used in

the forward file except that the first line begins with C rather

the other letters used  in the forward file.  You can thus use the

command C HOME in which case in the FWD directory you would have a

file called HOME.  An example of the contents of this file is

c0 wa8bxn

@olvgn

#120

+to

#180

+to

#300

+to

This example can be interpreted as follows.  In the first line the

C means this is a connect script, port 0 is to be used for the

initial connect, and the ultimate destination is WA8BXN.  The

second line says to make a connect to OLVGN (which is a network

node).  The following.  lines are commands sent to the node, the #

lines are times to wait for responses, and the +to lines indicate

the responses to look for that indicate success.  See the

descriptions for forwarding scripts for more information on these

var- ious option.  Another simple example of what you could put in

a connect script file is

C0 WA8BXN

@WA8BXN V TOMSYS

That's the entire file; it is a simple way to put the digipeater

(TOMSYS) needed to reach the destination station (WA8BXN).

CTS [port#] [on/OFF]

Allows specification for each port if hardware handshaking should

be used with the TNC or Modem.  If on, MSYS will suspend sending

data to the TNC or Modem when Clear to Send is not true.  This

will be quite useful with AEA TNCs which have problems when MSYS

sends packets too rapidly to them. Also useful with most Modems.

Make sure you have the proper wires in the cable between the

computer and TNC or Modem, and that you have your TNC or Modem

configured properly if you try to use this option.

MSYS 1.14                                                   Page 39

C# callsign [Via digi1 [digi2  ... [digi8]]]

This command is used to connect to another station.  The

letter C must be followed by a digit indicating the port number to

be used for the connection.  This is followed by a space.  Next

comes an optional window number to be used for the conversation.

Next comes the callsign of the station you want to connect to,

with SSID if needed.  Following this can come Via and a list of

digipeaters to use, separated by commas.

Examples:

c1 w1xxx

c0 w2xxx v k2xxx

c2 k3xxx-2 v n4xxx-3,a5xxx

When you issue this command the screen to be used for the

conversation will be cleared and you will see a line "Enter

converse for channel # port #".  When connection is made you will

get a message "*** Connection established to callsign".  If the

attempt retries out (determined by the RETries value), you will

get the message "Quit converse for channel # because no connect!".

In this case the default CALL process window may show "***

callsign busy" if this is why the connection could not be made.

Pressing any key before getting one of these messages will abort

the connect attempt and return you to the current command

processor window.  When you are connected to a station, you type

lines to be sent to the other station and will see lines typed by

the other station.  Once you begin typing a line it will not be

interrupted by lines from the other station until you press

return.  To terminate the connection, type ^C, the letter d (^Cd)

and press return.

If you want to save what you are getting on disk, press the \ key

three times and then press return.  You will get the message "***

Capture file opened ***".  All lines you receive will be displayed

as usual as well as being written to the file CAPTURE.DAT.  When

you turn capture on with the \\\ at the beginning of a line, you

can also specify a filename to be used in place of the default

CAPTURE.DAT.  For example you can type \\\ junk.abc to start

capturing your conversation into the file JUNK.ABC.  When you have

already turned capture on during a connection, if you type

something like \\\morejunk.xyz then the current capture file will

be closed and now your conversation will be saved in the file

MOREJUNK.XYZ.  Three \followed by return will alternately suspend

and resume the capturing of lines to disk.  When you disconnect

(or get disconnected) the capture file will be closed.  Note: That

the default capture file name is always CAPTURE.DAT.  These files

can be viewed by using the TYpe command or edited with the EFile

command You could for example connect to some other BBS, read a

message found there after turning on capture and then include the

capture file in one of your own messages.  This procedure is

explained in more detail for the BBs command above.

MSYS 1.14                                                   Page 40

You can also send any file on your system as if you typed it on

your keyboard to the station you are connected to by typing

\+\filename

at the beginning of a line. Example:

\+\ files\stuff

would send the file called stuff from the files subdirectory.

DEADletter (bbscall)

This command specifies a BBS call that will be used for private

messages entered on your system without @BBS and whose home bbs is

not known.  This permits you to forward the messages to a WP

server or another station that enjoys tracking down unknown souls.

DEBug

This command is primarily used during development of MSYS. Its

results will change from release to release and are not further

documented. Although it should not do any damage it will probably

be of little use to the normal Sysop.

DELEfile pathname

This command is used to delete a file.  You must give as a

parameter a single file path name with or without wildcards.  The

parameter can contain a drive specification and/or directory names

for the given file.

Example: DELE mail.bin\*.*

DELUDays number

Specifies the number of days after which a user that has not

connected will automatically be deleted from the system.  Default

is 100 days.  Locked users (see bit list in EUser cmd), BBS call

signs and bad users are not automatically deleted.

DELUSer callsign

This command removes a user call from the BBS user list.  The

callsign given as a parameter must match the a call seen on the

list produced by LUsers exactly (including case).  Use this

command to REMove users that haven't been active lately when you

are getting near the capacity of the list (currently 500 users).

The last time a given station con- connected to the BBS may be

obtained by using the P command while in the BBS. If you give *

for the callsign (as in DELUS *) then most user entries that

contain garbage callsigns or ? for the user's name. This options

is pretty safe to use, but if in doubt make a copy of the MSYS.USR

file if you wish.

DESt ADD [id:]destcall neighborcall neighborport obs# quality

Adds new network destination node which is routed through given

neighborcall using specified port. The obsolescence count and

quality values are stored as well.

DESt REP [id:]destcall neighborcall neighborport obs# quality

Changes obs# and quality for specified destination node

MSYS 1.14                                                   Page 41

DESt DEL [id:]destcall neighborcall neighborport

Deletes route to specified destination node through given

neighbor.  When all routes have been deleted, the destination

itself is also removed from the network node tables.  See also the

REMove command.

DFree [driveletter:]

Displays the number of bytes free on a disk. If no operands are

given the space is for the default disk (normally C:).  If a

letter is given as a parameter, the number of free bytes will be

given for that drive.  Be sure the drive is ready before using

this command. If its not and you get the DOS error message, type I

for ignore.

DIGipeaters

This command displays the list of digipeater definitions and

related information. The output of this command is the same as

using the ID command in the BBS.

DIRectory [path]

Displays directory for path specified Example DIR mail\*.* This

command displays a sorted directory listing.  Only the names of

the files (or directories) are given, not the sizes.  To get the

size of a file, use the FSize command described below.  Names of

directories are enclosed in <>.  If no argument is given, you will

get a directory for the current default directory (normally

C:\MSYS).  To list some other directory, give the path to that

directory and a file name of *.*.  Other file specifications

can be given for a selective directory list.  If you include a

drive specification in the pathname, be sure the drive is ready

before using this command.  Examples:

dir mail\*.*

dir a:*.*

dir files\*.c

DISconnect channel#

This command can be used to force a disconnect on the specified

active channel. If the channel you specify was in the process of

disconnecting when you use this command, then the channel will be

immediately disconnected. Otherwise the channel will begin

disconnecting when you use this command. Use it a second time for

that channel if you want to speed up the disconnect.

The disconnect command can be useful when you want to shut down

the system. Another good use is force a disconnect from the bbs

you are forwarding to after you have used the ABort command to

abort forwarding.

DOcmdfile filename

This command is used to execute the sysop commands found in the

specified file.  A command file will contain the same things you

would normally type as commands.  To execute a command file, type

DO followed by the command file name. Example:  DO cmds.dat

MSYS 1.14                                                   Page 42

DPorts [MASK)

A one in the mask for a given port permits digipeat on that port

DRoute callsign port# [digi1 ... digi8]

DRoute DELete

The first form of this command allows you to specify the TCP/IP

station to which IP frames are routed by default (Default Route).

You specify as parameters the callsign of that station (with SSID)

and the port number used to reach that station.  Optionally up to

8 digipeaters can also be specified to be used to reach that

station.   The second form of this command allows you to turn off

default routing of IP frames.  If you do have a default route

specified, then all incoming IP frames that are not otherwise

handled by existing ARP table entries will be sent to the default

station.  This station will also be used for any server functions

you initiate (telnet for example) to IP addresses that are not

currently in your ARP table.  Care must be taken in deciding if

this command is appropriate to use in your particular situation.

DTimedate

This command displays what the computer thinks the current date

and time is. The time can be changed with the TS command.  To

change the date you must exit MSYS and use the DOS date command.

DUmpbbs [ON/off]

Disconnect any BBS that has connected and uses a command other

than S or F.  A BBS is defined as a station that sends a SID

([...]) line.  This is useful in cases where the two BBB's get out

of sync and message titles/text are being interpreted as commands.

EDit filename

Screen editor for files.  Escape gets to command mode, do help for

command list. See section on EDITVIEW later in this document for

more information.

EFile filename

This command is used to edit an ASCII file with re- cords <= 80

bytes in length.  At least one operand is required which is the

pathname to the file you want to edit.  A second operand may

optionally be given that gives the size of the work area in lines

to be allocated (default is 200).  See the separate documentation

for EDITFILE for more information on the editor.

MSYS 1.14                                                   Page 43

EMsghdr msg#

This command allows you to edit the fields of a message header.

One operand is required which is the message number to edit.  This

command also automatically sets the message length to the actual

size of the associated message file.  This is useful when you have

replaced a message file with some other file as described above.

This command is also useful to change the status of messages to

ALL (or others) that have not been marked as being read (but

actually have been) so that they don't appear in the MAIL FOR list

sent out with each ID.  If you press Esc when editing you will

skip the current and remaining fields and see the header again.

This allows you to bypass having to press return for all the

fields you don't want to change.

If you place a string of characters that begins with a period

after the message number, it will be appended to the existing @BBS

field of the given message.  Thus if the @BBS of msg 123 was

WA8BXN then:

EM 123 .NTSOH

would cause the @BBS field of msg 123 to become WA8BXN.NTSOH.

If you type a blank for the BID field the Bid is cleared entirely

(you still need to get rid of the $ in the type).

EUser callsign

Edits user information. Also displays the meanings of the possible

user flags bits.

F2time number

The number of seconds to continue displaying monitored packets

after F2 (function key) was last pressed.  The default is 600

seconds (10 minutes).  Thus 10 minutes after you last press F2

monitoring will be turned off until you press F2 again.  If you

set F2 to 0 this function will be turned off. There is a fair

amount of overhead in formatting all the information displayed on

the F2 screen so on slower systems it is a good idea to allow the

display to be suspended when you aren't actively looking at it!

FBBCompat [port#] [0, 1 or 2]

This command determines the level of compatibility with FBB style

forwarding.  FBBC 0 means MSYS will forward non-compressed to all

stations as before.  FBBC 1 means that MSYS detects the "F" in the

SID, it will forward non-compressed, but in FBB style groups.

FBBC 2 Means that if a station identifies itself with a "B" and

"F" in the SID, it will forward compressed in groups per the FBB

style format.

FBBGroupsize #  [1-5]

This command determines the maximum number of messages that may be

forwarded out in one group using FBB compliant protocol.  The

maximum group size is 5.  In FBB systems, the maximum group size

is 10K bytes, and the number of messages proposed in the group for

forwarding will be limited by this value.  MSYS does not have this

limitation.

MSYS 1.14                                                   Page 44

FBBStimes [Port#] [time-range]

Allows specification of hours BBS is full function for each port.

Default is 0-23 which is all the time. The format of the time

range is the same as used on the ! lines in the forwarding file.

FCall callsign

This command finds the channel associated with a callsign.  The

single required operand is a callsign without SSID.  All channels

in use by that callsign (with any SSID) will be displayed giving

the Port, process number and name and complete callsign (with

SSID).  This can be useful when you were in the middle of a

conversation with some station (and still connected) but switched

the keyboard to some other process (maybe the command processor)

and now want to get back to that conversation.  Use the KEyboard

command with the appropriate process number to do so.

FFile filenamepattern [starting-path]

When you give a file name pattern and optionally a starting path

location this command will list all files that match the pattern.

The following would find all files that have the extension .DAT on

your C: disk

FF *.DAT C:/

The output from this command gives the date, time, size and

complete path names for all files that match the search

specification.

FIle msg# filename

Converts specified message into a file with name given. This

command removes the R: lines at the beginning of the file it

creates.  If you really want to keep them, use an appropriate

rename command instead.

FMsg pattern-string

Searches all messages in MAIL.BAK directory for given pattern

string.  For example, to find messages that refer to hurricanes

you might type:

FM hurricane

FOrward [callsign]

This command causes forwarding to begin immediately if it is not

currently going on.  You might want to use this command when you

have just entered a new message with an @BBS specification that

you want sent out now.  FO with a callsign of a single station

begins forwarding to that station.  This must match the callsign

(including SSID) found on the F/R/P line in the forward file.

MSYS 1.14                                                   Page 45

FRAck number

This commands allows you to set or display the number of seconds

to wait for an acknowledgment to arrive for a packet that is sent

out.  If it doesn't arrive within the number of seconds specified

the packet will be retransmitted.  The retry count for the channel

will also be incremented.  Making this parameter to small will

congest the channel and cause unwarranted disconnects.  The

default is 6 seconds.  If in doubt, error on the high side! (port

number optional)  All IP (and IP through netrom) frame retry

timing uses a dynamic FRACK value.

FREE chan#

This command allows you to immediately release an allocated

channel.  The process using the channel will see this as a

disconnect.  This command will not be needed very often.

Disconnecting which frees the channel will automatically occur

after the CHEck interval has expired with no activity.  Sometimes

unpredictable results can occur with this command.  It is best to

permit the DISC command do it's thing.

FSize filename

This command can be used to determine the size of a disk file.

One operand is required which is the pathname to the file you want

the size of.  It may include a drive specification and directory

paths in addition to the file name.  Wild cards are not allowed.

Be sure the drive containing the file is ready before using this

command.

FTIme number

This command sets the time in minutes after the hour that

forwarding will start.  If time is negative number such as -5 then

you will forward every 5 minutes

FTPgm host-id

Takes a host-id as an argument (it can be either an IP address in

the square brackets or a symbolic IP address).  It establishes a

connection to the FTP server of the specified system.  FTP is the

File Transfer Protocol used in TCP/IP.

FUllduplex [port#] [on/OFF]

Enables full duplex operation in the TNC on selected ports. This

may be useful for satellite operation or with full duplex

repeaters.  Examples:

FU ON     turns full duplex on for all ports

FU 2 OFF  turns full duplex off for port 2

GEtmsgotd

This command causes the Message of the day file (MSYS.OTD) to be

read and displayed.  The Message of the day is sent to all

stations connecting to the BBS.  You can create and/or change this

file using the EFile command.  Keep the contents short! See also

MEXpert command.

MSYS 1.14                                                   Page 46

GRep filename search-string

Will search the given file for the specified pattern, displaying

lines that contain the given string. Case is ignored in the

search. Wild cards are NOT allowed.

HApnstat

Provides HAPN/DRSI interface statistics.

HCAll string

This command allows you to properly specify your BBS hierarchical

callsign.  This is the callsign put in the R: lines of messages as

they pass through your system.  If you don't give this a value,

your ID callsign will be used.  Hint: if you run more than one

copy of MSYS for some reason and pass messages between the two

systems, make your HCAll slightly different on each.  I might use

for example WA8BXN.OH.USA on one and WA8BXN.OH.USA.NA on the

other.  This will prevent unnecessary holding (or deleting, see

OLdbids) of messages validly passed from one system to the other.

HCLean [number]

This command allows you to automatically delete specified messages

every day at specified time (see MSYS.HCL in index). If no operand

is given HCL shows time in minutes after 00:00 (midnight) that

housecleaning happens. If a number is given as an operand it sets

the time for housecleaning in minutes after 00:00 (midnight).

HCLean NOW

Manually initiates housecleaning.

HEard number

The J heard list recording is turned off if the number of

dispatches per second in the last minute is less than the number

given.  Default value is 3100 which j heard recording.  J Heard

list recording takes quite a bit of computation for every packet

heard.  This command was added to speed things up on slower

computers.  (you might want to use a value of 10) Help  This

command lists the names of the available commands with the

required part of the command name in capital letters.

HOLdlevel number

Parameter is a number which if the sum of word weights matched in

message exceeds the value the message is held.

HOStname string

Sets TCP/IP hostname.

HReplace [ON/off]

This command selects adding to end of existing hierarchical @BBS

field info from BBSTONTS.DAT file (if HRep is off) or replacing

anything past the first part with the information from the

BBSTONTS.DAT file (if HRep is ON).  Note that if some MSYS knows

how to forward from some existing part of the @BBS field, no

changes are made to it.

MSYS 1.14                                                   Page 47

ID [*]

This command will send out an ID on all the ports immediately.

You might use it to see if all your radios go into transmit as

they should.  It also will send out the MAIL FOR list.  The text

of the ID is not displayed at this time, but is shown once during

initialization.  Nothing is displayed by the automatic IDs sent

out (on the F1 screen).  To see what mail is outstanding for local

users, type ID.  To really force an ID to be sent, type:

ID *

IGNORE [on/OFF]

When on calls in the budcalls list will be ignored.

INitpr [on/OFF]

Command allows you to specify if printer should be initialized

before each file is printed.  Set it to ON if you need the

initialization.  If you need to press buttons (to select draft or

letter quality mode for example) that you don't want cleared

before each file, set it to OFF.

IPNCheck number

Inactivity timeout in seconds for TCP/IP connections, default 600.

IPNFrack number

Retry timer in seconds for TCP/IP connections, default 30.

IPNRetry number

Retries for TCP/IP connections, default 5.

IPTtl number

Sets TCP/IP time to live value. This is the number of times a

TCP/IP frame will get relayed before it gets discarded. Default is

16.

JBbs [port#]

Lists BBSs heard and their paths for given port or on all ports if

no port number is given.

JDigipeat [port#]

Lists Digipeaters heard and their paths for given port or on all

ports if no port number is given. JD command screens out invalid

callsigns.

JGateways [port#]

Lists Gateways heard and their paths for given port or on all

ports if no port number is given.

JHeard [port#]

Lists stations heard and those connected to BBS or PBBS  This

command gives a list of the last 10 stations that were heard on

the ports or connected to the BBS or PBBS. If a port number is

given only stations heard on that port are listed. An asterisk

after a callsign indicates that the station was heard through a

digipeater.

MSYS 1.14                                                   Page 48

JKanodes [port#]

Lists KANodes heard and their paths for given port or on all ports

if no port number is given.

JMsys [port#]

Lists other MSYS systems heard for given port or on all ports if

no port number is given.

JNetrom [port#]

Lists NET/ROM nodes heard and their paths for given port or on all

ports if no port number is given.

NOTE for all the J commands: These commands give the PBBS systems,

KANodes and others that have been heard on the ports.  A BBS is a

system that sends out a beacon that contains /B after what looks

something like a callsign.  A KANODe is identified by /N in its

beacon to ID.  For both of these commands the date, time, callsign

and path are given.  This pair of commands is useful to identify

the PBBS that goes with a given KANODe.  The Net nodes are also

included.  The real call for such nodes are given in [ ].  Real

NetRoms get the real call in { }.

JTcp/ip [port#]

List TCP/IP stations heard and their addresses for given port or

on all ports if no port number is given.

KEyboard chan#

The KEyboard command allows you to manually reassign the keyboard.

As a parameter you give the channel number of the process you want

to have the keyboard.  You can get the channel number from the

lower right part of the F1 screen.  To get the keyboard back to

the command processor so you can issue this command either press

^F6 to assign the keyboard to the command processor or press F6 to

temporarily send keyboard input to the command process up to and

including the next return key.  When you press a key, it is placed

in the input buffer of the process to which the keyboard is

logically connected.  Initially this is the command process.  When

you give certain commands (such as BB, PB, C) the keyboard will be

assigned to the new process created for the command.  When the

process is terminated, the keyboard is automatically assigned back

to the command scheduler.

KILLproc process#

Abort specified process number (Can be hazardous!). Never use this

command to kill a permanent process.  Better yet, just don't use

this command! Process numbers can be viewed using the PROcess

command.

KPorts [MASK]

This command allows you to selectively disable the K-node on any

ports.

A one bit for a given port number enables the K-node on that port.

MSYS 1.14                                                   Page 49

LCount filename

This command counts the number of lines, characters and words in a

given file.

LOCKkbd

This command "locks" the keyboard so that any keys pressed are

ignored.  I put it in for two reasons.  First, I am getting RF

into my keyboard from HF radios and it generates random

characters.  Second, my 3 year old son sometimes likes to press

the keys.  To "unlock" the keyboard, simply type "lock" again (no

CR needed this time).  When unlocking the keyboard, "lock" must be

all LOWER case.

LOGclose

This command closes the log file, renames it to form of yymmdd.LOG

and then opens a new log file.

LUsers

Use this command to see who has connected and more importantly how

many users are in the user file since it can hold only 500 users

currently.  LU takes an optional argument to list users flagged as

BAD, EXPert, LIMited, LOCked, XFAst, NEW and BBS system.  Example:

LU EXP

LU RESET clears all new user flags.

MAKEPrivate [ON/OFF]

Sets/resets making personal messages private by default.

MAStersrv hierarchical-address

Specifies the master WP server (default is AD8I.OH.USA.NA and

probably shouldn't be changed).  If MAS is set to NONE, no WP

messages will be sent.

MAXBullage number

This command may be used to discard bulletins older than a

specified amount as soon as they are received.  Default age is

32000.  A reasonable value might be 21 days.  Bulletin age is

determined by the date on the last R: line found in the bulletin.

If MAXBullage is set to a negative number, bulletins older than

the number given (in days) will be held rather than discarded.

MAXFrames number

Display or change number of frames (1 to 7) sent before waiting

for and ACK (RR) to be received. A value of 4 is generally good

for VHF ports. Use 1 or 2 on HF ports. For very good vhf paths and

RS-232 wire links, 7 may be a good value.

MAXRead [port#] number

Specifies maximum size message that can be read or file that can

be downloaded. If no port number is specified value applies to all

ports.

MSYS 1.14                                                   Page 50

MCForward [MASK]

Allows you to control Multiple Connections during Forwarding.  The

operand for this command is a port bit mask.  Default is all 1's

in the mask which allows forwarding to go on as in previous

versions.  If a particular port has its bit set to 0, then

forwarding on that port is counted as a BBS connect on that port.

Thus if you have BMaxcon set to 1 for that port and you are

forwarding on that port, a BBS connect will not be allowed (since

you are already using the port).  Also, if there are already

BMaxcon BBS connections for that port forwarding will be bypassed

for that port.  Careful use of these commands should reduce the

congestion on HF forwarding as you now have complete control over

the level of multiconnection for both BBS and forwarding

activities combined.  In the [MASK] argument you put 1's in the

ports on which you want forwarding to be counted as a BBS connect.

Using MCF along with BM (and BP if you have marked any stations as

BBSs), allows you to limit a given port to a single BBS connect or

forward at a time.  If you want this on port 0, use the following:

MCF $FE

BM 0 1

BP $01

MCOn [number]

Specifies number of connects allowed to the SYSOP keyboard at one

time if MCon 1, only allows an incoming call to the key board to

be answered if there are no other keyboard connections.  MCOn 0

prevents all incoming keyboard requests.

MDir directoryname

Makes specified directory.

MEMory

Displays amount of memory free.  This number may not include the

amount of memory available in the "holes" created by stations

disconnecting.

MERge

This command reads MSYSFWD.DEF and copies it to MSYS.FWD,

expanding any include files ($filename).  A new file is also

produced, MSYS.BBS that is a sorted list of all possible BBSs that

can be forwarded to (including things like ARRL, 44*, etc.).  This

command was added for 2 reasons: previously, if there were any

include files in the MSYS.FWD files they did not get read when a

PF command was used and secondly, the new file is needed for

hierarchical forwarding.  The leftmost part of the hierarchical

route only is used for forwarding and the contents of the new file

were needed for efficient processing.  So, the bottom line is

rename your existing MSYS.FWD file to MSYSFWD.DEF and then use the

MERge command.  Any future changes you make to your forward file

must be done to MSYSFWD.DEF followed by use of the MERge command.

Its called MERge because it merges in the include files.

MSYS 1.14                                                   Page 51

MEXpert [on/OFF]

If on, the message of the day will be displayed to all non-BBS

marked stations.  If off, it will only be displayed to non-expert,

non-BBS stations.

MHclear [letter [port or B]]

Clears the various J heard lists. Letters are HBKDN or G

(Heard, BBSs, K Nodes, Digi's, NetRoms).  Examples:

MH        clears everything.

MH D      clears all Digi's heard

MH D 1    clears Digi's heard on port 1

MIDchar string

Message ID indicator (max 4 chars) to be sent in SID. If non-

blank, MIDS will be sent for private messages when forwarding to

systems that have the same string in their SID (System IDentifier,

the stuff in [ ] when a connect is made to a BBS).

MINmem number

Allows specification of the minimum free memory as reported by the

MEM for which a new connect request will be accepted.  Default is

32000.  Values less than 16000 probably will be disastrous. A

value of 32000 or more is strongly recommended, particularly if

you are using compressed forwarding.

MISsing

Reports number of times forwarding aborted due to missing text

file.

MONBframe [ON/off]

Enable/disable monitoring of Beacon (UI) frames

MONCC [ON/off] - enables/disables monitoring of TCP/IP packets

MONCF [ON/off] - enables/disables monitoring of NetRom packets

MONIFrame [ON/off]

Enable/disable monitoring of Information frames

MONITor [ON/off]

This is the master command for monitoring packets heard.  If this

is set to OFF then there will be no monitoring.  If set to ON then

the types of packets selected will be monitored for calls as

selected by BUDCalls on the ports set by MONPorts.

MONOutgoing [ON/off]

Enable/disable display of outgoing frames

MONPorts [MASK]

This command is used to enable or disable monitoring of the

various ports on the F2 screen.  See the section on MASK on how to

set the mask for the desired ports.

MONSframe [ON/off]

Enable/disable monitoring of Supervisory frames (RR, RNR, REJ).

MSYS 1.14                                                   Page 52

MONUframe [ON/off]

Enable/disable monitoring of unnumbered frames (SABM, UA, DISC,

DM)

MONxx

Specifies any hex value for PIDs to ignore on screen 2 monitoring.

Example: MONCC ON  would enable monitoring of frames with PID of

CC

Note: You must specify exactly two hex digits

MSgtrailer [ON/off]

If on when a message is read on the BBS (except with RN or RE) a

line is sent at the end of the message indicating its number and

who it was from.  (Default is ON)

MYAlias string

Display or change BBS alias callsign. Must be 1 to 6 characters,

no SSID.

MYBbs callsign

Display or change BBS primary callsign. Can have SSID.

MYCall callsign

This command displays your keyboard callsign if there is no

argument, or sets the keyboard callsign (used to connect and

respond to connects) to the call given as an argument (including

SSID).

MYIpaddr ip-addr

Takes four integers each in the range 0 to 255.  It sets the IP

address that the system will respond to.  Example: MYI 44 70 4 6

If MYIpaddress is not specified, or set to all zeros, the TCP/IP

support will be inactive.

MYKnode callsign

Display or change K-node callsign. Should have SSID.

MYTcpcall callsign

Display or change a callsign (with SSID) that will be used when

sending ARP and TCP/IP frames.  It should be the same as the

network node call in most cases.

NBdcast [port#] [ON/off]

If OFF, node broadcasts will not go out on port, but the node will

otherwise operate on the port (assuming it is enabled using NP and

other commands).

NDwindow number

Network default window size in frames. Default is 4. Note that his

has nothing to do with windows on the screen.

NETBbs (ON/off)

Enable/Disable BBS command in network node.

MSYS 1.14                                                   Page 53

NETCon (ON/off)

Enable/Disable Connect command in network node.

NETFBBC number  NET/ROM FBB Compatibility Flag

0 = No FBB Compatibility (Default)

1 = FBB Non-Compressed Compatibility

2 = FBB Compression Compatibility

NETFBBG number

NetRom FBB forwarding group size (1-5 messages per forward group)

NETFBBStime [hourlist]

Allows specification of hours BBS is full function when accessed

through the network.  Default is 0-23 which is all the time. Hour

list has same format as that used in the ! lines in the forwarding

file.

NETNode [ON/OFF]

Enables the Network Node

NETOnly [on/OFF]

This command is used to indicate if BBSs only or any stations can

connect to the BBS via the network.  If ON, only BBSs are allowed

to connect via the network.  If OFF, then any station can connect

to BBS via network.

NETRead number

Specifies maximum size message that can be read or file that can

be downloaded when connected to the BBS through the network.

NETTalk (ON/off)

Enable Talk command in the network node.

NEWUser number

Allows you to set the initial user flags for all new users (see

EUuser command for the bit position meanings).  Thus you can

choose to make new users limited (or I suppose for a closed board,

"bad users") until they identify themselves somehow.

NETYapp [on/OFF]

Enables YAPP (Yet Another Packet Protocol used for binary file

transfer between user and BBS) for connections made to the BBS

through the network.  Users must be enabled with user bit $800 to

be able to upload files.

NInfo

Gives network node status display. Same output as U * command from

in the network node.

MSYS 1.14                                                   Page 54

NNode [window#]

This command takes you into the NODe as if you had connected to

it.  When in the NODe you can issue SYSOP commands if you need to

by putting \ or / in front of them as you can in the BBS.

Particularly useful are the NODe, DEStination and REMove commands

when in the NODe to modify neighbor nodes and DEStination nodes.

You can optionally specify a window number to use (4 might be a

good one).

NODETimeout number

Number of seconds of inactivity on network node or K node at

command prompt to disconnect user.  Default is 300 seconds (5

minutes).

NOBEacons [mask]

If a given port is set to 1 then there will be no ID, mail for, or

BText beacons.

NOBUlls (on/OFF)

When on, no bulletins will be forwarded during normal forwarding.

Useful in clearing backlog of messages, particularly during

earthquakes and hurricanes!

NODe ADD [nodeid:]call port quality [digi1 [digi2 ... [digi8]]]

Adds neighbor network node using given port and quality

NODe DEL call port

Deletes neighbor nodes & its DESts

NODe REP [nodeid:]call port quality [digi1 [digi2 ... [digi8]]]

Changes quality of neighbor network node

NODe REStore

Read net NODe Database from disk. If you are using the network

node, you would probably put this command in your MSYS.DO file.

NODe SAve

Write network node Database to disk.

NODe SEnd

Force known network node destination broadcast.

NPorts [mask]

Ports to which network node broadcasts are made. Default is 0.

NRCall [callsign]

Sets net work mode callsign. It should have an SSID.  Example:

NRC WA8BXN-2

NRId [mnemonic]

Sets net NODe mnemonic identifier for network node.  Example:

NRI 460406

NTtl [number]

Network node time to live (max hops). Default is 64.

MSYS 1.14                                                   Page 55

OCInit [number]

Initial obsolescence count for network node. Default is 6.

OCMin [port#] [number]

Minimum obsolescence to broadcast a destination node in network

node broadcast.  OCM is settable for each port. Default 10

PAClen [port#] [number]

Specifies maximum number of bytes that are put in a packet before

it is sent.  Possible range of values is 1 to 255. If packets will

go through a network node on their way to their destination 236

should be used. On HF a value of 40 is generally good.

PATh callsign

Displays information about given user callsign (same as P on BBS)

PC [on/OFF]

This command enables or disables the on-line Callsign Server.

PForward callsign

Displays information about given BBS callsign (same as PF on BBS)

PIng destination

Where destination is either an IP address like [44.70.4.6.] or a

mnemonic defined in MSYSHOST.NET.  The reply comes in window 2

(where the connect and disconnect messages appear)and gives the

round trip time in seconds.

PMessage message#

This will print the given message.  If you have MAIL.BAK as a

directory then the file MAIL.BAK/MSG###.DAT will be printed.  This

will look nice.  If you don't, then MAIL/MSG###.DAT will be

printed.  This will be only the message text.  This command just

makes it easier to print a message than using the PRInt command.

POrts [port#] [ON/OFF]

Various information is given by this command if no operands are

specified.  Interesting to look at but may not be very useful.

POrt can have operand on/off.  When off a port will not respond to

the id callsign (with any SSID) and will no longer digipeat or be

usable from the KANODe.  BBS will not id unless you use it.  Id

will be very short if it has to id (no text for the UI frame).If

you call a station with the port off this will still work because

incoming packets from that station don't include a connect packet.

The POrts command will take a single parameter (ON or OFF) in

addition to an optional port number.  With just the single

parameter, all ports are turned on or off.  Useful when you want

to shut down the system.  PO OFF allows the current users to

finish while preventing further connects.

MSYS 1.14                                                   Page 56

PPersist [port#] value

Sends number given to TNC 255 to 0 as persistence value. Basically

the higher the number the more likely the TNC will actually

transmit a frame (if it has one to transmit) when it hears the

channel is clear. Better overall channel throughput will result if

all stations use a moderate value of persistence (50 to 100). A

value of 255 says to transmit as soon as the channel is clear.

PQuality [port#] number

Default is 192. Sets quality for network nodes heard on port.

PRInt [filename]

Prints a file (on printer) The argument is the pathname to the

file to print.  Make sure the drive is ready before using this

command.  Make sure the printer is ready (and exists) before using

this command.  Printing is done to LPT1: via BIOS calls.  This

command will queue up to 25 print requests.  PRInt with no

operands lists the files in the queue.

PROcesses

This command displays for each active process its number, program

running for that process, an integer parameter passed to the

process (usually a port or channel number), the least amount of

memory left in the stack for the process (labeled SPmin), and the

window used for output by the process.  Of particular importance

is SPmin if the system does bizarre things.  If this number is

less than say 100 the stack is too small.  If its a negative

number, you have problems for sure!

PRTmsgs (on/OFF)

Enables printing of all incoming messages as they are received.

PWindow number number

If given one parameter this command displays the name and window

used by the given process number.  If two numbers are given as

parameters, the first is the process and the second number is the

window number to assign to that process.

QUIT

After you use QUIT command (or Ctrl/F4) the screen is cleared.  JK

etc. information is saved when you do a QUIT and reloaded when BBS

is started again.

RDir directoryname

REMove specified directory

Readbackup number

This command allows you to read a message in the MAIL.BAK

directory (if it exists).     Example: RE 12345

REMove DESt callsign

Deletes specified destination network node through all neighbors.

MSYS 1.14                                                   Page 57

REName Oldpathname newpathname

Renames a file. Both name must be on same drive.

REPlaces

Reads BBS call/@BBS replace file and stores it in memory. Use this

command every time you modify the MSYS.REP file while MSYS is

running.

RESPonsetime #  Where # is the number of seconds to wait before sending

out a response.  This is the T2 timer value in the AX.25 spec.  If

you set RESP to 0 it will work the old way, always sending out a

response to each I frame received as soon as possible.  With non-

zero values for RESP, the responses are held, the  latest one

replacing any one being held, until no  I frames have been

received for the time given.  It then  sends out its response.

RESPonstime Can be set for individual ports.

RESTore <msg#>  Restores a message from the MAIL.BAK dir to the active

mail directory.  IT DOES NOT maintain the old number, but assigns

a new MESSAGE number.  It WILL maintain the original BID (if any).

It MUST be done from the BBS with the syntax:

/REST <xxx>

RETries number

Displays or sets number of retries before disconnecting. A value

of 5 is good for VHF, 10 can be used on HF.

REQDIR [on/OFF]

Enables Request directory server in the BBS.

REQFIL [on/OFF]

Enables Request File server in the BBS.

REQMaxfile number

Used to set a limit on the size of files that may be requested

from the REQFIL server and also the files that will be displayed

in REQDIR requests.  The default is 5000 which is probably too

high.

REQQTH [on/OFF]

Enables Request QTH server in the BBS.

MSYS 1.14                                                   Page 58

RUn programname

Allows you a DOS program with many limitations.  For those of you

who have asked for some sort of DOS shell capability you may

recall I have said it can't be done.  But some of you said all you

needed to do was output 8 bits to some port or something like

that.  Well if you have something VERY SIMPLE and VERY SMALL and

very QUICK here is something that should work for you.  Note: You

must follow all of the rules given here or very strange things

might happen for which I claim no responsibility.  If you use the

RUn command and it doesn't work it is clearly you who have made

the error!  Here are the rules for programs you might consider

using with the run command (if you don't understand these rules or

aren't sure your program complies with them, don't run it!):

1. Program must be small (I'd say under 10K).  If you get a

message that says not enough memory, the program is too big.

2. Program must be in default MSYS directory.

3. Program must be fast.  If runs much over 1 minute, the system

will reboot.  While it is running no packets of any kind will

be transmitted.

4. Programs should not use any of the "standard" pre-opened files.

Console I/O must be done using ROM BIOS only.  If you open

any files, you must close them.  If you use any memory, you

must free it.  If you change anything in the computer you

must put it back for the most part.

5. Any screen output will be put in a file called $POOL.  The TYpe

command may be used to view the file.

6. You should write your programs in either assembly language or

C.  Here is an example of a C program that is suitable for

framing (I mean execution using the RUn command):

#include <stdio.h>

#include <stdlib.h>

main(int argc, char *argv[])

{

int i;

char *path;

path = getenv("PATH");

for (i=0;i<argc;i++)

cprintf("argv[%d] is '%s'\n",i,argv[i]);

if (path) cprintf("Path = '%s'\n",path);

exit(1);/* You didn't expect this to do anything useful I

hope*/

}

SCreensave number

If the number given is zero (which is the default) the screen will

not be blanked automatically.  The method used to blank the screen

will work with all display cards (the screen content is saved and

then blanks are written to the screen).  When you press a key to

unblank the screen you will always get to screen 1 (the one you

get with the F1 key).

MSYS 1.14                                                   Page 59

SEnd chan# message

Parameters are an active channel number and the text of the

message to send.  The text is prefixed with "MESSAGE FROM SYSOP AT

id-callsign".  Use this to give helpful hints to users you see

that are in trouble with the commands they are trying to use.

SLottime [port#] value

Sends the given value (0 to 255) to the TNC. This value is used in

conjunction with PPersist. For more information consult your TNC

manual.

SMTP

Allows local access to the SMTP server (mainly for test purposes).

The data channel port number for SMTP transfers is not correctly

displayed.  It does not affect operation and may be fixed in a

future release.

SNow  [on/OFF]

If you have a CGA adapter and there is a lot of snow during screen

changes (particularly when changing display pages with F1-F4) and

you don't like it, set SNow to ON.  Otherwise leave it off and the

display will run faster.

SOrt filename

Useful to sort your forward include files in the /fwd directory so

you can find things there more easily.  Will sort up to 200 eighty

byte records.

STatus

I put this one in for my own debugging (like some of the other

information  displays).  The current time is in something like

seconds since midnight of 1900.  If any channels are open you will

learn very little by what you see, but state 5 is the normal

connected state number.  The next to last part of this display is

somewhat important.  FREE is what is currently free in the buffer

used to get characters from the TNC (an interrupt driven  routine

is used).  LEAST is  the smallest amount  of space that was

available.  If least is small (say less than 100) you've got some

real problems.  I don't think I've ever seen it go below 3000.  If

it is small it means that characters are coming from the TNC

faster than they are being processed.  You might try a lower baud

rate between the computer and TNC but this probably won't really

help the situation much.  Dispatches per second is how many times

per second each process was given control of the CPU.  If this

goes to zero I'd worry a lot!  Note: will only display information

for real ports.  The STatus command may be given the operand RESET

which resets the least free values to 4096, the other counters

(over- run, framing and discarded) to zero.  It also copies the

last minute dispatches to min and max.  Also added to the STatus

command is display of the average number of dispatches per second

since MSYS was started.

STatus Interrupts

Displays additional information about interrupts that have

occurred.

MSYS 1.14                                                   Page 60

STatus Channels

Gives status for active channels only

STatus RESET

Clears forward abort msg as well as all counters

SUspendtrace number

Where number is the number of free bytes in the input buffers

below which the monitoring is turned off.  There is 1 buffer of

4096 bytes for each serial port (tnc).  The amount free at any

time is given by the STatus command.  When the free space gets

less than the SU value, F2 monitoring is turned off to allow

things to get processed more quickly.  Default value is 2000

bytes.  Again, F2 displays take a lot of time and when the system

gets behind it is a good idea to turn off the updating of the

display.  When the free space in all the buffers goes above the SU

value tracing is turned back on (unless otherwise suspended).

This function can be effectively disabled by setting SU to 0 and

then it will work as before.

TABs [ON/off]

when on tabs are expanded on local display; when off a tab shows

up as ^I.  Default is ON.

TACk [number]

Transport ack delay in seconds for network node. Default is 3.

Similar in function to RESPonsetime but for network node.

TBdelay [number]

Transport busy delay in seconds for network node. Default is 180.

TElnet host-id [server#]

Takes an IP address (or host mnemonic) and optional server number

as parameters.

Examples:

TE [44.70.4.10]

te eiw    (eiw would have to be defined in MSYSHOST.NET)

NOTE: To allow Telnet access to your BBS, you must modify the

entries in your MSYSHOST.NET file.  Basically you must add the

call (without SSID) in square brackets of those stations that are

to be allowed BBS access.  Example:

44.70.4.5 eiw K8EIW      #this IP address would not have access to

the BBS

44.70.4.32 [k8eiw] don   #this IP address would have access to the

BBS

Note: You can use any case within the [ ], do NOT use any SSID's!

TFrames Port#

Send test frames on a given port each time enter is pressed. Use

Esc key to terminate.

MSYS 1.14                                                   Page 61

TImezone [string]

Allows entry of your time zone.  Set timezone GMT to all caps to

get lower case z on message time R: lines.

TNatime [number]

Network node Transport no activity timeout time in seconds.

Default is 600.

TOdo

Causes the MSYSTODO.DAT file to be scanned immediately, doing

anything it is time to do and also setting the time of the next

thing to do.

NOTE: Be sure to use this command every time you change

MSYSTODO.DAT while MSYS is running!

TPorts [MASK]

This number is used the same way as other similar commands:

When the bit is set to 1 for a given port, that port is available

for TCP/IP use (particularly ARP broadcasts).

Trace25 [ON/OFF]

Enables/disables traces of AX.25 packets.  If you set it on you

will get more stuff that you can't understand faster than you ever

wanted to see it!  Mainly used for my debugging of AX.25 protocol

problems.

TraceIP [ON/OFF]

Enables/disables trace of TCP/IP problems. Again mainly for my

use!

TREtries number

Network node Transport retries. Default is 3.

TRUncate filename

Adds CR/LF to files as needed to make lines no longer than 80

characters so it can be edited with ED command.

TS   HHMMSS

TimeSet command - sets DOS time to given value (must be exactly 6

digits)

TTimeout number

Network node Transport timeout in seconds (FRACK for the network).

Default is 60 seconds.

TXDelay [port#] number

Keyup delay (usually in 10 mS increments) number given can be

between 0 and 255 (its best to err on the high side). This number

is sent to the TNC. For more information, check your TNC book.

MSYS 1.14                                                   Page 62

TXTail  [port#] number

Keydown delay - how long to wait after sending a packet to unkey

the transmitter. The number can be between 0 and 255. This command

should be used with TNC-2 clone TNCs (any tnc that uses hardware

switches to set baud rate going to radio) when on HF.  Without

this parameter the TNC keys up on the transmitter before the

complete packet is sent.  Try a value of 20 as a starting point.

TYpe filename

Allows you to type a file.  Use spacebar to stop/start and a

control Z or A to abort.

Users filename

Writes out all users info to formatted ASCII file

VErifysysop [ON/OFF]

If on then whenever any variation of the id call is used to

connect to the BBS (except from the local keyboard) then 4

characters from the REMOTE SYSOP phrase must be entered.)  NOTE:

THIS MAY NEED TO BE SET TO PREVENT UNWANTED ACCESS BY LESS-THAN-

HONEST (AB)USERS.  The REMOTE SYSOP phrase is in a file called

PASSWORD.RMT in your FWD directory that is the same as the

MSYS.RMT file on the other system.

VIdeo     [COLOR/MONOCHROME]

You can change the monitor you want to use while the system is

running (using COLOR or MONOCHROME as a parameter).  I've got both

connected to my systems here (at the same time) so it is

meaningful to switch back and forth on rare occasions (like to see

if it works).  You probably ought to specify the one you want to

use in your MSYS.OPT file and not otherwise use this command.

WAtch chan#

This command allows you to see all that occurs  in both directions

for the channel you specify.  Watching is turned off when the

channel is closed or you use the WAtch command a second time  for

that same channel.  The lines that go to the screen  are not

labeled in any way, they are just sent to the screen.  If you

WAtch more than one channel at a time it will work but you may

wonder which line came from which channel.  Using watch may have

some side effects like resetting the window being used by the

process using the channel you have selected.  You should probably

not "watch" a forwarding channel. That's a "feature" of this

program and will not be corrected!

WBackgrnd number

This command requires two parameters to set back- ground color:

Window# and Color#.  The color number should be between 0 and 7.

WColors

Gives list of color names and their numbers

WForegrnd number

This command requires two parameters to set fore- ground color:

Window# and Color#.  The color number should be between 0 and 15.

MSYS 1.14                                                   Page 63

WHO

Displays current users and messages.

Window

Use of this command is a good way to see what numbers mean in the

various window definition commands.

WHY msg#

This command will search for the words in wordfile or hold file in

the given msg#.  Useful when you do a LH and want to see why the

message was held.

WPAge number number

This command requires two parameters to set page number: Window#

and Page#.  The page number must currently be between 0 and 3.

Basically this command defines what page a window will appear in.

Example: WPA 5 3

would assign window 5 to page 3 (the one you see when you press

the F4 key). This example assignment, by the way, will not be

particularly useful!

WPEvery [number]

Specifies the number of days between sending out WP information

for users who haven't changed their  information.  Default is 60

days.

WQupdate [number]

Worst quality network node destination node route to record.

Default is one.

WRecord [number]

Window record sends all output to a disk file. The first time use

the command for a given window it begins recording everything sent

to that window in a file called window.dat where N is the window

number.  The second time you use the command for the same window

it turns off recording for that window.  Using the command with no

operands displays the status of recording.  NOTE: This function

produces a lot of disk output quickly.  It can fill up your disk.

It also slows down the system a fair amount due to all the disk

writes.  This command was added for debugging purposes and should

not be used as a general purpose capture command.  You can

determine the window numbers by using the W? command.

WSize number number number number number

Specifies coordinates for specified window This command requires 5

parameters.  The first is the window# being defined.  The

remaining parameters are two pairs of numbers giving the upper

left and lower right corners of the window.  The pair is column

number (1,80) then row number (1 to 25).  Overlapping windows may

produce amusing results.  To specify window 3 as being the entire

screen the following command may be used: WS 3 1 1 80 25

MSYS 1.14                                                   Page 64

W?

Use of this command is the easiest way to find  out which window

is which! It displays the window number of each window in the

upper left hand corner of that window.

Yapp [port#] [on/OFF]

Enables use of Yapp (binary file transfer between users and the

BBS) on selected ports.

<Esc>

Cancel partially typed command

Function key usage:

F1 through F4 select page to display (0 to 3 respectively)

F6 causes the following characters to the next return to be treated as

a command even if keyboard currently connected to some other

process (such as local BBS, etc.).

F8 key can be used to blank the screen.  Pressing any other key will

unblank the screen.  A new SYSOP command is also added that allows

you to specify the number of minutes after last keystroke to blank

the screen automatically:

Alt-F1 Alt-F2 Alt-F3 keys work like DOS F1 F2 and F3 commands to

retrieve/edit last SYSOP command typed.

Ctrl/F2  suspends  activity on the monitor  screen.   ^F2 restores

normal operation.  This is probably as close as  being able to

scroll the F2 screen that I will be implementing.

Ctrl/F4 terminates MSYS as if QUIT command was used

Ctrl/F5 is the same as the PROcesses command

Ctrl/F6 assigns keyboard to command processor

MOST COMMANDS GIVE INFORMATION ON THEIR  USAGE

IF  YOU  TYPE  COMMAND  NAME  FOLLOWED  BY   A

QUESTION MARK.

Example: WS ?

MOST COMMANDS GIVE  THE CURRENT  VALUE SET  IF

YOU TYPE JUST THE COMMAND NAME.

Example: FRAck

MSYS 1.14                                                   Page 65

MSYS BBS USER COMMANDS

A - Abort                N - Enter name/qth

B - Bye                  P - Path to station

C - Conference           R - Read message

D - Download             S - Send Message

G - Search file          T -Talk to SYSOP

H - Help                 U - Current users

I - Information          W - What files

J - Calls heard          V - Version

K - Kill message         X - Expert-Mode

L - List Messages        Y - YAPP Command

M - Message of the day   ?x - Info about command

* - Comment line

The A (abort) command to the BBS.  It can be used to abort output from

most BBS commands, like Download, List and Read.

The Bye command disconnects you from the BBS.  Use it when you are

done!

The C command enters the user into the Conference.  Following lines

typed will be sent to all stations in the local conference.  Control-ZQ

(^ZQ) is used to exit the conference.  When a station enters or exits

the conference, a message noting the event is sent to the stations in

the conference.  Other Conference commands are:

^ZH produces the help text

^ZU Lists the current BBS users

^ZA #  Asks the user on the given channel number to join the

conference

The CC command allows sending a "Carbon Copy" of a given message to

other stations.  Only the originator of the msg, the destination

station, or the SYSOP can do this for a given msg.  Syntax is: CC msg#

call1@BBS1; call2@bbs2 ...  The @bbs parts are optional if the calls

are users of your system.  Example: cc 4725 k8eiw@wb8bii; no8m   An

alternate syntax for this command is: CC msg# <filename Where filename

does not include a file type (.DIS is automatically appended).  This

file contains tocalls (and optionally @BBS) of the stations that should

get the copies.  There is one call per line of the file.  Example:

CC 3195 <locals

The file LOCALS.DIS could contain:

k8eiw @ wb8bii

NO8M

The Download command is used to read a file stored on the system.  The

format of the command is D filename where filename is the name of the

file to download.  To see what files are available for downloading, use

W or WN command.  To get a file from a subdirectory type

d <directory_name>/<filename)>

MSYS 1.14                                                   Page 66

The G command (for grep, a Unix utility) allows you to search a

downloadable file for a specified string of characters, printing all

lines that contain the string of characters.  Case is ignored in the

comparison.  No wild cards (like ? or *) are recognized.  When you use

this command you will be asked for a file name and then a search

string.  For file name enter the name that you would use if you were

downloading the entire file.  If you do not give a file name,

Database.0 will be assumed.  When the command is used, the user will be

asked for the file name to search and then the search pattern.

The default directory FILES\ will be prefixed to the file name the user

gives UNLESS the DOS SET command has been used for filesdir.  Thus

downloadable files may be pattern searched, and in the directory that

the sysop chooses to make available.

The G command is not available when the BBS is in limited service mode.

The GX command permits search on the directory specified by the DOS SET

msys:grepdir (default is FILES).

SEE ADVANCED USERS SECTION for details.

The H command will give you a short description of the commands.  For

more information about a particular command, type ?x where x is the

letter of the command.

The I command by itself gives hardware configuration of this system.

(the information displayed comes from HELP\INFO.HLP)

For example:

This system runs on an IBM PC type computer and is written in C.

It uses TNCs running in KISS mode which allows all the fancy stuff

it does.

Location is Port Huron, Michigan

TNCs KPC-4

PK-232

Frequency Radio Power Antenna

145.03 MOCOM 70 60 W Isopole at 80 ft

145.09 TR-7400A 30 W Isopole at 30 ft

If you have any questions please leave a message.

ID gives a list of the ports and digipeaters/gateways available.

JH command lists stations recently heard on the various ports and

stations that recently connected.  Use the P command for path to

station that have connected at some time.  For the following commands

the sponsor's callsign is given in {curly brackets}.  This is normally

NOT the call you would use.  This callsign is given to help identify

some of the "mystery" callsigns used for nodes and digipeaters such as

"MYDIGI".

JBbscommand gives a list of BBS's that have been heard along with date

and time they were heard and the path to them.

JDigipeat command gives a list of the Digipeaters that have been heard

along with the date and time they were heard and the path to them.

MSYS 1.14                                                   Page 67

JGateways command gives a list of the Gateways that have been heard

along with the date and time they were heard and the path to them.

JKanodes command gives a list of the KAnodes that have been heard along

with the date and time they were heard and the path to them.

JMsys command List other MSYS systems that have been heard.

JNetrom command gives a list of the NET/ROM nodes that have been heard

along with the date and time they were heard, the real callsign of the

NODe and the path to them (most are direct).

You can specify a port number as an argument to limit the display to

just that port.  For example:

JK 1 would display the KA Nodes heard on port 1 only.

The K command is used to kill (delete) old messages from the

system.  You can kill only those messages that are to or from your

station.  The format of the command is K, a space, and then the number

of the message to delete.  Multiple message numbers separated by a

space may also be killed.

Example: k 123 6789 1123.

KM - To delete all messages TO you.

KF - Kill messages that have been forwarded (sysop only)

KT - Msg# to kill NTS traffic you are going to deliver.

K* - (sysop only) Goes through ALL messages, allowing you to kill, keep

or hold any of them.  You can also read a given msg (it must be in the

mail.bak directory).  When reading a msg this way, use the space bar to

pause the display.

K@ - kills all messages with given @BBS.(sysop only) only compares the

leftmost part of the @BBS field (the normally visible part of a

hierarchical address) instead of any part which caused unexpected

deletions.

Example: K@ ARRL

The L command lists selected message headers.  The following format are

available:

L    List messages since you last used the B command

LH   (sysop only) List held messages.  A message gets held if any of

the R: lines contain the idcall of the receiving BBS.  This will catch

forwarding loops.

LM   List messages to or from you (List Mine)

LN   List all messages not yet read

LT   List NTS Traffic messages

LL # List the last # messages

Example: LL 10 - Lists last 10 messages

LO yymmdd      List messages older than date yymmdd given

MSYS 1.14                                                   Page 68

LU   Lists unread messages to you

LY   Lists msgs that have been read but not deleted

LW   Lists msgs with type W (for weather)

L?   (sysop only) List msgs that will not forward

Also reports msgs sent to your bbs for calls that are not

registered as users of your bbs.  For example, suppose someone sends a

msg to W1AW@mybbs.  The msg would show up in a regular L as going to

W1AW and @BBS of --- (what you normally see for messages to local

users).  If W1AW is not a registered user of my system, then such a

message would show up when L? is used.

L< callsign    List messages from callsign

Example: L< WA8BXN

L> callsign    List messages to callsign

Example: L> WA8BXN

L@ callsign    List messages being sent to BBS callsign

Example: L@ WA8BXN

L #  List messages >= given number

Example: L 827

L # #     List messages from # to #

Example: L 50 75

L"string" Lists messages with given string in title (case insensitive)

L'string' Lists messages with given string in title (case insensitive)

Examples:

L'Pfb'

L"ID's"

l"Modem users"

L$   List messages with type $

Example: L$

The L< and L> commands will also take wildcards as arguments.

Note: Messages that are Private are shown only to Sender, Addressee or

SYSOP.

The M command may be used to display the Message of the day.  Normally

this message (if not empty) is displayed when you connect unless you

have selected Expert mode (?X for more info on Expert mode).  If you

have selected Expert mode, using the M command is the only way to see

any Message of the day.  If there is no message of the day, you will

simply get the next command prompt.  The message of the day will advise

you of new features and other important information.

MSYS 1.14                                                   Page 69

N command can be used to register your name or QTH.  You should enter

both of these.  To enter your name type N your name.

Example: N Mike

To enter your QTH, use the command NQ your qth.

Example: NQ Kirtland, OH

To enter your ZIP or Postal Code, use NZ code.

Example: NZ 44026

To enter the primary BBS that you look for your mail on use NH

callsign.

Example: NH WA8BXN

The P command can be used to find the path last used by a station to

connect to this system.  The format is P callsign.

Example: P W1AW

The information given comes from the station's use of the N commands

and information recorded by the system.  Try the P command with your

own call to check the information the system has about you.

PF The PF command can be used to find out if this system knows how to

forward mail to a particular BBS (what you would enter as @bbs in

the S command).  The port number and call of the BBS to which the

message would be sent is displayed along with the VIA list used for

the connect.  If NetRom or KA Nodes are used to connect to the BBS

to which the message would be forwarded to the fact is noted but the

details are not given.  If this system does not contain the

requested BBS in its forward file you get a message that the

callsign is unknown.

Example: PF WA8BXN

The R command is used to read messages that appear when you do a L

command.  After the letter R and a space you should enter the

number of the message to read or multiple message numbers

separated by spaces.  Example:

r 725 1012 892.

RE # (for read export) This causes the given message(s) to be displayed

with a S command line, followed by title, message text and then

/EX in place of the normal way messages are displayed.  If this

form of output is saved using the output to file command [<

filename] you can easily export messages to an ASCII file.

RH <msg#>  Gives complete forwarding information including R: header

lines.

RM   Read unread messages addressed to you.

RN <msg#> This is like the R command but no headers are printed, just

the message text.  Good for marginal connections when you

otherwise know the message number you want to read.

RP   Read Preview.  It does a normal read operation but does not mark

the message as having been read so it will show up if you do a LU

command.

MSYS 1.14                                                   Page 70

RS   Reads all mail for SYSOP.  Note that when you read mail for SYSOP

(ALL CAPS) it gets changed to mail for Sysop so that it won't

continue to show up as a new msg.

R+   (Sysop only) Goes through all the held msgs using the hold file

trying to identify why each was held and allowing you to continue

to hold, kill, or release as private or bulletin each msg.

R*   (sysop only) It allows you to read all held messages.  After each

is read you are prompted to enter H to continue to holding the

message, K to kill, N to release as private, or $ to release as a

bulletin.

REPly     The REPly command is used to generate a reply to a message

without having to do a S command.  The advantage of using the

REPly command is that it will usually pick up the correct @BBS

(often with proper H route) from the R: lines in the message being

replied to.  You can use either:

REPly msg# (example: REP 1234) or

REPly msg#.  (example: REP 1234.).

In the first form you will have to supply the title.  In the

second form, a title is automatically generated of the form RE:

the-original-title Messages that passed through SMTP systems will

probably not generate the correct reply address.

S command      format: Sx tocall @bbs

Where x is message type (P=private, B=bulletin, T=traffic)

Tocall is DEStination callsign (without - number/SSID)

@bbs is DEStination BBS (again without any - number) [OPTIONAL]

---> If @BBS part is omitted, if the station has entered a Home

BBS (NH command) that BBS will be used.  Otherwise the tostation

callsign will also be used as the @BBS.

Example: S W1XXX @W0YYY

You will next be asked for a title for the message.  After this

you type the message, keeping all lines less than 80 characters.

When you are done with the message type ^Z or /EX on a new line.

If you type ^A in place of ^Z when you are done typing a message,

the message will not be stored.

Users who send messages to stations without giving any @BBS or

giving unknown @BBS are warned of the fact.

Use SB to send bulletins, ST to send traffic, etc.  All messages

sent to a specific callsign will be treated as if you used SP

(send private).  To make messages to a specific callsign readable

by everyone use SB instead of just S! Or make it to a non-callsign

(such as ALL).

The ST Command will NOT add a BID to messages.

MSYS 1.14                                                   Page 71

Notes about the @BBS:

Automatic generation of @BBS field for S command in BBS:  If the S

command contains an @BBS entry it is left untouched.  If there is

no @BBS as a parameter for the S command, the following happens.

If the to station is found in the user file then the home bbs of

that station is substituted for the given @BBS.  If the to station

is not a known user then the to station callsign is placed in the

@BBS field.

Any messages entered into the system with no @BBS that don't get a

@BBS from the user file will have ==== for the @BBS.  Only

messages with second type character of N will cause the user file

to be searched for @BBS.  Thus bulletins will never get their to

call sign (typically ALL) looked up in the user file.

In the bbs, if a S command has an SSID in the @BBS filed it is

removed in the leftmost part of the hierarchical address.  Thus  S

W1XYZ @ K1XXX-5 is taken as S W1XYZ @ K1XXX.

T    The T command (talk to sysop).  When a bbs user types T you will

get the connect music and then have 60 seconds to type something

or they will be put back to the bbs.  If you do type to them then

when you are done with your conversation and do ^Cd then they will

go back to the bbs.  The keyboard call can still be used for

incoming connects; note that it is accessible to all stations,

even on closed bbs ports.  Outgoing connects are the same as they

have always been.  When an incoming keyboard connect occurs, you

will automatically be taken to the F1 screen

U    The U command gives the current users of the bbs plus the number

of active messages and users and what they are doing.

UPload

The UPload command To use it, the user must be authorized (see

user flags ).  To do an upload, the user types UP on the BBS.

MSYS will then ask for the file name to be uploaded.  If no file

name is entered, or the file exists (in the FILES directory) the

upload is terminated.  If the filename is accepted, the user then

sends the ASCII file, ending with a line that contains only ^Z.

Once the transfer is started, it may be aborted by sending a line

with ^A.

Version

The V command gives the version of this software and date and time

MSYS was started.

W    The W command lists the available files for downloading.

Examples:

W    Lists all files in C:\MSYS\FILES

MSYS 1.14                                                   Page 72

W (Dir1)  Lists all files in the (Dir1) subdirectory of

C:\MSYS\FILES\(Dir1).

The directory list provided includes the dates & sizes of the

files.

X    Command can be used to set or reset Expert status and also to set

the number of lines sent before pausing during an R command.  If

no argument is given, X simply toggles and displays the new Expert

status.  When Expert status is set, the initial connect messages

are replaced by a compatibility identifier line ([MSYS-1.14-BFH$])

and the command prompt is simply >.   If a number is given after

X, this is the number of lines to display for a R command before

asking More?  To continue displaying the current message when

More? is sent, press return.  To abort the message, type N and

then return.  This facility has been added to prevent a long

message from scrolling off your screen faster than you can read

it.  Experiment with different values until you find one that

pleases you.  To not use this facility, set the number of lines to

zero.  Examples: X 20 (pause after every 20 lines) X 0 (don't

pause ever).  When you get More? in R cmd output (as a result of

using X cmd to set lines between pauses), you can type C for

continue without any more stops.

XF and XS Commands

XF sets "fast" mode, XS sets "slow" mode.  Fast mode means that

multiple lines are placed in packets for some commands (like D, R

and L).  Slow mode means one line per packet as it has been (the

default).  XF is faster for users with good paths.  XS is better

for users with poor paths (or busy channels!).

YD pathname filename

YAPP download of the given file.  Users must start their YAPP

function within 30 seconds of issuing the command.

YU pathname filename

YAPP upload of the given file.  Users can not delete or replace

existing files.  Users must be enabled with user bit $800 to be

able to upload files.

YW   List files in the default YAPP directory.  (See section on DOS SET

for further details on defaults.)

YW xxx    List the contents of the xxx directory under the YAPP

directory.

*    Putting * at the beginning of a line makes it a comment.  It also

suppresses the next command prompt (but the system will be waiting

for another command).  * is useful to answer the SYSOP if you get

a: MESSAGE FROM SYSOP

MSYS 1.14                                                   Page 73

< filename (sysop only)

Causes the commands found in the given file to be processed as if

they were typed from the keyboard.  This may be used as a message

import function if messages are placed in a file.  A sample of

this is:

SB ALL @ ARRL < W1AW $XYZ27

Sample title

This is the text of the message.  It is short.

/EX

You can of course have many messages in the file.  When end of

file is found, input is taken again from the keyboard.

> filename and >> filename

Both of these commands cause output normally sent to the screen to

be also saved in the indicated file.  Using just one > causes a

new file to be created (replacing any existing file by that name).

Use of two > before the filename causes output to be appended to

the end of the filename given (if it exists).  To terminate

writing to the file, use either of these commands without a file

name.  While output is being saved to a file, there will be NO

PROMPT FOR NEXT COMMAND.

/ To be used for sysop commands while in the BBS (see remote sysop)

MSYS 1.14                                                   Page 74

REQxxx SERVERS

A set of REQxxx servers are available in the BBS they are: REQDIR,

REQFIL and REQQTH.  Messages sent to your system to REQanything will be

processed.  If a server is not available for the requested function, a

reply msg will be generated stating that the server is not available.

The servers named above are enabled by new commands of the same names.

That is, using REQDIR ON will enable the REQDIR server, etc.  Enabling

the REQQTH server also requires that a file name containing the

Buckmaster CD ROM ASCII callbook file (normally S:HAM0\HAMCALL.129) be

specified with the CAllfile command.

The REQDIR server responds with a message to the requester containing

either the FILES directory listing or specified FILES directory

subdirectories given in the message title.  An @BBS to which the

message is to be returned may optionally be given in the title if the

reply is to go to a bbs other than the one at which the original

message was entered.  Examples:

SP REQDIR @

WA8BXN

(blank title)

^Z

would return a listing of the FILES directory to the bbs used to

originate this message.

SP REQDIR @ WA8BXN @NT8V  (title)

^Z

would return a listing of the FILES directory to the sending

station @ NT8V.

Subdirectories may be given in the title, proceeded by a slash and

separated with blanks.  The optional @BBS may also be specified.

Example:

SP REQDIR @ WA8BXN

/MODS /PGMS/BASIC

^Z

The file REQDIR.HLP included on the distribution disk should be placed

in the HELP directory.  The file REQFIL.HLP should be placed in the

FILES directory.

The REQFIL server responds by sending the requested file.  The file

name is specified in the title of the message.  Subdirectory names may

be given if needed (don't include the leading /).  An @BBS can be

specified in the title as described for REQDIR above.

Example:

SP REQFIL @ WA8BXN

mods/kam.1 mods/kam.2

^Z

MSYS 1.14                                                   Page 75

The REQQTH server responds with callbook information for calls given in

the title.  Currently only US hams are in the callbook file.  Calls

must be separated by blanks.  An @BBS may be specified as described

above.

Example:

SP REQQTH @ WA8BXN

nt8v no8m K8EIW

^Z

MSYS 1.14                                                   Page 76

CALLSIGN SERVER

The callbook server has split out of MSYS.  There are three servers

available, one for the Buckmaster CD ROM, one for the  and one for the

j-com disk files.  Note that the CAllfile command has been removed from

MSYS.  In its place you specify the name of the file (or directory in

case of j-com files) to be used when you start the appropriate call

server TSR.

To use either the Buckmaster CD ROM callsign files, or those available

from j-com, and SAM (c) RT Systems,  you must load the appropriate TSR

program before starting MSYS.  Once this is done, accessing the call

server is the same as it has been in the past (using either the PC

command in the BBS or sending a message to REQQTH @ the bbs call).

The TSR for the CD ROM files is CALLSRV1.  It optionally takes one

pararmeter, the name of the callsign file.  If none is given, the

default is S:\HAM0\HAMCALL.129.

The TSR for the j-com files is CALLSRV2.  It optionally takes one

parameter, the name of the directory in which you installed the several

j-com files.  If none is given, the default is F:HAMBASE.

The TSR for the SAM (c) files is CALLSRV3.  It optionally takes one

parameter, the name of the directory in which you installed the several

SAM (c) files.  If none is given, the default is F:HAMBASE.

Things needed to get the CD ROM call lookup to work:

Put:

LASTDRIVE=W

DEVICE=SONY_CDU.SYS /D:MSCD210 /U:1 /B:340 /M:P /V /L:S /Q:* /T:*

in  your CONFIG.SYS Reboot system.

Run the following before MSYS (probably put these statements in

AUTOEXEC.BAT):

MSCDEX /D:MSCD210 /M:4 /L:S /V

MSCDINFO

CALLSRV1 S:\HAMO\HAMCALL.129

Put:

REQQTH ON

in MSYS.DO

Notes: You may have to make changes in the above statements if you have

different device drivers for your CD ROM drive or set up the hardware

options differently.  Running without interrupts works pretty well on

the CD ROM drive and if you are like me you don't have any interrupts

to give to the drive anyway.  If you don't want to respond to REQQTH

queries but only enable the PC command on the BBS, set REQQTH OFF.

MSYS 1.14                                                   Page 77

For all the servers, the title must be less than 39 characters.

Things needed to get the SAM (c) data base call lookup to work:

Put:

F:\sam\api\ F:\sam     <--------This installs the SAM (c) API

interface.

C:\msys\callsrv3.exe F:\sam  <--This installs the MSYS interface to SAM

(c) in your AUTOEXEC.BAT file.

Of course, this presupposes that you have already installed the SAM (c)

data base on your F: drive.

You will also need to enable the Callsign Server in your MSYS.DO file

with the command PC ON

MSYS 1.14                                                   Page 78

MODEM SUPPORT

MSYS will support one or more modem ports.  The modems must be capable

of supporting true carrier detect (CD) operation.  Both the CTS and the

DTR line must be connected!

MSYS modem forwarding protocol

When MSYS attempts to forward to another BBS, the following protocol is

used for the initial logon. When MSYS sees the other system send

"Call:" or "Callsign :" it will respond with two lines. The first is

"echo off" and the second is the callsign indicated in the forward

file. Next, when it sees "Password:" or "Password :" it responds with a

line that is the password from the forward file. It then proceeds using

the normal packet protocol lines.

Incoming Call Protocol

When carrier is detected from the modem, MSYS sends a line like

MSYS Modem Interface (c) Copyright 1993 Hub Computers Inc.

It then attempts to open MSYSMOD.MSG. If the file is found, its

contents are sent to the modem. Next MSYS prompts with "Call:". At this

point a human operator would enter their callsign value. An MSYS

connecting for purposes of forwarding would reply with "echo off" which

disables echoing by the called MSYS system and then the callsign value

would be send on a second line.  MSYS next prompts with "Password:" and

expects the password to be entered that corresponds to the previously

given call sign.  These two words are then used to scan the

MSYSPASS.MOD file to find a match.  If a match is found, the third word

on the line in the MSYSPASS.MOD file is used as the apparent callsign

with which the user will appear to have connected to the system.

Here is an example of a normal user connecting into MSYS via modem:

MSYS                     User

-----                    --------

MSYS Modem Interface ...

[contents of MSYSMOD.MSG]

Call:

NO8M

Password:

camping

[from this point on it looks just like a packet connection was made]

MSYS 1.14                                                   Page 79

Here is an example of a MSYS system forwarding into another MSYS system

via modem:

MSYS                     Calling MSYS system

-----                    -------------------

MSYS Modem Interface ...

[contents of MSYSMOD.MSG]

Call:

echo off

WA8BXN

Password:

BINGO

[normal packet protocol follows]

Note: To accommodate exchanges with FBB systems, MSYS when forwarding

will respond to "Callsign :" in place of the expected "Call:" and

"Password :" in place of "Password:". However if the "echo off" part of

the protocol is not supported by FBB (or other systems) there will

probably still be problems.  Compressed forwarding may have problems

with any systems. Normal AX.25 protocol or FBBC 1 protocol should work

OK but there will be little benefit to using FBBC 1 protocol on a MODEM

port.

MSYS.OPT FILES

The name of the port MUST be (in capitals) "MODEM"  Radiospeed must be

set to 1200.  Port speed must be set to the speed of the modem for a

300 to 4800 baud modem.  For modems that can lock the port speed it

should be set to at least 19.2k for a 9.6 or 14.4k modem.

Here is an example:

port 5 at $260 int 5 speed 19200 name MODEM radiospeed 1200 tnctype 2

MSYS.DO

A line turning the Clear To Send (CTS) (EIA pin 5) to on must be

present.  "CTS 5 on" will do it for port 5.  The CTS line must be

connected.

MSYS 1.14                                                   Page 80

MSYSTNC.x FILES

There are a number of ways to set up the modem.  Here is a file for a

Zoom 2400 baud modem:

atz ---------- reset the modem to the power on defaults

atq1 --------- don't send responses

atm1 --------- speaker will be active until connect

ats0=1 ------- answer on one ring

The only problem that might show up with these files is that the

"ats0=1" *MUST* be last in all the files.  If it is another location

and someone is calling in at the time of the initialization, the modem

will answer and then ignore all the commands that follow and come up

with an improper initialization.

One 2400 baud modem was found with a unique need.  Register 27 on a

small plug-in was found to control the V22.bis routine.  If 27 was set

to one (the default), then the modem would *NOT* initiate a V22.bis

error correcting connect.  The factory default was that 27 was set to

never make an error correcting connect!  The line "ats27=0" cured the

problem.  AND ... it was found on a very generic 2400 baud modem that

this same register, which was not mentioned in the documentation,

worked the same way.  Check for register 27 on your 2400 baud modem!

Here is the MSYS.OPT string used with a V32.bis Zoom modem which is

able to connect from 300 to 14.4k (with automatic fallback):

port 5 at $260 int 5 speed 19200 name MODEM radiospeed 1200 tnctype 2

Here is the MSYSTNC.2 file that then sets it up:

atz ........ reset

at&f ....... load factory defaults

atl0 ....... speaker volume low

at&c1 ...... DCD follows carrier

at&d2 ...... DTR signal setting

atq1 ....... no responses

atm1 ....... speaker on until carrier

ats0=1 ..... answer in one ring

This may or may not work with your modem.  A book and a whole bushel

basket of experimentation may be necessary.  Without the book for your

modem, your work will be cut out for you.

NETNode USAGE

Access to a network node from the modem is possible.  It is

accomplished by adding the capitalized string "NETNODE" to the

appropriate line in the MSYSPASS.MOD.  USE THIS OPTION WITH CAUTION!

An example follows:

wombat poobear WA8BXN NETNODE

MSYS 1.14                                                   Page 81

FORWARDING ON THE MODEM PORT

LET'S PAUSE FOR A MOMENT ... DO NOT FORWARD WITH MSYS ON A

TELEPHONE LINE UNLESS YOU ARE ABLE TO INITIATE A ERROR CORRECTING

CONNECT WITH THE MODEM.  A NON-ERROR CORRECTING CONNECT WILL MAKE YOUR

STATION RESPONSIBLE FOR GARBLED, JUMBLED AND MESSED UP MESSAGES.

In your forwarding file, you put a  F#, R#, or P# line followed by the

things to forward. The format of the F, R and P lines are identical

except for the letter.  This format is different from what would be

used for packet forwarding and is as follows:

F# callsign phone# callword password

where # is the port number,

callsign is the call of the station you are forwarding to,

phone number is the string to send to the modem to dial,

callword is the reply to give to Call:, and

password is the reply to give to Password: (questions asked by the

destination MSYS system when you call it). Example:

F1 WA8BXN ATDP5551212 swiss normal

Note: Everything is converted to upper case. Thus the station

you are calling must have UPPER CASE values for your callword and

password.  The station you are calling must be running  MSYS 1.11 or

higher.

MODEM USAGE AT 4.8, 9.6 AND 14.4k

Hardware flow control between a modem using 9.6k or above is critical.

MSYS now hosts full CTS support.  This means that the modem's port

speed can be set to a speed higher than the maximum connect speed.

When a connect is achieved, the speed between the modem and the

computer remains at that higher speed no matter what the speed of the

modem to the remote person connected.  If a person checks in with an

old 300 baud modem, the speed between the computer and the modem

remains at what it was sent as.  The modem will accept data until its

buffer is full.  It will then signal the computer, via CTS control,

that it is full and will await the buffer emptying before it tells the

computer to send more data  A full RS-232 cable is necessary.  The

three wire cables that work in some TNCs will not work here.

FBB ON THE PHONE

Compression is not done on the phone line.  The time it takes to

compress and uncompress the files negates the time savings.

MSYS 1.14                                                   Page 82

ANONYMOUS CHECK-INS

It is possible to set up the modem port to take a call from an

anonymous check-in.  As long as all outgoing traffic is held, people

interested in ham radio, awaiting their license or whatever can read

and send traffic.  As long as ALL outgoing anonymous traffic is held,

there are no legal implications.  MSYS is designed so that if someone

uses the string "visit" for both the call and the password, and if

visit is in the MSYSPASS.MOD,  MSYS will allow a check in.  Here is how

the MSYSPASS.MOD should be set up:

VISIT VISIT V1SIT

Visit Visit V1SIT

visit visit V1SIT

... other calls follow

By having all the various combinations of case, a anonymous check-in

does not have to worry about the case that is used.  Now, it is

CRITICAL that MSYSHOLD.DAT have the lines "connected=v1sit" and

"from=v1sit".  All anonymous traffic will be held.  It can then be

routinely deleted or readdressed by the sysop (with the sysop's call

being used for the sending party).

It is also necessary to use the "EU V1SIT" command to add the user to

your system.  Here is what the file will look like when done:

V1SIT is VISITOR located at VISITOR  Last new message 19485

Last connected Fri Mar 19 23:10:57 1993 on port 5

Flags=$0040 Lines=20

Postal code: VISITOR    Home BBS: NO8M   Last WP Update: Mar 10

08:41:01 1993

Path to V1SIT is Direct

Other calls than V1SIT can be used, however MSYS is programmed to

disallow changes to the V1SIT callsign's information.  An anonymous

check-in can not go in and change V1SIT's home PBBS, name, etc.

MSYS 1.14                                                   Page 83

It helps to have a help file that tells the anonymous users something

about how to get help.  Here is an example MSYSMOD.MSG:

*******************************************

** Welcome to the NCARC MSYS Modem Port! **

*******************************************

For information on the system, use "I".  To a list of messages on

the board, use the command "L 0".  To see what is in the

download section, use the command "W".  (Use "D FILES.LST" and

"D README.DOC" for information on how to get those files.)  A

MSYS User's Manual is available and how to get it is in the

file USERMAN.INF.)  Please DO use this modem port for

downloading the LONG files I have here ... we have PART 97 of

the FCC Regulations and other very long files that are

inappropriate for downloading by a RF link.

If you are interested in amateur radio and how to get your license

leave me a message (SP NO8M or SP SYSOP) and I will get you in

touch with someone that can help.

-----end of MSYSMOD.MSG

Anonymous use of the port will be in direct proportion to the

advertising that is done outside of amateur circles.  If you make up

fliers and distribute them, you will see port usage rise.

ACOUSTIC COUPLED MODEMS

There are times that you may want to utilize a acoustically coupled

modem port.  This might be needed when you want to forward

automatically from a pay phone.  The MSYSFWD.DEF line in this set up

would read like this:

-----

R0 NO8M ATX1D call password

$FWD\NO8M.REV

-----

A call and password that is a unique, pretend user is set up.  If the

call and password were BOZO and CLOWN, the following line could be used

in your MSYSPASS.MOD:

BOZO CLOWN N08M

MSYS 1.14                                                   Page 84

The pretend callsign N08M would then be set up on the home PBBS to

forward what you wanted.  You would add the pretend call to your home

MSYSFWD.DEF like this:

-----

r5 n08m

!25

$fwd\n08m.rev

-----

This would cause the home MSYS to keep all the traffic that you wanted

for the acoustic system (and was listed in the N08M.REV) listed as PN

or B$ and ready to forward into the remote system.  Make sure you add

the pretend call to the MSYSBBSB.DAT if you will be forwarding

bulletins.

MISCELLANEOUS NOTES

ON USAGE       If you have a user who ties up a RF port for hours

reading each and every message, you may benefit by having him start up

a MSYS, check in on the phone port and get each and every message

automatically.  In that way, the user can read the stuff off-line at

their own computer.

ON R: LINES:   The above user may then try to automatically forward

messages out on that phone MSYS that was set up.  This will add a new

R: line that will be unknown to your network.  You can either begin

supporting each and every PBBS that comes and goes or demand that the

station send their message with NO R: line and that they send their

messages manually.

REQUEST

Please forward problems, concerns and hints to Steve,

NO8M@NO8M.#NEOH.OH.USA.NA.

MSYS 1.14                                                   Page 85

DATABASE HELP

Commands related to Database manipulation available on MSYS

MSYS allows BBS users to input information and search for information

in simple data bases.  The names of the databases are Database.0

through Database.999.  These files are located in the normal FILES

directory, which can be accessed by  the D command (download) on the

BBS.  These files can also be searched by the G command on the BBS.  To

use this command you enter the command G (with  no operands).  You will

then be asked for the file you want to search.  Simply pressing return

at this point will select Database.0 by default .  You are then asked

for the search  string. This is a set of characters that when found in

lines of the file cause those  lines to be displayed.  Case is ignored

during the search.  If a selected line ends with \ then the following

line(s) are also selected for display.  Data is  entered into a data

base by sending a message to MSYS with DBADD in the title.  The text of

the message will be added to the indicated Database (Database.0) by

default.  A header line is also inserted in the data base telling the

station that made the entry,  long with the date and time.  Here are

some examples:

SP MSYS   {this is a S command issued to the BBS}

DBADD 3   {the message title, Database.3 to be used}

wind damage mentor Ohio \

large tree down at 615 & 20 \

reported by mentor pd \

no injuries

^Z   {normal way to end message}

The text of this message would be added to Database.3.  Now if you

search Database 3 for either wind damage or mentor for example,  you

would get to see these lines, along with any other lines that have wind

damage or mentor in them.

MSYS 1.14                                                   Page 86

Example of using search command:

G              {BBS command to do search}

File search routine {Response from bbs}

Enter file name (or just press return for Database.0): Database.3

Enter search argument: Mentor

{lines from message above displayed}

>              {BBS ready for next command}

Another example:

Assume a file had the following lines:

Mary had a little lamb

its fleece was white as snow, \

and every where that Mary went

the lamb was sure to go.

It followed her to school one day

If you searched this for 'snow' you would get the following back:

its fleece was white as snow, \

and every where that Mary went

That's a brief outline of what can be done.  Its simple yet very

flexible.  Let me know of any applications you can think of for these

facilities.  Particular Database numbers can be assigned to different

uses.  Formats for information can be suggested for each different

Database.

MSYS 1.14                                                   Page 87

FORWARD FILE DESCRIPTION

NOTE: TO FORWARD BULLETINS YOU MUST SET UP MSYSBBS.DAT!

The MSYS forward file (MSYSFWD.DEF) is an ASCII file that can be

prepared with any program editor.  The file consists of groups of lines

separated by a line beginning with at least 3 dashes.  Each group of

lines specifies what should be sent to a given BBS along with other

information.

The first line has the following format:

Position       Content

--------        -------

1   The letter F      If you substitute R for F (F0 becomes R0

WA8BXN) then it enables reverse forwarding

request.  P0 Forces a reverse forward poll.

EXCEPT IN FBBC MODE!

2           The port number to use (a single digit)

3           A blank

4to N       Callsign of BBS (with SSID) optional:

N+1         A blank

N+2 to ?    Via list of digipeaters

Examples:

F0 W1XX V W2XX,W1XX-1

F2 WA0XYZ-2

When no nodes are being used, this first line is used as if it

were a connect command to the desired system .  When a NODe is used,

the Via digipeater list will have no effect.

If a NODe (KA NODe or NET/ROM) is to be used, the next set of

lines are a script of how to use the NODe(s).  This script may be up to

25 lines long.  Its format will be described below.

MSYS 1.14                                                   Page 88

Next comes a list of callsigns of BBS systems that should be

forwarded  to the system given on the first line (the one beginning

with F).  There is one call per line.  No SSID should be given.  The

callsign of the system given on the first line should normally be given

in this list.  Example forward file that doesn't use any nodes:

F0 W1XX

W1XX

W2ABC

W2XXX

----------

F0 W1XX V W1XYZ

W1XX

W2ABC

W2XXX

---------

F0 W1XX V K1XXX,W1XXX

W1XX

W2ABC

W2XXX

--------------

F0 W8XXX-12

W8XXX

WA8XXX

K7XXX

------------

Note: In the previous example we try three different paths to connect

to W1XX.  If we get  the messages forwarded the first way the other

connections will not be attempted.

F0 W1XX

!01-04,12,21-23          This will forward during the times indicated

W1XX                     (Use of this line is optional, but if used,

W2ABC                    must begin with a !)

W2XXX

----------

In the list of callsigns given indicating what BBS's msgs can be

forwarded to a particular BBS if a second entry is typed on the line

then the second entry must match some part of a message title to allow

it to be forwarded.  For example,

F1 W1XX

ARRL DX

----------

@ARRL  will be forwarded only if DX appears in their title.  Case of

letters is ignored in the comparison.

MSYS 1.14                                                   Page 89

You can use * in the forward file.  If you have NTS* then all msgs with

NTS as the first 3 characters of the @BBS will be forwarded.

For example:

F1 W1XX

NTS*

----------

*  used alone is treated specially.  Only messages that have been on

the BBS for more than about 2 days will be forwarded.  This gives time

for them to be forwarded by other specified connections.

IMPORTANT  NOTE:  In the forwarding file, use of * for the BBS names

cannot be the first call and the first bbs in the list of BBB's that

can be forwarded to through the given bbs MUST  begin with a letter.

Example:

F0 WA8BXN

*         <---- won't work

WA8BXN

-----------

F0 WA8BXN

44*       <----- won't work

WA8BXN

-----------

F0 WA8BXN

WA8BXN    <----- will work

*

44*

-----------

Note: That # is a wild card character.  If you want to put #XYZ as an

entry in your forward file, you will have to use "#XYZ (the " is an

escape character that says the character that follows must match

exactly and is not treated as a wildcard character as it normally would

be).

MSYS 1.14                                                   Page 90

NODe Scripts

The script consists of several sets of lines.  The first line begins

with  @  followed immediately by the call (and Via digipeater list if

needed) of the NODe to initially connect to.  The port that will be

used is taken from the digit after the F on the first line in the

group.  If the connection is established to the NODe, the following

sets of lines are used to talk to the first NODe and possibly other

nodes called through it.

Each set of lines for a given NODe begins with a line that has a period

in position one.  The following characters on the line are sent to the

NODe as a command.  The next line in the set is optional and if used

has  #  in the first position.

Following this is the number of seconds to wait before abandoning use

of this NODe.  If this line is not given then 60 seconds will be

assumed.

The next line is required and begins with  +  in the first position.

The following characters are what are looked for in lines that come

from the NODe to indicate that connection has been established to the

station called (another NODe or the desired BBS).  An exact match to

the characters given on the line (including case) must be found

somewhere in a line received to continue.

Next come two lines beginning with  ?  in the first position.  These

lines contain strings of characters that if found anywhere in a line

that comes from the NODe that the connection was not made.  Thus after

we issue the connect command (given on the line beginning with .) one

of three things can happen: a line from the NODe that matches the

+string is found and we continue with the script or talk to the BBS, a

line from the NODe matches a ?string which means we give up on making

the connection, or a time-out occurs waiting for a match which again

means we will give up  the connection.

Example scripts:

These strings are useful for KA Nodes.

@ANOD    (issue connect command to ANOD)

.C       (issue connect to desired call)

#25      (time out after 25 seconds if no match)

+MADE    (if MADE comes from NODe, continue)

?BUSY         (if BUSY comes from NODe, abandon attempt)

?RETRIED (if RETRIED comes from NODe, abandon attempt)

For a NET/ROM NODe the following would be useful:

+to

?Failure

?Busy

MSYS 1.14                                                   Page 91

Here is another example of a complete forward file:

F1 W8XXX

W8XXX

W5XYZ

---------

F0 W4XYZ

@W7XXX

.C W9XXX

+to

?Busy

?Failure

.C W4XXX

#120

+MADE

?RETRIED

?BUSY

W4XYZ

W4XXX

-------

F1 W9XXX-2 VIA K9XXX-3,KA9XXX

W9XXX

W1XXX

W3XXX

K0XXX

----------

NOTE:  The forward connect script for a given bbs is limited  to 25

lines.

MSYS 1.14                                                   Page 92

MSYSBBSB.DAT

(YOU MUST HAVE THIS FILE TO FORWARD BULLETINS!)

This file contains a set of bit position numbers and corresponding BBS

calls to which $ BID messages will be sent.  The format of the records

is:

Bit#  Callsign(0-31)

An example file content is:

25 WB8LVP

26 KA8Z

27 KB8AST

28 KQ8M

29 NO8M

The MAXIMUM BIT# IS 31

MSYSFWD.DEF:  If you put a line beginning with $ and followed by a file

name (where BBS callsigns normally go) the given file will be read at

that point as more BBS calls.

Example:

F0 WA8BXN

WA8BXN

$BXN.DAT

ARRL

--------

In this example, the contents of BXN.DAT will be used for more calls

that can be forwarded through WA8BXN.  Due to the extra overhead of

opening, reading, and closing the file, this should only be used for

more than just a few call signs.

(callsign).FWD/.REV Files

One way to reasonably manage large forwarding lists is to create a file

for each BBS that you forward to.  The file will take its name from the

BBS that you will be forwarding to.  For example, to establish the list

of @BBS items to forward to WB8BII, you would create a file called

WB8BII.FWD.  To help manage the lists, you would to set up a

C:/MSYS/FWD directory.  In this example,  the file will be:

C:/MSYS/FWD/WB8BII.FWD.  In this file, you will list all of the

different @BBS and hierarchical addresses you wish to forward.

MSYS 1.14                                                   Page 93

Example:

File entry     Comments (NOT IN THE FILE!)

WB8BII         Mail with an @BBS of WB8BII

KA8Z           Mail with an @BBS of KA8Z

OKIPN          Mail with an @BBS of OKIPN

NEOH           Mail with an @BBS of NEOH

NTSKY          Mail with an @BBS of NTSKY

ALL*           Mail with an @BBS of ALL(anything)

"#NEOH         Mail with a HIERARCHICAL ADDRESS of .NEOH

OH             Mail with a HIERARCHICAL ADDRESS of OH

EU             Mail with a HIERARCHICAL ADDRESS of EU (Europe)

442*           Mail to ZIP CODE 442xx

The last entry in the table MUST have a carriage return after it.

There should be NO blank lines in the file, INCLUDING AT THE END OF THE

FILE!

It is suggested that perhaps @BBS and hierarchical entries be listed in

the file ahead of the more general ALL-type entries.  In the event a

logical connection is lost during a forward cycle, at least some of the

personal mail will be forwarded ahead of the less urgent "for sale-

rusty tower" messages.

If you wish WB8BII to be able to REVERSE FORWARD (i.e. connect TO you

and pull the messages listed, rename the file to:  WB8BII.REV.  This

will permit MSYS then to examine the file and to forward any messages

and bulletins to WB8BII upon request.  Don't forget, before any

bulletins can be forwarded to WB8BII, it must be assigned a number in

the MSYSBBSB.DAT file!

To make the above file work, your MSYSFWD.DEF would look like this:

(previous entry)

---------

F0 WB8BII

$C:/MSYS/FWD/WB8BII.REV

---------

(next entry)

When you run the MERge command, the contents of the

C:/MSYS/FWD/WB8BII.REV file will be added to the contents of the

MSYSFWD.DEF file to get a combined file listing the routing to WB8BII

and all of the types of messages to be forwarded.  Because the file is

in the /FWD directory, and because it has a .REV extension, if WB8BII

should connect to you, your station will forward those items as well.

"Reverse" forwarding is very beneficial to a busy "hub" bbs as it will

help move the mail.  It is not uncommon in a metropolitan area to be

forwarding to one bbs while several others are reverse forwarding from

you, provided the network capacity is available to handle the traffic.

MERge: (MUST DO THIS IN ORDER TO FORWARD MAIL)

MSYS 1.14                                                   Page 94

This command reads MSYSFWD.DEF and copies it to MSYS.FWD, expanding any

include files ($filename).  A file is also produced, MSYS.BBS that is a

sorted list of all possible BBSs that can be forwarded to (including

things like ARRL, 44*, etc.).  This command is for 2 reasons:  if there

were any include files in the MSYS.FWD files they did not get read when

a pf command was used and secondly, the new file is needed for

hierarchical forwarding.  The leftmost part of the hierarchical route

only is  used for forwarding and the contents of the file is needed for

efficient processing.  Any changes you make to your forward file must

be done to MSYSFWD.DEF followed by use of the MERge command (IN THE

BBS).

Call/@BBS replacement facility:

Create a file called MSYS.REP.  In it place lines with the following

format

OLDCALL OLD@BBS   NEWCALL NEW@BBS

For example:

ALL  ALLONT   ALL  NEOH

(messages  to: ALL@ALLONT  become ALL@NEOH)

44* NTSOH @1 ---

(messages to 44*@NTSOH    become originaltocall@ ---)

NTS* * @2 @1

(messages coming in to NTS-anything get tocall and @bbs swapped)

***  IMPORTANT  *** There can be no more than 150 lines in this file!

The algorithm goes like this: When a message comes in, a check is made

comparing the to call and @bbs of the msg with the first two fields of

the records in MSYS.REP.  If a match is found (wild cards are allowed

in MSYS.REP) then the last 2 fields are used to replace the tocall and

@bbs of the message.  @1 or @2 in the last two fields in MSYS.REP have

a special meaning.  @1 means the original to call and @2 means the

original @bbs.  Note that wildcards are not expanded in these last 2

fields.  To  support this facility a new command is also added:

REPlace.  If an argument of L is specified, the current replacement

list is displayed.  If no argument is given, the current contents of

MSYS.REP are read and stored in memory.  (This is done when  MSYS is

started also).  Use the REP command after you have edited MSYS.REP to

make the changes effective.

REVERSE FORWARDING

MSYS can respond with messages to a connected BBS when it gives F>.  To

use this facility, you must have a directory C:\MSYS\FWD that contains

files for each of the bbs to which you wish to reverse forward.  These

files will be named <requesting_bbs's_call_sign>.REV.  These files

contain the .@BBS values that can be reverse forwarded to the

requesting station.

So you may want to set up your forward file as follows:

MSYS 1.14                                                   Page 95

[file MSYSFWD.DEF]:

F0 W8XYZ

$c:\msys\fwd\w8xyz.rev

--------

F0 K8XXX

$fwd/k8xxx.dat

--------

The file C:\MSYS\W8XYZ.REV could look like this:

W8XYZ

ARRL

ALLUS

The file C:\MSYS\FWD\K8XXX.DAT could look like this:

K8XXX

K9XYZ

ARRL

With these example files, reverse forwarding would be enabled for W8XYZ

but not K8XXX.  Recall that $ lines in the forward file MSYSFWD.DEF

cause the given file to be included at that point.

Also remember to be sure to use the MERge command  whenever  you make

ANY changes to the forward files!  Reverse forwarding of a given

message is not possible until there has been at least one normal

forwarding (either forced manually with the FOrward command or at your

forward time).  The other half of reverse forwarding (where MSYS would

send F> when it is done forwarding messages to a particular station) is

enabled with the R in  the first line of the forwarding file (F0

becomes R0 WA8BXN).

When done forwarding MSYS just disconnects and does not send *** done.

Reverse forwarding checks if the bbs is currently connected.  If so

MSYS will not request reverse forwarding.

Reverse forwarding POLL (connects and requests reverse forwarding even

if you don't have anything yourself to forward) can be implemented -

EXCEPT when using FBB compatible forwarding!  Simply place P where you

had R or F before.  Thus: F0 WA8BXN becomes P0 WA8BXN

A check is made to see if asking for another message to be reverse

forwarded will overfill the maximum number of messages.

Reverse forwarding will check if the required number of attempts at

forwarding a msg have occurred before reverse forwarding.  Thus if we

have OH 25 in the reverse forwarding file, a msg that hasn't undergone

25 forwarding attempts will not be reverse forwarded by this entry .

NOTE that selective title forwarding is ignored during reverse

forwarding.

MSYS 1.14                                                   Page 96

SELECTIVE FORWARDING:

Selective forwarding by title contents is limited to ONE selection per

route.  Thus if you have both ARRL LETTER and ARRL PFB in your forward

file for a given bbs,  only bulletins addressed @ARRL with LETTER in

their titles will be forwarded.  No other bulletins with @ARRL

(including those with PFB in their titles) will be forwarded.

In addition to selective forwarding title comparisons, two additional

selection criteria may be specified: number of forward attempts that

must be made before using this path or maximum message size to be

forwarded using this path.  Here some examples:

F0 KA0FPO

ARRL

NEOH  DX

KQ8M  10

NEOH  '15

OKIPN  <3000

OKIPN  '<I

-------

This set of lines will cause the following to happen.  All @ARRL

messages will be forwarded.  Messages with @NEOH will be forwarded if

they contain DX (upper and/or lower case) in the title.  Messages with

@KQ8M will be forwarded if type is PN and there have been at least 10

forwarding cycles with the message present on the BBS.  Messages with

@NEOH will be forwarded if there is 15 somewhere in the title.  The

single quote mark is used to indicate that the number following is not

the number of attempts that must be made to forward the message by

other means.  Messages with @OKIPN that are less than 3000 bytes long

will be forwarded.  And finally messages with @OKIPN and <I in the

title (again, case ignored) will be forwarded.  The ability to not use

a given forwarding possibility until there have been the specified

number of forwarding attempts is included so that preference can be

given to favored paths while still allowing alternate paths to be

automatically used if the favored ones fail.  Limits on message size

have been implemented for those who may forward to mini-BBS systems

with limited storage capacity.

TCP/IP FORWARDING:

TO forward to VE3GKM-4's TCP/IP mailbox the file would look  like

the following.

F0 VE3GKM-4

@VE3GKM-4

.

+NET (may not be needed)

VE3GKM

(MORE @BBS CALLS)

---------------

MSYS 1.14                                                   Page 97

SMTP FORWARDING

Sample entry in MSYSFWD.DEF:

T 44 70 4 10   (DO NOT USE DOTS IN THE ADDRESS)

N8HSP

-----

Note that no port number is specified, it does an arp if needed.  Also

note no digis (put them in the arp table if you have to have them).  No

nodes.  Just T and the IP address of the DEStination station as shown.

This corresponds to the line like F0 N8HSP.  Next comes the @BBS values

to send to this station.  There can be more  than one.  Then the ------

that ends this entry.  The same forward file is used for both normal

forwarding and tcp/ip stuff.  I have also made some changes to the SMTP

command.  After you use it, if you type PRompt it will prompt you for

To: (enter call@bbs) Reply-to: (just hit return if you like the default

given) Subject: (this is in the body of the text, not the title) Then

you enter your text and end with.  (not ^Z) as instructed.  The result

is a message that contains all the fancy extra lines often found in

tcp/ip messages.

Bulletins may be forwarded using SMTP.  To do so you must assign the

SMTP station an entry in the MSYSBBSB.DAT file like you would for

normal forwarding (don't use any SSIDs).  Also add the callsign of the

station in the T line in the forward file after the IP address.

Example: T 44 70 4 5 K8EIW  This should be done in the forwarding file

for all SMTP forwarding.

Bulletins are accepted from SMTP (a bulletin goes to a non-callsign).

BIDs are made up based on your msg number and callsign.

Hierarchical addresses are added to msgs that come in via SMTP.

NOTE: Since BIDs are not supported in this implementation of SMTP,

bulletins can either be sent to or received from TCP/IP systems but not

both ways.

MSYS will accept <RCPT TO>: lines in either the format a%b@c or b!a@c

where c is your bbs call, a is the destination stations call and b is

the destinations bbs.  Both of these get transformed into a@b on the

MSYS msg list.  The first form (the one with the %) is newly added in

1.10 to help out in what seems to be a difference of opinion as to the

correct way to do it.

MSYS 1.14                                                   Page 98

FORWARDING NOTES

Make sure you don't have any blank lines in your various forward files

(MSYSFWD.DEF and the files in the FWD directory).

Executing commands from within the forwarding file.  In the forwarding

file at the beginning of a set of forwarding lines (i.e., at the

beginning of the file or after a ------ line) you can place any sysop

command(s) by simply putting \ in front of them.

The MSYS.BBS file contains a number after each "BBS" that the system

knows how to forward to.  This is the ORing of all the BBS Masks that

can be used to forward the particular @BBS.  This is used to determine

when bulletins have been sent to all stations that should be getting

them; when such a bulletin is found, it gets its type changed from $ to

# so then it is no longer considered for forwarding.

Within the forward file a !timestring line may be placed before any

callsign to limit forwarding hours for that callsign (or bulletin

route).  The first ! line as used before limits all times for a bbs

entry you forward to.  The following ! lines remain in effect until

changed by another ! line.  When you start another bbs forward entry

group the forwarding hour defaults to 0-23.  At this point I would like

a little example of what this means, so here it is (the comments and

blank lines would NOT really be in the forward file):

F0 WA8BXN this is the beginning of a forward group}

!0-16,19-23    {don't initiate forwarding during hours of 5 or 6 P.M.}

WA8BXN         {forward the @BBS any time we connect}

!0-16,22-23    {don't forward the following during early evening hours}

NTSOH

OH

!1-4      {forward following only in early morning}

ARRL

ALLOH

-----

BID:

When a msg comes in to non-callsign without a bid a check is made with

generated BIDs of msg#_callsign and callsign_msg# to eliminate

duplicates.

Messages addressed to other than a what looks like a callsign

automatically get a BID generated for them.

Mail forwarded to BBSs identifying with either [MBL or [...  - $  ]get

BID included in S command.

MSYS 1.14                                                   Page 99

FORWARDING DISPLAY

As each of the BBSs to which msgs can be forwarded is scanned for a

given bbs you will see it.  When a msg is being forwarded you will see

a line that gives its number and the to station and @bbs of the

message.  In parenthesis you will see the entry in the forward file

that is being used.  Thus if the forwarding file contained.

F0 WA8BXN

WA8BXN

44*

---------

and there was a message for K8ZZZ@44123 being forwarded you would see

something like:

Fwding msg # 123 K8ZZZ@44123(44*). QSLDX @BBS:

Messages  that arrive with @BBS of IDCALL get the @BBS field changed to

--- .  This makes them distinctive

Messages sent to REQQTH, REQDIR and REQFIL are left private if they

arrive that way.

***  There can only be up to 400 different bbs names in the forwarding

file.  To check this do a LCount on the file MSYS.BBS.

Wildcards (where you can use *) have been enhanced to allow more than

just using *.  Based upon the recommendations of AA4RE and others, the

following have been implemented:

* Matches 0 or more occurrences of anything

@ a..z, A..Z

# 0..9

+ a..z, A..Z, 0..9

? a..f, A..F, 0..9 (hex digits)

< a..z

> A..Z

$ =@#+?<>$     (any one of those characters, but I don't have any

guesses as to why this group was included)

= any one character

" the escape character: the next character must match exactly

( beginning of group . (abc) matches a b or c.

Examples: 9v1A2q   matches #@#@#@

95===  matches anything 5 characters long beginning

with 95

(WK)6   matches anything starting with W6 or K6 (case

is important)

95###   matches any 5 digi number beginning with 95

MSYS 1.14                                                   Page 100

HF FORWARDING

HF Forwarding regulations do not allow transmitting 3rd party traffic

unless there is a control operator present, the station is under remote

control, or a STA has been issued.  To aid in compliance with the

regulations, the ATtended command has been added.  When set to ON all

forwarding is allowed.  If off, ports with radio speed of 300 will

forward only those messages in which the from call is the idcall

(messages that you originate) or the to station in the message is the

station you would directly connect to for forwarding.  HF reverse

forwarding is also inhibited if ATtended is OFF.  Note that the

regulations appear to allow you to receive messages unattended.  So if

you don't have a STA for HF turn AT OFF when you aren't there and

remember to turn it ON when you are.  To be on the safe side, put

ATtended OFF in your MSYS.DO file should the system reboot when you

aren't present.

MSYS NODe Forwarding

You can forward directly through your own MSYS node (assuming  it is

properly activated).  Here are some examples how to do it:

F0 W8XYZ  This is the station we are forwarding to, the number

%W8XYZ    after F is not important Then do a network connect to W8XYZ

- assuming  W8XYZ is a BBS that is in our local node

destination list

W8XYZ     stuff to forward to this bbs

-------

F0 W8XYZ

%W8XXX    {connect to node W8XXX which is in our local dest list}

+to

?busy

W8XYZ     stuff to forward to this bbs

-------

NOTE:  There should be NO BLANK LINES in this file!

MISSING TEXT TO Message

If the text for a message is not available when it is being forwarded,

the forwarding is aborted instead of sending out a missing text message

for the text.  The STatus command will now indicate how many times this

has occurred and the last message number for which it happened.  The

MISsing command may also be used to determine how many times this

occurred without generating all of the other STatus output.

**** BE SURE TO INCLUDE MSYS in your forward files for all other

stations you forward to so I can send bulletins about MSYS to all of

you.  The list of MSYS operators keeps growing!

MSYS 1.14                                                   Page 101

FBB COMPATIBLE FORWARDING

MSYS follows the standard established by F6FBB in his version 5.14 and

5.15 of his BBS software.

As described in the FBB manual, once the F letter in the SID (System

type IDentifier) has been detected, all command lines must start in the

first column with the 'F' character.  All command lines are terminated

by a return (CR) character.  When the connection to another bbs using

the FBB protocol has been made and the 'F' flag has been received, the

SID of the local station is sent with the first proposal.  The proposal

may look something like this:

FB P WA8BXN WB8BII K8EIW 12345_WA8BXN 765

F>

Where:

FB             Type of command (proposal)

P              Type of message (P = personal, B = bulletin)

WA8BXN         Sender (From field)

WB8BII         BBS of recipient (@BBS)

K8EIW          Recipient (To field)

12345_WA8BXN   BID or MID

765            Size of message in bytes

F>             End of proposal

In the case of multiple message proposals, the 'FB' line is sent for

each message proposed followed by one F>.

Upon receiving the proposals, the other BBS will accept or reject the

message with the command line:

FS -+=

where

-    means No, already have that message

+    means Yes, send the message

=    assumed by MSYS to be No, message being forwarded by another

system.

The FS line must have as many +,- signs as  FB lines in the proposal.

Each message is sent with the title on the first line, the text, and a

control Z in the last line.  There are NO blank lines between messages.

When the other BBS has received all the messages in the group, it

acknowledges by sending its proposal and the direction of forwarding is

reversed.

If the receiving bbs has no further messages to send, it responds with

a:   FF

This line must NOT be followed by a F>.

MSYS 1.14                                                   Page 102

If the sending BBS has no further messages, it responds with a:

FQ

and then disconnects.

FBB COMPATIBLE COMPRESSED FORWARDING

The protocol utilized for the transfer of compressed messages is an

extension to the standard FBB protocol.  Compressed forwarding is

validated by the presence of the letter B in the SID [MSYS-1.14-BFHM$].

In the case of compressed forwarding, the proposal line begins with a:

FA

FA means that the transfer will be an ASCII compressed message.

The proposal to send a message is in the form:

FA P WA8BXN WB8BII K8EIW 12345_WA8BXN 6789

Compressed message transfer is done in a binary mode, which is derived

from the YAPP protocol.  All transfers consist of a header, a block of

data, an end of message and a checksum.  Each transfer is equivalent to

the transfer of one message of the standard protocol, but is not

followed by a control Z.  The end of file specifier is defined by the

transmission of a EOT character followed by a 1 byte checksum..

Format of the header for a compressed message:

<SOH> 1 byte = 01 hex

Length of the header 1 byte = Length from the title,

Title of the message 1 to 80 bytes

<NUL> 1 byte = 00 hex

Offset 1 to 6 bytes

<NUL> 1 byte = 00 hex

To follow the FBB format, the title of the message is transmitted in

ASCII, not compressed.

The offset is also transmitted in ASCII and specifies the offset at

which the data should be inserted in the file (in case of a fragmented

file).  A data block contains from one to 256 bytes.  It begins by two

bytes which specify the format:

<STX> 1 byte = 02 hex

Number of data 1 byte = 00 to FF hex. (00 if length = 256 bytes).

Data bytes 1 to 256 bytes

MSYS 1.14                                                   Page 103

The last data block is followed by the end of transmission character

(EOT) and the checksum in the format:

<EOT> 1 byte = 04 hex

Checksum 1 byte = 00 to FF hex

The checksum is equal to the sum of all the data bytes of the

transmitted file, modulo 256 (8 bits) and then two's complemented.

The checking of the checksum is very simple:

The sum of the data from the file and the checksum received modulo

256 (anded with FF) shall be equal to zero.

In case of a checksum error, the message is not accepted and the system

sends the comment

*** Checksum error

and issues a disconnect request.

The transmission sequence is repeated for each message in the group.

MSYS 1.14                                                   Page 104

HIERARCHICAL FORWARDING and MSYS

There are several features in MSYS that combined with hierarchical

forwarding should almost completely automate routing of incoming

messages.  These features act at two separate times: first when a

message comes in and second during forwarding.

First let's review the way hierarchical forwarding works.  With

hierarchical forwarding the @BBS field can contain more than just the

callsign of the BBS that is the DEStination of the message.  Routing

information can also be placed in this field.  The @BBS field thus has

the syntax of part1.part2.part3.  ....  When it is time to forward a

message, the @BBS field is inspected and the leftmost part that is

found in the forwarding file is used to forward the message.  Consider

as an example @N8PIH.MI.  If N8PIH is found in the forwarding file then

it is forwarded that way and the MI part is ignored.  On the other

hand, if N8PIH is not found in the forwarding file, then if MI is found

in the forward file then it is forwarded that way.  If neither part is

found in the forwarding file, then the message will not go anywhere.

Such messages will show up (after at least one forwarding attempt after

the last time MSYS was started) if you do a L? command.

If all messages had both the actual callsign of the DEStination BBS and

the appropriate xx part in the @BBS field then the forwarding file

could be greatly shortened.  You would need to have the calls of the

BBS stations in your state in the forward file, along with each of the

xx designators for the remaining states.  If there were say 50 BBS

systems in your state then perhaps 99 different entries would be

sufficient in your forward file (ignoring for the moment bulletin

routes like ARRL).

NOTE that the limit of 400 different calls in the forward file is more

than sufficient!  Messages that are going to a BBS in your state get

routed as specified in your forwarding file; those going to another

state get routed to the BBS you have set up in the forwarding file to

get stuff for that state.  You could include a few explicit calls for

BBS systems in nearby states that you route directly to, if you want to

override the normal path for messages going to that state if you so

desire.

Now for the second part, the action taken by MSYS when a new message

arrives.  When a message comes in, MSYS looks at the @BBS field.  If it

is absent, then if the to callsign field contains something that MSYS

knows the routing for, the to field is copied to the @BBS field.

Otherwise the @BBS field is set to ===== and the message won't go

anywhere.  It will show up as noted above in a L? command.

MSYS 1.14                                                   Page 105

For forwarding NTS traffic, MSYS uses a file called BBSTONTS.BIN that

contains BBS callsigns and the states they belong in (this information

comes from several sources, more on this later).  Next MSYS inspects

the @BBS field to determine if it knows how to forward what is in the

given @BBS field.  If some part of the possibly hierarchical

specification is found in the forwarding file, it is left alone and

will be forwarded by that matching specification.  If no match is found

in the forwarding file, then the BBSTONTS.BIN file is searched to find

some part of the given @BBS field.  If a match is found, the

corresponding NTSxx is appended to the existing @BBS field.  Thus MSYS

will attempt to lookup the right state to send the message to.

Assuming the state is found, then forwarding will take place as

described above (see the N8PIH.MI example).  If no part of the @BBS

field can be recognized, then the message will not be sent out, but

will show up when you do a L? command as described above.  The

BBSTONTS.BIN look up will also handle zip codes given in the @BBS

field, making @12345 into @12345.xx as is appropriate.

Thus far we have examined what happens when in the process of

automating forwarding.  In the event the proper routing cannot be

determined by MSYS then you will have to use the sysop EM (edit

message) command to manually add the appropriate .xx designation,

assuming you can guess it somehow.  If you are lucky, the originator

put useful information in the title of the message or may have hidden

it in the text of the message.  Maybe the callbook will be of some use

to you.  Or maybe you can just force it in the opposite direction from

which it arrived.

MSYS 1.14                                                   Page 106

Creating the BBSTONTS.BIN file.

This file contains binary data and cannot easily be created with a text

editor.   in MUTIL.EXE has been provided to create this file.  Function

14 takes as its input a file called BBSTONTS.DAT and produces as output

BBSTONTS.BIN.  To create BBSTONTS.DAT, you run a function called

BBSTONTS (function 13 in MUTIL) which takes as its input BBSLIST.DAT.

BBSLIST.DAT is created and/or updated using data from message headers

that have passed through your system and/or AD8I bbs lists.  Function

10 of MUTIL is used to use messages as input; function 11 uses AD8I

lists as input.

Lets look at a picture of these files and programs:

+---------------+

| Message files |

| from MAIL.BAK |

+---------------+

|

|

|

V

MUTIL function 10

|

|

|

+-------------+

| BBSLIST.DAT |

|  file    |

+-------------+

|

|

MUTIL function 13  <--- Other standard files:

|          STATES.DAT

|          ZIPCODES.DAT

V          ROUTES.DAT

|

|

+-------------+

| BBSTONTS.DAT|

|   file      |

+-------------+

|

|

MUTIL function 14

|

|

V

|

|

+-------------+

| BBSTONTS.BIN|

|   file      |

+-------------+

MSYS 1.14                                                   Page 107

HOUSECLEANING (HCL command)

MSYS.HCL

You should create a file called MSYS.HCL into which you put lines

describing classes of messages you want to eradicate.  Each line should

contain one or more of the following keywords:

AT= TO= FROM= AGE= TITLE= TYPE= KEEP= SYSOP=

So for example:

AT=ARRL AGE=30

would delete all messages with @BBS of ARRL that are over 30 days old.

If specified, the values of AT TO and FROM must match exactly for a

message to be deleted.  TITLE and TYPE if specified must match some

part of their corresponding message header fields; case is ignored.

KEEP can be used to specify a number of msgs that match the other

values given that should be saved anyway.

Example:

TO=ALL AT=ARRL TITLE=PFB KEEP=2

Would keep the newest two messages to ALL@ARRL with PFB somewhere in

their title.  NOTE:  The value of title given cannot contain blanks.

SYSOP=Y has to be specified in any line that you want to delete

messages that are to the idcall of the bbs.  Thus to delete all read

messages (including one to you) you could have a line:

TYPE=PY SYSOP=Y

The following line deletes read messages, except those to you:

TYPE=PY

The effects of these lines can be far reaching.  For example, if you

put in the line:

KEEP=5

Then it will delete all but the 5 newest messages!  All this action

takes place when you type:

HCLean NOW

or at a time specified as a numeric parameter to HCLean.  HCL 120 would

cause it to happen at 02:00 (number given is minutes after 00:00

[midnight] to begin housecleaning).

MSYS 1.14                                                   Page 108

msys.hcl file as an example:

at=arrl age=21

at=okipn age=10 a

to=panet age=3 type=#

at=alloh age=10 type=#

at=allbbs age=7 type=#

at=amsat age=7 type=#

at=midnet age=3 type=#

at=arrl title=pfb keep=1 type=#

at=arrl title=dx keep=1 type=#

at=arrl title=kep keep=1

type=# title=sale age=2

type=# title=need age=3

type=# age=35

type=py keep=1

type=pf keep=2

type=tn age=21

type=pn age=60

to=sysop age=7 type=#

At the end of housecleaning, the file MSYSHCL.DO is done (as a DO file)

if it exists.

MSYS 1.14                                                   Page 109

MSYS NETWORK NODe INTRODUCTION

With version 1.06 is the ability to interface a variety of the

functions previously found in MSYS into networks built using any

combination of other MSYS systems, TheNet nodes, G8BPQ Nodes and NetRom

nodes.  The network NODe support in MSYS is by no means complete or

perfect at this time but it does seem to be usable.  Some basic

information concerning the implementation of the MSYS NODe interface

will help in making decisions related to configuration.  Two terms must

be understood by the system administrator (SYSOP).  These terms are

neighbor NODe and DEStination NODe.  A neighbor NODe is a compatible

NODe to which there is a direct connection path.  MSYS does not support

use of digipeaters in the path to neighbor nodes.  Normal AX.25

connections are made to neighbor nodes.  The data exchanged by these

connections consists of a mixture of supervisory and data frames that

together support the network operation.  DEStination nodes are also

network nodes and are the nodes listed when one does a Nodes command.

DEStination nodes are nodes that can be reached through the network

either directly if they are also neighbor nodes, or indirectly through

some neighbor NODe.

Routing

Routing of frames in the network is conceptually fairly simple,

handling all the details is not!  Each NODe does a "Nodes Broadcast"

periodically (usually every 30 minutes) which is a list of nodes that

it knows how to route to.  This may be due to directly hearing a

neighbor NODe or by hearing the nodes list from neighbor NODe that

claims to have a route to each NODe it lists in its NODe broadcast.

Information from the nodes broadcast is maintained by MSYS.  When it

receives a frame that is not for itself, MSYS looks to see which

neighbor NODe can handle the DEStination indicated in the frame and

simply passes the frame on to some neighbor NODe.

Data frames while they are inside the network as handled as what are

termed "datagrams".  What this means is that during a conversation

between two end users there is no guarantee by the network that the

data frames will always go by the same path or that they will make it

through the network.  Depending on conditions in the network the path

taken by frames can change and at times frames will be arbitrarily

discarded.  The end nodes (the ones the users connect to) are

responsible for error free end to end services.  While the AX.25

protocol insures that transmission of frame between any two nodes is

error free, the network protocol establishes error free operation

between users.

MSYS network NODe functions include both the routing of frames within

the network as well as the uplink/link logic to support the error free

end to end services.  Additionally, the TCP/IP servers can make use of

the network as an alternative to the existing IP routing facilities.

Connections from the network may be made directly to the BBS through

the enhanced NODe command set.

MSYS 1.14                                                   Page 110

An attempt has been made to make the MSYS network NODe user friendly

while still retaining compatibility with other network nodes.  To this

end there is a help command and headings on output generated by various

information listing commands.  Since MSYS is generally run at an

attended site, a Talk command is available to allow users to talk to

the sysop directly from the network.

Basic Decisions About the Network NODe

The really big question is should you enable the network NODe functions

of MSYS? Fortunately, you have more options than just yes or no! So

perhaps the question should be reworded as should you run the NODe in

full feature, fully automated mode?  The answer for most stations will

be NO.  The answer in your particular case will depend on the

capabilities of your station and the locations of other compatible

nodes.  If you run a high profile station already (wide coverage as a

digi/K-NODe) and have compatible network nodes at reasonable distances

from you (maybe at least 30 miles away), then you may fill a need by

running with everything enabled.  On the other hand, if the only way

you can talk to anything is through a digipeater (even when the

batteries in your ht are fully charged) and on a hazy day you can still

see the antennas of at least five network nodes, all still forwarding

the same frame they received 2 years ago, then may I suggest you run in

a very controlled minimal mode.  Make friends, not enemies through

these enhancements to MSYS!

Network NODe Quickstart

This section tells the minimal things you must do to get the NODe up

and running in some (probably not optimal) fashion.  You MUST define

your network NODe callsign using the NRC command.  If you are also

active in TCP/IP you should set NRCall and MYTcpcall to be the same

thing.  You MUST use the NPorts Command to enable the ports that you

want to be able to access the NODe.  For example, NP 3 would allow

access to the NODe on ports 0 and 1 but no others.  You SHOULD define

your network NODe mnemonic identifier using the NRI command.  You MUST

enable the NODe using the NETNode ON command.  You should probably turn

AUTONode and AUTODESt ON and set OCM to 3.

This should put your system into full service automatic mode as far as

the network NODe is concerned.  It probably will not be the way you

will want to run but its a starting point.

MSYS 1.14                                                   Page 111

Manual Operations on the Network NODe Database

To REMove a DEStination NODe completely, use

REMove DEStination nodecall

To REMove a DEStination for a given neighbor NODe, use

DESt DELete DEStcall neighborcall

To REMove a neighbor NODe (and all  the  DEStination  routes through

this                          neighbor NODe), use:

NODe DELete nodecall port#

To add a new neighbor NODe, use

NODe ADD nodecall port# quality [digicall1 .. .  digicall8]

Note:  The mnemonic identifier for the neighbor NODe will be obtained

from the first nodes broadcast heard from the NODe.

Use of the digipeater calls is optional.  Although you can specify up

to 8 digipeaters, rarely will more than 1 or 2 be practical to use.

To add a new DEStination for a given neighbor NODe, use

DESt ADD [destid:]destcall neighborcall neighborport obs# quality#

Notes:

The neighbor NODe callsign must be defined before any

DEStinations can be added for it.

destid or alias for the DEStination is optional.

obs# is the obsolescence count, 0 locks the DESt in for the NODe.

To change the quality of an existing neighbor NODe, use:

NODe REPlace callsign port# newquality [digi1 ... digi8]

To change the obsolescence count and quality of a DEStination through

some neighbor NODe, use:

DESt REPlace DEStcall neighborcall neighborport newobs# newquality

MSYS 1.14                                                   Page 112

How I Run My Network NODe

I run my NETNode in a moderate mode here that is somewhere between

fully automatic and low profile.  Here is some basic  info that should

help you understand my configuration: Computer is an old original IBM

PC 4.77 MHz clock 640 K ram,  two 10 M hard drives, 4 serial cards and

a printer card.

Port Freq.

0  223.7     High profile (150 W, beam at 80 ft .  aimed west)

1  145.01    Quite high profile (200 W omni ant at 90 ft.)

2  145.05    Low profile (10 W omni ant at 60 ft.)

3  80 M

4  40 M

5  20 M

6  serial connection to another computer

Nearby existing netrom nodes:

Port Call           Distance

0  WB8CQR-10      15 (Miles)

1  WB8CQR-1       15

1  K8EIW-1        45

1  W8QLY          75

1  W3LIF          80

2  KB8AST-1       25

Nearby NET.EXE stations using netrom interface:

Port Call           Distance

1  N8HSP-4        10

1  WB8LYJ-4       15

All of these stations are usually reliable connections.  Other netrom

nodes are usually heard but are not always reliable.  During band

openings many more are heard, some are reliable during the band

opening.

MSYS 1.14                                                   Page 113

NODE RELATED PARMS in MSYS.DO

Here are the network NODe related parameters I have in my MSYS.DO file:

OCM 3          Transmit DESt nodes heard that have obsolescence counts

3 to 6

NRI 460406     Right 3 bytes of my IP address [44.70.4.6] expressed in

hex  (quite useful this way)

NRC WA8BXN-2   Callsign net NODe responds to.  MUST be the same as

TCP/IP callsign (MYTcpcall)

NPorts $47     Network NODe uses ports 0,1,2 and 6 only

PQuality 100   Default port quality 100.  A low value assigned to

nodes not manually entered that connect to mine

NODe REStore   Read saved NODe file when MSYS starts

AUTONode OFF   Do NOT record NODe broadcasts from nodes not already in

neighbor lists.  This means that new nodes heard

(during band openings for example) will not be

automatically added, unless they do actually connect to

my NODe.

AUTODESt ON    DEStination NODe broadcasts (from nodes already in my

neighbor table) will be recorded.

NETNode ON          Enable network NODe

Here is the output I get from the JN sysop command (or R command in the

NODe):

Port Neighbor NODe Call Quality   DESts  Heard    Digipeater(s)

1  AKR:K8EIW-1         179       66     23:43

1  CLE1:WB8CQR-1       180       67     23:41

0  CLE220:WB8CQR-10    180       67     23:34

2  PRYTWP:KB8AST-1     192        2     23:25

1  YNG:W8QLY           180        7     23:36

The general philosophy I am using is that I only want to attempt to

connect to network nodes that I have a good path to.  These are also

the nodes that I broadcast myself (and the DEStinations they claim they

can handle).  I have manually entered these nodes into my neighbor NODe

table (using NODe ADD).  I see little sense in saying that I can really

connect to a NODe I hear once because a meteor hit at the right time!

This method does not particularly limit the number of DEStination nodes

my NODe knows about because the nearby nodes claim they can handle

everything they hear.  I just let the nearby nodes worry about finding

routes to everything they claim to hear.  My system serves as a link

from the one isolated NODe on 145.05 LAN frequency to the nodes on

145.01 which give access to the Ohio 4800 Baud UHF backbone frequency.

I also serve as an IP switch to and from the network.  I have adjusted

the neighbor NODe qualities to favor nodes (primarily TCP/IP stations)

to which I am a good route.  I have set most of the normal netrom nodes

to a lower quality value so that they will talk directly to each other

most of the time rather than trying to go through my NODe.  Should they

be unable to connect directly to each other, they will still be able to

go through my system.

MSYS 1.14                                                   Page 114

If you want to run very low profile in terms of getting used as a

network NODe, leave the default of 10 set for OCMin.  This means that

your NODe broadcasts will only send out your own call. This will get

the network to recognize you but not make it think you are a good relay

to anything.  Keep AUTOD ON so that your system will know how to reach

nodes in the network.  Its probably a good idea to set AUTON OFF and

manually add the calls of the best nearby nodes you can connect to (one

is all you need).

If you are totally confused about the network operations, try to

consult with who ever runs your local NODe(s).  Most of the parameters

you have, they have (with some changes in their names).

MSYS 1.14                                                   Page 115

Miscellaneous Comments on Network NODe

If you are low on SSIDs, you might consider the following.  Set the

IDCALL to your callsign without any SSID.  Do not define a BBS,

Keyboard, or K NODe callsign.  Pick an SSID and use your callsign with

it and set as the value for MYTcpcall, NRCall and FORWARD call  To get

to your BBS the network NODe BBS command can be used.  To talk to your

keyboard the BBS T command or network NODe T command can be used.  If

you want to make outgoing calls you will have to define a keyboard

callsign.

Should you use the network NODe to route TCP/IP frames?  I would say

only if you have to!  The normal IP routing is better than using the

network NODe but in those cases where using the network is the only way

to reach some distant IP station then do use the network.

Network node connects are allowed only on those ports enabled by NPorts

command.

K which lists KANodes that have been heard.  An optional parameter may

be given that specifies the port for which the list is desired.

Justheard which lists stations recently heard.  Here again a port

number may be given as an argument.  The N command is now described as

Netnodes (lists destination network nodes known).

In the network node, using the C# form of the connect command (example:

C2 WA8BXN) will always result in a normal AX.25 connect, even if the

destination station is a network node.

The network node does not record information in nodes broadcasts heard

on ports that are not enabled for network node use (NP command).

MSYS 1.14                                                   Page 116

NET NODe PARAMETERS SUMMARY

(PARM          Default   Comments)

PQuality [port#] # 192   Quality for nodes heard on port

BInterval seconds 3600   Nodes Broadcast interval

OCMin #            10    Minimum obsolescence to broadcast

OCInit #            6    Initial obsolescence count

WQupdate  #         1    Worst quality DESt route to record

NTtl #             64    Network time to live (max hops)

TTimeout seconds   60    Transport timeout (Net FRACK)

TREtries #          3    Transport retries

TACK sec            3    Transport ACK delay (RESPonsetime)

TBdelay seconds   180    Transport busy delay

NBdcast [port#]    ON    Enable or disable node broadcasts on a per

port basis

NDwindow #         4     Net default window size (frames)

NETCon            ON     Enable connect cmd in net NODe

NETBbs            ON     Enable BBS cmd in net NODe

NETTalk           ON     Enable Talk cmd in net NODe

AUTODESt         OFF     Add new DESts for known neighbors

AUTONode         OFF     Add new neighbors heard

NODe SEnd                Force known DESt transmission

NODe SAve                Write net NODe Database to disk

NODe REStore             Read net NODe Database from disk

NODe ADD call port qual [digis]    Adds neighbor NODe

NODe REP call port qual [digis]    Changes quality of neighbor NODe

NODe DEL call port       Deletes neighbor nodes & its DESts

DESt ADD [id:] destcall neighbor call neighborport obs# qual

Adds new DEStination

DESt REP destcall neighborcall neighborport obs# quality

Changes obs & quality

DESt DEL destcall neighborcall neighborport

Deletes DESt through given neighbor

REMove DESt callsign     Deletes DESt through all neighbors

NInfo                    Gives net NODe status

NRCall callsign          Sets net NODe callsign

NRId mnemonic            Sets net NODe mnemonic (alias) identifier

NPorts (MASK)         0  Ports to which nodes broadcast

TNatime seconds     600  Transport no activity timeout time

C BBS or C bbscallsign   Act like BBS command taking you to

the bbs.  You also now get a "connected to"

message with either of these commands.

A number of commands allow setting options for network connections.

Here are the network command names and their AX.25 port related

counterpart.

NETOnly        BPorts

NETRead        MAXRead

NETMax         BMaxcon

NETFBBStime    FBBStimes

NETFBBC        FBBC

NETFBBG        FBBG

MSYS 1.14                                                   Page 117

MSYS GENERAL INFORMATION

MEMORY MANAGEMENT

Buffer space for logical channels is allocated when the channel is

allocated (and released when the channel is released).  The MEM command

may or may not change results depending on position of the last user in

the memory space.  If you find that you are seeing ***MEMORY LOW BUSY

messages, you probably should investigate the reasons why.  Of course,

it is desirable to start out with as much available memory as possible.

One solution is to use some of the later versions of Disk Operating

Systems (MSDOS 5.0, 6.0 or DRDOS 6.0 or higher) that permit loading DOS

into upper memory to load all of your TSR programs in upper memory to

provide more room for MSYS.  Another suggestion is to use one of the

available memory managers to provide the maximum amount of program

area.

A third suggestion is to use the Quarterdeck program VIDRAM with the

"ON" switch set.  VIDRAM will utilize the memory just above the 640K

boundary that is normally used for graphics and add it to the available

program space.

If all that fails, or you just have a system with no expanded or

extended memory, you need to seriously consider your MSYS

configuration.  If you reduce the maximum number of messages (MUTIL 1)

to a smaller number, you will increase the available memory for users.

You may choose to limit the number of active channels.  If you are

experiencing low memory problems, you may also wish to disable the

conference feature.  While it is possible to reduce the size of the

various memory stacks used by MSYS, extreme care is needed to avoid

serious consequences.  See the section on STACKS on page 22 for further

details.

Another alternative is to provide a minimum service system by running

the "small" version of MSYS, which is distributed as MSYSB.EXE.  The

'B' version does not provide all the fancy NODE OR TCP/IP features, but

is will function quite nicely as a stand-alone BBS working behind or

along side of other network switches.  Probably the major loss to the

packet community is the fact that this version will not show up in a

NetRom node list.  You will still need the main distribution disks for

all of the documentation.  MSYSB.EXE is the executable version that

replaces the "big" MSYS.EXE

MSYS 1.14                                                   Page 118

EXPANDED MEMORY USAGE

MSYS will automatically detect if either EXTENDED or EXPANDED memory is

available.  If sufficient EXPANDED memory is present (at least 320K

must be available to be used) then all stacks will be around 8000 bytes

and come from expanded memory.  This should free up about 50K or so of

memory as shown by the MEM command.  Also each new connection will

require less memory.  If you have expanded memory and don't want it

used, start MSYS using:

MSYS NOEMM

Real expanded memory boards (that could be used with an 8086/8088

processor) will give better performance (faster) than simulating

expanded memory.  Expanded memory is used (if present) to contain the

message headers.  Now you can have 2000 msgs on your system and memory

to spare.  Be warned that with this many active msgs things will slow

down.

NOTE:  If you have problems running expanded memory try using a

different EMM program.

MSYS can also run using EXTENDED memory as well.  Of course, you will

need to utilize a EXTENDED memory manager sych as HIMEM.SYS or QEMM 5.0

or perhaps any one of several other quality memory manager programs on

the market.  If you have EXTENDED memory and do not want it used, start

MSYS using:

MSYS NOEXMEM

MSYS 1.14                                                   Page 119

REMOTE SYSOP

/Sysop command:  CASE SENSITIVE!

This command gets you into remote sysop mode which allows you to use

most of the sysop commands through the bbs.  The command must be typed

exactly as shown: backslash, Capital S and lower case ysop and then

return.  You will then get a line like

Usage stats: Users=4 Msgs rcvd=10 Msgs sent=5 Max=8

To this you must reply (there will be no prompt) four characters

corresponding to the numbers typed in order of characters found in the

file MSYS.RMT (case is important).  For example, if your MSYS.RMT file

contained MSYS is the greatest! then you would type <space> e i t which

are the corresponding characters (the first character is numbered 0).

Thus you would type: eit (Don't forget to count the spaces) You should

now get a prompt cmd:  If you are connected via radio, you issue /Sysop

command on bbs to verify sysop authority (enter 4 letters) You then get

the next bbs prompt (if you give correct letters or not).  If you have

given the correct reply then you may issue any sysop command by

starting the line with \ (while still in the bbs).  So when on the bbs

from the local keyboard, you can now type something like \em 1234 and

you will be able to edit message 1234 without having to leave the bbs.

As a remote sysop, once you give the right 4 letters, you retain sysop

status until you disconnect.  To get a list of available commands, type

h.  Most of the commands available duplicate those found normally as

sysop commands.  An added command, WHO, gives a list of who is on the

system and is the same as the new U command in the bbs.

NOTE: /Sysop is not required from local keyboard.

Make sure you mark yourself a remote sysop, even if you won't be

one.(User flag $10)

Automatic Password Reply

If your system receives the password prompt (the Usage state: ..  line)

from a system that you connected to during forwarding or from the

Connect commands, MSYS will automatically supply the correct response.

You must have a file called PASSWORD.RMT in your FWD directory that is

the same as the MSYS.RMT file on the other system.

NOTE:  If you are using MSYS to remote sysop a distant bbs, it will

respond ONLY ONCE per initial connect!  For example, if you are using

MSYS to sysop a remote system from the campground,  your local system

will respond to the "Usage..." statement the first time.  If you

disconnect from the remote bbs and drop back to a node, then reconnect

to the remote system, your local MSYS will NOT respond to the

"Usage..." statement.  You must disconnect completely and then re-

establish the connection to the remote system for the automatic

password response to be given.  This is done to help prevent repeated

automatic attempts to decipher the password.

MSYS 1.14                                                   Page 120

MSYS.DO

MSYS.DO is a file which you should create to do all of the sysop

commands needed to prepare your specific MSYS system.  This file is

processed as a series of commands and is far easier to do rather than

manually typing them every time you start MSYS.  When the system

starts, it automatically tries to do a DO command.

MSYSTODO.DAT

Timed execution of commands can be done in MSYS.  You can create  a

file called MSYSTODO.DAT with commands and times they should be done.

Wild cards can be specified for the times.  Each line consists of a

time (given as exactly 8 characters)  followed by  a space, and then

the command to execute at the  given  time. The wildcard character is ?

and they may only occupy positions in the  left  part of the time .

The format of the time  is  MMDDhhmm where  MM  is the month number (1

to 12), DD is the  day  of  the month  (1 to at most 31), hh is the

hour (0 to 23) and mm is  the minute (0 to 59) .  Here are some time

examples:

MMDDhhmm (pattern)

???????? every minute

???????0  every 10 minutes (times that are like 00 10 20 etc.  for

the minutes)

??????00 at the beginning of every hour

????1200 at noon every day

????0000 at midnight every day

???10000 at midnight of the 1st, 11th and 21st days of each month

??010400 at 4 am the first of every month

01010000 at the beginning of every year

You can also specify a day of the week in the DD field, in  place of

the day of the month .  To do this you would put D# in  the  DD field

(# is from 1 to 7 for Sunday to Saturday) .  Examples:

??D11200 Every Sunday at noon

11D52300 At 11 PM on each Thursday in November

That is how the times are set up, now here is a complete  example

of things that could go in MSYSTODO.DAT:

????0000 nod sav

????1200 nod sav

??010000 log

??d10000 copy msys.usr msysusr.bak

???????0 dt

This file would save the network node list twice a day, make a new log

file at the beginning of every month, make a backup of the user file

every Sunday, and display the time and date every 10 minutes.

MSYS 1.14                                                   Page 121

***LINKED

A line of the form *** LINKED to callsign is recognized as a connection

by the given callsign.  This occurs with some versions of RLI systems

when a user uses the C command on the RLI system to connect to the MSYS

BBS.  ***LINKED to callsign requires sysop verification if sysop call

is used.

KEYBOARD

When someone connects to your keyboard answer call you will get two

musical tune to alert you to the connect.  Any stations connecting to

your keyboard call when you are already connected to another station

will get a busy.  The actual number of connects allowed before busy is

given by the sysop command MCon.  MCon 1, for example, only allows an

incoming call to the keyboard to be answered if there are no other

keyboard connections.  MCon 0 prevents all incoming keyboard requests

(they get busy).  MCon N will allow N connections to the keyboard until

busy is returned.  When you type past column 72 you will get a beep for

each character.  Keep those lines less than 80 characters!  When a

station attempts to connect to your keyboard via the several ways

possible but is sent a busy the bell will also beep to alert you to

this.

Packet size

Data in packets is limited to 256 bytes.  Larger amounts of data in a

single packet will be discarded without causing the system to crash.

STACK Requirement

NOTE: NOT USED WITH EXPANDED OR EXTENDED MEMORY!

The stack space needed by the forward process has increased.  The

following line can be put in MSYS.OPT  (This may not be necessary):

STACK FORWARD 8000

A smaller value (maybe 7000) might work.  Try it if you want.  If the

system randomly crashed, make it 8000.

Device Errors

Device errors are trapped by MSYS (you would previously get the Abort,

Retry, Ignore? message).  The first occurrence of the error is given a

response of retry, the second ignore.  This will normally be caught by

particular function being performed which will terminate with an error

message, but MSYS should continue operation.

MSYS 1.14                                                   Page 122

Disk Full Message

If the disk is full when a message is sent to the system it will give a

msg to the connection station that the disk is full and then

disconnect.

MSYSTNC.#

If you use any tnc set up files be warned that the program waits 1 sec

after each line from the file is sent to the tnc to give it time to

think about it.

NOTE:

AEA tncs made after Jan. 89 can probably be treated like KPC-x tncs to

get them into kiss mode.  It should no longer be necessary to have a

special tnc file for them (but if it doesn't work without it, it is

suggested that the tnc file be used).  A good solution to the tnc to

kiss problem with MFJ 1270/4 TNC-2 compatibles is to install the JKISS

EPROM available from Tucson Amateur Packet Radio.  The EPROM forces the

tnc to ALWAYS be in the KISS mode, and no further action by MSYS is

needed.

MAIL BEACON

Messages to your IDCALL and to SYSOP are not displayed in the MAIL FOR

beacon.  You will see them on the screen.

LOG

Messages sent, forwarded, and killed get logged to MSYS.LOG.

A new log file record is being generated.  It is identical to the BBS

record except it has BYE in place of BBS.  It is generated when a user

exits the bbs by any means (disconnect, timeout, B cmd).  You may wish

to put an entry in the MSYSTODO.DAT file to rename MSYS.LOG

periodically (weekly?) as the log file CAN get very large, very fast!

MSYS 1.14                                                   Page 123

LOCKUPS

If the system hangs an attempt is made to reboot.  Thus you should make

sure you don't have a diskette in A: and that you have an appropriate

AUTOEXEC.BAT file set up.  It is strongly recommended that any

automatic restarts include the MUTIL 4 command to insure that the mail

file will be properly indexed.  If the mail files are not properly

indexed, MSYS will most likely hang on restart.

CHANNEL NUMBERS

The channel numbers use a rotational assignment rather than always

starting from 0

USER BIT SETTINGS:

$01   -  Expert user

$02   -  BBS

$04   -  Bad User Bit  (Disconnect immediately)

$08   -  Limited BBS  Permits only limited use of the bbs

$10   -  Remote sysop status

$20   -  Need to send WP Update

$40   -  Multiline packets (XF)

$80   -  Prevents use of the S command.  Users so marked can only read

messages.

$100  -  Lock user

$200  -  Authorizes use of the Upload command in the bbs.

$400  -  Must have user password

$800  -  YAPP Upload authorized

$8000 -  New User flag

NOTE: These settings may be added together.

CAUTION!  They ADD in HEXADECIMAL!  800 + 200 = A00

USER PASSWORDS

If a user has their flag bits set with the $400 bit on, when they

connect they will get the Usage stats msg line and must give the proper

4 characters just like a sysop would.  The password file for a user

must be in the PASSWORD directory and has a filename of the user's call

(no SSID, no file extension).  See also the reference to passwords

elsewhere in this document.

When a user logs on to the bbs for the first time (or after a long

while (see DELUDays), the file HELP/NEWUser.HLP will be sent to that

new user (if it exists) on the first connect (only).

MSYS 1.14                                                   Page 124

#

Command lines beginning with # on the BBS are ignored (they sometimes

come from KA Nodes).

MAX channels AND PORTS

Maximum for MAXCHANS is 50  (MSYS.OPT)

Maximum number of ports is 7 (numbered 0 to 6)

MSYS supports all IRQs (0-7) for serial ports.  I suggest the use of

IRQs 2,3,4 & 7 on PCclass computers and 3,4,5 & 7 on AT class and above

computers.  Use of 01 & 6 are not recommended but are supported in the

code.  Due to the use of these interrupts by other hardware

devices/functions results will generally be bad!  I really think that

the MS-400 (and similar) shared interrupt serial interface boards

should work properly.

END OF LINE BELLS:

When you type past column 72 you will get a beep for each character.

Keep those lines less than 80 characters!

WP SERVER:

Generation of messages to the white pages master server is included in

MSYS.  This sends the information users give in the N, NH, NZ and NQ

commands to the WP server when it is changed (or initially given) as

well as periodically.  Note that MSYS does not respond to messages to

WP but merely passes them on without modification.  WP messages are NOT

generated for users with any of the following characteristics: locked,

homebbs of ?, bbs  system, and/or baduser.

TESTING

You can connect two MSYS systems together through their serial ports by

the use of a NULL MODEM.

You can also connect a loopback by connecting pins 2 and 3 on the same

serial port.  This will allow you to see what the system is doing

without being on the air.

Automatic Back-up

of msys.msg and msys.usr has been removed.  If you still want to do

these backups once a day, put appropriate copy statements in your

MSYSHCL.DO file.

MSYS 1.14                                                   Page 125

SMTP

The \+\ filename include capability that is currently in the BBS S

command is also usable in SMTP with the PRompt command.  Both of these

are available only from the local keyboard.

Typing other than Y N C A or RETURN in response to More? in the BBS is

taken as a new command.

Titles Truncated

Titles of messages are truncated after 37 characters.

MAIL.BAK

To reduce disk read/write delays when a message is received and copied

to the MAIL.BAK directory, subdirectories are automatically created in

the MAIL.BAK directory.  100 messages are backed up in each directory.

This can substantially reduce the time it takes to make the backup copy

of incoming messages because much smaller number of file names must be

looked at to add the new file name to the directory.  The created

subdirectory names are of the form MSGxxx where xxx is the message

number divided by 100.  Thus in MSG025 messages 2500 to 2599 would be

found.  The msg /RESTore function from the BBS has been modified to

understand these changes.

TheNet & NetRom RS-232 Connections

RS-232 Connections of ports to tncs running NetRom and TheNet should

use > as the first character in the name given in the port statement in

MSYS.OPT.

To connect the computer to a single tnc per computer port, a straight

through cable is used (with a minimum of pins 2,3 and 7).  At the tnc

end, pins 10 and 23 must be jumpered.  DO NOT CONNECT PINS 10 or 23 at

the computer end!

In a multiple tnc connection using the standard diode matrix

interconnect,  the computer may be connected if you reverse pins 2&3

and pins 5&20 at the computer connection.

Using MSYS into a matrix with NetRom or TheNet equipped tncs can have

great advantages on a backbone system.  The tncs handle traffic on the

backbone not going to/from the MSYS system without any load being

placed on the MSYS computer.  MSYS then gains the advantage of a fast

reliable connection to the adjacent nodes.

Forwarding may be accomplished by connecting to the RS-232 connected

tnc or matrix using normal script files.  When using the NetRom Serial

protocol, remember the ">" character in the port statement!

MSYS 1.14                                                   Page 126

Type of Message

(the "TR" field)

The algorithm uses 3 pieces of information to determine the appropriate

TR to assign: the destination "callsign", the @"bbs", and the character

after the S in the Send command.

The to "callsign" is classified as one of the following:

0 - Looks like a real callsign

1 - A zipcode (5 digits)

2 - something else (like ALL, for example)

The @"bbs" is classified as one of the following:

0 - Looks like a real callsign

1 - none specified

2 - call of this bbs

3 - something else

The character after the S in the s command is classified as:

0 - B  (as in SB for send bulletin)

1 - P  (as in SP for send private)

2 - T  (as in ST for send traffic)

3 - none (as in just plain S)

4 - something else (SW for send weather bulletin, for example)

The following message types are generated:

BN   For  bulletins going to a specific bbs - these are

forwarded to a single station that handles that bbs.

PN  For private messages

TN  For NTS traffic

xN  For Sx where x is an "something else"

N   For individual non-private messages.

B$  For bulletins going with @route (like ARRL)

x$  For bulletins sent with Sx (x is "something else")

P$  For private bulletins going with @route

Note: [P]N below means that the type will be N is MAKEPrivate is off,

PN if MAKEPivate is ON (default).

TR Values Assigned to Messages

For messages going to Callsigns:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   [P]N xN

no bbs given             B$   PN   TN   [P]N x$

this bbs                 B$   PN   TN   [P]N x$

something else           B$   PN   TN   [P]N x$

MSYS 1.14                                                   Page 127

For messages going to ZIP Codes:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   TN   xN

no bbs given             B$   P$   TN   TN   x$

this bbs                 B$   P$   TN   TN   x$

something else           B$   P$   TN   TN   x$

For messages going to SOMETHING ELSE:

Character after S ->     B    P    T    None x (Other)

@BBS type:               ---  ---  ---  ---- ---------

callsign                 BN   PN   TN   BN   xN

no bbs given             B$   P$   TN   B$   x$

this bbs                 B$   BN   TN   B$   x$

something else           B$   P$   TN   B$   x$

Messages with second character of type N are forwarded to one place

(and then killed if AUTOKill is ON, the N is changed to F otherwise).

Messages with second character of type $ are flooded to all stations

that get the specified @BBS.  When sent to all such BBSs, the second

character gets changed to #.

Messages that come in with an R: line that contains the call (actually

HCAll) of this bbs are automatically held.  The R: line scan is

terminated by the first non-R: line found in the message.

Bulletins that arrive with a bid on the S command line are rejected if

their BID already exists in the BID file.  Bulletins are identified as

those messages that arrived via the SB command or S non-call.  All

messages other than bulletins are always received (never rejected).  If

a message identifier ($string) exists on the S command line AND

ACceptmid is ON, it is used.  In all other cases, a message identifier

is generated internally for all non-bulletins using the bbs call and

message number from the last R: line scanned.  If the message

identifier is found in the BID file and the message was not held as

noted above, the message is not saved (but it is acknowledged as being

received entirely by sending the > prompt when the ^Z is received).

The processing of internal message identifiers is not indicated in the

system identification line [MSYS-XXXX-H$].  If you set MIDchar to

something, then the character will appear between the H and the $.  For

example, if you set MIDchar to M, then the SID line will be [MSYS-XXXX-

HM$].  This will be sent to all  stations that connect to the BBS.

Setting MIDchar to a non-null character will also cause  the MID to be

sent during forwarding if the system to which you are forwarding has

MIDchar in its SID.

MSYS 1.14                                                   Page 128

Here are some combinations of the parameters to do selected processing:

For no BID (mid) processing on non-bulletins, set

BIDall OFF

ACceptmid OFF

don't set MIDchar

To emulate AA4RE MID processing (I think), set

BIDallON

ACceptmid OFF

MIDchar M

To use a MID if provided, or generate one otherwise, set

BIDall ON

ACceptmid ON

MIDchar M

Currently there is still a lot of discussion going on over  MIDS.

Hopefully I have give enough parameters so that MSYS can be used with

whatever standard wins out.

Automatic Message Holding

If you create a file called MSYSHOLD.DAT you can specify

characteristics of messages to hold automatically using parameters

similar to those used in the house cleaning file.  The available

parameters are:

TO=       wildcard representation of To callsign

FROM=     wildcard representation of From callsign

AT=       wildcard representation of @BBS

SIZE=     number that is size of msg

CONNECTED=  exact callsign of sending station (less SSID)

PORT=     port number msg is coming in on

BID=      This is useful in situations like the following:  Normally

ARRL           bulletins have BIDs similar to: ARLP037.  These bids

begin with          a letter.  BIDs automatically assigned to bulletins

that don't          have a bid usually begin with a digit.  So we could

use AT=ARRL BID=#*  To hold bulletins that probably don't have the

right bid.  Note         that #* is a wildcard specification that

matches anything that         starts with a digit.

TITLE=    If used, this should be the last parameter on the line.  If

the       value given for TITLE=  is found any where in the title of a

message (and all other specifications are met) the message

will           be held.

MSYS 1.14                                                   Page 129

Examples:

SIZE=2500           would hold any messages bigger than 2500 bytes

FROM=WA8BXN PORT=2  would hold any messages from WA8BXN that come in on

port 2

CONNECTED=W8XYZ     would hold all messages sent by station W8XYZ

(connected to the bbs)

AT=MSYS             would hold all messages with @MSYS

Note: the difference between FROM= and CONNECTED=; FROM is the from

call for the message (often supplied after < in the S  command) while

CONNECTED is the call of the station connected to the bbs sending the

message.

There is one other parameter that can be specified, a line  containing

only the letters BBS.  The lines following such a line in the file

would not apply to messages that are forwarded to you from another BBS.

For this use, a BBS is defined as a station that transmitted a [...$]

line.

NOTE:  Held messages are visible only to the SYSOP and sender.  They

are killable only by SYSOP.

Other options on the lines in MSYSHOLD.DAT:

* at the beginning of the line makes it into a comment  TEXT=filename

operand on a line When specified, if other tests are met, the text and

title of the msg will be compared with the contents of the given

filename .  If any of the words in the file match, the msg will be

held.  Case is ignored in the test. Example: TO=ALL TEXT=WORDLIST.1

would cause all messages going to ALL  to be compared with the file

WORDLIST.1.  Put one phrase  or word  per line in the file, enclosed in

either single or double quote  marks.  You may also put a number

between 1 and 255 at  the beginning of the line before the quoted

phrase to assign a weight to the word.  If no weight is given, the word

or phrase gets a weight of 255.  Here is a sample file of words:

'war'

"900"

"send check"

50 'send'

50 "check"

"That's it"

If the sum of the weights of these "words" found in the title or msg

text exceeds HOLdlevel (default 0) then the msg is held.  NOTE:

HOLdlevel is a new sysop command!  Caution: 'war'  would match  an

occurrence of the word 'forWARd' in a msg.  Its not a real problem as

it just means the msg will be held not killed off.

MSYS 1.14                                                   Page 130

DOS

If you are using MSDOS 3.3, you may need to put STACKS=0,0 in your

config.sys file.  MSYS has been run under MSDOS 5.0 and MSDOS 6.0 as

well as DRDOS 6.0 with no known problems or special commands other than

FILES=40 and BUFFERS=20.  With MSDOS 5.0 and MSDOS 6.0, you may chose

to use SMARTDRV.EXE instead of the buffers statement.  It all depends

on your personal preference and "RAM CRAM".

HEADERS

To create an ASCII file containing all the active message headers, use

the following sequence of commands on the bbs:

X 0       (turn off the More? message temporarily)

>msghdrs  (this is the name of the file to be created)

L$ 0      (or L 0 if you don't want to see the bids)

>         (close the file)

X 20      (restore the More? message)

KANTRONICS 3.0 EPROM Upgrade users:

I am told you will have to make a tnc setup file and include the

following:

INTFACE KISS

RESET

You may also need to include a line with one or more control-c

characters before these two lines in your file.

BBS ONLY ACCESS

Lets say you are running with ports 0 through 3 and want to make port 2

bbs only access with at most 1 connection allowed at a time (either

another BBS forwarding to you or you forwarding but not both at the

same time).  You also don't want to generate MAIL FOR beacon on port 2.

Use the following:

FB 2 25           turn off mail for beacon port 2

BP 4              port 2 bbs only

BM 2 1            max 1 connect port 2

MCF $B            no simultaneous forwarding on port 2

Note that BP and MCF both have port masks as arguments.

CONNECTS FROM NON-AMATEUR CALLSIGNS

Connects are no longer accepted from non-amateur callsigns  (like PK-

232 and NOCALL for example).  You can still make connections to non-

call signs.

MSYS 1.14                                                   Page 131

PASSWORDS

There are several PASSWORD types utilized in MSYS.

First, there is the remote SYSOP password.  The phrase for this is

stored in a file called MSYS.RMT in the C:\MSYS directory.  When the

/Sysop (CASE SENSITIVE!) command is sent to MSYS, it will pick four

characters from the MSYS.RMT file and send a prompt similar to:

Usage stats: Users=4 Msgs rcvd=10 Msgs sent=5 Max=8

to you.  You MUST respond with the EXACT character needed before SYSOP

privileges will be granted.

If you are running a remoted MSYS system (you DO go on vacation, don't

you?), you may automate that return process by having a file called

PASSWORD.RMT in your C:\MSYS\FWD directory.  When your remote system

sees the "Usage stats" message from your home system, it will

automatically respond with the proper characters.  Likewise, this is a

convenient way for your neighbor sysop to manage your system while you

are absent.

A third form of password protection is the USER password.  If the bits

for USER password have been set for a user, then you must have a

PASSWORD DIRECTORY.  The directory will contain a file for each user

with ONLY the callsign as the file name.  There are NO SSID's or

extensions used.  The sample directory and file name would look like

this:

C:\MSYS\PASSWORD\N8UDK

At a minimum, N8UDK's user bits would be set to $400.

A fourth password is the one used by the FTP function of TCP/IP.  It is

more fully described under the heading of MSYSPASS.DAT and can be found

on page 166 of the documentation.

A fifth form of the password is utilized in the modem access.

Discussion of the modem passwords used in the MSYSPASS.MOD file can be

found on Page 81 of the documentation.

MSYS 1.14                                                   Page 132

MSYS Utility Support Programs

(c) Copyright 1993 by HUB COMPUTERS, INC.

MUTIL.EXE

When you run MUTIL you will get a screen that is similar to:

==============================================================

MSYS Utilities 1.14 by WA8BXN (C) 1993 Hub Computers, Inc.

1    Change maximum number of messages

*    Restore messages from backup directory MAIL.BAK        ***

3    Print remote sysop phrase number table

4    Check Mail index file integrity

5    Test serial ports

6    Print forwarding tree

8    Run log file analyzer

9    Run SETUP program

10   Scan message headers to create/add to BBSLIST.DAT

11   Deletes old entries from BBSLIST.DAT and BBSTONTS.DAT  files.

12   Fix mail index file (can be hazardous to use!)

13   Process BBSLIST.DAT into BBSTONTS.DAT file

14   Scan BBSTONTS.DAT to build BBSTONTS.BIN (indexed)

15   Build list of archived messages

16   Printer status

17   MERge BBSLIST.DAT files

18   Check MSYSNODE.DAT integrity

19   Convert user files 1.05 & before <===> 1.06 and later

20   Sort calls in the BBSLIST.DAT file

21   Delete Old BIDS (and MIDS)

22   Specify file name to use in place of printer

99   Quit

Enter number of selection: _

***This function has been replaced by the /RESTore command in the BBS

MSYS 1.14                                                   Page 133

MUTIL Function 1 - Change max number of messages

Input file(s):  MSYS.MSG

Output files(s): MSYS.MSG

The maximum number of messages that can be stored in MSYS is determined

by the MSYS.MSG mail index file.  When MSYS starts running, it checks

to see if there is an existing MSYS.MSG file.  If there is one, it is

used (and contains the header lines of the messages that existed when

MSYS was last stopped).  If there is no existing MSYS.MSG file, then an

empty one is created, with a capacity of 100 messages.  This utility

function can be used to change the size of an existing MSYS.MSG file.

You can increase or decrease the size of the file.  You can't make it

smaller than the number of active messages.  The maximum number of

active messages has been increased to 2000.

NOTES:  There is a cost in memory requirements for each message

possible.  Each message slot costs 138 bytes of memory;  100 messages

(default maximum) thus takes 13800 bytes.  500 messages would take

69,000 bytes and 2000 messages will take approximately 270,000 bytes

(even if there weren't that many messages active).

VALUES I USE:  I run with a 500 message maximum size.  Normally I have

around 400 active messages.

DIALOG  WITH  FUNCTION: You are first asked for the name of the

MSYS.MSG file to modify:

Enter path to MSYS Mail Index file or just press return to use

MSYS.MSG: _

Normally you will want to just press the return key.  If the current

directory from which you are running MUTIL does not contain the

MSYS.MSG file that you want to modify, you can specify a different path

(or even a different file name if you have renamed MSYS.MSG for some

reason).

This function will next copy the specified MSYS.MSG file to a file

called MSYS$$$.MSG which can be used to restore MSYS.MSG if for some

reason this function fails.

Next you are asked if you have run the Check Mail function (function

4).  You should run Check Mail just before this function since

attempting to change the maximum number of  message slots can produce

very interesting (and usually bad) results if the original MSYS.MSG

file is corrupt.  If you respond N then  this function terminates and

you can then do the Check Mail function.  If you have just run Check

Mail, then press Y to continue.  Note that you do NOT press return

after either of these letters!

MSYS 1.14                                                   Page 134

If you responded Y, then the function continues, telling the current

maximum number of messages, the current number of active messages, and

some other information.  You are then asked for the new maximum number

of messages you want.  This number must be greater or equal to the

number of active messages.  It must also be less than or equal to 2000.

If the value you ask for is acceptable, the file will be changed to

accommodate the new number.

Next you will be asked to press a key to continue (try the return key).

This pause is put in the function so that you can see any additional

messages that were produced before the screen  is cleared.

MUTIL Function 2 - Restore Messages

MOVED INTO MSYS PROGRAM!

This function has been moved into the MSYS program with the SYSOP

command of /REST [msg#] which MUST be done while connected to the bbs.

MSYS 1.14                                                   Page 135

MUTIL Function 3 - Print Remote Sysop Phrase

INPUT FILE(S):  MSYS.RMT

OUTPUT FILE(S): Hardcopy printer output (optional)

This function prints the contents of the MSYS.RMT file (used  for

remote sysop verification).  The output of this function can be just

the screen or also to the printer.  The output consists of printing the

content of remote sysop phrase followed by a table that gives the

character at each position in the phrase.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output  (on  the  printer). Respond

Y or N as desired.

SAMPLE OUTPUT:

Sample phrase.

0   S

1   a

2   m

3   p

4   l

5   e

6

7   p

8   h

9   r

10  a

11  s

12  e

13  .

USAGE EXAMPLE:

If  you did the \Sysop command as a remote user to get sysop privileges

and got the following reply:

Usage stats: Users=7 Msgs rcvd=3 Msgs sent=6 Max=12   you would type

the following response:

pp e

MSYS 1.14                                                   Page 136

MUTIL Function 4 - Check Mail Index File Integrity

(CHKMAIL)

INPUT FILE(S):  MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE(S): MSYS.MSG

This function checks the integrity of the Mail index file MSYS.MSG.  It

checks for such things as message slots on both the free and active

lists, loops in either of these lists and  active message numbers that

do not have text files in the MAIL directory (and  vice-versa).  It

attempts to fix any errors if it can (and its generally pretty good at

doing it).

NOTE:  It will not delete any message text files in the MAIL directory

if they don't  have active headers.  They will be reported but you will

have to delete them yourself if you really don't want them.  You

probably won't really want them, but  you decide!

NOTE:  This function may be run in a batch file through MUTIL (without

having to give its function number interactively) by running:

MUTIL 4

SAMPLE NORMAL OUTPUT:

CHKMAIL - An MSYS Utility to verify mail index integrity

MSYS Mail Index Checker

head=0 free=1 next=2

Reading Index file:

There are 100 total message headers

Pass 1:

No cycles detected

No headers on both lists

All headers accounted for

No message files without headers

All active headers have files

Press a key to continue_

COMMENTS:  I would strongly suggest you run this function EVERY time

MSYS crashes for any reason (it should not crash often, if at all).  To

be on the safe side, many MSYS SYSOPS always run it when they start

MSYS.

MSYS 1.14                                                   Page 137

MUTIL Function 5 - Test serial ports

INPUT FILE(S):  None

OUTPUT FILE(S): None

This function provides a very basic test of serial interfaces.  It is

designed mostly to see if you got the addresses/int  numbers right for

your machine.  It is not intended to be a brutal test of their

operation!

DIALOG WITH FUNCTION:

After a table is displayed that lists the normal addresses for serial

cards and corresponding interrupts, you will be asked if your machine

has an 8086/8088 CPU chip in it.  If its a basic PC/XT compatible

system, respond Y.  If you have an AT (80286) or higher level machine,

respond with N.  If you have an 8088/8086 based system (PC class) then

it will look for ints from  2,3,4, and 7.  If you say not 8088/8086 (AT

class and above) it will look for ints from 3,4,5, and 7.  Depending on

your response, certain interrupt handlers will  be installed.  You will

then be asked for the address of the serial card you wish to check.

The answer you give must be in HEXADECIMAL.  A valid response might be

3F8, for example.  A simple check is made to see if there might be a

serial port at the address you specify.  It is easily fooled if you

specify the address of some other kind of I/O interface.

If  there seems to be a device at the address you gave, a quick check

will be made to see if it looks like an 8250 UART chip is used in your

serial interface.  Internal loop back tests cannot be done with the

newer UART chips.  If the test fails when you select internal loopback,

try the external loopback option.  This requires a connector that will

connect pins 2 & 3 together in place of the tnc you would normally

connect.  External loopback can be used to test all types of UART

chips.

Next you will next be asked for a baud rate to test at.  Here  you give

a decimal number as an answer.  It probably doesn't make much

difference what you give . (Don't try 0!).  I usually test at 1200

baud.

Now its your turn to type characters.  As indicated, press Esc  to

terminate the test.  Normal response should be something like:

Had IRQ 4 1 time(s),  A received

If you press the letter 'A' key.  If interrupts aren't working you

would see:

Received A without interrupt.

NOTE: This is not a performance test, just an address test.

MSYS 1.14                                                   Page 138

MUTIL Function 6 - Print forwarding tree

INPUT FILE(S):  MSYS.FWD

OUTPUT FILE(S): Printer hard copy (optional)

This function displays your forwarding file in two ways to tell you

what callsigns you forward to each bbs you forward to and also to which

bbs you forward each callsign.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

The first part of the output consists of lines like

AD8I ---> KA8Z WB8BII

which means that messages with @AD8I are sent to either KA8Z or

WB8BII(which ever one you can connect to first).

The second part of the output consists of lines like

KA3NVP <---   KA3NVP NTSNJ NTSNY NTSPA

which means that you forward messages with KA3NVP NTSNJ NTSNY and NTSPA

in some part of the @BBS field to KA3NVP.

NOTE:     To pause the display, you must use ^NumLock (^S  won't work).

To resume scrolling, press any other key.

MSYS 1.14                                                   Page 139

MUTIL Function 8 - Run Log File Analyzer

INPUT FILE(S):  MSYS.LOG

MSYS.MSG

OUTPUT FILE(S): Printer hard copy (optional)

This program produces a listing of BBS usage.

DIALOG WITH FUNCTION:

You  are  asked if you want hard copy output  (on  the  printer).

Respond Y or N as desired.

SAMPLE OUTPUT:

==============================================================

From Wed Mar 01 15:23:48 1989 to Mon May 229 00:57:03 1989

Station  Connects     Sends   Reads     Kills     Forwards

HCLean      0           0       0        27         0

N8UDK       3           1       1         0        12

NT8V        0           1       1         0         0

WA8BXN      4           0       0         1         9

WB8ZZS    168          82      31        24         0

LAST READ       MSG# TO   @BBS  Count  Title

May 18            1 TEST         1    test

Mar 23            7 TEST         2    test

May 28            2 TEST         3    test

Mar 02         8073 TEST         7    no title

MSGINs 84  MSGKILLs 97  MSGFWDs 21  MSGREADs 33 RESTARTs 331  QUITs 311

CONNECTS 175

The following messages were received but not killed:

1   2   3   20   21   8073

(These should be active messages on your BBS!)

==============================================================

NOTES:  This data is taken from my test system which uses  WB8ZZS as

its call (Thanks to my wife Ruth!) Many strange things happen on this

system as I am testing new software changes.

The first line of the report gives the period of time covered  by the

contents of the MSYS.LOG file that was read.

The next section of the report lists stations that were active on the

BBS and what they did.  The headings mean the following:

Station - Callsign of the station

Connects- Number of times the station connected (or the

SYSOP used the BB command).

Sends  -  Number of messages sent by this station

Reads  -  Number of messages read by this station

Kills  -  Number of messages killed by this station

Forwards- Number of messages forwarded to this station

NOTE:  The callsign HCLean indicates activity by the House Cleaning

routine.

MSYS 1.14                                                   Page 140

The next section lists messages that were read during the period,

sorted in increasing order of number of reads .  The meaning of the

headings is:

LAST READ - Date the message was last read

MSG#    - The number of the msg on the bbs

TO      - The "TO" station callsign of the message

@BBS    - The "@" field of the message.

Count   - The number of times the message was read

Title   - The title of the message

NOTE:  Private messages are not shown,  only bulletins.  This

section is useful to see what interest there is in the various types of

messages.

The next section provides overall counts of various activities:

MSGINs      The number of messages that were sent to the BBS

MSGKILLs    The number of messages that were killed

MSGFWDS     The number of messages forwarded by the BBS to other

systems.  Note: If a bulletin is forwarded to 4

different systems, it counts as 4 forwards

MSGREADs    Total number of Read commands used on BBS

RESTARTs    Number of times MSYS was started

QUITs       Number of times MSYS was gracefully shut down using ^F4

or Quit command.  Restarts - quits is the number of

crashes for whatever reason.

CONNECTS    Number of user connects to BBS + SYSOP BB cmds

The final section of the report gives a list of message numbers that

were assigned to messages sent to the system but not killed.  Normally

these message numbers should be active messages on the BBS.  If they

aren't, they may be messages to restore since they got lost somehow.

MSYS 1.14                                                   Page 141

MUTIL Function 9 - Run Setup Program

INPUT FILE(S):  MSYS.OPT

OUTPUT FILE(S): MSYS.DEF

This function sets default initialization parameters.  IT MUST BE RUN

BEFORE YOU CAN RUN MSYS THE FIRST TIME and any time you change

MSYS.OPT.  It is documented elsewhere in this document.

DIALOG WITH FUNCTION:

You are asked if you want hard copy output (on the printer).  Respond Y

or N as desired.

MSYS 1.14                                                   Page 142

MUTIL Function 10 - Scan Message Headers

to create/add to BBSLIST.DAT

INPUT FILES:   MAIL.BAK\MSG*.DAT (or specified directory) BBSLIST.DAT

(if it exists)

OUTPUT FILE:   BBSLIST.DAT

This program scans backup mail message files that are (or were

originally found) in the MAIL.BAK directory to determine what BBS

stations have been "seen" (using the R: lines added by each BBS as it

forwards a message).

DIALOG WITH FUNCTION:

You will be asked to give the name of the directory that contains the

messages to be scanned or to press return to use the MAIL.BAK directory

directly.

NOTE: This function may be run from a batch file by using either:

MUTIL 10

or:

MUTIL 10 dirname

In the first form (MUTIL 10) it will be assumed that the input files

are in the MAIL.BAK directory.  In the second form (MUTIL 10 dirname)

the files are assumed to be in the given directory name.

OUTPUT FILE FORMAT:

The first line of BBSLIST.DAT gives the date this function was run to

create or add to the file.

The remaining lines in the file consist of the following fields:

callsign of bbs

location of bbs

postal code of bbs

number of times this call has been seen

Note: If you choose to edit this file, be sure to keep things in the

same columns.  Also note that the callsigns are lined up using the

digit of the callsign.  The list is sorted  by  digit in callsign,

letters after the digit, and then letters before the digit (pretty much

callbook order).  Calls that do not begin with A, K, N, or W are

grouped together at the end of the list, with an * in the first

position in the record (non-US calls).

MSYS 1.14                                                   Page 143

MUTIL Function 11 - Deletes Old Entries

From BBSLIST.DAT and BBSTONTS.DAT

This utility function will prompt the user to determine what the oldest

record date you wish to keep in a yymmdd sequence.  It will then edit

the files to delete BBS records older than the specified date.  It

might be a good idea to then run MUTIL Function 10 to add any new BBS's

to the list before running MUTIL Function 13.

MUTIL Function 12 - Fix Mail Index File

***** WARNING ***** This function can DESTROY the mail index file

INPUT FILE(S): MSYS.MSG

MAIL\MSG*.DAT

OUTPUT FILE:   MSYS.MSG

This function allows manual inspection, checking and modification of

the mail index file.  It is a utility I wrote to see what destroyed a

mail file and perhaps fix it.  There is one function that may be useful

to most SYSOPS, that allows you to specify the next message number to

be assigned to incoming messages.  You may want to do this if you were

running some other bbs and want to continue using message numbers from

where you left off.  Personally I would suggest that you don't do this

but rather start with the default value 1.  But its up to you!

NOTE:  Make a copy of your existing MSYS.MSG file before using this

function!

NOTE:  Most of the checking and repair functions have been automated in

the CHKMAIL function (MUTIL Function 4).

DIALOG WITH FUNCTION: You are first asked for the name of the MSYS.MSG

file to modify:

Enter MSYS message header file name or simply press return for

MSYS.MSG: _

Normally you will want to just press the return key.  If the current

directory from which you are running MUTIL does not contain the

MSYS.MSG file that you want to modify,  you can specify a different

path (or even a different file name if you have renamed MSYS.MSG for

some reason).

To change the number that will be assigned to the next message, use the

following procedure:

Select F8 (Manual changes)

Select F5 (Change Next)

Enter new value for next message number to be used.

Select F10 (Return to previous menu)

Select F9 (Save changes and quit)

MSYS 1.14                                                   Page 144

RENUMBERING MESSAGES

All active messages may be renumbered using MUTIL function 12.

You should renumber your messages when you reach about message number

64000.  As message numbers increase above this they will wrap around

back to 0 if you don't use this function.  Some of the L command

options will not work properly if newer messages have numbers smaller

than older ones.  To renumber the messages, go through the following

steps:

1 - Start the MUTIL program

2 - Type 12 and return

3 - Press return to accept use of MSYS.MSG

4 - Press F8 for manual changes

5 - Press F6 for Renumber messages

6 - Type new starting message number (1 is good) then

press return

7 - Press F10 to return to previous menu

8 - Press F9 to save changes

9 - Press return to go back to main menu

10 - Type 99 and return to exit

MSYS 1.14                                                   Page 145

MUTIL Function 13 - BBSLIST.DAT to BBSTONTS.DAT

INPUT FILES:  BBSLIST.DAT

ZIPCODES.DAT

STATES.DAT

ROUTES.DAT

OUTPUT FILE:  BBSTONTS.DAT

This function builds a list of BBS calls (and optionally zip codes) and

the corresponding xx designator to which they should be forwarded.

If you select processing US BBS calls by state the data found in

BBSLIST.DAT is scanned for state and zip code.  The state is identified

as being something after a comma in the location field that matches the

list of state two letter codes found in  the STATES.DAT file.  If only

one state is found and it corresponds to the zip code found, then the

BBS call is automatically assigned to  xx  where xx is the state

abbreviation.  If either of these pieces of information is missing, or

they do not match the test fails and you are asked to decide what state

the bbs belongs in.

DIALOG WITH FUNCTION:

You will be asked if you want to include state routings for ZIPCODES.

You should reply Y the first time you are creating the BBSTONTS.DAT

file.  Otherwise reply N.  If you delete BBSTONTS.DAT for some reason,

you should reply Y the first time you create a new one.

If the state of the BBS cannot be automatically determined, you will

get a screen that looks like this:

==============================================================

WB8ZZS   KIRTLAND OH        44026   (4)

State/ZIP check failed - Zipstate OH

Enter two letter state code then <Return key> or press <Escape key>

then <Return key> to ignore or just press <Return key> to use OH

=============================================================

The first line of this screen is the line from BBSLIST.DAT that is

being considered.  The next line gives the state that corresponds to

the zip code (if available).  The third line gives the guess that can

be accepted if you just press return.  If the guess is good, hit

return.  If you want to use a different state, type the two letter code

for that state and then press return.  If you want to omit this call

from the output file, press some non-letter key before pressing return.

Space bar, return works well.

In this example, the state could not be identified because  there is no

comma between the city and the state.  (It would be nice if everyone

used standardized R: lines!) The guess of OH was made from the zip code

alone.

MSYS 1.14                                                   Page 146

When the US calls have been processed, you will be asked if you want to

process non-US calls.  If you have earlier chosen not to process US

calls by states/zipcodes, the US calls will be processed with all the

other calls if you choose to process non-US calls.  This feature is

useful for non-US hams that handle all US messages as just going to the

US.

If you choose to process the calls at this point, you will be presented

with screens that  list an entire entry from the BBSLIST.DAT file.

Following this line will be the callsign of the BBS identified from the

line.  Using the call prefixes found in the ROUTES.DAT file, a guess

will be made as to what routing should be used for the given call.  You

may then choose to ignore the call, to use the routing obtained from

the call prefix (if any) or to type the complete route you desire.

A sample screen might look like this:

=============================================================

*  WB8ZZS   KIRTLAND OH        44026   (4)

Callsign:    WB8ZZS

Possible route: USA.NA

Press <Escape key> <Return key> to ignore

Type desired routing then press <Return key> or

just press <Return key> to use USA.NA

=============================================================

BBSTONTS.DAT FILE FORMAT:

This  is an ASCII file containing in bytes 1 to 9 a BBS  call  or

zipcode (left justified, possibly with wildcard * indicator), and the

corresponding designator to which messages for that BBS  call or zip

code should be forwarded beginning in position 10 .  Example lines:

89*     NV

WB8ZZS  OH

VK1XX   AUS.AS

NOTE:  Bulletin routing designator (like ARRL) should NOT appear in

this file.

NOTE:  The file is sorted in ascending order using the entire record as

key.

NOTE:  There should be no duplicates of calls/zip codes found in the

first 9 bytes of the records.

MSYS 1.14                                                   Page 147

MUTIL Function 14 - BBSTONTS.DAT to BBSTONTS.BIN

INPUT FILE(S):  BBSTONTS.DAT

OUTPUT FILE(S): BBSTONTS.BIN

This function builds an index of the calls in  the BBSTONTS.DAT file,

placing the index at the beginning of the BBSTONTS.BIN file.  After the

index, the contents of BBSTONTS.DAT is copied to BBSTONTS.BIN.

DIALOG WITH FUNCTION:

This function does not require any keyboard input.

NOTE:  The format of the resulting file is such that no attempt should

be made to edit it.  With care, BBSTONTS.DAT may be edited and the

result then processed by this function.

NOTE:  The file BBSTONTS.BIN is used by MSYS.  The files BBSTONTS.DAT

and BBSLIST.DAT are NOT used directly used by MSYS and may be stored on

a diskette, if desired, when MSYS is running.  I just keep them in the

normal MSYS directory.

MUTIL Function 15 - Build List of Archived Messages

This function will look at messages found in a given directory (in

MAIL.BAK format) and construct a file of the message headers.  The

result is a file of all your messages like doing a L command on the BBS

for every message that has passed through your system.  You can use the

grep function on it to find messages with some particular string of

characters in their header line.  I use it to find a message when I

think to myself "I remember there was a message about ...  one time".

Then knowing the message number you can go back to your archive

diskettes and extract the complete text.

MUTIL Function 16 - Printer Status

This function provides a very simple printer test to see if its

connected and can be initialized.

MSYS 1.14                                                   Page 148

MUTIL Function 17 - MERge BBSLIST.DAT Files

This function takes as its input your current BBSLIST.DAT file (which

it first renames to BBSLIST.BAK) and another file you specify  in the

same format as BBSLIST.DAT) and merges the two files into one called

BBSLIST.DAT.  I use this function to merge the BBSLIST.DAT file that

some of the users of MSYS have sent me.  This allows me to include in

my file some bbs calls that normally would not have been seen in

messages passing through my system.

MUTIL Function 18   Check MSYSNODE.DAT Integrity

This function performs the following functions:

MSYSNODE.DAT viewing and repair program

0     Printer echo toggle (now OFF)

1     List active neighbors

2     List all neighbor slots

3     List active known destinations

4     List all known destinations slots

5     Data consistency check

6     View given neighbor or destination

99     Exit

Any output from the program will be directed to the screen unless the

printer has been toggled on.

MUTIL Function 19 - Convert User Files

1.05 & Before To 1.06 and later

Version 1.05 and earlier used a different format for storing user

files.  This utility probably will be of little use to most current

users.

MSYS 1.14                                                   Page 149

MUTIL Function 20 - Sort BBSLIST.DAT File

This utility will sort the callsigns in the BBSLIST.DAT file.  It will

sort the bbs callsigns based on number, suffix after the number, prefix

to the number and generate a new file called BBSLIST.NEW  At the

conclusion of this operation, it will suggest that you rename the files

in the proper manner.  This program is quite processor intensive, so it

is suggested that if you are in a hurry, that you do not run this

program concurrently with any other program that may use processor

cycles, such as DesqView.  It will function properly under DesqView,

but its sort time is lengthened by the number of cycles allocated to

the other program(s).

MUTIL Function 21 - Delete Old BIDs (and MIDs)

This function deletes old BIDs/MIDs from the file BIDLIST.DAT.  It asks

for the number of days worth of bids that should be kept.  Run it

periodically (maybe once a week when you do your hard disk backups).

The bid file can hold up to 6500 entries  The more it has, the slower

it works.  The maximum number of BIDs in the BID file is settable.

MUTIL Function 22 - Specify Printer File Name

The file you specify will be opened and will receive any output that

would normally go to the printer.  Any existing data in the given file

will be lost.  If you want to abort this  function, simply press return

when asked for the file name.

MSYS 1.14                                                   Page 150

SAMPLES OF MSYS FILES

(c) Copyright 1993 by HUB COMPUTERS, INC.

MSYS.OPT:

port 0 at $3E0 int 7 speed 9600 name 145.01 RADIOSPEED 1200 TNCTYPE 1

port 1 at $2E8 int 7 speed 9600 name 223.68 RADIOSPEED 1200 TNCTYPE 1

port 2 at $3E8 int 7 speed 9600 name 145.05 RADIOSPEED 1200 TNCTYPE 1

port 3 at $2F0 int 7 speed 9600 name >AKRNODE RADIOSPEED 9600

numchans 15

bbs alias akrbbs

bbs call wb8bii

id call k8eiw

id every 30

forward call wb8bii-1

answer call k8eiw-15

kanode call wb8bii-3

sysop name Don

sysop qth Cuyahoga Falls

sysop zip 44223

process id window 2

monitor outgoing on

window display color

window 0 size 1 1 80 13

Window 6 size 1 23 48 25

make

MSYS 1.14                                                   Page 151

MSYS.DO:

*----------------CALLSIGN STUFF-------------------------------

HCA WB8BII.#NEOH.OH.USA.NOAM

HOST K8EIW

NRI AKRON

NRC K8EIW

MYT K8EIW

myip 44 70 4 5

mya akrbbs

*-----------------TNC PARAMETERS------------------------------

monsf on

check 0500

frack 0 12

frack 1 12

frack 2 12

frack 3 5

resp 1

tac 2

txt 5

txd 0 35

txd 1 30

txd 2 35

txd 3 5

paclen 236

maxfr 0 4

maxfr 1 7

maxfr 2 4

maxfr 3 7

*----------Net Default Window Size-----------

ND 7

PP 255

DPORTS $00

DIGI OFF

*-----------------NET/ROM PARAMETERS -------------------------

*------------Path Quality of heard nodes port 0

PQ 0 120

*------------Path Quality of heard nodes port 1

PQ 1 120

*------------Path Quality of heard nodes port 2

PQ 2 120

*------------Path Quality of heard nodes port 3

PQ 3 200

*------------Min. DESt Quality to record

WQ 120

*------------Broadcast Interval 30 minutes

BIN 1800

*------------Mail-for beacons turned off if 1 (port MASK)

NOBE $0

*------------KaNode Turned off

KPORTS $0

*------------TCP ON PORTS 0,1,2,3

TPORTS $F

*------------NetRom PORTS 0,1,2,3

NPORTS $F

MSYS 1.14                                                   Page 152

*------------Enable BBS in NetNode

NETB 1

*------------Enable Connect Comand in NetNode

NETC ON

*------------Net Full Function Hours

NETFBBS 0-23

*------------NetNode On

NETN ON

*------------Network Connect for BBSs Only (Off permits ALL)

NETO 0

*------------Adds New Neighbors Heard if On

AUTOn off

*------------Obsolescence Count Initial

OCI 6

*------------Obsolescence Count Min for Broadcast

OCM 5

*------------Network Time to Live

NTL 15

*--------------------BBS PARMS--------------------------------

TIMezone GMT

*------------Kill on Forward OFF

AUtokill OFF

*------------Turns on AX25 Level 2

AX25L2 on

*------------Turns On AX25 Level 2 backoff (RNR)

AX25b $F

*------------Turns on BBS Node broadcast to network

BBSNODE 1

*------------Turns on BID function

BID 1

*------------Forward Time interval (- = every x minutes)

fti -10

*------------Display Time for F2 Key (0=infinite)

f2timer 0

*------------FBB Compatibility/Compression On

FBBC 2

*------------FBB Forward Group Number of Messages

FBBG 5

*------------Network FBB Compatability/Compression On

NETFBBC 2

*------------Network FBB Forward Group Number of Messages

NETFBBG 5

*------------House Clean at x Minutes after 00:00

HCL 310

*------------Heard List turn off point if Dispatches < #

HE 10

*------------Initializes Printer before each File Printed (0=NO)

IN 0

*------------Makes Messages Private by default

MAKEP ON

*------------Maximum Bulletin Age

MAXB 10

*------------Allow Connects to SYSOP Keyboard

MCON ON

*------------Min Memory for Memory Low Busy Msg

MSYS 1.14                                                   Page 153

MINM 32000

*------------Turns on Message Trailer Function

MS on

*------------Establishes New User default bits

NEW $00

*------------Turns on CallSign Server

PC ON

REQQTH ON

REQDIR ON

REQFIL ON

*------------Screen Save Setting

SC 10

*------------Turns on SYSOP Verification

VERIFY on

*----------------NET/ROM NEIGHBORS----------------------------

NODE ADD AKR:K8EIW-1 3 192

NODE ADD #AKRU:K8EIW-2 3 200

NODE ADD #AKRU2:WB8BII-8 3 200

NODE ADD AKROH:WB8BII-10 3 200

AUTODEST On

NETNODE ON

NODE RESTORE

DESt REP AKR:K8EIW-1 K8EIW-1 3 0 230

*-------------ARP ROUTING TABLE-------------------------------

arp add ka8tef-4 -2 44 70 16 1 /24

arp add ka8z-4 -2 44 70 4 4

arp add k8eiw-4 -2 44 70 4 32

arp add wa8bxn-2 -2 44 70 4 6

arp add wb8lvp-2 -2 44 70 4 57

arp add n8nvw-1 0 44 70 5 14

*------------Default TCP/IP ROUTE--------------

DRoute k8eiw-4 3

MSYS 1.14                                                   Page 154

MSYS.OTD:

<<< RADIO CLUB MEETING TONIGHT @ 8:00 PM - CITY BUILDING >>>

***

MSYSK.OTD

< Please try  ### W8UPD  Gateway 145.01 <-> 223.68 >

MSYS.REP:

NTS* ##### @2 @1    COMMENTS (NOT IN FILE!!!)

440* * @1 NO8M      This sends ALL 440xx Zip Code to NO8M

44001 * @1 N8ITP    This redirects 44001 Zip Code to N8ITP

44003 * @1 KB8AST   This redirects 44003 Zip Code to KB8AST

4403* * @1 N8ITP    This redirects 4403x Zip Code to N8ITP

44030 * @1 KB8AST   This redirects 44030 Zip Code to KB8AST

The result of the above is that all ZIP coded mail

not specifically addressed to the 44001,03 or 3x

Zip Codes will go to NO8M.

441* * @1 W8GRG

44107 * @1 NO8M

46* * @1 KA8TEF     This redirects all Zip Code 46xxx to KA8TEF

* OHNET @1 ALLOH

* ARL @1 ARRL

* ALLUSA @1 ALLUS

* USA @1 ALLUS

* K8EIW @1 WB8BII

* WB8CQR @1 W8GRG

* WB8BXN @1 WA8BXN

NOTE:  THERE SHOULD BE NO BLANK LINES IN THE FILE!

MSYS.RMT:

PASSWORD TEST PHRASE

MSYSBBSB.DAT:

1  KA8Z

2  NO8M

3  KB8DRN

7  W8GRG

8  WA8BXN

9  NG8P

10

16 AD8I

17 W8CQK

18 KA0FPO

19

22 WA8DCE

23 K1LT

31 W8UPD

NOTE: 31 is the MAXIMUM number of BBS's you can forward bulletins to!

MSYS 1.14                                                   Page 155

MSYSFWD.DEF:

FILE CONTENTS  COMMENTS  (DO *NOT* INCLUDE COMMENTS IN FILE!)

F1 w8upd       This is a straight forward out port 1 to a

w8upd          local PBBS.  Only Messages @ W8UPD will be forwarded.

-----          Necessary dashes! (5 minimum - no maximum)

F3 kq8m

@k8eiw-2       Forward through an adjacent TheNet node.

.c #cledx      Connect command given to TheNet node.

+to            Necessary triggers.

?from

?with

.c kq8m

#120           This extends the wait for connect to 120

+to            seconds after the .C KQ8M command was issued.

?with

?Busy

kq8m

-----

R3 ka0fpo      This is a forward with a request for Reverse Forward

@k8eiw-2

.c #cledx

+to

?from                    This example shows how to use the

?with                    include file KA0FPO.REV.  The .rev

.c kq8m-1                extension indicates ka0fpo may

#120                     reverse forward from this system.

+to

?with

?Busy

.c ka0fpo

#120

+to

?with

?Busy

$c:\msys\fwd\ka0fpo.rev

------

f1 wa8dce

!25                      This indicates that WA8DCE will never be

$c:\msys\fwd\wa8dce.rev  forwarded to, but will reverse forward all

--------                 messages.

MSYS 1.14                                                   Page 156

MSYSHOST.NET:

44.70.4.4 [ka8z] ka8z-4 ka8z.ampr.org        #Tom Barnes,canton,ka8z

44.70.4.6 [wa8bxn] wa8bxn wa8bxn.ampr.org    #Mike Pechura,lake

county,@wa8bxn

44.70.4.39 [kb7yw] kb7yw kb7yw.ampr.org #Fred Peachman, brookfield,

@wb8lvp

44.70.16.1 [ka8tef] ka8tef-4 ka8tef.ampr.org #Phil,Republic Ohio,

@ka8tef

NOTE:  ALL ENTRIES FOR EACH HOST MUST BE ON ONE LINE!

MSYSPASS.DAT:

guest * 3 c:/msys/public

anonymous * 3 c:/msys/public

ka8z tom 7 c:/msys/public

mike wa8bxn 7 c:/msys/public

fred kb7yw 7 c:/msys/public

ka8tef phil 7 c:/msys/public

Typical MSYSTNC.1

(AEA PK-232)

VHF OFF

HB 300

AWLEN 8

PARITY 0

CONMODE TRAN

TRACE OFF

HID OFF

BE 0

passall off

RAWHDLC ON

HP OFF

PP ON

KISS ON

HOST ON

NOTE:  The MSYSTNC.x files are associated with the corresponding

tnctype.x specified in the MSYS.OPT file.  The above file COULD be

named MSYSTNC.2 depending on your configuration.  Only the Kantronics

TNCs are fixed at MSYSTNC.0  Be sure to specify the proper parameters

for your tnc to put it into the KISS mode.

The TNC-2 clones, such as the MFJ 1270B or 1274, may benefit greatly by

using the JKISS EPROM which forces the tnc to be in the KISS mode all

the time.  The JKISS file is included on the distribution disk or is

available as an EPROM from the Tucson Amateur Packet Radio

organization.

MSYS 1.14                                                   Page 157

INFORMATION FOR ADVANCED USERS

REPLACEABLE DIRECTORY NAMES

Beginning in MSYS 1.04 you can specify alternate directory names to be

used by MSYS in place of the standard ones by using appropriate DOS set

commands.  The directory names that can be replaced and their default

values are:

Default             Set DOS variable

Dir Name            to replace

--------            ----------------

FILES               MSYS:FILES

FWD                 MSYS:FWD

GREP                MSYS:GREPDIR

HELP                MSYS:HELP

MAIL                MSYS:MAIL

YAPP                MSYS:YAPPDIR

As an example, assume you have a RAM disk at drive E: and have set up a

directory called HELP on E: and copied the normal help files to this

directory.  You can then issue the following DOS command before

starting MSYS:

set MSYS:HELP=E:HELP

and now MSYS will get the help files from the E:HELP directory.

NOTE:  You must be very careful in using a RAMDISK since its contents

are easily lost.  Also, you should probably not us a RAMDISK that takes

space out of the normal 640K address space.

NOTE:  You are not limited to using RAMDISKS in specifying the disk

and/or directory to be used for the above directories.  You could

specify floppy and/or hard disks as well.

To establish a separate directory that is "GReppable", use the DOS SET

command similar to this:   SET MSYS:GREPDIR=C:MSYS\GREP    Don't forget

to create the C:\MSYS\GREP directory!

MSYS 1.14                                                   Page 158

Replaceable File Names.

You can also replace the following file names of files normally found

in  the "MSYS" (default) directory.  Judicious choices can speed up

operation and/or let you make use of multiple disk drives.  Poor

choices can be a disaster!

Default File   Set This DOS

Name           Parameter to replace:   Notes

-------------  --------------------- -----

MAIL.BAK       MSYS:MAIL.BAK          (6)

MAIL.BIN       MSYS:MAIL.BIN

BBSTONTS.BIN   MSYS:BBSTONTS.BIN      (1)

MSYS.FWD       MSYS:MSYS.FWD          (2)

MSYS.HRD       MSYS:MSYS.HRD          (3)

MSYS.LOG       MSYS:MSYS.LOG          (3)

MSYS.MSG       MSYS:MSYS.MSG          (4)

MSYS.USR       MSYS:MSYS.USR          (4)

MSYSHOST.NET   MSYS:MSYSHOST.NET      (5)

MSYSBBSB.DAT   MSYS:MSYSBBSB.DAT      (2)

MSYSPASS.DAT   MSYS:MSYSPASS.DAT      (5)

Notes:

1. Very good candidate for ram disk (read only)

2. Good to put on ram disk if you are careful

3. These can be made NUL if you don't want them

4. If you like to live very dangerously, consider putting them on ram

disk.  Otherwise keep them on hard disk .  Updated frequently

during normal operation, they are very important files.

5. Good for ram disk if you run tcp/ip much.  Read mostly.

6. A very strong suggestion would be to put this file on a different

disk partition.  Backup message files tend to get forgotten and

expand at a very high rate.  If they fill up the primary MSYS disk,

MSYS will no longer be able to accept messages.  If the MAIL.BAK

directory is on a different drive, the most that can happen when

that drive is full is that MSYS just will not make backup copies of

the incoming messages.

Example 1.  Lets move the mail.bak directory to the D: drive.  It is

appropriate to put the SET command in the AUTOEXEC.BAT so that it will

happen on a reboot.  You would put the statement:

SET MSYS:MAIL.BAK=D:\MAIL.BAK

in the AUTOEXEC.BAT file.  MSYS will then look for the mail backup

directory on the D: drive.

CAUTION  MUTIL Functions always assume the C:\MSYS directory path!

When doing MUTIL functions, you MUST specify the changed path!

Example 2.  Assume you still have a ram disk at E: and have copied the

normal contents of BBSTONTS.BIN to that disk.  You could do the

following DOS command:

MSYS 1.14                                                   Page 159

SET MSYS:BBSTONTS.BIN=E:BBSTONTS.BIN

to cause MSYS to read the file from your RAMDISK.

NOTE: With all these changes you can make, you can configure a lot in

terms of files now.  Be careful of file sizes when thinking about where

to put things.  MSYS.LOG grows endlessly as does the MAIL.BAK

directory.  These two are VERY good candidates for another drive so as

to NOT accidentally run out of disk space and bring the bbs to a

humbled state.

NOTE:  The programs in MUTIL assume the files are where they should be

and totally ignore any replacements you may have made of file names

and/or directory names.

NOTE: If you are not absolutely sure of what you are doing, don't (do

anything with these facilities).

NOTE: Before you try any of this, MAKE BACKUPS OF EVERYTHING!

MSYS 1.14                                                   Page 160

MSYS MSG FILE FORMAT:(For the adventuresome only!)

(If you know what you are doing, this will make some sense to you)

int head;   /* slot number of head of active list */

int free;   /* slot number of head of free list */

int next;   /* next message number to be assigned */

/* Each slot has the following definition: */

struct msghdrdef {

unsigned  mmsgnr;

unsigned char mmsgtype[3];

int  mmsgsize;

unsigned char mmsgto[7];

unsigned char mmsgfrom[7];

unsigned char mmsgatbbs[39];

unsigned char mmsgdate[7];

unsigned char mmsgtitle[40];

long mmsgbits;

unsigned char mmhbbs[8];

unsigned char mmsgtries;

unsigned char mmsgbid[15];

int  mmsglink;

};

/* Note: word alignment is forced for all ints & longs */

MSYS 1.14                                                   Page 161

ROUTES.DAT FILE FORMAT:

This is an ASCII file that contains callsign prefixes and routings.

The callsign prefix begins in the first byte.  Comparisons are made

with the BBS call for the length of the prefix from this file.  The

routing field begins in byte 9 (a tab character can be used to get

here).  Here are some sample entries:

4X    ISR.AS

9M2   MYS.AS

A     USA.NA

The idea here is that incoming messages for a given bbs will get the

routing appended to the @BBS field.  Thus if someone sent a message to

4X1ABC  @  4X1XYZ  the @ field would become @4X1XYZ.ISR.AS

It is expected in your forward file you would either have provisions to

route the country part (ISR) to some bbs,  or the continent part (AS)

to some bbs.  Thus to route foreign messages, you might just have all

the continent designators (like EU,  AS, NA  etc.) in your forwarding

file or you can be more selective. Remember that you can have some

countries along with the continent routings in your forward file.  The

part of the @BBS from left to right that is found in the forwarding

file is the one that is used.  If you do a lot of direct forwarding to

various countries that can be accommodated; if you just dump all

foreign stuff  on a single station that is easy too.  Feel free to

modify the contents of ROUTES.DAT as you see fit;  I have just included

country prefixes for countries for which I seen message headers pass

through my system.

MSYS 1.14                                                   Page 162

The following program may be used to create a BBSLIST.DAT file

from the BBSTONTS.DAT file supplied on distribution disk:

10 OPEN "BBSTONTS.DAT" FOR INPUT AS 1

20 OPEN "BBSLIST.NEW" FOR OUTPUT AS 2

30 IF EOF(1) THEN CLOSE:END

40 LINE INPUT #1,A$

50 I=1:C$="":  WHILE  MID$(A$,I,1)<>" "  :C$=C$+MID$(A$,I,1):

I=I+1: WEND

60 D$=MID$(A$,35,6)

70 H$=MID$(A$,10,25)

90 IF LEFT$(D$,2)="99" THEN 30

100 X$=LEFT$(C$,1): IF X$>="0" AND X$<="9" THEN 120

110 X$=MID$(C$,2,1): IF X$>="0" AND X$<="9" THEN C$=" "+C$

120 PRINT #2,C$;TAB(8);H$;TAB(74);D$

130 GOTO 30

You may use a text editor to extract the program from this file. The

result of this program is put in a file called BBSLIST.NEW.  It may be

renamed to BBSLIST if you don't have one already.  Or you can merge

your file with this one by using the following commands:

copy BBSLIST.DAT+BBSLIST.NEW TEMP

del BBSLIST.DAT

ren TEMP BBSLIST.DAT

Then run MUTIL function 20 to sort the file and discard the duplicates

from the BBSLIST.NEW file.

MSYS under WINDOWS 3.1

To run MSYS in the background mode with Windows 3.1 you must make a PIF

file called MSYS.PIF.  The secret is when you get ready to save the

MSYS.PIF file, show the path as:

c:\windows\msys.pif

NOT

c:\msys\msys.pif.

Try using the variable 130 for background and 110 for foreground to

give the best results for timesharing.

MSYS 1.14                                                   Page 163

MSYS under DesqView

To run MSYS under DesqView 386 Version 2.4 on a 386 or a 486 machine,

try these values in the "Add Program" (AP) setup:

O (for Other) F1 (to specify Program Information)

PROGRAM NAME = MSYS

Keys To Use = MS (or your choice!)

Memory Size = (as big as you can make it - try 620K)

Program  = C:\MSYS\MSYS.EXE (be sure to specify proper drive and

sub-directory)

Directory = C:\MSYS

Writes text directly to screen     [Y]

Displays graphics information [N]

Virtualize text/graphics (Y,N,T)   [Y]

Uses serial ports (Y,N,1,2)        [Y]

Requires floppy diskette           [N]

F1 for advanced options

Starting Height:                   25

Starting Row.                       0

Starting Width.                    80

Starting Column:                    0

Close on exit (Y,N,blank)          [ ]

Uses its own colors                [Y]

Allow Close Window  command        [Y]

Runs in background (Y,N,blank)     [ ]

Uses math  coprocessor             [Y]

Keyboard conflict (0-F):           [0]

Share   CPU   when   foreground    [Y]

Share EGA when foreground/zoomed   [Y]

Can be swapped out (Y,N,blank)     [ ]

Protection level (0-3)             [0]

To run MSYS under DesqView on a 386 machine, try these values in the

"performance" section:

foreground 3

background 2

common memory 15

dos buffer for ems 5

optimize communications Y

allow swapping of programs N

manage printer contention N

If you choose to use QEMM as your memory manager, the following line in

your CONFIG.SYS file should give satisfactory results:

DEVICE=C:\QEMM\QEMM386.SYS RAM ROM

NOTE:  A WORD OF CAUTION:

When using QEMM-386, the /ST:M option in the DEVICE=  line will cause

MSYS to not load properly.  There may be other options which will also

cause grief.  These suggestions are not necessarily the optimum

settings for your system,  but they have been known to work.

MSYS 1.14                                                   Page 164

SYSOP COMMANDS RELATED TO TCP/IP

MYIpaddr

Takes four integers in the range 0 to 255.  It sets the IP address

that the system will respond to.  Example: MYI 44 70 4 6  If

MYIpaddress is not specified, or set to all zeros, the TCP/IP

support will be inactive.

MYTcpcall

Takes a callsign (with SSID) that will be used when sending arp

and TCP/IP frames.  It can be the same as the ID  call in most

cases.

TElnet

Takes an IP address and optional server number as parameters.  A

symbolic IP address may given to be looked up in the file

MSYSHOST.NET.  Default server is 23.  This command is used to

establish keyboard to keyboard connection with a TCP/IP station.

Examples: TE [44.70.4.10]

te hsp   (hsp would have to be defined in MSYSHOST.NET)

TElnet operates much like C# normally used to connect to AX.25

stations.  ^Cd is used to disconnect.

TPorts

Takes a port mask value as an argument.  This number is used the

same way as in the other similar commands: each port corresponds

to a port (port 0 is the rightmost bit) and when the bit is set to

1 for a given port that port is available for TCP/IP use

(particularly ARP broadcasts).

ARP

Used without an argument lists the known callsign/IP address

correspondence determined by TElnet connects.  An optional

argument, CLEAR, may be used to erase all of the ARP entries.

FTP

Takes a host-id as an argument (it can be either an IP address in

the square brackets or a symbolic IP address).  It establishes a

connection to the FTP server of the specified system.  FTP is the

File Transfer Protocol used in TCP/IP.

SMTP

Allows local access to the SMTP server (mainly for test purposes).

Note: The following servers are currently implemented in MSYS:

7     Echo

9     Discard

21    BBS

23    Telnet

25    Simple Mail Transfer Protocol

87    Local keyboard

MSYS 1.14                                                   Page 165

Files related to TCP/IP

MSYSHOST.NET

This is an ASCII file you create with an editor (such as the MSYS

EF command).  Each line in the file begins with an IP address that

includes the periods but not the square brackets around it.

Following this there is at least one space and/or tab character,

then the list of symbolic names (or aliases, or nicknames, what

ever you want to call them).  You may place comments on the line

by beginning the comments with a pound sign (#).  Example entry

line for this file:

44.70.4.6 bxn mike wa8bxn # kirtland, oh

This line would allow "bxn" "mike" or "wa8bxn" to be used as

arguments in the TElnet and FTP commands. Note that WA8BXN here

just happens to look like a callsign.  The actual callsign (and

SSID) associated with a particular IP address is determined by ARP

(address resolution protocol) broadcasts.  These symbolic names

you define in this file simply relieve you of having to remember

(and type) complete IP addresses.  Nothing more, nothing less!

See sample on page 157 for MSYSHOST.NET file.

MSYSPASS.DAT

This file contains the user names and passwords for FTP users.

Each line contains 3 or 4 items: username password access

directory Username is the name the user will use to log in.  It

can be anything for a given user.  It could be their callsign or

something else.  The password can also be anything.  If you put *

as the password in the file for a particular, any password given

by the user will be accepted.  The third entry on each line,

access, is a number that says how much access the user will have

to your system: 1 means can only read files, 3 means read and

write (but not replace or delete) and 7 means read, write, replace

and delete.  You shouldn't set 7 for any user since the passwords

given by a user can be monitored by others.  The last item which

should always be given is the root directory that the user will

have. They may never go closer to the real root than what you give

here.

MSYS 1.14                                                   Page 166

Here are some examples:

anonymous * 1 files      This allows user anonymous to read

any files in MSYS/FILES and any of

its subdirectories.  Any password can be

given for this user.  Anonymous with *

for the password is an expected

"standard" entry for  TCP/IP systems to

give access to totally public files.

Include it in your system.

apple runtime 3          This allows user apple (who must give

password runtime) read/write access

to your entire disk (DEFAULT DIRECTORY

IS THE REAL ROOT DIRECTORY!)

USER ACCESS level chart

Level 1: CD DIR GET PWD QUIT

Level 3: PUT MKDIR

Level 7: PUT (to replace existing file) DELE RMDIR

If you are going to allow level 7 access (i.e., can do everything) I

would suggest you also assign a directory that doesn't allow access to

any needed files.  For example, you might set up the following entry in

the MSYSPASS.DAT file:

WA8BXN MIKE 7 /MSYS/FILES/WA8BXN

You will have to create a directory WA8BXN in the files directory.  Now

stations can log in with WA8BXN MIKE and do anything in

/MSYS/FILES/WA8BXN and any directories found in this directory.

If you set up ANONYMOUS * 1  /MSYS/FILES then the user ANONYMOUS can

read anything put in the WA8BXN subdirectory of FILES (as well as in

FILES itself) but not write or otherwise change anything.  The rest of

your files should be pretty well protected.

MSYS 1.14                                                   Page 167

FTP COMMANDS:

When you use the FTP command to connect to another system you can type:

HELP to get a list of the supported commands.  They currently include:

CD dirname     To move around in the directory structure

CD             with no arguments gives the current directory

DELE filename  To delete given file

DIR filespec   gives a directory listing

GET filename   downloads a the given file

MKDIR          makes a directory

PUT filename   uploads the given file  Syntax is:

put [localfilename] remotefilename

If the optional localfile name is omitted it is assumed to

be the same as the remote file name.

PWD            gives the current directory

QUIT           disconnects

RMDIR dirname  removes the given directory

FTP function   will give an estimate on the number of bytes

transferred during GET and PUT commands.

MSYS 1.14                                                   Page 168

IP Routing:

What does this mean?  Its sort of like digipeaters for the TCP/IP

world.  If a TCP/IP user sets you up as a route for IP frames going to

a particular IP address MSYS will now repeat them if it has (or can get

through an ARP request broadcast) a  known route to the DEStination IP

address.  Consider the following fictitious example: W9AAA [44.60.1.1]

is in Indiana and wishes to connect to W3AAA [44.80.1.1] in

Pennsylvania but can't do it directly.  W9AAA can reach W8AAA

[44.70.1.1] in Ohio who can in turn reach W3AAA.  W9AAA will set up the

route to [44.80.1.1] to be W8AAA (an  MSYS system).  When attempting to

connect to W3AAA (using Telnet,  for example) will send out a SYN IP

frame (connect request) to callsign W8AAA but with an IP address of

[44.80.1.1], the IP address of W3AAA.  W8AAA upon hearing this will

check its ARP table to see if it knows how to get to [44.80.1.1].  If

the entry is not there, W8AAA will do an ARP broadcast on all the ports

enabled for TCP/IP.  W3AAA will respond on one of them and this will

put an entry in the ARP table.  Now when a SYN is heard from W9AAA it

will be repeated by W8AAA and thus heard by W3AAA.

To use IP ROUTING yourself in MSYS you use the arp add command.  When

adding a given IP address you would put the call of the TCP/IP station

you want to use as an IP repeater in place of the call that belongs

with the IP address.  You can still use normal AX.25 digipeaters (up to

8) to reach the TCP/IP station.  If the path from W8AAA to W3AAA in the

above example was not direct,  then the following arp entry could be

done at W8AAA, assuming that K8AAA is a normal AX.25 digipeater: arp

add W3AAA port# 44 80 1 1 K8AAA

NOTE:  For more on TCP/IP refer to The KA9Q Internet Software Package.

In the TCP/IP related ARP command, use: -2 as the port number for

routing through the network interface.

TCP/IP connects are rejected if not enough memory or MCON is exceeded.

MSYS 1.14                                                   Page 169

EDITVIEW (ED Command)

(c) Copyright 1993 by HUB COMPUTERS, INC.

Commands    The ED command used from the MSYS command mode can be used

to edit small text files.  The file is limited to records each 80 bytes

or less in length.  By default up to 200 records may be in the file.

This can be modified by giving the maximum number of lines after the

file name.  The intent of this editor is to do little things without

having to take down the system to run a more powerful editor.  These

might include editing message texts, the forward file, etc.

ED is a full window editor (it adjusts its operation to fill the window

it is run in).  It must be run in a window that is 80 characters wide.

Thus using the default window definitions it can be run in the normal

command window (0) or you can use the CMdwindow command to first go to

window 4 (the one you get when you press F4) and then use the ED

command.  You will then be using the entire screen for your editing.

ED is pretty much a what you see is what you get editor.  There are two

modes of operation.  When you start the editor you are in full screen

or edit mode in which changes you make to the screen are also made to

the file.  In this mode you get a line at the bottom of the window that

gives such information as the current line number and the name of the

file being edited.  The other mode is command mode in which you give

commands to be done by the editor.  To enter command mode press the

Escape key.  The bottom line will change to Edit Cmd: and await your

command.  The command you type will appear on this bottom line.  To get

a list of available commands type Help.  You should see something that

looks like the following:

Find - locates given string .  Example:  F wa8bxn

Notes: Not case sensitive .  F10 from edit mode is find

same string again.

Insert - puts blank line at beginning of file

QUIT - Abandons edit

SAVE - Saves file and exits

DELete  -  Delete specified # of lines from  file  beginning with

current line

Return key in edit mode with insert on inserts blank lines

Press a key to return to edit

Use of the keypad to move around in the file is what is expected of the

labeled keys, with the following enhancements.  If you are somewhere in

a line and you press the Home key,  you go to the beginning of the

current line.  If you are the beginning of the current line and press

Home,  you go to the beginning of the screen.  If you are at the

beginning of the screen, pressing Home takes you to the beginning of

the file.

MSYS 1.14                                                   Page 170

EDITFILE (EF Command)

(c) Copyright 1993 by HUB COMPUTERS, INC.

The EF command used from the MSYS command mode can be used to edit

small text files.  The file is limited to records each 80 bytes or less

in length.  By default up to 200 records may be in the file.  This can

be modified by giving the maximum number of lines after the file name.

The intent of this editor is to do little things without having to take

down the system to run a more powerful editor .  These might include

editing message texts, the forward file, etc.  This isn't a full screen

editor but rather a line editor that was influenced in its design by

the UNIX editor.  Plain and simple  nothing fancy is found in it.  To

begin editing, use the command EF filename when you have the cmd:

prompt.  The filename can contain drive and path information.  If the

file does not exist you will be given the option to create one by that

name.  To create a new file this way, use the Append command.  Type the

lines of the new file and then a line with a period at the beginning to

exit append mode.  Then use the  commands Write and Quit (each followed

by the return key) and you have created a new file.  All of the

commands can be shortened to the first letter of the command, except

for the DElete command which requires at least the first two letters.

Here is information about the commands:

#  (i.e.,  a line number) makes that line the current line and also

displays it

+ # Makes the current line # more than it is now .  If # is omitted,

then the current line is incremented by one.

- # Same as + # except current line number is decremented.

<ESCAPE> Using the Escape key followed by the Return key abandons edit

with no changes made to the file (unless you used the Write command

yourself).

<RETURN> Pressing the Return key alone displays the next line and makes

it the current line.

Append enters input mode, placing the following lines of input  AFTER

the current line.  A line beginning with a period terminates append

mode.

Bottom makes the last line the current line and displays it.

Change can be used to replace part of the current line.  The first

non-blank character after the command is the quote character.  The

operands are quote-character string-to-replace quote-character

replacement-string quote-character .  For example,

C /YXZ/abc/

would replace XYZ in the current line with abc.  The line is

printed after it is displayed.

MSYS 1.14                                                   Page 171

DElete can be used to delete the current line.  If a number is

specified as an operand, it is the number of lines to delete beginning

with the current line.

Down moves the current line down the number of lines specified (or 1 if

none is specified).

Find uses the first non-blank character after the command word as the

beginning of the search string.  The search is from the current line on

in the file.  Case is NOT important.  If a match is found the line is

displayed and that line becomes the current line.  If no match is found

the current line remains unchanged.

Help displays the list of commands.

Insert is like Append except that the new lines are placed BEFORE

the current line.

Near displays the contents of the file 5 lines before and 4 lines after

the current line which remains unchanged.

Print with no operands displays the current line.  Give 1 operand it

displays the line with that number.  Given 2 operands (separated with a

blank or comma) it displays from the first to the second line number.

The current line number remains unchanged.

Quit is used to exit the editor after the file has been written.

Top makes the first line (numbered 0) the current line and displays it.

Up decrements the current line number by the number given or 1.

View displays the current line and the 9 lines that follow it.  The

current line remains unchanged.

Write causes the file to be written to disk with any changes that have

been made.

^F is used to search again for the same search string give with last

use of Find.

To start the editor, type EFile filepathname [# of lines max].

Examples:

ef msys.fwd 250

ef mail/msg25.dat

efile a:f1.c

MSYS 1.14                                                   Page 172

Epilog

Notes on sending me messages with questions/suggestions/etc.:  I do try

to respond to your personal messages.  Keep in mind I do get quite a

few at times.  If you want an answer from the author, you need to send

the question to the author personally!  Messages that contain demands

(add this feature or correct this bug immediately and put out a new

release or I won't use your program) generally are saved in the bit

bucket.  For a free program, please be considerate!  Try to put just

one thought/question in each message.  If you bundle up a whole bunch,

I don't respond until I can address all your issues.  If you send

follow-up messages, please give a sentence or two summary of the

problem so that I may understand what you are talking about.  Just

saying "I tried what you suggested and it still doesn't work, anything

else to try?" doesn't give me much of a clue about the situation.

Although you may have only one problem you are dealing with and can

easily remember its entire history,  I don't have that luxury here!  If

you find what you think is a bug,  please try to tell me how to

reproduce it, what you are seeing, and what you think should be

happening.  The harder the problem, the longer it takes me to work on

it and respond.  When you run into any sort of problem, simplify

everything as much as possible.  Remove all statements from config.sys,

autoexec.bat, msys.opt and msys.do that are not essential to reproduce

the bug.  Believe me, this will probably help you find at least half of

the problems and their solutions yourself.

Use and Enjoy!

MSYS 1.14                                                   Page 173

INDEX

ARP, 33, 53, 165

!, 99                                 arp, 98

!timestring, 99                       ARP ADD, 34

ARP CLEAR, 33

#, 33, 90                             ARP DEL, 34

ARP REP, 34

$POOL, 59                             ATtended, 34, 101

AUTODESt, 34, 111, 117

*, 73                                 AUTOEXEC.BAT, 77, 159

*** done, 96                          AUTOHold, 34

*comment, 33                          AUTOKill, 34, 128

Automatic Back-up, 125

+, 91                                 Automatic Message Holding, 129

Automatic Password Reply, 120

.FWD, 93                              AUTONode, 35, 111, 117

.REV, 93                              AX25 CHECK, 20

AX25 FRACK, 20

/, 74                                 AX25 MAXFRAMES, 20

/RESTore, 10                          AX25 RETRIES, 20

/Sysop, 120                           AX25Backoff, 35

AX25L2v2, 35

<, 74

<Esc>, 65                             BADuser, 35

BBS, 165

>, 18, 74, 126                        BBs, 35

BBS ALIAS, 19

?, 65, 67, 91                         BBS CALL, 19

BBS callsign, 56

@, 91                                 BBS ONLY ACCESS, 131

@BBS, 71, 72                          BBSLIST.DAT, 143, 144, 149

@home, 33                             BBSNode, 36

BBSTimeout, 36

\+\, 41, 126                          BBSTONTS, 107

BBSTONTS.BIN, 106, 107, 148, 159

^A, 71                                BBSTONTS.DAT, 47, 144, 146, 147,

^Z, 71                                148

BDigi, 36

A, 66                                 BElloff, 36

ABortfwd, 33                          BID, 71, 99, 128

ACceptmid, 33                         BIDall, 36

ACOUSTIC COUPLED MODEMS, 84           BIDLIST.DAT, 36

ADDUser, 33                           BINterval, 37

Alt-F1, 65                            BInterval, 117

Alt-F2, 65                            blank lines, 99

Alt-F3, 65                            BMaxcon, 37

ANONYMOUS CHECK-INS, 83               BOOT, 37

ANSWER CALL, 20                       BPorts, 37

APpend, 33                            BText, 37, 55

ARChive, 33                           BUDCall, 35

archived messages, 148                BUDCalls, 37, 52

MSYS 1.14                                                   Page 174

BUDList, 37                           DESt DEL, 42, 112, 117

BUFFERS, 12                           DESt REP, 41, 112, 117

BUSY, 37                              DEStination, 98, 112

Bye, 66                               DEStination NODe, 110

Device Errors, 122

C, 66, 117                            DFree, 42

C script, 39                          digipeaters, 110

C#, 40                                DIGipeats, 42

Call/@BBS replacement facility,       DIRectory, 42

95                                    Discard, 165

CALLSIGN SERVER, 77                   DISconnect, 42

Callsign Server, 56                   Disk Full Message, 123

CALLSRV1, 77                          DOcmdfile, 42

CALLSRV2, 77                          DOS, 131

CALLSRV3, 77                          DOS 3.3, 14

callword, 82                          DOS SET, 67

Cancel, 65                            Download, 66

CC, 66                                DPorts, 43

CD ROM, 77                            DRDOS 6.0, 118

CGA, 60                               DRoute, 43

Change maximum number of              DRoute DELete, 43

messages, 134                         DRSI CARDS, 26

CHANNEL NUMBERS, 124                  DTimedate, 43

CHAnsta, 38                           DUmpbbs, 43

CHEck, 38

Checksum error, 104                   Echo, 165

CHKMAIL, 137                          EDit, 43

CL B#, 38                             EDITFILE (EF Command), 171

Clear To Send, 80                     EDITVIEW (ED Command), 170

CLrscrn, 38                           EFile, 43

CMdwindow, 38                         EMsghdr, 44

COLOR, 63                             END OF LINE BELLS, 125

CONference, 10                        EU, 83

CONFIG.SYS, 12, 77                    EUser, 44

COPy, 10, 38                          EXPANDED MEMORY, 119

CRetries, 38                          EXTENDED, 119

Ctrl/F2, 65

Ctrl/F4, 57, 65                       F:HAMBASE, 77

Ctrl/F5, 65                           F1, 65

Ctrl/F6, 65                           F2 Time, 44

CTS, 39, 80                           F6, 65

F8, 65

DATABASE HELP, 86                     FA, 103

Database.0, 67, 86                    FB, 102

datagram, 110                         FBB, 10, 82, 96

DBADD, 86                             FBB COMPATIBLE COMPRESSED

DEADletter, 41                        FORWARDING, 103

DEBug, 41                             FBB COMPATIBLE FORWARDING, 102

DELEfile, 41                          FBB ON THE PHONE, 82

Delete Old BIDS, 150                  FBBC, 10, 44, 88

DELUDays, 41                          FBBG, 10

DELUSer, 41                           FBBStimes, 45

DesqView, 164                         FCall, 45

DESt, 64                              FF, 102

DESt ADD, 41, 112, 117                FFile, 45

MSYS 1.14                                                   Page 175

FIle, 45                              INitpr, 48

FILES, 12, 158                        IP, 61

Files related to TCP/IP, 166          IP Routing, 169

files=40, 7                           IPNCheck, 48

Fix mail index file, 144              IPNFrack, 48

FMsg, 45                              IPNRetry, 48

FOrward, 45                           IPTtl, 48

FORWARD CALL, 20, 116                 IRQs, 125

Forward File Description, 88

FORWARDING DISPLAY, 100               J, 116

FORWARDING NOTES, 99                  j-com, 77

FORWARDING ON THE MODEM PORT, 82      JBbs, 48, 67

FQ, 103                               JDigipeat, 48, 67

FRACK, 62                             JGateways, 48, 68

FRAck, 46                             JH, 67

FREE, 46                              JHeard, 48

FREQUENT PROBLEMS, 12                 JKanodes, 49, 68

FS, 102                               JKISS EPROM, 123

FTIme, 46                             JMsys, 49, 68

FTP, 165                              JNetrom, 49, 68

FTP COMMANDS, 168                     JTcp/ip, 49

FTPgm, 46                             Justheard, 116

FUllduplex, 46

Function key usage, 32, 65            K, 68

FWD, 158                              K*, 68

fwd directory, 99                     K@, 68

KANODE CALL, 19

G, 87                                 KANTRONICS 3.0 EPROM, 131

GEtmsgotd, 46                         KEYBOARD, 122

GMT, 62                               KEyboard, 49

GR, 67                                KF, 68

GREP, 158                             Kill, 68

GRep, 47                              KILLproc, 49

grepdir, 67                           KM, 68

GX, 67                                KPorts(MASK), 49

KT, 68

H, 67

HApnstat, 47                          L, 68

HCAll, 47, 128                        L #, 69

HCL, 108                              L"string", 69

HCLean, 47, 140                       L$, 69

HEADERS, 131                          L<, 69

HEard, 47                             L>, 69

HELP, 15, 158                         L?, 69

HF Forwarding, 101                    L@, 69

Hierarchical, 98                      LCount, 50, 100

HIERARCHICAL FORWARDING, 105          LH -, 68

HOLdlevel, 47, 130                    LINKED, 122

HOStname, 47                          lists, 68

HReplace, 47                          LL #, 68

LM, 68

I, 67                                 LN, 68

ID, 48, 67                            LO, 68

ID CALL, 20                           Local keyboard, 165

ID EVERY, 20                          LOCKkbd, 50

MSYS 1.14                                                   Page 176

LOCKUPS, 124                          MONPorts, 52

LOG, 123                              MONSframe, 52

log file analyzer, 140                MONUframe, 53

LOGclose, 50                          MONxx, 53

LT, 68                                More?, 126

LU, 69                                MS-400, 125

LUsers, 50                            MSDOS 5.0, 118

LW, 69                                MSDOS 6.0, 118

LY, 69                                MSgtrailer, 53

MSYS COMMAND LINE, 11

M, 69                                 MSYS Command Summary, 27

MAIL, 15, 158                         MSYS DIRECTORY LAYOUT, 15

MAIL BEACON, 123                      MSYS MSG FILE FORMAT, 161

MAIL.BAK, 15, 58, 126, 159            MSYS Network NODe, 110

MAIL.BIN, 15, 159                     MSYS NODe Forwarding, 101

MAKEPrivate, 50, 127                  MSYS SYSOP Commands, 33

MASK, 62                              MSYS.BBS, 15, 95, 99

MASK INFORMATION, 24                  MSYS.DEF, 17

MAStersrv, 50                         MSYS.DO, 77, 80, 101, 114, 121,

MAX, 125                              152

max hops, 55                          MSYS.EXE, 15

MAXBullag, 50                         MSYS.FWD, 12, 51, 95, 159

MAXCHANS, 125                         MSYS.HCL, 108

MAXFrames, 50                         MSYS.HRD, 12, 159

maximum number of messages, 134       MSYS.LOG, 159

MAXRead, 50                           MSYS.MSG, 134, 159

MCForward, 51                         MSYS.OPT, 12, 16

MCOn, 51                              MSYS.OPT FILES, 80

MDir, 51                              MSYS.OTD, 155

MEMory, 51                            MSYS.REP, 95, 155

MEMORY LOW BUSY, 118                  MSYS.RMT, 63, 120, 132, 136, 155

MEMORY MANAGEMENT, 118                MSYS.USR, 159

MERge, 8, 51, 94, 149                 MSYS:BBSTONTS.BI, 159

MEXpert, 52                           MSYS:FILES, 158

MFJ KISS MODE, 14                     MSYS:FWD, 158

MHclear, 52                           MSYS:GREPDIR, 158

MIDchar, 52                           MSYS:HELP, 158

MINmem, 52                            MSYS:MAIL, 158

MISsing, 52                           MSYS:MAIL.BAK, 159

MISSING TEXT TO Message, 101          MSYS:MAIL.BIN, 159

MODE MONO, 8                          MSYS:MSYS.FWD, 159

MODEM, 18, 80                         MSYS:MSYS.HRD, 159

MODEM SUPPORT, 79                     MSYS:MSYS.LOG, 159

MONBframe, 52                         MSYS:MSYS.MSG, 159

MONCC, 52                             MSYS:MSYS.USR, 159

MONCF, 52                             MSYS:MSYSBBSB.DAT, 159

MONIFrame, 52                         MSYS:MSYSHOST.NET, 159

MONITor, 52                           MSYS:MSYSPASS.DAT, 159

MONITOR BFRAMES, 21                   MSYS:YAPPDIR, 158

MONITOR IFRAMES, 20                   MSYSB.EXE., 118

MONITOR OUTGOING, 21                  MSYSBBS.DAT, 88

MONITOR PORTS, 21                     MSYSBBSB.DAT, 12, 85, 93, 155,

MONITOR SFRAMES, 21                   159

MONOCHROME, 63                        MSYSFWD.DEF, 8, 12, 51, 88, 93,

MONOutgoing, 52                       95, 96, 156

MSYS 1.14                                                   Page 177

MSYSHCL.DO, 109                       NETNode, 54, 81, 111

MSYSHOLD.DAT, 83, 129                 NETOnly, 54, 117

MSYSHOST.NET, 56, 157, 159            NETRead, 54, 117

MSYSK.OTD, 155                        NetRom Serial protocol, 18, 126

MSYSMOD.MSG, 84                       NETTalk, 54, 117

MSYSPASS.DAT, 132, 157, 159, 166      Network time to live, 55

MSYSPASS.MOD, 81, 83, 132             NETYapp, 54

MSYSTNC.#, 123                        NEWUser, 54

MSYSTNC.1, 18, 157                    NH, 70

MSYSTNC.2, 81                         NInfo, 54, 117

MSYSTNC.x FILES, 81                   NNode, 55

MSYSTODO.DAT, 13, 62, 121             NOBEacons, 55

MUTIL, 12, 107                        NOBUlls, 55

MUTIL 4, 12, 13                       NODe, 55, 110, 111, 112, 113

MUTIL Function 1, 134                 NODe ADD, 55, 112, 117

MUTIL Function 10, 143                NODe DEL, 55, 117

MUTIL Function 11, 144                NODe DELete, 112

MUTIL Function 12, 144                NODe REP, 55, 112, 117

MUTIL Function 13, 146                NODe REStore, 55, 117

MUTIL Function 14, 148                NODe SAve, 55, 117

MUTIL Function 15, 148                NODe Scripts, 91

MUTIL Function 16, 148                NODe SEnd, 117

MUTIL Function 17, 149                NODETimeout, 55

MUTIL Function 18, 149                NOEMM, 11, 119

MUTIL Function 19, 149                NOEXMEM, 11

MUTIL Function 2, 135                 NOHEARD, 11

MUTIL Function 20, 150                NON-AMATEUR CALLSIGNS, 131

MUTIL Function 21, 150                NONODE, 11

MUTIL Function 22, 150                NORAMHDRS, 11

MUTIL Function 3, 136                 NPorts, 55, 116, 117

MUTIL Function 4, 137                 NQ, 70

MUTIL Function 5, 138                 NRCall, 55, 111, 116, 117

MUTIL Function 6, 139                 NRId, 55, 117

MUTIL Function 8, 140                 NRS, 18

MUTIL Function 9, 142                 NTS traffic, 68

MUTIL.EXE, 7, 133                     NTtl, 55, 117

MYAlias, 53                           NUMCHANS, 19

MYBbs, 53                             NZ, 70

MYCall, 53

MYIpaddr, 53, 165                     obsolescence, 56

MYKnode, 53                           OCInit, 56, 117

MYTcp, 53, 111, 116, 165              OCMin, 56, 115, 117

N, 70                                 P, 70

NAME, 17                              Packet size, 122

NBdcast, 53, 117                      PAClen, 56

NDwindow, 53, 117                     password, 15, 82, 83, 132

neighbor NODe, 110                    PASSWORD DIRECTORY, 132

NET NODe Parameters Summary, 117      PASSWORD.RMT, 63, 120, 132

NETBbs, 53, 117                       PATh, 56

NETCon, 54, 117                       PC, 56, 77, 78

NETFBBC, 10, 54, 117                  PF, 70

NETFBBG, 10, 54, 117                  PForward, 56

NETFBBStime, 54, 117                  PIng, 56

NETMax, 117                           PMessage, 56

MSYS 1.14                                                   Page 178

poll, 96                              RS-232 Connections, 126

Port speed, 80                        RUn, 59

POrts, 56

PPersist, 57                          S, 71

PQuality, 57, 117                     SAM (c), 77

PRInt, 57                             SAMPLES OF MSYS FILES, 151

Print forwarding tree, 139            SB, 71

Print remote sysop phrase number      SCreensave, 59

table, 136                            SELECTIVE FORWARDING:, 97

printer, 150                          SEnd, 60

Printer status, 148                   servers, 165

PROcesses, 57                         SET, 159

PRTmsgs, 57                           Set, 159

PUBLIC, 15                            SET TZ, 14

PWindow, 57                           SETUP, 9

setup, 142

QEMM, 14, 164                         Simple Mail Transfer Protocol,

QUIT, 57, 65                          165

SLottime, 60

R, 70                                 SMTP, 60, 71, 98, 126, 165

R*, 71                                SMTP FORWARDING, 98

R+, 71                                SNow, 60

RADIOSPEED, 18                        SOrt, 60

Radiospeed, 80                        SSID, 116

RDir, 57                              ST, 71

RE, 70                                ST RESET, 61

Readbackup, 57                        STACK, 22, 122

Register 27, 81                       STACKS, 119, 131

REMOTE SYSOP, 63, 120                 STatus, 60, 101

REMove, 57, 112                       Status, 61

REMove DESt, 117                      SUspendtrace, 61

REName, 58                            SYSOP, 71

RENUMBERING MESSAGES, 145             SYSOP NAME, 23

REPLACEABLE DIRECTORY NAMES, 158      SYSOP QTH, 23

Replaceable File Names, 159           SYSOP ZIP, 23

REPlaces, 58

REPly, 71                             T, 72

REQDIR, 58, 75                        TABs, 61

REQFIL, 58, 75                        TACK, 61, 117

REQMaxfil, 58                         TBdelay, 61, 117

REQQTH, 58, 75, 77                    TCP/IP, 53, 62, 111, 116, 165

REQxxx SERVERS, 75                    TCP/IP FORWARDING:, 97

RESPonsetime, 58                      TElnet, 61, 165

RESTore, 58                           Telnet, 165

RETries, 58                           Test serial ports, 138

REVERSE FORWARD, 94                   TESTING, 125

REVERSE FORWARDING, 95                TFrames, 61

Reverse forwarding, 96                TImezone, 62

RH, 70                                Titles Truncated, 126

RM, 70                                TNatime, 62, 117

RN <msg#>, 70                         TNC-2, 63

ROUTES.DAT FILE FORMAT, 162           TOdo, 62

Routing, 110                          TPorts, 62, 165

RP, 70                                Trace25, 62

RS, 71                                TraceIP, 62

MSYS 1.14                                                   Page 179

TREtries, 62, 117                     YU, 73

TRUncate, 62                          YW, 73

TS, 62

TSR, 77

TTimeout, 62, 117

Tucson Amateur Packet Radio, 123

Tuscon Amateur Packet Radio, 157

TXDelay, 62

TXTail, 63

TYpe, 63

Type of Message, 127

U, 72

UPload, 72

USER BIT SETTINGS, 124

USER Commands, 66

USER PASSWORD, 124, 132

Users, 63

V, 72

V1SIT, 83

VErifysysop, 63

VIdeo, 63

VIDRAM, 118

visit, 83

W, 72

WAtch, 63

WBackgrnd, 63

WColors, 63

WForegrnd, 63

WHO, 64, 120

WHY, 64

Wildcards, 100

wildcards, 69

WINDOW, 21

Window, 64

WINDOW DISPLAY MONOCHROME, 8

WINDOWS 3.1, 163

WP SERVER, 125

WPAge, 64

WPEvery, 64

WQupdate, 64, 117

WRecord, 64

WSize, 64

X, 73

XF, 73

XS, 73

YAPP, 9, 15, 73, 158

Yapp, 65

YAPP download, 73

YAPP upload, 73

YD, 73

MSYS 1.14                                                   Page 180