TN210_12.DOC

REMOTE SYSOP PARAMETERS, 1 through 5

****************************************

Parameter 1,  MINIMUM QUALITY FOR UPDATE

----------------------------------------

Sets  the  minimum path  quality value  of other nodes that will be accepted

into  the  routing table.   Nodes included in the broadcasts will be limited to

those  with path  quality values equal or greater than this parameter.   If the

value  is set low,  distant nodes  with  lower path values  (poorer probability

of connect)  will be allowed into the tables.    Higher values will improve the

probabilities   of  a  successful  connection.    A setting of "0" disables the

automatic update function and does not  allow  ANY  nodes to be recognized (not

recommended).    The  value  selected  will depend on several factors.    These

include the purpose of the node, network type and size.   A suggested value for

a multi-user simplex configured node is "86."

(Range:  0-255)

Parameter 2, HDLC CHANNEL QUALITY

---------------------------------

HDLC is an OSI level 2 function  and is  what  the AX.25  protocol is based

upon.    Here, the term is synonymous with RADIO PATH QUALITY.    Since much of

the automatic network  management  techniques of TheNet are centered around the

path  quality concept, the basic conventions will be discussed.  Certain  types

of  packet  networks  (wire links, HF/VHF/UHF radio  with varying  user access)

are more  efficient than others.    The most ideal packet link is a full duplex

wire line between two TNCs.  This, because there isn't any outside interference

to corrupt the circuit.  Least ideal is a multi-user  accessed  HF link subject

to  interference and  changing propagation conditions  between  the  two  TNCs.

Through  observations and studies the following conventions were established:

TYPE OF PATH BETWEEN TNCs      PATH QUALITY    RELIABILITY

RS-232 wire line (2 port)          255            99

Satellite link                     252            98

RS-232 wire line (3 port) link     248            97

UHF radio non-user radio link      240            94

VHF radio non-user radio link      224            88

VHF/UHF user accessed radio link   192            75

10 Meter user accessed radio link  180            70

HF user accessed radio link        128            50

It should be remembered these path quality values are for ideal situations.

Path  congestion and propagation conditions will lower the values  accordingly.

But  by  convention, the  above  values  will  be used  in  parameters  2 and 3

throughout the system.  The above chart explains why it is undesirable to allow

user access to backbone trunks.    By doing  so, the path  quality is degraded.

Recommended default for a  1200 baud VHF/UHF  user accessed  simplex radio link

is 100 with  the reliable neighbors  permed to 192.  (Range:  0-255)

Parameter 3, RS-232 CHANNEL QUALITY

-----------------------------------

Describes the path  (or port)  via the RS-232 connector  (DB-25 or DB-9) at

the rear of the TNC.    If configured for  multi-node operation, this connector

ordinarily attaches to either another TNC/node, or to a diode matrix connecting

three or more TNC/nodes.    As indicated in the above chart, the probability of

a successful connection is highest for two TNCs RS-232 connected.    Therefore,

the highest path quality value is assigned to this configuration.  Diode matrix

connected TNC's have a lower  probability  of connect.  This is because any one

given TNC may be held off  while an active packet is in process to another  TNC

on the matrix.   Thus throughput over diode  matrixed  TNCs  is not as  high as

two direct connected TNCs.    Path quality values for this parameter ordinarily

are  set at  255 for two nodes,  and perhaps  as low as  245  for several diode

matrixed nodes.

NOTE: SYSOPing  the  existing  value  of  Parms 2 and 3 to a different value on

active nodes showing in the ROUTES will not be immediately reflected.  If these

nodes decrement out (fail) and later come back, they will reappear with the new

SYSOPed  Path  Quality  value  assigned  to  them.   A procedure to get the new

value to quickly go into effect  (short of RESETTING the node)  would be to use

the  routes  locking  command.   Lock them in at the new value, then remove the

locks.

The following illustrates how HDLC (radio) CHANNEL QUALITY and RS-232 CHANNEL

QUALITY are displayed in response to a (R)outes command:

CLAMS:WA1ZDA-3} Routes:

0 ME220:WA2YVL-3 150 16

1 PENBAY:WA1ZDA-1 255 15

1 LOBSTR:WA1ZDA-7 255 15

The  0  and 1's seen in the left column correspond to the port  identifiers

listed  in  parameters 2 and 3.  The 0 means it is a radio port and the 1 is an

RS-232 TNC port.  The 3rd  column is the default settings (in this example) for

parameters  2 and 3.  Here we note the NodeOp has determined the radio path  to

ME220 is not a good "standard" 192 quality path.   Therefore he has assigned it

a value of  150.   If a neighbor node HAD a good 192  quality path, parameter 2

would be set for 192.  Then the  ROUTES  locking technique would be used to set

ME220  to  the  150  value.     In this example, the NodeOp has assigned RS-232

CHANNEL QUALITY values of 255 to both the PENBAY and LOBSTR nodes.   Though not

pertinent  to  this  discussion,  the  final  column  indicates the  number  of

destination routes through that path.

(Range:  0-255)

Parameter 4, OBS COUNT INITIAL VALUE

------------------------------------

The  initial  value  for the  NODES  obsolescence  counter.    This counter

displays  how current  a path is to a  destination node.   The node keeps track

time-wise of all nodes heard during neighbor broadcasts.   By  convention, this

counter is normally defaulted to "6."  If for some reason (QRM or node failure)

a known  node  isn't heard upon receipt of  the  next  neighbor broadcast,  the

obsolescence value for that node  will decrement to "5."    If not heard at the

next broadcast, it goes to "4," etc.  Once the value goes to "0," knowledge  of

that  node is removed from the routing table.  If the node IS heard  before the

value  falls to  "0,"  it automatically is reassigned a "6."   By comparing the

broadcast timer value in parameter 6 against  the  obsolescence count, one  can

calculate how "fresh" a path is.

The  purpose  of  the  obsolescence counter  is to  purge failed nodes from the

routing  tables.     With a  one-half hour  broadcast cycle (1800 seconds),  an

initial value of "6" will allow a  failed node to remain in the  routing tables

of immediate neighbors for up to 3 hours.    Depending  upon the parameters  of

more  distant  nodes,  the  failed node  may  appear  within the  network for a

longer period before finally being decremented out.

Nodes  configured as HF gateways  are subject to variable and sometimes sudden,

propagation shifts.    An initial obsolescence counter  value of "4" on  the HF

node  and on  adjacent  RS-232 connected VHF/UHF nodes  will hasten the purging

of stale nodes from the system.

(Range:  0-255)

Parameter 5, OBS COUNT MINIMUM for BCST

---------------------------------------

Sets  a limit on the  minimum obsolescence value associated with other nodes

to be  included  in the NODES broadcast.    For instance, if a neighbor has not

heard the  CARBON:VE6RCB  node  for 2 or 3 broadcast periods, the likelihood is

high CARBON has failed.  Thus it is good network practice to avoid  sending out

useless node data.   By setting parameter 5 to a value of one less than that of

parameter 4,  freshly  failed  nodes  will not be  included in the  forthcoming

broadcast.    Knowledge of the missed node will remain until decremented out by

parameter 4.  Therefore, if the broadcast was simply missed due to QRM, distant

users would still be able to connect to that node.     If set to a higher value

than parameter 4, the NODES destination table will include only itself.

(Range:  1-255)