<< HIGH SPEED MODEM USERS NOTE >>

If you use a high speed modem (9600 or 14.4) to connect to a BBS and you

experience problems with the UU screen displaying, then you may need to

make some adjustments. The following are 3 ideas that can be used to find

the right setup for UUTERM on any particular BBS.

1) By default UUTERM assigns a default handshake of CTS/RTS. This may not

be appropriate for your situation. You may want to choose a different

method of handshaking. To override the default values set by UUTERM,

simply give UUTERM one of the following parameters at the command line

when you start UUTERM:

0 = No Handshaking

1 = XON/XOFF

2 = CTS/RTS (default that is used)

3 = Both XON/XOFF and CTS/RTS

Example: UUTERM 0  to start with no handshaking

UUTERM 1  to use XON/XOFF only

2) If you have tried all 4 methods, and you still find the screen display

locking up, you may want to try a special key combination I have set up

for such an emergency. Try pressing "<ALT> Y". This should reset the

modem so you can continue playing without having to exit UUTERM and

reentering. Sometimes you may have to push this key combination a few

times before it works.

3) Another method that has worked successfully. After logging onto a BBS,

go to your configuration settings at that BBS. Lock your modem in at

9600. Then go into UU, exit your comm program and start UUTERM. You

should still be able to operate at 14.4 without any problems.

4) If all this fails. Hit <ALT> X in UUTERM to exit UUTERM without dropping

carrier. You can then fire up your comm program to get things going

again as you should still be on-line.