ACECOMM  Copyright 1990,1991,1992,1993 A.C.E. Software & Michael G Phelps

CONTENTS

Chapter 1   FILE REQUESTS..........................................2

NODELIST or optional .NOD file (FBL.EXE output)........2

Making a File Request..................................3

Using ACECOMM with the full blown raw NODELIST.........4

Using ACECOMM with FBL'S .NOD file.....................5

MAKING A FIDO STYLE EMSI FILE REQUEST..................7

Selecting a ZONE:NET/NODE Address......................9

NODE Display Menu.....................................10

Entering Filenames for Request........................12

Dialing the NODE......................................12

EMSI Handshaking......................................13

.REQ File.............................................14

Further Explanations..................................16

FREQ.DOC       Page 1

Chapter 1  FILE REQUESTS

NODELIST or optional .NOD file (FBL.EXE output)

NODELIST  is  a  specially  formatted ascii file listing all BBS

NODES which are members of a  NETWORK  known  as  FODONET.   The

NODELIST  contains  the  each  system's  name,  location,  phone

number, modem capability, mail capability.  The NODELIST  serves

as  a  giant  phone  directory for all FIDONET BBS systems world

wide.  As of the writing of this manual, February 18, 1993,  the

FidoNet nodelist is approximately 1.7 megabytes.  This may sound

huge  at first, but carrying the entire nodelist all the time is

optional.

.NOD file   (FBL.EXE output)

FBL can extract a NET from the NODELIST.  Sample .NOD files  are

contained  inside  FON.ARJ  (included  in  the  ACECOMM package)

Generally a NET will cover  an  entire  city,  and  you  may  be

content  holding  a .NOD file for your city's NET, rather than a

full blown copy of the NODELIST.

To obtain a .NOD file for your city, you will initially  need  a

NODELIST  to  use with FBL.EXE.  Your local SysOp may, if asked,

keep a local current .BBS, and .NOD file for your NET  available

for  download.   .BBS  ?  FBL produces two files.  An ascii .BBS

file in BBS LIST format, and a .NOD file, raw  NODELIST  format.

ACECOMM  can  directly  read  the .NOD file for MAIL activities.

CBF.EXE (included in the ACECOMM  package)  is  a  utility  that

converts FBL's .BBS file into an ACECOMM compatible .FON file.

In short, FBL.EXE can produce a local listing of all FIDONET BBS

systems  for your city, for both a dialing directory, and mailer

directory.

FREQ.DOC       Page 2

Chapter 1  FILE REQUESTS

Making a File Request

Make sure that your NODELIST or .NOD file is  in  the  directory

"NODELIST" specified within PRM-data AREA.  You may have defined

this  area  on  installation,  and can check or edit it now from

within ACECOMM itself with the  <PrmDataMenus_LUI>  MACRO  AREA.

ACECOMM   comes   with   the   ALT-P   key   configured  to  the

<PrmDataMenus_LUI> MACRO.

����������ACE.PRM Data  Edit-Menus������ͻ To get here, load

� General Controls                       � ACECOMM, then from

� Serial Port Setup                      � the terminal screen

� KeyBoard Text Macros                   � press ALT-P (in the

�#Directories in use                     � default configuration).

� Executable Menu                        � Select the DIRECTORIES

� AutoDnLd Trigger Sequences             � IN USE selection by

� Curtain color                          � using the arrow keys.

� Mass BOX-MENU attributes               �

� AutoSave is ON                         �

���� Select  Help  Color  Quit ���������ͼ

Press the [enter] key and a new screen pops in over it.

����������ACE.PRM Data  Edit-Menus������ͻ

� General Controls                       �

� Serial Port Setup                      �

� KeyBoard Text Macros                   �

��� Directories in Use Menu ��������������������������������������

�� .FON files: C:\ACE\FON

�� .CAP files: C:\ACE\CAPTURE

�� .SCR files: C:\ACE\SCRIPT

�� .KTS files: C:\ACE\KEY

��  Downloads: C:\ACE\FILES

Ⱥ#  Nodelist: C:\ACE\NODELIST

���� Edit  Help  Color  Quit ������������������������������������

Be sure that your NODELIST  or  .NOD  file  resides  within  the

PRM-data NODELIST directory.

FREQ.DOC       Page 3

Chapter 1  FILE REQUESTS

Using ACECOMM with the full blown raw NODELIST

This sub chapter applies  to  those  who  wish  to  use  the  raw

NODELIST with ACECOMM.

There is a handy utility provided with  ACECOMM  named  IDX.EXE.

IDX  will  create  a  small  index  file  so ACECOMM can use the

nodelist  more  efficiently.   Otherwise  the  NET/NODE   search

function  would  have  to scan the entire nodelist each time you

wanted  to  make  a  file  request.   The  index  file  created,

NLIST.IDX is about 5k in size.  NLIST.IDX contains as it's first

data  entry,  the name of the NODELIST it has been compiled for.

If your system maintains a NODELIST applying NODEDIFF  files  or

is changed by other means, IDX must be re run accordingly.

Run  IDX.EXE  to  create  this  NLIST.IDX  file.   There is more

specific documentation on IDX named IDX.DOC, but the  syntax  to

run IDX is: (at the DOS prompt)

IDX nodelist.nnn

Where nnn is the day number of the FidoNet<tm> nodelist that you

currently have.

Copy   NODELIST.NNN   and  NLIST.IDX  into  the  PRM-data  AREA

directory label NODELIST.

FREQ.DOC       Page 4

Chapter 1  FILE REQUESTS

Using ACECOMM with FBL'S .NOD file

OK,  you're reading this section because you don't want to carry

a ~2~ megabyte nodelist on your drive all the time.   For  those

who  run nodes, it's almost a necessity, but if you don't, there

are options for you.  When making a File Request, ACECOMM  looks

for a NODELIST.nnn file and a NLIST.IDX file.  If ACECOMM cannot

find  those, it next looks for a .NOD file.  .NOD files are made

with an supplied utility called FBL.EXE.   FBL  will  extract  a

particular  NET  out  of  the  entire  nodelist.   For NET 1:114

(Phoenix,AZ,USA), the  1_114.NOD  file.   You  still  require  a

current  nodelist,  but  at least you do not need to keep it, at

the ready for ACECOMM.  What you will need to do is:

NOTE: You may find it handy to  contact  your  favorite  FIDONET

SysOp  and  ask  him to carry FBL.EXE output .NOD and .BBS

files for you.  He will get a weekly current BBS list  for

all  FIDO NODES for his NET, and you can use the .NOD file

with ACECOMM'S mailer functions, and  run  the  .BBS  file

past  CBF.EXE  for a current .FON file of FIDO BBS systems

for your area.

- Instructions for creating a .NOD file with FBL.EXE

1.  Put the NODELIST and FBL together in the same directory.

2.  Decide on what NET it is that you want to File Request from.

3.  Run FBL with  the  following  syntax:

FBL.EXE  nodelist.nnn [ZONE:]NET [phone-filter]

where 'nodelist.nnn' is a standard nodelist file.

* 'ZONE' is optional (defaults to Zone,1)

* 'NET' is any Net that is defined in the

nodelist beginning as:

Host,NET. FBL defaults to using the first Zone

found.

* phone-filter' is an optional parameter. This

will tell FBL to filter phone numbers found,

creating a local BBS list.

FREQ.DOC       Page 5

Chapter 1  FILE REQUESTS

Using ACECOMM with FBL'S .NOD file (continued)

Example: FBL NODELIST.115 282

Result: Will generate listing for NET 282, leave phone  numbers

intact with the 1-(area code) so they can be dialed as

long distance if you live outside of that net.

Example: FBL NODELIST.115 1:114 1-602-

Result: Will generate listing for NET 114, stripping off the LD

part  of  phone  numbers  (1-602)  to  make them local

dialing numbers

FBL creates 2 files: ZONE_NET.nod, and ZONE_NET.bbs

ZONE_NET.NOD  -  is a listing for NET in raw-nodelist format.

This file is what you are going to be  needing

for ACECOMM to do a File Request.

ZONE_NET.BBS - is a listing for NET in BBS-List format.  This

one is just to read, display or print.

FBL (with no arguments will display a help screen)

After you are done with FBL, make sure that the  .NOD  file  you

created is residing in the PRM-data AREA directory for NODELIST.

�� Directories in Use Menu ����������ͻ

� .FON files: C:\ACE\FON              �

� .CAP files: C:\ACE\CAPTURE          �

� .SCR files: C:\ACE\SCRIPT           �

Be sure the NOD   � .KTS files: C:\ACE\KEY              �

file is in this   �  Downloads: C:\ACE\FILES            �

directory ----->  �#  Nodelist: C:\ACE\NODELIST         �

���� Edit  Help  Color  Quit ��������ͼ

FREQ.DOC       Page 6

Chapter 1  FILE REQUESTS

MAKING A FIDO STYLE EMSI FILE REQUEST

From the terminal screen:

call <RProtocols_LUI> macro

ACECOMM default setup  "PgDn" key

Press the PgDn key (3 on the keypad).   The  INTERNAL  RECEIVE

PROTOCOL menu pops up.

_____________________________________________________________________

A.C.E.  Communications  Software

Evaluation copy, Version (Test)

Initializing Modem

ATZ

OK                              �� Internal Protocols (Recv) �ķ

ATEL1S7=40                      ��Batch Capable���������������ĺ

OK                              �#Zmodem                       �

� Ymodem                       �

� Ymodem 1k/Y                  �

� YmodemG                      �

��Single File�����������������ĺ

� Xmodem                       �

� Xmodem 1k                    �

��User Defined����������������ĺ

� Externals                    �

� AutoDnlds                    �

��FIDONET EMSI FReq.����������ĺ

� FileRequest                  �

�� Select  Help  Color  Quit �Ľ

ACECOMM � 1,2400,8N1 � Ansi � 11:05:48pm �

FREQ.DOC       Page 7

Chapter 1  FILE REQUESTS

MAKING A FIDO STYLE EMSI FILE REQUEST (continued)

Again,   with   the  arrow  keys,  move  the  selector  down  to

FileRequest and press the [enter] key.

�� Internal Protocols (Recv) �ķ

��Batch Capable���������������ĺ

� Zmodem                       �

� Ymodem                       �

� Ymodem 1k/Y                  �

� YmodemG                      �

��Single File�����������������ĺ

� Xmodem                       �

� Xmodem 1k                    �

��User Defined����������������ĺ

� Externals                    �

� AutoDnlds                    �

��FIDONET EMSI FReq.����������ĺ

�#FileRequest                  � <-- EMSI File Request selection

�� Select  Help  Color  Quit �Ľ

When you press [enter] now, an INPUT WINDOW appears asking for

the Zone, Net and Node that you want to REQUEST the file from.

��Input Window������������������������������������������������������

�#

� Format =>  ZONE:NET/NODE

��������������������������������������������������������������������

ATZ

OK

ATEL1S7=40

OK

...

ACECOMM � 1,2400,8N1 � Ansi � 11:07:03pm �

FREQ.DOC       Page 8

Chapter 1  FILE REQUESTS

Selecting a ZONE:NET/NODE Address

��Input Window������������������

�#           1:114/35          <-- enter target address here

� Format =>  ZONE:NET/NODE

��������������������������������

ATZ

OK

Enter  the  target  systems  address in the fashion, ZONE number

first (1-6, Zone 1 is North America), followed by a  colon  (:),

followed  by  the NET number (114 is for the Phoenix area), next

is a forward slash (/), then the target systems NODE number each

FidoNet<tm> node has it's own unique  address).   An  incomplete

address  will  result in an error as will entering a nonexistent

address.  ACECOMM must be able  to  find  that  address  in  the

nodelist or it will not continue.

��Input Window������������������

�#           1:114/35

� Format =>  ZONE:NET/NODE

������� Message Alert �������������������������������������������ͻ

ATZ � Address not found                                            �

OK  �                                                              �

ATEL���� Press any key to continue��������������������������������ͼ

OK

The  error  above  is what is displayed if ACECOMM has exhausted

it's resources in attempting to find a  match  to  the  inputted

target address.

Otherwise,  the  address  box  (below)  appears when ACECOMM has

found a match to the inputted address.

FREQ.DOC       Page 9

Chapter 1  FILE REQUESTS

NODE Display Menu

��Input Window����������������������

���1:114/35���������������������������������������ķ

��    System Name A.C.E._Software_Support          �

Ӻ           City Phoenix_AZ                       �

�  Operator Name Michael_Phelps                   �

�   Phone Number 1-602-846-2940                   �

�      Baud Rate 9600                             �

�          Flags V32,CM,XA                        �

��File_Request  Human_call  Poll_for_mail  Quit��Ľ

This box appears so you, the user can be  certain  this  is  the

target system you desire to File Request from.

If this is the correct node you wish to  FReq  from,  press  the

enter]  key and ACECOMM will then query you for a filename(s) to

Request from the target system.

If  a  mistake  was  made  and/or this was not the node that you

wanted to File Request from, hit [ESC] and  ACECOMM  will  bring

you out.

System  Name  and  City  are  self  explanatory, the name of the

system that you have addressed your File Request to.

Operator Name is the name of the Sysop  of  the  node  you  have

addressed your File Request to.

Phone  number, of course is the phone number of the addressee of

your File Request.  Do not be alarmed if the system is local  to

you  and the phone number still contains 1-(area code).  ACECOMM

will filter that out for local calls.

Some systems do not carry the CM flag.  The CM flag  stands  for

Continuous  Mail,  which  means the system is up 24 hours a day,

waiting  to  send  or  receive  mail,  or  in  this  case,  File

Requests.  If this node were to not have this CM flag, you would

not  be  able to File Request from it because it is probably not

up and waiting 24 hours  a  day.   ACECOMM  should  give  you  a

warning if you attempt to File Request from a non-CM system.

FREQ.DOC       Page 10

Chapter 1  FILE REQUESTS

NODE Display Menu (continued)

��Input Window������������������������������������������������������

�#FileName:

��Input Window<ENTER> alone when done�������������������������������

��������������������������������������������������������������������

This window is used to input the names of the files that you are

going to be requesting from the target node.

��Input Window������������������������������������������������������

�#FileName:  files

��Input Window<ENTER> alone when done�������������������������������

��������������������������������������������������������������������

Enter  a  filename that you desire to File Request and press the

[enter] key.  You may continue requesting  more  files  just  by

entering another name when the INPUT WINDOW reappears.  When you

are  done with the list of files you wish to request, just press

[enter] when ACECOMM asks you for another filename.

Consider File Requesting  the  filename  "FILES"  first  from  a

system.  Generally, systems have a master list of files that are

available on their system.

So, you would type (when the INPUT  WINDOW  appears)  is:  files

<cr> <cr>

Next  will  appear the dialing box.  ACECOMM will now attempt to

connect with the system whose node number you gave.

FREQ.DOC       Page 11

Chapter 1  FILE REQUESTS

Dialing the NODE

���Record Number��������������������������������������������ķ

�#       System Name A.C.E._Software_Support                 �

�       Phone Number 1-602-846-2940                          �

�       Line Control 19200,8N1                               �

� Terminal Emulation Ansi                                    �

�     Dialing Prefix                                         �

�   AutoLogOn Script                                         �

�       Capture File                                         �

�           Password                                         �

��Call History����������������������������������������������ĺ

�  Last Connect Time 17 Feb 01:05:56am                       �

�   Past Connections 0                                       �

��Dialing Status��������������������������������������������ĺ

�        Dial String ATEQ0X4L1DT846-2940                     �

�     Modem Response                                         �

�            ReDials 0                                       �

�            Elapsed 00:01                            00:02  �

�            Timeout 48                                      �

�� Quit  <SpaceBar>-ReCycle ��������������������������������Ľ

After you get a connect with the target  system,  you  will  see

something similar to what is below.  Different front-end-mailers

will look differently, however, the results will be the same.

FREQ.DOC       Page 12

Chapter 1  FILE REQUESTS

EMSI Handshaking

ACECOMM  currently  is limited to EMSI handshaking for it's mail

session activities.  Almost all  BBS  systems  using  front  end

mailers  such  as  Binkley, FrontDoor, DBridge are EMSI capable.

EMSI  handshaking  is  the  means  the  two  softwares  use  for

exchanging  identification  data.   Your  identification data is

contained inside the file ACE.CFG.  You  will  have  edited  the

file previous to making a File Request.  The following screen is

the result of a successful EMSI log on with a Binkley system.

CONNECT 9600

Address 1:114/35@fidonet.org Using BinkleyTerm Version 2.56

490I_REQA77E your Escape key to enter the BBS, or wait a few moments.

A.C.E. Software Support

Michael Phelps, Phoenix,AZ1-602-846-2940

BinkleyTerm, 9600, CM,XA,V32,V42b

Outgoing  00720023.REQ

FileSize  9

0        9        0 CPS        00:00        ZGHDR

With  successful  EMSI log on, ACECOMM sends the .REQ file.  The

.REQ file contains a list of filenames for request.

FREQ.DOC       Page 13

Chapter 1  FILE REQUESTS

.REQ File

The line below gives the status on the transfer of the REQ file.

After  the  target  system,  1:114/35  reads   the   REQ   file,

BinkleyTerm responds by sending back the file requested.

CONNECT 9600

Address 1:114/35@fidonet.org Using BinkleyTerm Version 2.56

490I_REQA77E your Escape key to enter the BBS, or wait a few moments.

A.C.E. Software Support

Michael Phelps, Phoenix,AZ1-602-846-2940

BinkleyTerm, 9600, CM,XA,V32,V42b

Outgoing  00720023.REQ

FileSize  9

0        9        0 CPS        00:00        ZGHDR

Incoming  c:\ace\files\1_114-35.arj

FileSize  15211

6144        2048        236 CPS        00:26        ZDATA

After  the  OUTGOING  is  an  INCOMING.   To  the  right it says

c:\ace\files\1_114-35.arj.  This is where the file you requested

is going to and the actual name of the file sent in response  to

"FILES".

I know, I know, you requested FILES, but you're getting  a  file

named  1_114-35.arj.   Strange eh?  Well, some systems allow you

to request files by the use of what is called  MAGIC  names.   I

won't  go  into  detail on how that works, but suffice it to say

that you can define MAGIC  names  for  whatever  you  want.   An

example  of  this  would  be for the nodelist.  Each nodelist is

named differently, but the Sysop can tell  his  front-end-mailer

to  send  the  appropriate  one just by you requesting the MAGIC

name, NODELIST.  Or for a nodes master list of files.  Just FILE

REQUEST the name FILES and you should receive that  file.   This

can  also be done for other files as well.  Say the most current

version of the compression utility ARJ.  You might not know  the

version  number,  but possibly, all you would need to do is FILE

REQUEST the filename ARJ and see if that does it.

FREQ.DOC       Page 14

Chapter 1  FILE REQUESTS

.REQ File (continued)

When the .REQ file is done being sent, it will be  deleted.   If

you find an .REQ file at some time in your ACECOMM directory, it

is  probably the result of a file request that wasn't completed.

If you examine the .REQ file, you can tell  what  you  requested

and  where  it  was  intended  on going to.  If you read what is

inside the REQ file, with LIST or even TYPE it out, it will tell

you what file(s) you were requesting.  To  find  out  where  the

system was you had intended on requesting this from, look at the

name of the .REQ file.  In this case, it was:

00720023.REQ

Take it apart this way..

0072 (hex) = 114 (decimal)

0023 (hex) =  35 (decimal)

So your request was targeted at 114/35 (Zone 1 assumed).

FREQ.DOC       Page 15

Chapter 1  FILE REQUESTS

Further Explanations

������������ Terminal screen during File Request ��������������������

CONNECT 9600                    ( Connect acknowledgement and baud )

Address 1:114/35@fidonet.org Using BinkleyTerm Version 2.56

(target FidoNet address and

front-end-mailer identification)

490I_REQA77E your Escape key to enter the BBS, or wait a few moments.

(target mailer REQuesting an EMSI

packet, that line partially

overwritten by target mailer

attempting to verify if the current

connect is human or another mailer.

A.C.E. Software Support

(target system name)

Michael Phelps, Phoenix,AZ1-602-846-2940

(target system Sysop, location and

phone number)

BinkleyTerm, 9600, CM,XA,V32,V42b

�  �   �   � (mailer I.D., baud rate and flags)

�  �   �   �

�  �   �   � modem is V.42bis capable

�  �   ����� modem is V.32 capable

�  ��������� mailer type I.D.

������������ system is of a Continuous Mail type

Incoming  c:\ace\files\1_114-35.arj

(pathname of the file or packet that

is being exchanged)

FileSize  15211               (total size of packet/file)

FREQ.DOC       Page 16

Chapter 1  FILE REQUESTS

Further Explanations (continued)

6144        2048        236 CPS        00:26        ZDATA

�           �            �              �           �� information

�           �            �              �              about what

�           �            �              �� amount      ZModem is

�           �            �                 of time     currently

�           �            �� Characters     already     doing.

�           �               Per            spent

�           �               Second         transferring

�           �               transfer       this file

�           �               rate, higher

�           �� size of      the better

�              the current

�              packet being

�              transferred.

�              In this case,

�              the packet size

�� number of   is 2048 bytes (2k)

bytes that

have already

been transferred