FASTA20.SYS, A SOLUTION TO USING SOME FAST MACHINES WITH QEMM-386

Some fast computers which can run at 66 MHZ, have a bug in their

BIOSes.  When you use Function 87 of Interrupt 15, the BIOS call which you use

to move data into extended memory, the BIOS does not wait long enough for the

8042 (the keyboard controller chip, a port of which you use to enable/disable

address line A20, the address line you must enable to use any memory above one

megabyte) to enable the A20 address line.  Because the 8042 is typically slow,

QEMM-386 always enables it twice when it must enable it.  When you run such a

computer at the 66 MHz speed, it does not give the 8042 enough time to honor

the BIOS's request to enable line A20 so when you try to use any of the memory

you need to enable line A20 to use (all even megabytes), then try to use this

memory, you crash.

This device driver, FASTA20.SYS, hooks Int 15, and, when a function 87

comes in, does an enable of A20 so that by the time the BIOS enables A20, it

will be the second time, and, by the time the machine expected to have enabled

A20, it will be ready.

Load this driver in the CONFIG.SYS before QEMM386.SYS:

device=c:\qemm\fasta20.sys

device=c:\qemm\qemm386.sys ram st:m

Currently the only machine we know this works on is the Swan 486DB.  But

the problem and this solution are generic and you do yourself no harm by

trying loading FASTA20.SYS before QEMM-386 if you crash on loading QEMM-386

when in the clock-doubled mode.

The reason you do not have this problem with HIMEM/EMM386 is that HIMEM

enables the A20 line but does not use the memory, while EMM386 uses the

memory, and enough time has elapsed between HIMEM's enabling of the A20 line

and EMM386's use of the memory for the 8042 to have enabled A20.

QEMM-386 enables the A20 line only once, when it loads, so using FASTA20

entails no performance degradation.

************************************************************************

*This technical note may be copied and distributed freely as long as it*

*is distributed in its entirety and it is not distributed for profit.  *

*          Copyright (C) 1993 by Quarterdeck Office Systems            *

************************ E N D   O F   F I L E *************************