To forward to a station, that station must now be designated a BBS or a

personal BBS.  Use the EU command and set the proper flag.

B flag -- Old BBS programs that don't have the [xxx] info

A flag -- Advanced BBS that does send [xxx].  When in doubt,

use this flag

P flag -- Personal BBS.

Rather than the current way of having a list of destinations for each

route, a list of routes for each destination is used.  The data has been

broken into two pieces.  How to connect with a distant mailbox is in the

PATH file while the information on what to send is in the ROUTE file.

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

The ROUTE File

--------------

A line from the ROUTE file is organized as:

W3IWI W0RLI WB6ASR 80 KE6BX

This says to send mail for W3IWI mailbox down the path named either

W0RLI, WB6ASR, and then KE6BX.  The forwarding will be performed in that

order.  W0RLI will be tried first and then WB6ASR.  If a message is more

than 80 hours old, KE6BX will be used (See special case:  numbers below)

Wildcards are supported.   For full details on these, see WILDCARD.DOC.

Just more than * is supported.

Once a message (or a member of a distribution list has been selected, it

cannot be selected for another route.

95020   AA4RE

95*     W0RLI

A message with a route code of 95020 will go to AA4RE.  All others go to

W0RLI. 95020 will not since it was selected for the AA4RE route already.

The ROUTE files also allows ALIASES.  These are of the form

SOCAL = K6IYK W6PW

Anywhere SOCAL appears in a route, K6IYK W6PW will be substituted.  So

if the ROUTE file says

WB6YMH SOCAL

this is the same as

WB6YMH K6IYK W6PW

Aliases only affect the path to use.  Aliases must be listed before they

can be used.

>>> Hierarchial addresses

The full hierarchical address is checked.  The "#" and "." are not

ignored.  Search is right to left.

Examples:

Address #NOCAL.CA.USA.NA

Matches

NA

USA\.NA

CA\.USA\.NA

"#NOCAL\.CA\.USA\.NA

Note the use of the double quote.  This is because the "#" is also a

wild card character for numbers!  The double quote says to interpret the

next character literally.

The backslash "\" is used for optional material.  Thus the pattern

USA\.NA matches both USA and USA.NA.

>>> Special case:  LEAVE

If you send something to LEAVE then it will stay on the BBS and not

be forwarded.  This is handy to prevent just certain messages to stay.

Example:

95020 LEAVE

95*   KE6BX

A message to 95020 will stay but all other 95anything will go to

KE6BX.

>>> Special case:  @BLANK

If the special @BLANK is used, this will match any message where

the @BBS field is blank.  Combining LEAVE and BLANK make some

route tables easy:

AA4RE  LEAVE

@BLANK LEAVE

*      WB6ZVW

Any message addressed to @AA4RE stays.  Any message addressed to

anycall without @ will stay.  All others will go to WB6ZVW.  This

is very handy for a small BBS where everything but local calls go

to one other BBS.

>>> Special case:  ?

If you send something to ? then it will stay on the BBS and not

be forwarded.  It will show up if you execute the L? command.

Example:

USA   ?

A message to USA will be marked with the ? flag

>>> Special case:  numbers

The numbers in a route file pertain to time delay

Example: W3IWI W0RLI 20 WB6ASR +5 WB6ZVW 80 KE6BX

This means send traffic to W0RLI to W3IWI anytime.  If a message if more

than 20 hours old then also try to forward it to WB6ASR.  If a message

is 25 hours old (20 from before WB6ASR +5) to WB6ZVW and after 80 hours

to KE6BX.  The use of the "+5" should be confined to ALIAS statements

since it requires more computation.  I would use something like:

OTHER = WB6ASR +5 WB6ZVW +10 KE6BX

W3IWI KA6FUB 10 OTHER

This would be interpreted as

W3IWI 10 WB6ASR 15 WB6ZVW 20 KE6BX

>>> Special case:  DONE

If you send something to DONE then it will sbe marked as already

forwarded.  This is handy marking message done for dead systems or

for giving up on forwarding bulletins after a certain length of time.

Example:

ALLUS AA4RE 6 DONE

A message to USA will be forwarded to AA4RE.  After trying for

6 hours, the message will be marked as forwarded.

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

The PATH File

-------------

The PATH file has the entries that describe how to connect to a

mailbox we want to forward to.   A typical entry:

PATH W0RLI  B W0RLI

T 0000 0600

T 1000 1600 1 5

T 2200 2359

O TA

C W6AMT-10

PF 12

SC #SFO3

R* to *

SC SNTCRZ

R* to *

SC W0RLI

R* to *

PATH pathname port callsign force

"pathname" is the name of this path.  Any arbitrary 12 characters

will do.  This matches the entry in the ROUTE file.

"port" is the letter of the port to use to connect to this

mailbox on.  If this is a DRSI card using TNC-TSR, you must also

code the port on the C(onnect) statement.  If this is a BPQ

switch port of any type, you may have to code the port number

also.

Note that you can designate subports by extending this field.

Example:  B-1 would indicate a subport.  You can then control

forwarding by subport by using commands like "X PORT B-1".  If

you want to automatically forward by subport then code

"no_out_fwd = Y" for this port in your PARMS.BB and then issue

the X commands via WAKEUP.BB

For forwarding to a file, use a port of "L"

"callsign" is used to set certain parameters found in the USER

file (like MAXPAC etc).  Any call can be used here as long as the

USER file contains the information.  This call is also used for

REVERSE FORWARDING.

"FORCE" -- If the actual word FORCE appears after the callsign,

connect via this path even if no messages.

C(onnect) callsign

This is the actual connect command to be used.  For the DRSI

card using TNC-TSR, don't forget to include port.  Example:

C 1:AA4RE

Note 1:  You must have a "C" statement in each PATH even if

it will never be used.

Note 2:  If you are forwarding to a file (the port letter is

"L", then the "C" statement will contain the name of the

file.

P(ort) cmd

This is a command to be issued to this port.  Can be used to

change all sorts of things.  Be careful to affect only this

"channel".  See the documentation for your TNC as to what is

allowed.

S(end) text

Send the text of this line to the distant station.

R(ecieve) pattern

If the next line received from the distant station does not match

this line disconnect.  Full pattern matching can be used.  "R* to

*" is great for the NETROM response.  Be careful when

constructing your own pattern.  It starts immediately after the

"R" without an intervening blank.

W(ait for) pattern

If the next line received from the distant station does not match

this line then ignoreit.  Full pattern matching can be used.  "R*

to *" is great for the NETROM response.  Be careful when

constructing your own pattern.  It starts immediately after the

"R" without an intervening blank.  Warning!!  The only exit from

this other than a successful match is the port timeout value.

O(Order) order_info

This controls how messages are ordered before forwarding.

order_info can have up to three characters:

A -- Order by age with oldest first

T -- Order by type.  T before P before B.  This is

actually an alphabetical sort.  Special case

of "P" with distribution list is same as "B"

S -- Order by size.  Smallest first

D -- Order by day.  Similar to A but only the day matters

and not the time.  Oldest first.

R -- Reverse of D.  Latest is first.

If you code the letters TA then the messages will be sent

by type then age within type.  Thus the first message sent

will be the oldest "T" message.  The order will look

like this:

oldest "T" message

.

newest "T" message

oldest "P" message

.

newest "P" message

oldest "B" message

.

newest "B" message

Note that using "A" as a parameter except as the last letter

is kinda useless.  If you gave "AT" then messages would be

ordered by AGE then type.  The probability of two messages

being the same AGE (down to the second) is very very small.

This is why the "D" is given.  "DT" would order by DAY and

then TYPE within DAY.

Note 1:  The "O" statement must be before any "C" statements

Note 2:  There is a DEFAULT_FORWARD_ORDER parameter that

applies on a port basis in PARMS.BB

Note 3:  Any "O" statement will wipe out the ORDER parm on

the "T" statement

T(ime) start_time end_time start_day_of_week end_day_of_week ...options...

Use this path during certain times.

start_time = start time we can use path.  HHMM format

stop_time  = stop time we can use path.  If not given then

we can forward anytime after start time.

start_day_of_week = Day of week we can use this path.

Sunday = 0, Monday = 1, etc.  If omitted, can

use any day.  If no end_day_of_week given then

path can be used this day ONLY.

end_day_of_week = Last day path can be used.

options = options for more control.

HOUR x/y    If the hour divided by x has a

remainder of y then forward.

Examples:

HOUR 2/0   Only even hours

HOUR 2/1   Only odd hours

HOUR 3/1   Every 3 hrs (0100 is ok)

ORDER ccc   Same as "O" but only for this time

period.  Must be last thing on line.

SIZE nnn    Max size to forward in this time period

TYPE aaa    Type of message to forward.  Use

an underscore '_' for blank types.

Wildcards not allowed.  Max ten types.

REVERSE     Apply this statement only when reverse

forwarding

NOREVERSE   Apply this statement only when not

reverse forwarding

FORCE       Apply this statement only when force

forwarding

NOFORCE     Apply this statement only when not

force forwarding

RPROMPT     Send the reverse forward prompt F>

NORPROMPT   Do not send the reverse forward prompt F>

Note 1:  All start/end are INCLUSIVE.

Note 2:  You can have many "T" statements.  If ANY of them

is true, you can forward.

Note 3:  The "T" statement must be right after the PATH statement

Note 4:  The end day-of-week must be greater than or equal to

the start day-of-week.  The format 6 0 will not execute

on Saturday and Sunday but will simply be ignored.  To

accomplish something like this you will need two "T"

statements:  One for Saturday and one for Sunday

Note 5:  The ORDER, SIZE, TYPE, etc operands from the FIRST

"T" statement that is found to be true will be used.

Note 6:  If REVERSE and NOREVERSE are both specified then the

"T" statement will always be ignored

Note 7:  If FORCE and NOFORCE are both specified then the "T"

statement will always be ignored

Note 8:  The parameters FORCE/NOFORCE/REVERSE/NOREVERSE/HOUR

should appear BEFORE the SIZE/TYPE/ORDER parameters.

Examples

T 0900 2300                 Use this path from 0900 to 2300

T 0000 2359 1 5             Use this path from Mon to Fri

T 0000 2359 0 6 HOUR 2/0    Any day but only even hours

T 0000 0800 6 6 HOUR 2/1    Odd hours between midnight and

eight AM on Saturday only

T 1200 2359 0 6 ORDER DT    Forward between noon and midnight

on any day with order sequence DT

T 1200 2200 0 6 SIZE 100    Forward between noon and 10 PM

on any day but only messages of 100

bytes or less

T 1200 2359 0 6 ORDER DT SIZE 100

Forward between noon and midnight

on any day with order sequence DT

but only messages of 100 bytes or

less

T 0000 2359 0 6 TYPE PT     Forward anytime and anyday but only

message types "P" and "T'

T 0000 0000 0 6 NOREVERSE   Never forward down this path except

for reverse forward

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

Some notes on proper coding of the "T" command.  Prior to V2.12, the "T"

command only applied to direct forward out.  It was ignored on reverse

and force forwards.  On newer releases, this is not true.  The "T"

command now applies to all forwards.  This allows you to fine tune the

messages to be forwarded under all conditions.

Several examples follow:

1) You only want to reverse or force forward to the station:

T 0000 2359 0 6 REVERSE       <- This handles reverse

T 0000 2359 0 6 FORCE         <- This handles force

Since there is no "T" line for normal forwarding, it is skipped.

2) You only want to forward private mail during the day but want to

force forward anything.

T 0800 1900 0 6 TYPE PT NOFORCE     <- P or T mail during the day

T 0000 2359 0 6                     <- Everything else

This coding exploits the fact that the first "T" command that is

true is used.  The other "T" commands are ignored

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

To ensure proper operation, execute the GF command after updating

ROUTE.BB Warning!!!  If forwarding is in progress when you issue the GF

command then you will start to receive "routes in use" messages.  This

means that the data from the old ROUTE.BB file is still being used by

someone connected to the BBS.  The condition should clear up when those

stations disconnect.

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

You can forward to a file by setting the port letter to "L" and the name

of the file on the "C" statement.  Example:

PATH PACSAT L PACSAT

C D:\PACSATS\FILE.OUT

Note that the call given as the 4th word on the path line MUST be in the

user file as a BBS.

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

Forwarding thru NETROM, TheNet, G8BPQ switch

--------------------------------------------

Use the following snippets for NETROM forwarding

C NODE1                    <---Local Node

SC NODE2                   <---Distant Node

R* to *                    <--- to MUST be in lower case!

SC Z6ZZZ                   <---The other BBS

R* to *

Repeat the 2nd and 3rd lines as necessary for all the nodes needed

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

Forwarding thru ROSE

--------------------

SC KD5SL V W5JJW-2,504926  <--- ROSE Connect Command

R* to *                    <---}

R* being *                 <---} Three Lines for ROSE Response

R* Complete *              <---}

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

NETROM/ROSE/NETROM

------------------

CO MSVKS

R* to *

SC N5HOF-2 V AE5V-3,318323

R* to *

R* being *

R* Complete *

SC N5HOF

R* to *

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

Using the G8BPQ switch

----------------------

If you are using the G8BPQ switch, you will always connect to

the switch first.

C SW

SC X6XXX

R* to *

* = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = * = *

Simple Example

--------------

A simple BBS might have the following

ACTION.BB:

DISTRIBUTE DISTLIST B

DISTLIST.DIS

GILROY WB6ZVW KE6BX

MERCED K6RAU  N6OA

ROUTE.BB

GILROY WB6ZVW

MERCED K6RAU

PATH.BB

PATH WB6ZVW A WB6ZVW

C WB6ZVW

PATH K6RAU A K6RAU

C K6RAU

Now the explanation.

1) The ACTION.BB says to distribute all bulletins via DISTLIST.DIS

2) DISTLIST.DIS says to send the bulletins to two destinations, MERCED

and GILROY (I like to use town names).  On the Gilroy line, the calls

WB6ZVW and KE6BX mean that if a bulletin has already gone thru either of

these stations, do not send it to "GILROY"

3) The ROUTE.BB says to send things for GILROY to WB6ZVW and for

MERCED to K6RAU.

The use of town names for bulletins allows them to be routed in

ROUTE.BB differently than private mail.