Digitized Voice Programmer's Toolkit for the PC

-----------------------------------------------

Version 2.0

Copyright (c) 1988,1989,1990,1991 Farpoint Software

Recording Procedure Call Documentation

------------------------------------------------------------------------------

Calibration routine

-------------------

Prototype:

long pascal far RCALIBRATE(void);

This must be called once before RECORDVOICE is called. RECORDVOICE may be

called multiple times thereafter, as long as the program remains in memory.

Return value:

Low word      Meaning

--------      -------

0          success

1          this CPU is too slow to accomplish normal-speed playback

2          operation not possible under Microsoft Windows

in "Enhanced" mode

The return value high word is the actual speed calibration constant for this

computer.

------------------------------------------------------------------------------

Force Calibration Routine

-------------------------

Prototype:

void pascal far RSETCAL(unsigned int calfactor);

Normally never used, this call can be used to set the internal calibration

constant normally determined by the RCALIBRATE routine. A possible use for

RSETCAL would be to alter the recording sample rate.

There is no return value.

------------------------------------------------------------------------------

Recording Routine

-----------------

Prototype:

long pascal far RECORDVOICE( unsigned char far *buffer,

long bytecount,

int comport );

This routine actually performs the voice recording. The CPU will be fully

occupied by this operation. Any keypress during playback will terminate the

operation. The keystroke will still be in the BIOS keystroke buffer when

RECORDVOICE returns. Note that literally ANY keyboard action, including key

releases and shift keys, will be sufficient to end playback. There is also

a "stop" switch on the digitizer board which may be used to terminate

recording.

The pointer "buffer" passed to the routine will be treated as a huge pointer,

with 64k segment boundaries taken care of automatically. Also, the "bytecount"

parameter is a 32 bit number. Thus, the practical limit to the size of one

contiguous block of data to be recorded is the amount of memory available to

your program.

The return value indicates the actual number of bytes recorded. This number

will be less than "bytecount" unless the routine was allowed to run until

the buffer was full.

------------------------------------------------------------------------------

Memory Available Routine

------------------------

Prototype:

long pascal far GETMEMAVAIL(void);

This is just a convenient way to find out how much memory can be allocated

from DOS, under the assumption that it will all be used to create the largest

possible recording buffer.

The return value is the number of bytes available.