Compiling New PCroute code

The PCroute distribution comes with a compiled PCROUTE.EXE file that is

configured for two Ethernet (WD8003E) interfaces.  If you want a different

interface configuration (say three ethernets, or an ethernet and a localtalk)

you will need to make some small changes to the source code and recompile

it.  This document describes how to do this.

WHAT YOU WILL NEED.

The original source code was written using TURBO ASSEMBLER 1.0.  I have

made no attempt to check if the code will assemble with other assemblers,

but you are welcome to try.  Here I will assume that you have the TASM and

TLINK utilities that come with TURBO ASSEMBLER.

CONFIGURING PCroute.

In order to change the interface configuration of PCroute it is only

necessary to change one file, DECLARE.INC.  This file contains a description

of each card in the router and it is this description that needs to be changed.

------------------------------------------------------------------------------

Ethernet Card:   For each WD8003E card there are lines like

WDE_DECLARE %cur_dl, 280H, 0D000H, 00000H ;;<name>, <I/O>, <seg>, <off>

IF_DECLARE %cur_dl, %cur_dl, WDE        ;; <if name>, <real if>, <prefix>

ETH_DECLARE %cur_dl, %cur_dl, %cur_dl   ;; <eth name>, <if>, <task>

ARP_DECLARE %cur_dl, %cur_dl            ;; <arp name>, <ether>

DL_IP_DECLARE %cur_dl, %cur_dl, ARP     ;; <dlip name>, <real dlip> <prefix>

cur_dl = cur_dl + 1

WD_DECLARE declares that this card will be called 'cur_dl', and

that the card has its I/O address set to 280H, and its shared memory

address to D000:0000 (seg:offset).  The rest of the declarations

describe how this card is interacts with the rest to PCroute software.

In particular

1) IF_DECLARE declares that the WDE card is an ethernet card

2) ETH_DECLARE declares that this IF will use ethernet addressing

(as opposed to say 802.3 addressing)

3) ARP_DECLARE declares that the ARP protocol is to be used to

map ethernet addresses to IP addresses

4) DL_IP declares that the card is to be used by a an IP code

The key feature of all of this is that other ethernet cards differ

from the example shown above in only their I/O address and the shared

memory offset number.  I use the following convention.

WD8003 Card #    I/O address    shared mem seg  shared mem off

1              280H           0D000H           0000H

2              2A0H           0D000H           2000H

3              2C0H           0D000H           4000H

4              2E0H           0D000H           6000H

...

This insures the requirement that the I/O address have 20H entries and

the shared memory point to 8K of unused memory address space.

------------------------------------------------------------------------------

Starlan Card:  For each WD8003S card there is an entry like

WD_DECLARE %cur_dl, 2A0H, 0D000H, 02000H,10,16 ;;<name>, <I/O>, <seg>, <off>

IF_DECLARE %cur_dl, %cur_dl, WD         ;; <if name>, <real if>, <prefix>

ETH_DECLARE %cur_dl, %cur_dl, %cur_dl   ;; <eth name>, <if>, <task>

ARP_DECLARE %cur_dl, %cur_dl            ;; <arp name>, <ether>

DL_IP_DECLARE %cur_dl, %cur_dl, ARP     ;; <dlip name>, <real dlip> <prefix>

cur_dl = cur_dl + 1

The starlan entry is very similar to the Ethernet entry.  The big

difference is that there are two additional parameters to the

WD_DECLARE declaration (note also the string WD (not WDE) is passed

to the IF_DECLARE statement).  The parameter 10 is the name of what

I call a timer object and will NEVER change.  The parameter 16 is

the number of 256 byte blocks (of the 8K available) to allocate to

output buffering.  This can be tuned, but in general it should not

change.

Also, if the line 'include wd8003.inc' has been commented out, you

should comment it back in and comment out code for some other interface

you are not using (atalk.inc for appletalk, slip.inc for slip)

Thus the configuration of a starlan entry is very similar to a

Ethernet entry, you simply make sure that the I/O address and

shared memory offset do not conflict with any other card.  I

strongly recommend following the chart given above for selecting

these two numbers.

It is interesting to note that WD_DECLARE and WDE_DECLARE differ

only in that WD_DECLARE has output queuing and WDE_DECLARE does not.

You can in fact use WD_DECLARE to declare an Ethercard, but since

Ethernet always transmits packets faster than the PC can place them

in the output queue the output queuing does not make sense for

ethernet.   Thus you can think of WDE as a version of WD that has

been optimized for ethernet.

-------------------------------------------------------------------------

Localtalk:  For every localtalk entry there is a line of the form

ATP_DECLARE %cur_dl, 60H, %cur_dl, 10, 2000H;; <name> <int> <task> <wqueue>

DL_IP_DECLARE %cur_dl, %cur_dl, ATP     ;; <dlip name>, <real dlip> <prefix>

cur_dl = cur_dl + 1

The ATP_DECLARE declares that this card will be called 'cur_dl' and

that it can access the Appletalk software driver that comes with

the card though interrupt 60H.  (this is the correct number for Apple's

version of the localtalk driver).  The next parameter is the task

number needed by ATP and should ALWAYS be %cur_dl.  The next two

parameters is the number of entries in the write queue (in this case 10)

and the total space to allocate to the write queue (in this case 2000H

bytes).  These are tunable, but in general should not be changed.

Also, if the line 'include atalk.inc' has been commented out, you

should comment it back in and comment out code for some other interface

you are not using (wd8003.inc for starlan, slip.inc for slip)

At present, only ONE such localtalk entry is allowed.  The reason is

the appletalk driver that comes with the card can only support one

card.  It should be possible to install two cards (with different

I/O addresses), and install two drivers at different interrupts and

thus allow two localtalk cards.  I will try this to see if it works.

But for now, we only support one localtalk card. (:-(

-------------------------------------------------------------------------

Slip:  For every Slip entry there is a line of the form

SLIP_DECLARE %cur_dl, %cur_dl, 1, 20, 10000 4, 2500

;; <name> <task> <port> <wqueue> <wbuff> <rqueue> <rbuff>

DL_IP_DECLARE %cur_dl, %cur_dl, SLIP    ;; <dlip name>, <real dlip> <prefix>

cur_dl = cur_dl + 1

This line declares a SLIP interface called 'cur_dl'.  The next parmeter

is the name of the task needed by this interface and should ALWAYS be

the same as the name of the interface.  The next parameter is the name

of the COM port (in this case we are using COM1) to use (numbers 1-4

are legal entries here).  The next two parameters are the length (in

packets and byte respectivly) of the output queue.  This queue should

be fairly long since SLIP is a slow interface.  Finally the last two

parameters are the length (in packets and bytes) of the SLIP input queue.

This queue can be quite short, but show be at least 3, 2200 (enough

to hold two full sized SLIP packets.

Also, if the line 'include slip.inc' has been commented out, you

should comment it back in and comment out code for some other interface

you are not using (atalk.inc for appletalk, wd8003.inc for starlan)

=============================================================================

The basic procedure now is to edit DECLARE.INC to make sure that it has

all the proper declarations for the cards you want to use.  I have made

this easy by including declarations for many common card configurations

and commenting out all but the ones pertaining to the two ethernet card

router.  So under almost all situations all you need to do is uncomment

the proper declarations.  Please, doublecheck that the I/O address and

shared memory offset addresses are do not conflict and match the settings

on the WD8003 cards.

Notice that DECLARE.INC also defines the naming of the cards as far as

'config' is concerned.  The first card declared in DECLARE.INC is interface

1, the second, interface 2, and so on.

Once you have edited DECLARE.INC, the rest is easy.  Simply type

tasm /ks170 *.asm

tlink pcroute ip rip log dlog ether atalk wd8003 wde8003 slip

ip_dl1 ip_dl2 ip_dl3 ip_dl4 ip_dl5 ip_dl6

(that is link all the object files create with the tasm command)

This will create a pcroute.exe file.  You can then copy this file

to where config.exe lives and configure the router by following the

in instructions in the installation document.