INSTALLATION of PCROUTE

Installation of PCroute consists of essentially four parts.

1) Compiling the software

2) Setting up the hardware.

3) Configuring the software.

4) Testing.

This section tells you the 'theory' behind the installation

process.  INSTALL.EXA is an example installation of the precompiled

PCrouter provided with this release.

1) COMPILING THE SOFTWARE

PCroute can have as many as 6 network interfaces attached

to it, however, for efficiency reasons, the number and type of

interface must be COMPILED into the code.  For convenience, PCroute

is distributed with compiled version of the Ethernet-Ethernet (called

ether.exe) and and Ethernet-Slip router (called slip.exe).  If either

of these are the type of router you want, simply rename the proper

one to 'pcroute.exe' and go on to step 2

PCroute was written with TURBO assembler 1.0.  MASM does NOT

work, for reasons I have not looked into.  If you want, you can

try porting it but since Turbo assembler cost only $69, it is probably

not worth porting.

Before you compile and link the code, you must first declare

how many and what types of interfaces the router will have.  You do

this by modifying the DECLARE.INC file.  No other files need to be

changed!  For most configurations, it is just a mater of uncommenting

the right declarations.  The general case, however, requires some

understanding of DECLARE.INC.  For a full discussion on this, see

the document COMPILE.DOC.

After you have modified DECLARE.INC simply compile and link

the files as shown below

TASM /KS=170 *.ASM

TLINK PCROUTE IP RIP LOG DLOG ETHER WD8003 WD8003E ATALK SLIP

IP_DL1 IP_DL2 IP_DL3 IP_DL4 IP_DL5 IP_DL6

--------------------------------------------------------------------------

2) HARDWARE SETUP

Hardware setup simply involves getting the proper networking

cards, jumpering them properly, and installing them in the PC

to be used as a router.

The type of cards, and the jumper settings for them are all

determined by the declaration in DECLARE.INC.  In addition each

interface is given a number (starting with one) based upon its

order in DECLARE.INC.  This number will be used below when we

configure IP addresses.

WD8003 Cards (Starlan or ethernet)

There are only two settings that have to be made on each

of the WD8003 Cards.  The first is configuring it for THICK/THIN

ethernet, and the other is the I/O address of the card (interrupts

are not used so the Interrupt number is irrelevant.

Refer to the DECLARE.INC file to determine the I/O addresses

for the ethernet cards.  In DECLARE.INC there will be a line

WDE_DECLARE (for ethernet) or WD_DECLARE (for starlan) for each

card.  The second parameter on this line is the I/O address of

the card.  Set jumpers on the respective card to agree with this

number.   For precompiled routers the first WD8003 card has I/O

address 280H, and the second (in the case of the ether-ether router)

has I/O address 2A0H.

Also make sure to make sure the card is configured for the

correct type of ethernet.  From the factory these cards are

configured for THIN Ethernet (BNC connector), if you are not using

THIN Ethernet (That is you are using the 15 Pin DSUB connector),

you will need to change a bank of jumpers.  Refer to the manual

that came with the card on exactly how to do this.

If you are using a 6Mhz AT (and only a 6Mhz AT), you should

set one more jumper on the wd80003e.  This jumper determines the

number of wait states used when accessing the shared memory and

for a 6Mhz AT the default is not optimal.  Consult the Manual that

came with the Ethercard for more details.

SLIP USERS: If you plan to use slip you should change the

IRQ (interupt) setting on the WD8003 cards to IRQ 2 instead of

the default of IRQ3.  The reason is that COM2 uses IRQ3 and even

though the Ethernet cards to not use interupts, COM2 does, and

the confict will cause the SLIP interface using COM2 to fail.

Localtalk Card

PCroute does not directly access the Localtalk card.  Instead it

goes though the driver that comes with the card (ATALK.EXE).

For this reason PCroute can only support one localtalk interface.

The card should be configured as specified in the manual that

came with the software driver.  The only concern is that there is

no confict (in I/O address or IRQ level and DMA channel) with any

other cards.  The driver (ATALK.EXE) also must be loaded BEFORE

PCroute is run.

Slip Cards

The SLIP interfaces use a standard COM port for a network interface.

At present up to two SLIP interfaces are supported (COM1 and COM2)

To determine which one the software is configured to use consult

the DELCARE.INC file in the source code.  For those using the

precompiled ether-slip router, that router uses the COM1 port.

Make sure that the COM1 card is configured to use IRQ4 and the

COM2 card (if present) is set for IRQ3 (These are the default IRQ

settings for COM1 and COM2).

If you are using COM2, make sure the ethernet cards are jumpered

so that their IRQ is 2.  Otherwise, the confict will cause COM2

to fail.

Finally, if you really need more than two SLIP interfaces, it is

possible, but difficult.  You can contact me if you want details.

-------------------------------------------------------------------------

2) SOFTWARE SETUP

A) Creating a PCROUTE.CFG file

All the configuration data that PCroute needs is in a file

called 'pcroute.cfg'.  To ease programming of PCroute, this file

is NOT an ascii file.  Instead, it is a binary file, and there

is an program called 'config' that edits/creates this file.

Thus the procedure to configure PCroute is to run config in

the directory that PCROUTE.EXE is in.  Config then looks in

PCROUTE.EXE and determines how many and what type of interface

information is needed. It then prompts for this data.  Among the

data config will ask for is

For every interface it will ask for

IP address      - self explanatory

Subnet mask     - MUST begin with 255.255

Flags           - Meaning below

Every interface on IP network must have a unique IP address.

In addition IP has two level hierarchy in its addressing.  Of the

32 bits in a IP address, a certain number of bits is considered the

'network' part of the address and the others are the 'host' part.

IT IS A REQUIREMENT that all hosts on the same network (ethernet)

have the same network part.  Originally, the network part was

encoded into the IP address itself for example if the IP address has

the form

<0-127>.X.X.X       Network part first 8 bits (Class A)

<128-191>.X.X.X     Network part first 16 bits (Class B)

<192-223>.X.X.X     Network part first 24 bits (Class C)

It turned out that this encoding schemed was too limited, so a

'subnetting' mask was added.  Every bit in the subnet mask that

is a '1' implies that this bit in the IP address is part of the

network part of the address.  Note, however, that the subnet mask

have several restrictions

1) Subnet masks can only cause bits that would have been

interpreted as the host part to be interpreted as the network

part.  Thus a class A number MUST have a subnet mask of the

form 255.X.X.X, a class B number 255.255.X.X and a class C

number 255.255.255.X.

2) PCroute network masks must ALWAYS have the form 255.255.X.X

3) A subneted network MUST have a consistant subnet mask.  For

example, let us assume IF1 has IP address 129.105.1.2

and network mask 255.255.255.0, then if IF2 has IP address

129.105.3.1, then it MUST also have the mask 255.255.255.0.

This is because 129.105.1.2 and 129.105.3.1 have the same

network part, 129.105 (as determined by the number's Class)

thus they must have a consistant view of how that network

is subnetted, and thus the same network mask.

Flags - This is a HEX number that determines several configuration

options for the interface.

Bit 0 - Don't send routing (RIP) info out this interface.

Bit 1 - Don't listen to routing (RIP) info from this interface.

Bit 2 - Proxy Arp on this interface.  Normally the router will

only respond to ARP requests for itself.  With this option

turned on, it will reply for itself and any IP address such

that that is in the same network as the interface, but NOT

in the same subnet (For example if the interface was

129.105.3.2 and the network mask was 255.255.255.0 then

it would respond to any ARP asking for an address that begins

129.105 but does not begin with 129.105.3).   This is useful

if you have hosts that do not know how to subnet.

Bit 3 - Disable Directed Broadcasts

Normally it is perfectly legitimate for and IP host to send

a packet the broadcast address of another network.  The

router on the destination network then broadcasts the the packet

on behalf of the sender.  Unfortunately, if you have hosts

running old IP code that handle broadcasts improperly, these

hosts will hear the broadcast and forward it back to the

router, who in turns rebroadcasts it.  This is called a

broadcast storm, and will quickly saturate the network.  To

avoid this problem, you can disable directed broadcasts.

Bit 4 - Disable Sending ICMP redirects

Normally when a PCrouter receives a packet it has to route

out the same interface the packet came in, it sends a

redirect message to the sender.  Many hosts ignore these

packets, so this bit allows you to turn this feature

off and conserve network bandwidth.

Bit 5 - Broadcast using old (0's) format

Normally when PCroute sends out an IP broadcast (sending RIP

for example), it uses the 1's format.  That is it fills the

host part of the address with the number which when views

as a binary number is all 1's.  Old hosts use a simmilar

convention, except they fill with 0's instead of 1's.  Turning

this flag on makes PCroute use the old format.  NOTE THAT

YOU ARE STRONGLY ADVISED TO USE THE NEW FORMAT IF AT ALL

POSSIBLE.  The reason for this is if there is a conflicts

can sometimes lead to broadcast loops which quickly saturate

the network.  Also if you use this option you should also

use option Bit 3 (turn off directed broadcast).

Interface Metric - This is a HEX number that determines the RIP

metric to be added to routes going through this interface.

A large number means that this is an undesirable (slow)

interface.  I recommend the following

Ethernet interfaces             metric 1

Starlan interfaces              metric 2

Localtalk interfaces            metric 3 or 4

Slip interfaces                 metric 4

Next the CONFIG program will ask you for interface specific

information.  In particular

FOR LOCALTALK:

Zone names: CONFIG asks you for a list of zone names

that contain all the other PCrouter (or Macs using

NCSA telnet).  THIS LIST MUST INCLUDE THE ZONE OF

THE ROUTER ITSELF (you can either use its full name

or use the abbreviation '*' (the current zone))

When PCroute needs to find and IP address, it looks

in all of the zones listed here to find it.

FOR SLIP:

Baud rate: Slip allows you to configure the baud rate

of each of the SLIP lines.  Baud rates up to 19200

have been tried.  38400 should work but has not been

tried.   Even 56K is possible, but that is the limit

Note that the IP address for a SLIP network can be done

in two ways.  The first it to treat the SLIP line EXACTLY

like a ethernet, complete with its own network number.

THIS IS THE PREFERED METHOD.  On the other hand, you can

configure SLIP so that it borrows its IP address from the

network that it is connected to.  If you do this, make sure

to turn off RIP on the slip interface.  Also this method

ONLY works with a single SLIP interface.  See SLIP.DOC for

an example of this.

Next the CONFIG program will ask you for the static routes.  For

each static route CONFIG will ask you for

Network number

Gateway to send it to

RIP metric (1-16)

Flags

1) The network number looks much like a IP address except that

it has zeros for the host bits.  PCroute does NOT except host

routes.  The network number 0.0.0.0 has a special meaning,

it is the default route.  This route will be used as a route

of last resort.  Note that as far as RIP is concerned the

default route is just another route.

2) The gateway it the host to send the packet to.  It must be

a host directly attached to a network the router is directly

attached to (that is 1 hop away).

3) The Metric should be the number of 'hops' the network is away

from the router.  Note that this number can be inflated for

less preferred routes (ie though slow SLIP lines for example)

4) The flag bits have the following meaning

Bit 0 - Silent.  This route will not be propagated via

RIP, it will only be used for packet routing.

Bit 1 - Transient.  Normally any static routes are permanent

and cannot be removed by RIP.  By setting this bit

you allow the route to be changed by RIP (which

means this route will be expired in 90 sec if a RIP

update does not come along)

Static routes should be used SPARINGLY.  Use RIP if you can,

and you will save yourself a lot of work when your network

grows.  I have found it useful to put Transient Default route

in the table with a metric of 15.  This allows that router

to operate to between the time that it is booted up and the

time it receives its first RIP update.  Because the route is

transient, it will be replaced or deleted shortly thereafter.

Next the CONFIG program will ask you for the address to forward

BOOTP packets to.  This can be a directed broadcast (and directed

broadcasts are not turned off on all routers connected to the

destination net).  If you have no diskless machines booting across

the gateway, just leave this value 0.0.0.0.

Next the CONFIG program asks you to specify where and what kind

of error messages it should print.  It asks you for a

1) A syslogd host

2) A facility mask

3) A priority (0-9)

1) The syslog host is a BSD UNIX 4.3 system that is running

the syslog daemon.  This daemon listens for error reports

and sends them to a person or a file (as directed by

/etc/syslog.conf).  PCroute logs errors to syslogd as

facility 'LOCAL0'  Thus by putting the line (on 4.3 systems)

local0.debug        /usr/adm/router

and restarting syslogd, the daemon will log all messages

from a PCrouter to the file /usr/adm/router.

2) The facility mask allows you to filter out messages from

certain parts of PCroute.  This is really only of use for

specific debugging, and to this mask should usually be 0.

3) Error messages from PCroute are given priorities anywhere

from 0 (Critical) to 7 (debug).  By setting the priority

low, you can filter out messages.  In addition by specifying

priorities above 8, you make PCroute log a status message

every minute.  This is useful in the beginning to make sure

PCroute is still alive.  For production system, you should

set this value to 7 (everything but debugging).

B) Create a Floppy to boot from

At this point we are ready to create a floppy that will be

used by the PC that will actually be the router.  To do so create

a bootable floppy using the FORMAT with the /S option.  Do this on

a low density drive it the PCrouter has a low density drive.

Next copy the PCROUTE.EXE and PCROUTE.CFG files.  If you are

running any localtalk interfaces, copy the ATALK.EXE driver too.

Finally create a AUTOEXEC.BAT file that will execute PCROUTE

when the machine boots.  If you are running any localtalk interfaces,

the AUTOEXEC.BAT file should load the ATALK.EXE driver before

running PCroute.

------------------------------------------------------------------------------

3) RUNNING and TESTING

We are now ready to go.  The machine has been installed with the

proper cards and a bootable floppy with the software on it has been

created.  Insert the floppy into the PCrouter and try booting it up.

1) Pinging the PCrouter.  Try pinging the PCrouter from a local host

on a directly connected network.

2) Check the log file on the syslogd host.  It should say the the

router is up (unless you filtered out the message)

3) If the router seems dead, remove the floppy and go to a PC.

Look at the file PCROUTE.LOG on the floppy.  This file was

created when PCroute booted up.  In particular check to

see that the Ethernet address are reasonable and that the file

ends with the message 'PCroute closing log file'.

4) If the PCrouter replies to the Ping, try pinging it from distant

hosts.

5) Also try looking at the routing tables of host that are running

RIP (routed, gated).  See if the routes that PCroute is announcing

are being picked up.

Congratulations!! you just installed PCroute.  If you have any NFS

file systems mounted through the router you should edit the /etc/fstab

(or equivalent) and set the rsize and wsize parameters to 4K so that NFS

will not send more than 4K of data in a quick spirt. (see readme.doc)

------------------------------------------------------------------------

REFERENCES

For those who are unfamiliar with IP ARP, RIP etc here are

some good references.

IP Tutorial - Internetworking with TCP/IP by David Comer

available at your local bookstore

RIP		- rip.doc. Available most places RFC's are available.

In particular it is available by anonymous ftp from

uxc.cso.uiuc.edu (128.174.5.50) in dir RFC

nic.ddn.mil (10.0.0.51) in dir RFC

RFC		- These are the definitive references for internet standards.

They are referenced by numbers, and ther is a master

index that is updated periodicly.  The are available

from (among other places)

uxc.cso.uiuc.edu (128.174.5.50) in dir RFC

nic.ddn.mil (10.0.0.51) in dir RFC

Of particular interest might be

IP protocol - RFC791

IP subnets - RFC950

ARP protocol - RFC1011

ICMP protocol - RFC792

UDP protocol - RFC768

BOOTP protocol - RFC951, 1048, 1084