YAMAHA PATCH EDITOR/LIBRARIAN FOR THE PSS SERIES

VERSION 2.0

copyright 1991, 1993, 1994  by Mike Silverstein

Well, here goes!  About 5 years ago I purchased a Yamaha PSS-480

keyboard synthesizer to accompany my new Atari 1040 ST.  I was amazed

at the MIDI capability of this synthesizer which, at the time, cost me

a mere 150 bucks.  Six months later, I purchased a modem, joined

GEnie, and searched for anything that would support this synth in both

the Atari ST and MIDI Roundtables.  I found an excellent PD program

which would allow you to send a bulk dump from the synth to the

program and it would successfully capture all the System Exclusive

Data (SYSEX) to disk which includes all the voice banks, memory/chord

banks, and custom drummer (somewhere around 13K of MIDI data).

It was at this point where I became very intrigued in SYSEX stuff and

I began searching for other SYSEX programs, but I could only find

demos of 'Universal' programs like Dr. T's XOR, Hybrid Arts GenEdit,

etc.  I even purchased PixelPro's Superlibrarian which claimed to let

you write your own library routines through a 'simple' interface.  If

you ask me, this was 200 dollars worth of nothing but pain and

turmoil.  You see, these commercial universal SYSEX programs are

geared towards more expensive synths.  And even so, what you pay for,

you only use about 10% because your setup probably uses no more than a

couple of synths and a drum machine.  None of these programs support

this 'KMART' type of synthesizer (i.e.  profiles) and you would have

to go through a lot of nonsense to attempt to write your own profile.

Well, I eventually joined an IEMUG board and found a European import

which supported the Yamaha PSS series called PSSED.  It worked in ST

LOW rez only and had a real nice graphic interface.  Unfortunately,

the 'Quit' function and the librarian feature didn't seem to work.

The only possible thing to do was receive the five patch banks and

save them all as a set.  Well, now I was REALLY intrigued.  I'm not

sure if these features didn't work because it's from Europe and the

ST's there are different or what.  No way was I able to contact the

author so I finally realized that it was time to say, "The heck with

it!" I'll write my own."  Thus, PSSLIB was born (but much inspired by

PSSED).

I was amazed to learn that (only thru software) there's 3 times more

parameters you can edit when building patches.  And having the ST

accompanying you for MIDI work is practically a must have!  After

reading this doc, I hope you enjoy this program as much as I had

writing it and using it!

The original version of PSSLIB goes back to late 1990/early 1991 when

the Yamaha PortaSounds were more popular.  But even now (1994), I

still enjoy the original AM/FM tone generaration/synthesis sounds vs.

what the newer synths offer via Pulse Code modulation.  Although these

synths are outdated, there are still many in use today and I still

believe that they are good MIDI synthesizers, especially for beginners

to the MIDI arena who don't want a lot of fancy controls/gizmos.

=========================================================================

NEW IN VERSION 1.1 (DEC 1991)

Since 2 years have gone by since the original release, I haven't

really done anything exciting since I've been using my ST on other

ventures.  However, I have added Active Sensing capability and you'll

notice that if your MIDI cables are not hooked up right or your PSS is

turned off, you will not be able to send or receive MIDI data (makes

sense, doesn't it??).  I haven't come across one MIDI program which

uses this feature and since it is in the MIDI spec, why not be the

first person to implement it (GEMwise, that is!).

I have also deleted the necessity of the OLDMIDI/NEWMIDI external

files which expanded the system MIDI buffer; this is now done in the

GFA source code.

I have also "channelized" the 5 banks whenever a program change is

performed as opposed to Version 1.0 which just sent all patches on

channel 1.  Now, Bank 4 will be transmitted on Channel 4. This works

for both sending and receiving and adds some MIDI mechanization when

sequencing.

Finally, I have made the LZH archive capability more versatile when

other Accessories or programs are running simultaneously. Thanx to

help from GEnie members in the GFA Basic topic, I have learned how to

be more efficient with memory mangement and have gotten rid of the

RESERVE command when performing an EXEC call to the LHA.TTP program.

I have used the $m command and reserved 50K bytes which handles the

PSSLIB and LHA applications with no problems.  I have also tested this

with a Falcon030 and it runs fine (and no, I don't own one yet -

uggh!)

The most important thing is that I have decided to let this program

package be freeware although you are still encouraged to send the

donation.  I have also included the Source Code and all programmers

can feel free to see the internals of the patch editor/librarian

features.  It is heavily commented and very straightforward

programming.  I have also included the source to the PSSKEY and you

can see how the code word was implemented.  Basically, if you modify

the source, you need to use a hex editor (i.e. LookIt from Codehead or

Edhak from Clear Thinking) to find the keyword and then use the 2

decimal offsets and replace them in the PSSKEY source (this is on the

actual executable PSSLIB.PRG - make sure it is unsquished or unpacked

when you perform the search). Oh, BTW, the key word is 'midilation'.

========================================================================

NEW IN VERSION 1.5 (FEB 1993)

It was discovered that the 50K used in the $m compiler command was

slightly lower than I originally tested.  After receiving SYSEX data,

certain help dialog boxes would not operate and certain file

functions (i.e. save patch) would not work. A GFA Error 8 would occur

which indicates memory is full.  I've increased this size to 100K

which means other programs and DA's will have 50K less memory than

before.  The current version now takes about 220K of memory which is

still better than the original which sucked up everything it could

grab.

I have also compiled the PRG with the Interrupts Option flag (I+) set.

This has eliminated some checksum errors involved when receiving MIDI

and assists with the system overhead when many TSR applications are

loaded in the ST AUTO folder.  I used to have these checksum errors

with both Codekeys and Hotwire from Codehead Technologies and this

option appears to eliminate the checksum errors (we're talking maybe 1

bit of 1 byte of data would be incorrect.)

==========================================================================

USING CODEHEAD'S MIDI SPY WITH PSSLIB

Note for all ST MIDI people:  I have recently purchased the MIDI Spy

/ MIDIMAX package from the Codehead's and, I must say, is an excellent

addition to your software collection.  MIDI Spy's SYSEX capabilities

compliment PSSLIB giving you the capability to send all patches to the

PSS in MIDI Spy's MIDIMAX Mode.  I don't have to worry about writing a

separate Desk Accessory since MIDI Spy does this already!  For those

interested, I wrote a very commendable review in the December 1993

issue of ST Informer (hey, I finally made the big time!) so feel free

to read about it there.

With MIDI Spy running in the background, you will be able to load

patches and send them to a bank on the synthesizer and MIDI Spy will

capture this (as a Song) in it's internal buffer.  It is a requirement

to have the MIDIMAX button enabled in MIDI Spy's configuration,

otherwise, PSSLIB will not be able to send any data since MIDI Spy

disregards Active Sensing, thus, PSSLIB's MIDI functions will be

disabled.  To conclude, the MIDIMAX button in MIDI Spy may be

considered as 'Communicate with PSSLIB' (or other MIDI programs as

well).  If you don't want to capture ala MIDI Spy but it just happens

to be there, you can always disable the MIDI IN button in MIDI Spy

which will allow the active sensing to turn on.

Once SYSEX data is captured to MIDI Spy, you can do all sorts of

things.  Disable the MIDIMAX button and start recording stuff (with

'Thru' on and the Yamaha PSS in MIDI mode [c.99 - sound source].

Assign the voice/channel via the bank number of what you sent via

PSSLIB.  (i.e. bank 3 would be voice 102, channel 3).  After you've

recorded your song and save it, the SYSEX transmission to send the

patch is embedded in the song file!  This is a convenient way to save

your SYSEX patches as .MID Standard MIDI files to be used with other

ST sequencers (i.e. Omega 2 from Dr. T, Cubase from Steinberg, etc.).

It is best to place all your embedded patches at the beginning (as

opposed to within a song).  For some bizarre reason, although the

SYSEX transmission is usually successful, there is a 3-4 second delay

requirement between each SYSEX message so you'll need to embed this

with your sequencer.  The Yamaha's internal processor must need this

time to properly set the tone generator's correctly.

I have also noticed that PSSLIB MIDI receive functions do not work

well with MIDI Spy running in the background due to MIDI Spy's

overhead in the system.  You usually will get checksum error's so its

best to not use MIDI Spy with PSSLIB if you plan on receiving patches.

The same thing applies to other programs like MultiTOS, Geneva.  Bear

in mind that I use a standard 8 Mhz 1040 STfm 4 Meg system.  Perhaps the

receive functions work with an accelerated ST/STe/TT or Falcon030.

Both MIDI Spy/MIDIMAX may be purchased from Codehead Technologies for

$99.95 or MIDI Spy (alone) for $79.95.  Believe me, they're well

worth the price.  Here's how to order:

CodeHead Technologies

P.O. Box 74090

Los Angeles, CA 90004

Voice: (213) 386-5735

Fax:   (213) 386-5789

BBS:   (213) 461-2095

==========================================================================

NEW IN VERSION 2.0  (APRIL 1994)

SYSEX FILES

I've added a lot of new features to PSSLIB.  The 1st feature is the

ability to IMPORT/EXPORT a SYSEX file.  I've decided to include this

because I've noticed that there are other programs available which

deal with SYSEX stuff and it wasn't too painful to implement this.  I

still promote the internal LZH archive method since patch files are

so small and don't need to be considered 1K in size to the system,

wasting "valuable" disk space.  However, storage is getting cheap

nowadays and I've reasoned that there are practical uses in keeping a

patch as a separate file.

I know of 2 programs which use embedded SYSEX files: The newest

version of Dave Henry's shareware MIDI Music Maker (MMM) and ST MIDIEX

by Harry Wootan of Polk Products (both GFA programs I might add).  I

have added these functions to PSSLIB's online help to learn more about

them.  I have also included several SYSEX (.SYX) in a separate PATCHES

Directory.  The number in the file corresponds to the synth bank

embedded within the file.

Note that when you save a patch, one gets saved (with comments) to the

LZH archive and you are given a choice as to export a new .SYX file

with the same name you provided for the patch.

You must also realize that PSSLIB's buffer only represents current

data from the LZH archive only and that import/export bypasses this.

I've made sure that if you use the 'Load Patch' selection and enter a

.SYX file in the file selector, an error message alerts you of your

mistake.  At this point if you View the Buffer as Hex data, you will

see that the box looks funny and some bytes have 3 characters which is

a definite suggestion that the data is bad.  Doing a 'Send Bank'

selection will always cause a transmission error (Hey, this is a good

way to check that your 'ERR' display function is working on your PSS).

It is also important to explain the difference between the MIDI data

in PSSLIB's buffer vs. the data in a .SYX patch file.  The buffer

contains 64 bytes which does not include any SYSEX header/trailer

information.  The .SYX file contains 72 bytes which includes the Start

of Sysex (F0 hex), followed by the MIDI Instument ID for Yamaha (43

hex), followed by the Instrument Class (76 hex), followed by the

SYSEX mode (00 hex for patches), followed by the bank number.  The

trailer information includes an embedded checksum and the End of

SYSEX message (F7 hex).  At the time you load an existing patch from

the archive, you are given a choice of 5 banks to send the data.

Therefore, to save (export) to a SYSEX file you must first send it to

the desired bank and then receive from that same bank.  Here's an

example:

Let's say you want to have the WIRY2 patch (bank 2/channel 2)to be

used on bank 4/ channel 4 (WIRY4). You would:

a) Load Patch and enter WIRY2 in File Selector

b) Send to Bank 4

c) Receive from Bank 4

d) Save Patch as WIRY4 and export it.

If you disregard steps b and c (that is, don't send and receive), the

patch you save/export will include the wrong bank and no embedded

checksum, therefore, you will definitely get a MIDI Transmission

error when using this .SYX file, elsewhere.

CUSTOM DRUMMER

PSSLIB now supports the programmable CUSTOM DRUMMER which is part of

the Yamaha PortaSound's internal drum machine.  To review how to use

it: make sure the PSS is in MIDI mode [c.00 - keyboard assign mode],

pick 1 of the 100 internal Styles and press the 'Program/End' button

underneath 'Custom Drummer' on the PSS.  You can now edit the style by

Clearing/Accenting/Rolling with the MIDI C1/D1/E1 keys (at the very

left) and selecting the drum note with the MIDI keys (E4 - C5) at the

very right of the keyboard. (Note - this may not be the same for the

PSS-680 but I don't own that so you'll have to consult your manual).

After editing, you can always play it back via the green button 'Play

On/Off'.  To save it to disk, use the Export Drummer selection and it

will be saved as a .SYX file.  Since the Custom Drummer SYSEX Message

is the very last message defined for the PSS, the internal MIDI

receive buffer is temporarily adjusted to 13.8 Kbytes, therefore,

taking a lot more time to receive the data.  They should have put it

after the Patch SYSEX messages at the beginning.

You can load the Drummer SYSEX data back via Import .SYX just like

patches (and/or use it with other MIDI compatible programs).  I have

included several modified Styles in a separate DRUMMER directory.  As

long as you're in mode [c.00] and the Custom Drummer 'Play On/Off'

button is enabled, you can cycle thru the various drum creations you

saved since PSSLIB automatically sends MIDI Start/Stop commands when

interfacing to the drum machine.

Once again, a reminder: don't use 'Load Patch' for loading SYSEX drum

files (690 bytes).  Sending this data will cause transmission errors.

AUDITION MODE

As promised, I have implemented an audition mode feature when either

sending data to a bank or importing a .SYX file.  It will always cause

three MIDI notes to play, thus, allowing you to hear your patch

creations to get it to sound the way you like before saving them to

disk.  Initially selecting this mode via the mouse/menu will cause a

new dialog to allow you to enter 4 possible DELAY intervals between

notes.  You would use short delays for patches whose envelope

parameters contain high attack rates, decay rates, and sustain rates.

You would use longer delays for patches with lower envelope values. I

have chosen MIDI notes C1, C3 and C5 so that 3 distinct octaves may

be heard, thus, aiding you in defining LKS/RKS (Level/Rate Key

Scaling) patch parameters.  To disable Audition Mode, you may uncheck

it again.  Once again, I have added this to the on-line help.

Because the dialog box may be awkward to some users, I have provided

a keyboard equivalent approach to the audition mode as well.  Note

that a Custom Drummer file is always "auditioned" regardless of the

mode as long as your PSS is in MIDI mode [c.00].

OTHER STUFF

Other changes to PSSLIB:

- increased the memory allocation to allow for all the new features.

PSSLIB now consumes about 280K of memory.

- Active sensing now uses the AES libraries to pause for the required

300 milliseconds.  This speeds up all the commands, especially,

using the mouse/menus.

- provided key equivalents to all new features. Selecting 'K' will

display a summary of all new key commands.

- adjusted hi rez help screens for monochrome users.  The text should

be more readable now.

- PSSLIB will no longer exit if you type in an incorrect patch file

when loading a patch.  You will get a 'File Doesn't Exist' message

and the File Selector will allow you to re-enter the correct name.

Use the Verbose command to find the right name.  This works for

importing SYSEX files, as well.

==========================================================================

ARCHIVE  CONTENTS

You should have the following files in the same directory:

PSSLIB.PRG - the PSS Patch Editor/Librarian program

PSSLIB.LST - source code in GFA BASIC

PATCHES.LZH - system archive file used to store and load all work

PSSKEY.PRG - key program to unlock the 'Save' feature

PSSKEY.LST - source code in GFA BASIC

PSSLIB.DOC - this file

LHA.TTP

WHATS.NEW

LHA.MAN

README.1ST - LZH utility + support files

A sub-directory (PATCHES) containing several .SYX patch files.

A sub-directory (DRUMMER) containing several .SYX drummer files.

==========================================================================

FEATURES

This program was written in GFA BASIC 3.07.  The program is completely

GEM driven and includes online help for basic definitions and patch

editing concepts.

Runs in both medium and high resolution on any STf/STe/Falcon030 (1Meg

minimum). Not sure about TT030.

There are 2 view modes - displaying the buffer in its 'English form'

where everything you edit is summed up in 1 screen display.  The other

is viewing the buffer in hexadecimal form which is what is sent thru

the MIDI ports.

MIDI Receive Capability - you can receive any sound stored within the

5 banks.  This can be used for obtaining any of the 100 built-in

voices and modifying it with your own additions.

Keyboard Equivalents for every GEM menu drop-down with the exception

of Editing all patch parameters. The mouse is used extensively here.

The left button will increase and the right button will decrease a

patch parameter.  Both Buttons will always bring you back to the GEM

Menus.

LZH Archive storage and retrieval support. This saves a lot of disk

storage space since each patch need only be 64 bytes. Since TOS (like

MSDOS) makes a file a minimum of 1024 bytes, this was a logical

approach.  You can also add comments to your patch when saving which

will also be saved within the archive.

==========================================================================

OTHER CONSIDERATIONS

I'm not really gonna go into a 'how to run this program' deal.  That's

what the online help is for.  I am going to point out that if you just

have a floppy drive only, the program will run a lot slower so may I

suggest a  300K - 400K  RAM disk??

I chose GFA BASIC because it was simple and straight forward.  The

only thing I had a hard time with was dealing with the MIDI overrun

error when receiving a memory bulk dump from the synth.  Yamaha was

nice enough to put the 5 bank patch stuff at the beginning so I made

the buffer 512 bytes which is enough and speeds up the MIDI interrupt

service routine.

I hope to have provided as much error checking as possible.  If

support files are missing, there should be enough alert boxes that

handles each situation.  In the case of receiving MIDI data, once the

program is waiting for MIDI data, that's it!  If you don't do the bulk

dump, you'll have to reboot.  However, you will not get to this point

unless your MIDI connections are valid and your PSS is powered on.

The purpose of turning off and on some Menu Items was more

experimentation on my part.  Logically, when you start the program,

you wouldn't want to save anything, just like you wouldn't want to

load anything after you've just edited because you haven't heard it

yet.  However, you can always choose 'View' if you always want to be

able to select something.  I hope this is not an annoyance but more of

an aid.

All of the key equivalents override the toggling of menu items, that

is, you can do anything with keys (some have alert boxes to start you

off).  I've done this because I'm a firm believer that it is an

absolute necessity to have key equivalents if you're forced to use the

keyboard within a program.  I originally wanted the program to be 100

% mouse driven.  However, since I  decided to go with archiving files,

this does force you to use the keyboard.  This was a tradeoff but I

felt it more necessary to go with archives to save as much disk space

possible (I'm kind of a miser in that respect). I may experiment and

see if I can get the patch file into a box graphic to allow 100 %

mouse capability.  But then again....................

The MIDI specs in the Yamaha PSS manual can be pretty confusing.

However, it didn't take long to figure out how to convert the decimal

parameters to binary nibbles (that's 4 bits or half a byte) and vice

versa.  GFA's powerful bit manipulation instructions were a big aid in

this effort.  In fact, I can't recall another BASIC language on any

platform that allows bit testing, bit shifting, etc.  It's the next

best thing to C but a whole lot easier to program!

Since there is a 50 character limitation when providing comments to

your saved patch, here is a short glossary of SYSEX acronyms which I

use which you can use as a reference:

atk - attack rate

dcr - decay rate

dcl - decay level

mod - modulator

car - carrier

freq - frequency

LKS - Level Key Scaling

RKS - Rate Key Scaling

rr - release rate

ss - sustain rate

Feel free to use your own nomenclature to your own preferences when

describing your patch sounds.

There has been some confusion concerning the 2 distinct MIDI modes on

the Yamaha Portasound.  You would use Mode [c.00 - Keyboard Assign

Mode] to play the synth by itself, rhythm machine and all.  You select

a voice or bank and it plays - simple.  Mode [c.99 - Sound Source] is

used only with a Program supporting MIDI Echo or MIDI Thru capability,

turning the PSS into a 12 note polyphonic multitimbral keyboard so you

can hear many voices/banks simultaneously (so long as there are no

more than 12 on together).  For all intent purposes, you should use

Mode [c.00] with PSSLIB since we're assigning and playing back one

sound at a time. It is also the only way to deal with the Custom

Drummer and the audition feature will not adjust to the right sound

unless mode [c.00] is used.

Note that with the Audition mode turned on, if you don't happen to be

looking at the PSS when sending MIDI data and there is a transmission

error, the sound you'll hear we'll be one of the first five default

voices (00 - 04) depending on the bank that was sent.

==========================================================================

QUIRKS AND BUGS

I only own a color system but I handled monochrome via an emulator.

I'm not sure if some displays may come up messy but I did the best I

could.  I'm pretty convinced the program is totally runnable in high

rez.

Codekeys from Codehead Software, when installed, interferes with MIDI

reception in that there are usually checksum errors encountered when

attempting to receive a bank from the Yamaha instrument.  I was told

they use the same Keyboard/MIDI interrupt coming from the ACIA chip

which was an Atari design flaw.  Hey! It's nice to see those error

checking routines doing their thing! And if you look closely at what

was received, 1 or 2 MIDI parameters hold the wrong value.  This same

concept applies to Geneva/MultiTOS or if there is a lot of system

overhead due to Desk Accessories.  It is for this reason that I

personally prefer Hotwire from the Codeheads as my shell/desktop

since running stuff from Hotwire seems to eliminate these overhead

problems.

When you choose 'Build Patch', bear in mind that certain parameters

are stuck with some of the Envelope parameters (i.e. Course detune

goes with am enable, decay 1 rate).  Don't know why Yamaha implemented

it like that - there's plenty of 'don't cares' which could have

avoided this.  At any rate, the idea of 'building' is mainly to learn

how a patch sound is formed.

When using both mouse buttons to exit Help or Editing Parameters, you

don't necessarily have to press them both simulataneously, but rather

press 1, then the other easily.  Occasionally, the GEM pull-down items

will get 'stuck'.  In this situation, just press either button and

they will reappear.

==========================================================================

FINAL NOTES

I hope you use this program to your enjoyment.  Your Yamaha synth is

nothing fancy and that is why this program is not fancy, but it gets

the job done.  I have included a handful of patches in the archive to

get you started with.  The only thing left is your creativity.  You

will find through experimentation and trial and error that this synth

is capable of producing some pretty nifty sounds.  Some of the more

'eerie' ones sound pretty neat through your stereo system.  My setup

includes a 'Y' stereo adapter cable attached to the Headphone input

of the PSS with an RCA stereo splitter on the other end.  These are

connected to the Left/Right Plugs on my stereo receiver.  Although

only the drums are "stereoized", I can still record to my tape deck to

hear my MIDI creations in the car/on the road!

A special thanks goes to Roger Burrows for his LHA archiving utility.

Please support him by registering LHA130.

Another special thanks goes to Steve Johns of Johnson Software for his

interesting MIDI Programming Tips in START Magazine.

Still another round of applause goes to several GEnie members in the

GFA topic who have countlessly explained to me the internals of ST

programming and helping me fix bugs.

And finally, to my darling wife Carrie and 4 yr. old daughter

Brittany, who  have put up with me these past months while undergoing

this 'MIDI obsession.'

Remember to run the PSSKEY program and enter 'midilation' to change

the patch editor from DEMO to Registered mode.  This will allow you to

save your patches with a description, as well as export a .SYX file if

you desire.  I have included the source code for your enjoyment.

At this point, I can't think of anything new to add to PSSLIB.  Sure,

I could support the Song Memory SYSEX features which include 5 Melody

Banks and 5 Chord Banks, however, since this represents about 92 % of

the entire SYSEX data, there are other programs around which will do

the job.  STMIDIEX (shareware) has the ability to save any SYSEX

message as a separate file.  YAMA_PSS is a quick utility which saves

an entire bulk dump of all supported PSS SYSEX messages into 1 file.

For now, unless there are severe problems with this new version

(2.0), you can probably expect that this is it!

This program is freeware and copyrighted!  Please feel free to

distribute it so long as all support documentation is included (that

means all of the LHA documentation as well).

Don't forget to download the freeware program:

YAMV_ED Voice and Style Selector - This program lets you select any

voice or rhythm via color menu displays.  It also remembers what

you've selected and auto-switches each voice and rhythm in 'pseudo'

real time when playing the Yamaha synthesizer.  Search for PSS,PSS480

on GEnie to get the file (it's in the ST Library as well as the MIDI

library.)

For MIDI Spy Owners, I have written a quick & dirty Jukebox creator

utility (MAKEJUKE.ZIP), which uses the Tomshell Command Line

Interpreter to allow for days and days (or another word for unlimited

amounts) of constant music!.

Please address all comments and concerns to M.SILVERSTE3 here on GEnie

EMAIL.  You may freely distribute this program on any BBS, however,

everything in the archive should be included.  I encourage everybody

to create their own sounds and upload them as well for others to share

with.

If anyone's interested, I have a massive collection of Standard MIDI

files (type 0, 1 track) which consist of some great sounding tunes

ranging from the Beatles to Pink Floyd, Jazz, POP, Hard Rock, Steely

Dan, Oldies, etc, etc.  They are all configured for Yamaha

Portasounds and sound really good.  I do this as a hobby, that is,

download them here from GEnie and rechannel/revoice all of the tracks,

as well as play with the velocities and pitch bends.  Leave EMAIL if

you're interested in getting a floppy full of tunes!

Enjoy!

Mike Silverstein 05/07/94