MIDITHRU - A Small Convenience by Clinton Keith

This desk accessory simply takes the input MIDI data and sends it though

the MIDI output port.  This makes the ST MIDI out a MIDI thru which I

found usefull especially if you have more than one MIDI device (such as

a keyboard and keyboard module).

Since the ST doesn't have a thru, its a little difficult to use as a

controller.  If you want to have your ST MIDI output go to the module

and use the keyboard to control the module, you'll either have to switch

cables everytime or buy a thru box to throw in the loop.  MIDITHRU allows

you to switch between the two configurations on the desktop.

The MIDI cable connections I use for this are as follows.  Connect the

MIDI out from the ST into the MIDI in of the Keyboard and vice versa for

the other side.  Then put a cable from the keyboard thru to the module.

To have the ST play the module, leave the accessory off.  To play the

module through the keyboard, activate the accessory.

I tried to make this accessory stay in the backround, but, obviously,

performance will bite it if the ST is hard at work.  The only dangerous

tendencies MIDITHRU has exhibited are when the accessory is enabled and

a MIDI application is run at the same time.  So much for resource

management!

MIDITHRU is in the public domain.  Please send any sugestions, curses

and threats to C.KEITH2 on GEnie.

Clinton Keith