************************** 520ST SOUND WAVES *********************************

**************************** by JIM LUCZAK ***********************************

USING SOUND WAVES

The display screen is divided into 8 sections. One section for each of

the 3 VOICES, and sections for NOISE,PERIOD,CONTROLS,WAVE SHAPE, and

BASIC CODE DISPLAY. To activate a voice, simply move the mouse pointer

to the TONE or OCTAVE value desired and click the mouse. You will hear

a short BEEP and the value that you selected will turn RED. To change

a value, point to a new value and click the mouse. The old value will

return to normal and the new value will turn RED. To turn a voice off,

click the OFF box for that voice . For the voice to play, you must have

both a TONE and OCTAVE value selected. This same procedure is used for all

the sections of the display. To activate or deactivate a section click on

the BOX or VALUE desired.

The PERIOD section (ENVELOPE FREQUENCY) is slightly different. The values

in the COURSE and FINE controls, determine the frequency of the envelope.

These values are cumulative. For example, if you click on COURSE 1 and then

click on FINE 7 both values will turn RED. The values will add togeather

to produce a longer period. To deactivate a PERIOD value, click on the

value that is active (RED). The value will return to normal. You can have

all of the values or any combination in the COURSE and FINE controls

active at once.

As you select different values, the BASIC CODE required to produce the

tone is displayed in the BASIC CODE section of the display. This section

is automatically updated each time you make a change. If you create a

sound you want to reproduce in your own program, simply jot down the code

that appears in the BASIC CODE section.

THE WAVE COMMAND

Listing 1 breaks down the WAVE command and it's parameters. The first

parameter (ENABLE) can actually have a value from 1 to 63, depending

on what combination of voices and noise you select. The values in the

chart for the ENABLE parameter are typical values that may be used.

The ENABLE parameter is a six bit word that enable the TONE and NOISE

channels. Listing 2 shows a breakdown of the ENABLE WORD. The ENVELOPE,

SHAPE, PERIOD, and DELAY parameters shown in Listing 1 are the actual

values to use.

********************************************************************************

Listing 1

THE WAVE COMMAND

SYNTAX:  WAVE enable,envelope,shape,period,delay

ENABLE:     ( Enables VOICE or NOISE or VOICE with NOISE )

CHANNEL		TONE VALUE 	NOISE VALUE 	NOISE with TONE VALUE

1		1		8		9

2		2		16		18

3		4		32		36

1 + 2		3		24		27

1 + 3		5		40		45

2 + 3		6		48		54

1 + 2 + 3	7		56		63

----------------------------------------------------------------------------

ENVELOPE:     ( Enables ENVELOPE for VOICES )

VOICE		VALUE

1		1

2		2

3		4

1 + 2		3

1 + 3		5

2 + 3		6

1 + 2 + 3	7

-----------------------------------------------------------------------------

SHAPE:     ( SHAPE/CYCLE control )

VALUE		DESCRIPTION

1-3		alternate,hold

4-7		attack,alternate,hold

8		continue

9		continue,hold

10		continue,alternate

11		continue,alternate,hold

12		continue,attack

13		continue,attack,hold

14		continue,attack,alternate

15		continue,attack,alternate,hold

-----------------------------------------------------------------------------

PERIOD:     ( Controls frequency of the ENEVOLPE )

VALUE		DESCRIPTION

001-128		FINE tune value

256-32768	COURSE tune value

-----------------------------------------------------------------------------

DELAY:     ( Sets time in 1/50 second increments before

BASIC resumes execution )

Use any desired value. If your WAVE command does'nt seem to be

working the way you think it should, try increasing this value.

----------------------------------------------------------------------------

HOW TO USE THE WAVE COMMAND.

First use the SOUND command to select the voice or voices desired, and Set

the NOTE and OCTAVE values. Set the VOLUME and DURATION to 0. When using the

WAVE command the VOLUME and DURATION values in the SOUND command are disabled.

Next set the values for the WAVE command. As your program runs, the SOUND

command will take on the parameters set in the WAVE command.

********************************************************************************

LISTING 2

THE ENABLE PARAMETER

BIT	FUNCTION	  DECIMAL VALUE

0	Tone  Channel 1		1

1	Tone  Channel 2		2

3	Tone  Channel 3		4

4	Noise Channel 1		8

5	Noise Channel 2		16

6	Noise Channel 3		32