USING THE LOGITECH MODEL DEXXA 2B (M-M27) SERIAL MOUSE WITH

ATARI 680x0 COMPUTERS (ST/STe/Mega/Mega STe/Falcon/TT)

A good mouse is hard to find for the Motorola based Atari

computers.  I've found that the Atari OEM mice just don't hold

up very long and you have to mail order to get one that will

work.  I recently ran across some software that allows serial

mice to work with the Atari ST.  You can get a pretty decent

mouse for just around $10.00 if you buy the Logitech model Dexxa

2B (M-M27) 400dpi serial IBM PC compatible mouse.  This is great

if you don't mind losing your serial port and having to convert

from the DB-9 connector to the DB-25.  The best option is to

make the serial mouse work with the ST in the conventional

manner by following the step-by-step procedure that I have

outlined below.  If you don't have the time nor the patience to

perform this then ask someone who can or give us a call and we

can help.

WARNING: Custom Design Technology will not be held responsible

for any damage to your computer or any related hardware or

software problems that may be caused by any mistakes that you

may have made or by your inability to properly modify this

equipment.  If you have any problems, we are here to help.

It is our option to limit the amount of time we spend regarding

over the phone assistance.  We also reserve the option to refuse

assistance if we are busy with our regular business priorities.

Before you run out and buy one of these mice to perform

surgery on, be sure you save the cord assembly from your

ST mouse since you will need it for this one (The original

Logitech mouse uses only 6 conductors and the ST requires 8

total).  You might be able to makeup a cable if you can locate

some flexible small diameter 8-conductor cable and a standard

DB-9 female connector.  Some soldering experience is required

along with some familiarization with replacing surface mount

devices (SMD).

WHAT YOU NEED

(1).  330 ohm surface mount type resistor.

(2).  Phillips screwdriver (not the beverage)

(3).  Diagonal cutting pliers

(4).  Low-wattage soldering pencil

(5).  Solder sucker or solder wick

(6).  Needle nose pliers.

(7).  Short piece of small gauge bare tin plated wire or similar

(Approx. 1").

Outside from having to replace a surface mount device on the

circuit board, the modification is pretty straightforward.

MOUSE DISASSEMBLY

(1).  Rotate and remove the locking plate securing the mouse

ball.

(2).  Remove the ball.

(3).  Remove the two screws that hold the mouse together.

(4).  Carefully separate the two halves of the mouse starting

with the palm end first to prevent from breaking the two

locking tabs at the switch end of the upper shell.

(5).  Lift the circuit board assembly out of the mouse proper,

being careful not to break or bend the two plastic

interrupter wheels.  Try not to misalign the devices

marked LD1, LD2, LQ1, LQ2.  If you do, then it's not too

hard to line them back up again.

(6).  Carefully remove the cord "tail" by unplugging from J1 on

the mouse's circuit board.  It will make a nice addition

to your junk box.

MOUSE MODIFICATION

(1).  With solder wick or sucker, remove the integrated circuit

U1 without tearing up the circuit traces.  Clear the holes

of solder using low heat and solder wick.  Discard U1.

(2).  With the same care, remove the SMD device marked Z1 and

discard.

(3).  Remove SMD resistor R3 and discard.  It should be marked

with a tiny "390" on it.  If necessary, use two soldering

pencils with one on each end of the resistor to remove it.

Clean up the circuit board pads with low heat and solder

wick to prep it for the new resistor.

(4).  Install the new 330 ohm SMD resistor at the location marked

R3 and solder.  The new resistor should have a tiny "331"

on it.

(5).  Familiarize yourself with the proper numbering of the holes

where U1 used to reside.  On the component side of the

circuit board you will see a rectangular shaped pattern

with a notch on one end.  Hold the board so that the

notched end of the rectangle is facing toward you.  Pin

number 1 will be the hole nearest you to the right of the

notch with pin number 16 nearest you to the left of the

notch.  You simply count up in a counter-clockwise fashion

around the rectangle.

(6).  Fashion the 1" piece of bare wire into the shape of a "U".

This wire is installed on the component side of the board

by first feeding one end into U1's pin no. 10 hole and the

other end into U1's pin no. 14 hole.  Leave enough slack in

the wire so that other wires can be soldered into other

nearby holes at a later time.  Solder both ends at this

time.

(7).  This completes the actual board modifications.

(8).  Now it's time to prep the 8-conductor Atari mouse cord.

Cut the cord off right at the Atari mouse strain relief

since the new mouse already has an integral strain relief.

(9).  Strip the outer insulation back leaving about one inch of

lead length.  Strip each of the 8 wires to leave approx-

imately 1/8" of conductor.  Twist the strands together to

keep them from coming apart.  Tin the ends of all 8 leads

with solder.  These tinned wires must be able to fit into

the holes where U1 used to be.

(10). Take note that NOT all Atari mouse cords have the same

color coding scheme.  In the table below I have included

two color code schemes that I have seen in use.  If you see

a black wire use color code 1's column.  If you see a grey

wire then use code 2's column.  Cross out the code column

that does not apply to your mouse cord with a marking pen.

Double check the connections between the keyboard DB-9

connector and the colors against the table below just to be

on the safe side.  The best way to do this is with an

ohmeter.

(11). Refer to the wiring chart to solder each of the designated

8 wires of the mouse cord into U1's holes if there are no

discrepencies with your cord.  Read step 5 of "Mouse

modifications" again if you have forgotten U1's numbering

scheme.

ATARI ST to DEXXA MOUSE WIRING CHART

DB-9      SIGNAL      COLOR    COLOR       MOUSE

FEMALE   DESCRIPTION   CODE 1   CODE 2   IC1 Hole No.

------   -----------   ------   ------   ------------

1 --- Horizontal    Brown    Brown  ----->  5

2 --- Horizontal    Orange   Red    ----->  6

3 --- Vertical      Green    Orange ----->  7

4 --- Vertical      Blue     Yellow ----->  8

6 --- Right Button  Yellow   Green  ----->  2

7 --- +5 Volts      Red      Blue   -----> 13

8 --- Ground        BLACK    GREY   ----->  9

9 --- Left Button   White    White  ----->  3

(12). Be sure that you DO NOT cross-wire the leads to holes

numbered 9 and 13.  Doing so will pretty much toast your

newly modified mouse not to mention the hassles you might

get from the animal rights activists.

(13). All modifications are now complete.  Clip off any excess

lead lengths on the foil side (trace side) of the board

and inspect for any solder bridges between adjacent leads

or pads.

MOUSE PRELIMINARY CHECKOUT AND ASSEMBLY

(1).  Carefully install the completed board into the lower half

of the mouse and work the cord through the strain relief

channel leaving enough slack in the leads to the circuit

board to keep them from breaking off.

(2).  Plug the mouse into the ST and power up making sure that

the power indicator on the computer instantly comes on.

Kill power if you suspect anything wrong and re-check all

of your steps, otherwise, proceed as follows.

(3).  Observe the mouse pointer on the display and see if you

can move it both vertically and horizontally by running

your finger along the rollers inside of the ball cavity.

If you can do this, then it's a pretty safe bet that you

can proceed with the final assembly.

(4).  Ascertain clearance for upper shell nomenclature by

observing the circuit board wiring etc.

(5).  Insert the upper shell locking tabs into the lower shell

tab recesses and close.

(6).  Insert the two screws and tighten.

(7).  Locate ball and insert into cavity.

(8).  Locate the ball locking plate and install.  Lock it into

place by rotating in a clockwise direction.

FINAL CHECKOUT

(1).  Again check operation.  If the mouse pointer moves opposite

the direction of the mouse travel in a vertical direction

then the leads going to U1's hole no. 7 and 8 will need to

reversed.  If horizontal movement is opposite, then leads

to hole no. 5 and 6 need reversing.

(2).  Check to make sure that the left and right buttons operate

as expected.  If they are opposite, then simply swap the

leads to U1 holes numbered 2 and 3.

(3).  Check the mouse pointer travel.  As you move the mouse from

top to bottom and left to right it should move smoothly

without irratic results.  Keep in mind that there is a

limit to the rate at which the mouse can be moved in any

direction without showing signs of irratic movement due to

the limit in speed at which the ST's keyboard processer can

decode the X and Y phases from the mouse.  If you feel that

the maximum speed of smooth travel is less than that of the

original mouse then you may need to optimize the alignment

of optical components LD1, LD2, LQ1, and LQ2.

(4).  You did it!  Yay!  Enjoy.  Order a pizza.

A WORD ABOUT OUR COMPANY

Custom Design Technology started in 1980 under the name

Christian Computer Products.  We started out designing hardware

and software projects for the Radio Shack Color Computer under

the original name.  We became more diverse designing and building

several special projects for television stations in our

surrounding area; one of which was an aural telemetry encoder for

television remote trucks.  Because of this diversity, we made the

decision to change our name.

Custom Design Technology has always utilized Motorola

products in our designs and continues to enjoy the power and

trouble free performance that their microprocessors provide.  We

employ the Atari TT030 exclusively for design and CAD work and

the Hazelwood '020 running Microware's OS-9 for our financial and

record keeping.

If you have a special design request, please call us.  We

consider it a privilege to serve you and our Lord and Savior

Jesus Christ in our business.

Custom Design Technology

751 W. Ewing

South Bend, Indiana  46613

Phone: (219) 234-7754

Serving the Broadcasting Industry for over 15 years.