Here is a helpful hint on using "FLASH" with a Hayes-compatible

modem. If you would like to have your modem automatically re-dial the

number when no "Carrier Signal" is detected, without an overly-long

wait (the default of FLASH is 30 seconds!), then read on...

1.  Determine the "delay" (in seconds) between the dialing

process of your modem and the "carrier detect" signal

(usually CONNECT) that is returned by the BBS.

2.  The Hayes Command Register (S7) is assigned to the time delay

(in seconds) that the modem will wait until it receives

the "carrier signal" before aborting.

3.  Let's assume that you want to log-on to your local BBS,

and that the phone number is 555-1212.

4.  Let's also assume that from experience, you have determined

that it takes about 10 seconds to connect with the BBS

after you initiate the dial command.

5.  Set up the FLASH "Dial Directory" phone number to read:

555-1212;S7=10O

6.  Here is a breakdown of the dial command:

a)  555-1212 -- Auto dial 555-1212...

b)  ; -- The modem goes "off-line" and...

c)  S7=0 -- Loads the modem register "S7" with a

value of 10 seconds (the modem waits for 10

seconds for a response "carrier" from the other

modem)...

d)  O (oh) -- The modem goes back on-line to wait for

the carrier for 10 seconds before "re-dialing".

e)  If a "carrier" signal is recieved by your modem

it will go "on-line" and send a "CONNECT"

message. If not, it will send a "NO CARRIER"

message and look to the FLASH default to find

out how many times you want to "re-dial" the BBS

number before giving up.

7.  NOTE: Be sure that you end the "Dial Directory" command

with a CAPITAL O (oh)... NOT A 0 (zero)! That is what

puts the modem back to "on-line" status!

If you really want to get the best out of FLASH and your modem,

then check out the "QuikCard" quick-reference card designed especially

for FLASH, by "HIRED HAND GRAPHICS" (916 NE 10th Street, Grants Pass,

Oregon 97526).

The card sells for only $7.95 (+$1.50 shipping) direct from the

company, and includes a complete set of the "command-level" & "Alt-key"

commands, as well as a special space right above the function keys,

to enter your "function key macro" labels. They also include two extra

"function key macro" cards for your use.

The cards slip between the top of the function keys and the

computer case, and don't get in the way of keyboard use. They are

printed on sturdy stock, and are scored so that they can be adjusted to

any convenient viewing angle.

The company also offers a "Bonus Pak" of QuikCards that support

the PD word processor "ST-Writer", and the popular "ST-Talk"

terminal program. Both of these QuikCards are in the same package, and

sell for only $5.95 for the pair.

If you can't find these QuikCards at your local dealer, they can

be ordered by mail order from SERIOUS SOFTWARE (probably at a

discount!). SERIOUS SOFTWARE carries only ATARI ST hardware and

software, so know what works best! They can be reached at 1-800-782-7007.

---Warren Tryk (W.TRYK)

################