------------------------------------------------------------------------

PCPDIAL PC Pursuit Dialer v1.4 by michael moncur

------------------------------------------------------------------------

NOTE:  PCPDIAL.PRG, as included here, is a BBS-independent program, and

should NOT be confused with the STadel-specific dialer of the same name.

------------------------------------------------------------------------

This is a PC Pursuit dialer program.  It accepts the outdial and

phone number on the command line.  It should work with any STadel or

<fnord>adel system, or any BBS or terminal program that can run an

external program with a command line.

USAGE:  pcpdial <outdial> <phone> [baud]

examples:  pcpdial UTSLC 261-9372        <--this uses 2400 baud

pcpdial UTSLC 261-9372 1200   <--this uses 1200 baud

Simply placing 1200, or simply 12, in the third argument specifies

1200 baud.  Otherwise, 2400 baud is used.  There are no current

provisions for 300 baud - however, the baud rates only determine which

phone number to use (see below), so you should be able to dial 300-baud

systems using "1200" on the command line, since the phone number is

usually the same.  [This will only work if your BBS program properly sets

the modem baud rate, which <fnord>adel or STadel will.]

Setting up PCPDIAL

------------------

Before PCPDIAL can be used, it needs to know a few things.  Place a

file called PCPDIAL.DAT in the directory that will be current when PCPDIAL

runs (usually your BBS "home" directory) with the following data:

SprintNet phone number (2400 baud)

SprintNet phone number (1200 baud)

Your PC Pursuit user id

Your PC Pursuit password

modem "CONNECT" response(2400)

modem "CONNECT" response(1200)

<optional> path\filename for PCPDIAL log file

PCPDIAL.DAT should contain 7 lines of raw ascii text, as above.  An

example PCPDIAL.DAT file is included with this archive - you can

modify it to fit your needs, or create your own with any text editor

or word processor that will save an ASCII file.

The first two lines of PCPDIAL.DAT should contain the phone numbers

that you use for SprintNet (PC Pursuit) access at 2400 and 1200 baud.  If

these are the same, or if you will only be using 2400, include the same

number on both lines.

Lines 3 and 4 of the file are your user ID and password for PC Pursuit.

Lines 5 and 6 are the strings that your modem returns to indicate a

successful connection - these will usually be CONNECT 2400 and CONNECT

1200.  If your modem is set up to return numeric result codes, the

strings are 10 and 5.  See your modem manual for further details.

Line 7 is the full pathname for a PCDIAL log file, for example

I:\BBS\AUDIT\PCPDIAL.LOG.  If this line is not specified, no log file

will be created.

Setting up your BBS to use PCPDIAL

----------------------------------

[These instructions are specific to <fnord>adel.  STadel should work

the same way.]

Copy PCPDIAL.PRG to your #netdir, and rename it to DIAL_1.PRG (or

DIAL_n.PRG if you use more than one external dialer.)

Set up a network node in the normal manner.  Edit the node and

select an external dialer number (usually 1.)  Change the "Access String"

for the node to the outdial and phone number of the BBS, for example

UTSLC 261-9372 .  If you want PCPDIAL to use 1200 baud, add "12" to the

access string, i.e. UTSLC 261-9372 12 .

NOTE:  Your node definition must be set up with the baud rate you want

to use, as usual.  PCPDIAL does not reconfigure the modem.  For 300 baud

nodes, use "12" as above, but set the baud rate in the node definition

to 300 baud.

Notes / Features / Etc.

-----------------------

If PCPDIAL is unable to find the PCPDIAL.DAT file in the current

directory, it will look for it in the root directory of the current

drive - you may want to put an extra copy there in case the wrong

directory is current when it runs.

The log file (see above) contains the outdial, phone number, date

and time for unusual conditions that occur in PCPDIAL.  These include

problems with SprintNet and the outdial modem, and timeouts during the

dialing process.  If the remote modem returns BUSY or NO CARRIER when

dialing the phone number, this is not logged.  Optional logging for

such events will probably be added in a later version of PCPDIAL.

While PCPDIAL is working, the UNDO key can be used to abort the

dialing process.

If the BBS being called is busy, or doesn't answer, PCPDIAL attempts

to do a "legal" disconnect from the PC Pursuit outdial before hanging

up.  However, if the connection is successful, PCPDIAL returns control

to the BBS software, which will probably just hang up when the network

session is finished.  This hasn't caused any problems yet for me, though.

MANUAL MODE:  If you run PCPDIAL with the word "MANUAL" (all caps,

without the quotes) as the first argument, it will prompt you for an

outdial and phone number.  You can set up a dummy network node with

MANUAL in the access string, and use it to dial any PC Pursuitable

phone number.  Be careful that the node won't be called in a network

session, though - it will sit and mindlessly wait for you to type the

outdial.  Making the node receive-only will prevent this.

If you run PCPDIAL in manual mode by mistake (which is difficult to

do, I hope) just press RETURN to abort.  If you need to use manual mode

at 1200 baud, use MANUAL 1200 as the command line (access string.)

Be sure that you don't use a space within the phone number on the

command line; this will be interpreted as two separate arguments, and

will not be a Good Thing.

pcpdial UTSLC 261-9372   <correct>

pcpdial UTSLC 2619372    <correct>

pcpdial UTSLC 261 9372   <NO, NO, NO.>

----------------------------------------------------------------------

Caveats and Credits

----------------------------------------------------------------------

Although PCPDIAL has been running nightly on my BBS for half a year

in various forms, I'm sure it is not completely bullet-proof.  Let me

know if you have any problems.  The author of PCPDIAL can not be held

responsible for any damage, real or imagined, caused by incorrect or

correct use of this program.  Note that said damage is extremely

unlikely.

This is the first publicly released version of PCPDIAL.  There will

undoubtedly be later versions - I've thought of a few potential features

while writing these instructions.

PCPDIAL is freeware.  It may be freely distributed anywhere and

everywhere, so long as no charge is made for it (aside from BBS connect-

time charges and shareware collections sold for a nominal fee under $5.)

A program called PCPDIAL.TOS was included with some versions of

STadel.  It was an STadel-specific dialer program.  My program, also

called PCPDIAL, is in no way descended from that program, which I did

not write.

Send bug reports, complaints, comments, questions, and large sums of

money to one of the following addresses:

BBS:  The Only BBS, (801) 261-9372.  Accessible by the UTSLC outdial in

PC Pursuit.  Leave mail to "sysop" with any comments.

GEnie:  M.MONCUR

DELPHI:  MONCUR

Internet:  mgm@world.std.com

sysop@only_bbs.via.mind.org

Citanet:  sysop@only_bbs.UT, undermind!only_bbs!sysop,

mast!only_bbs!sysop, C-86 Test System!only_bbs!sysop

US Snail-Mail: PO Box 26426, Salt Lake City, UT, 84126-0426

PCPDIAL 1.4, (c) 1992 by michael moncur.  All rights reversed.

This documentation file will self-destruct in 5 minutes.  Good luck, Jim.