PCPFLASH Version 5.

Do Files for PC Pursuit and Flash v1.52 and up.

Kenneth R Martin - Oct 26, 1987

3615 Cayuga Lane

York, PA 17402

Internet address: kmartin%eclus.DNET%tron.UUCP@umbc3.UMD.EDU

REVISION HISTORY

Revised: Dec 24, 1988 - Simplified the user set up so that only one

file (setup.do) needs to be modified for any user system in addition

to entering your Telenet number in the dial directory. Incorporated some

ideas from Mark Asbell's gwpcpd20 dialer and provided an interface to Mike

Lewis's pas*.prg scanner. Also provided an interface to zmodem and a text

find facility for locating a file containing a text string.

Revised: Nov 22, 1988 - The user interface has been improved and

the files set up for use on a hard drive or floppy drive. User set-up has

been simplified and is achieved my modifications to the first part of the

setup.do file and dial directory.

CONTENTS

--------

1.       INTRODUCTION

2.       INSTALLATION

3.       ENHANCEMENTS

4.       INSTRUCTIONS ON USE

5.       FILE DESCRIPTIONS

5.1      DIAL.DIR

5.2      PCPDIR Folder

5.3      Telenet Links

5.4      Flash Configuration Files

5.5      SETUP.DO

5.6      PCP.DO

5.7      BBS DO Files

5.8      Local and Long Distance Calling

6.       IN SUMMARY

7.       PROBLEMS

8.       SUMMARY OF MODIFICATIONS

9.       FLOPPY DISK SYSTEMS

1. INTRODUCTION

The following Flash do files make operation with PC Pursuit a

breeze. The methods shown here will not work with earlier versions of

Flash. The following files have been arc'ed together and should be tailored

to your needs as explained below. This set-up has been checked out and is

in use daily.

Included are the following files:

PCPFLASH.DOC  - the file you are reading

DIAL.DIR      - dial directory for local and long distance calls

(put in your local Telenet number)

PCPDIR.ARC    - contains the dial directories for PCP nodes

TENET3.DO     - Telenet dial file for 300 baud

TENET12.DO    - Telenet dial file for 1200 baud

TENET24.DO    - Telenet dial file for 2400 baud

FLSHRV3.CNF   - Configuration file for 300 baud

FLSHRV12.CNF  - Configuration file for 1200 baud

FLSHRV24.CNF  - Configuration file for 2400 baud

BREAK.TOS     - Program to break back to Telenet

SETUP.DO      - Set up variables and function keys (add your setups)

PCP.DO        - PC Pursuit dialer. This is the heart of the system

G.DO          - Generic do file for specific BBS's

(use as a template to build do files)

PRCHIP.DO     - Example do file for the Prarie Chip BBS

My set-up uses drive D:\FLASH for Flash and all the do and

supporting files. Drive D:\FLASH\DL is used for downloads and capture

buffer saves. Drive D:\FLASH\PCPDIR contains all the dial directories for

the PC Pursuit cities. Drive B: is used for uploads. Follow the simple

installation instructions to set up to your configuration.

2. INSTALLATION

Decide on your folder structure then put all the files except

pcpdir.arc in the same folder with flash. Create a folder for the PC

Pursuit dial directories and dearc pcpdir.arc into that folder. Now you

make a few changes to only two files and you're ready to roll.

Get into the capture window of Flash and select edit dial directory.

This is the main dial.dir which is in your flash folder. You can use mine

which is included. Just add your non-PC Pursuit local and long distance

phone numbers. Add the Telenet numbers for your area as shown below. Don't

forget the /baud numbers and the @ link to the tnet*.do files listed.

Precede each Telenet name with a space so that these numbers show up first

when the dial directory is pulled up.

i.e.

Telenet/12@tnet12.do     846-6550

Telenet/24@tnet24.do

Telenet/3@tnet3.do       846-6550

MichTron                  1-313-332-5452

QMI@qmi.do                1-315-457-7216

Supra Support             1-503-926-1980

The Prarie Chip@prchip.do 1-307-635-0148

The Rabbit Hutch          1-717-354-5027

The TelephoneCo@teleco.do 1-817-778-2506

Don't forget to save the changes in dial.dir.

The last file to update is setup.do. Load it into your capture

buffer. You're going to put your set-up into the 10 variables A through J.

Don't change any of the punctuation and don't change any other items in

this or any other file. Put your information between the quotes.

i.e.

>** add your own information below|

>*****************************|

>$A "PCP12345"|

>$B "PASSWORD"|

>$C "John Doe"|

>$D "abcde"|

>$E ".....the Lone STranger"|

>$F ".....Take Care,|>pa 1|.....John"|

>$G "d:\flash"|

>$H "d:\flash\pcpdir"|

>$I "d:\flash\dl"|

>$J "b:"|

>*****************************|

The first ten items in this file must be updated with your

information. Replace the area between the quotes (leaving the quotes

intact).

Variable                   Function

-------- -----------------------------------------------------

A      PC Pursuit id

B      PC Pursuit password

C      Name or id used on most bulletin boards

D      Password used on most bulletin boards

E      Handle used for signing messages

F      Name used for signing messages

G      The complete path name for your do files and Flash program

H      The complete path name for the PC Pursuit node directories

I      The complete path name where downloads and capture buffer

saves are to be stored

J      The complete path name from which to obtain files for uploads

That's it, now you're all set. You may want to add the enhancements

listed in the next section but it's not necessary.

3. ENHANCEMENTS

The following files can be found on various bulletin boards and

if used will greatly add to your enjoyment of PCPFLSH. They should be

unarced into the same directory as Flash. My program is set-up to handle

them and no further changes to PCPFLSH are required. The pas*.arc will

need to be updated with your PC Pursuit id and password though. Look for the

following files on your BBS's:

PAS402M.ARC - Selecting SCAN from PCPFLSH will link to pas*.prg.

This particular version has been updated to the latest PC Pursuit

information.

ZMDM.ARC - This set of files is used for Zmodem file transfers.

If you havn't tried Zmodem you should if the BBS supports it. It's

faster and the file name is handled automatically.

TEXTFIND.ARC - This is a useful file if you want to call a BBS but

forget which node it's in. Just invoke the textfind function from within

PCPFLSH, provide the BBS name (or part of the name), and provide the path

name for your PCP dial directories. The program will then search through

all your dial directories and show the directory containing the BBS.\

4. INSTRUCTIONS ON USE

Turn on modem and double click on Flash. Select the Left Mouse

Button (<LM>), and click on the Telenet entry for the desired baud rate.

At the prompt select Node to use the PCPFLSH node dialer, Scan to

use the pas*.prg scanner, or NetExc to dial the PC Pursuit Net Exchange.

If Node was selected, enter the node at the prompt. After

connection click on the BBS desired. If busy, no answer, or other problem

press <LM> and select Same node, New Node or Quit.

After connecting to the BBS, select F9 in response to user name

prompt and F8 in response to password prompt.

The Flash time will switch to the timer and be reset for monitoring

the amount of time on the BBS.

When in the message base select F20 to sign your messages (or F19

if using a handle).

For downloading use F2, F3, or F4 for Xmodem, Ymodem, or Zmodem

respectively. For uploading use F12 (<SHIFT>F2), F13, or F14 for Xmodem,

Ymodem, or Zmodem respectively. After downloading F10 will display the

default directory contents for checking the transfer.

Prior to logging off you may want to search the directories for the

node a particular BBS is located in. Select F5, supply the BBS name, and

path name for the PC Pursuit dial directories. If textfind is installed,

the directories will be searched and the node where the BBS is located will

be displayed.

After logging off a BBS, press F1, supply the name for saving the

capture buffer (Return for no save), then Same Node, New Node, or Quit. The

timer will be changed to indicate real-time.

Same Node will bring you back to the previous dial directory. New

Node will bring you back to the Node, NetExc, or Scan prompt. Quit will

take you to a prompt for Hangup or Terminal.

Terminal will break to the PAD without disconnecting from the node

so you have manual control. You may want to disconnect (D^M) prior to going

back to PCPFLSH with the F1.

Quit will disconnect and hangup Telenet.

For local or non-PC Pursuit calling select <ALT>A and select file

attphone.do for getting into the PCPFLSH system and using its features.

5. FILE DESCRIPTIONS

Now for a walk through of the files:

5.1     DIAL.DIR

Telenet/12@tnet12.do     846-6550

Telenet/24@tnet24.do

Telenet/3@tnet3.do       846-6550

MichTron                  1-313-332-5452

QMI@qmi.do                1-315-457-7216

Supra Support             1-503-926-1980

The Prarie Chip@prchip.do 1-307-635-0148

The Rabbit Hutch          1-717-354-5027

The TelephoneCo@teleco.do 1-817-778-2506

Use your main dial.dir for your local and non-PC Pursuit long

distance calls. Link to a do file for any BBS specific setups like id

and password. Structure these do files like g.do described later.

Your change to this file is to add your local Telenet numbers.

Clicking on one of the Telenet numbers starts the PCPFLSH program.

5.2     PCPDIR Folder

MABOS.DIR (typical of all node directories)

Boston, MA       (MABOS) (617)

!Boston Cmputer Society   396-4607

!Games People Play        471-2270

!Toad Hall BBS            567-8642

Blade Runner              891-7338

Computer Cache            472-1502

Freeport Express          825-3124

GLENDA'S OASIS            770-0026

Harbour Light             472-1503

Metro ST                  536-6974

South Shore Atari Group   331-9640

Syntronics                266-5039

This is typical of directories for the other PC Pursuit nodes.

Each directory must be named with the node name (i.e. NJNEW.DIR). Include

the node name on the first line of the file. This is because FLASH forgets

the name of the dial directory loaded and if an edit is made, when you go

to save it, FLASH defaults to DIAL.DIR in the file selector box and you may

forget which one you edited. With the name in the first line you should

have no problem. If you have a unique id and/or password for a BBS,

generate a do file patterned after g.do, rename it to the BBS unique name,

save it in the Flash (not pcpdir) folder, and @link it to the BBS in the

corresponding "node".dir.

5.3     Telenet Links

These three files take care of the Telenet link up, Flash

configuration, PCPFLSH variables and function key setups, and calling the

pcp.do file which is the main PCPFLSH program.

TNET3.DO

>$N "3"|

>pa 1|

|D|

>wa TERMINAL=|

D1|

>wa @|

>lo co flshrv3.cnf|

>$O "BBS"|

>do setup.do|

>do pcp.do|

>end|

TNET12.DO

>$N "12"|

>pa 1|

|D|

>wa TERMINAL=|

D1|

>wa @|

>lo co flshrv12.cnf|

>$O "BBS"|

>do setup.do|

>do pcp.do|

>end|

TNET24.DO

>$N "24"|

>pa 1|

@D

>pa 1|

|

>wa TERMINAL=|

D1|

>wa @|

>lo co flshrv24.cnf|

>$O "BBS"|

>do setup.do|

>do pcp.do|

>end|

5.4     Flash Configuration Files

These files are for the Flash configurations. The difference is

the baud rate. Other configuration items of interest are:

Strip bit off

Xmodem 1K with loose coupling

ASCII metering off

RS232 set to 8N2

Dial commands set to Racal-Vadic mode

FLASHRV3.CNF

FLSHRV12.CNF

FLSHRV24.CNF

5.5     SETUP.DO

>** add your own information below|

>*****************************|

>$A "PCP12345"|

>$B "PASSWORD"|

>$C "John Doe"|

>$D "abcde"|

>$E ".....the Lone STranger"|

>$F ".....Take Care,|>pa 1|.....John"|

>$G "d:\flash"|

>$H "d:\flash\pcpdir"|

>$I "d:\flash\dl"|

>$J "b:"|

>*****************************|

>$K ""|

>$L ""|

>$M "6"|

>** $N reserved for baud rate code|

>** $O reserved for normal phone, PC Pursuit, or Net-Exchange flag|

>** $P reserved for prompts|

>fk 1 ">do $G\pcp.do|"|

>fk 2 ">dl xm "|

>fk 3 ">dl yb|"|

>fk 4 ">exec $G\rz.ttp -B|"|

>fk 5 ">gem $G\textfind.prg|"|

>fk 6 "c d/$K/$N,$A,$B|"|

>fk 8 "$D|"|

>fk 9 "$C|"|

>fk 10 ">dd|"|

>fk 11 ">ca on|>b1|>rt|>ti|>dp $I|>'$L' '$O'|"|

>fk 12 ">ul xm $J"|

>fk 13 ">ul yb $J"|

>fk 14 ">exec $G\sz.ttp -B $J"|

>fk 19 "$E|"|

>fk 20 "$F|"|

>end|

Variables A-J have been described in the installation section.

Other variables are used as follows:

Variable K is used to hold the PC Pursuit node name.

Variable L is used as a flag in the do file logic to indicate if a

BBS has been connected to or not.

Variable M holds the delay time in seconds for the dialing loops.

Variable N is used to hold the baud rate.

Variable O holds a flag to distinguish between node, net-exchange,

and local/long distance calling.

Variable P is used for prompts such as filename for capture buffer

saves and other short term responses.

The remaining part of the setup.do file sets up the function keys to

perform the following actions:

F1       Use after signing off a BBS to save the capture buffer and

to call the next node

F2       Xmodem downloads

F3       Ymodem downloads

F4       Zmodem downloads

F5       Find file containing text string

F6       Used by the do files in calling an outdial

F8       Your BBS password

F9       Your BBS name or id

F10      Use to show a directory listing of the default directory

F11      Used by the pcp.do file after connecting to a BBS to turn

on the capture buffer,reset and select the timer, ring the

bell, set the default path to the download directory, and

set flag L to indicate BBS, PCP (Net Exchange), or PHONE

(local/long distance)

F12      Xmodem uploads

F13      Ymodem uploads

F14      Zmodem uploads

F19      For signing messages with your handle

F20      For signing messages with your name

When the do file connects to a BBS it executes F11 as a subroutine

turning on the capture buffer, ringing the bell, resetting and selecting

the timer, setting the default path for downloads and capture buffer

saves, and setting a variable (L) for logic control of the do files. This

variable indicates that a connection has been made to a BBS.

After logging on to a BBS, F9 will send your user name (or id) and F8

will send your password.

F2 is used for xmodem downloads and F12 for xmodem  uploads. Enter

the file name at the prompt. F3 is used for ymodem batch downloads and F13

is used for ymodem batch uploads. F4 is used for Zmodem downloads and F14

is used for Zmodem uploads. Note that some BBS's when referring to Ymodem

really mean Xmodem 1-K which is on F2 and F12. Also note that if a BBS

only supports Xmodem CRC, you don't have to change Flash. It will

automatically drop back. Not so for uploads however. Zmodem requires the

file zmdm.arc.

Selecting  F10 will provide a listing of your default directory.

Prior to logging off a BBS, if you know the name of the BBS you want

to connect to next but forget which node it's on, hit F5 (requires

textfind.arc) and supply the BBS name and PC Pursuit directory pathname.

The directory containing the BBS will be displayed.

After logging off a BBS, hit F1. This executes a do file which

closes and saves the capture buffer, changes the default path back

to FLASH and returns to the prompt for Same Node, New Node, or Quit.

5.6     PCP.DO

This is the heart of the system. Pressing F1 usually will get you

back into the program at the correct point because of the internal flags.

>if L  jmp CAPSAVE|

>ifi $O "PHONE" jmp DIAL|

>L: TELENET|

>type "#E||#b1#f                              PCP FLASH Version 5.0 #b3||"|

>type "                                15 December 1988||"|

>al "[0][          Select            ][ Node | NetExc | Scan ]" $P 1|

>type "#e"|

>ifi $P "NetExc" jmp NETEXC|

>ifi $P "Scan" jmp SCAN|

>L: MENU|

>ty "#E 201-NJNEW 216-OHCLE 312-ILCHI 408-CASJO 415-CASFA 617-MABOS 713-TXHOU 801-UTSLC|"|

>ty " 202-DCWAS 301-DCWAS 313-MIDET 414-WIMIL 503-ORPOR 618-MOSLO 714-CACOL 813-FLTAM|"|

>ty " 203-CTHAR 303-CODEN 314-MOSLO 415-CAOAK 602-AZPHO 619-CASDI 714-CASAN 815-ILCHI|"|

>ty " 206-WASEA 305-FLMIA 404-GAATL 415-CAPAL 612-MNMIN 703-DCWAS 718-NYNYO 816-MOKCI|"|

>ty " 212-NYNYO                                                             817-TXDAL|"|

>ty " 213-CALAN                                                             818-CAGLE|"|

>ty " 214-TXDAL                                                             913-MOKCI|"|

>ty " 215-PAPHI                                                             916-CASAC|"|

>ty "                                                                       919-NCRTP|"|

>$K ""|

>qu "||        Enter Node Access Code|           (return to quit)" $K|

>if ~K jmp QUIT|

>fk 6 "c d/$K/$N,$A,$B|"|

>type "#E|"|

>L: OUTDIAL|

>go 6|

>wa "@" $M CONNECT|

>jmp OUTDIAL|

>L: CONNECT|

>b1|

I|

>pa 1|

ATZ|

>wa OK|

@|

>wa @|

SET 1:0,7:8|

>wa @|

CONT|

>L: LOADDIR|

>$O "BBS"|

>lo di $H\$K.dir|

ATZ|

>wa OK|

#|

>wa *|

>L: DIAL|

>type "#E#b1Connected to  $K:#b3|||"|

>di|

>if ~di jmp NEXT|

>go 11|

>end|

>L: NETEXC|

>$O "PCP"|

>fk 6 "C PURSUIT|"|

>** These SETs are per the instructions on PCPNET|

SET 0:0,57:1,63:0|

>wa @|

>L: LOOP|

>go 6|

>wa "CONNECTED" $M LOOP|

@|

>wa @|

SET 4:1,2:0|

>wa @|

CONT|

>go 11|

>end|

>L: SCAN|

>exec pas*.prg|>open pasnode.dat|>inp $K|>open blk:|

>b1|

I|

>pa 1|

>jmp LOADDIR|

>L: CAPSAVE|

>ca off|

>qu "|Enter ""filename"" for capture buffer|(return for no-save)" $P|

>if ~P jmp NOSAVE|

>sa ca $P|

>ca cl|

>L: NOSAVE|

>dp $G|

>fk 8 "$D|"|

>fk 9 "$C|"|

>$L ""|

>ifi $O "PCP" jmp TELENET|

>ifi $O "PHONE" jmp NEXT|

I|

>pa 1|

ATZ|

>wa OK|

#|

>wa *|

>L: NEXT|

>cl|

>type "#E"|

>al "[1][|     Destination        ][Same Node|New Node|Quit]" $P 1|

>ifi $P "Same Node" jmp DIAL|

>ifi $O "PHONE" jmp QUIT|

>exec break.tos|

>ifi $P "New Node" jmp DISCONNECT|

>al "[0][                  ][Hangup|Terminal]" $P 1|

>ifi $P "Terminal" jmp END|

>L: DISCONNECT|

>pa 1|

D|

>pa 2|

>ifi $P "New Node" jmp TELENET|

>L: QUIT|

hangup|

>pa 2|

>hg|

>lo di $G\dial.dir|

>lo co $G\flash.cnf|

>lo fk $G\flash.def|

>L: END|

>end|

The dial delay is set for 6 seconds ($M in the setup.do file). If

Telenet is particularly slow you may want to increase this redial delay.

Ok, you'll get an alert box asking for the Node, Net-Exc, or Scan.

If you select Net-Exc the PC Pursuit Net-Exchange will be dialed. Scan

will call the PAS*.prg (if it's installed) for scanning multiple nodes.

Node will present an alert box for entering the node you want. All the

nodes and their telephone area codes are displayed for reference. Suppose

you want to call the Toad Hall BBS in MABOS, just enter MABOS and the

dialing loop starts.  It will loop every $M seconds until connected to the

node. Then the  bell sounds, the node modem is reset and the dial directory

for the correct node is displayed. You also arrive at the same point by

using Scan. At this point select the desired BBS.

If you can't get past the busy just press UNDU to break out of the

node loop and press F1 to continue or if in the BBS dial loop, press the

left mouse button . A prompt for Same node, New node, or Quit next appears.

Same Node will bring up the same dial directory for selecting another BBS.

New node will disconnect from the current node and revert back to the Node,

Net-Exc, and Scan prompt. Quit will go to a prompt for Hangup or Terminal.

Hangup will disconnect from the node, hangup from Telenet, and restore

the normal Flash configuration. Terminal will exit the do file so you can

manually control the outdial modem. To re-enter the do file, press F1.

After you log off a BBS, select F1. This will return to the pcp.do

file and prompt you for a filename for saving the capture buffer. Next you

will get the prompt for Same node, New node, or Quit as explained above.

The file break.tos is automatically called when required to break back to

the Telenet @ prompt.

The pcp.do file takes care of setting your default paths depending

on whether or not you have connected to a BBS or have logged off. You may

also note that the clock is commanded by the do file. After connecting to

a BBS the timer is activated and reset. At any time you can tell at a

glance how long you have been on a BBS. After logging off and pressing F1,

the time reverts back to real-time (so you are reminded it's really 2 AM

in the morning!).

5.7     BBS DO Files

G.DO

Here's a generic do file for setting a BBS specific id and password.

Link it to BBS names in your directories as required.

>** this is used as a template for do files|

>** add your default username or id|

>fk 9 "user name|"|

>** add your default password|

>fk 8 "password|"|

PRCHIP.DO

Here's an example of the DO file for a long distance call. Notice

it's the same as for the PCP calls. This is an interesting and fun BBS run

by a real nice gal (Calamity Jane). Tell her the Nite-Owl sent you.

>fk 9 "123456|"|

>fk 8 "junk|"|

5.8      Local and Long Distance Calling

ATTPHONE.DO

You can also use many of the features of PCPFLSH on local and long

distance calls. Features such as capture saves, default path management,

and timer/clock management are available even if not calling via Telenet

and PC Pursuit. To use PCPFLSH in this mode get into it by executing the

file attphone.do. This is accomplished by pressing <ALT> A and selecting

attphone.do. Once in pcp.do you can continue with the non-PC Pursuit

service or follow the prompts to break into Telenet and PC Pursuit.

>$O "PHONE"|

>do setup.do|

>do pcp.do|

6.                          IN SUMMARY

----------

1.  Execute your FLASH program.

2.  Double click on TELENET in the DIAL directory.

3.  Select Node.

4.  Enter node at prompt.

5.  When dial directory appears, click on desired BBS.

6.  Hit F9 at user name prompt.

7.  Hit F8 at password prompt.

8.  To see what files are on your default directory hit F10.

9.  To d/l a file (xmodem), hit F2 and enter filename.

To u/l a file (xmodem), hit F12 and enter filename.

10. To d/l a file (ymodem batch), hit F3.

To u/l a file (ymodem batch), hit F13 and enter filename.

11. To d/l a file (zmodem), hit F4.

To u/l a file (zmodem), hit F14 and enter filename.

(requires zmdm.arc de-arced into Flash directory)

12. To put a signature on your messages, hit Shift F10 (or Shift F9 if

using a handle).

13. To search a directory of files for a string, hit F5, supply the string,

and the path.

(requires textfind.arc de-arced into Flash directory).

14. After logging off a BBS, hit F1, give name for saving capture buffer,

then select same node, new node, or quit.

15. To call a BBS on the same node just select from displayed dial

directory.

16. To disconnect and call another node enter new access code.

17. To abort a call to a busy node and select another, hit UNDO then F1.

18. To hang up from Telenet, select QUIT at the prompt.

7.                           PROBLEMS

--------

1. None I know of.

8.                     SUMMARY OF MODIFICATIONS

------------------------

DIAL.DIR       Add your own Telenet number and your favorite non-PC Pursuit

BBS local and long distance numbers.

SETUP.DO      Var A      add PC Pursuit id

B      add PC Pursuit password

C      add typical BBS user name

D      add typical BBS password

E      add handle signature

F      add typical name signature

G      add do file path name (For floppy disk users this

might be A:, A:\FLASH, etc.)

H      add PC Pursuit directories path name (Floppy users

might use A:, A:\PCPDIR, A:\FLASH\PCPDIR, etc.)

I      add path name to hold downloads (For floppy disk

users this may be A:, A:\DL, A:\FLASH\DL, B:, etc.)

J      add path name for uploads (Floppy disk users might

use A:, A:\ul, B:, etc.)

9.                       FLOPPY DISK SYSTEMS

-------------------

See Summary of Modifications above and pay particular

attention to SETUP.DO (variables G ,H, I, and J).