ST-Keep System Files Format Reference

Copyright 1993 by Andrew P. Studer

Updated:  26Nov92

* WARNING *  This file contains information critical to the security

of your BBS, do not hand it out freely.

NOTE:  Use this information with extreme caution, I will not be

responsible for loss of data due to misuse or inaccuracy of this

document.

////////////////////

/// STK_INFA.SYS /// This file contains various defaults for the BBS.

////////////////////

Key:   * = DO NOT MODIFY THIS LINE, SYSTEM CRITICAL INFORMATION

$ = CHANGE ONLY IF YOU MOVE THE CORRESPONDING FILES

Line #  Description

------  -----------

1     Maximum baud rate code:

0 = 19200    1 =  9600    2 =  4800

4 =  2400    7 =  1200    9 =   300

192 = port locked at 19200

2  *  Maximum userlog size

3  *  Maximum number of rooms

4  *  System flags - bit 0 = real names only, no handles

1 = append new users if userlog full

2 = allow deletion of msgs in room 4 and

the system activity room?

3 = allow users to create rooms

4 = install new user accounts

5 = time bank enabled

5  *  System flags - bit 0 = secure mode

1 = modem off-hook on local logon

2 = verbose file transfers

3 = unused

4 = unused

5 = status line @ logon

6 = chat mode disabled?

7 = bbs disabled?

6     Number of calls since system initialization

7     Chat exec path/filename

8  *  Maximum number of messages per room

9     System Password

10  $  System files pathname (BBS.PRG & *.SYS files)

11  $  Blurb files pathname (*.BLB)

12  $  Help files pathname (*.HLP)

13  $  Bulletin files (*.BU?)

14  $  Vote files (*.VOT, *.ANS)

15  $  Drive/path/filename of file transfer executable pathname (XYZ.TTP)

16  $  Pathname for temporary files

17     Post-logon exec door number

18     Number of records in STK_CALL.SYS

19     Door to run if key in line 28 is pressed, 0 for door menu

20  *  BBS Name (do not change this if you are registered)

21     Modem initialization string:

Requires - ATS0=1 to answer on the first ring

ATS2=255 to disable remote control of the modem

DCD should not be set to always on

22     System-wide miscellaneous comment field

23     Default time limit, in minutes, for new users, 255 = unlimited

24     Default download ratio for new users, 255 = unlimited

25     SysOp's first name

26     SysOp's last name

27     Logoff Exec door number

28     ASCII code for hot-key to auto-door (line 19)

29     Room number where ST-Keep will save download messages, 0=room 3

30  $  Drive/path/filename of STZIP.PRG for verbose listing

31  $  Drive/path/filename of LHARC.TTP for verbose listing

32     Password to bypass disabled bbs

33     Minimum baud rate to call system

34     Minimum baud rate for file transfers

35     Maximum minutes on-line per day

36  $  Message base path

37     Time-out in minutes

38     Use file selector (0=off, 1=on)

39     New user defaults A (use BBS_CNFG.PRG to modify)

40     New user defaults B ("")

41     New user screen length (0-25)

42     New user emulation 0-5

43     New user tabs 0-20

44     New user file xfer protocol (0-4)

45  $  Terminal program path/filename

46     Serial number (obtain from Andrew Studer)

////////////////////

/// STK_INFB.SYS /// Contains message base information

////////////////////

* WARNING *  Never edit this file or you will destroy the integrity

of your message base.

Line #  Description

------  -----------

1     Total number of bytes written to the message file

2     Oldest message number in the message file

3     Number of the next message to be written to the message file

4     File offset location to write the next message

////////////////////

/// STK_INFC.SYS /// Contains ST-Keep color palette data

////////////////////

32 bytes of information, 2 bytes for each color register (0-15) used

by the ST's setpalette() routine (XBIOS #6).

////////////////////

/// STK_HDRS.SYS /// Room header information

////////////////////

Each line in this file contains the room header for the relative room

number.  Line 1 contains the header for the room 1, line 2 for room 2 (Mail),

etc.  [R][I] shows the room number you are in.  If used, this file

must contain a number of lines equal to the maximum number of rooms

in your system with blank lines for rooms without headers.

///////////////////

/// STK_ARC.SYS /// Room archive filenames

///////////////////

Each line in this file contains the pathname/filename of each room

that is being archived.  If used, this file must contain a number of

lines equal to the maximum number of rooms in your system with blank

lines for rooms that aren't being archived.

///////////////////

/// STK_GRP.SYS /// Group names

///////////////////

15 lines, each containing the name of the relative group.  If the

group is unused the line must be blank.

////////////////////

/// STK_MODR.SYS /// Room Moderator names

////////////////////

A number of lines equal to your total maximum rooms, each line contains the

username of the Moderator for the valid room.  It is blank if there is no

moderator.

////////////////////

/// STK_ROOM.SYS /// Room data

////////////////////

This block of data is repeated a number of times equal to the maximum

number of rooms.

Offset    Description

--------  -----------

1 - 30   Name, space padded to the right

31 - 60   Directory pathname, space padded to the right

61        Generation number, 0=inactive, 1-255 gen

62        First flag

63        Second flag

64        Group number, if non-zero this room is only available to

users of this group

65        Floor number that it exists on

The following two fields are repeated for each message in the room.  If

your system has 50 messages per room, this group of bytes will be repeated

50 times before the next room information structure begins.

66 - 69   Message number

70 - 73   Message location

~~ - ~~   Repeated for each message in this room

~~   This block repeated for each room

////////////////////

/// STK_USER.SYS /// User account information

////////////////////

This block of data is repeated a number of times equal to the maximum

number of user accounts.

Offset    Description

--------  -----------

1 - 20   Password

21 - 50   Username

51 - 70   Personal Comment field

71 - 83   Phone Number

84        Access Level

85 - 86   Number of calls (hi/low)

87 - 88   Last call date (hi/low MS/DOS format)

89 - 90   Last call time (hi/low MS/DOS format)

91 - 100  Users birth date (mm/dd/yyyy format)

101       Unused

102       Minutes per day

103       Minutes left today

104       Screen width

105       Userflag A

106       Userflag B

107 - 117 Unused

118 - 199 Download count (hi/low)

120 - 121 Upload count (hi/low)

122       Upload protocol

123 - 124 Number of messages written (hi/low)

125       Download ratio

126 - 127 Unused

128       Graphic emulation (0=ascii, 1=vt52, 2=ansi)

129 - 130 Group bits (0-15)

131       Tab spaces (0+)

132 - 134 Unused

135 - 164 Real Name

165 - 184 City, State

The following two fields are repeated a number of times equal to the

maximum number of messages allowed in each room.  These are user mail

pointers.

185 - 188 Message number

189 - 192 Message location

~~~ - ~~~ Repeated for each message in the mail room, even empty messages

The following two fields are repeated once for every room on the BBS.  This

includes unused room slots.

193       Generation number for this room

194 - 197 Last message read in this room

"     "  Next user structure