This is an amateur radio RTTY program for the Atari ST.

Capabilities:

1.	BAUDOT (60,	75, & 100 WPM)

ASCII (110 & 300 Baud)

2.  Variable size split Screen

Drag the MOVE bar on the transmit window to change size

Type ahead to transmit window during receive.

3.  Write Sessions to disk

Received and transmitted characters are written to disk

in the order they occur on the air.

4.  Read disk files into transmit buffer.

5.  Desk accessory support.

NOTE--received characters are written to receive window

even while desk accessory is active.  This works OK as

long as the desk accessory is running in a GEM window because

RTTY uses GEM to determine if its window is obscured by

another window.  However, GEM forms (e.g. the standard file

selector form) are not windows and are not protected from

being overwritten on the screen by the RTTY program.

6.  Limited line editing for transmitting.

(delete back to	beginning of current line)

7.  Drop down menus are used for all functions except:

F1 switches unconditionally and immediately to

receive mode, and sets RTS off.

F2 switches unconditionally and immediately to

transmit mode, sets RTS on, and starts

transmitting characters from the transmit

buffer.  When there are none, it holds the

modem in the MARK state (no diddling).

F3 inserts a character in the transmit buffer

(displayed as a greek letter phi) that is

not transmitted, but switches back to receive

mode when it is reached.  Any characters following

the F3 character remain in the transmit buffer

and will be sent when F2 is selected again.

F4 allows deletion of all characters in the transmit

buffer (after a confirmation).  The deleted

characters remain in the transmit window in

the display, however.

NOTES:

RTTY.PRG and RTTY.RSC must reside in the same folder.  The

program exits immediately if the resource file is missing.

The ASCII/Baudot conversion is from the 1982 ARRL Radio

Amateur's Handbook, the chapter on Specialized Communications

Systems.  The FIGS code used is the US Figures (versus the

CCITT No 2 Figures).  The Bell character (FIGS 5) does not

ring a bell, but prints a diamond character.  When transmitting

Baudot, the conversion from ASCII is done as characters are

removed from the type ahead buffer.  Characters that do not

translate into Baudot are simply discarded at that time, even

though they appear on the transmit half of  the screen.

I use the NOTEPAD desk accessory from COMPUTE!  April, 1987

Atari ST disk if I need a real text editor to compose for

transmission, which makes up somewhat for the limited

editing capability of the RTTY program itself.  This is

the main purpose of the "SWITCH RECORDING..." menu entry.

It allows you to close the file currently being recorded,

open a new one for recording, and then NOTEPAD can open

the old one for inspection, or retransmitting text if you like.

I used Megamax C compiler and resource editor to develop

this program (but you don't need a C compiler to run it).

This program was written and tested on an Atari 1040ST

with single disk drive and monochrome monitor.  My RTTY modem

is a Heathkit HD3030 and my rig is a Kenwood TS-440S.

I don't know whether this program will work on any other

hardware although I think it should.  Specifically, the

color monitor could be a problem. I'd like to hear from anyone that

tries this program on a color monitor, as well as from anyone that

finds this program useful at all.  Please send me a QSL at:

1805 Mayfield

Round Rock, Texas 78681