Telephone Monitor Documentation

Version 1.00

by Lonny Pursell and ENCOM

FreeWare

(C)1993 by Lonny L. Pursell and ENCOM

All Rights Reserved.

Note: Please skip to the end of this file for revision information.

Purpose

-------

This little program records phone calls while you are away.  It's sort

of a poor mans version of caller ID.  :^)  Some people don't have caller

ID in their area (like me) or perhaps can't afford the feature even if it

was offered.  Simply run this little program and it will record the date,

time, and number of rings per call to your phone while you are away.

Configuration

-------------

The program must be configured specifically for your systen due to the

many different combinations of compters and modems.  To configure the

program you need to create a 5 line ASCII file called TELE_MON.INF and

place it in the same directory as the program.

Config file format: 5 lines of ASCII text and each line ending with CR/LF

Example below: (These are also the defaults if the file is not found.)

6

2400

ATZ

6

CON:

Explaination of each line:

Line 1: Serial port your modem is connected to.  The MegaST and TT have

4 serial ports therefore the program needs to know which one to

watch.  This line must be present in the config file even if

you have only one serial port, therefore use the value 6.

Valid settings are:

6 for Modem port 1 (ST compatible port) [default]

7 for Modem port 2

8 for Serial port 1

9 for Serial port 2

Line 2: Baud rate.  Valid settings are 300, 600, 1200, 2400, 4800, 9600,

and 19200.  [default=2400]

Line 3: Modem initialization string.  This varies from modem to modem

so refer to your modems manual for the proper settings.

[default=ATZ]

Line 4: Calibration factor.  This value determines the amout of time

between calls in seconds.  This is how the program counts calls.

You can try different values until you find one that works.

A value of 6 works on my phone and unless the time between rings

varies from system to system, then you may have to tweak the

value.  [default=6]

Line 5: Output device.  Valid settings are:

CON: for screen output [default]

PRN: for printer output

d:\file (must be a full path like C:\CALL_LOG.TXT)

Running the program

-------------------

This program configures the serial port, therefore you don't need

to load the Atari Control Panel.  Make sure your modem is turn on before

you run the program or it may not be initialized properly.

Run the program and leave.  It's that simple!  Use the sample

TELE_MON.INF file as an example if you have any problems.  You should

turn your monitor off before leaving to avoid image burn in.  If you

decided on printer output make sure your printer is on-line and hopefully

you trust it enough not to jam while you are away.  It should work in any

resolution, but obviously the more lines of text you can display the

better for screen output.  All output is completely directed to the

output device you specified in the config file.  This means if you

selected a disk file or the printer then nothing will be displayed on the

screen when the phone rings.

Note:  This program probably won't work if you have any other programs

resident that intercept or attempt to read incoming serial data.

This includes terminal programs that work as desk accessories.

About the program

-----------------

This program was written in GFA-Basic v3.6TTe on an Atari TT030.

It should run on any ST compatible machine in theory.  This program was

tested on my phone system with my computer by calling my own phone number

and allowing the phone to ring several times.  If you press the ALTERNATE

key you can simulate the phone ringing once to test the program or just

to see how it works.

Support

-------

This program is FreeWare.  Should you find that the program fails to

function as advertised please contact me.  Please report any bugs or

ideas you might have for a feature.

I can be reached via the following methods:

U.S. Postal Service...      Lonny L Pursell

PO Box 145

Pandora  OH  45877

USA

Voice...                    Telephone: (419)-384-3594 after 4:00 PM [est]

Modem...                    CodeHead Quarters BBS

(213)-461-2095 [1200/2400/9600]

username: Lonny Pursell

Credits

-------

Atari, GEM, ST, and TT are registered trademarks of Atari Corp.

GFA-Basic is a registered trademark of GFA Systemtechnik

All other items mentioned are trademarked by their respective owners.

Disclaimer

----------

There are NO implied warranties on this program, use at your own risk!

The author is NOT responsible (in no way shape or form) for lost data

or damaged hardware due to the use or misuse of this program.

Not for sale!  This progam must remain unmodified along with this

doc file if it is to be distributed via PD libraries or uploaded to

any type of media storage system.

Revision history

----------------

v1.00  First release, hopefully bug free.

Atari Rules!

--EOF--