Ymodem Batch Transfer Desk Accessory

Ver 1.1, 14 May 1987

by David B. Chiquelin

Note:  This program is released into the public domain, however

portions of it are Copyright 1985 by OSS and CCD.

This program runs in medium and high resolutions only and was written

in OSS Personal Pascal.

This is a pretty simple desk accessory, but if you call FoReM ST BBSs

and use Flash to do so, you are missing out on the 'better' file

transfer methods.  A very nice one is Ymodem Batch.  You simply mark

the files you want to receive, then select Ymodem Batch download, and

the BBS will send them all, and the receiving terminal program will

save them on your disk with the same names as on the BBS.  Easy!

Ymodem Batch is a nice way to D/L files (you can only D/L using Ymodem

Batch with this accessory, and also on FoReM ST BBSs.)  FoReM ST

supports _many_ other transfer methods, and to get the most from a

FoReM ST BBS you should use ST-TERM.  This is especially true if you

have an HST modem, as the Fmodem Batch file transfer method is 25%

faster with the HST than Ymodem Batch.

But, as I said, Flash (which seems to be the terminal program of choice

for a lot of users) does not support Ymodem Batch.  Until now that is!

If you boot your ST with YBATCH.ACC you will see the item Batch Ymodem

has been added to the Desk drop-down menu.  First - how to exit the

accessory if you should accidentally select it or just need to get out

of it.  Press the Undo key and wait until you hear the bell.  You

should also see the message that it is aborting.  Whenever you hear the

bell ringing (once/second) press ANY key and the desk accessory will

disappear.

How to use it.

Call your favorite FoReM ST BBS.  (I know you must have one, all the

better BBSs use FoReM!)  You can use any terminal program that allows

you to select the desk accessory, or, in a pinch you can use the VT-52

accessory to call the BBS.  When you are in one of the file sections of

the BBS, type a B to enable Batch file selection.  This is important,

as the desk accessory will not do normal Ymodem D/Ls!  The BBS  should

tell you that the batch modem is enabled.  If it tells you otherwise,

press it again as it was already on.  Now list the files.  (Don't scan,

you must List to be able to mark the files for batch D/L.)  You will

see one file description at a time and if you want to D/L it type a B

to mark it for batch D/L.  If you don't want to D/L it, press <RETURN>

to see the next file.  (The N for next is not required as Next is the

default.)  When you are ready to D/L the files selected (1 to xxx

files, depending on the system set-up) go back to the file menu where

Ymodem Batch is listed.  Tell the BBS to send them using Ymodem Batch

by typing a Y.  When the BBS says it is ready to send, press your RIGHT

mouse button to get to the capture window.  Now select the Ymodem Batch

desk accessory and sit back!

The files will be saved with the name as they were on the BBS.  They

will be saved to whatever the default drive/directory is.  (ie., if you

are running Flash on Hard drive C, within folder FLASH, the files will

be saved in FLASH on drive C.  If you are running FLASH from drive A,

the files will be saved on drive A.)  If a duplicate file is on the

destination drive it will be replaced by the new file.  If you run out

of room on the disk or if there is a write error the D/L will be

aborted.

When all files selected have been received the bell will ring at 1

second intervals until you press ANY key.  The desk accessory will

disappear and you will be back 'live' in the capture window of Flash

(if using Flash of course).  Press the right mouse button again to get

back to the main window.

The program has been tested at up to 9600 bps using Flash with the

expanded RS-232 buffer.  I did not test it using the VT-52 emulator at

9600 bps but feel it would probably lose some data at that speed with

the small RS-232 buffer.

How to use with the VT-52 emulator.

Run the VT-52 emulator accessory, set up the RS-232 parameters (baud

rate, etc) and then call the FoReM ST BBS.  Mark your files to be

received, and when you get the 'Ready to send' cue, exit the accessory

by pressing Undo.  Now select the Ymodem Batch accessory and sit back.

When completed, press ANY key to get back to the desktop and then re-

select the VT-52 accessory.

To abort the transfer at any point, press Undo.  If when you are back

in your terminal mode the BBS is not responding, type a <CONTROL X>.

Changes for ver 1.1

Ver 1.1 will now D/L files that have the underscore character in the

name.  It has also been modified and tested on a non-FoReM ST BBS (non-

Atari ST) that has Ymodem Batch and should work now on all systems with

the Ymodem Batch option.  Aborting if there is a write error has also

been added, as well as the abort sequence being modified so you should

not have to type a <CONTROL X> after aborting a D/L.  If you call a

non-FoReM ST BBS and D/L using Ymodem Batch and the BBS does not send

the file size with the name you should see ???? as the file size and

the file will be saved with the padding added to the end of the last

block.  The block count in the accessory is also set to match what

FoReM ST BBSs send, and may show too low if non-FoReM ST BBSs are

called, but it will not affect the D/L.

I hope you enjoy this accessory and that it makes your modeming a

little more pleasant.

David Chiquelin

SysOp, Atari-O! BBS

(402) 592-4435

300/1200/2400/9600 (HST)

GEnie - D.CHIQUELIN

CIS - 71336,1443

Delphi - DCHIQUELIN