A-Real Time Clock Version 3.60

Copyright (C) 1988,89,90,91 by

by Bill Penner

Are  you one of the many users who set the clock whenever you turn

your ST computer on?  Maybe the process takes more effort than it seems

to be worth and so you don't use the internal clock.  Did you know that

when  the  ST creates a new file on the disk, the date and time is also

saved?  Have you had a disk with many files you had created on it, only

to  forget which file was the most current version?  If you had set the

clock,  all  you  would have to do is to check the date and time on the

file.

Now  you can have a simple hardware and software combination which

will  automatically  set the time and date.  Software is provided which

will  set  the  clock and display the time and/or date.  Even if you do

not  have  a  module,  the  accessory  can still be used to display the

time.

Do you have need for two different times?  Well, version 3 has the

answer.   Support has been provided to support multiple clocks (up to 6

in the computer and 4 in a cartridge).  The clock in use can be changed

by  either  using the configuration program or by setting up disks with

different AREAL36.INF files.

THE HARDWARE

The  hardware  required  for  this  modification  is  the  DS1216E

SmartWatch  made by Dallas Semiconductor Corporation located in Dallas,

TX.   The DS1216E is a module which contains a clock chip, crystal, and

a lithium battery.  The lithium battery should be good for more than 10

years  and  the  module  is accurate to within 1 minute per month at 25

degrees  C.   It  is  designed to be placed in a read only memory (ROM)

socket  and  then  the  ROM plugged into the module.  The module is the

same as various clock modules which you may have seen available for the

PC compatible machines, usually for $49.

The module is available from many sources, but the best prices are

generally  from commercial electronic supply stores.  To find a dealer,

you  can  call  Dallas Semiconductor at (214) 450-0400.  Make sure that

the  module  is  the  E version as the C or unlettered version will not

work  for this application.  The C and unlettered versions are designed

for  low power static random access memory (SRAM) sockets.  The typical

price  for  the  DS1216E SmartWatch module is less than $30 (and lately

less than $25).

THE SOFTWARE

Three  programs  are  part  of  the A-Real Time Clock 3.6 software

package.     The   programs   are:   AREAL36.ACC,   AR36BOOT.PRG,   and

AR36_CNF.PRG.   The  software was written to allow operation in several

configurations.   The  accessory  program, AREAL36.ACC provides support

for systems with or without the Dallas Semiconductor Clock Module.  The

accessory  is  used to display the time on the screen.  The name of the

accessory  must not be changed, else the time setting and configuration

program, AR36_CNF.PRG, will not totally work.  The program AR36_CNF.PRG

allows  the  user  set  the  time  in the clock module and to configure

several  parameters  dealing  with the clock display.  The program will

create an .INF file.  If you do not wish to use the accessory, but wish

to  set  the  clock  on powerup, place the AR36BOOT.PRG file in an AUTO

folder.

AREAL36.ACC  is an accessory for setting the system clock from the

clock module and displaying the time.  To use the desk accessory, place

the  program  on  the  disk on the top level (not in a folder) and then

boot  the  disk.  The accessory will then be loaded in and useable.  To

display  the time, click on the A-Real Clock 3.0 accessory.  To disable

the clock, click on the accessory again.  The accessory will reload the

module time into the system clock at least once an hour.  If you change

the  time  using the CONTROL accessory, the time in the module will not

be  changed.   When  the  accessory  does the hourly update or the user

turns  on  the clock display, the system time will then be changed back

to  the  module  time.  To set the time in the module, you will have to

use  the AR36_CNF.PRG program.  If you have multiple clock modules, you

can  set  the  default  module  by using the CNF program and writing an

AREAL36.INF  file.   The AREAL36 program first looks for an AREAL36.INF

file  and  reads  the  module  address.   If a clock is located at that

address,  then  the  system  time  is  set  and  the  time used for the

accessory.  If no clock exists there, then a search is made of each ROM

position  from  $FA0000  to $FE0001.  If no modules are found, then the

system time is not changed and the system time is loaded for use by the

accessory program.

AR36BOOT.PRG  is  an  alternate  way of setting the clock.  If you

cannot  use  an accessory (either you have too many already or need the

memory  space),  you  can  place  the  AR36BOOT.PRG  program in an AUTO

folder.   When  you  boot  the  disk, the system time will be set.  The

program  can  also be run directly when desired, to set the time to the

module time.

AR36_CNF.PRG  is  a program for use in setting the module time and

configuring the AREAL36.ACC accessory.  When the customizer is started,

following  a  credits screen, you will be presented with a dialog box.

The  module  time can be set by changing the entry in the time and date

box  and then clicking on the Make Settings button.  If the time box is

unaltered,  the  time  will  not  be  written to the module.  The clock

configuration  is set by setting the various parameter buttons and then

clicking  on  the Make Settings button or pressing [RETURN].  Four time

and  display  layouts  are  supported.  The layouts are time only, date

only,  time  and  date shown alternately, or time and date shown on two

lines.   To set the time only, turn the time button on, the date button

off,  and the two line button off.  To set the date only, turn only the

date  button on.  If you wish to have the time and date alternate, turn

the time and date buttons on and leave the two line button off.  If you

want  the  time and date to be shown on two separate lines, turn on the

two  line  button.   The time can be shown in either 12 hour or 24 hour

format.   The  12  hour  format is selected when the button is on.  The

position  of  the clock can be changed by changing the X and Y values.

The  X  value  is  the character position of the left most digit on the

screen (0-79 for high or medium resolution and 0-39 for low resolution)

and  the Y value is the screen raster line from the top (0-399 for high

resolution  and  0-199  for color).  The update rate, the rate at which

the  display  is  written to the screen, can also be altered.  For most

applications  the  update rate should be set at 1000 ms (1 second), but

for  programs  which  change  the  screen  often,  the  number  may  be

decreased.  The minimum practical number (screen updated every vertical

blank  interupt) is 14 for high resolution and 16 for color.  The clock

color can be changed.  The color for the clock is shown in a box to the

right  of the Color: statement.  Two boot options are configurable when

the  program  boots up.  The clock can be set to automatically start up

when  the program boots and a double bell sound can be made to indicate

the  accessory  has  been  loaded.  Once the desired settings have been

made and loaded into the clock, the option save config will then create

a  file called AREAL36.INF.  A .INF file can be loaded by selecting the

Save  Config  button  and then placing the disk in the boot drive.  The

settings  from the file are displayed on the dialog screen, but are not

loaded  into the clock until the Make Settings button is selected.  Not

all  settings  make  sense  in all cases, but the program will make the

best guess available.  If the X or Y values are too large, they will be

limited  to  the  right side or the bottom of the screen.  If the color

selected  is  not available, the color will default to black.  Multiple

clock  modules  are supported.  Upon booting up, the program will first

attempt  to  get  the settings of the AREAL36.ACC.  If the accessory is

not  loaded, the system is searched from the from the cartridge through

the  operating  system  ROMS and the first clock found is used.  If you

wish  to  interact  with a different module or make the accessory use a

different  module, single click on either the up or down arrow near the

module  address  box.   The  program  will  attempt  to  find  the next

available  clock.  If no other clocks are found, the current entry will

return.   The clock address can be forced (who know's why I added this)

by double clicking on either the up or down arrow.

READ BEFORE ATTEMPTING TO INSTALL!

Version  3.0  software  supports  modules  in  any  of the six ROM

sockets, or in any of the four cartridge positions.  If you do not want

to  disassemble  your  computer,  the  module  can  be  placed in a ROM

cartridge and plugged into the cartridge port.

The  author  assumes  no  responsiblity for installation or damage

occurring   from  installation  of  the  A-Real  Time  Clock.   If  the

instructions  are  followed carefully, the installation should be quick

and  simple.   The  installation  requires  disassembly of the case and

shield,  removing  a  ROM  chip,  placing the module in the socket, and

replacement  of  the  ROM  chip.   If  you  have one of the early 520ST

machines,  you  may  have  to  desolder a couple of tabs which hold the

shield  on.   If  you  do  not  feel  confident in performing the above

operations, take it to someone who is, such as a computer repair center

or your local ATARI dealer.

CARTRIDGE INSTALLATION

In  addition to purchasing the module, you will need to purchase a

standard  ATARI  ROM cartridge or other suitable cartridge for 2764 (64

Kbits)  or  larger ROM.  One source for the standard ATARI cartridge is

Best Electronics, San Jose, CA for about $15.

Unfortunately,  the  module will not simply fit in the cartridge.

One  of  two things will have to be done to get the module to fit.  The

prefered  method  is  to remove one of the sockets in the cartridge and

then solder the module in place of the socket.  The alternate method is

to  cut  the  case  of the cartridge to provide space for the module to

fit.

INTERNAL INSTALLATION

IF  YOU DO NOT FEEL CONFIDENT IN DISASSEMBLING YOUR COMPUTER, TAKE

IT TO SOMEONE WHO IS!

Installation  in either the 520ST (no internal disk drive) or some

of the newer 520STFM and 1040STFM machines is the easiest.  If you have

an  older  520STFM  or  1040STFM,  the ROMs are located under the power

supply.   For  systems  with  the ROMs under the power supply, you will

have  to  cut a piece out of the power supply bracket to allow room for

the  module  and the ROM chip or make a short jumper using a 28 pin dip

header  made  from  cutting  up a 40 pin header, a 28 pin socket, and a

short piece of 28 pin ribbon cable.

1.   Remove the screws on the bottom of the computer.

2.    Turn the computer over (bottom down, be careful not to loose

the screws).

3.   Lift the cover off, leaving the keyboard on the computer.

4.   Carefully unplug the keyboard connector which is plugged into

the  mother board near the upper right corner of the keyboard.  You may

have to remove the tape holding the keyboard cable to the shield.

5.   Lift the keyboard out and place to the side.

6.    If  you  have an STFM (internal disk drive) carefully remove

the power supply (rear left side) and the disk drive (rear right side).

To  disconnect  the disk drive, remove the 34 pin connector at the disk

drive  (remember  which  direction the connector is connected) and then

remove the four pin power connector.

7.    For the following steps, sit down and do not get up and walk

around  during  the following steps! The circuitry is very sensitive to

static  electricity  and  walking around can build up enough charges to

destroy  your  computer.   If  you have a static wrist strap, use it by

connecting  the  clip on to a shield on the board or to a ground on the

computer.   You  do  not  have  to  have the strap, but without it, you

increase  the  chance  of  causing  damage to your computer.  The wrist

straps are available from Radio Shack.

8.    Remove  the  shield.  Across the front of the shield, either

three screws must be removed or the tabs desoldered.  Some of the first

520ST  had  the  tabs soldered down, but most have screws.  Eleven tabs

connecting  the  top  shield  to  the  bottom  shield  will  need to be

straightened.   Nine  of  the  tabs  are located around the edge of the

shield,  and two can be found in holes near the back of the case.  Lift

the shield and place to the side.

9.    Using  a screwdriver, CAREFULLY remove one of the ROM chips.

The  ROMs  are  the  28  pin chips located either under the power power

supply  (left  side,  near  the  back)  or on the left side.  Recently,

machines  coming  from  ATARI  have  been coming with two 1 Megabit ROM

chips instead of six 256Kbit chips.  If you have one of the Megabit ROM

machines  (two  ROMs  and  four  empty sockets), place the module under

either  one  of the ROMs.  DO NOT PLACE THE MODULE IN AN EMPTY SOCKET.

Pick  a ROM chip which interferes the least with the shield or anything

else.  If you have the 256Kbit ROM machine, you can place the module in

any  socket.   Using a screwdriver, carefully remove the ROM by placing

the  screwdriver  under  the chip and rotating the screwdriver to cause

the spade of the screwdriver to lift the chip.  Lay the chip down on an

anti-static pad (or on the shield if one is not available).

10.   Remove the module from the package, and plug the module into

the ROM socket.  Make sure the notch on the module is pointing the same

direction  as the notch located on the other ROM chips.  All of the ROM

chips  should  be pointing the same direction, however, I have heard of

cases  where  the  notch  on  one  the  sockets  was pointing the wrong

direction.

11.    Reinstall  the  ROM  chip into the socket on the top of the

module  with  the  notch pointing the same direction as the rest of the

ROM  chips  (same direction it was when you removed it).  Make sure all

of  the pins are inserted into the socket, as it is pretty easy to fold

a pin or have a pin bend away from the socket.

12.    Get  some electrical tape and locate where the shield could

touch  the  module.   Place  tape on the shield to prevent the ROM pins

from  possiblibly  shorting to the shield.  If you fail to do this, you

may  not  get  a  chance  to  correct  the  problem later (poof! smoke,

smoke).

13.    Replace  the  shield  and  bend  the  tabs  as found during

disassembly.   The  shield  may  be lifted slightly near the module for

some  installations.   You can get the tab to hold by pressing down the

shield slightly (the reason for the tape).

14.    If  you have an STFM, replace the power supply and the disk

drive,  making  sure  the  34  pin  disk  drive  connector  is properly

installed.

15.  Replace the keyboard plug.  The plug has one hole plugged and

can be installed only in one direction.

16.  Replace the case top, turn over, and replace the screws.

Installation  is  now  complete.  Power up the computer and verify

that it will boot up.  If it does not, disassemble and carefully verify

the module and chip are plugged in correctly and that the keyboard plug

is  installed  correctly.   If  you  want  to  test the computer before

placing  the  case on the computer, make sure you attach the keyboard.

Start the AR36_CNF.PRG program and set the time.  The time clock should

now operate correctly.

SOFTWARE DETAILS

The source code was written to be compiled using the Megamax Laser

C  version  1.1  compiler.   If  another  compiler  is to be used, some

registers  required by the compiler to execute C code during a vertical

blank may have to be saved and restored.

The  clock  module  does  nothing  until  a special sequence of 64

addresses are read from the ROM in the clock module.  Once the sequence

is  completed,  the ROM is disabled and the module becomes active.  The

address line A3 is used to tell the module if a read or write operation

is desired.  The address line A1 contains the data if a write operation

is  executed.   Data  line  D8 contains the data if a read operation is

performed.   To read a data bit from the module (assuming the module is

located  at  $FE0000 low byte), read location $FE0008, to write a 0 bit

to the module read location $FE0000, and to write a 1 bit read location

$FE0008.

To  activate  the  module,  execute  a read operation to reset the

module,  and  then  write  the value $5CA36AC5 beginning with the least

significant bit, out to the module twice.  At this point, the module is

enabled and the ROM is disabled until a sequence of 64 read or 64 write

operations are performed to read the time from the module or to write a

new  time  into the module.  According to the module specification, the

chance  of  an accidental sequence causing the module  to  be activated

is  less than one in 10,000,000,000,000,000,000.  If you write your own

software to access the module, be sure you disable all interupts during

this  period of time.  Some of the interupts which occur need to access

some  code  in  the  ROM on the module.  If an interupt occurs when the

module  is  activated  and  the  ROM  disabled,  bad data will be read,

generally resulting in a system crash or a lock up.

The  clock  display  is accomplished by writing directly to screen

memory  using  the built in character font.  When the accessory starts,

the  routine  vbi_init()  is  executed.  The routine locates the proper

character  font,  dimensions  of the font, and then sets up a couple of

resolution  dependent  variables.  To start the vertical blank routine,

the routine vbi_install() is executed.  When a vertical blank occurs, a

user   C   routine   called  vbimain()  is  executed.   For  the  clock

accessories,  the  vbimain() routine determines what the time is and if

it    is    time   to   print   the   clock   display.    The   routine

vbi_line(x,y,p,tcol) is executed to write the time character string, p,

to the raster position x, y, using the color register number tcol.  The

vbi_line  routine  was  included  since  the  Cconout()  type routines,

including  printf,  will  not  work  in  during  a VBI.  To disable the

vertical  blank  routine,  the routine vbi_remove() is executed.  These

five  routines,  a  user  routine  vbimain(),  and the global variables

indicated  for  use  with  the VBI routines shown in the source are all

that is required to include a vertical blank routine in your programs.

The  configuration  program  uses  the  GEM message pipe system to

communicate  with the AREAL36.ACC program, if it is loaded.  GEM allows

processes to talk to each other through the same mechanism used to tell

the  programs  when  mouse  operations  occur,  buttons pressed, and so

forth.   The  commands  which were built into the accessory are ones to

enable  or  disable  the  clock display, or to enable the clock display

after  loading a new configuration, or to return the address of a table

of  current  parameters.   The  program uses a dialog box, but does not

require  a  .RSC  file.   The  .RSC file data is built in to the source

code.

CONCLUSION

I hope that you will find the information and software useful.  If

any  problems  with  the  program are found, I would appreciate hearing

about it so that I can try to correct the problem.

Bill Penner

3235 Wright Avenue

Bremerton, WA 98310

VERSION 3.60 INFORMATION

The  major  change  with this version over previous versios was to

add STe compatiblity.